3GPP TSG RAN WG3 NR AdHoc	R3-172199

Qingdao, China, 27th -29th June 2017

Agenda item:		10.10.1

Source:	Huawei.

Title:					TP on F1 interface management to 38.470

Document for:		pCR

Text proposal for 38.470

A	Procedures of the F1 interface

## A.B	Control plane procedures

A.B.x	Interface Management procedures

A.B.x.y1	Reset procedure

The purpose of the Reset procedure is to re-initialize the peer entity or part of the peer entity after node setup and after a failure event occurred. This procedure is initiated by both the gNB-DU and gNB-CU. The receiving node releases the corresponding references and resources and replies with an acknowledgement to confirm that the resources and references are cleared.

A.B.x.y2		Error Indication procedure

The Error Indication procedure is initiated by the gNB-DU and the gNB-CU, to report detected errors in one incoming message, if an appropriate failure message cannot be reported to the sending entity.

A.B.x.y3	F1 Setup procedure

The F1 Setup procedure is used to exchange configured data which is required in the gNB-CU and in the gNB-DU respectively to ensure a proper interoperation. The F1 Setup procedure is triggered by the gNB-DU. The gNB-CU responds to indicate the successful or unsuccessful outcome of the procedure.