3GPP TSG RAN WG3 Meeting NR AdHoc	R3-17xxxx

Qingdao, China, 27th – 29th June 2017

Agenda item:	10.3.2

Source:		KT Corp.

Title:	TNL address discovery for 5G network

Document for:	Discussion and Decision

Introduction

Many candidate solutions with regards to the TNL address discovery for NR SN in NSA (Option 3 and 7) network has been proposed (e.g., [1], [2], and [3]). In this contribution, we would like to further discuss it by considering 5G network migration aspects.

Discussions on TNL address discovery for 5G network migration

NG-RAN may consist of NR gNBs and/or LTE eNBs. Especially, in NSA with Option 3 family architecture, LTE eNB as master node and NR gNB as a secondary node are interworked with EN-DC through X2 interface and LTE MN is connected to EPC through S1 interface. Note that the TNL address discovery for NR SN through CN is not feasible because there is no S1-C interface between MME and NR SN.

Candidate solutions proposed in [1] are as follows:

- Connection of eNB and gNB is configured by O&M

- Some eNBs serve as proxy towards MME

- gNB connected to MME for TNL address discovery purpose

- Encrypted broadcasting of X2-C TNL address to UE

- TNL address discovery through inter-CN-inter-system interface

Since operators have different deployment and migration path for 5G, we should also consider these for selecting a solution for the TNL address discovery.

Proposal 1: Selecting and specifying solutions for the TNL address discovery shall consider 5G deployment and migration path.

Firstly, we think that Solutions of 2), 3) and 4) can be excluded due to their specification or implementation complexity.

Secondly, it is expected that many operators prefer the Option 3 deployment at the initial phase or Option 3 + Option 2 mixed deployment by introducing more NR gNBs and 5GC. Moreover, new inter-CN-inter-system interface, Nx, will be specified between EPC and 5GC for interworking at core network level.

Figure 1 shows 5G network migration from Option 3 (NSA-only) to Option 3/7 + Option 2 (NSA and SA mixed). In general, different types of commercial 5G UEs will co-exist and can be served by 5G network. That is, the support of NR-5GC UEs (single-mode), LTE and NR-EPC UEs (dual-mode), and LTE and NR-5GC UEs (dual-mode) is assumed. Note that LTE UEs that can connect to 5GC are excluded here.

Figure 1: A 5G network migration from NSA to NSA+SA.

Therefore, the solution of TNL address discovery through inter-CN-inter-system interface is relatively simple and shall be preferred when 5GC becomes available

Thirdly, in case network is migrated to the 5G network supporting both the Option 7/3 and Option 2 by adding Xn and NG interfaces from Option 3 network, the TNL address discovery though 5GC becomes feasible by using Xn setup. From operators’ perspective, although it is undesirable that Option 3 and Option 7 based on 5GC are co-operated due to operational and maintenance inefficiency, both the options shall be simultaneously supported to serve LTE and NR-EPC UEs as well as LTE and NR-5GC UEs.

Finally, Solution 1) that requires neighbour eNBs information through X2 setup can also be a promising solution in case NSA Option 3 network is deployed.

Proposal 2: Both the Solutions 1) and/or 5) for the TNL address discovery shall be supported.

Proposals Summary

Having discussed above, it is proposed that RAN WG3 is kindly asked to reflect the followings onto the relevant specifications.

- Proposal 1: Selecting and specifying solutions for the TNL address discovery shall consider 5G deployment and migration path.

- Proposal 2: Both the Solutions 1) and/or 5) for the TNL address discovery shall be supported.

References

- R3-171150, Option 3: TNL address discovery and inter-NR-node signalling

- R3-171865, TNL address discovery for Option 3/3a/3x

- R3-171606, Discussion on gNB-eNB interface setup