3GPP TSG-RAN WG3 NR#2 AdHoc                                                                                                R3-172451

Qingdao, China, 27– 29 June, 2017

Agenda item:	10.10.3

Source:	CMCC

Title:	Text Proposal for Intra-DU Inter-Cell Mobility

Document for:	pCR

TP for 38.401

- We provide a TP for 38.401 to capture the proposals in R3-172450.

Start of Text Proposal for TS 38.401

- 8.2	Inter-DU Mobility

Editor Note: This subclause shows overall procedures for inter-DU mobility.

Editor Note: The name of this subclaues is FFS.

- 8.x Intra-DU Inter-Cell Mobility

Figure X: CU takes decision and DU generates UE handover command

0.  Cell Measurements are configured and collected by CU;

- Based on collected measurement reports, CU makes handover decision and sends to DU the UE context modification request, including target cell information, and possibly new configuration for the UE; part of new UE configuration, e.g. neighbour cell information, may be sent to UE via later RRC connection reconfiguration as in step 5;

- Upon reception of CU message, DU commands to UE the intra-DU inter-cell handover decision, e.g. via MAC. This command mainly includes mobility control information, e.g., target cell identity.

- Upon reception of handover command from DU, UE releases source cell configuration and if instructed, performs random access procedure towards target cell;

- Upon reception of MAC command acknowledgement from UE, DU releases source cell dedicated configuration and responds to CU UE context modification response;

- CU further sends to UE the necessary RRC configuration, e.g. measurement configuration information;

- The UE applies the new RRC configuration, and replies with RRC connection reconfiguration complete message;

End of Text Proposal for TS 38.401