3GPP TSG-RAN WG3 Meeting #ad-hoc2	R3-172501

Qingdao, P.R. China, 27th – 29th June 2017

Agenda Item:	10.1

Source:	Ericsson

Title:	TP for Data forwarding at handover

Document for:	Discussions & Approval

# 1	Introduction

This paper provides TP for data forward in stage 3 TS 38.420

<<<<<<<<<<<<<<<<<<<< Begin Text Proposal >>>>>>>>>>>>>>>>>>>>

### X.Y.Z	Data Forwarding at handover

To ensure lossless intra-system mobility the PDCP-SN status will be transferred from source to target node and that the packets that have already been assigned a PDCP SN in the source should be transmitted to the target on a per-bearer basis, which implies creating one GTP tunnel per each bearer over the Xn interface. On the other hand, a single GTP tunnel per PDU session can be used to transfer the new incoming IP packtes in downlink (DL) direction, i.e., the packets that have not received a SN in the source node. An example of data forwarding over the Xn is provided in Figure 1.

Figure 1: Example of data forwarding over Xn.

For data forwarding between soure and target over the Xn:

(1) One GTP tunnel per bearer is utilized to transmit the packets that have already been assigned a SN in the source;

(2) One GTP tunnel per PDU session is utilized to transmit new incoming IP packets in downlink.

The solution for lossless intra-system HO, i.e. lossless, in-sequence and without duplication to upper layers is to use the same QoS-flows to DRBs mapping in the source and target nodes

<<<<<<<<<<<<<<<<<<<< End Text Proposal  >>>>>>>>>>>>>>>>>>>>