3GPP TSG-RAN WG3 AH #2	Tdoc R3-172521

Qingdao, P. R. China, 27th – 29th June 2017

Agenda Item:	10.10.3

Source:	Ericsson

Title:	TP for Xn handover in disaggregated gNB

Document for:	pCR

# Introduction

This contribution contains a text proposal to capture the proposal in R3-172520.

# TP for TS 38.401

Start of Text Proposal for TS 38.401

# 8	Overall procedures in disaggregated gNB deployment

Editor Note: This clause shows the overall procedures including signalling flows in gNB-CU / gNB-DU deployment e.g., initial access from the UE, mobility etc. The main focus is on gNB-CU and gNB-DU while also UE and 5GC are shown for understanding the interaction.

## 8.1	UE Initial Access

Editor Note: This subclause shows overall procedures for initial access from the UE.

Editor Note: The name of this subclause is FFS.

## 8.2	Mobility

This subclause presents the Xn handover in the disaggregated gNB deployment. In the Xn handover the UE changes both the gNB-DU and the gNB-CU. It is the most comprehensive of the Xn handover procedures. The other handover procedures can be logically derived from the procedure illustrated in this section. The Xn handover call-flow is shown in Figure 8.2-1.

- Fig. 8.2-1: Xn handover in the disaggregated gNB deployment.

In Figure 8.2-1, the following terminology is utilized to describe the entities involved in the procedure:

- S-gNB: source gNB

- T-gNB: target gNB

- S-DU: source gNB-DU

- S-CU: source gNB-CU

- T-DU: target gNB-DU

- T-CU: target gNB-CU

The call-flow can be logically divided into three phases.

-	The first phase is the Xn handover preparation:

1.	The UE sends a RRC MEASUREMENT REPORT to the S-DU. The event that triggers the measurement report depends on the UE measurement configuration.

2.	The S-DU employs the F1AP UL RRC TRANSFER message to forward the RRC measurement report to S-CU.

3.	The S-CU takes the handover decision, based on the content of the RRC measurement report. The S-gNB (involving the S-CU) sends the XnAP HANDOVER REQUEST message to T-gNB (T-CU), passing necessary information to prepare the handover at the target side.

4.	The T-CU performs admission control, creates a UE context, and identifies the T-DU. The T-CU configures the higher-layers, including security configuration and QoS-flows to DRB mapping. Then, the T-CU sends the F1-AP UE CONTEXT SETUP REQUEST message to the T-DU, including UE context information and CU-UL-TEID for data radio bearers.

5.	The T-DU performs admission control, configures the lower-layers, and creates a local UE context (including a C-RNTI for the UE). Then, the T-DU sends the F1-AP UE CONTEXT SETUP RESPONSE message to T-CU, including lower-layers configuration, C-RNTI, and DU-DL-TEID for data radio bearers.

6.	The T-gNB (T-CU) sends the XnAP HANDOVER REQUEST ACKNOWLEDGE to the S-gNB (S-CU). The XnAP HANDOVER REQUEST ACKNOWLEDGE message includes a transparent container to be sent to the UE as an RRC message to perform the handover (includes C-RNTI). The XnAP HANDOVER REQUEST ACKNOWLEDGE message may also include RNL/TNL information for the data forwarding tunnels (if applicable).

-	The second phase is the Xn handover execution:

7.	The S-CU sends the F1AP UE CONTEXT MODIFICATION REQUEST message to the S-DU. The F1AP UE CONTEXT MODIFICATION REQUEST message includes a transparent container that carries the RRC message to perform handover, which was generated by the T-CU and ciphered and integrity protected by S-CU. The F1AP UE CONTEXT MODIFICATION REQUEST message contains a specific indication to inform the S-DU the UE Context medication is triggered due to handover.

8.	The S-DU sends the F1AP UE CONTEXT MODIFICATION RESPONSE message to the S-CU to confirm that the handover is accepted.

9.	The S-DU sends the RRC CONNECTION RECONFIGURATION MESSAGE to the UE.

10.	 The S-gNB (S-CU) sends the XnAP SN STATUS TRANFER message to the T-gNB (T-CU) to convey the uplink PDCP SN receiver status and the downlink PDCP SN transmitter status.

11.	Data forwarding may start from S-gNB (S-CU) to T-gNB (T-CU) (if applicable).

12.	UE sends a RANDOM ACCESS preamble to the T-DU. It may employ a dedicated RACH preamble if that was included in the RRC connection reconfiguration message.

13.	The T-DU sends a RANDOM ACCESS RESPONSE to UE.

14.	The UE sends the RRC CONNECTION CONFIGURATION COMPLETE to T-DU (include the C-RNTI to identify the UE).

15.	The T-DU employs the F1AP UL RRC TRANSFER message to forward the RRC connection reconfiguration complete message to T-CU.

16.	The T-gNB (T-CU) sends a NGAP PATH SWITCH REQUEST message to The AMF to inform that the UE has changed cell. The AMF should then contact the SMF/UPF to modify the PDU session (not shown).

17.	The AMF confirms the NGAP PATH SWITCH REQUEST message with the NGAP PATH SWITCH REQUEST ACKNOWLEDGE message.

-	The last phase is the resource release in the source:

18.	The TgNB (T-CU) sends the XnAP UE CONTEXT RELEASE message to the S-gNB (S-CU). The XnAP UE CONTEXT RELEASE message informs the S-CU that the handover was successful.

19.	The S-CU sends the F1AP UE CONTEXT RELEASE message to the S-DU, to allow the T-DU to release the resources allocated to the UE.

End of Text Proposal for TS 38.401