3GPP TSG-RAN WG3 AH #2	Tdoc R3-172523

Qingdao, P. R. China, 27th – 29th June 2017

Agenda Item:	22.2

Source:	Ericsson

Title:	TP for SgNB addition in disaggregated gNB

Document for:	pCR

# Introduction

This contribution contains a text proposal to capture the proposal in R3-172522.

# TP for TS 36.401

Start of Text Proposal for TS 36.401

8.3 Dual-connectivity operation

8.3.1 SgNB addition

This section presents the SgNB addition procedure. The SgNB addition call-flow is shown in Figure 8.3.1-1.

The following terminology is utilized to describe the entities involved in the procedure:

-	MeNB: master eNB

-	SgNB: secondary gNB

-	DU: secondary gNB-DU

-	CU: secondary gNB-CU

The UE is initially connected to E-UTRA. The call-flow for adding the secondary gNB node encompasses the fowling steps.

1.	The UE sends a RRC MEASUREMENT REPORT to the MeNB. The RRC MEASUREMENT REPORT indicates the presence of an NR cell with good radio quality. The event that triggers the measurement report depends on the UE measurement configuration.

2.	The MeNB sends the X2AP SgNB ADDITION REQUEST message to the SgNB (CU). The MeNB requests radio resources for specific E-RABs, indicating E-RAB characteristics (E-RAB parameters, TNL address information corresponding to bearer type). The MeNB can provide the latest measurement results for the SCG cell(s) requested to be added.

3.	The CU performs admission control, creates a UE context, and identifies the DU. Then, the CU sends the F1-AP UE CONTEXT SETUP REQUEST message to the DU, including UE context information and CU-UL-TEID for data radio bearers.

4.	The DU performs admission control, configures the lower-layers, and creates a local UE context. Then, the DU sends the F1-AP UE CONTEXT SETUP RESPONSE message to the CU, including lower-layers configuration, and DU-DL-TEID for data radio bearers.

5.	The SgNB (CU) sends the X2AP SgNB ADDITION REQUEST ACKNOWLEDGE message to the MeNB. For SCG and SCG split bearers, the X2AP SgNB ADDITION REQUEST ACKNOWLEDGE message may provide the S1 DL TNL address information for the respective E-RAB and security algorithm, for split bearers together with X2 DL TNL address information.

6.	The MeNB sends the RRC CONNECTION RECONFIGURATION message to the UE.

7.	The UE applies the new configuration and replies with the RRC CONNECTION RECONFIGURATION COMPLETE message to the MeNB.

8.	The MeNB sends the SgNB ADDITION RECONFIGURATION COMPLETE message to SgNB (CU) to indicate the success of the procedure.

9.	The UE sends a RANDOM ACCESS preamble to the DU.

10.	The DU replies with a RANDOM ACCESS RESPONSE to UE.

11.	The MeNB sends the X2AP SN STATUS TRANSFER message to the SgNB (CU) for data forwarding (if applicable).

12.	For SCG and SCG split bearers, the MeNB sends the E-RAB MODIFICATION INDICATION message to the MME to update the UP path.

13.	The MME confirms the new UP path by sending the E-RAB MODIFICATION CONFIRMATION message to MeNB.

14.	and 15: UL and DL data transmission.

- Fig. 8.3.1-1: SgNB addition in option 3 with a disaggregated gNB deployment.

End of Text Proposal for TS 36.401