| 3GPP TS 36.133 V18.2.0 (2023-06) |  |
| --- | --- |
| Technical Specification¦Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	92

1	Scope	94

2	References	94

3	Definitions, symbols and abbreviations	96

3.1	Definitions	96

3.2	Symbols	98

3.3	Abbreviations	99

3.4	Test tolerances	102

3.5	Additional notation	102

3.5.1	Groups of bands	102

3.5.1A	Groups of bands for satellite access	104

3.6	General	105

3.6.1	Applicability of requirements in this specification version	105

3.6.1.1	Applicability of requirements for UE capable of network-based CRS interference mitigation	111

3.6.1.2	Applicability of requirements with CRS muting for category M1 UE capable of CRS muting	113

3.6.1.3	Applicability of requirements with CRS muting for category M2 UE capable of CRS muting	114

3.6.2	Applicability of requirements for EN-DC operation	115

3.6.3	Applicability of requirements for NE-DC operation	116

3.6.4	Applicability of requirements for NGEN-DC operation	117

4	E-UTRAN RRC_IDLE state mobility	117

4.1	Cell Selection	117

4.2	Cell Re-selection	117

4.2.1	Introduction	117

4.2.2	Requirements	117

4.2.2.1	Measurement and evaluation of serving cell	119

4.2.2.2	Void	120

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	120

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	121

4.2.2.5	Measurements of inter-RAT cells	124

4.2.2.5.1	Measurements of UTRAN FDD cells	124

4.2.2.5.2	Measurements of UTRAN TDD cells	125

4.2.2.5.3	Measurements of GSM cells	127

4.2.2.5.4	Measurements of HRPD cells	128

4.2.2.5.5	Measurements of cdma2000 1X	129

4.2.2.5.6	Measurements of NR cells	130

4.2.2.5.7	Measurements of NR cells subject to CCA	133

4.2.2.5.8	Measurements of NR cells for RedCap	134

4.2.2.6	Evaluation of cell re-selection criteria	136

4.2.2.7	Maximum interruption in paging reception	137

4.2.2.8	void	137

4.2.2.9	UE measurement capability	137

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	138

4.2.2.10	Reselection to CSG cells	138

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	139

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	139

4.2.2.11	Void	140

4.2.2.12	Void	140

4.2.2.13	Void	140

4.2.2.14	UE measurement capability for RedCap	140

4.3	Minimization of Drive Tests (MDT)	141

4.3.1	Introduction	141

4.3.2	Measurements	141

4.3.2.1	Requirements	141

4.3.3	Relative Time Stamp Accuracy	141

4.3.3.1	Requirements	141

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	142

4.3.4.1	 Requirements	142

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	142

4.3.5.1	Requirements for timeSinceFailure	142

4.4	MBSFN Measurements	142

4.4.1	Introduction	142

4.4.2	MBSFN RSRP measurements	142

4.4.3	MBSFN RSRQ measurements	143

4.4.4	MCH BLER measurements	143

4.5	Proximity-based Services	143

4.5.1	Introduction	143

4.5.2	Requirements	143

4.5.2.1	Interruptions with ProSe Direct Discovery	143

4.5.2.2	Interruptions with ProSe Direct Communication	143

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	144

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	144

4.6	Cell Selection and Re-selection Requirements for UE category NB1	145

4.6.1	Cell Selection	145

4.6.2	Cell Re-selection	145

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	145

4.6.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	146

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	147

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	148

4.6.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	150

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	151

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	152

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	153

4.6.2.7	Maximum interruption in paging reception in normal coverage	155

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	155

4.6.2.8	UE measurement capability	155

4.6.2.9	WUS receptions for NB1	155

4.6.3	Requirements for transmission using preconfigured uplink resources for UE category NB1	156

4.6.3.1	Introduction	156

4.6.3.2	Requirements on UE synchronization for transmission using PUR	156

4.6.3.3	Requirements on TA validation for transmission using PUR	156

4.6A	Cell Selection and Re-selection Requirements for UE category NB-IoT for Satellite Access	157

4.6A.1	Cell Selection	157

4.6A.2	Cell Re-selection for UE category NB-IoT for Satellite Access	157

4.6A.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	158

4.6A.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	159

4.6A.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	160

4.6A.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	162

4.6A.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	164

4.6A.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	165

4.6A.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	167

4.6A.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	168

4.6A.2.7	Maximum interruption in paging reception in normal coverage	170

4.6A.2.7A	Maximum interruption in paging reception in enhanced coverage	171

4.6A.2.8	UE measurement capability	171

4.6A.2.9	WUS receptions for NB1	171

4.6A.3	Requirements for transmission using preconfigured uplink resources for UE category NB-IoT for Satellite Access	172

4.6A.3.1	Introduction	172

4.6A.3.2	Requirements on UE synchronization for transmission using PUR	172

4.6A.3.3	Requirements on TA validation for transmission using PUR	172

4.7	Cell Selection and Re-selection Requirements for UE category M1	172

4.7.1	Cell Selection	172

4.7.2	Cell Re-selection	172

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	173

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	173

4.7.2.1.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in normal coverage	173

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	174

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	176

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	178

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	178

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	179

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	179

4.7.2.2.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in enhaned coverage	180

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	181

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	184

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	185

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in enhanced coverage	186

4.7.2.3	WUS receptions for UE category M1	186

4.7.3	Channel quality report for UE Category M1 in idle mode	186

4.7.4	Requirements for transmission using preconfigured uplink resources for UE category M1	187

4.7.4.1	Introduction	187

4.7.4.2	Requirements on UE synchronization for transmission using PUR	187

4.7.4.3	Requirements on TA validation for transmission using PUR	187

4.7A	Cell Selection and Re-selection Requirements for UE category M1 for Satellite Access	188

4.7A.1	Cell Selection	188

4.7A.2	Cell Re-selection for UE category M1 for Satellite Access	188

4.7A.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	189

4.7A.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	189

4.7A.2.1.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in normal coverage	191

4.7A.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	192

4.7A.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	195

4.7A.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	196

4.7A.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	197

4.7A.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	197

4.7A.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	197

4.7A.2.2.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in enhaned coverage	199

4.7A.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	201

4.7A.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	203

4.7A.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	205

4.7A.2.2.5	Maximum interruption in paging reception for Category M1 UEs in enhanced coverage	206

4.7A.2.3	WUS receptions for UE category M1	206

4.7A.3	Channel quality report for UE Category M1 in idle mode for Satellite Access	206

4.7A.4	Requirements for transmission using preconfigured uplink resources for UE category M1 for Satellite Access	207

4.7A.4.1	Introduction	207

4.7A.4.2	Requirements on UE synchronization for transmission using PUR	207

4.7A.4.3	Requirements on TA validation for transmission using PUR	207

4.8	Idle State Positioning Measurement Requirements for UE category NB1	208

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	208

4.8.1.1	RSTD Measurement Reporting Delay	209

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	210

4.8.2.1	RSTD Measurement Reporting Delay	211

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	212

4.8.3.1	RSTD Measurement Reporting Delay	213

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	214

4.8.4.1	RSTD Measurement Reporting Delay	215

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	216

4.8.5.1	Measurement Reporting Delay	217

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	217

4.8.6.1	Measurement Reporting Delay	218

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	219

4.8.7.1	Measurement Reporting Delay	220

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	220

4.8.8.1	Measurement Reporting Delay	221

4.9	Idle Mode CA Measurement	222

4.9.1	Introduction	222

4.9.2	Requirement	222

4.9.2.1	Detected cell requirement during state transition and Idle mode	222

4.9.2.2	Measurements of inter-frequency CA candidate cells	223

4.9.2.3	Measurements on serving cell	223

4A	E-UTRAN RRC_INACTIVE state mobility	224

4A.1	Cell Re-selection	224

4A.1.1	Introduction	225

4A.1.2	Requirements	225

4A.1.2.1	UE measurement capability	225

4A.1.2.2	Measurement and evaluation of serving cell	225

4A.1.2.3	Measurements of intra-frequency E-UTRAN cells	225

4A.1.2.4	Measurements of inter-frequency E-UTRAN cells	225

4A.1.2.5	Evaluation of cell re-selection criteria	225

4A.1.2.6	Maximum interruption in paging reception	225

4A.1.2.7	Measurements of inter-RAT NR cells	225

4A.1.2.8	UE measurement capability for RedCap	225

4A.1.2.9	Measurements of inter-RAT NR cells for RedCap	225

4A.2	Requirements for UE Category M1	226

4A.2.1	Introduction	226

4A.2.2	Cell Selection	226

4A.2.3	Cell Reselection	226

4A.2.3.1	Cell Re-selection requirements for UE category M1 in normal coverage	226

4A.2.3.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	226

4A.2.3.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	227

4A.2.3.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	227

4A.2.3.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	227

4A.2.3.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	227

4A.2.4	Channel quality report for UE Category M1 in idle mode	229

5	E-UTRAN RRC_CONNECTED state mobility	229

5.1	E-UTRAN Handover	230

5.1.1	Introduction	230

5.1.2	Requirements	230

5.1.2.1	E-UTRAN FDD – FDD	230

5.1.2.1.1	Handover delay	230

5.1.2.1.2	Interruption time	230

5.1.2.2	E-UTRAN FDD – TDD	231

5.1.2.2.1	(Void)	232

5.1.2.2.2	(Void)	232

5.1.2.3	E-UTRAN TDD – FDD	232

5.1.2.3.1	(Void)	232

5.1.2.3.2	(Void)	232

5.1.2.4	E-UTRAN TDD – TDD	232

5.1.2.4.1	Handover delay	232

5.1.2.4.2	Interruption time	232

5.1.2.5	E-UTRAN HD–FDD	234

5.1.2.5.1	Handover delay	234

5.1.2.5.2	Interruption time	234

5.1.2.6	E-UTRAN FDD – FDD conditional handover	235

5.1.2.6.1	Handover delay	235

5.1.2.6.2	Measurement time	236

5.1.2.6.3	Preparation time	236

5.1.2.6.4	Interruption time	236

5.1.2.7	E-UTRAN FDD – TDD conditional handover	236

5.1.2.8	E-UTRAN TDD – FDD conditional handover	237

5.1.2.9	E-UTRAN TDD – TDD conditional handover	237

5.2	Void	237

5.3	Handover to other RATs	237

5.3.1	E-UTRAN - UTRAN FDD Handover	237

5.3.1.1	Introduction	237

5.3.1.1.1	Handover delay	237

5.3.1.1.2	Interruption time	237

5.3.2	E-UTRAN - UTRAN TDD Handover	238

5.3.2.1	Introduction	238

5.3.2.2	Requirements	238

5.3.2.2.1	Handover delay	238

5.3.2.2.2	Interruption time	238

5.3.3	E-UTRAN - GSM Handover	239

5.3.3.1	Introduction	239

5.3.3.2	Requirements	239

5.3.3.2.1	Handover delay	239

5.3.3.2.2	Interruption time	239

5.3.4	E-UTRAN - NR FR1 Handover	239

5.3.4.1	Introduction	239

5.3.4.2	Handover delay	240

5.3.4.3	Interruption time	240

5.3.4A	E-UTRAN - NR FR1 Handover to target cell using CCA	240

5.3.4A.1	Introduction	240

5.3.4A.2	Handover delay	241

5.3.4A.3	Interruption time	241

5.3.4B	E-UTRAN - NR FR1 Handover for RedCap	242

5.3.4B.1	Introduction	242

5.3.4B.2	Requirements	242

5.3.5	E-UTRAN - NR FR2 Handover	242

5.3.5.1	Introduction	242

5.3.5.2	Handover delay	242

5.3.5.3	Interruption time	242

5.4	Handover to Non-3GPP RATs	243

5.4.1	E-UTRAN – HRPD Handover	243

5.4.1.1	Introduction	243

5.4.1.1.1	Handover delay	243

5.4.1.1.2	Interruption time	243

5.4.2	E-UTRAN – cdma2000 1X Handover	244

5.4.2.1	Introduction	244

5.4.2.1.1	Handover delay	244

5.4.2.1.2	Interruption time	244

5.5	E-UTRAN Handover for Cat-M1 UEs	244

5.5.1	Introduction	244

5.5.2	Requirements in CEModeA	245

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	245

5.5.2.1.1	Handover delay	245

5.5.2.1.2	Interruption time	245

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	245

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	246

5.5.2.3.1	Void	246

5.5.2.3.2	Void	246

5.5.3	Requirements in CEModeB	246

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	246

5.5.3.1.1	Handover delay	246

5.5.3.1.2	Interruption time	246

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	247

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	247

5.5A	E-UTRAN Handover for Cat-M1 UEs for Satellite Access	247

5.5A.1	Introduction	247

5.5A.2	Requirements in CEModeA	247

5.5A.2.1	E-UTRAN FDD – FDD HO for Cat-M1 FDD UEs	247

5.5A.2.1.1	Handover delay	247

5.5A.2.1.2	Interruption time	247

5.5A.2.2	E-UTRAN FDD – FDD HO for Cat-M1 HD – FDD UEs	248

5.5A.2.3	E-UTRAN FDD – FDD conditional HO for Cat-M1 FDD UEs	248

5.5A.2.3.1	Handover delay	248

5.5A.2.3.2	Measurement time	248

5.5A.2.3.3	Preparation time	249

5.5A.2.3.4	Interruption time	249

5.5A.2.4	E-UTRAN FDD – FDD conditional HO for Cat-M1 HD – FDD UEs	249

5.5A.3	Requirements in CEModeB	250

5.5A.3.1	E-UTRAN FDD – FDD HO for Cat-M1 FDD UEs	250

5.5A.3.1.1	Handover delay	250

5.5A.3.1.2	Interruption time	250

5.5A.3.2	E-UTRAN FDD – FDD HO for Cat-M1 HD – FDD UEs	251

5.5A.3.3	E-UTRAN FDD – FDD conditional HO for Cat-M1 FDD UEs	251

5.5A.3.3.1	Handover delay	251

5.5A.3.3.2	Measurement time	251

5.5A.3.3.3	Preparation time	251

5.5A.3.3.4	Interruption time	252

5.5A.3.4	E-UTRAN FDD – FDD conditional HO for Cat-M1 HD – FDD UEs	252

5.6	Void	252

5.7	E-UTRAN DAPS Handover	252

5.7.1	Introduction	252

5.7.2	Requirements	253

5.7.2.1	E-UTRAN FDD – FDD	253

5.7.2.1.1	DAPS Handover delay	253

5.7.2.1.2	Interruption time	254

5.7.2.2	E-UTRAN FDD – TDD	254

5.7.2.3	E-UTRAN TDD – FDD	254

5.7.2.4	E-UTRAN TDD – TDD	254

5.8	EN-DC Handover with PSCell	254

5.8.1	Introduction	254

5.8.1.1	Handover with PSCell Interruption time	255

5.8.1.2	Handover with PSCell - NR PSCell Changing Delay requirements	255

5.9	EN-DC Handover with PSCell using CCA	256

5.9.1	Introduction	256

5.9.1.1	Handover with PSCell – E-UTRA HO Interruption time	256

5.9.1.2	Handover with PSCell - NR PSCell Change Delay requirements	257

6	RRC Connection Mobility Control	257

6.1	RRC Re-establishment	257

6.1.1	Introduction	258

6.1.2	Requirements	258

6.1.2.1	UE Re-establishment delay requirement	258

6.2	Random Access	258

6.2.1	Introduction	258

6.2.2	Requirements	258

6.2.2.1	Contention based random access	259

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	259

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	259

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	259

6.2.2.1.4	Void	259

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	259

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	259

6.2.2.2	Non-Contention based random access	259

6.2.2.2.1	Correct behaviour when receiving Random Access Response	259

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	259

6.2.3	Requirements for Cat-M1 UEs	260

6.2.3A	Random Access Requirements for Cat-M1 UEs for Satellite Access	260

6.3	RRC Connection Release with Redirection	260

6.3.1	Introduction	260

6.3.2	Requirements	260

6.3.2.1	RRC connection release with redirection to UTRAN FDD	260

6.3.2.2	RRC connection release with redirection to GERAN	261

6.3.2.3	RRC connection release with redirection to UTRAN TDD	261

6.3.2.4	RRC connection release with redirection to NR	262

6.3.2.5	RRC connection release with redirection to NR carrier subject to CCA	262

6.3.2.6	RRC connection release with redirection to NR Redcap	263

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	265

6.4.1	Introduction	265

6.4.2	Requirements	265

6.5	RRC Re-establishment for NB-IoT UEs	265

6.5.1	Introduction	265

6.5.2	Requirements	265

6.5.2.1	UE Re-establishment delay requirement in normal coverage	265

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	266

6.5A	RRC Re-establishment for NB-IoT UEs for Satellite Access	266

6.5A.1	Introduction	266

6.5A.2	Requirements	267

6.5A.2.1	UE Re-establishment delay requirement in normal coverage	267

6.5A.2.2	UE Re-establishment delay requirement in enhanced coverage	267

6.6	Random Access for UE category NB1	268

6.6.1	Introduction	268

6.6.2	Requirements	268

6.6.2.1	Correct behaviour when receiving Random Access Response reception	268

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	268

6.6.2.3	Correct behaviour when receiving a NACK on msg3	269

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	269

6.6.2.5	Correct behaviour when contention Resolution timer expires	269

6.6.2.6	MSG3-based channel quality report for UE Category NB1	269

6.6.3	Requirements for NPRACH configuration	269

6.6A	Random Access for UE category NB-IoT for Satellite Access	270

6.6A.1	Introduction	270

6.6A.2	Requirements	270

6.6A.2.1	Correct behaviour when receiving Random Access Response reception	270

6.6A.2.2	Correct behaviour when not receiving Random Access Response reception	270

6.6A.2.3	Correct behaviour when receiving a NACK on msg3	270

6.6A.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	270

6.6A.2.5	Correct behaviour when contention Resolution timer expires	271

6.6A.2.6	MSG3-based channel quality report for UE Category NB1	271

6.6A.3	Requirements for NPRACH configuration	271

6.7	RRC Re-establishment for Cat-M1 UEs	272

6.7.1	Introduction	272

6.7.2	Requirements	272

6.7.2.1	UE Re-establishment delay requirement for CEModeA	272

6.7.2.2	UE Re-establishment delay requirement for CEModeB	272

6.7A	RRC Re-establishment for Cat-M1 UEs for Satellite Access	273

6.7A.1	Introduction	273

6.7A.2	Requirements	273

6.7A.2.1	UE Re-establishment delay requirement for CEModeA	273

6.7A.2.2	UE Re-establishment delay requirement for CEModeB	274

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	275

6.8.1	Introduction	275

6.8.2	Requirements	275

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	275

6.8A	RRC Connection Release with Redirection for UE Category M1 for Satellite Access	276

6.8A.1	Introduction	276

6.8A.2	Requirements	276

6.8A.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	276

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	277

6.9.1	Introduction	277

6.9.2	Requirements	277

6.9A	RRC Connection Redirection to Non-anchor Carrier in NB-IoT for Satellite Access	277

6.9A.1	Introduction	277

7	Timing and signalling characteristics	278

7.1	UE transmit timing	278

7.1.1	Introduction	278

7.1.2	Requirements	279

7.2	UE timer accuracy	280

7.2.1	Introduction	280

7.2.2	Requirements	280

7.3	Timing Advance	280

7.3.1	Introduction	280

7.3.2	Requirements	281

7.3.2.1	Timing Advance adjustment delay	281

7.3.2.2	Timing Advance adjustment accuracy	281

7.4	Cell phase synchronization accuracy (TDD)	281

7.4.1	Definition	281

7.4.2	Minimum requirements	281

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	282

7.5.1	Introduction	282

7.5.2	eNodeB Synchronization Requirements	282

7.5.2.1	Synchronized E-UTRAN	282

7.5.2.2	Non-Synchronized E-UTRAN	282

7.6	Radio Link Monitoring	282

7.6.1	Introduction	282

7.6.2	Requirements	284

7.6.2.1	Minimum requirement when no DRX is used	284

7.6.2.2	Minimum requirement when DRX is used	285

7.6.2.3	Minimum requirement at transitions	286

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	286

7.6.2.5	Minimum requirement under IDC Interference	287

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	287

7.7.1	Introduction	287

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	287

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	289

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	290

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	291

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	292

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	293

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	293

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	293

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	294

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	295

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	296

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	297

7.7.14	SCell Activation Delay Requirement for Dormant SCell	297

7.7.15	SCell Hibernation Delay Requirement for Activated SCell	298

7.7.16	SCell Hibernation Delay Requirement for Deactivated SCell	299

7.7.17	SCell Deactivation Delay Requirement for Dormant SCell	301

7.7.18	Direct SCell Activation and Hibernation Delay Requirement	301

7.7.19	Direct SCell Activation and Hibernation Delay Requirement at RRC Reconfiguration during Handover	303

7.8	Interruptions with Carrier Aggregation	305

7.8.1	Introduction	305

7.8.2	Requirements	305

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	305

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	305

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	305

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	305

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	306

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	306

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	306

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	306

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	307

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	308

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	308

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	308

7.8.2.13	Interruptions at SRS carrier based switching	309

7.8.2.14	Interruptions at SCell activation and deactivation of dormant SCell for intra-band CA	309

7.8.2.15	Interruptions at SCell activation and deactivation of dormant SCell for inter-band CA	310

7.8.2.16	Interruptions at SCell activation and deactivation of multiple dormant SCells	310

7.8.2.17	Interruptions during CQI measurement on dormant SCell	310

7.8.2.18	Interruptions during RRM measurement on dormant SCell for intra-band CA	310

7.8.2.19	Interruptions during RRM measurement on dormant SCell for inter-band CA	311

7.8.2.20	Interruptions at SCell hibernation	311

7.8.2.21	Interruptions at direct SCell activation and hibernation	311

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	312

7.9.1	Introduction	312

7.9.2	Minimum Requirements for Interband Carrier Aggregation	312

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	312

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	313

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	313

7.10.1	Introduction	313

7.10.2	Requirements	313

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	313

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	313

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	314

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	314

7.11	Radio Link Monitoring for UE Category 0	314

7.11.1	Introduction	314

7.11.2	Requirements for FD-FDD and TDD	316

7.11.2.1	Minimum requirement when no DRX is used	316

7.11.2.2	Minimum requirement when DRX is used	316

7.11.2.3	Minimum requirement at transitions	317

7.11.3	Requirements for HD-FDD	317

7.11.3.1	Minimum requirement when no DRX is used	317

7.11.3.2	Minimum requirement when DRX is used	317

7.11.3.3	Minimum requirement at transitions	318

7.12	Interruptions with Dual Connectivity	318

7.12.1	Introduction	318

7.12.2	Requirements	318

7.12.2.1	Interruptions at PSCell addition/release	318

7.12.2.2	Interruptions at transitions between active and non-active during DRX	319

7.12.2.3	Interruptions at transitions from non-DRX to DRX	319

7.12.2.4	Interruptions at SCell addition/release	319

7.12.2.5	Interruptions at SCell activation/deactivation	319

7.12.2.6	Interruptions during measurements on SCC	320

7.12.2.7	Interruptions at SRS carrier based switching	320

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	321

7.13.1	Definition	321

7.13.2	Minimum requirements	321

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	321

7.14.1	Introduction	321

7.14.2	PSCell Addition Delay Requirement	321

7.14.3	PSCell Release Delay Requirement	322

7.15	Maximum Receive Timing Difference in Dual Connectivity	322

7.15.1	Introduction	322

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	322

7.16	Proximity-based Services	323

7.16.1	Introduction	323

7.16.2	Requirements	323

7.16.2.1	ProSe UE transmission timing	323

7.16.2.1.1	Serving cell or PCell as timing reference	323

7.16.2.1.2	SCell or non-serving cell as timing reference	323

7.16.3	Interruptions with ProSe	323

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	324

7.16.3.2	Interruptions at ProSe Direct Communication configuration	324

7.16.3.3	Interruptions during ProSe Direct Discovery	324

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	324

7.16.3.5	Interruptions during ProSe Direct Communication	325

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	325

7.16.4.1	Measurement and evaluation of selected cell	325

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	325

7.16.5	Selection / Reselection of ProSe relay UE	326

7.16.6	ProSe operation under deactivated SCell	326

7.17	Maximum Transmission Timing Difference in Dual Connectivity	327

7.17.1	Introduction	327

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	327

7.18.1	Introduction	327

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	327

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	327

7.19	Radio Link Monitoring for UE Category M1	327

7.19.1	Introduction	327

7.19.2	Requirements for FD-FDD and TDD CE mode A	328

7.19.2.1	Minimum requirement when no DRX is used	330

7.19.2.2	Minimum requirement when DRX is used	331

7.19.2.3	Minimum requirement at transitions	332

7.19.3	Requirements for HD-FDD with CE mode A	332

7.19.3.1	Minimum requirement when no DRX is used	332

7.19.3.2	Minimum requirement when DRX is used	332

7.19.3.3	Minimum requirement at transitions	334

7.19.4	Requirements for FD-FDD and TDD with CE mode B	334

7.19.4.1	Minimum requirement when no DRX is used	336

7.19.4.2	Minimum requirement when DRX is used	337

7.19.4.3	Minimum requirement at transitions	338

7.19.5	Requirements for HD-FDD with CE mode B	338

7.19.5.1	Minimum requirement when no DRX is used	338

7.19.5.2	Minimum requirement when DRX is used	338

7.19.5.3	Minimum requirement at transitions	340

7.19A	Radio Link Monitoring for UE Category M1 for Satellite Access	340

7.19A.1	Introduction	340

7.19A.2	Requirements for FD-FDD and CE mode A	340

7.19A.2.1	Minimum requirement when no DRX is used	342

7.19A.2.2	Minimum requirement when DRX is used	343

7.19A.2.3	Minimum requirement at transitions	344

7.19A.3	Requirements for HD-FDD with CE mode A	344

7.19A.3.1	Minimum requirement when no DRX is used	344

7.19A.3.2	Minimum requirement when DRX is used	344

7.19A.3.3	Minimum requirement at transitions	346

7.19A.4	Requirements for HD-FDD with CE mode B	346

7.19A.4.1	Minimum requirement when no DRX is used	346

7.19A.4.2	Minimum requirement when DRX is used	346

7.19A.4.3	Minimum requirement at transitions	347

7.20	UE transmit timing for NB-IoT	347

7.20.1	Introduction	347

7.20.2	Requirements	348

7.20A	UE transmit timing for NB-IoT for Satellite Access	348

7.20A.1	Introduction	348

7.20A.2	Requirements	348

7.21	 UE timer accuracy for NB-IoT	349

7.21.1	Introduction	349

7.21.2	Requirements	349

7.21A	UE timer accuracy for NB-IoT for Satellite Access	350

7.21A.1	Introduction	350

7.21A.2	Requirements	350

7.22	Timing Advance for NB-IoT	350

7.22.1	Introduction	350

7.22.2	Requirements	350

7.22.2.1	Timing Advance adjustment delay	350

7.22.2.2	Timing Advance adjustment accuracy	350

7.22A	Timing Advance for NB-IoT for Satellite Access	351

7.22A.1	Introduction	351

7.22A.2	Requirements	351

7.22A.2.1	Timing Advance adjustment delay	351

7.22A.2.2	Timing Advance adjustment accuracy	351

7.23	Radio Link Monitoring for Category NB1 UE	351

7.23.1	Introduction	351

7.23.2	Requirements for Category NB1 UE	351

7.23.2.1	Minimum requirement when no DRX is used	352

7.23.2.2	Minimum requirement when DRX is used	352

7.23.2.3	Minimum requirement at transitions	353

7.23A	Radio Link Monitoring for Category NB-IoT UE for Satellite Access	353

7.23A.1	Introduction	353

7.23A.2	Requirements for Category NB1 UE	353

7.23A.2.1	Minimum requirement when no DRX is used	354

7.23A.2.2	Minimum requirement when DRX is used	354

7.23A.2.3	Minimum requirement at transitions	355

7.24	UE transmit timing for Category M1	355

7.24.1	Introduction	355

7.24.2	Requirements	355

7.24A	UE transmit timing for Category M1 for Satellite Access	356

7.24A.1	Introduction	356

7.24A.2	Requirements	356

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	357

7.25.1	Definition	357

7.25.2	Minimum requirements	357

7.26	UE transmit timing for Category M2	358

7.26.1	Introduction	358

7.26.2	Requirements	358

7.27	UE timer accuracy for category M1	358

7.27.1	Introduction	358

7.27.2	Requirements	358

7.27A	UE timer accuracy for category M1 for Satellite Access	358

7.27A.1	Introduction	358

7.27A.2	Requirements	358

7.28	Timing Advance for Category M1	359

7.28.1	Introduction	359

7.28.2	Requirements	359

7.28A	Timing Advance for Category M1 for Satellite Access	359

7.28A.1	Introduction	359

7.28A.2	Requirements	359

7.28A.2.1	Timing Advance adjustment delay	359

7.28A.2.2	Timing Advance adjustment accuracy	359

7.29	Interruptions requirements with FeMBMS	359

7.29.1	Introduction	359

7.29.2	Requirements	359

7.30	Numerology switching delay requirements with FeMBMS	360

7.30.1	Introduction	360

7.30.2	Requirements	360

7.31	NR PSCell Addition and Release Delay for E-UTRA - NR Dual Connectivity	360

7.31.1	Introduction	360

7.31.2	NR PSCell Addition Delay Requirement	360

7.31.3	NR PSCell Release Delay Requirement	361

7.31A	Addition and Release Delay of NR PSCell Operating with CCA for E-UTRA - NR Dual Connectivity	361

7.31A.1	Introduction	361

7.31A.2	NR PSCell Addition Delay Requirement	361

7.31A.3	NR PSCell Release Delay Requirement	362

7.32	Interruptions with EN-DC	363

7.32.1	Introduction	363

7.32.2	Requirements	363

7.32.2.1	Interruptions at PSCell addition/release	363

7.32.2.2	Interruptions at transitions between active and non-active during DRX	363

7.32.2.3	Interruptions at transitions from non-DRX to DRX	364

7.32.2.4	Interruptions at SCell addition/release	364

7.32.2.5	Interruptions at SCell activation/deactivation	364

7.32.2.6	Interruptions during measurements on SCC	365

7.32.2.6.1	Interruptions during measurements on deactivated NR SCC	365

7.32.2.6.2	Interruptions during measurements on deactivated E-UTRA SCC	365

7.32.2.6.3	Interruptions during CQI measurements on dormant E-UTRA SCell	365

7.32.2.6.4	Interruptions during RRM measurements on dormant E-UTRA SCC	365

7.32.2.7	Interruptions at active BWP switching	365

7.32.2.8	Interruptions at SCell activation and deactivation of dormant SCell	366

7.32.2.9	Interruptions at SCell activation and deactivation of multiple dormant SCell	366

7.32.2.10	Interruptions at SCell hibernation	366

7.32.2.11	Interruptions at direct SCell activation and hibernation	366

7.32.2.12	DL Interruptions at UE switching between two uplink carriers	367

7.32.2.13	Interruptions at NR SRS carrier based switching	367

7.32.2.14	Interruptions at NR SCell dormancy	368

7.32.2.14.1	Interruptions due to NR SCell dormancy switch	368

7.32.2.14.2	Interruptions due to CSI and RRM measurements during SCell dormancy	368

7.32.2.15	Interruption during NR measurement with autonomous gaps	368

7.32.2.16	Interruptions at SRS carrier based switching	369

7.32.2.17	Interruptions at SCG activation/deactivation	369

7.32.2.18	Interruptions due to NR SRS antenna port switching	369

7.32.2.19	Interruptions at fast SCell activation/deactivation	370

7.32.2.20	Interruptions due to RRM measurements on deactivated NR SCG	370

7.33	Maximum Transmit/Receive Timing Difference in Carrier Aggregation for sTTI and 1ms-TTI with 3 subframe HARQ processing	371

7.33.1	Introduction	371

7.33.2	Requirements	371

7.34	Void	371

7.35	Interruptions with SFTD measurements	371

7.35.1	Introduction	371

7.35.2	Requirements	371

7.36	Interruptions with NE-DC	372

7.36.1	Introduction	372

7.36.2	Requirements	372

7.36.2.1	Interruptions at transitions between active and non-active during DRX	372

7.36.2.2	Interruptions at transitions from non-DRX to DRX	372

7.36.2.3	Interruptions at SCell addition/release	372

7.36.2.4	Interruptions at SCell activation/deactivation	373

7.36.2.5	Interruptions during measurements on SCC	373

7.36.2.5.1	Interruptions during measurements on deactivated NR SCC	373

7.36.2.5.2	Interruptions during measurements on deactivated E-UTRA SCC	373

7.36.2.5.3	Interruptions during CQI measurements on dormant E-UTRA SCell	373

7.36.2.5.4	Interruptions during RRM measurements on dormant E-UTRA SCC	374

7.36.2.6	Interruptions at active BWP switching	374

7.36.2.7	Interruptions at SCell activation and deactivation of dormant SCell	375

7.36.2.8	Interruptions at SCell activation and deactivation of multiple dormant SCell	375

7.36.2.9	Interruptions at SCell hibernation	375

7.36.2.10	Interruptions at direct SCell activation and hibernation	375

7.36.2.11	Interruptions at NR SRS carrier based switching	375

7.36.2.12	Interruptions at NR SCell dormancy	376

7.36.2.12.1	Interruptions due to NR SCell dormancy switch	376

7.36.2.12.2	Interruptions due to CSI and RRM measurements during SCell dormancy	376

7.36.2.13	Interruption during E-UTRA measurement with autonomous gaps	376

7.36.2.14	Interruption during NR measurement with autonomous gaps	377

7.36.2.15	Interruptions at SRS carrier based switching	377

7.36.2.16	Interruptions due to NR SRS antenna port switching	378

7.37	Interruptions during NR measurement with autonomous gaps	378

7.37.1	Introduction	378

7.37.2	Requirements	379

7.38 SCG Activation and Deactivation Delay	379

7.38.1	Introduction	379

7.38.2	SCG Activation Delay Requirement	379

7.38.3	SCG Deactivation Delay Requirement	380

8	UE Measurements Procedures in RRC_CONNECTED State	380

8.1	General Measurement Requirements	380

8.1.1	Introduction	380

8.1.2	Requirements	381

8.1.2.1	UE measurement capability	381

8.1.2.1.1	Monitoring of multiple layers using gaps	389

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	390

8.1.2.1.1b	Monitoring of multiple layers using gaps (EN-DC)	391

8.1.2.1.1c	Monitoring of multiple layers using gaps (NE-DC)	393

8.1.2.1.1d	Monitoring of multiple layers using gaps (RedCap)	394

8.1.2.1.2	Network controlled small gap	394

8.1.2.2	E-UTRAN intra frequency measurements	396

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	396

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	401

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	405

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	406

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	407

8.1.2.3	E-UTRAN inter frequency measurements	407

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	408

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	413

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	420

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	420

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	420

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	421

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	423

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	424

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	425

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	432

8.1.2.4	Inter RAT measurements	432

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	432

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	437

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	437

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	441

8.1.2.4.5	E-UTRAN FDD – GSM measurements	441

8.1.2.4.6	E-UTRAN TDD – GSM measurements	446

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	446

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	448

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	448

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	448

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	449

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	449

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	449

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	449

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	451

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	451

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	451

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	451

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	452

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	452

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	454

8.1.2.4.21	E-UTRAN FDD – NR measurements	454

8.1.2.4.21A	E-UTRAN FDD – NR measurements when CCA is used	458

8.1.2.4.22	E-UTRAN TDD – NR measurements	462

8.1.2.4.22A	E-UTRAN TDD – NR measurements when CCA is used	462

8.1.2.4.23	Void	462

8.1.2.4.24	Void	462

8.1.2.4.25	E-UTRAN FDD – NR SFTD Measurements	462

8.1.2.4.26	E-UTRAN TDD – NR SFTD Measurements	464

8.1.2.4.27 	E-UTRA FDD - NR measurements with autonomous gaps	464

8.1.2.4.28 	E-UTRA TDD - NR measurements with autonomous gaps	465

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	465

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	465

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	467

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	469

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	471

8.1.2.6.5	Void	472

8.1.2.6.6	Void	472

8.1.2.6.7	Void	472

8.1.2.6.8	Void	472

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	472

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	473

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	474

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	476

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	478

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	480

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	481

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	483

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	485

8.1.2.7	E-UTRAN E-CID Measurements	487

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	487

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	488

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	489

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	490

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	490

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	491

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	493

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	496

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	500

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	503

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	503

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	504

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	504

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	505

8.1.2.10	Void	505

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	505

8.2.1	Introduction	505

8.2.2	Requirements	505

8.3	Measurements for E-UTRA carrier aggregation	510

8.3.1	Introduction	510

8.3.2	Measurements of the primary component carrier	510

8.3.3	Measurements of a secondary component carrier	510

8.3.3.1	Measurements of a secondary component carrier with active SCell	510

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	511

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	511

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	512

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	514

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	514

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	514

8.4.1	Introduction	514

8.4.2	Measurements on the primary component carrier	515

8.4.3	Measurements on a secondary component carrier	516

8.4.4	Measurements on both primary component carrier and a secondary component carrier	516

8.4.5	Measurements on different secondary component carriers	517

8.5	Measurements for UE category 0	518

8.5.1	Introduction	518

8.5.2 	Requirements	519

8.5.2.1 	E-UTRAN intra frequency measurements	519

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	519

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	522

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	524

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	528

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	528

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	529

8.6	Discovery signal measurements	530

8.6.1	Introduction	530

8.6.2	Requirements for CRS based discovery signal measurements	530

8.6.2.1	E-UTRAN intra frequency measurements	530

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	530

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	533

8.6.2.2	E-UTRAN inter frequency measurements	535

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	536

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	538

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	541

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	541

8.6.3	Requirements for CSI-RS based discovery signal measurements	541

8.6.3.1	E-UTRAN intra frequency measurements	541

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	542

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	544

8.6.3.2	E-UTRAN inter frequency measurements	546

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	547

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	549

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	552

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	552

8.7	Discovery signal measurements for E-UTRA carrier aggregation	552

8.7.1	Introduction	552

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	553

8.7.2.1	Measurements of the primary component carrier	553

8.7.2.2	Measurements of a secondary component carrier	553

8.7.2.3	Measurements of a secondary component carrier with active SCell	553

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	553

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	553

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	554

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	556

8.7.3.1	Measurements of the primary component carrier	556

8.7.3.2	Measurements of a secondary component carrier	556

8.7.3.3	Measurements of a secondary component carrier with active SCell	556

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	556

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	556

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	558

8.8	Measurements for E-UTRA dual connectivity	559

8.8.1	Introduction	559

8.8.2	Intra-frequency measurements requirements on PCell	559

8.8.3	Intra-frequency measurements requirements on PSCell	560

8.8.4	Inter-frequency and inter-RAT measurement requirements	560

8.8.5	Intra-frequency measurements with autonomous gaps	560

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	560

8.8.5.2	ECGI reporting delay	561

8.8.6	Inter-frequency measurements with autonomous gaps	561

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	561

8.8.6.2	ECGI reporting delay	562

8.8.7	SSTD Measurements	562

8.8.7.1	Introduction	562

8.8.7.2	SSTD Measurement requirements	562

8.8.7.3	SSTD Measurement Reporting Delay	563

8.8.8	Intra-frequency measurements requirements on SCell	563

8.9	MBSFN Measurements	563

8.9.1	Introduction	563

8.9.2	MBSFN RSRP Measurements	563

8.9.3	MBSFN RSRQ Measurements	564

8.9.4	MCH BLER Measurements	564

8.10	Proximity-based Services	564

8.10.1	Introduction	564

8.10.2	Requirements	564

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	564

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	565

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	565

8.11.1	Introduction	565

8.11.2	CRS based discovery signal measurements	566

8.11.2.1	E-UTRAN intra-frequency measurements	566

8.11.2.1.1	Requirements	566

8.11.2.1.1.1	Requirements when no DRX is used	566

8.11.2.1.1.1.1	Measurement Reporting Requirements	568

8.11.2.1.1.2	Requirements when DRX is used	569

8.11.2.1.1.2.1	Measurement Reporting Requirements	571

8.11.2.2	E-UTRAN inter-frequency measurements	572

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	572

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	576

8.11.3	CSI-RS based discovery signal measurements	576

8.11.3.1	E-UTRAN intra-frequency measurements	576

8.11.3.1.1	Requirements	576

8.11.3.1.1.1	Requirements when no DRX is used	576

8.11.3.1.1.1.1	Measurement Reporting Requirements	578

8.11.3.1.1.2	Requirements when DRX is used	578

8.11.3.1.1.2.1	Measurement Reporting Requirements	579

8.11.3.2	E-UTRAN inter-frequency measurements	580

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	580

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	584

8.11.4	RSSI measurements	584

8.11.4.1	E-UTRAN intra-frequency measurements	584

8.11.4.2	E-UTRAN inter-frequency measurements	584

8.11.5	Channel occupancy measurements	585

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	585

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	585

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	585

8.12.1	Introduction	585

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	585

8.12.2.1	Introduction	585

8.12.2.2	Measurements of a secondary component carrier	585

8.12.2.3	Measurements of a secondary component carrier with active SCell	585

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	586

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	586

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	588

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	591

8.12.3.1	Introduction	591

8.12.3.2	Measurements of a secondary component carrier	591

8.12.3.3	Measurements of a secondary component carrier with active SCell	591

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	591

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	591

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	593

8.13	Measurements for UE Category M1	595

8.13.1	Introduction	595

8.13.2	Requirements for UE category M1 with CE mode A	595

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	596

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	596

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	601

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	603

8.13.2.2	Void	607

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	607

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	607

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	610

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	612

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	613

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	613

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	615

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	617

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	618

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	618

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	618

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	619

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	619

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	619

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	620

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	620

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	621

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	622

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	622

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	623

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	627

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	629

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	633

8.13.2.8	Channel quality report for UE Category M1 in connected mode with CE mode A	633

8.13.3	Requirements for UE category M1 with CE mode B	634

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	634

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	635

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	639

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	642

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	647

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	648

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	648

8.13.3.2	Void	649

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	649

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	649

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	652

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	654

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	655

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	655

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	655

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	656

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	656

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	656

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	657

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	657

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	657

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	662

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	664

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	668

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	669

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	669

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	671

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	673

8.13.3.8	Channel quality report for UE Category M1 in connected mode with CE mode B	674

8.13A	Measurements for UE Category M1 for Satellite Access	674

8.13A.1	Introduction	674

8.13A.2	Requirements for UE category M1 with CE mode A	675

8.13A.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	675

8.13A.2.1.1	E-UTRAN FDD intra frequency measurements	676

8.13A.2.1.1.1	E-UTRAN intra frequency measurements when no DRX is used	676

8.13A.2.1.1.2	E-UTRAN intra frequency measurements when DRX is used	678

8.13A.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	679

8.13A.2.1.2.1	E-UTRAN intra frequency measurements when no DRX is used	679

8.13A.2.1.2.2	E-UTRAN intra frequency measurements when DRX is used	680

8.13A.2.2	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	682

8.13A.2.2.1	E-UTRAN FDD - FDD inter frequency measurements	682

8.13A.2.2.1.1	E-UTRAN FDD - FDD inter frequency measurements when no DRX is used	682

8.13A.2.2.1.2	E-UTRAN inter frequency measurements when DRX is used	684

8.13A.2.2.2	E-UTRAN inter-frequency measurements for HD-FDD	686

8.13A.2.2.2.1	E-UTRAN inter-frequency measurements when no DRX is used	686

8.13A.2.2.2.2	E-UTRAN inter frequency measurements when DRX is used	686

8.13A.2.3	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	688

8.13A.2.4	Channel quality report for UE Category M1 in connected mode with CE mode A	688

8.13A.3	Requirements for UE category M1 with CE mode B	688

8.13A.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	689

8.13A.3.1.1	E-UTRAN FDD intra frequency measurements	689

8.13A.3.1.1.1	E-UTRAN intra frequency measurements when no DRX is used	689

8.13A.3.1.1.2	E-UTRAN intra frequency measurements when DRX is used	691

8.13A.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	693

8.13A.3.1.2.1	E-UTRAN intra frequency measurements when no DRX is used	693

8.13A.3.1.2.2	E-UTRAN intra frequency measurements when DRX is used	694

8.13A.3.2	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	696

8.13A.3.2.1	E-UTRAN FDD - FDD inter frequency measurements	696

8.13A.3.2.1.1	E-UTRAN FDD - FDD inter frequency measurements when no DRX is used	696

8.13A.3.2.1.2	E-UTRAN inter frequency measurements when DRX is used	698

8.13A.3.2.2	E-UTRAN inter-frequency measurements for HD-FDD	700

8.13A.3.2.2.1	E-UTRAN inter-frequency measurements when no DRX is used	700

8.13A.3.2.2.2	E-UTRAN inter frequency measurements when DRX is used	700

8.13A.3.3	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	702

8.13A.3.4	Channel quality report for UE Category M1 in connected mode with CE mode B	702

8.14	Measurements for UE category NB1	702

8.14.1	Introduction	702

8.14.2	NB-IoT intra frequency measurements under normal coverage	703

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	703

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	703

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	703

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	703

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	703

8.14.4	Connected mode channel quality report for UE Category NB1	703

8.14.5	Connected mode channel quality report for UE Category NB2 supporting 16QAM	704

8.14.6	NB-IoT neighbour cell measurements	704

8.14.6.1	Introduction	704

8.14.6.2	Requirements	704

8.14.6.3	Intra-frequency neighbour cell measurements	705

8.14.6.4	Inter-frequency neighbour cell measurements	705

8.14.6.5	 Requirements for monitoring multiple carriers	706

8.14A	Measurements for UE category NB-IoT for Satellite Access	706

8.14A.1	Introduction	706

8.14A.2	NB-IoT intra frequency measurements under normal coverage	707

8.14A.2.1	NB-IoT intra frequency measurements when no DRX is used	707

8.14A.2.2	NB-IoT intra frequency measurements when DRX is used	707

8.14A.3	NB-IoT intra frequency measurements under enhanced coverage	707

8.14A.3.1	NB-IoT intra frequency measurements when no DRX is used	707

8.14A.3.2	NB-IoT intra frequency measurements when DRX is used	707

8.14A.4	Connected mode channel quality report for UE Category NB1	707

8.15	Void	708

8.16	Measurements for UE Category M2	708

8.16.1	Introduction	708

8.16.2	Requirements for UE category M2 with CE mode A	708

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	708

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	709

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	709

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	710

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	710

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	711

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	711

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	711

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	714

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	716

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	717

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	717

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	719

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	721

8.16.3	Requirements for UE category M2 with CE mode B	722

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	722

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	722

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	725

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	727

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	728

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	728

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	730

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	732

8.17	Measurements for E-UTRA – NR Dual Connectivity	733

8.17.1	Introduction	733

8.17.1.1	Measurement Gap Sharing	733

8.17.1A	Intrafrequency Measurements	734

8.17.2	SFTD Measurements	734

8.17.2.1	Introduction	734

8.17.2.2	SFTD Measurement requirements	734

8.17.2.2.a	SFTD Measurement requirements with CCA on target frequency	735

8.17.2.3	SFTD Measurement Reporting Delay	736

8.17.3	E-UTRA Inter-frequency Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	736

8.17.3.1	Introduction	736

8.17.3.2	E-UTRAN FDD inter frequency measurements	736

8.17.3.2.1	E-UTRAN FDD inter frequency measurements when no DRX is used	736

8.17.3.2.2	E-UTRAN FDD inter frequency measurements when DRX is used	738

8.17.3.3	E-UTRAN TDD inter frequency measurements	740

8.17.3.3.1	E-UTRAN TDD inter frequency measurements when no DRX is used	740

8.17.3.3.2	E-UTRAN TDD inter frequency measurements when DRX is used	742

8.17.4	E-UTRA Inter-RAT NR Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	744

8.17.4.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	744

8.17.4.1.1	NR Inter-RAT cell identification	744

8.17.4.1.2	NR Inter-RAT measurement	746

8.17.4.1.3	NR Inter-RAT measurement reporting	746

8.17.4.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	747

8.17.4A	E-UTRA Inter-RAT NR Measurements when CCA is used when Configured with E-UTRA-NR Dual Connectivity Operation	747

8.17.4A.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	747

8.17.4A.1.1	NR Inter-RAT cell identification	748

8.17.4A.1.2	NR Inter-RAT measurement	749

8.17.4A.1.3	NR Inter-RAT measurement reporting	750

8.17.4A.1.4	NR inter-RAT RSSI measurements	750

8.17.4A.1.5	NR inter-RAT channel occupancy measurements	751

8.17.4A.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	751

8.17.5	E-UTRAN FDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	752

8.17.5.1	Introduction	752

8.17.5.2	E-UTRAN FDD – UTRAN FDD measurements when no DRX is used	752

8.17.5.2.1	Identification of a new UTRA FDD cell	752

8.17.5.2.2	Enhanced UTRA FDD cell identification requirements	752

8.17.5.2.3	UE UTRA FDD CPICH measurement capability	752

8.17.5.2.4	Periodic Reporting	753

8.17.5.2.5	Event Triggered Reporting	753

8.17.5.2.6	Event-triggered Periodic Reporting	753

8.17.5.3	E-UTRAN FDD – UTRAN FDD measurements when DRX is used	754

8.17.5.3.1	Periodic Reporting	755

8.17.5.3.2	Event Triggered Reporting	755

8.17.5.3.3	Event-triggered Periodic Reporting	755

8.17.6	E-UTRAN TDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	756

8.17.7	E-UTRAN FDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	756

8.17.7.1	Introduction	756

8.17.7.2	Identification of a new UTRA FDD cell for SON	756

8.17.7.2.1	Requirements when no DRX is used	756

8.17.7.2.2	Requirements when DRX is used	756

8.17.7.2.3	Reporting Delay	757

8.17.8	E-UTRAN TDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	758

8.17.9	E-UTRAN TDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	758

8.17.9.1	Introduction	758

8.17.9.2	E-UTRAN FDD – UTRAN TDD measurements when no DRX is used	758

8.17.9.2.1	Identification of a new UTRA FDD cell	758

8.17.9.2.2	Enhanced UTRA TDD cell identification requirements	758

8.17.9.2.3	UE UTRA TDD P-CCPCH RSCP measurement capability	759

8.17.9.2.4	Periodic Reporting	759

8.17.9.2.5	Event Triggered Reporting	759

8.17.9.2.6	Event-triggered Periodic Reporting	760

8.17.9.3	E-UTRAN TDD – UTRAN TDD measurements when DRX is used	760

8.17.9.3.1	Periodic Reporting	761

8.17.9.3.2	Event Triggered Reporting	761

8.17.9.3.3	Event-triggered Periodic Reporting	761

8.17.10	E-UTRAN FDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	762

8.17.11	E-UTRAN TDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	762

8.17.11.1	Introduction	762

8.17.11.2	Identification of a new UTRA TDD cell for SON	762

8.17.11.2.1	Requirements when no DRX is used	762

8.17.11.2.2	Requirements when DRX is used	762

8.17.11.2.3	Reporting Delay	763

8.17.12	E-UTRAN FDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	763

8.17.13	E-UTRAN FDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	764

8.17.13.1	Introduction	764

8.17.13.2	E-UTRAN FDD – GSM measurements when no DRX is used	764

8.17.13.2.1	GSM carrier RSSI	764

8.17.13.2.2	BSIC verification	764

8.17.13.2.3	Enhanced BSIC verification	766

8.17.13.2.4	Periodic Reporting	766

8.17.13.2.5	Event Triggered Reporting	766

8.17.13.2.6	Event-triggered Periodic Reporting	767

8.17.13.3	E-UTRAN FDD – GSM measurements when DRX is used	767

8.17.13.3.1	GSM carrier RSSI	767

8.17.13.3.2	BSIC verification	767

8.17.13.3.3	Periodic Reporting	769

8.17.13.3.4	Event Triggered Reporting	769

8.17.13.3.5	Event-triggered Periodic Reporting	769

8.17.14	E-UTRAN TDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	769

8.17.15	E-UTRAN Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	769

8.17.15.1	E-UTRAN FDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	769

8.17.15.1.1	RSTD Measurement Reporting Delay	770

8.17.15.2	E-UTRAN TDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	770

8.17.15.2.1	RSTD Measurement Reporting Delay	770

8.17.15.3	E-UTRAN TDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	770

8.17.15.3.1	RSTD Measurement Reporting Delay	771

8.17.15.4	E-UTRAN FDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	771

8.17.15.4.1	RSTD Measurement Reporting Delay	771

8.17.16	E-UTRAN intra-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	772

8.17.16.1	Introduction	772

8.17.16.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	772

8.17.16.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	772

8.17.17	E-UTRAN inter-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	772

8.17.17.1	Introduction	772

8.17.17.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	772

8.17.17.3	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	772

8.17.17.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	772

8.17.17.5	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	772

8.17.18	E-UTRA FDD - NR CGI measurements with autonomous gaps	772

8.17.19	E-UTRA TDD - NR CGI measurements with autonomous gaps	772

8.18	Measurements for non-BL/CE UE	772

8.18.1	Introduction	772

8.18.2	Requirements for non-BL/CE UE with CE Mode B	773

8.18.2.1	E-UTRAN intra frequency measurements	773

8.18.2.1.1	E-UTRAN FDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	773

8.18.2.1.2	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD non-BL/CE with CE Mode B	773

8.18.2.1.3	E-UTRAN TDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	774

8.19	Measurements for NR – E-UTRA Dual Connectivity	775

8.19.1	Introduction	775

8.19.2	Intra-frequency Measurements	775

8.19.3	Inter-frequency Measurements	775

8.19.4	Void	775

8.19.5	Intra-frequency E-CID Measurements	775

8.19.6	Intra-frequency measurements with autonomous gaps	776

8.19.6.1	Introduction	776

8.19.6.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	776

8.19.6.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	776

8.19.7	Inter-frequency measurements with autonomous gaps	776

8.19.7.1	Introduction	776

8.19.7.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	776

8.19.7.3	E-UTRAN TDD-FDD inter frequency measurements with autonomous gaps	776

8.19.7.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	776

8.19.7.5	E-UTRAN FDD-TDD inter frequency measurements with autonomous gaps	776

8.20	Inter-RAT NR Measurements for RedCap UE	776

8.20.1	Introduction	776

8.20.2	E-UTRAN FDD – NR measurements	776

8.20.2.1	Identification of a new NR cell	776

8.20.2.2	Periodic Reporting	780

8.20.2.3	Event Triggered Reporting	780

8.20.2.4	Event-triggered Periodic Reporting	780

8.20.3	E-UTRAN TDD – NR measurements	780

9	Measurements performance requirements for UE	781

9.1	E-UTRAN measurements	781

9.1.1	Introduction	781

9.1.2	Intra-frequency RSRP Accuracy Requirements	781

9.1.2.1	Absolute RSRP Accuracy	781

9.1.2.2	Relative Accuracy of RSRP	782

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	783

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	783

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	784

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	785

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	786

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	787

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	788

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	788

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	788

9.1.2B	Intra-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	789

9.1.2B.1	Introduction	789

9.1.2B.2	Intra-frequency Absolute RSRP Accuracy for CA Idle Mode Measurements	789

9.1.3	Inter-frequency RSRP Accuracy Requirements	790

9.1.3.1	Absolute RSRP Accuracy	790

9.1.3.2	Relative Accuracy of RSRP	791

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	792

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	792

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	793

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	793

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	794

9.1.3B	Inter-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	795

9.1.3B.1	Introduction	795

9.1.3B.2	Inter-frequency Absolute RSRP Accuracy for Overlapping Carrier	795

9.1.3B.3	Inter-frequency Absolute RSRP Accuracy for Overlapping and Non-overlapping Carrier	795

9.1.4	RSRP Measurement Report Mapping	796

9.1.5	Intra-frequency RSRQ Accuracy Requirements	796

9.1.5.1	Absolute RSRQ Accuracy	796

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	797

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	798

9.1.5.4	Absolute WB-RSRQ Accuracy	799

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	800

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	801

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	801

9.1.5B	Intra-frequency RSRQ Accuracy requirements for CA Idle Mode Measurements	801

9.1.5B.1	Introduction	801

9.1.5B.2	Intra-frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements	801

9.1.6	Inter-frequency RSRQ Accuracy Requirements	802

9.1.6.1	Absolute RSRQ Accuracy	802

9.1.6.2	Relative Accuracy of RSRQ	803

9.1.6.3	Absolute WB-RSRQ Accuracy	804

9.1.6.4	Relative WB-RSRQ Accuracy	804

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	805

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	806

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	806

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	806

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	807

9.1.6B	Inter-frequency absolute RSRQ Accuracy requirements for CA Idle Mode Measurements	808

9.1.6B.1	Introduction	808

9.1.6B.2	Inter-frequency Absolute RSRQ Accuracy for Overlapping Carrier	808

9.1.6B.3	Inter-frequency absolute RSRQ Accuracy for Overlapping and Non-overlapping Carrier	808

9.1.7	RSRQ Measurement Report Mapping	809

9.1.8	Power Headroom	809

9.1.8.1	Period	810

9.1.8.2	Reporting Delay	810

9.1.8.3	Void	810

9.1.8.4	Report Mapping	810

9.1.9	UE Rx – Tx time difference	810

9.1.9.1	Measurement Requirement	810

9.1.9.2	Measurement Report mapping	811

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	812

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	813

9.1.10	Reference Signal Time Difference (RSTD)	814

9.1.10.1	Intra-Frequency Accuracy Requirement	814

9.1.10.2	Inter-Frequency Accuracy Requirement	815

9.1.10.3	RSTD Measurement Report Mapping	817

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	817

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	818

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	819

9.1.11	Carrier aggregation measurement accuracy	820

9.1.11.1	Primary component carrier accuracy requirement	821

9.1.11.2	Secondary component carrier accuracy requirement	821

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	821

9.1.11.4	Secondary component carrier relative accuracy requirement	821

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	821

9.1.13	Measurement accuracy for UE category 0	822

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	822

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	822

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	823

9.1.14	Accuracy requirements for Discovery Signal Measurements	824

9.1.14.1	Introduction	824

9.1.14.2	RSRP measurements in discovery signal occasions	824

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	824

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	824

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	824

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	825

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	826

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	826

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	826

9.1.14.3.3	CSI-RSRP measurement report mapping	827

9.1.14.4	RSRQ measurements in discovery signal occasions	827

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	827

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	828

9.1.15.1.1	Primary component carrier accuracy requirement	828

9.1.15.1.2	Secondary component carrier accuracy requirement	828

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	828

9.1.15.1.4	Secondary component carrier relative accuracy requirement	828

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	828

9.1.15.2.1	Primary component carrier accuracy requirement	828

9.1.15.2.2	Secondary component carrier accuracy requirement	828

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	828

9.1.15.2.4	Secondary component carrier relative accuracy requirement	828

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	829

9.1.17	RS-SINR Measurements	829

9.1.17.1	Measurement Report Mapping	829

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	830

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	830

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	830

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	830

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	831

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	832

9.1.18.1	Introduction	832

9.1.18.2	RSRP measurements	832

9.1.18.2.1	RSRP measurement report mapping	832

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	832

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	833

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	833

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	834

9.1.18.3	RSRQ measurements	834

9.1.18.3.1	RSRQ measurement report mapping	834

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	834

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	835

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	835

9.1.18.4	CSI-RSRP measurements	836

9.1.18.4.1	CSI-RSRP measurement report mapping	836

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	836

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	836

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	837

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	837

9.1.18.5	RSSI measurements	838

9.1.18.5.1	RSSI measurement report mapping	838

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	838

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	839

9.1.18.6	Channel occupancy measurements	839

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	839

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	839

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	840

9.1.19.1	Introduction	840

9.1.19.2	Accuracy requirements for measurements on SCC	840

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	840

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	840

9.1.20	SFN and Subframe Time Difference (SSTD)	841

9.1.20.1	SSTD Accuracy Requirement	841

9.1.20.2	SSTD Measurement Report Mapping	841

9.1.21	Measurement accuracy for UE category M1	842

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	842

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	844

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	845

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	847

9.1.21.5	RSRP Measurement Report Mapping	848

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	848

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	849

9.1.21.8	RSRQ Measurement Report Mapping	850

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	850

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	851

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	852

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	853

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	854

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	855

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	856

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	857

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	858

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	859

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	860

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	861

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	864

9.1.21.22	Downlink Channel Report Mapping for UE Category M1	867

9.1.21.23	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode A	867

9.1.21.24	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode B	870

9.1.22	Measurement accuracy for UE Category NB1	871

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	871

9.1.22.2	Void	872

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	872

9.1.22.4	Void	873

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	873

9.1.22.6	Void	874

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	874

9.1.22.8	Void	875

9.1.22.9	NRSRP Measurement Report Mapping	875

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	875

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	876

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	877

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	878

9.1.22.14	NRSRQ Measurement Report Mapping	879

9.1.22.15	MSG3-based Measurement Report Mapping for UE Category NB1	880

9.1.22.16	Downlink Channel Quality Measurement Accuracy for UE Category NB1	880

9.1.22.17	Channel quality reporting for UE Category NB2 with 16-QAM	881

9.1.23	Power Headroom for UE Category NB1	881

9.1.23.1	Period	882

9.1.23.2	Reporting Delay	882

9.1.23.3	Report Mapping for UE Category NB1	882

9.1.23.3.1	Void	883

9.1.23.3.2	Void	883

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	883

9.1.24	Void	884

9.1.25	Measurement accuracy for UE category M2	884

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	884

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	885

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	886

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	887

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	889

9.1.26	Measurement Accuracy for non-BL CE UE	890

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	891

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	893

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	893

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	895

9.1.26.5	RSRP Measurement Report Mapping	896

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	896

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	896

9.1.26.8	RSRQ Measurement Report Mapping	897

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	897

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	898

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	899

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	900

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	901

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	901

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	901

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	902

9.1.27	SFN and frame Timing Difference (SFTD)	903

9.1.27.1	SFTD Accuracy Requirement	903

9.1.28	SFN and Frame Timing Difference (SFTD) under CCA	905

9.1.28.1	SFTD Accuracy Requirement under CCA	905

9.2	UTRAN FDD Measurements	906

9.2.1	UTRAN FDD CPICH RSCP	906

9.2.2	Void	907

9.2.3	UTRAN FDD CPICH Ec/No	907

9.3	UTRAN TDD Measurements	907

9.3.1	UTRAN TDD P-CCPCH RSCP	908

9.3.2	Void	908

9.3.3	Void	908

9.4	GSM Measurements	908

9.4.1	GSM carrier RSSI	908

9.5	CDMA2000 1x RTT Measurements	908

9.5.1	CDMA2000 1x RTT Pilot Strength	908

9.6	PCMAX,c	909

9.6.1	Report Mapping	909

9.6.2 	Estimation Period	909

9.6.3	Reporting Delay	909

9.7	IEEE802.11 Measurements	909

9.7.1	WLAN RSSI	909

9.7.2	WLAN RSSI Measurement Report Mapping	909

9.8	MBSFN Measurements	910

9.8.1	Introduction	910

9.8.2	MBSFN RSRP	910

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	910

9.8.2.2	MBSFN RSRP measurement report mapping	911

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	911

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	912

9.8.2.5	MBSFN RSRP measurement report mapping for 2.5 kHz subcarrier spacing	912

9.8.2.6	MBSFN RSRP measurement report mapping for 370.37Hz subcarrier spacing	912

9.8.3	MBSFN RSRQ	913

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	913

9.8.3.2	MBSFN RSRQ measurement report mapping	913

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	914

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	914

9.8.3.5	MBSFN RSRQ measurement report mapping for 2.5 kHz subcarrier spacing	914

9.8.3.6	MBSFN RSRQ measurement report mapping for 370.37 kHz subcarrier spacing	915

9.8.4	MCH BLER	915

9.8.4.1	Measurement report mapping for MCH BLER	915

9.8.4.2	Measurement report mapping for MCH Block Number	916

9.9	ProSe Measurements	917

9.9.1	Introduction	917

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	917

9.9.2.1	Absolute S-RSRP Accuracy	917

9.9.2.2	Relative Accuracy of S-RSRP	918

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	919

9.9.3.1	Absolute SD-RSRP Accuracy	919

9.9.3.2	Relative Accuracy of SD-RSRP	919

9.10	V2X Measurements	920

9.10.1	Introduction	920

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	920

9.10.2.1	Absolute S-RSRP Accuracy	920

9.10.2.2	Relative Accuracy of S-RSRP	921

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	921

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	921

9.10.4	S-RSSI Measurement Accuracy Requirements	922

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	922

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	922

9.11	NR Measurements	923

9.11.1	NR SS-RSRP Measurements	923

9.11.1A	NR SS-RSRP Measurements for DC Idle Mode Measurements	923

9.11.2	NR SS-RSRQ Measurements	924

9.11.2A	NR SS-RSRQ Measurements for DC Idle Mode Measurements	924

9.11.3	NR SS-SINR Measurements	924

9.11.4	NR SS-RSRP Measurements under CCA	925

9.11.5	NR SS-RSRQ Measurements under CCA	925

9.11.6	NR SS-SINR Measurements under CCA	925

9.11.7	NR RSSI Measurements under CCA	925

9.11.8	NR Channel Occupancy Measurements under CCA	926

10	Measurements Performance Requirements for E-UTRAN	926

10.1	Received Interference Power	926

10.1.1	Absolute accuracy requirement	926

10.1.2	Relative accuracy requirement	926

10.1.3	Received Interference Power measurement report mapping	927

10.2	Angle of Arrival (AOA)	927

10.2.1	Range/mapping	927

10.3	Timing Advance (TADV)	927

10.3.1	Report mapping	927

11	ProSe Requirements in Any Cell Selection state	928

11.1	Introduction	928

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	928

11.2.1	Introduction	928

11.2.2	ProSe UE transmission timing	928

11.3	Initiation/Cease of SLSS Transmissions	929

11.3.1	Introduction	929

11.3.2	Requirements	929

11.4	Measurements for ProSe in Any Cell Selection State	929

11.4.1	Introduction	929

11.4.2	Requirements	929

11.4.2.1	E-UTRA FDD	929

11.4.2.2	E-UTRA TDD	930

11.5	Selection / Reselection of ProSe Synchronization Reference	930

11.5.1	Introduction	930

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	930

11.5.2.1	Introduction	930

11.5.2.2	Requirements	930

11.6	Void	931

11.7	Selection / Reselection of ProSe relay UE	931

11.7.1	Introduction	931

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	931

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	931

12.1	Introduction	931

12.2	Transmit Timing	932

12.2.1	GNSS as timing reference	932

12.3	Interruption	932

12.4	Reliability of GNSS signal	932

13	V2X Requirements	932

13.1	Introduction	932

13.2	UE Transmit Timing	933

13.2.1	Introduction	933

13.2.2	GNSS as synchronization reference source	933

13.2.3	Serving cell/PCell as synchronization reference source	933

13.2.4	SyncRef UE as synchronization reference source	933

13.3	Initiation/Cease of SLSS Transmissions	933

13.3.1	Introduction	933

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	933

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	934

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	934

13.4	Selection / Reselection of V2X Synchronization Reference Source	935

13.5	Autonomous Resource Selection/Reselection measurements	936

13.5.1	Introduction	936

13.5.2	PSSCH-RSRP measurements	936

13.5.3	S-RSSI measurements	936

13.6	Congestion Control measurements	936

13.7	Interruption	936

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	936

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	936

13.7.3	Interruptions to WAN due to V2X Carrier Aggregation	937

13.7.4	Interruptions to WAN due to NR V2X sidelink communication	937

13.8	Reliability of GNSS signal	937

13.9	Component Carrier Addition and Release Delay for V2X Sidelink Carrier Aggregation	938

13.10	Selection / Reselection of V2X Synchronization Reference Source for V2X Carrier Aggregation	938

Annex A (normative): Test Cases	818

A.1	Purpose of annex	818

A.2	Requirement classification for statistical testing	818

A.2.1	Types of requirements in TS 36.133	818

A.2.1.1	Time and delay requirements on UE higher layer actions	818

A.2.1.2	Measurements of power levels, relative powers and time	818

A.2.1.3	Implementation requirements	819

A.2.1.4	Physical layer timing requirements	819

A.3	RRM test configurations	820

A.3.1	Reference Measurement Channels	820

A.3.1.1	PDSCH	820

A.3.1.1.1	FDD	820

A.3.1.1.2	TDD	825

A.3.1.1.3	FDD for UE category 0	828

A.3.1.1.4	HD-FDD for UE category 0	829

A.3.1.1.5	TDD for UE category 0	830

A.3.1.1.6	Frame Structure 3	831

A.3.1.2	PCFICH/PDCCH/PHICH	832

A.3.1.2.1	FDD	832

A.3.1.2.2	TDD	832

A.3.1.2.3	HD-FDD for UE category 0	833

A.3.1.2.4	FS 3	833

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	833

A.3.1.3.1	FDD in CEModeA	834

A.3.1.3.2	HD-FDD in CEModeA	834

A.3.1.3.3	TDD in CEModeA	835

A.3.1.3.4	FDD in CEModeB	835

A.3.1.3.5	HD-FDD in CEModeB	836

A.3.1.3.6	TDD in CEModeB	836

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	837

A.3.1.4.1	FDD in CEModeA	837

A.3.1.4.2	HD-FDD in CEModeA	838

A.3.1.4.3	TDD in CEModeA	839

A.3.1.4.4	FDD in CEModeB	840

A.3.1.4.5	 HD-FDD in CEModeB	841

A.3.1.4.6	TDD in CEModeB	841

A.3.1.5	NPDSCH Reference Channel for UE category NB1	842

A.3.1.5.1	HD-FDD in-band operation	842

A.3.1.5.2	Void	843

A.3.1.5.3	HD-FDD standalone operation	843

A.3.1.5.4	Void	844

A.3.1.5.5	HD-FDD guard band operation	844

A.3.1.5.6	Void	845

A.3.1.5.7	TDD in-band operation	845

A.3.1.5.8	TDD standalone operation	845

A.3.1.5.9	TDD guard band operation	846

A.3.1.6	NPDCCH Reference Channel for UE category NB1	846

A.3.1.6.1	In-band operation	846

A.3.1.6.2	Void	847

A.3.1.6.3	Standalone operation	847

A.3.1.6.4	Void	847

A.3.1.6.5	Guard band operation	847

A.3.1.6.6	Void	848

A.3.2	OFDMA Channel Noise Generator (OCNG)	848

A.3.2.1	OCNG Patterns for FDD	848

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	849

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	849

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	850

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	851

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	851

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	852

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	852

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	852

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	853

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	854

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	854

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	855

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	855

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	856

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	856

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	857

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	857

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	858

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	858

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	859

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	859

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	860

A.3.2.2	OCNG Patterns for TDD	860

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	861

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	861

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	862

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	862

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	863

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	864

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	865

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	866

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	866

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	867

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	868

A.3.2.3	OCNG Patterns for Narrowband IoT	868

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	870

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	871

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	871

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	872

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	873

A.3.2.3.6	Narrowband IoT OCNG TDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	874

A.3.2.3.7	Narrowband IoT OCNG TDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	875

A.3.2.3.8	Narrowband IoT OCNG TDD pattern 3: standalone NB-IoT	876

A.3.2.4	OCNG Patterns for V2X sidelink	877

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	877

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	878

A.3.3	Reference DRX Configurations	878

A.3.4	ABS Transmission Configurations	879

A.3.4.1	Non-MBSFN ABS Transmission Configurations	879

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	879

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	879

A.3.4.2	MBSFN ABS Transmission Configurations	880

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	880

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	881

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	881

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	881

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	882

A.3.6.1	Introduction	882

A.3.7	Test Cases with Different Channel Bandwidths	882

A.3.7.1	Introduction	882

A.3.7.2	Principle of testing	882

A.3.8	Antenna Configuration	882

A.3.8.1	Antenna connection for 4 Rx capable UEs	882

A.3.8.1.1 Introduction	882

A.3.8.1.2 Principle of testing	882

A.3.8.1.2.1 Single carrier tests	882

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	883

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	884

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	884

A.3.8.2	Antenna connection for 8 Rx capable UEs	884

A.3.8.2.1	Introduction	884

A.3.8.2.2	Principle of testing	884

A.3.8.2.2.1	Single carrier tests	884

A.3.8.2.2.2	Carrier aggregation and Dual connectivity tests	885

A.3.8.2.2.3	Antenna connection for bands where 2RX is supported	885

A.3.8.2.2.4	Antenna connection for bands where 4RX is supported	885

A.3.8.2.2.5	Antenna connection for bands where 8RX is supported	885

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	885

A.3.9.1	Introduction	885

A.3.9.2	Principle of testing	885

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	886

A.3.10.1	Introduction	886

A.3.10.2	Principle of testing	886

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	886

A.3.11.1	Introduction	886

A.3.11.2	Principle of Testing	886

A.3.12	Proximity-based Services	886

A.3.12.1	Introduction	886

A.3.12.2	Reference DRX configurations for ProSe tests	886

A.3.12.3	Test Cases with Different Channel Bandwidths	887

A.3.12.3.1	Introduction	887

A.3.12.3.2	Principle of testing	887

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	887

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	894

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	897

A.3.12.6.1	FDD	897

A.3.12.7	Reference measurement channels for ProSe Direct Communication	897

A.3.12.7.1	FDD	897

A.3.12.8	ProSe Receive Traffic Generator	898

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	898

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	898

A.3.13	Time Offset between Cells	899

A.3.13.1	Introduction	899

A.3.13.2	Definition	899

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	899

A.3.14.1	Introduction	899

A.3.14.2	Principle of testing	899

A.3.15	Dual connectivity test cases with different combination of duplex mode	899

A.3.15.1	Introduction	899

A.3.15.2	Principle of testing	899

A.3.16	Reference PRACH Configurations	900

A.3.17	Listen before talk model	900

A.3.17.1	Introduction	900

A.3.17.2	Definition	900

A.3.18	Reference NPRACH Configurations	901

A.3.19	Dual connectivity test cases with different bandwidth combinations	902

A.3.19.1	Introduction	902

A.3.19.2	Principle of testing	902

A.3.20	Category M1 UE Test Cases	903

A.3.20.1	Introduction	903

A.3.20.2	Principle of Cat-M1 UE Testing	903

A.3.20.3	Principle of Cat-M1 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	904

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	905

A.3.21.1	Introduction	905

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	905

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	906

A.3.22	Category 1bis UE Test Cases	907

A.3.22.1	Introduction	907

A.3.22.2	Principle of Category 1bis UE Testing	907

A.3.23	Category NB2 UE Test Cases	911

A.3.23.1	Introduction	911

A.3.23.2	Principle of Category NB2 UE Testing	911

A.3.24	V2X sidelink communication	914

A.3.24.1	Introduction	914

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	915

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	919

A.3.25	Category M2 UE Test Cases	920

A.3.25.1	Introduction	920

A.3.25.2	Principle of Cat-M2 UE Testing	920

A.3.25.3	Principle of Cat-M2 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	921

A.3.26	sTTI and processing time reduction test cases with different sTTI/processing time reduction scheme	922

A.3.26.1	Introduction	922

A.3.26.2	Principle of testing	922

A.3.27	LTE INACTIVE Cell Re-selection Test Cases	922

A.3.27.1	Introduction	922

A.3.27.2	Principle of INACTIVE cell re-selection Testing	922

A.4	E-UTRAN RRC_IDLE state	922

A.4.2	Cell Re-Selection	922

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	922

A.4.2.1.1	Test Purpose and Environment	922

A.4.2.1.2	Test Requirements	924

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	925

A.4.2.2.1	Test Purpose and Environment	925

A.4.2.2.2	Test Requirements	926

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	927

A.4.2.3.1	Test Purpose and Environment	927

A.4.2.3.2	Test Requirements	928

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	929

A.4.2.4.1	Test Purpose and Environment	929

A.4.2.4.2	Test Requirements	930

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	931

A.4.2.5.1	Test Purpose and Environment	931

A.4.2.5.2	Test Requirements	932

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	933

A.4.2.6.1	Test Purpose and Environment	933

A.4.2.6.2	Test Requirements	934

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	935

A.4.2.7.1	Test Purpose and Environment	935

A.4.2.7.2	Test Requirements	936

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	937

A.4.2.8.1	Test Purpose and Environment	937

A.4.2.8.2	Test Requirements	938

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	939

A.4.2.9.1	Test Purpose and Environment	939

A.4.2.9.2	Test Requirements	939

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	939

A.4.2.10.1	Test Purpose and Environment	939

A.4.2.10.2	Test Requirements	943

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	943

A.4.2.11.1	Test Purpose and Environment	943

A.4.2.11.2	Test Requirements	947

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	947

A.4.2.12.1	Test Purpose and Environment	947

A.4.2.12.2	Test Requirements	949

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	950

A.4.2.13.1	Test Purpose and Environment	950

A.4.2.13.2	Test Requirements	951

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	952

A.4.2.14.1	Test Purpose and Environment	952

A.4.2.14.2	Test Requirements	953

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	954

A.4.2.15.1	Test Purpose and Environment	954

A.4.2.15.2	Test Requirements	955

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	956

A.4.2.16.1	Test Purpose and Environment	956

A.4.2.16.2	Test Requirements	957

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	958

A.4.2.17.1	Test Purpose and Environment	958

A.4.2.17.2	Test Requirements	959

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	960

A.4.2.18.1	Test Purpose and Environment	960

A.4.2.18.2	Test Requirements	962

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	963

A.4.2.19.1	Test Purpose and Environment	963

A.4.2.19.2	Test Requirements	965

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	966

A.4.2.20.1	Test Purpose and Environment	966

A.4.2.20.2	Test Requirements	967

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	968

A.4.2.21.1	Test Purpose and Environment	968

A.4.2.21.2	Test Requirements	969

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	970

A.4.2.22.1	Test Purpose and Environment	970

A.4.2.22.2	Test Requirements	971

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	972

A.4.2.23.1	Test Purpose and Environment	972

A.4.2.23.2	Test Requirements	973

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	974

A.4.2.24.1	Test Purpose and Environment	974

A.4.2.24.2	Test Requirements	977

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	978

A.4.2.25.1	Test Purpose and Environment	978

A.4.2.25.2	Test Requirements	979

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	980

A.4.2.26.1	Test Purpose and Environment	980

A.4.2.26.2	Test Requirements	981

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	982

A.4.2.27.1	Test Purpose and Environment	982

A.4.2.27.2	Test Requirements	983

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	984

A.4.2.28.1	Test Purpose and Environment	984

A.4.2.28.2	Test Requirements	985

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	986

A.4.2.29.1	Test Purpose and Environment	986

A.4.2.29.2	Test Requirements	987

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	988

A.4.2.30.1	Test Purpose and Environment	988

A.4.2.30.2	Test Requirements	989

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	990

A.4.2.31.1	Test Purpose and Environment	990

A.4.2.31.2	Test Requirements	991

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	992

A.4.2.32.1	Test Purpose and Environment	992

A.4.2.32.2	Test Requirements	993

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	994

A.4.2.33.1	Test Purpose and Environment	994

A.4.2.33.2	Test Requirements	995

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	996

A.4.2.34.1	Test Purpose and Environment	996

A.4.2.34.2	Test Requirements	997

A.4.2.35	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	998

A.4.2.35.1	Test Purpose and Environment	998

A.4.2.35.2	Test Requirements	1000

A.4.2.36	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	1001

A.4.2.36.1	Test Purpose and Environment	1001

A.4.2.36.2	Test Requirements	1003

A.4.2.37	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	1004

A.4.2.37.1	Test Purpose and Environment	1004

A.4.2.37.2	Test Requirements	1006

A.4.2.38	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation	1007

A.4.2.38.1	Test Purpose and Environment	1007

A.4.2.38.2	Test Requirements	1009

A.4.2.39	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1010

A.4.2.39.1	Test Purpose and Environment	1010

A.4.2.39.2	Test Requirements	1011

A.4.2.40	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1012

A.4.2.40.1	Test Purpose and Environment	1012

A.4.2.40.2	Test Requirements	1013

A.4.2.41 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1014

A.4.2.41.1	Test Purpose and Environment	1014

A.4.2.41.2	Test Requirements	1017

A.4.2.42	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1018

A.4.2.42.1	Test Purpose and Environment	1018

A.4.2.42.2	Test Requirements	1020

A.4.2.43	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1021

A.4.2.43.1	Test Purpose and Environment	1021

A.4.2.43.2	Test Requirements	1023

A.4.2.44	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1024

A.4.2.44.1	Test Purpose and Environment	1024

A.4.2.44.2	Test Requirements	1026

A.4.2.45	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1027

A.4.2.45.1	Test Purpose and Environment	1027

A.4.2.45.2	Test Requirements	1029

A.4.2.46	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1030

A.4.2.46.1	Test Purpose and Environment	1030

A.4.2.46.2	Test Requirements	1032

A.4.2.47	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation with UE specific DRX	1033

A.4.2.47.1	Test Purpose and Environment	1033

A.4.2.47.2	Test Requirements	1035

A.4.2.48	E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1036

A.4.2.48.1	Test Purpose and Environment	1036

A.4.2.48.2	Test Requirements	1037

A.4.2.49	E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1038

A.4.2.49.1	Test Purpose and Environment	1038

A.4.2.49.2	Test Requirements	1040

A.4.2.50	E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1041

A.4.2.50.1	Test Purpose and Environment	1041

A.4.2.50.2	Test Requirements	1043

A.4.2.51 E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1044

A.4.2.51.1	Test Purpose and Environment	1044

A.4.2.51.2	Test Requirements	1046

A.4.2.52 E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1047

A.4.2.52.1	Test Purpose and Environment	1047

A.4.2.52.2	Test Requirements	1049

A.4.2.53 E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1050

A.4.2.53.1	Test Purpose and Environment	1050

A.4.2.53.2	Test Requirements	1051

A.4.2.54	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1052

A.4.2.54.1	Test Purpose and Environment	1052

A.4.2.54.2	Test Requirements	1054

A.4.2.55	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1055

A.4.2.55.1	Test Purpose and Environment	1055

A.4.2.55.2	Test Requirements	1057

A.4.2.56	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	1058

A.4.2.56.1	Test Purpose and Environment	1058

A.4.2.56.2	Test Requirements	1059

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	1060

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	1060

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	1060

A.4.3.1.1.1	Test Purpose and Environment	1060

A.4.3.1.1.2	Test Requirements	1062

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	1062

A.4.3.1.2.1	Test Purpose and Environment	1062

A.4.3.1.2.2	Test Requirements	1065

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	1065

A.4.3.1.3.1	Test Purpose and Environment	1065

A.4.3.1.3.2	Test Requirements	1068

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	1068

A.4.3.1.4.1	Test Purpose and Environment	1068

A.4.3.1.4.2	Test Requirements	1069

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	1069

A.4.3.1.5.1	Test Purpose and Environment	1069

A.4.3.1.5.2	Test Requirements	1072

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	1073

A.4.3.2.1	Test Purpose and Environment	1073

A.4.3.2.1.1	Void	1073

A.4.3.2.1.2	1.28Mcps TDD option	1073

A.4.3.2.1.3	Void	1075

A.4.3.2.2	Test Requirements	1075

A.4.3.2.2.1	1.28Mcps TDD option	1075

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	1075

A.4.3.2A.1	Test Purpose and Environment	1075

A.4.3.2A.2	Test Requirements	1079

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	1079

A.4.3.3.1	Test Purpose and Environment	1079

A.4.3.3.2	Test Requirements	1082

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	1082

A.4.3.3A.1	Test Purpose and Environment	1082

A.4.3.3A.2	Test Requirements	1087

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	1088

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	1088

A.4.3.4.1.1	Test Purpose and Environment	1088

A.4.3.4.1.2	Test Requirements	1090

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	1090

A.4.3.4.2.1	Test Purpose and Environment	1090

A.4.3.4.2.2	Test Requirements	1092

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	1092

A.4.3.4.3.1	Test Purpose and Environment	1092

A.4.3.4.3.2	Test Requirements	1095

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	1095

A.4.3.4.4.1	Test Purpose and Environment	1095

A.4.3.4.4.2	Test Requirements	1099

A.4.4	E-UTRAN to GSM Cell Re-Selection	1099

A.4.4.1	E-UTRAN FDD – GSM:	1099

A.4.4.1.1	Test Purpose and Environment	1099

A.4.4.1.2	Test Requirements	1101

A.4.4.2	E-UTRAN TDD – GSM:	1101

A.4.4.2.1	Test Purpose and Environment	1101

A.4.4.2.2	Test Requirements	1103

A.4.5	E-UTRAN to HRPD Cell Re-Selection	1104

A.4.5.1	E-UTRAN FDD – HRPD	1104

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	1104

A.4.5.1.1.1	Test Purpose and Environment	1104

A.4.5.1.1.2	Test Requirements	1106

A.4.5.2	E-UTRAN TDD – HRPD	1106

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	1106

A.4.5.2.1.1	Test Purpose and Environment	1106

A.4.5.2.1.2	Test Requirements	1109

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	1109

A.4.6.1	E-UTRAN FDD – cdma2000 1X	1109

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1109

A.4.6.1.1.1	Test Purpose and Environment	1109

A.4.6.1.1.2	Test Requirements	1112

A.4.6.2	E-UTRAN TDD – cdma2000 1X	1112

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1112

A.4.6.2.1.1	Test Purpose and Environment	1112

A.4.6.2.1.2	Test Requirements	1115

A.4.7	Idle State Positioning Measurement for UE category NB1	1115

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1115

A.4.7.1.1	Test Purpose and Environment	1115

A.4.7.1.2	Test Requirements	1120

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1121

A.4.7.2.1	Test Purpose and Environment	1121

A.4.7.4.2	Test Requirements	1125

A.4.7.3	TDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1126

A.4.7.3.1	Test Purpose and Environment	1126

A.4.7.3.2	Test Requirements	1130

A.4.7.4	TDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1131

A.4.7.4.1	Test Purpose and Environment	1131

A.4.7.4.2	Test Requirements	1135

A.5	E-UTRAN RRC CONNECTED Mode Mobility	1136

A.5.1	E-UTRAN Handover	1136

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	1136

A.5.1.1.1	Test Purpose and Environment	1136

A.5.1.1.2	Test Requirements	1138

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	1138

A.5.1.2.1	Test Purpose and Environment	1138

A.5.1.2.2	Test Requirements	1140

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	1140

A.5.1.3.1	Test Purpose and Environment	1140

A.5.1.3.2	Test Requirements	1142

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	1142

A.5.1.4.1	Test Purpose and Environment	1142

A.5.1.4.2	Test Requirements	1144

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	1144

A.5.1.5.1	Test Purpose and Environment	1144

A.5.1.5.2	Test Requirements	1146

A.5.1.6 	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	1146

A.5.1.6.1	Test Purpose and Environment	1146

A.5.1.6.2	Test Requirements	1148

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	1148

A.5.1.7.1	Test Purpose and Environment	1148

A.5.1.7.2	Test Requirements	1151

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	1151

A.5.1.8.1	Test Purpose and Environment	1151

A.5.1.8.2	Test Requirements	1154

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	1154

A.5.1.9.1	Test Purpose and Environment	1154

A.5.1.9.2	Test Requirements	1155

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	1155

A.5.1.10.1	Test Purpose and Environment	1155

A.5.1.10.2	Test Requirements	1157

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	1157

A.5.1.11.1	Test Purpose and Environment	1157

A.5.1.11.2	Test Requirements	1159

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	1159

A.5.1.12.1	Test Purpose and Environment	1159

A.5.1.12.2	Test Requirements	1161

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1161

A.5.1.13.1	Test Purpose and Environment	1161

A.5.1.13.2	Test Requirements	1163

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1164

A.5.1.14.1	Test Purpose and Environment	1164

A.5.1.14.2	Test Requirements	1165

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	1166

A.5.1.15.1	Test Purpose and Environment	1166

A.5.1.15.2	Test Requirements	1167

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1168

A.5.1.16.1	Test Purpose and Environment	1168

A.5.1.16.2	Test Requirements	1169

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1170

A.5.1.17.1	Test Purpose and Environment	1170

A.5.1.17.2	Test Requirements	1171

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	1172

A.5.1.18.1	Test Purpose and Environment	1172

A.5.1.18.2	Test Requirements	1173

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	1174

A.5.1.19.1	Test Purpose and Environment	1174

A.5.1.19.2	Test Requirements	1175

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	1176

A.5.1.20.1	Test Purpose and Environment	1176

A.5.1.20.2	Test Requirements	1177

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	1178

A.5.1.21.1	Test Purpose and Environment	1178

A.5.1.21.2	Test Requirements	1179

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	1179

A.5.1.22.1	Test Purpose and Environment	1179

A.5.1.22.2	Test Requirements	1181

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	1181

A.5.1.23.1	Test Purpose and Environment	1181

A.5.1.23.2	Test Requirements	1183

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	1183

A.5.1.24.1	Test Purpose and Environment	1183

A.5.1.24.2	Test Requirements	1185

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	1185

A.5.1.25.1	Test Purpose and Environment	1185

A.5.1.25.2	Test Requirements	1187

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	1188

A.5.1.26.1	Test Purpose and Environment	1188

A.5.1.26.2	Test Requirements	1189

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	1190

A.5.1.27.1	Test Purpose and Environment	1190

A.5.1.27.2	Test Requirements	1191

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	1192

A.5.1.28.1	Test Purpose and Environment	1192

A.5.1.28.2	Test Requirements	1193

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	1194

A.5.1.29.1	Test Purpose and Environment	1194

A.5.1.29.2	Test Requirements	1195

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	1196

A.5.1.30.1	Test Purpose and Environment	1196

A.5.1.30.2	Test Requirements	1197

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	1198

A.5.1.31.1	Test Purpose and Environment	1198

A.5.1.31.2	Test Requirements	1199

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	1200

A.5.1.32.1	Test Purpose and Environment	1200

A.5.1.32.2	Test Requirements	1201

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1202

A.5.1.33.1	Test Purpose and Environment	1202

A.5.1.13.2	Test Requirements	1203

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1204

A.5.1.34.1	Test Purpose and Environment	1204

A.5.1.34.2	Test Requirements	1205

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1206

A.5.1.35.1	Test Purpose and Environment	1206

A.5.1.35.2	Test Requirements	1207

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1208

A.5.1.36.1	Test Purpose and Environment	1208

A.5.1.36.2	Test Requirements	1209

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1210

A.5.1.37.1	Test Purpose and Environment	1210

A.5.1.37.2	Test Requirements	1211

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1212

A.5.1.38.1	Test Purpose and Environment	1212

A.5.1.38.2	Test Requirements	1213

A.5.1.39	E-UTRAN FDD - FDD Intra frequency handover with direct SCell activation	1214

A.5.1.39.1	Test Purpose and Environment	1214

A.5.1.39.2	Test Requirements	1215

A.5.1.40	E-UTRAN TDD - TDD Intra frequency handover with direct SCell activation	1216

A.5.1.40.1	Test Purpose and Environment	1216

A.5.1.40.2	Test Requirements	1217

A.5.1.41	E-UTRAN FDD – FDD Intra-band Inter-frequency sync DAPS handover	1218

A.5.1.41.1	Test Purpose and Environment	1218

A.5.1.41.2	Test Requirements	1220

A.5.1.42	E-UTRAN FDD – FDD Intra-band Inter-frequency async DAPS handover	1221

A.5.1.42.1	Test Purpose and Environment	1221

A.5.1.42.2	Test Requirements	1222

A.5.1.43	E-UTRAN FDD – FDD Inter-band Inter-frequency sync DAPS handover	1223

A.5.1.43.1	Test Purpose and Environment	1223

A.5.1.43.2	Test Requirements	1224

A.5.1.44	E-UTRAN FDD – FDD Inter-band Inter-frequency async DAPS handover	1225

A.5.1.44.1	Test Purpose and Environment	1225

A.5.1.44.2	Test Requirements	1226

A.5.1.45	E-UTRAN FDD - FDD Intra frequency DAPS handover	1227

A.5.1.45.1	Test Purpose and Environment	1227

A.5.1.45.1	Test Requirements	1228

A.5.1.46	E-UTRAN TDD - TDD Intra frequency DAPS handover	1229

A.5.1.46.1	Test Purpose and Environment	1229

A.5.1.46.2	Test Requirements	1230

A.5.1.47	E-UTRAN FDD - FDD Intra frequency conditional handover	1231

A.5.1.47.1	Test Purpose and Environment	1231

A.5.1.47.2	Test Requirements	1232

A.5.1.48	E-UTRAN TDD - TDD Intra frequency conditional handover	1233

A.5.1.48.1	Test Purpose and Environment	1233

A.5.1.48.2	Test Requirements	1234

A.5.1.49	E-UTRAN FDD - FDD Inter frequency conditional handover	1235

A.5.1.49.1	Test Purpose and Environment	1235

A.5.1.49.2	Test Requirements	1236

A.5.1.50	E-UTRAN TDD - TDD Inter frequency conditional handover	1237

A.5.1.50.1	Test Purpose and Environment	1237

A.5.1.50.2	Test Requirements	1238

A.5.1.51	E-UTRAN FDD - TDD Inter frequency conditional handover	1239

A.5.1.51.1	Test Purpose and Environment	1239

A.5.1.51.2	Test Requirements	1241

A.5.1.52	E-UTRAN TDD - FDD Inter frequency conditional handover	1241

A.5.152.1	Test Purpose and Environment	1241

A.5.1.52.2	Test Requirements	1244

A.5.1.53	E-UTRAN TDD – TDD Intra-band Inter-frequency sync DAPS handover	1245

A.5.1.53.1	Test Purpose and Environment	1245

A.5.1.53.2	Test Requirements	1247

A.5.1.54	E-UTRAN TDD – TDD Inter-band Inter-frequency sync DAPS handover	1248

A.5.1.54.1	Test Purpose and Environment	1248

A.5.1.54.2	Test Requirements	1250

A.5.1.55	E-UTRAN FDD - TDD inter-band inter-frequency synchronous DAPS handover	1251

A.5.1.55.1	Test Purpose and Environment	1251

A.5.1.56	E-UTRAN TDD - FDD inter-band inter-frequency synchronous DAPS handover	1255

A.5.1.56.1	Test Purpose and Environment	1255

A.5.1.56.2	Test Requirements	1258

A.5.1.57	E-UTRAN FDD – TDD Inter-band Inter-frequency async DAPS handover	1259

A.5.1.57.1	Test Purpose and Environment	1259

A.5.1.57.2	Test Requirements	1261

A.5.1.58	E-UTRAN TDD – FDD Inter-band Inter-frequency async DAPS handover	1261

A.5.1.58.1	Test Purpose and Environment	1261

A.5.1.58.2	Test Requirements	1264

A.5.2	E-UTRAN Handover to other RATs	1264

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	1264

A.5.2.1.1	Test Purpose and Environment	1264

A.5.2.1.2	Test Requirements	1266

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	1267

A.5.2.2.1	Test Purpose and Environment	1267

A.5.2.2.2	Test Requirements	1270

A.5.2.3 E-UTRAN FDD- GSM Handover	1270

A.5.2.3.1	Test Purpose and Environment	1270

A.5.2.3.2	Test Requirements	1271

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	1272

A.5.2.4.1	Test Purpose and Environment	1272

A.5.2.4.1.1	Void	1272

A.5.2.4.1.2	1.28 Mcps TDD option	1272

A.5.2.4.1.3	Void	1274

A.5.2.4.2	Test Requirements	1274

A.5.2.4.2.1	Void	1274

A.5.2.4.2.2	1.28 Mcps TDD option	1274

A.5.2.4.2.3	Void	1274

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	1274

A.5.2.5.1	Test Purpose and Environment	1274

A.5.2.5.1.3	Void	1277

A.5.2.5.2	Test Requirements	1277

A.5.2.5.2.1	Void	1277

A.5.2.5.2.2	1.28 Mcps TDD option	1277

A.5.2.5.2.3	Void	1277

A.5.2.6	E-UTRAN TDD - GSM Handover	1277

A.5.2.6.1	Test Purpose and Environment	1277

A.5.2.6.2	Test Requirements	1279

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	1280

A.5.2.7.1	Test Purpose and Environment	1280

A.5.2.7.2	Test Requirements	1282

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	1282

A.5.2.8.1	Test Purpose and Environment	1282

A.5.2.8.2	Test Requirements	1283

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	1284

A.5.2.9.1	Test Purpose and Environment	1284

A.5.2.9.2	Test Requirements	1285

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	1286

A.5.2.10.1	Test Purpose and Environment	1286

A.5.2.10.2	Test Requirements	1288

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	1288

A.5.2.10A.1	Test Purpose and Environment	1288

A.5.2.10A.2	Test Requirements	1291

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	1291

A.5.2.10B.1	Test Purpose and Environment	1291

A.5.2.10B.2	Test Requirements	1294

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	1294

A.5.2.11.1	Test Purpose and Environment	1294

A.5.2.11.2	Test Requirements	1294

A.5.3	E-UTRAN Handover to Non-3GPP RATs	1295

A.5.3.1	E-UTRAN FDD – HRPD Handover	1295

A.5.3.1.1	Test Purpose and Environment	1295

A.5.3.1.2	Test Requirements	1297

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	1297

A.5.3.2.1	Test Purpose and Environment	1297

A.5.3.2.2	Test Requirements	1300

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	1300

A.5.3.3.1	Test Purpose and Environment	1300

A.5.3.3.2	Test Requirements	1303

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	1303

A.5.3.4.1	Test Purpose and Environment	1303

A.5.3.4.2	Test Requirements	1304

A.5.3.5	E-UTRAN TDD – HRPD Handover	1305

A.5.3.5.1	Test Purpose and Environment	1305

A.5.3.5.2	Test Requirements	1308

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	1308

A.5.3.6.1	Test Purpose and Environment	1308

A.5.3.6.2	Test Requirements	1311

A.6	RRC Connection Control	1311

A.6.1	RRC Re-establishment	1311

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	1311

A.6.1.1.1	Test Purpose and Environment	1311

A.6.1.1.2	Test Requirements	1312

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	1313

A.6.1.2.1	Test Purpose and Environment	1313

A.6.1.2.2	Test Requirements	1314

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	1315

A.6.1.3.1	Test Purpose and Environment	1315

A.6.1.3.2	Test Requirements	1316

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	1317

A.6.1.4.1	Test Purpose and Environment	1317

A.6.1.4.2	Test Requirements	1318

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	1319

A.6.1.5.1	Test Purpose and Environment	1319

A.6.1.5.2	Test Requirements	1319

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	1319

A.6.1.6.1	Test Purpose and Environment	1319

A.6.1.6.2	Test Requirements	1321

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	1322

A.6.1.7.1	Test Purpose and Environment	1322

A.6.1.7.2	Test Requirements	1323

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	1324

A.6.1.8.1	Test Purpose and Environment	1324

A.6.1.8.2	Test Requirements	1325

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1326

A.6.1.9.1	Test Purpose and Environment	1326

A.6.1.9.2	Test Requirements	1327

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1328

A.6.1.10.1	Test Purpose and Environment	1328

A.6.1.10.2	Test Requirements	1329

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1330

A.6.1.11.1	Test Purpose and Environment	1330

A.6.1.11.2	Test Requirements	1331

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1332

A.6.1.12.1	Test Purpose and Environment	1332

A.6.1.12.2	Test Requirements	1333

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1334

A.6.1.13.1	Test Purpose and Environment	1334

A.6.1.13.2	Test Requirements	1335

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1336

A.6.1.14.1	Test Purpose and Environment	1336

A.6.1.14.2	Test Requirements	1337

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1338

A.6.1.15.1	Test Purpose and Environment	1338

A.6.1.15.2	Test Requirements	1340

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1341

A.6.1.16.1	Test Purpose and Environment	1341

A.6.1.16.2	Test Requirements	1343

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1344

A.6.1.17.1	Test Purpose and Environment	1344

A.6.1.17.2	Test Requirements	1345

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1346

A.6.1.18.1	Test Purpose and Environment	1346

A.6.1.18.2	Test Requirements	1347

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1348

A.6.1.19.1	Test Purpose and Environment	1348

A.6.1.19.2	Test Requirements	1349

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1350

A.6.1.20.1	Test Purpose and Environment	1350

A.6.1.20.2	Test Requirements	1351

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1352

A.6.1.21.1	Test Purpose and Environment	1352

A.6.1.21.2	Test Requirements	1353

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1354

A.6.1.22.1	Test Purpose and Environment	1354

A.6.1.22.2	Test Requirements	1355

A.6.1.23	E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1356

A.6.1.23.1	Test Purpose and Environment	1356

A.6.1.23.2	Test Requirements	1358

A.6.1.24	E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1359

A.6.1.24.1	Test Purpose and Environment	1359

A.6.1.24.2	Test Requirements	1361

A.6.2	Random Access	1362

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1362

A.6.2.1.1	Test Purpose and Environment	1362

A.6.2.1.2	Test Requirements	1363

A.6.2.1.2.1	Random Access Response Reception	1363

A.6.2.1.2.2	No Random Access Response Reception	1363

A.6.2.1.2.3	Receiving a NACK on msg3	1363

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1364

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1364

A.6.2.1.2.6	Contention Resolution Timer expiry	1364

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1364

A.6.2.2.1	Test Purpose and Environment	1364

A.6.2.2.2	Test Requirements	1365

A.6.2.2.2.1	Random Access Response Reception	1366

A.6.2.2.2.2	No Random Access Response Reception	1366

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1366

A.6.2.3.1	Test Purpose and Environment	1366

A.6.2.3.2	Test Requirements	1368

A.6.2.3.2.1	Random Access Response Reception	1368

A.6.2.3.2.2	No Random Access Response reception	1368

A.6.2.3.2.3	Receiving a NACK on msg3	1368

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1369

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1369

A.6.2.3.2.6	Contention Resolution Timer expiry	1369

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1369

A.6.2.4.1	Test Purpose and Environment	1369

A.6.2.4.2	Test Requirements	1371

A.6.2.4.2.1	Random Access Response Reception	1371

A.6.2.4.2.2	No Random Access Response Reception	1371

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1371

A.6.2.5.1	Test Purpose and Environment	1371

A.6.2.5.2	Test Requirements	1372

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1372

A.6.2.6.1	Test Purpose and Environment	1372

A.6.2.6.2	Test Requirements	1372

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1373

A.6.2.7.1	Test Purpose and Environment	1373

A.6.2.7.2	Test Requirements	1375

A.6.2.7.2.1	Random Access Response Reception	1375

A.6.2.7.2.2	No Random Access Response Reception	1375

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1375

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1376

A.6.2.8.1	Test Purpose and Environment	1376

A.6.2.8.2	Test Requirements	1378

A.6.2.8.2.1	Random Access Response Reception	1378

A.6.2.8.2.2	No Random Access Response Reception	1378

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1378

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1379

A.6.2.9.1	Test Purpose and Environment	1379

A.6.2.9.2	Test Requirements	1382

A.6.2.9.2.1	Random Access Response Reception	1382

A.6.2.9.2.2	No Random Access Response Reception	1383

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1383

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1384

A.6.2.10.1	Test Purpose and Environment	1384

A.6.2.10.2	Test Requirements	1386

A.6.2.10.2.1	Random Access Response Reception	1386

A.6.2.10.2.2	No Random Access Response Reception	1387

A.6.2.10.2.3	Receiving a NACK on msg3	1387

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1387

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1387

A.6.2.10.2.6	Contention Resolution Timer expiry	1387

A.6.2.10.2.7	PRACH Resource Selection	1388

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1388

A.6.2.11.1	Test Purpose and Environment	1388

A.6.2.11.2	Test Requirements	1390

A.6.2.11.2.1	Random Access Response Reception	1390

A.6.2.11.2.2	No Random Access Response Reception	1391

A.6.2.11.2.3	Receiving a NACK on msg3	1391

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1391

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1391

A.6.2.11.2.6	Contention Resolution Timer expiry	1391

A.6.2.11.2.7	PRACH Resource Selection	1392

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1392

A.6.2.12.1	Test Purpose and Environment	1392

A.6.2.12.2	Test Requirements	1394

A.6.2.12.2.1	Random Access Response Reception	1394

A.6.2.12.2.2	No Random Access Response Reception	1395

A.6.2.12.2.3	Receiving a NACK on msg3	1395

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1395

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1395

A.6.2.12.2.6	Contention Resolution Timer expiry	1395

A.6.2.12.2.7	PRACH Resource Selection	1396

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1396

A.6.2.13.1	Test Purpose and Environment	1396

A.6.2.13.2	Test Requirements	1398

A.6.2.13.2.1	Random Access Response Reception	1398

A.6.2.13.2.2	No Random Access Response Reception	1399

A.6.2.13.2.3	Receiving a NACK on msg3	1399

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1399

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1399

A.6.2.13.2.6	Contention Resolution Timer expiry	1399

A.6.2.13.2.7	PRACH Resource Selection	1400

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1400

A.6.2.14.1	Test Purpose and Environment	1400

A.6.2.14.2	Test Requirements	1402

A.6.2.14.2.1	Random Access Response Reception	1402

A.6.2.14.2.2	No Random Access Response Reception	1403

A.6.2.14.2.3	Receiving a NACK on msg3	1403

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1403

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1403

A.6.2.14.2.6	Contention Resolution Timer expiry	1403

A.6.2.14.2.7	PRACH Resource Selection	1404

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1404

A.6.2.15.1	Test Purpose and Environment	1404

A.6.2.15.2	Test Requirements	1406

A.6.2.15.2.1	Random Access Response Reception	1406

A.6.2.15.2.2	No Random Access Response Reception	1407

A.6.2.15.2.3	Receiving a NACK on msg3	1407

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1407

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1407

A.6.2.15.2.6	Contention Resolution Timer expiry	1407

A.6.2.15.2.7	PRACH Resource Selection	1408

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1408

A.6.2.16.1	Test Purpose and Environment	1408

A.6.2.16.2	Test Requirements	1411

A.6.2.16.2.1	Random Access Response Reception	1411

A.6.2.16.2.2	No Random Access Response Reception	1412

A.6.2.16.2.3	Receiving a NACK on msg3	1412

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1412

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1412

A.6.2.16.2.6	Contention Resolution Timer expiry	1412

A.6.2.16.2.7	NPRACH Resource Selection	1412

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1413

A.6.2.17.1	Test Purpose and Environment	1413

A.6.2.17.2	Test Requirements	1416

A.6.2.17.2.1	Random Access Response Reception	1416

A.6.2.17.2.2	No Random Access Response Reception	1417

A.6.2.17.2.3	Receiving a NACK on msg3	1417

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1417

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1417

A.6.2.17.2.6	Contention Resolution Timer expiry	1417

A.6.2.17.2.7	NPRACH Resource Selection	1417

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1418

A.6.2.18.1	Test Purpose and Environment	1418

A.6.2.18.2	Test Requirements	1421

A.6.2.18.2.1	Random Access Response Reception	1421

A.6.2.18.2.2	No Random Access Response Reception	1422

A.6.2.18.2.3	Receiving a NACK on msg3	1422

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1422

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1422

A.6.2.18.2.6	Contention Resolution Timer expiry	1422

A.6.2.18.2.7	NPRACH Resource Selection	1422

A.6.2.19	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1423

A.6.2.19.1	Test Purpose and Environment	1423

A.6.2.19.2	Test Requirements	1425

A.6.2.19.2.1	Random Access Response Reception	1425

A.6.2.19.2.2	No Random Access Response Reception	1426

A.6.2.19.2.3	Receiving a NACK on msg3	1426

A.6.2.19.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1426

A.6.2.19.2.5	Reception of a Correct Message over Temporary C-RNTI	1426

A.6.2.19.2.6	Contention Resolution Timer expiry	1426

A.6.2.19.2.7	NPRACH Resource Selection	1426

A.6.2.20	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage	1427

A.6.2.20.1	Test Purpose and Environment	1427

A.6.2.20.2	Test Requirements	1429

A.6.2.20.2.1	Random Access Response Reception	1429

A.6.2.20.2.2	No Random Access Response Reception	1430

A.6.2.20.2.3	Receiving a NACK on msg3	1430

A.6.2.20.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1430

A.6.2.20.2.5	Reception of a Correct Message over Temporary C-RNTI	1430

A.6.2.20.2.6	Contention Resolution Timer expiry	1430

A.6.2.20.2.7	NPRACH Resource Selection	1430

A.6.2.21	TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1431

A.6.2.21.1	Test Purpose and Environment	1431

A.6.2.21.2	Test Requirements	1433

A.6.2.21.2.1	Random Access Response Reception	1433

A.6.2.21.2.2	No Random Access Response Reception	1434

A.6.2.21.2.3	Receiving a NACK on msg3	1434

A.6.2.21.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1434

A.6.2.21.2.5	Reception of a Correct Message over Temporary C-RNTI	1434

A.6.2.21.2.6	Contention Resolution Timer expiry	1434

A.6.2.21.2.7	NPRACH Resource Selection	1434

A.6.3	RRC Connection Release with Redirection	1435

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1435

A.6.3.1.1	Test Purpose and Environment	1435

A.6.3.1.2	Test Requirements	1437

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1437

A.6.3.2.1	Test Purpose and Environment	1437

A.6.3.2.2	Test Requirements	1439

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1439

A.6.3.3.1	Test Purpose and Environment	1439

A.6.3.3.2	Test Requirements	1440

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1441

A.6.3.4.1	Test Purpose and Environment	1441

A.6.3.4.2	Test Requirements	1442

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1443

A.6.3.5.1	Test Purpose and Environment	1443

A.6.3.5.2	Test Requirements	1445

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1445

A.6.3.6.1	Test Purpose and Environment	1445

A. 6.3.6.2	Test Requirements	1448

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1448

A.6.3.7.1	Test Purpose and Environment	1448

A.6.3.7.2	Test Requirements	1451

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1451

A.6.3.8.1	Test Purpose and Environment	1451

A.6.3.8.2	Test Requirements	1454

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1454

A.6.3.9.1	Test Purpose and Environment	1454

A.6.3.9.2	Test Requirements	1456

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1456

A.6.3.10.1	Test Purpose and Environment	1456

A.6.3.10.2	Test Requirements	1458

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1458

A.6.3.11.1	Test Purpose and Environment	1458

A.6.3.11.2	Test Requirements	1460

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1460

A.6.3.12.1	Test Purpose and Environment	1460

A.6.3.12.2	Test Requirements	1463

A.6.3.13	Redirection from E-UTRA to NR FR1 for redcap UE	1463

A.6.3.13.1	Test Purpose and Environment	1463

A.6.3.13.2	Test Parameters	1463

A.6.3.13.3	Test Requirements	1467

A.7	Timing and Signalling Characteristics	1443

A.7.1	UE Transmit Timing	1443

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1443

A.7.1.1.1	Test Purpose and Environment	1443

A.7.1.1.2	Test Requirements	1444

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1445

A.7.1.2.1	Test Purpose and Environment	1445

A.7.1.2.2	Test Requirements	1447

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1448

A.7.1.3.1	Test Purpose and Environment	1448

A.7.1.3.2	Test Requirements	1450

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1451

A.7.1.4.1	Test Purpose and Environment	1451

A.7.1.4.2	Test Requirements	1452

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1453

A.7.1.4A.1	Test Purpose and Environment	1453

A.7.1.4A.2	Test Requirements	1453

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1453

A.7.1.5.1	Test Purpose and Environment	1453

A.7.1.5.2	Test Requirements	1454

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1454

A.7.1.6.1	Test Purpose and Environment	1454

A.7.1.6.2	Test Requirements	1456

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1457

A.7.1.7.1	Test Purpose and Environment	1457

A.7.1.7.2	Test Requirements	1458

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1459

A.7.1.7A.1	Test Purpose and Environment	1459

A.7.1.7A.2	Test Requirements	1459

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1459

A.7.1.7B.1	Test Purpose and Environment	1459

A.7.1.7B.2	Test Requirements	1459

A.7.1.8	Void	1460

A.7.1.8.1	Void	1460

A.7.1.8.2	Void	1460

A.7.1.9	Void	1460

A.7.1.9.1	Void	1460

A.7.1.9.2	Void	1460

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1460

A.7.1.10.1	Test Purpose and Environment	1460

A.7.1.10.2	Test Requirements	1462

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1462

A.7.1.11.1	Test Purpose and Environment	1462

A.7.1.11.2	Test Requirements	1464

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1464

A.7.1.12.1	Test Purpose and Environment	1464

A.7.1.12.2	Test Requirements	1466

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1466

A.7.1.13.1	Test Purpose and Environment	1466

A.7.1.13.2	Test Requirements	1468

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1469

A.7.1.14.1	Test Purpose and Environment	1469

A.7.1.14.2	Test Requirements	1470

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1470

A.7.1.15.1	Test Purpose and Environment	1470

A.7.1.15.2	Test Requirements	1471

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1472

A.7.1.16.1	Test Purpose and Environment	1472

A.7.1.16.2	Test Requirements	1473

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1474

A.7.1.17.1	Test Purpose and Environment	1474

A.7.1.17.2	Test Requirements	1476

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1477

A.7.1.18.1	Test Purpose and Environment	1477

A.7.1.18.2	Test Requirements	1479

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1480

A.7.1.19.1	Test Purpose and Environment	1480

A.7.1.19.2	Test Requirements	1481

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1481

A.7.1.20.1	Test Purpose and Environment	1481

A.7.1.20.2	Test Requirements	1483

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1483

A.7.1.21.1	Test Purpose and Environment	1483

A.7.1.21.2	Test Requirements	1485

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1485

A.7.1.22.1	Test Purpose and Environment	1485

A.7.1.22.2	Test Requirements	1487

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1487

A.7.1.23.1	Test Purpose and Environment	1487

A.7.1.23.2	Test Requirements	1489

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1489

A.7.1.24.1	Test Purpose and Environment	1489

A.7.1.24.2	Test Requirements	1490

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1491

A.7.1.25.1	Test Purpose and Environment	1491

A.7.1.25.2	Test Requirements	1492

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1493

A.7.1.26.1	Test Purpose and Environment	1493

A.7.1.26.2	Test Requirements	1494

A.7.1.27	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1495

A.7.1.27.1	Test Purpose and Environment	1495

A.7.1.27.2	Test Requirements	1497

A.7.1.28	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1497

A.7.1.28.1	Test Purpose and Environment	1497

A.7.1.28.2	Test Requirements	1499

A.7.2	UE Timing Advance	1500

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1500

A.7.2.1.1	Test Purpose and Environment	1500

A.7.2.1.2	Test Requirements	1501

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1502

A.7.2.2.1	Test Purpose and Environment	1502

A.7.2.2.2	Test Requirements	1504

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1504

A.7.2.3.1	Test Purpose and Environment	1504

A.7.2.3.2	Test Requirements	1504

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1504

A.7.2.4.1	Test Purpose and Environment	1504

A.7.2.4.2	Test Requirements	1507

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1507

A.7.2.5.1	Test Purpose and Environment	1507

A.7.2.5.2	Test Requirements	1509

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1509

A.7.2.5A.1	Test Purpose and Environment	1509

A.7.2.5A.2	Test Requirements	1510

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1510

A.7.2.5B.1	Test Purpose and Environment	1510

A.7.2.5B.2	Test Requirements	1510

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1510

A.7.2.6.1	Test Purpose and Environment	1510

A.7.2.6.2	Test Requirements	1513

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1513

A.7.2.7.1	Test Purpose and Environment	1513

A.7.2.7.2	Test Requirements	1515

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1515

A.7.2.8.1	Test Purpose and Environment	1515

A.7.2.8.2	Test Requirements	1517

A.7.2.9.2	Test Requirements	1518

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1519

A.7.2.10.1	Test Purpose and Environment	1519

A.7.2.10.2	Test Requirements	1520

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1520

A.7.2.11.1	Test Purpose and Environment	1520

A.7.2.11.2	Test Requirements	1522

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1522

A.7.2.12.1	Test Purpose and Environment	1522

A.7.2.12.2	Test Requirements	1524

A.7.2.13	E-UTRAN FDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1525

A.7.2.13.1	Test Purpose and Environment	1525

A.7.2.13.2	Test Requirements	1526

A.7.2.14	E-UTRAN TDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1527

A.7.2.14.1	Test Purpose and Environment	1527

A.7.2.14.2	Test Requirements	1529

A.7.2.15	E-UTRAN TDD – TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage	1529

A.7.2.15.1	Test Purpose and Environment	1529

A.7.2.15.2	Test Requirements	1531

A.7.3	Radio Link Monitoring	1531

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1531

A.7.3.1.1	Test Purpose and Environment	1531

A.7.3.1.2	Test Requirements	1535

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1535

A.7.3.2.1	Test Purpose and Environment	1535

A.7.3.2.2	Test Requirements	1539

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1539

A.7.3.3.1	Test Purpose and Environment	1539

A.7.3.3.2	Test Requirements	1543

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1543

A.7.3.4.1	Test Purpose and Environment	1543

A.7.3.4.2	Test Requirements	1547

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1547

A.7.3.5.1	Test Purpose and Environment	1547

A.7.3.5.2	Test Requirements	1550

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1551

A.7.3.6.1	Test Purpose and Environment	1551

A.7.3.6.2	Test Requirements	1554

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1554

A.7.3.7.1	Test Purpose and Environment	1554

A.7.3.7.2	Test Requirements	1557

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1557

A.7.3.8.1	Test Purpose and Environment	1557

A.7.3.8.2	Test Requirements	1560

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1560

A.7.3.9.2	Test Requirements	1563

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1563

A.7.3.10.1	Test Purpose and Environment	1563

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1567

A.7.3.11.1	Test Purpose and Environment	1567

A.7.3.11.2	Test Requirements	1572

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1572

A.7.3.12.1	Test Purpose and Environment	1572

A.7.3.12.2	Test Requirements	1577

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1577

A.7.3.13.1	Test Purpose and Environment	1577

A.7.3.13.2	Test Requirements	1580

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1580

A.7.3.14.1	Test Purpose and Environment	1580

A.7.3.14.2	Test Requirements	1583

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1583

A.7.3.15.1	Test Purpose and Environment	1583

A.7.3.15.2	Test Requirements	1587

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1587

A.7.3.16.1	Test Purpose and Environment	1587

A.7.3.16.2	Test Requirements	1592

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1592

A.7.3.17.1	Test Purpose and Environment	1592

A.7.3.17.2	Test Requirements	1596

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1596

A.7.3.18.1	Test Purpose and Environment	1596

A.7.3.18.2	Test Requirements	1600

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1600

A.7.3.19.1	Test Purpose and Environment	1600

A.7.3.19.2	Test Requirements	1605

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1605

A.7.3.20.1	Test Purpose and Environment	1605

A.7.3.20.2	Test Requirements	1610

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1610

A.7.3.21.1	Test Purpose and Environment	1610

A.7.3.21.2	Test Requirements	1615

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1615

A.7.3.22.1	Test Purpose and Environment	1615

A.7.3.22.2	Test Requirements	1620

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1620

A.7.3.23.1	Test Purpose and Environment	1620

A.7.3.23.2	Test Requirements	1621

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1621

A.7.3.24.1	Test Purpose and Environment	1621

A.7.3.24.2	Test Requirements	1622

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1622

A.7.3.25.1	Test Purpose and Environment	1622

A.7.3.25.2	Test Requirements	1623

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1623

A.7.3.26.1	Test Purpose and Environment	1623

A.7.3.26.2	Test Requirements	1626

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1626

A.7.3.27.1	Test Purpose and Environment	1626

A.7.3.27.2	Test Requirements	1629

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1629

A.7.3.28.1	Test Purpose and Environment	1629

A.7.3.28.2	Test Requirements	1632

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1632

A.7.3.29.1	Test Purpose and Environment	1632

A.7.3.29.2	Test Requirements	1635

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1635

A.7.3.30.1	Test Purpose and Environment	1635

A.7.3.30.2	Test Requirements	1638

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1638

A.7.3.31.1	Test Purpose and Environment	1638

A.7.3.31.2	Test Requirements	1641

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1641

A.7.3.32.1	Test Purpose and Environment	1641

A.7.3.32.2	Test Requirements	1644

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1644

A.7.3.33.1	Test Purpose and Environment	1644

A.7.3.33.2	Test Requirements	1647

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1647

A.7.3.34.1	Test Purpose and Environment	1647

A.7.3.34.2	Test Requirements	1650

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1650

A.7.3.35.1	Test Purpose and Environment	1650

A.7.3.35.2	Test Requirements	1653

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1653

A.7.3.36.1	Test Purpose and Environment	1653

A.7.3.36.2	Test Requirements	1656

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1656

A.7.3.37.1	Test Purpose and Environment	1656

A.7.3.37.2	Test Requirements	1659

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1659

A.7.3.38.1	Test Purpose and Environment	1659

A.7.3.38.2	Test Requirements	1662

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1662

A.7.3.39.1	Test Purpose and Environment	1662

A.7.3.39.2	Test Requirements	1665

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1666

A.7.3.40.1	Test Purpose and Environment	1666

A.7.3.40.2	Test Requirements	1668

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1669

A.7.3.41.1	Test Purpose and Environment	1669

A.7.3.41.2	Test Requirements	1672

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1672

A.7.3.42.1	Test Purpose and Environment	1672

A.7.3.42.2	Test Requirements	1675

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1675

A.7.3.43.1	Test Purpose and Environment	1675

A.7.3.43.2	Test Requirements	1678

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1678

A.7.3.44.1	Test Purpose and Environment	1678

A.7.3.44.2	Test Requirements	1681

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1682

A.7.3.45.1	Test Purpose and Environment	1682

A.7.3.45.2	Test Requirements	1684

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1685

A.7.3.46.1	Test Purpose and Environment	1685

A.7.3.46.2	Test Requirements	1688

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1688

A.7.3.47.1	Test Purpose and Environment	1688

A.7.3.47.2	Test Requirements	1691

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1691

A.7.3.48.1	Test Purpose and Environment	1691

A.7.3.48.2	Test Requirements	1694

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1694

A.7.3.49.1	Test Purpose and Environment	1694

A.7.3.49.2	Test Requirements	1697

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1697

A.7.3.50.1	Test Purpose and Environment	1697

A.7.3.50.2	Test Requirements	1700

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1700

A.7.3.51.1	Test Purpose and Environment	1700

A.7.3.51.2	Test Requirements	1703

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1703

A.7.3.52.1	Test Purpose and Environment	1703

A.7.3.52.2	Test Requirements	1706

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1706

A.7.3.53.1	Test Purpose and Environment	1706

A.7.3.53.2	Test Requirements	1709

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1709

A.7.3.54.1	Test Purpose and Environment	1709

A.7.3.54.2	Test Requirements	1712

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1712

A.7.3.55.1	Test Purpose and Environment	1712

A.7.3.55.2	Test Requirements	1715

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1715

A.7.3.56.1	Test Purpose and Environment	1715

A.7.3.56.2	Test Requirements	1718

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1718

A.7.3.57.1	Test Purpose and Environment	1718

A.7.3.57.2	Test Requirements	1721

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1721

A.7.3.58.1	Test Purpose and Environment	1721

A.7.3.58.2	Test Requirements	1724

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1724

A.7.3.59.1	Test Purpose and Environment	1724

A.7.3.59.2	Test Requirements	1727

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1727

A.7.3.60.1	Test Purpose and Environment	1727

A.7.3.60.2	Test Requirements	1731

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1731

A.7.3.61.1	Test Purpose and Environment	1731

A.7.3.61.2	Test Requirements	1735

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1735

A.7.3.62.1	Test Purpose and Environment	1735

A.7.3.62.2	Test Requirements	1739

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1739

A.7.3.63.1	Test Purpose and Environment	1739

A.7.3.63.2	Test Requirements	1743

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1743

A.7.3.64.1	Test Purpose and Environment	1743

A.7.3.64.2	Test Requirements	1746

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1747

A.7.3.65.1	Test Purpose and Environment	1747

A.7.3.65.2	Test Requirements	1750

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1751

A.7.3.66.1	Test Purpose and Environment	1751

A.7.3.66.2	Test Requirements	1753

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1754

A.7.3.67.1	Test Purpose and Environment	1754

A.7.3.67.2	Test Requirements	1757

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1758

A.7.3.68.1	Test Purpose and Environment	1758

A.7.3.68.2	Test Requirements	1760

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1760

A.7.3.69.1	Test Purpose and Environment	1760

A.7.3.69.2	Test Requirements	1762

A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1762

A.7.3.70.1	Test Purpose and Environment	1762

A.7.3.70.2	Test Requirements	1764

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1764

A.7.3.71.1	Test Purpose and Environment	1764

A.7.3.71.2	Test Requirements	1767

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1767

A.7.3.72.1	Test Purpose and Environment	1767

A.7.3.72.2	Test Requirements	1769

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1769

A.7.3.73.1	Test Purpose and Environment	1769

A.7.3.73.2	Test Requirements	1771

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1771

A.7.3.74.1	Test Purpose and Environment	1771

A.7.3.74.2	Test Requirements	1774

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1774

A.7.3.75.1	Test Purpose and Environment	1774

A.7.3.75.2	Test Requirements	1776

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1777

A.7.3.76.1	Test Purpose and Environment	1777

A.7.3.76.2	Test Requirements	1780

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1780

A.7.3.77.1	Test Purpose and Environment	1780

A.7.3.77.2	Test Requirements	1783

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1783

A.7.3.78.1	Test Purpose and Environment	1783

A.7.3.78.2	Test Requirements	1786

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1786

A.7.3.79.1	Test Purpose and Environment	1786

A.7.3.79.2	Test Requirements	1789

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1789

A.7.3.80.1	Test Purpose and Environment	1789

A.7.3.80.2	Test Requirements	1792

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1792

A.7.3.81.1	Test Purpose and Environment	1792

A.7.3.81.2	Test Requirements	1795

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1795

A.7.3.82.1	Test Purpose and Environment	1795

A.7.3.82.2	Test Requirements	1798

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1798

A.7.3.83.1	Test Purpose and Environment	1798

A.7.3.83.2	Test Requirements	1799

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1800

A.7.3.84.1	Test Purpose and Environment	1800

A.7.3.84.2	Test Requirements	1802

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1802

A.7.3.85.1	Test Purpose and Environment	1802

A.7.3.85.2	Test Requirements	1803

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1804

A.7.3.86.1	Test Purpose and Environment	1804

A.7.3.86.2	Test Requirements	1806

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1806

A.7.3.87.1	Test Purpose and Environment	1806

A.7.3.87.2	Test Requirements	1808

A.7.3.88	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1809

A.7.3.88.1	Test Purpose and Environment	1809

A.7.3.88.2	Test Requirements	1813

A.7.3.89	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1813

A.7.3.89.1	Test Purpose and Environment	1813

A.7.3.89.2	Test Requirements	1817

A.7.3.90	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1817

A.7.3.90.1	Test Purpose and Environment	1817

A.7.3.90.2	Test Requirements	1821

A.7.3.91	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1821

A.7.3.91.1	Test Purpose and Environment	1821

A.7.3.91.2	Test Requirements	1825

A.7.3.92	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1825

A.7.3.92.1	Test Purpose and Environment	1825

A.7.3.92.2	Test Requirements	1828

A.7.3.93	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1829

A.7.3.93.1	Test Purpose and Environment	1829

A.7.3.93.2	Test Requirements	1832

A.7.3.94	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1833

A.7.3.94.1	Test Purpose and Environment	1833

A.7.3.94.2	Test Requirements	1835

A.7.3.95	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1836

A.7.3.95.1	Test Purpose and Environment	1836

A.7.3.95.2	Test Requirements	1839

A.7.3.96	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1840

A.7.3.96.1	Test Purpose and Environment	1840

A.7.3.97	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1841

A.7.3.97.1	Test Purpose and Environment	1841

A.7.3.97.2	Test Requirements	1844

A.7.3.98	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1844

A.7.3.98.1	Test Purpose and Environment	1844

A.7.3.98.2	Test Requirements	1847

A.7.3.99	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1847

A.7.3.99.1	Test Purpose and Environment	1847

A.7.3.99.2	Test Requirements	1850

A.7.3.100	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1850

A.7.3.100.1	Test Purpose and Environment	1850

A.7.3.100.2	Test Requirements	1852

A.7.3.101	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1852

A.7.3.101.1	Test Purpose and Environment	1852

A.7.3.101.2	Test Requirements	1854

A.7.4	Interruption for Dual Connectivity	1854

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1854

A.7.4.1.1	Test Purpose and Environment	1854

A.7.4.1.2	Test Requirements	1857

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1857

A.7.4.2.1	Test Purpose and Environment	1857

A.7.4.2.2	Test Requirements	1859

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	1859

A.7.4.3.1	Test Purpose and Environment	1859

A.7.4.3.2	Test Requirements	1861

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1861

A.7.4.4.1	Test Purpose and Environment	1861

A.7.4.4.2	Test Requirements	1864

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1864

A.7.4.5.1	Test Purpose and Environment	1864

A.7.4.5.2	Test Requirements	1866

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	1866

A.7.4.6.1	Test Purpose and Environment	1866

A.7.4.6.2	Test Requirements	1868

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	1868

A.7.4.7.1	Test Purpose and Environment	1868

A.7.4.7.2	Test Requirements	1872

A.7.5	Proximity-based Services	1872

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1872

A.7.5.1.1	Test Purpose and Environment	1872

A.7.5.1.2	Test Requirements	1873

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1874

A.7.5.2.1	Test Purpose and Environment	1874

A.7.5.1.2	Test Requirements	1874

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	1875

A.7.5.3.1	Test Purpose and Environment	1875

A.7.5.3.2	Test Requirements	1876

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	1877

A.7.5.4.1	Test Purpose and Environment	1877

A.7.5.4.2	Test Requirements	1878

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	1879

A.7.5.5.1	Test Purpose and Environment	1879

A.7.5.5.2	Test Requirements	1881

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	1882

A.7.5.6.1	Test Purpose and Environment	1882

A.7.5.6.2	Test Requirements	1883

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	1884

A.7.5.7.1	Test Purpose and Environment	1884

A.7.5.7.2	Test Requirements	1886

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	1886

A.7.5.8.1	Test Purpose and Environment	1886

A.7.5.8.2	Test Requirements	1888

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	1889

A.7.5.9.1	Test Purpose and Environment	1889

A.7.5.9.2	Test Requirements	1891

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	1892

A.7.5.10.1	Test Purpose and Environment	1892

A.7.5.10.2	Test Requirements	1894

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	1895

A.7.5.11.1	Test Purpose and Environment	1895

A.7.5.11.2	Test Requirements	1897

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	1897

A.7.5.12.1	Test Purpose and Environment	1897

A.7.5.12.2	Test Requirements	1901

A.7.6	Interruption for carrier aggregation	1902

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	1902

A.7.6.1.1	Test Purpose and Environment	1902

A.7.6.1.2	Test Requirements	1905

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	1905

A.7.6.2.1	Test Purpose and Environment	1905

A.7.6.2.2	Test Requirements	1908

A.8	UE Measurements Procedures	1908

A.8.1	E-UTRAN FDD Intra-frequency Measurements	1908

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1908

A.8.1.1.1	Test Purpose and Environment	1908

A.8.1.1.2	Test Requirements	1909

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1910

A.8.1.2.1	Test Purpose and Environment	1910

A.8.1.2.2	Test Requirements	1911

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1912

A.8.1.3.1	Test Purpose and Environment	1912

A.8.1.3.2	Test Requirements	1914

A.8.1.4	Void	1914

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1914

A.8.1.5.1	Test Purpose and Environment	1914

A.8.1.5.2	Test Requirements	1916

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1916

A.8.1.6.1	Test Purpose and Environment	1916

A.8.1.6.2	Test Requirements	1918

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1918

A.8.1.7.1	Test Purpose and Environment	1918

A.8.1.7.2	Test Requirements	1920

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1921

A.8.1.8.1	Test Purpose and Environment	1921

A.8.1.8.2	Test Requirements	1923

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	1924

A.8.1.9.1	Test Purpose and Environment	1924

A.8.1.9.2	Test Requirements	1924

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	1924

A.8.1.10.1	Test Purpose and Environment	1924

A.8.1.10.2	Test Requirements	1925

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1925

A.8.1.11.1	Test Purpose and Environment	1925

A.8.1.11.2	Test Requirements	1927

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1928

A.8.1.12.1	Test Purpose and Environment	1928

A.8.1.12.2	Test Requirements	1929

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1930

A.8.1.13.1	Test Purpose and Environment	1930

A.8.1.13.2	Test Requirements	1932

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1932

A.8.1.14.1	Test Purpose and Environment	1932

A.8.1.14.2	Test Requirements	1934

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1935

A.8.1.15.1	Test Purpose and Environment	1935

A.8.1.15.2	Test Requirements	1936

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1937

A.8.1.16.1	Test Purpose and Environment	1937

A.8.1.16.2	Test Requirements	1939

A.8.1.17	Void	1939

A.8.1.18	Void	1939

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1939

A.8.1.19.1	Test Purpose and Environment	1939

A.8.1.19.2	Test Requirements	1941

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1942

A.8.1.20.1	Test Purpose and Environment	1942

A.8.1.20.2	Test Requirements	1944

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1944

A.8.1.21.1	Test Purpose and Environment	1944

A.8.1.21.2	Test Requirements	1946

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1947

A.8.1.22.1	Test Purpose and Environment	1947

A.8.1.22.2	Test Requirements	1949

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1949

A.8.1.23.1	Test Purpose and Environment	1949

A.8.1.23.2	Test Requirements	1950

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1951

A.8.1.24.1	Test Purpose and Environment	1951

A.8.1.24.2	Test Requirements	1953

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1953

A.8.1.25.1	Test Purpose and Environment	1953

A.8.1.25.2	Test Requirements	1955

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1955

A.8.1.26.1	Test Purpose and Environment	1955

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1958

A.8.1.27.1	Test Purpose and Environment	1958

A.8.1.27.2	Test Requirements	1960

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1960

A.8.1.28.1	Test Purpose and Environment	1960

A.8.1.28.2	Test Requirements	1962

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1962

A.8.1.29.1	Test Purpose and Environment	1962

A.8.1.29.2	Test Requirements	1964

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1965

A.8.1.30.1	Test Purpose and Environment	1965

A.8.1.30.2	Test Requirements	1967

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1967

A.8.1.31.1	Test Purpose and Environment	1967

A.8.1.31.2	Test Requirements	1969

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1970

A.8.1.32.1	Test Purpose and Environment	1970

A.8.1.32.2	Test Requirements	1971

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1972

A.8.1.33.1	Test Purpose and Environment	1972

A.8.1.33.2	Test Requirements	1973

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1974

A.8.1.34.1	Test Purpose and Environment	1974

A.8.1.34.2	Test Requirements	1975

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1976

A.8.1.35.1	Test Purpose and Environment	1976

A.8.1.35.2	Test Requirements	1977

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1978

A.8.1.36.1	Test Purpose and Environment	1978

A.8.1.36.2	Test Requirements	1979

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1980

A.8.1.37.1	Test Purpose and Environment	1980

A.8.1.37.2	Test Requirements	1982

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1982

A.8.1.38.1	Test Purpose and Environment	1982

A.8.1.38.2	Test Requirements	1983

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1984

A.8.1.39.1	Test Purpose and Environment	1984

A.8.1.39.2	Test Requirements	1986

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1986

A.8.1.40.1	Test Purpose and Environment	1986

A.8.1.40.2	Test Requirements	1988

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1988

A.8.1.41.1	Test Purpose and Environment	1988

A.8.1.41.2	Test Requirement	1990

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1990

A.8.1.42.1	Test Purpose and Environment	1990

A.8.1.42.2	Test Requirement	1991

A.8.1.43	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	1991

A.8.1.43.1	Test Purpose and Environment	1991

A.8.1.43.2	Test Requirements	1994

A.8.1.44	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	1994

A.8.1.44.1	Test Purpose and Environment	1994

A.8.1.44.2	Test Requirements	1997

A.8.1.45	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	1998

A.8.1.45.1	Test Purpose and Environment	1998

A.8.1.45.2	Test Requirements	2000

A.8.1.46	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2001

A.8.1.46.1	Test Purpose and Environment	2001

A.8.1.46.2	Test Requirements	2002

A.8.1.47	TDD Intra-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2003

A.8.1.47.1	Test Purpose and Environment	2003

A.8.1.47.2	Test Requirements	2005

A.8.1.48	TDD Intra-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2006

A.8.1.48.1	Test Purpose and Environment	2006

A.8.1.48.2	Test Requirements	2008

A.8.1.49	TDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2009

A.8.1.49.1	Test Purpose and Environment	2009

A.8.1.49.2	Test Requirements	2010

A.8.150	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2011

A.8.1.50.1	Test Purpose and Environment	2011

A.8.1.50.2	Test Requirements	2013

A.8.1.51	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2014

A.8.1.51.1	Test Purpose and Environment	2014

A.8.1.51.2	Test Requirements	2016

A.8.1.52	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2017

A.8.1.52.1	Test Purpose and Environment	2017

A.8.1.52.2	Test Requirements	2018

A.8.1.53	TDD Inter-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2019

A.8.1.53.1	Test Purpose and Environment	2019

A.8.1.53.2	Test Requirements	2021

A.8.1.54	TDD Inter-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2022

A.8.1.54.1	Test Purpose and Environment	2022

A.8.1.54.2	Test Requirements	2024

A.8.1.55	TDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2025

A.8.1.55.1	Test Purpose and Environment	2025

A.8.1.55.2	Test Requirements	2026

A.8.2	E-UTRAN TDD Intra-frequency Measurements	2027

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	2027

A.8.2.1.1	Test Purpose and Environment	2027

A.8.2.1.2	Test Requirements	2028

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	2028

A.8.2.2.1	Test Purpose and Environment	2028

A.8.2.2.2	Test Requirements	2031

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2031

A.8.2.3.1	Test Purpose and Environment	2031

A.8.2.3.2	Test Requirements	2033

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2034

A.8.2.4.1	Test Purpose and Environment	2034

A.8.2.4.2	Test Requirements	2036

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2036

A.8.2.5.1	Test Purpose and Environment	2036

A.8.2.5.2	Test Requirements	2038

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2039

A.8.2.6.1	Test Purpose and Environment	2039

A.8.2.6.2	Test Requirements	2042

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2043

A.8.2.7.1	Test Purpose and Environment	2043

A.8.2.7.2	Test Requirements	2044

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2045

A.8.2.8.1	Test Purpose and Environment	2045

A.8.2.8.2	Test Requirements	2047

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2047

A.8.2.9.1	Test Purpose and Environment	2047

A.8.2.9.2	Test Requirements	2049

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2050

A.8.2.10.1	Test Purpose and Environment	2050

A.8.2.10.2	Test Requirements	2052

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	2052

A.8.2.11.1	Test Purpose and Environment	2052

A.8.2.11.2	Test Requirements	2054

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	2054

A.8.2.12.1	Test Purpose and Environment	2054

A.8.2.12.2	Test Requirements	2056

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	2057

A.8.2.13.1	Test Purpose and Environment	2057

A.8.2.13.2	Test Requirements	2059

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2059

A.8.2.14.1	Test Purpose and Environment	2059

A.8.2.14.2	Test Requirement	2061

A.8.2.15	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	2061

A.8.2.15.1	Test Purpose and Environment	2061

A.8.2.15.2	Test Requirements	2063

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	2063

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2063

A.8.3.1.1	Test Purpose and Environment	2063

A.8.3.1.2	Test Requirements	2065

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2065

A.8.3.2.1	Test Purpose and Environment	2065

A.8.3.2.2	Test Requirements	2068

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	2068

A.8.3.3.1	Test Purpose and Environment	2068

A.8.3.3.2	Test Requirements	2071

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2071

A.8.3.4.1	Test Purpose and Environment	2071

A.8.3.4.2	Test Requirements	2073

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2073

A.8.3.5.2	Test Requirements	2075

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	2075

A.8.3.6.1	Test Purpose and Environment	2075

A.8.3.6.2	Test Requirements	2077

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2078

A.8.3.7.1	Test Purpose and Environment	2078

A.8.3.7.2	Test Requirements	2080

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2081

A.8.3.8.1	Test Purpose and Environment	2081

A.8.3.8.2	Test Requirements	2084

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2084

A.8.3.9.1	Test Purpose and Environment	2084

A.8.3.9.2	Test Requirements	2088

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2089

A.8.3.10.1	Test Purpose and Environment	2089

A.8.3.10.2	Test Requirements	2090

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	2090

A.8.3.11.1	Test Purpose and Environment	2090

A.8.3.11.2	Test Requirement	2092

A.8.3.12	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2093

A.8.3.12.1	Test Purpose and Environment	2093

A.8.3.12.2	Test Requirement	2094

A.8.3.13	E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2095

A.8.3.13.1	Test Purpose and Environment	2095

A.8.3.13.2	Test Requirement	2096

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2097

A.8.3.14.1	Test Purpose and Environment	2097

A.8.3.14.2	Test Requirement	2098

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2099

A.8.3.15.1	Test Purpose and Environment	2099

A.8.3.15.2	Test Requirement	2100

A.8.3.16	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	2101

A.8.3.16.1	Test Purpose and Environment	2101

A.8.3.16.2	Test Requirement	2103

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2103

A.8.3.17.1	Test Purpose and Environment	2103

A.8.3.17.2	Test Requirement	2106

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2106

A.8.3.18.1	Test Purpose and Environment	2106

A.8.3.18.2	Test Requirement	2109

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2109

A.8.3.19.1	Test Purpose and Environment	2109

A.8.3.19.2	Test Requirement	2112

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	2112

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2112

A.8.4.1.1	Test Purpose and Environment	2112

A.8.4.1.2	Test Requirements	2114

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	2114

A.8.4.2.1	Test Purpose and Environment	2114

A.8.4.2.2	Test Requirements	2117

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	2117

A.8.4.3.1	Test Purpose and Environment	2117

A.8.4.3.2	Test Requirements	2119

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2120

A.8.4.4.1	Test Purpose and Environment	2120

A.8.4.4.2	Test Requirements	2122

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2123

A.8.4.5.1	Test Purpose and Environment	2123

A.8.4.5.2	Test Requirements	2125

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	2125

A.8.4.6.1	Test Purpose and Environment	2125

A.8.4.6.2	Test Requirements	2126

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2126

A.8.4.7.1	Test Purpose and Environment	2126

A.8.4.7.2	Test Requirements	2130

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2130

A.8.4.8.1	Test Purpose and Environment	2130

A.8.4.8.2	Test Requirements	2133

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2133

A.8.4.9.1	Test Purpose and Environment	2133

A.8.4.9.2	Test Requirements	2138

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2138

A.8.4.10.1	Test Purpose and Environment	2138

A.8.4.10.2	Test Requirements	2140

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	2140

A.8.4.11.1	Test Purpose and Environment	2140

A.8.4.11.2	Test Requirement	2142

A.8.4.12	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2142

A.8.4.12.1	Test Purpose and Environment	2142

A.8.4.12.2	Test Requirement	2144

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2145

A.8.4.13.1	Test Purpose and Environment	2145

A.8.4.13.2	Test Requirement	2146

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2147

A.8.4.14.1	Test Purpose and Environment	2147

A.8.4.14.2	Test Requirement	2149

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2149

A.8.4.15.1	Test Purpose and Environment	2149

A.8.4.15.2	Test Requirement	2152

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	2152

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2152

A.8.5.1.1	Test Purpose and Environment	2152

A.8.5.1.2	Test Requirements	2154

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	2154

A.8.5.2.1	Test Purpose and Environment	2154

A.8.5.2.2	Test Requirements	2156

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	2156

A.8.5.3.1	Test Purpose and Environment	2156

A.8.5.3.2	Test Requirements	2159

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	2159

A.8.5.4.1	Test Purpose and Environment	2159

A.8.5.4.2	Test Requirements	2161

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2161

A.8.5.5.1	Test Purpose and Environment	2161

A.8.5.5.2	Test Requirements	2164

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	2164

A.8.5.6.1	Test Purpose and Environment	2164

A.8.5.6.2	Test Requirements	2165

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	2166

A.8.5.7.1	Test Purpose and Environment	2166

A.8.5.7.2	Test Requirements	2166

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2166

A.8.5.8.1	Test Purpose and Environment	2166

A.8.5.8.2	Test Requirements	2169

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	2169

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	2169

A.8.6.1.1	Test Purpose and Environment	2169

A.8.6.1.2	Test Requirements	2171

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2171

A.8.6.2.1	Test Purpose and Environment	2171

A.8.6.2.2	Test Requirements	2174

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2174

A.8.6.3.1	Test Purpose and Environment	2174

A.8.6.3.2	Test Requirements	2177

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	2177

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2177

A.8.7.1.1	Test Purpose and Environment	2177

A.8.7.1.1.1	Void	2177

A.8.7.1.1.2	1.28 Mcps TDD option	2177

A.8.7.1.1.3	Void	2179

A.8.7.1.2	Test Requirements	2179

A.8.7.1.2.1	Void	2179

A.8.7.1.2.2	1.28 Mcps TDD option	2179

A.8.7.1.2.3	Void	2179

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	2179

A.8.7.2.1	Test Purpose and Environment	2179

A.8.7.2.2	Test Requirements	2182

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	2183

A.8.7.3.1	Test Purpose and Environment	2183

A.8.7.3.2	Test Parameters	2183

A.8.7.3.3	Test Requirements	2184

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2185

A.8.7.4.1	Test Purpose and Environment	2185

A.8.7.4.2	Test Requirements	2187

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2187

A.8.7.5.1	Test Purpose and Environment	2187

A.8.7.5.2	Test Requirements	2189

A.8.7A	E-UTRAN FDD – UTRAN TDD Measurements	2189

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2189

A.8.7A.1.1	Test Purpose and Environment	2189

A.8.7A.1.2	Test Requirements	2192

A.8.8	E-UTRAN FDD – GSM Measurements	2192

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	2192

A.8.8.1.1	Test Purpose and Environment	2192

A.8.8.1.2	Test Requirements	2194

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	2194

A.8.8.2.1	Test Purpose and Environment	2194

A.8.8.2.2	Test Requirements	2196

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	2197

A.8.8.3.1	Test Purpose and Environment	2197

A.8.8.3.2	Test Requirements	2198

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	2199

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	2199

A.8.9.1.1	Test Purpose and Environment	2199

A.8.9.1.2	Test Requirements	2200

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2201

A.8.9.2.1	Test Purpose and Environment	2201

A.8.9.2.2	Test Requirements	2203

A.8.10	E-UTRAN TDD – GSM Measurements	2203

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	2203

A.8.10.1.1	Test Purpose and Environment	2203

A.8.10.1.2	Test Requirements	2204

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	2205

A.8.10.2.1	Test Purpose and Environment	2205

A.8.10.2.2	Test Requirements	2207

A.8.11	Monitoring of Multiple Layers	2207

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	2207

A.8.11.1.1	Test Purpose and Environment	2207

A.8.11.1.2	Test Requirements	2209

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	2210

A.8.11.2.1	Test Purpose and Environment	2210

A.8.11.2.2	Test Requirements	2211

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	2212

A.8.11.3.1	Test Purpose and Environment	2212

A.8.11.3.2	Test Requirements	2214

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	2214

A.8.11.4.1	Test Purpose and Environment	2214

A.8.11.4.2	Test Requirements	2217

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2217

A.8.11.5.1	Test Purpose and Environment	2217

A.8.11.5.2	Test Requirements	2219

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2220

A.8.11.6.1	Test Purpose and Environment	2220

A.8.11.6.2	Test Requirements	2222

A.8.12	RSTD Intra-frequency Measurements	2223

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	2223

A.8.12.1.1	Test Purpose and Environment	2223

A.8.12.1.2	Test Requirements	2227

A.8.12.1.2A	Test Requirements for UE Category 1bis	2227

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	2227

A.8.12.2.1	Test Purpose and Environment	2227

A.8.12.2.2	Test Requirements	2232

A.8.12.2.2A	Test Requirements for UE Category 1bis	2232

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	2232

A.8.12.3.1	Test Purpose and Environment	2232

A.8.12.3.2	Test Requirements	2238

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	2238

A.8.12.4.1	Test Purpose and Environment	2238

A.8.12.4.2	Test Requirements	2243

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	2243

A.8.12.5.1	Test Purpose and Environment	2243

A.8.12.5.2	Test Requirements	2248

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	2248

A.8.12.6.1	Test Purpose and Environment	2248

A.8.12.6.2	Test Requirements	2253

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	2253

A.8.12.7.1	Test Purpose and Environment	2253

A.8.12.7.2	Test Requirements	2258

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	2258

A.8.12.8.1	Test Purpose and Environment	2258

A.8.12.8.2	Test Requirements	2263

A.8.12.9	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2263

A.8.12.9.1	Test Purpose and Environment	2263

A.8.12.9.2	Test Requirements	2268

A.8.12.10	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2268

A.8.12.10.1	Test Purpose and Environment	2268

A.8.12.10.2	Test Requirements	2273

A.8.12.11	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2273

A.8.12.11.1	Test Purpose and Environment	2273

A.8.12.11.2	Test Requirements	2278

A.8.12.12	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2278

A.8.12.12.1	Test Purpose and Environment	2278

A.8.12.12.2	Test Requirements	2283

A.8.12.13	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2283

A.8.12.13.1	Test Purpose and Environment	2283

A.8.12.13.2	Test Requirements	2288

A.8.12.14	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2288

A.8.12.14.1	Test Purpose and Environment	2288

A.8.12.14.2	Test Requirements	2293

A.8.13	RSTD Inter-frequency Measurements	2293

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2293

A.8.13.1.1	Test Purpose and Environment	2293

A.8.13.1.2	Test Requirements	2298

A.8.13.1.2A	Test Requirements for UE Category 1bis	2298

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2298

A.8.13.2.1	Test Purpose and Environment	2298

A.8.13.2.2	Test Requirements	2304

A.8.13.2.2A	Test Requirements for UE Category 1bis	2304

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	2304

A.8.13.3.1	Test Purpose and Environment	2304

A.8.13.3.2	Test Requirements	2311

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	2311

A.8.13.4.1	Test Purpose and Environment	2311

A.8.13.4.2	Test Requirements	2317

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	2317

A.8.13.5.1	Test Purpose and Environment	2317

A.8.13.5.2	Test Requirements	2323

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	2323

A.8.13.6.1	Test Purpose and Environment	2323

A.8.13.6.2	Test Requirements	2329

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	2329

A.8.13.7.1	Test Purpose and Environment	2329

A.8.13.7.2	Test Requirements	2335

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	2335

A.8.13.8.1	Test Purpose and Environment	2335

A.8.13.8.2	Test Requirements	2341

A.8.13.9	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2341

A.8.13.9.1	Test Purpose and Environment	2341

A.8.13.9.2	Test Requirements	2347

A.8.13.10	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2347

A.8.13.10.1	Test Purpose and Environment	2347

A.8.13.10.2	Test Requirements	2353

A.8.13.11	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2353

A.8.13.11.1	Test Purpose and Environment	2353

A.8.13.11.2	Test Requirements	2359

A.8.13.12	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2359

A.8.13.12.1	Test Purpose and Environment	2359

A.8.13.12.2	Test Requirements	2365

A.8.13.13	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2365

A.8.13.13.1	Test Purpose and Environment	2365

A.8.13.13.2	Test Requirements	2371

A.8.13.14	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2371

A.8.13.14.1	Test Purpose and Environment	2371

A.8.13.14.2	Test Requirements	2377

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	2377

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2377

A.8.14.1.1	Test Purpose and Environment	2377

A.8.14.1.2	Test Requirements	2379

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2379

A.8.14.2.1	Test Purpose and Environment	2379

A.8.14.2.2	Test Requirements	2382

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2382

A.8.14.3.1	Test Purpose and Environment	2382

A.8.14.3.2	Test Requirements	2384

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	2385

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2385

A.8.15.1.1	Test Purpose and Environment	2385

A.8.15.1.2	Test Requirements	2386

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2386

A.8.15.2.1	Test Purpose and Environment	2386

A.8.15.2.2	Test Requirements	2389

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2389

A.8.15.3.1	Test Purpose and Environment	2389

A.8.15.3.2	Test Requirements	2391

A.8.16	E-UTRAN Carrier Aggregation Measurements	2392

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	2392

A.8.16.1.1	Test Purpose and Environment	2392

A.8.16.1.2	Test Requirements	2394

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	2394

A.8.16.2.1	Test Purpose and Environment	2394

A.8.16.2.2	Test Requirements	2396

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2396

A.8.16.3.1	Test Purpose and Environment	2396

A.8.16.3.2	Test Requirements	2398

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	2399

A.8.16.3A.1	Test Purpose and Environment	2399

A.8.16.3A.2	Test Requirements	2400

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	2401

A.8.16.4.1	Test Purpose and Environment	2401

A.8.16.4.2	Test Requirements	2402

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2403

A.8.16.4A.1	Test Purpose and Environment	2403

A.8.16.4A.2	Test Requirements	2405

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2406

A.8.16.5.1	Test Purpose and Environment	2406

A.8.16.5.2	Test Requirements	2406

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2406

A.8.16.6.1	Test Purpose and Environment	2406

A.8.16.6.2	Test Requirements	2407

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2407

A.8.16.7.1	 Test Purpose and Environment	2407

A.8.16.7.2	 Test Requirements	2408

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2408

A.8.16.8.1	 Test Purpose and Environment	2408

A.8.16.8.2	 Test Requirements	2409

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2409

A.8.16.9.1	Test Purpose and Environment	2409

A.8.16.9.2	Test Requirements	2410

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2410

A.8.16.10.1	Test Purpose and Environment	2410

A.8.16.10.2	Test Requirements	2411

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2411

A.8.16.11.1	 Test Purpose and Environment	2411

A.8.16.11.2	 Test Requirements	2412

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2412

A.8.16.12.1	 Test Purpose and Environment	2412

A.8.16.12.2	 Test Requirements	2412

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	2412

A.8.16.13.1	Test Purpose and Environment	2412

A.8.16.13.2	Test Requirements	2413

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	2413

A.8.16.14.1	Test Purpose and Environment	2413

A.8.16.14.2	Test Requirements	2414

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	2414

A.8.16.15.1	 Test Purpose and Environment	2414

A.8.16.7.2	 Test Requirements	2414

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	2414

A.8.16.16.1	 Test Purpose and Environment	2414

A.8.16.16.2	 Test Requirements	2415

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	2415

A.8.16.17.1	Test Purpose and Environment	2415

A.8.16.17.2	Test Requirements	2417

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	2418

A.8.16.17A.1	Test Purpose and Environment	2418

A.8.16.17A.2	Test Requirements	2418

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2418

A.8.16.17B.1	Test Purpose and Environment	2418

A.8.16.17B.2	Test Requirements	2419

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2419

A.8.16.17C.1	Test Purpose and Environment	2419

A.8.16.17C.2	Test Requirements	2419

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	2419

A.8.16.18.1	Test Purpose and Environment	2419

A.8.16.18.2	Test Requirements	2421

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	2422

A.8.16.18A.1	Test Purpose and Environment	2422

A.8.16.18A.2	Test Requirements	2422

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2422

A.8.16.18B.1	Test Purpose and Environment	2422

A.8.16.18B.2	Test Requirements	2423

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2423

A.8.16.18C.1	Test Purpose and Environment	2423

A.8.16.18C.2	Test Requirements	2423

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	2423

A.8.16.18D.1	Test Purpose and Environment	2423

A.8.16.18D.2	Test Requirements	2424

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	2424

A.8.16.19.1	Test Purpose and Environment	2424

A.8.16.19.2	Test Requirements	2426

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2426

A.8.16.19A.1	Test Purpose and Environment	2426

A.8.16.19A.2	Test Requirements	2427

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2427

A.8.16.19B.1	Test Purpose and Environment	2427

A.8.16.19B.2	Test Requirements	2427

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2427

A.8.16.19C.1	Test Purpose and Environment	2427

A.8.16.19C.2	Test Requirements	2428

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	2428

A.8.16.20.1	Test Purpose and Environment	2428

A.8.16.20.2	Test Requirements	2430

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2431

A.8.16.20A.1	Test Purpose and Environment	2431

A.8.16.20A.2	Test Requirements	2431

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2431

A.8.16.20B.1	Test Purpose and Environment	2431

A.8.16.20B.2	Test Requirements	2432

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2432

A.8.16.20C.1	Test Purpose and Environment	2432

A.8.16.20C.2	Test Requirements	2432

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	2432

A.8.16.20D.1	Test Purpose and Environment	2432

A.8.16.20D.2	Test Requirements	2433

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	2433

A.8.16.21.1	Test Purpose and Environment	2433

A.8.16.21.2	Test Requirements	2435

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	2435

A.8.16.22.1	Test Purpose and Environment	2435

A.8.16.22.2	Test Requirements	2437

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	2437

A.8.16.23.1	Test Purpose and Environment	2437

A.8.16.23.2	Test Requirements	2440

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	2440

A.8.16.24.1	Test Purpose and Environment	2440

A.8.16.24.2	Test Requirements	2443

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	2443

A.8.16.25.1	Test Purpose and Environment	2443

A.8.16.25.2	Test Requirements	2446

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	2446

A.8.16.26.1	Test Purpose and Environment	2446

A.8.16.26.2	Test Requirements	2449

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2449

A.8.16.27.1	Test Purpose and Environment	2449

A.8.16.27.2	Test Requirements	2454

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2454

A.8.16.28.1	Test Purpose and Environment	2454

A.8.16.28.2	Test Requirements	2459

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2459

A.8.16.29.1	Test Purpose and Environment	2459

A.8.16.29.2	Test Requirements	2464

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2464

A.8.16.30.1	Test Purpose and Environment	2464

A.8.16.30.2	Test Requirements	2469

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	2469

A.8.16.31.1	Test Purpose and Environment	2469

A.8.16.31.2	Test Requirements	2474

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	2474

A.8.16.32.1	Test Purpose and Environment	2474

A.8.16.32.2	Test Requirements	2479

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2479

A.8.16.33.1	Test Purpose and Environment	2479

A.8.16.33.2	Test Requirements	2484

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2484

A.8.16.34.1	Test Purpose and Environment	2484

A.8.16.34.2	Test Requirements	2489

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	2489

A.8.16.35.1	Test Purpose and Environment	2489

A.8.16.35.2	Test Requirements	2491

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	2492

A.8.16.36.1	Test Purpose and Environment	2492

A.8.16.36.2	Test Requirements	2494

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	2495

A.8.16.37.1	Test Purpose and Environment	2495

A.8.16.37.2	Test Requirements	2497

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	2498

A.8.16.38.1	Test Purpose and Environment	2498

A.8.16.38.2	Test Requirements	2500

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	2501

A.8.16.39.1	Test Purpose and Environment	2501

A.8.16.39.2	Test Requirements	2504

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	2504

A.8.16.40.1	Test Purpose and Environment	2504

A.8.16.40.2	Test Requirements	2507

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2507

A.8.16.41.1	Test Purpose and Environment	2507

A.8.16.41.2	Test Requirements	2509

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2510

A.8.16.42.1	Test Purpose and Environment	2510

A.8.16.42.2	Test Requirements	2512

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	2513

A.8.16.43.1	Test Purpose and Environment	2513

A.8.16.43.2	Test Requirements	2516

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	2516

A.8.16.44.1	Test Purpose and Environment	2516

A.8.16.44.2	Test Requirements	2519

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	2519

A.8.16.45.1	Test Purpose and Environment	2519

A.8.16.45.2	Test Requirements	2522

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	2522

A.8.16.46.1	Test Purpose and Environment	2522

A.8.16.46.2	Test Requirements	2524

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2525

A.8.16.47.1	Test Purpose and Environment	2525

A.8.16.47.2	Test Requirements	2527

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2528

A.8.16.48.1	Test Purpose and Environment	2528

A.8.16.48.2	Test Requirements	2530

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2531

A.8.16.49.1	Test Purpose and Environment	2531

A.8.16.49.2	Test Requirements	2534

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2534

A.8.16.50.1	Test Purpose and Environment	2534

A.8.16.50.2	Test Requirements	2536

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2537

A.8.16.51.1	Test Purpose and Environment	2537

A.8.16.51.2	Test Requirements	2542

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2542

A.8.16.52.1	Test Purpose and Environment	2542

A.8.16.52.2	Test Requirements	2547

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2547

A.8.16.53.1	Test Purpose and Environment	2547

A.8.16.53.2	Test Requirements	2551

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2551

A.8.16.54.1	Test Purpose and Environment	2551

A.8.16.54.2	Test Requirements	2555

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2555

A.8.16.55.1	Test Purpose and Environment	2555

A.8.16.55.2	Test Requirements	2561

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2561

A.8.16.56.1	Test Purpose and Environment	2561

A.8.16.56.2	Test Requirements	2567

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2567

A.8.16.57.1	Test Purpose and Environment	2567

A.8.16.57.2	Test Requirements	2569

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2570

A.8.16.58.1	Test Purpose and Environment	2570

A.8.16.58.2	Test Requirements	2572

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2573

A.8.16.59.1	Test Purpose and Environment	2573

A.8.16.59.2	Test Requirements	2577

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2578

A.8.16.60.1	Test Purpose and Environment	2578

A.8.16.60.2	Test Requirements	2581

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2582

A.8.16.61.1	Test Purpose and Environment	2582

A.8.16.61.2	Test Requirements	2585

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2585

A.8.16.62.1	Test Purpose and Environment	2585

A.8.16.62.2	Test Requirements	2587

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2588

A.8.16.63.1	Test Purpose and Environment	2588

A.8.16.63.2	Test Requirements	2591

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2592

A.8.16.64.1	Test Purpose and Environment	2592

A.8.16.64.2	Test Requirements	2596

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2596

A.8.16.65.1	Test Purpose and Environment	2596

A.8.16.65.2	Test Requirements	2601

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2601

A.8.16.66.1	Test Purpose and Environment	2601

A.8.16.66.2	Test Requirements	2606

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2606

A.8.16.67.1	Test Purpose and Environment	2606

A.8.16.67.2	Test Requirements	2611

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2611

A.8.16.68.1	Test Purpose and Environment	2611

A.8.16.68.2	Test Requirements	2616

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2616

A.8.16.69.1	Test Purpose and Environment	2616

A.8.16.69.2	Test Requirements	2620

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2620

A.8.16.70.1	Test Purpose and Environment	2620

A.8.16.70.2	Test Requirements	2624

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2624

A.8.16.71.1	Test Purpose and Environment	2624

A.8.16.71.2	Test Requirements	2629

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2629

A.8.16.72.1	Test Purpose and Environment	2629

A.8.16.72.2	Test Requirements	2634

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2634

A.8.16.73.1	Test Purpose and Environment	2634

A.8.16.73.2	Test Requirements	2640

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2640

A.8.16.74.1	Test Purpose and Environment	2640

A.8.16.74.2	Test Requirements	2646

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2646

A.8.16.75.1	Test Purpose and Environment	2646

A.8.16.75.2	Test Requirements	2652

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2652

A.8.16.76.1	Test Purpose and Environment	2652

A.8.16.76.2	Test Requirements	2658

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2658

A.8.16.77.1	Test Purpose and Environment	2658

A.8.16.77.2	Test Requirements	2662

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2662

A.8.16.78.1	Test Purpose and Environment	2662

A.8.16.78.2	Test Requirements	2664

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2665

A.8.16.79.1	Test Purpose and Environment	2665

A.8.16.79.2	Test Requirements	2672

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2672

A.8.16.80.1	Test Purpose and Environment	2672

A.8.16.80.2	Test Requirements	2679

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2679

A.8.16.81.1	Test Purpose and Environment	2679

A.8.16.81.2	Test Requirements	2685

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2685

A.8.16.82.1	Test Purpose and Environment	2685

A.8.16.82.2	Test Requirements	2691

A.8.16.83	3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2691

A.8.16.83.1	Test Purpose and Environment	2691

A.8.16.83.2	Test Requirements	2696

A.8.16.84	3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2696

A.8.16.84.1	Test Purpose and Environment	2696

A.8.16.84.2	Test Requirements	2702

A.8.16.85	3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2702

A.8.16.85.1	Test Purpose and Environment	2702

A.8.16.85.2	Test Requirements	2706

A.8.16.86	3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2706

A.8.16.86.1	Test Purpose and Environment	2706

A.8.16.86.2	Test Requirements	2709

A.8.16.87	4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2709

A.8.16.87.1	Test Purpose and Environment	2709

A.8.16.87.2	Test Requirements	2715

A.8.16.88	4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2715

A.8.16.88.1	Test Purpose and Environment	2715

A.8.16.88.2	Test Requirements	2723

A.8.16.89	4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2723

A.8.16.89.1	Test Purpose and Environment	2723

A.8.16.89.2	Test Requirements	2730

A.8.16.90	4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2730

A.8.16.90.1	Test Purpose and Environment	2730

A.8.16.90.2	Test Requirements	2737

A.8.16.91	5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2737

A.8.16.91.1	Test Purpose and Environment	2737

A.8.16.91.2	Test Requirements	2743

A.8.16.92	5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2743

A.8.16.92.1	Test Purpose and Environment	2743

A.8.16.92.2	Test Requirements	2751

A.8.16.93	5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2751

A.8.16.93.1	Test Purpose and Environment	2751

A.8.16.93.2	Test Requirements	2758

A.8.16.94	5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2758

A.8.16.94.1	Test Purpose and Environment	2758

A.8.16.94.2	Test Requirements	2765

A.8.16.95	6 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2765

A.8.16.95.1	Test Purpose and Environment	2765

A.8.16.95.2	Test Requirements	2773

A.8.16.96	6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2773

A.8.16.96.1	Test Purpose and Environment	2773

A.8.16.96.2	Test Requirements	2782

A.8.16.97	6 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2782

A.8.16.97.1	Test Purpose and Environment	2782

A.8.16.97.2	Test Requirements	2789

A.8.16.98	6 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2789

A.8.16.98.1	Test Purpose and Environment	2789

A.8.16.98.2	Test Requirements	2797

A.8.16.99	7 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2797

A.8.16.99.1	Test Purpose and Environment	2797

A.8.16.99.2	Test Requirements	2807

A.8.16.100	7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2807

A.8.16.100.1	Test Purpose and Environment	2807

A.8.16.100.2	Test Requirements	2817

A.8.16.101	7 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2817

A.8.16.101.1	Test Purpose and Environment	2817

A.8.16.101.2	Test Requirements	2827

A.8.16.102	7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2827

A.8.16.102.1	Test Purpose and Environment	2827

A.8.16.102.2	Test Requirements	2838

A.8.16.103	Hibernation and Activation of Known SCell in Non-DRX with generic duplex modes	2838

A.8.16.103.1	Test Purpose and Environment	2838

A.8.16.103.2	Test Requirements	2843

A.8.16.104	Hibernation and Activation of Unknown SCell in Non-DRX with generic duplex modes	2844

A.8.16.104.1	Test Purpose and Environment	2844

A.8.16.104.2	Test Requirements	2848

A.8.16.105	Idle Mode measurements of inter-frequency CA candidate cells for early reporting	2849

A.8.16.105.1	Test Purpose and Environment	2849

A.8.16.105.2	Test Requirements	2853

A.8.16.106	Direct Activation of Known SCell in Non-DRX with generic duplex modes	2854

A.8.16.106.1	Test Purpose and Environment	2854

A.8.16.106.2	Test Requirements	2858

A.8.16.107	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	2858

A.8.16.107.1	Test Purpose and Environment	2858

A.8.16.107.2	Test Requirements	2860

A.8.16.108	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	2861

A.8.16.108.1	Test Purpose and Environment	2861

A.8.16.108.2	Test Requirements	2863

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	2864

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	2864

A.8.17.1.1	Test Purpose and Environment	2864

A.8.17.1.2	Test Requirements	2870

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	2870

A.8.17.2.1	Test Purpose and Environment	2870

A.8.17.2.2	Test Requirements	2877

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	2877

A.8.17.3.1	Test Purpose and Environment	2877

A.8.17.3.2	Test Requirements	2878

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	2878

A.8.17.4.1	Test Purpose and Environment	2878

A.8.17.4.2	Test Requirements	2879

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2879

A.8.17.5.1	Test Purpose and Environment	2879

A.8.17.5.2	Test Requirements	2880

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2880

A.8.17.6.1	Test Purpose and Environment	2880

A.8.17.6.2	Test Requirements	2881

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	2882

A.8.17.7.1	Test Purpose and Environment	2882

A.8.17.7.2	Test Requirements	2882

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	2883

A.8.17.8.1	Test Purpose and Environment	2883

A.8.17.8.2	Test Requirements	2883

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	2884

A.8.17.9.1	Test Purpose and Environment	2884

A.8.17.9.2	Test Requirements	2885

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	2885

A.8.17.10.1	Test Purpose and Environment	2885

A.8.17.10.2	Test Requirements	2892

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	2893

A.8.17.11.1	Test Purpose and Environment	2893

A.8.17.11.2	Test Requirements	2899

A.8.18	E-UTRAN TDD – HRPD Measurements	2900

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	2900

A.8.18.1.1	Test Purpose and Environment	2900

A.8.18.1.2	Test Requirements	2902

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	2902

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	2902

A.8.19.1.1	Test Purpose and Environment	2902

A.8.19.1.2	Test Requirements	2903

A.8.20	Inter-frequency/RAT Measurements in CA mode	2904

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2904

A.8.20.1.1	Test Purpose and Environment	2904

A.8.20.1.2	Test Requirements	2905

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2905

A.8.20.2.1	Test Purpose and Environment	2906

A.8.20.2.2	Test Requirements	2907

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	2908

A.8.20.2A.1	Test Purpose and Environment	2908

A.8.20.2A.2	Test Requirements	2908

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	2908

A.8.20.2B.1	Test Purpose and Environment	2908

A.8.20.2B.2	Test Requirements	2911

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2911

A.8.20.3.1	Test Purpose and Environment	2911

A.8.20.3.2	Test Requirements	2913

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2913

A.8.20.4.1	Test Purpose and Environment	2913

A.8.20.4.1.1	1.28 Mcps TDD option	2913

A.8.20.4.2	Test Requirements	2915

A.8.20.4.2.1	1.28 Mcps TDD option	2915

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2915

A.8.20.4A.1	Test Purpose and Environment	2915

A.8.20.4A.1.1	1.28 Mcps TDD option	2915

A.8.20.4A.2	Test Requirements	2915

A.8.20.4A.2.1	1.28 Mcps TDD option	2915

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2916

A.8.20.4B.1	Test Purpose and Environment	2916

A.8.20.4B.1.1	1.28 Mcps TDD option	2916

A.8.20.4B.2	Test Requirements	2918

A.8.20.4B.2.1	1.28 Mcps TDD option	2918

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	2918

A.8.21.1	Test Purpose and Environment	2918

A.8.21.2 Test Requirements	2922

A.8.22	E-UTRAN Discovery Signal Measurements	2922

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2922

A.8.22.1.1	Test Purpose and Environment	2922

A.8.22.1.2	Test Requirements	2925

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2925

A.8.22.2.1	Test Purpose and Environment	2925

A.8.22.2.2	Test Requirements	2928

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2928

A.8.22.3.1	Test Purpose and Environment	2928

A.8.22.3.2	Test Requirements	2931

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2931

A.8.22.4.1	Test Purpose and Environment	2931

A.8.22.4.2	Test Requirements	2934

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2934

A.8.22.5.1	Test Purpose and Environment	2934

A.8.22.5.2	Test Requirements	2938

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2938

A.8.22.6.1	Test Purpose and Environment	2938

A.8.22.6.2	Test Requirements	2942

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2942

A.8.22.7.1	Test Purpose and Environment	2942

A.8.22.7.2	Test Requirements	2946

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	2946

A.8.22.8.1	Test Purpose and Environment	2946

A.8.22.8.2	Test Requirements	2950

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2950

A.8.22.9.1	Test Purpose and Environment	2950

A.8.22.9.2	Test Requirements	2952

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2953

A.8.22.10.1	Test Purpose and Environment	2953

A.8.22.10.2	Test Requirements	2955

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2956

A.8.22.11.1	Test Purpose and Environment	2956

A.8.22.11.2	Test Requirements	2959

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2959

A.8.22.12.1	Test Purpose and Environment	2959

A.8.22.12.2	Test Requirements	2963

A.8.23	E-UTRAN Dual Connectivity Measurements	2963

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2963

A.8.23.1.1	Test Purpose and Environment	2963

A.8.23.1.2	Test Requirements	2966

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	2966

A.8.23.2.1	Test Purpose and Environment	2966

A.8.23.2.2	Test Requirements	2969

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2969

A.8.23.3.1	Test Purpose and Environment	2969

A.8.23.3.2	Test Requirements	2972

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2972

A.8.23.4.1	Test Purpose and Environment	2972

A.8.23.4.2	Test Requirements	2975

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	2975

A.8.23.5.1	Test Purpose and Environment	2975

A.8.23.5.2	Test Requirements	2978

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2978

A.8.23.6.1	Test Purpose and Environment	2978

A.8.23.6.2	Test Requirements	2981

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	2981

A.8.23.7.1	Test Purpose and Environment	2981

A.8.23.7.2	Test Requirements	2983

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	2984

A.8.23.8.1	Test Purpose and Environment	2984

A.8.23.8.2	Test Requirements	2986

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	2987

A.8.23.9.1	Test Purpose and Environment	2987

A.8.23.9.2	Test Requirements	2990

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2990

A.8.23.10.1	Test Purpose and Environment	2990

A.8.23.10.2	Test Requirements	2993

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2993

A.8.23.11.1	Test Purpose and Environment	2993

A.8.23.11.2	Test Requirements	2996

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2996

A.8.23.12.1	Test Purpose and Environment	2996

A.8.23.12.2	Test Requirements	2999

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2999

A.8.23.13.1	Test Purpose and Environment	2999

A.8.23.13.2	Test Requirements	3002

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	3002

A.8.23.14.1	Test Purpose and Environment	3002

A.8.23.14.2	Test Requirements	3005

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	3005

A.8.23.15.1	Test Purpose and Environment	3005

A.8.23.15.2	Test Requirements	3008

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	3008

A.8.23.16.1	Test Purpose and Environment	3008

A.8.23.16.2	Test Requirements	3009

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	3010

A.8.23.17.1	Test Purpose and Environment	3010

A.8.23.17.2	Test Requirements	3012

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3012

A.8.23.18.1	Test Purpose and Environment	3012

A.8.23.18.2	Test Requirements	3013

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	3014

A.8.23.19.1	Test Purpose and Environment	3014

A.8.23.19.2	Test Requirements	3015

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3016

A.8.23.20.1	Test Purpose and Environment	3016

A.8.23.20.2	Test Requirements	3017

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3018

A.8.23.21.1	Test Purpose and Environment	3018

A.8.23.21.2	Test Requirements	3019

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	3020

A.8.23.22.1	Test Purpose and Environment	3020

A.8.23.22.2	Test Requirements	3021

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3022

A.8.23.23.1	Test Purpose and Environment	3022

A.8.23.23.2	Test Requirements	3023

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	3024

A.8.23.24.1	Test Purpose and Environment	3024

A.8.23.24.2	Test Requirements	3026

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	3027

A.8.23.25.1	Test Purpose and Environment	3027

A.8.23.25.2	Test Requirements	3029

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	3030

A.8.23.26.1	Test Purpose and Environment	3030

A.8.23.26.2	Test Requirements	3032

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3033

A.8.23.27.1	Test Purpose and Environment	3033

A.8.23.27.2	Test Requirements	3036

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	3036

A.8.23.28.1	Test Purpose and Environment	3036

A.8.23.28.2	Test Requirements	3040

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3040

A.8.23.29.1	Test Purpose and Environment	3040

A.8.23.29.2	Test Requirements	3044

A.8.24	Proximity-based Services	3044

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3044

A.8.24.1.1	Test Purpose and Environment	3044

A.8.24.1.2	Test Requirements	3045

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3046

A.8.24.2.1	Test Purpose and Environment	3046

A.8.24.2.2	Test Requirements	3047

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	3047

A.8.24.3.1	Test Purpose and Environment	3048

A.8.24.3.2	Test Requirements	3049

A.8.25	E-UTRAN-WLAN Measurements	3050

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3050

A.8.25.1.1	Test Purpose and Environment	3050

A.8.25.1.2	Test Requirements	3052

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3052

A.8.25.2.1	Test Purpose and Environment	3052

A.8.25.2.2	Test Requirements	3054

A.8.26 Frame Structure 3 (FS3)	3054

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	3054

A.8.26.1.1	Test Purpose and Environment	3054

A.8.26.1.2	Test Requirements	3056

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	3057

A.8.26.2.1	Test Purpose and Environment	3057

A.8.26.2.2	Test Requirements	3059

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3060

A.8.26.3.1	Test Purpose and Environment	3060

A.8.26.3.2	Test Requirements	3063

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3063

A.8.26.3A.1	Test Purpose and Environment	3063

A.8.26.3A.2	Test Requirements	3066

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	3066

A.8.26.4.1	Test Purpose and Environment	3066

A.8.26.4.2	Test Requirements	3070

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3070

A.8.26.4A.1	Test Purpose and Environment	3070

A.8.26.4A.2	Test Requirements	3074

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3074

A.8.26.5.1	Test Purpose and Environment	3074

A.8.26.5.2	Test Requirements	3077

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3077

A.8.26.5A.1	Test Purpose and Environment	3077

A.8.26.5A.2	Test Requirements	3081

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3081

A.8.26.6.1	Test Purpose and Environment	3081

A.8.26.6.2	Test Requirements	3085

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3085

A.8.26.6A.1	Test Purpose and Environment	3085

A.8.26.6A.2	Test Requirements	3089

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3089

A.8.26.7.1	Test Purpose and Environment	3089

A.8.26.7.2	Test Requirements	3092

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3092

A.8.26.8.1	Test Purpose and Environment	3092

A.8.26.8.2	Test Requirements	3096

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3096

A.8.26.9.1	Test Purpose and Environment	3096

A.8.26.9.2	Test Requirements	3099

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3099

A.8.26.10.1	Test Purpose and Environment	3099

A.8.26.10.2	Test Requirements	3102

A.9	Measurement Performance Requirements	3104

A.9.1	RSRP	3104

A.9.1.1	FDD Intra frequency case	3104

A.9.1.1.1	Test Purpose and Environment	3104

A.9.1.1.2	Test parameters	3104

A.9.1.1.3	Test Requirements	3107

A.9.1.2	TDD Intra frequency case	3107

A.9.1.2.1	Test Purpose and Environment	3107

A.9.1.2.2	Test parameters	3107

A.9.1.2.3	Test Requirements	3108

A.9.1.3	FDD—FDD Inter frequency case	3109

A.9.1.3.1	Test Purpose and Environment	3109

A.9.1.3.2	Test parameters	3109

A.9.1.3.3	Test Requirements	3112

A.9.1.4	TDD—TDD Inter frequency case	3112

A.9.1.4.1	Test Purpose and Environment	3112

A.9.1.4.2	Test parameters	3112

A.9.1.4.3	Test Requirements	3114

A.9.1.5	FDD—TDD Inter frequency case	3115

A.9.1.5.1	Test Purpose and Environment	3115

A.9.1.5.2	Test parameters	3115

A.9.1.5.3	Test Requirements	3116

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	3117

A.9.1.6.1	Test Purpose and Environment	3117

A.9.1.6.2	Test parameters	3117

A.9.1.6.3	Test Requirements	3120

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	3120

A.9.1.7.1	Test Purpose and Environment	3120

A.9.1.7.2	Test parameters	3120

A.9.1.7.3	Test Requirements	3123

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3123

A.9.1.8.1	Test Purpose and Environment	3123

A.9.1.8.2	Test parameters	3123

A.9.1.8.3	Test Requirements	3127

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3127

A.9.1.9.1	Test Purpose and Environment	3127

A.9.1.9.2	Test parameters	3127

A.9.1.9.3	Test Requirements	3130

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	3130

A.9.1.10.1	Test Purpose and Environment	3130

A.9.1.10.2	Test parameters	3130

A.9.1.10.3	Test Requirements	3133

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	3133

A.9.1.11.1	Test Purpose and Environment	3133

A.9.1.11.2	Test parameters	3133

A.9.1.11.3	Test Requirements	3137

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	3137

A.9.1.12.1	Test Purpose and Environment	3137

A.9.1.12.2	Test parameters	3137

A.9.1.12.3	Test Requirements	3138

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	3138

A.9.1.13.1	Test Purpose and Environment	3138

A.9.1.13.2	Test parameters	3138

A.9.1.13.3	Test Requirements	3139

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3139

A.9.1.14.1	Test Purpose and Environment	3139

A.9.1.14.2	Test parameters	3139

A.9.1.14.3	Test Requirements	3143

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3143

A.9.1.15.1	Test Purpose and Environment	3143

A.9.1.15.2	Test parameters	3143

A.9.1.15.3	Test Requirements	3145

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	3146

A.9.1.16.1	Test Purpose and Environment	3146

A.9.1.16.2	Test parameters	3146

A.9.1.16.3	Test Requirements	3147

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	3147

A.9.1.17.1	Test Purpose and Environment	3147

A.9.1.17.2	Test parameters	3148

A.9.1.17.3	Test Requirements	3149

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	3149

A.9.1.18.1	Test Purpose and Environment	3149

A.9.1.18.2	Test parameters	3149

A.9.1.18.3	Test Requirements	3150

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	3151

A.9.1.19.1	Test Purpose and Environment	3151

A.9.1.19.2	Test parameters	3151

A.9.1.19.3	Test Requirements	3151

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	3151

A.9.1.20.1	Test Purpose and Environment	3151

A.9.1.20.2	Test parameters	3151

A.9.1.20.3	Test Requirements	3153

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	3153

A.9.1.21.1	Test Purpose and Environment	3153

A.9.1.21.2	Test parameters	3153

A.9.1.21.3	Test Requirements	3153

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	3153

A.9.1.22.1	Test Purpose and Environment	3153

A.9.1.22.2	Test parameters	3154

A.9.1.22.3	Test Requirements	3158

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	3158

A.9.1.23.1	Test Purpose and Environment	3158

A.9.1.23.2	Test parameters	3158

A.9.1.23.3	Test Requirements	3162

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	3162

A.9.1.24.1	Test Purpose and Environment	3162

A.9.1.24.2	Test parameters	3162

A.9.1.24.3	Test Requirements	3163

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3163

A.9.1.25.1	Test Purpose and Environment	3163

A.9.1.25.2	Test parameters	3163

A.9.1.25.3	Test Requirements	3166

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3166

A.9.1.26.1	Test Purpose and Environment	3166

A.9.1.26.2	Test parameters	3166

A.9.1.26.3	Test Requirements	3169

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3169

A.9.1.27.1	Test Purpose and Environment	3169

A.9.1.27.2	Test parameters	3169

A.9.1.27.3	Test Requirements	3172

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	3172

A.9.1.28.1	Test Purpose and Environment	3172

A.9.1.28.2	Test parameters	3172

A.9.1.28.3	Test Requirements	3175

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3175

A.9.1.29.1	Test Purpose and Environment	3175

A.9.1.29.2	Test parameters	3175

A.9.1.29.3	Test Requirements	3178

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3178

A.9.1.30.1	Test Purpose and Environment	3178

A.9.1.30.2	Test parameters	3179

A.9.1.30.3	Test Requirements	3182

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3182

A.9.1.31.1	Test Purpose and Environment	3182

A.9.1.31.2	Test parameters	3182

A.9.1.31.3	Test Requirements	3185

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	3186

A.9.1.32.1	Test Purpose and Environment	3186

A.9.1.32.2	Test parameters	3186

A.9.1.32.3	Test Requirements	3189

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3189

A.9.1.33.1	Test Purpose and Environment	3189

A.9.1.33.2	Test parameters	3189

A.9.1.33.3	Test Requirements	3192

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3192

A.9.1.34.1	Test Purpose and Environment	3192

A.9.1.34.2	Test parameters	3192

A.9.1.34.3	Test Requirements	3195

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3195

A.9.1.35.1	Test Purpose and Environment	3195

A.9.1.35.2	Test parameters	3195

A.9.1.35.3	Test Requirements	3198

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3199

A.9.1.36.1	Test Purpose and Environment	3199

A.9.1.36.2	Test parameters	3199

A.9.1.36.3	Test Requirements	3202

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	3202

A.9.1.37.1	Test Purpose and Environment	3202

A.9.1.37.2	Test parameters	3202

A.9.1.37.3	Test Requirements	3205

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	3206

A.9.1.38.1	Test Purpose and Environment	3206

A.9.1.38.2	Test parameters	3206

A.9.1.38.3	Test Requirements	3209

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3210

A.9.1.39.1	Test Purpose and Environment	3210

A.9.1.39.2	Test parameters	3210

A.9.1.39.3	Test Requirements	3216

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3216

A.9.1.40.1	Test Purpose and Environment	3216

A.9.1.40.2	Test parameters	3217

A.9.1.40.3	Test Requirements	3223

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	3223

A.9.1.41.1	Test Purpose and Environment	3223

A.9.1.41.2	Test parameters	3223

A.9.1.41.3	Test Requirements	3226

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	3226

A.9.1.42.1	Test Purpose and Environment	3226

A.9.1.42.2	Test parameters	3226

A.9.1.42.3	Test Requirements	3229

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	3229

A.9.1.43.1	Test Purpose and Environment	3229

A.9.1.43.2	Test parameters	3229

A.9.1.43.3	Test Requirements	3230

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3231

A.9.1.44.1	Test Purpose and Environment	3231

A.9.1.44.2	Test parameters	3231

A.9.1.44.3	Test Requirements	3236

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3236

A.9.1.45.1	Test Purpose and Environment	3236

A.9.1.45.2	Test parameters	3236

A.9.1.45.3	Test Requirements	2416

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2416

A.9.1.46.1	Test Purpose and Environment	2416

A.9.1.46.2	Test parameters	2416

A.9.1.46.3	Test Requirements	2425

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2426

A.9.1.47.1	Test Purpose and Environment	2426

A.9.1.47.2	Test parameters	2426

A.9.1.47.3	Test Requirements	2435

A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2435

A.9.1.48.1	Test Purpose and Environment	2435

A.9.1.48.2	Test parameters	2436

A.9.1.48.3	Test Requirements	2446

A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2447

A.9.1.49.1	Test Purpose and Environment	2447

A.9.1.49.2	Test parameters	2447

A.9.1.49.3	Test Requirements	2457

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2458

A.9.1.50.1	Test Purpose and Environment	2458

A.9.1.50.2	Test parameters	2458

A.9.1.50.3	Test Requirements	2465

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2466

A.9.1.51.1	Test Purpose and Environment	2466

A.9.1.51.2	Test parameters	2466

A.9.1.51.3	Test Requirements	2472

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2473

A.9.1.52.1	Test Purpose and Environment	2473

A.9.1.52.2	Test parameters	2473

A.9.1.52.3	Test Requirements	2476

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2476

A.9.1.52A.1	Test Purpose and Environment	2476

A.9.1.52A.2	Test parameters	2476

A.9.1.52A.3	Test Requirements	2477

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2477

A.9.1.53.1	Test Purpose and Environment	2477

A.9.1.53.2	Test parameters	2478

A.9.1.53.3	Test Requirements	2481

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2481

A.9.1.53A.1	Test Purpose and Environment	2481

A.9.1.53A.2	Test parameters	2481

A.9.1.53A.3	Test Requirements	2482

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2483

A.9.1.54.1	Test Purpose and Environment	2483

A.9.1.54.2	Test parameters	2483

A.9.1.54.3	Test Requirements	2485

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	2485

A.9.1.55.1	Test Purpose and Environment	2485

A.9.1.55.2	Test parameters	2485

A.9.1.55.3	Test Requirements	2489

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	2489

A.9.1.56.1	Test Purpose and Environment	2489

A.9.1.56.2	Test parameters	2489

A.9.1.56.3	Test Requirements	2492

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2493

A.9.1.57.1	Test Purpose and Environment	2493

A.9.1.57.2	Test parameters	2493

A.9.1.57.3	Test Requirements	2496

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2496

A.9.1.57A.1	Test Purpose and Environment	2496

A.9.1.57A.2	Test parameters	2496

A.9.1.57A.3	Test Requirements	2497

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2497

A.9.1.58.1	Test Purpose and Environment	2497

A.9.1.58.2	Test parameters	2498

A.9.1.58.3	Test Requirements	2501

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2501

A.9.1.58A.1	Test Purpose and Environment	2501

A.9.1.58A.2	Test parameters	2501

A.9.1.58A.3	Test Requirements	2502

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2502

A.9.1.59.1	Test Purpose and Environment	2502

A.9.1.59.2	Test parameters	2503

A.9.1.59.3	Test Requirements	2505

A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	2505

A.9.1.60.1	Test Purpose and Environment	2505

A.9.1.60.2	Test parameters	2505

A.9.1.60.3	Test Requirements	2508

A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	2509

A.9.1.61.1	Test Purpose and Environment	2509

A.9.1.61.2	Test parameters	2509

A.9.1.61.3	Test Requirements	2512

A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2512

A.9.1.62.1	Test Purpose and Environment	2512

A.9.1.62.2	Test parameters	2512

A.9.1.62.3	Test Requirements	2515

A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2515

A.9.1.53.1	Test Purpose and Environment	2515

A.9.1.53.2	Test parameters	2515

A.9.1.63.3	Test Requirements	2518

A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2518

A.9.1.64.1	Test Purpose and Environment	2518

A.9.1.64.2	Test parameters	2518

A.9.1.64.3	Test Requirements	2519

A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2520

A.9.1.65.1	Test Purpose and Environment	2520

A.9.1.65.2	Test parameters	2520

A.9.1.65.3	Test Requirements	2526

A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2526

A.9.1.66.1	Test Purpose and Environment	2526

A.9.1.66.2	Test parameters	2526

A.9.1.66.3	Test Requirements	2532

A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2532

A.9.1.67.1	Test Purpose and Environment	2532

A.9.1.67.2	Test parameters	2532

A.9.1.67.3	Test Requirements	2533

A.9.1.68	3 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2534

A.9.1.68.1	Test Purpose and Environment	2534

A.9.1.68.2	Test parameters	2534

A.9.1.68.3	Test Requirements	2538

A.9.1.69	4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2538

A.9.1.69.1	Test Purpose and Environment	2538

A.9.1.69.2	Test parameters	2538

A.9.1.69.3	Test Requirements	2542

A.9.1.70	5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2543

A.9.1.70.1	Test Purpose and Environment	2543

A.9.1.70.2	Test parameters	2543

A.9.1.70.3	Test Requirements	2555

A.9.1.71	6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2556

A.9.1.71.1	Test Purpose and Environment	2556

A.9.1.71.2	Test parameters	2556

A.9.1.71.3	Test Requirements	2568

A.9.1.72	7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2569

A.9.1.72.1	Test Purpose and Environment	2569

A.9.1.72.2	Test parameters	2569

A.9.1.72.3	Test Requirements	2581

A.9.1.73	FDD Intra frequency case for CA Idle Mode Measurements	2582

A.9.1.73.1	Test Purpose and Environment	2582

A.9.1.73.2	Test parameters	2582

A.9.1.73.3	Test Requirements	2585

A.9.1.74	FDD—FDD Inter frequency case for CA Idle Mode Measurements for overlapping carrier	2585

A.9.1.74.1	Test Purpose and Environment	2585

A.9.1.74.2	Test parameters	2585

A.9.1.74.3	Test Requirements	2586

A.9.1.75	FDD—FDD Inter frequency case for CA Idle Mode Measurements for non-overlapping carrier	2586

A.9.1.75.1	Test Purpose and Environment	2586

A.9.1.75.2	Test parameters	2587

A.9.1.75.3	Test Requirements	2588

A.9.1.76	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	2588

A.9.1.76.1	Test Purpose and Environment	2588

A.9.1.76.2	Test parameters	2588

A.9.1.76.3	Test Requirements	2591

A.9.1.77	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	2591

A.9.1.77.1	Test Purpose and Environment	2591

A.9.1.77.2	Test parameters	2591

A.9.1.77.3	Test Requirements	2594

A.9.1.78	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	2594

A.9.1.78.1	Test Purpose and Environment	2594

A.9.1.78.2	Test parameters	2594

A.9.1.78.3	Test Requirements	2597

A.9.1.79	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	2597

A.9.1.79.1	Test Purpose and Environment	2597

A.9.1.79.2	Test parameters	2597

A.9.1.79.3	Test Requirements	2600

A.9.1.80	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	2600

A.9.1.80.1	Test Purpose and Environment	2600

A.9.1.80.2	Test parameters	2600

A.9.1.80.3	Test Requirements	2603

A.9.1.81	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	2603

A.9.1.81.1	Test Purpose and Environment	2603

A.9.1.78.2	Test parameters	2603

A.9.1.81.3	Test Requirements	2606

A.9.2	RSRQ	2606

A.9.2.1	FDD Intra frequency case	2606

A.9.2.1.1	Test Purpose and Environment	2606

A.9.2.1.2	Test parameters	2606

A.9.2.1.3	Test Requirements	2609

A.9.2.2	TDD Intra frequency case	2609

A.9.2.2.1	Test Purpose and Environment	2609

A.9.2.2.2	Test parameters	2609

A.9.2.2.3	Test Requirements	2611

A.9.2.3	FDD—FDD Inter frequency case	2611

A.9.2.3.1	Test Purpose and Environment	2611

A.9.2.3.2	Test parameters	2611

A.9.2.3.3	Test Requirements	2614

A.9.2.4	TDD—TDD Inter frequency case	2614

A.9.2.4.1	Test Purpose and Environment	2614

A.9.2.4.2	Test parameters	2614

A.9.2.4.3	Test Requirements	2616

A.9.2.4A	FDD—TDD Inter frequency case	2617

A.9.2.4A.1	Test Purpose and Environment	2617

A.9.2.4A.2	Test parameters	2617

A.9.2.4A.3	Test Requirements	2619

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	2620

A.9.2.5.1	Test Purpose and Environment	2620

A.9.2.5.2	Test parameters	2620

A.9.2.5.3	Test Requirements	2623

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	2623

A.9.2.6.1	Test Purpose and Environment	2623

A.9.2.6.2	Test parameters	2624

A.9.2.6.3	Test Requirements	2627

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2627

A.9.2.7.1	Test Purpose and Environment	2627

A.9.2.7.2	Test parameters	2627

A.9.2.7.3	Test Requirements	2631

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2631

A.9.2.8.1	Test Purpose and Environment	2631

A.9.2.8.2	Test parameters	2631

A.9.2.8.3	Test Requirements	2635

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	2635

A.9.2.9.1	Test Purpose and Environment	2635

A.9.2.9.2	Test parameters	2635

A.9.2.9.3	Test Requirements	2639

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	2639

A.9.2.10.1	Test Purpose and Environment	2639

A.9.2.10.2	Test parameters	2639

A.9.2.10.3	Test Requirements	2643

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2643

A.9.2.11.1	Test Purpose and Environment	2643

A.9.2.11.2	Test parameters	2643

A.9.2.11.3	Test Requirements	2644

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2644

A.9.2.12.1	Test Purpose and Environment	2644

A.9.2.12.2	Test parameters	2645

A.9.2.12.3	Test Requirements	2645

A.9.2.13	Void	2646

A.9.2.13.1	Void	2646

A.9.2.13.2	Void	2646

A.9.2.13.3	Void	2646

A.9.2.14	Void	2646

A.9.2.14.1	Void	2646

A.9.2.14.2	Void	2646

A.9.2.14.3	Void	2646

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2646

A.9.2.15.1	Test Purpose and Environment	2646

A.9.2.15.2	Test parameters	2646

A.9.2.15.3	Test Requirements	2650

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2650

A.9.2.16.1	Test Purpose and Environment	2650

A.9.2.16.2	Test parameters	2650

A.9.2.16.3	Test Requirements	2655

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	2655

A.9.2.17.1	Test Purpose and Environment	2655

A.9.2.17.2	Test parameters	2655

A.9.2.17.3	Test Requirements	2657

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	2657

A.9.2.18.1	Test Purpose and Environment	2657

A.9.2.18.2	Test parameters	2657

A.9.2.18.3	Test Requirements	2660

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	2660

A.9.2.19.1	Test Purpose and Environment	2660

A.9.2.19.2	Test parameters	2660

A.9.2.19.3	Test Requirements	2662

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	2662

A.9.2.20.1	Test Purpose and Environment	2662

A.9.2.20.2	Test parameters	2662

A.9.2.20.3	Test Requirements	2665

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2665

A.9.2.21.1	Test Purpose and Environment	2665

A.9.2.21.2	Test parameters	2665

A.9.2.21.3	Test Requirements	2668

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2668

A.9.2.22.1	Test Purpose and Environment	2669

A.9.2.22.2	Test parameters	2669

A.9.2.22.3	Test Requirements	2669

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2669

A.9.2.23.1	Test Purpose and Environment	2669

A.9.2.23.2	Test parameters	2669

A.9.2.23.3	Test Requirements	2672

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2672

A.9.2.24.1	Test Purpose and Environment	2672

A.9.2.24.2	Test parameters	2672

A.9.2.24.3	Test Requirements	2673

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2673

A.9.2.25.1	Test Purpose and Environment	2673

A.9.2.25.2	Test parameters	2673

A.9.2.25.3	Test Requirements	2678

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2678

A.9.2.26.1	Test Purpose and Environment	2678

A.9.2.26.2	Test parameters	2678

A.9.2.26.3	Test Requirements	2682

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	2682

A.9.2.27.1	Test Purpose and Environment	2682

A.9.2.27.2	Test parameters	2682

A.9.2.27.3	Test Requirements	2683

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2683

A.9.2.28.1	Test Purpose and Environment	2683

A.9.2.28.2	Test parameters	2683

A.9.2.28.3	Test Requirements	2686

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2686

A.9.2.29.1	Test Purpose and Environment	2686

A.9.2.29.2	Test parameters	2686

A.9.2.29.3	Test Requirements	2689

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2689

A.9.2.30.1	Test Purpose and Environment	2689

A.9.2.30.2	Test parameters	2689

A.9.2.30.3	Test Requirements	2692

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2692

A.9.2.31.1	Test Purpose and Environment	2692

A.9.2.31.2	Test parameters	2692

A.9.2.31.3	Test Requirements	2695

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2695

A.9.2.32.1	Test Purpose and Environment	2695

A.9.2.32.2	Test parameters	2695

A.9.2.32.3	Test Requirements	2698

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2698

A.9.2.33.1	Test Purpose and Environment	2698

A.9.2.33.2	Test parameters	2698

A.9.2.33.3	Test Requirements	2702

A.9.2.34	FDD—FDD Inter frequency new RSRQ	2702

A.9.2.34.1	Test Purpose and Environment	2702

A.9.2.34.2	Test parameters	2702

A.9.2.34.3	Test Requirements	2705

A.9.2.35	TDD—TDD Inter frequency new RSRQ	2705

A.9.2.35.1	Test Purpose and Environment	2705

A.9.2.35.2	Test parameters	2705

A.9.2.35.3	Test Requirements	2708

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	2708

A.9.2.36.1	Test Purpose and Environment	2708

A.9.2.36.2	Test parameters	2708

A.9.2.36.3	Test Requirements	2709

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	2710

A.9.2.37.1	Test Purpose and Environment	2710

A.9.2.37.2	Test parameters	2710

A.9.2.37.3	Test Requirements	2711

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2712

A.9.2.38.1	Test Purpose and Environment	2712

A.9.2.38.2	Test parameters	2712

A.9.2.38.3	Test Requirements	2716

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2716

A.9.2.39.1	Test Purpose and Environment	2716

A.9.2.39.2	Test parameters	2716

A.9.2.39.3	Test Requirements	2720

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2720

A.9.2.40.1	Test Purpose and Environment	2720

A.9.2.40.2	Test parameters	2720

A.9.2.40.3	Test Requirements	2723

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2724

A.9.2.41.1	Test Purpose and Environment	2724

A.9.2.41.2	Test parameters	2724

A.9.2.41.3	Test Requirements	2727

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	2727

A.9.2.42.1	Test Purpose and Environment	2727

A.9.2.42.2	Test parameters	2727

A.9.2.42.3	Test Requirements	2730

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	2730

A.9.2.43.1	Test Purpose and Environment	2730

A.9.2.43.2	Test parameters	2730

A.9.2.43.3	Test Requirements	2733

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	2733

A.9.2.44.1	Test Purpose and Environment	2733

A.9.2.44.2	Test parameters	2733

A.9.2.44.3	Test Requirements	2735

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	2735

A.9.2.45.1	Test Purpose and Environment	2735

A.9.2.45.2	Test parameters	2735

A.9.2.45.3	Test Requirements	2739

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	2740

A.9.2.46.1	Test Purpose and Environment	2740

A.9.2.46.2	Test parameters	2740

A.9.2.46.3	Test Requirements	2744

A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2745

A.9.2.47.1	Test Purpose and Environment	2745

A.9.2.47.2	Test parameters	2745

A.9.2.47.3	Test Requirements	2749

A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2749

A.9.2.48.1	Test Purpose and Environment	2749

A.9.2.48.2	Test parameters	2750

A.9.2.48.3	Test Requirements	2754

A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2755

A.9.2.49.1	Test Purpose and Environment	2755

A.9.2.49.2	Test parameters	2755

A.9.2.49.3	Test Requirements	2761

A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2762

A.9.2.50.1	Test Purpose and Environment	2762

A.9.2.50.2	Test parameters	2762

A.9.2.50.3	Test Requirements	2768

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	2768

A.9.2.51.1	Test Purpose and Environment	2768

A.9.2.51.2	Test parameters	2769

A.9.2.51.3	Test Requirements	2773

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	2773

A.9.2.52.1	Test Purpose and Environment	2773

A.9.2.52.2	Test parameters	2773

A.9.2.52.3	Test Requirements	2776

A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2777

A.9.2.53.1	Test Purpose and Environment	2777

A.9.2.53.2	Test parameters	2777

A.9.2.53.3	Test Requirements	2783

A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2784

A.9.2.54.1	Test Purpose and Environment	2784

A.9.2.54.2	Test parameters	2784

A.9.2.54.3	Test Requirements	2790

A.9.2.55	3 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2790

A.9.2.55.1	Test Purpose and Environment	2790

A.9.2.55.2	Test parameters	2791

A.9.2.55.3	Test Requirements	2796

A.9.2.56	4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2796

A.9.2.56.1	Test Purpose and Environment	2796

A.9.2.56.2	Test parameters	2796

A.9.2.56.3	Test Requirements	2801

A.9.2.57	5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2801

A.9.2.57.1	Test Purpose and Environment	2801

A.9.2.57.2	Test parameters	2802

A.9.2.57.3	Test Requirements	2806

A.9.2.58	6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2807

A.9.2.58.1	Test Purpose and Environment	2807

A.9.2.58.2	Test parameters	2807

A.9.2.58.3	Test Requirements	2811

A.9.2.59	7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2812

A.9.2.59.1	Test Purpose and Environment	2812

A.9.2.59.2	Test parameters	2812

A.9.2.59.3	Test Requirements	2817

A.9.2.60	FDD Intra frequency case for CA Idle Mode Measurements	2818

A.9.2.60.1	Test Purpose and Environment	2818

A.9.2.60.2	Test parameters	2818

A.9.2.60.3	Test Requirements	2821

A.9.2.61	FDD—FDD Inter frequency case for CA Idle Mode Measurements on overlapping carrier	2821

A.9.2.61.1	Test Purpose and Environment	2821

A.9.2.61.2	Test parameters	2821

A.9.2.61.3	Test Requirements	2824

A.9.2.62	FDD—FDD Inter frequency case for CA Idle Mode Measurements on non-overlapping carrier	2824

A.9.2.62.1	Test Purpose and Environment	2824

A.9.2.62.2	Test parameters	2824

A.9.2.62.3	Test Requirements	2827

A.9.3	UTRAN FDD CPICH RSCP	2827

A.9.3.1	E-UTRAN FDD	2827

A.9.3.1.1	Test Purpose and Environment	2827

A.9.3.1.2	Parameters	2827

A.9.3.1.3	Test Requirements	2830

A.9.3.2 E-UTRAN TDD	2830

A.9.3.2.1	Test Purpose and Environment	2830

A.9.3.2.2	Parameters	2830

A.9.3.2.3	Test Requirements	2833

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	2833

A.9.3.3.1	Test Purpose and Environment	2833

A.9.3.3.2	Parameters	2833

A.9.3.3.3	Test Requirements	2834

A.9.4	UTRAN FDD CPICH Ec/No	2835

A.9.4.1	E-UTRAN FDD	2835

A.9.4.1.1	Test Purpose and Environment	2835

A.9.4.1.2	Parameters	2835

A.9.4.1.3	Test Requirements	2837

A.9.4.2	E-UTRAN TDD	2838

A.9.4.2.1	Test Purpose and Environment	2838

A.9.4.2.2	Parameters	2838

A.9.4.2.3	Test Requirements	2840

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	2841

A.9.4.3.1	Test Purpose and Environment	2841

A.9.4.3.2	Parameters	2841

A.9.4.3.3	Test Requirements	2842

A.9.5	UTRAN TDD measurement	2842

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	2842

A.9.5.1.1	Test Purpose and Environment	2842

A.9.5.1.2	Test parameters	2842

A.9.5.1.3	Test Requirements	2844

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	2845

A.9.5.2.1	Test Purpose and Environment	2845

A.9.5.2.2	Test parameters	2845

A.9.5.2.3	Test Requirements	2846

A.9.6	GSM Carrier RSSI	2847

A.9.6.1	E-UTRAN FDD	2847

A.9.6.1.1	Test Purpose and Environment	2847

A.9.6.1.2	Test Requirements	2848

A.9.6.2	E-UTRAN TDD	2848

A.9.6.2.1	Test Purpose and Environment	2848

A.9.6.2.2	Test Requirements	2850

A.9.7	UE Rx – Tx Time Difference	2851

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	2851

A.9.7.1.1	Test Purpose and Environment	2851

A.9.7.1.2	Test parameters	2851

A.9.7.1.3	Test Requirements	2853

A.9.7.2	E-UTRA TDD UE Rx – Tx time difference case	2853

A.9.7.2.1	Test Purpose and Environment	2853

A.9.7.2.2	Test parameters	2853

A.9.7.2.3	Test Requirements	2855

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2855

A.9.7.3.1	Test Purpose and Environment	2855

A.9.7.3.2	Test parameters	2855

A.9.7.3.3	Test Requirements	2858

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2858

A.9.7.4.1	Test Purpose and Environment	2858

A.9.7.4.2	Test Parameters	2858

A.9.7.4.3	Test Requirements	2861

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2861

A.9.7.5.1	Test Purpose and Environment	2861

A.9.7.5.2	Test parameters	2861

A.9.7.5.3	Test Requirements	2864

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2864

A.9.7.6.1	Test Purpose and Environment	2864

A.9.7.6.2	Test Parameters	2864

A.9.7.6.3	Test Requirements	2867

A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2867

A.9.7.7.1	Test Purpose and Environment	2867

A.9.7.7.2	Test parameters	2867

A.9.7.7.3	Test Requirements	2868

A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2869

A.9.7.8.1	Test Purpose and Environment	2869

A.9.7.8.2	Test parameters	2869

A.9.7.8.3	Test Requirements	2870

A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2870

A.9.7.9.1	Test Purpose and Environment	2870

A.9.7.9.2	Test parameters	2870

A.9.7.9.3	Test Requirements	2871

A.9.8	RSTD	2872

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	2872

A.9.8.1.1	Test Purpose and Environment	2872

A.9.8.1.2	Test Requirements	2874

A.9.8.1.2A	Test Requirements for UE Category 1bis	2874

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	2875

A.9.8.2.1	Test Purpose and Environment	2875

A.9.8.2.2	Test Requirements	2878

A.9.8.2.2A	Test Requirements for UE Category 1bis	2878

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	2879

A.9.8.3.1	Test Purpose and Environment	2879

A.9.8.3.2	Test Requirements	2881

A.9.8.3.2A	Test Requirements for UE Category 1bis	2881

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	2882

A.9.8.4.1	Test Purpose and Environment	2882

A.9.8.4.2	Test Requirements	2884

A.9.8.4.2A	Test Requirements for UE Category 1bis	2884

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	2885

A.9.8.5.1	Test Purpose and Environment	2885

A.9.8.5.2	Test Requirements	2887

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	2887

A.9.8.6.1	Test Purpose and Environment	2887

A.9.8.6.2	Test Requirements	2890

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2890

A.9.8.7.1	Test Purpose and Environment	2890

A.9.8.7.2	Test Requirements	2891

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2891

A.9.8.8.1	Test Purpose and Environment	2891

A.9.8.8.2	Test Requirements	2892

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2892

A.9.8.9.1	Test Purpose and Environment	2892

A.9.8.9.2	Test Requirements	2893

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2893

A.9.8.10.1	Test Purpose and Environment	2893

A.9.8.10.2	Test Requirements	2894

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	2894

A.9.8.11.1	Test Purpose and Environment	2894

A.9.8.11.2	Test Requirements	2895

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	2895

A.9.8.12.1	Test Purpose and Environment	2895

A.9.8.12.2	Test Requirements	2896

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	2896

A.9.8.13.1	Test Purpose and Environment	2896

A.9.8.13.2	Test Requirements	2897

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2897

A.9.8.14.1	Test Purpose and Environment	2897

A.9.8.14.2	Test Requirements	2904

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2904

A.9.8.15.1	Test Purpose and Environment	2904

A.9.8.15.2	Test Requirements	2910

A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2910

A.9.8.16.1	Test Purpose and Environment	2910

A.9.8.16.2	Test Requirements	2913

A.9.8.17	HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2913

A.9.8.17.1	Test Purpose and Environment	2913

A.9.8.17.2	Test Requirements	2916

A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2916

A.9.8.18.1	Test Purpose and Environment	2916

A.9.8.18.2	Test Requirements	2919

A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2919

A.9.8.19.1	Test Purpose and Environment	2919

A.9.8.19.2	Test Requirements	2922

A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A	2922

A.9.8.20.1	Test Purpose and Environment	2922

A.9.8.20.2	Test Requirements	2925

A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA	2926

A.9.8.21.1	Test Purpose and Environment	2926

A.9.8.21.2	Test Requirements	2929

A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A	2930

A.9.8.22.1	Test Purpose and Environment	2930

A.9.8.22.2	Test Requirements	2933

A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B	2934

A.9.8.23.1	Test Purpose and Environment	2934

A.9.8.23.2	Test Requirements	2937

A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B	2937

A.9.8.24.1	Test Purpose and Environment	2937

A.9.8.24.2	Test Requirements	2941

A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B	2942

A.9.8.25.1	Test Purpose and Environment	2942

A.9.8.25.2	Test Requirements	2945

A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2946

A.9.8.26.1	Test Purpose and Environment	2946

A.9.8.26.2	Test Requirements	2948

A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2948

A.9.8.27.1	Test Purpose and Environment	2948

A.9.8.27.2	Test Requirements	2951

A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A	2951

A.9.8.28.1	Test Purpose and Environment	2951

A.9.8.28.2	Test Requirements	2954

A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2954

A.9.8.29.1	Test Purpose and Environment	2954

A.9.8.29.2	Test Requirements	2958

A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2959

A.9.8.30.1	Test Purpose and Environment	2959

A.9.8.30.2	Test Requirements	2962

A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B	2963

A.9.8.31.1	Test Purpose and Environment	2963

A.9.8.31.2	Test Requirements	2967

A.9.8.32	TDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2967

A.9.8.32.1	Test Purpose and Environment	2967

A.9.8.32.2	Test Requirements	2970

A.9.8.33	TDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2970

A.9.8.33.1	Test Purpose and Environment	2970

A.9.8.33.2	Test Requirements	2973

A.9.8.34	TDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2973

A.9.8.34.1	Test Purpose and Environment	2973

A.9.8.34.2	Test Requirements	2976

A.9.8.35	TDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2976

A.9.8.35.1	Test Purpose and Environment	2976

A.9.8.35.2	Test Requirements	2979

A.9.9	RSRP and RSRQ on the serving cell	2979

A.9.9.1	FDD Intra frequency serving cell case	2979

A.9.9.1.1	Test Purpose and Environment	2979

A.9.9.1.2	Test parameters	2979

A.9.9.1.3	Test Requirements	2982

A.9.9.2	TDD Intra frequency serving cell case	2982

A.9.9.2.1	Test Purpose and Environment	2982

A.9.9.2.2	Test parameters	2982

A.9.9.2.3	Test Requirements	2984

A.9.10	SSTD	2984

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	2984

A.9.10.1.1	Test Purpose and Environment	2984

A.9.10.1.2	Test parameters	2984

A.9.10.1.3	Test Requirements	2986

A.9.10.2	Void	2986

A.9.10.3	Void	2986

A.9.10.4	Void	2986

A.9.11 RSSI	2986

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	2986

A.9.11.1.1	Test Purpose and Environment	2986

A.9.11.1.2	Test parameters	2986

A.9.11.1.3	Test Requirements	2989

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	2989

A.9.11.2.1	Test Purpose and Environment	2989

A.9.11.2.2	Test parameters	2989

A.9.12	Channel occupancy	2992

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	2992

A.9.12.1.1	Test Purpose and Environment	2992

A.9.12.1.2	Test parameters	2992

A.9.12.1.3	Test Requirements	2995

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	2995

A.9.12.2.1	Test Purpose and Environment	2995

A.9.12.2.2	Test parameters	2995

A.9.12.2.3	Test Requirements	2998

A.9.13	RS-SINR	2998

A.9.13.1	FDD Intra-Frequency Case	2998

A.9.13.1.1	Test Purpose and Environment	2998

A.9.13.1.2	Test parameters	2998

A.9.13.1.3	Test Requirements	3001

A.9.13.2	TDD Intra-Frequency Case	3001

A.9.13.2.1	Test Purpose and Environment	3001

A.9.13.2.2	Test parameters	3001

A.9.13.2.3	Test Requirements	3004

A.9.13.3	FDD—FDD Inter frequency case	3004

A.9.13.3.1	Test Purpose and Environment	3004

A.9.13.3.2	Test parameters	3004

A.9.13.3.3	Test Requirements	3007

A.9.13.4	TDD—TDD Inter frequency case	3007

A.9.13.4.1	Test Purpose and Environment	3007

A.9.13.4.2	Test parameters	3007

A.9.13.4.3	Test Requirements	3011

A.9.13.5	FDD—TDD Inter frequency case	3012

A.9.13.5.1	Test Purpose and Environment	3012

A.9.13.5.2	Test parameters	3012

A.9.13.5.3	Test Requirements	3018

A.9.13.6	TDD—FDD Inter frequency case	3018

A.9.13.6.1	Test Purpose and Environment	3018

A.9.13.6.2	Test parameters	3018

A.9.13.6.3	Test Requirements	3022

A.9.14	Channel quality reporting accuracy	3022

A.9.14.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	3022

A.9.14.1.1	Test Purpose and Environment	3022

A.9.14.1.2	Test parameters	3023

A.9.14.1.3	Test Requirements	3023

A.9.14.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	3024

A.9.14.2.1	Test Purpose and Environment	3024

A.9.14.2.2	Test parameters	3024

A.9.14.2.3	Test Requirements	3025

A.9.14.3	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage	3025

A.9.14.3.1	Test Purpose and Environment	3025

A.9.14.3.2	Test parameters	3025

A.9.14.3.3	Test Requirements	3026

A.9.14.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage	3026

A.9.14.4.1	Test Purpose and Environment	3026

A.9.14.4.2	Test parameters	3026

A.9.14.4.3	Test Requirements	3027

A.10	Proximity-based Services in Any Cell Selection State	3050

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	3050

A.10.1.1	Test Purpose and Environment	3050

A.10.1.2	Test Requirements	3051

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	3052

A.10.2.1	Test Purpose and Environment	3052

A.10.2.2	Test Requirements	3053

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	3053

A.10.3.1	Test Purpose and Environment	3053

A.10.3.2	Test Requirements	3055

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	3056

A.10.4.1	Test Purpose and Environment	3056

A.10.4.2	Test Requirements	3058

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	3058

A.11.1	V2V UE Transmission Timing Accuracy Test	3058

A.11.1.1	Test Purpose and Environment	3058

A.11.1.2	Test requirements	3059

A.11.2	Interruptions due to V2V sidelink communication	3059

A.11.2.1	Test Purpose and Environment	3059

A.11.2.2	Test Requirements	3061

A.12	3061

A.12.1	V2X UE Transmission Timing Accuracy Test	3061

A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference	3061

A.12.1.1.1	Test Purpose and Environment	3061

A.12.1.1.2	Test requirements	3062

A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference	3063

A.12.1.2.1	Test Purpose and Environment	3063

A.12.1.2.2	Test Requirements	3063

A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	3064

A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference	3064

A.12.2.1.1	Test Purpose and Environment	3064

A.12.2.1.2	Test Requirements	3065

A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference	3066

A.12.2.2.1	Test Purpose and Environment	3066

A.12.2.2.2	Test Requirements	3067

A.12.3	V2X Synchronization Reference Selection/Reselection Tests	3067

A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority	3067

A.12.3.1.1	Test Purpose and Environment	3067

A.12.3.1.2	Test Requirements	3070

A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority	3071

A.12.3.2.1	Test Purpose and Environment	3071

A.12.3.1.2	Test Requirements	3073

A.12.4	Congestion Control Measurement Test for V2X UE	3073

A.12.4.1	Test Purpose and Environment	3073

A.12.4.2	Test Requirements	3075

A.12.5	Interruptions due to V2X Sidelink Communication	3075

A.12.5.1	Test Purpose and Environment	3075

A.12.5.2	Test Requirements	3077

A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test	3077

A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements	3077

A.12.6.1.1	Test Purpose and Environment	3077

A.12.6.1.2	Test Requirements	3079

A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements	3079

A.12.6.2.1	Test Purpose and Environment	3079

A.12.6.1.2	Test Requirements	3081

A.12.7	V2X Synchronization Reference Selection/Reselection Tests for V2X Carrier Aggregation	3081

A.12.7.1	Test Purpose and Environment	3081

A.12.7.2	Test Requirements	3084

A.12.8	Interruptions due to V2X Carrier Aggregation	3085

A.12.8.1	Interruptions on a FDD PCell	3085

A.12.8.1.1	Test Purpose and Environment	3085

A.12.8.1.2	Test Requirements	3086

A.12.8.2	Interruptions on a TDD PCell	3086

A.12.8.2.1	Test Purpose and Environment	3086

A.12.8.2.2	Test Requirements	3088

Annex B (normative): Conditions for RRM requirements applicability for operating bands	3089

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	3089

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	3089

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	3089

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1	3089

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	3091

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	3092

B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis	3092

B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2	3092

B.1.7.1	Conditions for measurements of intra-frequence E-UTRAN cells for cell selection	3092

B.1.7.2	Condition for measurements of inter-frequence E-UTRAN cells for cell selection	3094

B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1	3095

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	3095

B.2.1	Conditions for E-UTRAN intra-frequency measurements	3095

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	3096

B.2.3	Conditions for E-UTRAN inter-frequency measurements	3096

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	3097

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	3097

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	3097

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	3098

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	3098

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3099

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	3099

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	3099

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	3099

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	3100

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3100

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3100

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	3101

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	3101

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3101

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3102

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	3102

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	3103

B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1	3104

B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1	3105

B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1	3106

B.2.19	Conditions for E-UTRAN measurements by UE Category M2	3107

B.2.19.1	Conditions for E-UTRAN intra-frequency measurements	3107

B.2.19.2	Conditions for E-UTRAN inter-frequency measurements	3107

B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1	3107

B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2	3108

B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1	3108

B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2	3110

B.2.24	Conditions for intra-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1	3110

B.2.25	Conditions for inter-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1	3110

B.3	Conditions for measurements performance requirements for UE	3110

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	3110

B.3.2	Void	3111

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	3111

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	3111

B.3.5	Conditions for UE Rx – Tx time difference	3111

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	3111

B.3.7	Conditions for inter-frequency RSTD measurements	3111

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	3112

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	3112

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	3112

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3112

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3113

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	3113

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3113

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	3113

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	3113

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3114

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	3114

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	3114

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3114

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	3114

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	3114

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3115

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	3115

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	3115

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	3115

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	3115

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	3115

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3115

B.3.21.1	Conditions for RSRP measurements	3115

B.3.21.2	Conditions for RSRQ measurements	3116

B.3.21.3	Conditions for CSI-RSRP measurements	3116

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3116

B.3.22.1	Conditions for RSRP measurements	3116

B.3.22.2	Void	3116

B.3.22.3	Conditions for CSI-RSRP measurements	3116

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3117

B.3.23.1	Conditions for RSRP measurements	3117

B.3.23.2	Conditions for RSRQ measurements	3117

B.3.23.3	Conditions for CSI-RSRP measurements	3117

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3117

B.3.24.1	Conditions for RSRP measurements	3117

B.3.24.2	Conditions for RSRQ measurements	3117

B.3.24.3	Conditions for CSI-RSRP measurements	3117

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3118

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3118

B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0	3118

B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0	3118

B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3119

B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3119

B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3119

B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3119

B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3119

B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3119

B.4	RRM Requirements Exceptions	3120

B.4.1	General	3120

B.4.2	Receiver sensitivity relaxation for UE supporting CA	3120

B.4.3	Receiver sensitivity relaxation for UE configured with CA	3120

B.4.3.1	Inter-band carrier aggregation	3120

B.4.3.2	Intra-band non-contiguous carrier aggregation	3120

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	3120

B.5	Conditions for Measurement Performance Requirements for ProSe UE	3121

B.5.1	Conditions for S-RSRP Accuracy Requirements	3121

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	3121

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3121

B.5.4	Conditions for SD-RSRP Accuracy Requirements	3122

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	3122

B.6	Conditions for V2X	3123

B.6.1	Test parameters for GNSS signals	3123

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	3123

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	3123

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3123

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	3124

B.7	Conditions for sTTI and 1ms-TTI with 3 Subframe HARQ Processing	3124

B.7.1	Conditions for Maximum Timing Difference Between Uplink and Downlink Carriers in Carrier Aggregation	3124

Annex C (informative): Change history:	3126