| 3GPP TS 38.133 V18.12.0 (2025-12) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for support of radio resource management (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	162

1	Scope	164

2	References	164

3	Definitions, symbols and abbreviations	166

3.1	Definitions	166

3.2	Symbols	167

3.3	Abbreviations	168

3.4	Test tolerances	171

3.5	Frequency bands grouping	171

3.5.1	Introduction	171

3.5.2	NR operating bands in FR1	172

3.5.2A	NR operating bands for satellite access in FR1	148

3.5.3	NR operating bands in FR2	148

3.6	Applicability of requirements in this specification version	149

3.6.1	RRC connected state requirements in DRX	149

3.6.2	Number of serving carriers	150

3.6.2.1	Number of serving carriers for SA	150

3.6.2.2	Number of serving carriers for EN-DC	150

3.6.2.3	Number of serving carriers for NE-DC	150

3.6.2.4	Number of serving carriers for NR-DC	150

3.6.3	Applicability for intra-band FR2	150

3.6.4	Applicability for FR2 UE power classes	150

3.6.5	Applicability for SDL bands	151

3.6.6	Applicability of requirements for NGEN-DC operation	151

3.6.7	Applicability of QCL	151

3.6.9	Applicability of requirements for scheduling availability	152

3.6.10	Applicability of requirements for measurement restrictions	152

3.6.11	Applicability of requirements for Redcap UEs	152

3.6.11.1	RRC connected state requirements in DRX	152

3.6.11.2	Applicability for FR2 Redcap UE power classes	152

3.6.11.3	Applicability of QCL	152

3.6.12	Applicability of requirements for Satellite Access	152

3.6.13	Applicability of requirements for FR2	152

3.6.14	Applicability of requirements for FR2 Power Class 6	153

3.6.15	Applicability of requirements for per-FR gap	153

3.6.16	Applicability of requirements for ATG	153

3.6.17	Applicability of requirements for MUSIM gaps	153

3.6.18	Applicability of requirements for a UE operating on a cell with less than 5 MHz BW	153

3.6.19	Applicability of requirements for multi-Rx operation in FR2-1	153

4	SA: RRC_IDLE state mobility	153

4.1	Cell Selection	153

4.2	Cell Re-selection	154

4.2.1	Introduction	154

4.2.2	Requirements	154

4.2.2.1	UE measurement capability	154

4.2.2.2	Measurement and evaluation of serving cell	154

4.2.2.3	Measurements of intra-frequency NR cells	156

4.2.2.4	Measurements of inter-frequency NR cells	160

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	165

4.2.2.6	Maximum interruption in paging reception	167

4.2.2.7	General requirements	168

4.2.2.8	Minimum requirement at transitions	168

4.2.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	169

4.2.2.9.1	Introduction	169

4.2.2.9.2	Measurements for UE fulfilling low mobility criterion	169

4.2.2.9.3	Measurements for UE fulfilling not-at-cell edge criterion	171

4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	173

4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	174

4.2.2.10.1	Introduction	174

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion	174

4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion	176

4.2.2.10.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	179

4.2.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	179

4.2.2.11.1	Introduction	179

4.2.2.11.2	Measurements for UE fulfilling low mobility criterion	180

4.2.2.11.3	Measurements for UE fulfilling with not-at-cell edge criterion	181

4.2.2.11.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	183

4.2.2.12	 Measurements of inter-frequency NR cells with NTN carrier	183

4.2A	Cell Re-selection when subject to CCA	186

4.2A.1	Introduction	186

4.2A.2	Requirements	187

4.2A.2.1	UE measurement capability	187

4.2A.2.2	Measurement and evaluation when subject to CCA on the serving cell	187

4.2A.2.3	Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell	188

4.2A.2.4	Measurements of inter-frequency NR cells when subject to CCA on the target cell	189

4.2A.2.5	Measurements of inter-RAT E-UTRAN cells when subject to CCA on the serving cell	191

4.2A.2.6	Maximum interruption in paging reception when subject to CCA on the target cell	191

4.2A.2.7	General requirements	192

4.2B	Cell Re-selection for RedCap	192

4.2B.1	Introduction	192

4.2B.2	Requirements	192

4.2B.2.1	UE measurement capability for RedCap	192

4.2B.2.1.1	UE measurement capability for 1 Rx RedCap	192

4.2B.2.1.2	UE measurement capability for 2 Rx RedCap	192

4.2B.2.2	Measurement and evaluation of serving cell for RedCap UE	192

4.2B.2.3	Measurements of intra-frequency NR cells for RedCap UE	194

4.2B.2.4	Measurements of inter-frequency NR cells for RedCap UE	196

4.2B.2.5	Measurements of inter-RAT E-UTRAN cells for RedCap UE	199

4.2B.2.6	Maximum interruption in paging reception for RedCap	201

4.2B.2.7	General requirements for RedCap	201

4.2B.2.8	Minimum requirement at transitions	201

4.2B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion for RedCap	202

4.2B.2.9.1	Introduction	202

4.2B.2.9.2	Measurements for UE fulfilling stationary criterion	203

4.2B.2.9.3	Measurements for a UE fulfilling not-at-cell edge while stationary criterion	205

4.2B.2.9.3A	Measurements for a UE fulfilling stationary and not-at-cell-edge criteria	206

4.2B.2.9.4	Measurements for a UE fulfilling low mobility and stationary criteria	206

4.2B.2.9.5	Measurements for a UE fulfilling low mobility and not-at-cell-edge while stationary criteria	206

4.2B.2.9.6	Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria	207

4.2B.2.9.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria	207

4.2B.2.9.8	Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary criterion	207

4.2B.2.9.9	Measurements for UE fulfilling low mobility criterion	207

4.2B.2.9.10	Measurements for UE fulfilling not-at-cell edge criterion	210

4.2B.2.9.11	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	212

4.2B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	213

4.2B.2.10.1	Introduction	213

4.2B.2.10.2	Measurements for UE fulfilling stationary criterion	213

4.2B.2.10.3	Measurements for a UE fulfilling not-at-cell edge while stationary  criterion	215

4.2B.2.10.3A	Measurements for a UE fulfilling stationary and not-at-cell-edge criterion	216

4.2B.2.10.4	Measurements for a UE fulfilling low mobility and stationary criteria	216

4.2B.2.10.5	Measurements for a UE fulfilling low mobility and not-at-cell-edge while stationary criteria	216

4.2B.2.10.6	Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria	217

4.2B.2.10.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria	217

4.2B.2.10.8	Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary  criteria	217

4.2B.2.10.9	Measurements for UE fulfilling low mobility criterion	217

4.2B.2.10.10	Measurements for UE fulfilling not-at-cell edge criterion	220

4.2B.2.10.11	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	222

4.2B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	222

4.2B.2.11.1	Introduction	222

4.2B.2.11.2	Measurements for UE fulfilling stationary criterion	223

4.2B.2.11.3	Measurements for a UE fulfilling not-at-cell edge while stationary criterion	224

4.2B.2.11.3A	Measurements for a UE fulfilling stationary and not-at-cell-edge criterion	224

4.2B.2.11.4	Measurements for a UE fulfilling low mobility and stationary criteria	225

4.2B.2.11.5	Measurements for a UE fulfilling low mobility and not-at-cell-edge while stationary  criteria	225

4.2B.2.11.6	Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria	225

4.2B.2.11.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria	225

4.2B.2.11.8	Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary  criteria	226

4.2B.2.11.9	Measurements for UE fulfilling low mobility criterion	226

4.2B.2.11.10	Measurements for UE fulfilling with not-at-cell edge criterion	227

4.2B.2.11.11	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	228

4.2C	Cell Re-selection for NR UE for Satellite Access	229

4.2C.1	Introduction	229

4.2C.2	Requirements	229

4.2C.2.1	UE measurement capability	229

4.2C.2.2	Measurement and evaluation of serving cell	229

4.2C.2.3	Measurements of intra-frequency NR cells	230

4.2C.2.4	Measurements of inter-frequency NR cells	233

4.2C.2.5	Maximum interruption in paging reception	236

4.2C.2.6	Minimum requirement at transitions	237

4.2C.2.7	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	237

4.2C.2.8	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	237

4.2C.2.9	General requirements	237

4.2C.2.10	Measurements of inter-frequency NR cells with TN carrier	237

4.2C.2.11	Measurements of inter-RAT E-UTRAN cells with TN carrier	240

4.2C.3	Void	241

4.2C.4	Void	241

4.2D	Cell Re-selection for ATG	241

4.2D.1	Introduction	241

4.2D.2	Requirements	242

4.2D.2.1	UE measurement capability	242

4.2D.2.2	Measurement and evaluation of serving cell	242

4.2D.2.3	Measurements of intra-frequency NR cells	242

4.2D.2.4	Measurements of inter-frequency NR cells	243

4.2D.2.5	Maximum interruption in paging reception	245

4.2D.2.6	General requirements	245

4.3	Minimization of Drive Tests (MDT)	246

4.3.1	Introduction	246

4.3.2	Measurement Requirements	246

4.3.3	Requirements for Relative Time Stamp Accuracy	246

4.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	247

4.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	247

4.3C	Minimization of Drive Tests (MDT) for Satellite Access	247

4.3C.1	Introduction	247

4.3C.2	Measurement Requirements	247

4.3C.3	Requirements for Relative Time Stamp Accuracy	248

4.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	248

4.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	248

4.4	Idle Mode CA/DC Measurements	249

4.4.1	Introduction	249

4.4.2	Measurement Requirements	249

4.4.2.1	Detected cell requirement during state transition and Idle mode	249

4.4.2.2	Measurements of inter-frequency CA/DC candidate cells	249

4.4.2.3	Measurements on serving cell	250

4.4.2.4	Measurements of E-UTRAN inter-RAT DC candidate cells	251

4.5	NR measurements for positioning	251

4.5.1	Introduction	251

4.5.2	RSTD measurements	252

4.5.2.1	Introduction	252

4.5.2.2	Requirements Applicability	252

4.5.2.3	Measurement Capability	252

4.5.2.4	Measurement Reporting Requirements	252

4.5.2.5	Measurements Period Requirements	252

4.5.2.6	Measurements Period Requirements with Bandwidth Aggregation	255

4.5.3	PRS-RSRP measurements	259

4.5.3.1	Introduction	259

4.5.3.2	Requirements applicability	259

4.5.3.3	Measurement Capability	259

4.5.3.4	Measurement Reporting Requirements	259

4.5.3.5	Measurement Period Requirements	260

4.5.4	PRS-RSRPP measurements	262

4.5.4.1	Introduction	262

4.5.4.2	Requirements Applicability	262

4.5.4.3	Measurement Capability	262

4.5.4.4	Measurement Reporting Requirements	262

4.5.4.5	Measurement Period Requirements	263

4.5.5	Measurement requirements for DL RSCPD reported with RSTD	263

4.5.5.1	Introduction	263

4.5.5.2	Requirements Applicability	263

4.5.5.3	Measurement Capability	263

4.5.5.4	Measurement Reporting Requirements	263

4.5.5.5	Measurements Period Requirements	264

4.6	NR measurements for positioning for RedCap	268

4.6.1	Introduction	268

4.6.2	RSTD measurements for RedCap	268

4.6.2.1	Introduction	268

4.6.2.2	Requirements Applicability	268

4.6.2.3	Measurement Capability	269

4.6.2.4	Measurement Reporting Requirements	269

4.6.2.5	Measurements Period Requirements without RX FH	269

4.6.2.6	Measurement Period Requirements with RX FH	270

4.6.3	PRS-RSRP measurements for RedCap	272

4.6.3.1	Introduction	272

4.6.3.2	Requirements applicability	272

4.6.3.3	Measurement Capability	272

4.6.3.4	Measurement Reporting Requirements	272

4.6.3.5	Measurement Period Requirements without RX FH	272

4.6.3.6	Measurement Period Requirements with RX FH	275

4.6.4	PRS-RSRPP measurements for RedCap	277

4.6.4.1	Introduction	277

4.6.4.2	Requirements Applicability	277

4.6.4.3	Measurement Capability	277

4.6.4.4	Measurement Reporting Requirements	277

4.6.4.5	Measurement Period Requirements without RX FH	278

4.6.4.6	Measurement Period Requirements with RX FH	278

4.7	Measurement report for fast CA/DC setup	278

4.7.1	Introduction	278

4.7.2	Void	278

4.7.3	Measurement Report Requirements	278

5	SA: RRC_INACTIVE state mobility	279

5.1	Cell Re-selection	279

5.1.1	Introduction	279

5.1.2	Requirements	279

5.1.2.1	UE measurement capability	279

5.1.2.2	Measurement and evaluation of serving cell	279

5.1.2.3	Measurements of intra-frequency NR cells	281

5.1.2.4	Measurements of inter-frequency NR cells	283

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	285

5.1.2.6	Maximum interruption in paging reception	286

5.1.2.7	General requirements	286

5.1.2.8	Measurement of inter-frequency NR cells with NTN carrier	287

5.1.2.9	Minimum requirement at transitions	287

5.1.2.10	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	287

5.1.2.11	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	288

5.1.2.12	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	289

5.1A	Cell Re-selection with CCA	289

5.1A.1	Introduction	289

5.1A.2	Requirements	290

5.1A.2.1	UE measurement capability	290

5.1A.2.2	Measurement and evaluation when CCA is used on the serving cell	290

5.1A.2.3	Measurements of intra-frequency NR cells when CCA is used on the serving cell and target cell	290

5.1A.2.4	Measurements of inter-frequency NR cells when CCA is used on the target cell	290

5.1A.2.5	Measurements of inter-RAT E-UTRAN cells when CCA is used on the serving cell	290

5.1A.2.6	Maximum interruption in paging reception when CCA is used on the target cell	290

5.1A.2.7	General requirements	290

5.1B	Cell Re-selection for RedCap	290

5.1B.1	Introduction	290

5.1B.2	Requirements	290

5.1B.2.1	UE measurement capability	290

5.1B.2.2	Measurement and evaluation of serving cell	290

5.1B.2.3	Measurements of intra-frequency NR cells	293

5.1B.2.4	Measurements of inter-frequency NR cells	295

5.1B.2.5	Measurements of inter-RAT E-UTRAN cells	297

5.1B.2.6	Maximum interruption in paging reception	298

5.1B.2.7	General requirements	298

5.1B.2.8	Minimum requirement at transitions	298

5.1B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	298

5.1B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	300

5.1B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	303

5.1C	Cell Re-selection for Satellite Access	304

5.1C.1	Introduction	304

5.1C.2	Requirements	304

5.1C.2.1	UE measurement capability	304

5.1C.2.2	Measurement and evaluation of serving cell	304

5.1C.2.3	Measurements of intra-frequency NR cells	304

5.1C.2.4	Measurements of inter-frequency NR cells	305

5.1C.2.5	Maximum interruption in paging reception	305

5.1C.2.6	General requirements	305

5.1C.2.7	Measurements of inter-frequency NR cells with TN carrier	305

5.1C.2.8	Measurements of inter-RAT E-UTRAN cells with TN carrier	305

5.1C.3	Void	305

5.1C.4	Void	305

5.1D	Cell Re-selection for ATG	305

5.1D.1	Introduction	305

5.1D.2	Requirements	305

5.1D.2.1	UE measurement capability	305

5.1D.2.2	Measurement and evaluation of serving cell	305

5.1D.2.3	Measurements of intra-frequency NR cells	305

5.1D.2.4	Measurements of inter-frequency NR cells	305

5.1D.2.5	Maximum interruption in paging reception	306

5.1D.2.6	General requirements	306

5.2	Void	306

5.2B	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	306

5.2B.1	Introduction	306

5.2B.2	Requirements on UE synchronization for small data transmissions for RedCap	306

5.2B.2.1	Void	306

5.2B.3	TA validation requirements for RedCap	306

5.2B.3.1	Void	307

5.2B.3.2	Void	307

5.2B.4	Scheduling restriction	307

5.2B.5	Applicability conditions for CG-SDT for RedCap	308

5.3	Minimization of Drive Tests (MDT)	308

5.3.1	Introduction	308

5.3.2	Measurement Requirements	308

5.3.3	Requirements for Relative Time Stamp Accuracy	308

5.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	308

5.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	308

5.3.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	308

5.3C	Minimization of Drive Tests (MDT) for Satellite Access	309

5.3C.1	Introduction	309

5.3C.2	Measurement Requirements	309

5.3C.3	Requirements for Relative Time Stamp Accuracy	309

5.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	309

5.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	309

5.3C.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	309

5.4	Inactive Mode CA/DC Measurements	310

5.4.1	Introduction	310

5.4.2	Measurement Requirements	310

5.4.2.1	Detected cell requirement during state transition and inactive mode	310

5.4.2.2	Measurements of inter-frequency CA/DC candidate cells	310

5.4.2.3	Measurements on serving cell	310

5.4.2.4	Measurements on E-UTRAN inter-RAT DC candidate cells	310

5.5	Configured Grant based Small Data Transmissions (CG-SDT)	310

5.5.1	Introduction	310

5.5.2	Requirements on UE synchronization for small data transmissions	310

5.5.3	TA validation requirements	310

5.5.4	Scheduling restriction	312

5.5.4.1	Scheduling availability of UE performing measurements in TDD bands on FR1	312

5.5.4.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	312

5.5.4.3	Scheduling availability of UE performing measurements on FR2	312

5.5.5	Applicability conditions for SDT	313

5.5D	Configured Grant based Small Data Transmissions (CG-SDT) for ATG	313

5.6	NR measurements for positioning	313

5.6.1	Introduction	313

5.6.1A	Cell re-selection for positioning	314

5.6.1A.1	Measurement and evaluation of serving cell	314

5.6.1A.2	Measurements of intra-frequency NR cells	315

5.6.2	RSTD measurements	316

5.6.2.1	Introduction	316

5.6.2.2	Requirements Applicability	316

5.6.2.3	Measurement Capability	316

5.6.2.5	Measurements Period Requirements	317

5.6.2.6	Measurements Period Requirements with Bandwidth Aggregation	320

5.6.3	PRS-RSRP measurements	323

5.6.3.1	Introduction	323

5.6.3.2	Requirements applicability	323

5.6.3.3	Measurement Capability	323

5.6.3.4	Measurement Reporting Requirements	323

5.6.3.5	Measurement Period Requirements	324

5.6.4	UE Rx-Tx time difference measurements	327

5.6.4.1	Introduction	327

5.6.4.2	Requirements Applicability	327

5.6.4.3	Measurement Capability	327

5.6.4.4	Measurement Reporting Requirements	327

5.6.4.5	Measurement Period Requirements	327

5.6.4.6	Measurement Period Requirements with Bandwidth Aggregation	331

5.6.5	PRS-RSRPP measurements	334

5.6.5.1	Introduction	334

5.6.5.2	Requirements Applicability	334

5.6.5.3	Measurement Capability	334

5.6.5.4	Measurement Reporting Requirements	334

5.6.5.5	Measurement Period Requirements	335

5.6.6	TA validation requirements for positioning	335

5.6.6.1	Introduction	335

5.6.6.2	TA validation requirements	335

5.6.6.3	TA validation requirements when configured with validity area	336

5.6.7	Measurement requirements for DL RSCPD reported with RSTD	337

5.6.7.1	Introduction	337

5.6.7.2	Requirements Applicability	337

5.6.7.3	Measurement Capability	337

5.6.7.4	Measurement Reporting Requirements	337

5.6.7.5	Measurements Period Requirements	337

5.6.8	Measurement requirements for DL RSCP reported with UE Rx-Tx time difference	340

5.6.8.1	Introduction	340

5.6.8.2	Requirements Applicability	340

5.6.8.3	Measurement Capability	340

5.6.8.4	Measurement Reporting Requirements	340

5.6.8.5	Measurement Period Requirements	341

5.6A	NR measurements for positioning for RedCap	343

5.6A.1	Introduction	343

5.6A.2	Cell re-selection for positioning	344

5.6A.2.1	Measurement and evaluation of serving cell	344

5.6A.2.2	Measurements of intra-frequency NR cells	345

5.6A.3	TA validation requirements for positioning SRS	346

5.6A.3.1	Introduction	346

5.6A.3.2	TA validation requirements	346

5.6A.3.3	TA validation requirements when configured with validity area	346

5.6A.4	RSTD measurements for RedCap	346

5.6A.4.1	 Introduction	346

5.6A.4.2	Requirements applicability	346

5.6A.4.3	Measurement Capability	347

5.6A.4.4	Measurement Reporting Requirements	347

5.6A.4.5	Measurement Period Requirement without RX FH	347

5.6A.4.6	Measurement Period Requirement with RX FH	351

5.6A.5	PRS-RSRP measurements for RedCap	352

5.6A.5.1	Introduction	352

5.6A.5.2	Requirements applicability	353

5.6A.5.3	Measurement Capability	353

5.6A.5.4	Measurement Reporting Requirements	353

5.6A.5.5	Measurement Period Requirements without RX FH	354

5.6A.5.6	Measurement Period Requirement with RX FH	356

5.6A.6	UE Rx-Tx time difference measurements for RedCap	358

5.6A.6.1	Introduction	358

5.6A.6.2	Requirements Applicability	358

5.6A.6.3	Measurement Capability	359

5.6A.6.4	Measurement Reporting Requirements	359

5.6A.6.5	Measurement Period Requirements without RX FH	359

5.6A.6.6	Measurement Period Requirements with RX FH	360

5.6A.7	PRS-RSRPP measurements for RedCap	362

5.6A.7.1	 Introduction	362

5.6A.7.2	Requirements applicability	362

5.6A.7.3	Measurement Capability	362

5.6A.7.4	Measurement Reporting Requirements	362

5.6A.7.5	Measurement Period Requirements without FH	363

5.6A.7.6	Measurement period requirement with FH	363

5.7	Random access based Small Data Transmissions (RA-SDT)	363

5.7.1	Introduction	363

5.7.2	Requirements for small data transmissions based on 2-step RA	363

5.7.3	Requirements for small data transmissions based on 4-step RA	363

5.7.4	Applicability conditions for SDT	363

5.7B	Random access based Small Data Transmissions (RA-SDT) for RedCap	364

5.7B.1	Introduction	364

5.7B.2	Requirements for small data transmissions based on 2-step RA	364

5.7B.3	Requirements for small data transmissions based on 4-step RA	364

5.7B.4	Applicability conditions for RA-SDT for RedCap	364

5.7D	Random access based Small Data Transmissions (RA-SDT) for ATG	364

5.8	Measurement report for fast CA/DC setup	364

5.8.1	Introduction	364

5.8.2	Void	365

5.8.3	Measurement Report Requirements	365

6	RRC_CONNECTED state mobility	365

6.1	Handover	365

6.1.1	NR Handover	365

6.1.1.1	Introduction	365

6.1.1.2	NR FR1 - NR FR1 Handover	365

6.1.1.2.1	Handover delay	365

6.1.1.2.2	Interruption time	365

6.1.1.3	NR FR2- NR FR1 Handover	367

6.1.1.3.1	Handover delay	367

6.1.1.3.2	Interruption time	367

6.1.1.4	NR FR2- NR FR2 Handover	368

6.1.1.4.1	Handover delay	368

6.1.1.4.2	Interruption time	368

6.1.1.5	NR FR1- NR FR2 Handover	369

6.1.1.5.1	Handover delay	369

6.1.1.5.2	Interruption time	369

6.1.2	NR Handover to other RATs	371

6.1.2.1	NR – E-UTRAN Handover	371

6.1.2.1.1	Introduction	371

6.1.2.1.2	Handover delay	371

6.1.2.1.3	Interruption time	371

6.1.2.2	NR – UTRAN Handover	371

6.1.2.2.1	Introduction	371

6.1.2.2.2	Handover delay	372

6.1.2.2.3	Interruption time	372

6.1.3	NR DAPS Handover	372

6.1.3.1	Introduction	372

6.1.3.2	NR FR1 - NR FR1 DAPS Handover	373

6.1.3.2.1	DAPS handover delay	373

6.1.3.2.2	Interruption time	374

6.1.3.3	NR FR2- NR FR1 DAPS Handover	375

6.1.3.3.1	DAPS handover delay	376

6.1.3.3.2	Interruption time	376

6.1.3.4	NR FR1- NR FR2 DAPS Handover	376

6.1.3.4.1	DAPS handover delay	377

6.1.3.4.2	Interruption time	377

6.1.4	NR Conditional Handover	378

6.1.4.1	Introduction	378

6.1.4.2	NR FR1 – NR FR1 conditional handover	378

6.1.4.2.2	Measurement time	378

6.1.4.3	NR FR2 – NR FR1 conditional handover	380

6.1.4.4	NR FR2 – NR FR2 conditional handover	380

6.1.4.4.1	Handover delay	380

6.1.4.4.2	Measurement time	381

6.1.4.4.3	Preparation time	382

6.1.4.4.4	Interruption time	382

6.1.4.5	NR FR1 – NR FR2 conditional handover	382

6.1.5	NR Handover with PSCell	382

6.1.5.1	Introduction	382

6.1.5.2	Handover with PSCell from NR SA to EN-DC	383

6.1.5.2.1	Interruption time for inter-RAT HO from NR to E-UTRAN	383

6.1.5.2.2	PSCell addition in HO with PSCell for NR SA to EN-DC	383

6.1.5.3	HO with PSCell from NE-DC to NE-DC	384

6.1.5.3.1	Handover delay	384

6.1.5.3.2	HO with PSCell - PCell Interruption time	384

6.1.5.3.3	PSCell addition/change in NE-DC to NE-DC HO with PSCell	384

6.1.5.4	HO with PSCell from NR-DC to NR-DC	385

6.1.5.5	Handover with PSCell from NR SA to EN-DC with PSCell using CCA	386

6.1.5.5.1	Introduction	386

6.1.5.5.2	NR SA to EN-DC HO with PSCell- NR to E-UTRA HO Interruption time	386

6.1.5.5.3	NR SA to EN-DC HO with PSCell - NR PSCell Addition Delay requirements	387

6.1.6.1	Conditional handover including target MCG in FR1 and target SCG in FR1 in NR-DC	388

6.1.6.1.1	CHO with PSCell – PCell Interruption time	388

6.1.6.1.2	CHO with PSCell – PSCell change delay	389

6.1.6.2	Conditional handover including target MCG in FR1 and target SCG in FR2 in NR-DC	389

6.1.6.2.2	CHO with PSCell – PSCell change delay	390

6.1.7.1.2	PSCell conditional change delay	393

6.1.7.2	Conditional handover including target MCG in FR1 and Candidate SCG for CPC in FR2 in NR-DC	394

6.1.7.2.1	PCell handover delay	394

6.1.7.2.1.1	Measurement time	395

6.1.7.2.2	PSCell conditional change delay	395

6.1A	Void	397

6.1A.1	Void	397

6.1A.1.1	Void	397

6.1A.1.2	Void	397

6.1A.1.2.1	Void	397

6.1A.1.2.2	Void	397

6.1B	Handover to target cell using CCA	397

6.1B.1	NR Handover	397

6.1B.1.1	Introduction	397

6.1B.1.2	NR FR1 - NR FR1 Handover	397

6.1B.1.2.1	Handover delay	397

6.1B.1.2.2	Interruption time	397

6.1B.1.3	NR FR2-2 NR FR2-2 Handover	398

6.1B.1.3.1	Handover delay	398

6.1B.1.3.2	Interruption time	398

6.1B.1.4	NR FR1- NR FR2-2 Handover	399

6.1B.1.4.1	Handover delay	399

6.1B.1.4.2	Interruption time	400

6.1C	Handover for SAN	401

6.1C.1	NR SAN Handover	401

6.1C.1.1	Introduction	401

6.1C.1.2	NR SAN FR1 – NR SAN FR1 Handover	401

6.1C.1.2.1	Handover delay	401

6.1C.1.2.2	Interruption time	401

6.1C.1.3	NR SAN FR2-NTN – NR SAN FR2-NTN Handover	402

6.1C.1.3.1	Handover delay	402

6.1C.1.3.2	Interruption time	403

6.1C.2	NR SAN Conditional Handover	403

6.1C.2.1	Introduction	403

6.1C.2.2	NR SAN FR1 – NR SAN FR1 conditional handover	403

6.1C.2.2.1	Handover delay	404

6.1C.2.2.2	Measurement time	404

6.1C.2.2.3	Preparation time	405

6.1C.2.2.4	Interruption time	405

6.1C.2.3	NR SAN FR1 – NR SAN FR1 conditional handover without L3 measurement criteria	406

6.1C.2.3.1	Handover delay	406

6.1C.2.3.2	Preparation time	407

6.1C.2.3.3	Interruption time	407

6.1C.2.4	NR SAN FR2-NTN – NR SAN FR2-NTN conditional handover	407

6.1C.3	NR SAN Satellite switching with re-synchronization	408

6.1C.3.1	Introduction	408

6.1C.3.2	NR SAN FR1 – NR SAN FR1 Satellite switching with re-synchronization	408

6.1C.3.2.1	Satellite switching delay	408

6.1C.3.2.2	Interruption time for hard satellite switch with re-sync	408

6.1D	Handover for RedCap	409

6.1D.1	NR Handover	409

6.1D.1.1	Introduction	409

6.1D.1.2	NR FR1 - NR FR1 Handover	410

6.1D.1.2.1	Handover delay	410

6.1D.1.2.2	Interruption time	410

6.1D.1.3	NR FR2- NR FR2 Handover	411

6.1D.1.3.1	Handover delay	411

6.1D.1.3.2	Interruption time	412

6.1D.2	NR Handover to other RATs	413

6.1D.2.1	NR – E-UTRAN Handover	413

6.1E	Handover for ATG	413

6.1E.1	NR Handover	413

6.1E.1.1	Introduction	413

6.1E.1.2	NR FR1 - NR FR1 Handover	413

6.1E.1.2.1	Handover delay	413

6.1E.1.2.2	Interruption time	413

6.1E.2	NR Conditional Handover	414

6.1E.2.1	Introduction	414

6.1E.2.2	NR FR1 – NR FR1 conditional handover	414

6.1E.2.2.1	Handover delay	414

6.1E.2.2.2	Measurement time	415

6.1E.2.2.3	Preparation time	415

6.1E.2.2.4	Interruption time	416

6.2	RRC Connection Mobility Control	416

6.2.1	SA: RRC Re-establishment	416

6.2.1.1	Introduction	416

6.2.1.2	Requirements	416

6.2.1.2.1	UE Re-establishment delay requirement	416

6.2.1A	RRC Re-establishment with CCA	418

6.2.1A.1	Introduction	418

6.2.1A.2	Requirements	418

6.2.1A.2.1	UE Re-establishment with CCA delay requirement	418

6.2.1B	SA: RRC Re-establishment for RedCap	420

6.2.1B.1	Introduction	420

6.2.1B.2	Requirements	420

6.2.2	Random access	421

6.2.2.1	Introduction	421

6.2.2.2	Requirements for 4-step RA type	421

6.2.2.2.1	Contention based random access	421

6.2.2.2.1.1	Correct behaviour when transmitting Random Access Preamble	421

6.2.2.2.1.2	Correct behaviour when receiving Random Access Response	422

6.2.2.2.1.3	Correct behaviour when not receiving Random Access Response	422

6.2.2.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	422

6.2.2.2.1.5	SA: Correct behaviour when receiving a message over Temporary C-RNTI	422

6.2.2.2.1.6	Correct behaviour when contention Resolution timer expires	422

6.2.2.2.2	Non-Contention based random access	422

6.2.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	422

6.2.2.2.2.2	Correct behaviour when receiving Random Access Response	423

6.2.2.2.2.3	Correct behaviour when not receiving Random Access Response	423

6.2.2.2.3	UE behaviour when configured with supplementary UL	423

6.2.2.3	Requirements for 2-step RA type	423

6.2.2.3.1	Contention based random access	424

6.2.2.3.1.1	Correct behaviour when transmitting MsgA	424

6.2.2.3.1.2	Correct behaviour when receiving MsgB	424

6.2.2.3.1.3	Correct behaviour when not receiving MsgB	424

6.2.2.3.2	Non-Contention based random access	424

6.2.2.3.2.1	Correct behaviour when transmitting MsgA	424

6.2.2.3.2.2	Correct behaviour when receiving MsgB	425

6.2.2.3.2.3	Correct behaviour when not receiving MsgB	425

6.2.2.3.3	UE behaviour when configured with supplementary UL	425

6.2.2A	Random access when CCA is used on target frequency	425

6.2.2A.1	Introduction	425

6.2.2A.2	Requirements for 4-step RA type	425

6.2.2A.2.1	Contention based random access	426

6.2.2A.2.1.1	Correct behaviour when transmitting Random Access Preamble	426

6.2.2A.2.1.2	Correct behaviour when receiving Random Access Response	426

6.2.2A.2.1.3	Correct behaviour when not receiving Random Access Response	426

6.2.2A.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	426

6.2.2A.2.1.6	Correct behaviour when contention Resolution timer expires	427

6.2.2A.2.2	Non-Contention based random access	427

6.2.2A.2.2.1	Correct behaviour when transmitting Random Access Preamble	427

6.2.2A.2.2.2	Correct behaviour when receiving Random Access Response	427

6.2.2A.2.2.3	Correct behaviour when not receiving Random Access Response	428

6.2.2A.3	Requirements for 2-step RA type	428

6.2.2A.3.1	Contention based random access	428

6.2.2A.3.1.1	Correct behaviour when transmitting MsgA	428

6.2.2A.3.1.2	Correct behaviour when receiving MsgB	429

6.2.2A.3.1.3	Correct behaviour when not receiving MsgB	429

6.2.2A.3.2	Non-Contention based random access	429

6.2.2A.3.2.1	Correct behaviour when transmitting MsgA	429

6.2.2A.3.2.2	Correct behaviour when receiving MsgB	430

6.2.2A.3.2.3	Correct behaviour when not receiving MsgB	430

6.2.2B	Random access for RedCap	430

6.2.2B.1	Introduction	430

6.2.2B.2	Requirements	430

6.2.2C	PDCCH ordered Random Access for LTM	431

6.2.2C.1	Introduction	431

6.2.2C.2	PDCCH ordered Random Access delay	431

6.2.3	SA: RRC Connection Release with Redirection	432

6.2.3.1	Introduction	432

6.2.3.2	Requirements	432

6.2.3.2.1	RRC connection release with redirection to NR	432

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	433

6.2.3.2.3	RRC connection release with redirection to NR carrier subject to CCA	433

6.2.3A	SA: RRC Connection Release with Redirection for RedCap	434

6.2.3A.1	Introduction	434

6.2.3A.2	Requirements	435

6.2.3A.2.1	RRC connection release with redirection to NR	435

6.2.3A.2.2	RRC connection release with redirection to E-UTRAN	435

6.2C	RRC Connection Mobility Control for Satellite Access	435

6.2C.1	SA: RRC Re-establishment for Satellite Access	435

6.2C.1.1	Introduction	435

6.2C.1.2	Requirements	435

6.2C.1.2.1	UE Re-establishment delay requirement	436

6.2C.1.2.2	UE Re-establishment delay requirement for VSAT	437

6.2C.2	Random access for satellite access	437

6.2C.2.1	Introduction	437

6.2C.2.2	Requirements for 4-step RA type	437

6.2C.2.2.1	Contention based random access	438

6.2C.2.2.1.1	Correct behaviour when transmitting Random Access Preamble	438

6.2C.2.2.1.2	Correct behaviour when receiving Random Access Response	438

6.2C.2.2.1.3	Correct behaviour when not receiving Random Access Response	438

6.2C.2.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	438

6.2C.2.2.1.5	SA: Correct behaviour when receiving a message over Temporary C-RNTI	438

6.2C.2.2.1.6	Correct behaviour when Contention Resolution Timer expires	438

6.2C.2.2.2	Non-Contention based random access	439

6.2C.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	439

6.2C.2.2.2.2	Correct behaviour when receiving Random Access Response	439

6.2C.2.2.2.3	Correct behaviour when not receiving Random Access Response	439

6.2C.2.3	Requirements for 2-step RA type	440

6.2C.2.3.1	Contention based random access	440

6.2C.2.3.1.1	Correct behaviour when transmitting MsgA	440

6.2C.2.3.1.2	Correct behaviour when receiving MsgB	440

6.2C.2.3.1.3	Correct behaviour when not receiving MsgB	441

6.2C.2.3.2	Non-Contention based random access	441

6.2C.2.3.2.1	Correct behaviour when transmitting MsgA	441

6.2C.2.3.2.2	Correct behaviour when receiving MsgB	441

6.2C.2.3.2.3	Correct behaviour when not receiving MsgB	441

6.2C.3	SA: RRC Connection Release with Redirection for Satellite Access	441

6.2C.3.1	Introduction	441

6.2C.3.2	Requirements	441

6.2C.3.2.1	RRC connection release with redirection to NR	441

6.2D	RRC Connection Mobility Control for ATG	442

6.2D.1	SA: RRC Re-establishment	442

6.2D.1.1	Introduction	442

6.2D.1.2	Requirements	443

6.2D.1.2.1	UE Re-establishment delay requirement	443

6.2D.2	Random access	444

6.2D.2.1	Introduction	444

6.2D.2.2	Requirements for 4-step RA type	444

6.2D.2.3	Requirements for 2-step RA type	444

6.2D.3	SA: RRC Connection Release with Redirection	444

6.2D.3.1	Introduction	444

6.2D.3.2	Requirements	445

6.2D.3.2.1	RRC connection release with redirection to NR	445

6.3	L1/L2-Triggered Mobility	445

6.3.1	LTM PCell Cell Switch	445

6.3.1.1	Introduction	445

6.3.1.2	LTM Cell Switch delay	447

6.3.1.3	Interruption time	447

7	Timing	449

7.1	UE transmit timing	449

7.1.1	Introduction	449

7.1.2	Requirements	450

7.1.2.1	Gradual timing adjustment	452

7.1.2.2	Void	452

7.1.2.3	One shot large UL timing adjustment for FR2 Power Class 6 UE	452

7.1.2.4	UE transmit timing for positioning measurements	453

7.1A	UE transmit timing for RedCap	453

7.1A.1	Introduction	453

7.1A.2	Requirements	454

7.1A.2.1	Gradual timing adjustment	455

7.1A.2.2	UE transmit timing for positioning measurements	455

7.1C	UE transmit timing for Satellite Access	455

7.1C.1	Introduction	455

7.1C.2	Requirements	455

7.1C.2.1	Gradual timing adjustment	457

7.1D	UE transmit timing for ATG	457

7.1D.1	Introduction	457

7.1D.2	Requirements	457

7.1D.2.1	Gradual timing adjustment	458

7.2	UE timer accuracy	459

7.2.1	Introduction	459

7.2.2	Requirements	459

7.2A	UE timer accuracy for RedCap	459

7.2A.1	Introduction	459

7.2A.2	Requirements	459

7.2C	UE timer accuracy for satellite access	459

7.2C.1	Introduction	459

7.2C.2	Requirements	460

7.2D	UE timer accuracy for ATG	460

7.2D.1	Introduction	460

7.2D.2	Requirements	460

7.3	Timing advance	460

7.3.1	Introduction	460

7.3.2	Requirements	461

7.3.2.1	Timing Advance adjustment delay	461

7.3.2.2	Timing Advance adjustment accuracy	461

7.3A	Timing Advance for RedCap	461

7.3A.1	Introduction	461

7.3A.2	Requirements	461

7.3A.2.1	Timing Advance adjustment delay	461

7.3A.2.2	Timing Advance adjustment accuracy	461

7.3C	Timing advance for satellite access	461

7.3C.1	Introduction	461

7.3C.2	Requirements	462

7.3C.2.1	Timing Advance adjustment delay	462

7.3C.2.2	Timing Advance adjustment accuracy	462

7.3D	Timing advance for ATG	462

7.3D.1	Introduction	462

7.3D.2	Requirements	462

7.3D.2.1	Timing Advance adjustment delay	462

7.3D.2.2	Timing Advance adjustment accuracy	462

7.4	Cell phase synchronization accuracy	463

7.4.1	Definition	463

7.4.2	Minimum requirements	463

7.5	Maximum Transmission Timing Difference	463

7.5.1	Introduction	463

7.5.2	Minimum requirements for inter-band EN-DC	463

7.5.2.1	Minimum requirements for inter-band synchronous EN-DC	464

7.5.3	Minimum requirements for intra-band EN-DC	464

7.5.4	Minimum requirements for NR Carrier Aggregation	465

7.5.5	Minimum requirements for inter-band NE-DC	465

7.5.5.1	Minimum requirements for inter-band synchronous NE-DC	466

7.5.6	Minimum requirements for inter-band NR-DC	466

7.5.7	Minimum requirements for multi-TRP	467

7.6	Maximum Receive Timing Difference	467

7.6.1	Introduction	467

7.6.2	Minimum requirements for inter-band EN-DC	468

7.6.2.1	Minimum requirements for inter-band synchronous EN-DC	468

7.6.3	Minimum requirements for intra-band EN-DC	469

7.6.4	Minimum requirements for NR Carrier Aggregation	469

7.6.5	Minimum requirements for inter-band NE-DC	470

7.6.5.1	Minimum requirements for inter-band synchronous NE-DC	470

7.6.6	Minimum requirements for inter-band NR-DC	471

7.6.7	Minimum requirements for PC6 UE in FR2	472

7.6.8	Minimum requirements for Multi-TRPs	472

7.7	deriveSSB-IndexFromCell tolerance	473

7.7.1	Minimum requirements	473

7.7A	deriveSSB-IndexFromCell tolerance for RedCap	473

7.7A.1	Minimum requirements	473

7.7D	DeriveSSB-IndexFromCell tolerance for ATG	473

7.7D.1	Minimum requirements	473

7.8	Void	474

7.9	deriveSSB-IndexFromCellInter-r17 tolerance	474

7.9.1	Minimum requirements	474

7.9D	DeriveSSB-IndexFromCellInter-r17 tolerance for ATG	474

7.9D.1	Minimum requirements	474

8	Signalling characteristics	503

8.1	Radio Link Monitoring	503

8.1.1	Introduction	503

8.1.1.1	Introduction of Requirement on Radio Link Monitoring for UE Configured with Relaxed Measurement Criteria	504

8.1.2	Requirements for SSB based radio link monitoring	505

8.1.2.1	Introduction	505

8.1.2.2	Minimum requirement	506

8.1.2.3	Measurement restrictions for SSB based RLM	510

8.1.2.4	Minimum requirement of SSB based radio link monitoring for UE fulfilling relaxed measurement criteria	511

8.1.3	Requirements for CSI-RS based radio link monitoring	512

8.1.3.1	Introduction	512

8.1.3.2	Minimum requirement	512

8.1.3.3	Measurement restrictions for CSI-RS based RLM	516

8.1.3.4	Minimum requirement of CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria	517

8.1.4	Minimum requirement at transitions	518

8.1.5	Minimum requirement for UE turning off the transmitter	519

8.1.6	Minimum requirement for L1 indication	519

8.1.7	Scheduling availability of UE during radio link monitoring	519

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	519

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	519

8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2	520

8.1.7.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	521

8.1.8	Minimum requirement under IDC Interference	521

8.1A	Radio Link Monitoring with CCA on Target Frequency	521

8.1A.1	Introduction	521

8.1A.2	Requirements for SSB Based Radio Link Monitoring	522

8.1A.2.1	Introduction	522

8.1A.2.2	Minimum Requirement	523

8.1A.3	Minimum requirement at transitions	526

8.1A.4	Minimum requirement for UE turning off the transmitter	526

8.1A.5	Minimum requirement for L1 indication	526

8.1A.6	Scheduling availability of UE during radio link monitoring	527

8.1A.6.3	Scheduling availability of UE performing radio link monitoring on FR2-2	527

8.1A.6.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	528

8.1B	Radio Link Monitoring for RedCap	528

8.1B.1	Introduction	528

8.1B.2	Requirements for SSB based radio link monitoring	529

8.1B.2.1	Introduction	529

8.1B.2.2	Minimum requirement	529

8.1B.2.3	Measurement restrictions for SSB based RLM	531

8.1B.3	Requirements for CSI-RS based radio link monitoring	532

8.1B.3.1	Introduction	532

8.1B.3.2	Minimum requirement	532

8.1B.3.3	Measurement restrictions for CSI-RS based RLM	535

8.1B.4	Minimum requirement at transitions	535

8.1B.5	Minimum requirement for UE turning off the transmitter	536

8.1B.6	Minimum requirement for L1 indication	536

8.1B.7	Scheduling availability of UE during radio link monitoring	536

8.1B.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	536

8.1B.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	536

8.1B.7.3	Scheduling availability of UE performing radio link monitoring on FR2	537

8.1C	Radio Link Monitoring for Satellite Access	537

8.1C.1	Introduction	537

8.1C.2	Requirements for SSB based radio link monitoring	538

8.1C.2.1	Introduction	538

8.1C.2.2	Minimum requirement	539

8.1C.2.3	Measurement restrictions for SSB based RLM	540

8.1C.3	Requirements for CSI-RS based radio link monitoring	540

8.1C.3.1	Introduction	540

8.1C.3.2	Minimum requirement	541

8.1C.3.3	Measurement restrictions for CSI-RS based RLM	542

8.1C.4	Minimum requirement at transitions	543

8.1C.5	Minimum requirement for UE turning off the transmitter	543

8.1C.6	Minimum requirement for L1 indication	543

8.1C.7	Scheduling availability of UE during radio link monitoring	544

8.1C.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1-NTN and FR2-NTN	544

8.1C.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1-NTN and FR2-NTN	544

8.1D	Radio Link Monitoring for ATG	544

8.1D.1	Introduction	544

8.1D.2	Requirements for SSB based radio link monitoring	545

8.1D.2.1	Introduction	545

8.1D.2.2	Minimum requirement	545

8.1D.2.3	Measurement restrictions for SSB based RLM	546

8.1D.3	Requirements for CSI-RS based radio link monitoring	547

8.1D.3.1	Introduction	547

8.1D.3.2	Minimum requirement	547

8.1D.3.3	Measurement restrictions for CSI-RS based RLM	548

8.1D.4	Minimum requirement at transitions	548

8.1D.5	Minimum requirement for UE turning off the transmitter	548

8.1D.6	Minimum requirement for L1 indication	548

8.1D.7	Scheduling availability of UE during radio link monitoring	549

8.1D.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	549

8.1D.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	549

8.2	Interruption	549

8.2.1	EN-DC Interruption	549

8.2.1.1	Introduction	549

8.2.1.2	Requirements	550

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	550

8.2.1.2.2	Interruptions at transitions from non-DRX to DRX	550

8.2.1.2.3	Interruptions at SCell addition/release	550

8.2.1.2.4	Interruptions at SCell activation/deactivation	552

8.2.1.2.5	Interruptions during measurements on SCC	554

8.2.1.2.5.1	Interruptions during measurements on deactivated NR SCC	554

8.2.1.2.5.2	Interruptions during measurements on deactivated E-UTRAN SCC	554

8.2.1.2.5.3	Interruptions during CQI measurements on dormant E-UTRAN SCell	554

8.2.1.2.5.4	Interruptions during RRM measurements on dormant E-UTRAN SCC	555

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	555

8.2.1.2.7	Interruptions due to Active BWP switching Requirement	556

8.2.1.2.8	Interruptions at direct SCell activation and hibernation	557

8.2.1.2.8.1	Interruptions during direct SCell activation and hibernation of E-UTRA SCell	557

8.2.1.2.8.2	Interruptions during direct SCell activation	557

8.2.1.2.9	Interruptions at SCell hibernation	557

8.2.1.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	558

8.2.1.2.11	Interruptions due to UE-specific CBW change	558

8.2.1.2.12	Interruptions at NR SRS carrier based switching	558

8.2.1.2.13	Interruptions at E-UTRA SRS carrier based switching	560

8.2.1.2.14	DL Interruptions at switching between two uplink carriers	561

8.2.1.2.15	Interruptions due to SCell dormancy	561

8.2.1.2.15.1	Interruptions due to SCell dormancy switch	561

8.2.1.2.15.2	Interruptions due to CQI measurements during SCell dormancy	561

8.2.1.2.15.3	Interruptions due to RRM measurements during SCell dormancy	562

8.2.1.2.16	Interruptions when identifying CGI of an NR cell with autonomous gaps	562

8.2.1.2.17	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	562

8.2.1.2.18	Interruptions at NR SRS antenna port switching	563

8.2.1.2.19	Interruptions at fast SCell activation	564

8.2.1.2.20	Interruptions due to PUCCH SCell activation/deactivation	564

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation	565

8.2.2.1	Introduction	565

8.2.2.2	Requirements	566

8.2.2.2.1	Interruptions at SCell addition/release	566

8.2.2.2.2	Interruptions at SCell activation/deactivation	567

8.2.2.2.3	Interruptions during measurements on deactivated SCC	568

8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration	569

8.2.2.2.5	Interruptions due to Active BWP switching Requirement	569

8.2.2.2.6	Interruptions at inter-frequency SFTD measurement	570

8.2.2.2.7	Interruptions at SCell activation/deactivation with multiple downlink SCells	571

8.2.2.2.8	Interruptions due to UE-specific CBW change	572

8.2.2.2.9	Interruptions at NR SRS carrier based switching	572

8.2.2.2.10	DL Interruptions at UE switching between two uplink carriers	573

8.2.2.2.10A	DL Interruptions at UE switching between two uplink carriers with two transmit antenna connectors	574

8.2.2.2.10B	DL Interruptions at UE switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	574

8.2.2.2.10C	DL Interruptions at UE switching between two uplink bands with two transmit antenna connectors	575

8.2.2.2.10D	DL Interruptions at UE switching across three or four uplink bands	575

8.2.2.2.11	Interruptions at direct SCell activation	576

8.2.2.2.12	Interruptions due to SCell dormancy	576

8.2.2.2.12.1	Interruptions due to SCell dormancy switch	576

8.2.2.2.12.2	Interruptions due to CQI measurements during SCell dormancy	576

8.2.2.2.12.3	Interruptions due to RRM measurements during SCell dormancy	577

8.2.2.2.13	Interruptions at transitions between active and non-active during DRX	577

8.2.2.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	577

8.2.2.2.15	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	577

8.2.2.2.16	Interruptions at NR SRS antenna port switching	578

8.2.2.2.17	Interruptions at fast SCell activation	579

8.2.2.2.18	Interruptions due to PUCCH SCell activation/deactivation	579

8.2.2.2.19	Interruptions due to measurements without gap carried out by UE supporting NeedForInterruptionInfoNR	580

8.2.2.2.20	Interruptions due to PDCCH ordered RACH on target LTM cell	581

8.2.2.2.21	Interruptions at NR SRS bandwidth aggregation for positioning	581

8.2.3	NE-DC Interruptions	584

8.2.3.1	Introduction	584

8.2.3.2	Requirements	584

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	584

8.2.3.2.2	Interruptions at transitions from non-DRX to DRX	585

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	585

8.2.3.2.4	Interruptions at SCell activation/deactivation	586

8.2.3.2.5	Interruptions during measurements on SCC	587

8.2.3.2.5.1	Interruptions during measurements on deactivated NR SCC	587

8.2.3.2.5.2	Interruptions during measurements on deactivated E-UTRAN SCC	587

8.2.3.2.5.3	Interruptions during CQI measurements on dormant E-UTRAN SCC	588

8.2.3.2.5.4	Interruptions during RRM measurements on dormant E-UTRAN SCC	588

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	588

8.2.3.2.7	Interruptions due to Active BWP switching Requirement	589

8.2.3.2.8	Interruptions at direct SCell activation and hibernation	589

8.2.3.2.9	Interruptions at SCell hibernation	589

8.2.3.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	590

8.2.3.2.11	Interruptions at NR SRS carrier based switching	590

8.2.3.2.12	Interruptions at E-UTRA SRS carrier based switching	591

8.2.3.2.13	Interruptions due to SCell dormancy	592

8.2.3.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	592

8.2.3.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	593

8.2.3.2.17	Interruptions at fast SCell activation	595

8.2.3.2.18	Interruptions due to UE-specific CBW change	595

8.2.3.2.19	Interruptions due to PUCCH SCell activation/deactivation	595

8.2.4	NR-DC: Interruptions	596

8.2.4.1	Introduction	596

8.2.4.2	Requirements	596

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	596

8.2.4.2.2	Interruptions at SCell activation/deactivation	597

8.2.4.2.3	Interruptions during measurements on SCC	598

8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration	598

8.2.4.2.5	Interruptions due to Active BWP switching Requirement	599

8.2.4.2.6	Interruptions at transitions between active and non-active during DRX	599

8.2.4.2.7	Interruptions at transitions from non-DRX to DRX	599

8.2.4.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	600

8.2.4.2.9	Interruptions at NR SRS carrier based switching	600

8.2.4.2.10	Interruptions at direct SCell activation	601

8.2.4.2.11	Interruptions when identifying CGI of an NR cell with autonomous gaps	602

8.2.4.2.12	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	602

8.2.4.2.13	 Interruptions due to SCell dormancy	603

8.2.4.2.14	Interruptions at NR SRS antenna port switching	603

8.2.4.2.15	Interruptions at fast SCell activation	604

8.2.4.2.16	Interruptions at SCG activation/deactivation	605

8.2.4.2.17	Interruptions due to RRM measurements on deactivated SCG	605

8.2.4.2.18	Interruptions during RLM/BFD measurements on deactivated PSCell	605

8.2.4.2.19	Interruptions due to UE-specific CBW change	605

8.2.4.2.20	Interruptions due to PDCCH ordered RACH on target LTM cell	606

8.2.4.2.21	Interruptions at PSCell Cell switch	606

8.2.4.2A	Void	607

8.2.4.2A.1	Void	607

8.2.4.2A.2	Void	607

8.2.4.2A.3	Void	607

8.3	SCell Activation and Deactivation Delay	607

8.3.1	Introduction	607

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	607

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	614

8.3.4	Direct SCell Activation at SCell addition	614

8.3.5	Direct SCell Activation at Handover	616

8.3.7	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	618

8.3.8	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	622

8.3.9	Direct SCell Activation of Multiple Downlink SCells at SCell addition	622

8.3.10	Direct SCell Activation of Multiple Downlink SCells at Handover	623

8.3.12	SCell Activation Delay Requirement for Deactivated PUCCH SCell	625

8.3.13	SCell activation delay Requirement for Deactivated PUCCH SCell with Multiple SCells	628

8.3.14	SCell Deactivation Delay Requirement for Activated PUCCH SCell	630

8.3.15	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple Downlink SCells	631

8.3.16	Fast SCell Activation Delay Requirement for Deactivated SCell	631

8.3.17	SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation	634

8.3.18	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells with L3 reporting	637

8.3A	SCell Activation and Deactivation Delay in Carriers with CCA	640

8.3A.1	Introduction	640

8.3A.2	SCell Activation Delay Requirement for Deactivated SCell	640

8.3A.3	SCell Deactivation Delay Requirement for Activated SCell	645

8.4	UE UL carrier RRC reconfiguration delay	645

8.4.1	Introduction	645

8.4.2	UE UL carrier configuration delay requirement	645

8.4.3	UE UL carrier deconfiguration delay requirement	645

8.5	Link Recovery Procedures	646

8.5.1	Introduction	646

8.5.1.1	Introduction of Requirement on Link Recovery Procedures for UE configured with relaxed measurement criteria	647

8.5.2	Requirements for SSB based beam failure detection	648

8.5.2.1	Introduction	648

8.5.2.2	Minimum requirement	648

8.5.2.3	Measurement restriction for SSB based beam failure detection	652

8.5.2.4	Minimum requirement of SSB based beam failure detection for UE fulfilling relaxed measurement criteria	653

8.5.3	Requirements for CSI-RS based beam failure detection	654

8.5.3.1	Introduction	654

8.5.3.2	Minimum requirement	654

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection	658

8.5.3.4	Minimum requirement of CSI-RS based beam failure detection for UE fulfilling relaxed measurement criteria	660

8.5.4	Minimum requirement for L1 indication	660

8.5.5	Requirements for SSB based candidate beam detection	661

8.5.5.1	Introduction	661

8.5.5.2	Minimum requirement	661

8.5.5.3	Measurement restriction for SSB based candidate beam detection	664

8.5.6	Requirements for CSI-RS based candidate beam detection	665

8.5.6.1	Introduction	665

8.5.6.2	Minimum requirement	665

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection	669

8.5.7	Scheduling availability of UE during beam failure detection	670

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	670

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	670

8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2	670

8.5.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	671

8.5.8	Scheduling availability of UE during candidate beam detection	671

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	671

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	672

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	672

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	673

8.5.9	Requirements for Beam Failure Recovery in SCell	673

8.5.9.1	Introduction	673

8.5.9.2	Requirement	673

8.5.10	Minimum requirement at transitions for beam failure detection	673

8.5.11	Minimum requirement under IDC Interference	674

8.5A	Link Recovery Procedures when CCA is used on target frequency	674

8.5A.1	Introduction	674

8.5A.2	Requirements for SSB based beam failure detection	675

8.5A.2.1	Introduction	675

8.5A.2.2	Minimum requirement	675

8.5A.2.3	Measurement restriction for SSB based beam failure detection	677

8.5A.3	Void	678

8.5A.4	Minimum requirement for L1 indication	678

8.5A.5	Requirements for SSB based candidate beam detection	678

8.5A.5.1	Introduction	678

8.5A.5.2	Minimum requirement	678

8.5A.5.3	Measurement restriction for SSB based candidate beam detection	680

8.5A.6	Void	681

8.5A.7	Scheduling availability of UE during beam failure detection	681

8.5A.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH	681

8.5A.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH	681

8.5A.7.3	Scheduling availability of UE performing beam failure detection on FR2-2	681

8.5A.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	681

8.5A.8	Scheduling availability of UE during candidate beam detection	681

8.5A.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	682

8.5.8A.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	682

8.5B	Link Recovery Procedures for Redcap	682

8.5B.1	Introduction	682

8.5B.2	Requirements for SSB based beam failure detection for Redcap	682

8.5B.2.1	Introduction	682

8.5B.2.2	Minimum requirement	683

8.5B.2.3	Measurement restriction for SSB based beam failure detection	684

8.5B.3	Requirements for CSI-RS based beam failure detection for Redcap	685

8.5B.3.1	Introduction	685

8.5B.3.2	Minimum requirement	685

8.5B.3.3	Measurement restrictions for CSI-RS beam failure detection	687

8.5B.4	Minimum requirement for L1 indication for Redcap	688

8.5B.5	Requirements for SSB based candidate beam detection for Redcap	688

8.5B.5.1	Introduction	688

8.5B.5.2	Minimum requirement	688

8.5B.5.3	Measurement restriction for SSB based candidate beam detection	690

8.5B.6	Requirements for CSI-RS based candidate beam detection for Redcap	690

8.5B.6.1	Introduction	690

8.5B.6.2	Minimum requirement	691

8.5B.6.3	Measurement restriction for CSI-RS based candidate beam detection	693

8.5B.7	Scheduling availability of UE during beam failure detection for Redcap	693

8.5B.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	693

8.5B.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	693

8.5B.7.3	Scheduling availability of UE performing beam failure detection on FR2	693

8.5B.8	Scheduling availability of UE during candidate beam detection for Redcap	694

8.5B.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	694

8.5B.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	694

8.5B.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	694

8.5B.9	Minimum requirement at transitions for beam failure detection for Redcap	695

8.5C	Link Recovery Procedures for Satellite Access	695

8.5C.1	Introduction	695

8.5C.2	Requirements for SSB based beam failure detection	695

8.5C.2.1	Introduction	695

8.5C.2.2	Minimum requirement	696

8.5C.2.3	Measurement restriction for SSB based beam failure detection	697

8.5C.3	Requirements for CSI-RS based beam failure detection	697

8.5C.3.1	Introduction	697

8.5C.3.2	Minimum requirement	697

8.5C.3.3	Measurement restrictions for CSI-RS beam failure detection	698

8.5C.4	Minimum requirement for L1 indication	699

8.5C.5	Requirements for SSB based candidate beam detection	699

8.5C.5.1	Introduction	699

8.5C.5.2	Minimum requirement	699

8.5C.5.3	Measurement restriction for SSB based candidate beam detection	700

8.5C.6	Requirements for CSI-RS based candidate beam detection	701

8.5C.6.1	Introduction	701

8.5C.6.2	Minimum requirement	701

8.5C.6.3	Measurement restriction for CSI-RS based candidate beam detection	702

8.5C.7	Scheduling availability of UE during beam failure detection	702

8.5C.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1-NTN	702

8.5C.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1-NTN	702

8.5C.8	Scheduling availability of UE during candidate beam detection	702

8.5C.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1-NTN	703

8.5C.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1-NTN	703

8.5C.9	Minimum requirement at transitions for beam failure detection	703

8.5D	Link Recovery Procedures for ATG	703

8.5D.1	Introduction	703

8.5D.2	Requirements for SSB based beam failure detection	704

8.5D.2.1	Introduction	704

8.5D.2.2	Minimum requirement	704

8.5D.2.3	Measurement restriction for SSB based beam failure detection	705

8.5D.3	Requirements for CSI-RS based beam failure detection	705

8.5D.3.1	Introduction	705

8.5D.3.2	Minimum requirement	705

8.5D.3.3	Measurement restrictions for CSI-RS beam failure detection	706

8.5D.4	Minimum requirement for L1 indication	707

8.5D.5	Requirements for SSB based candidate beam detection	707

8.5D.5.1	Introduction	707

8.5D.5.2	Minimum requirement	707

8.5D.5.3	Measurement restriction for SSB based candidate beam detection	708

8.5D.6	Requirements for CSI-RS based candidate beam detection	708

8.5D.6.1	Introduction	708

8.5D.6.2	Minimum requirement	708

8.5D.6.3	Measurement restriction for CSI-RS based candidate beam detection	709

8.5D.7	Scheduling availability of UE during beam failure detection	709

8.5D.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	710

8.5D.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	710

8.5D.8	Scheduling availability of UE during candidate beam detection	710

8.5D.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	710

8.5D.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	710

8.5D.9	Minimum requirement at transitions for beam failure detection	710

8.6	Active BWP switch delay	710

8.6.1	Introduction	710

8.6.2	DCI and timer based BWP switch delay on a single CC	711

8.6.2A	DCI based BWP switch delay on multiple CCs	712

8.6.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	712

8.6.2A.2	Non-simultaneous DCI based BWP switch delay on multiple CCs	714

8.6.2B	Timer based BWP switch delay on multiple CCs	714

8.6.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	714

8.6.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	714

8.6.3	RRC based BWP switch delay on a single CC	715

8.6.3A	RRC based BWP switch delay on multiple CCs	716

8.6.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	716

8.6.3A.2	Non-simultaneous RRC based BWP switch delay on multiple CCs	716

8.6.4	BWP switch delay on Consistent UL CCA recovery	717

8.6A	Active BWP switch delay for RedCap	717

8.6A.1	Introduction	717

8.6A.2	DCI and timer based BWP switch delay on a single CC	717

8.6A.3	RRC based BWP switch delay on a single CC	719

8.6C	Active BWP switch delay for satellite access	719

8.6C.1	Introduction	719

8.6C.2	DCI and timer based BWP switch delay on a single CC	719

8.6C.3	RRC based BWP switch delay on a single CC	721

8.6D	Active BWP switch delay for ATG	721

8.6D.1	Introduction	721

8.6D.2	DCI and timer based BWP switch delay	721

8.6D.3	RRC based BWP switch delay on a single CC	722

8.7	Void	723

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	723

8.8.1	Introduction	723

8.8.2	E-UTRAN PSCell Addition Delay Requirement	723

8.8.3	E-UTRAN PSCell Release Delay Requirement	723

8.9	NR-DC: PSCell Addition and Release Delay	724

8.9.1	Introduction	724

8.9.2	PSCell Addition Delay Requirement	724

8.9.3	PSCell Release Delay Requirement	725

8.9A	Conditional PSCell Addition Delay	725

8.9A.1	Introduction	725

8.9A.2	Conditional PSCell Addition Delay Requirement	725

8.9A.2.1	Measurement time	726

8.9B	NR-DC: PSCell Addition and Release Delay in Carriers with CCA	726

8.9B.1	Introduction	726

8.9B.2	PSCell Addition Delay Requirement	726

8.9B.3	PSCell Release Delay Requirement	727

8.9C	Subsequent Conditional PSCell Addition Delay	727

8.9C.1	Introduction	727

8.9C.2	Subsequent Conditional PSCell Addition Delay Requirement	727

8.9C.2.1	Measurement time	728

8.10	Active TCI state switching delay	728

8.10.3A	MAC-CE based TCI state switch delay in HST FR2 scenarios	729

8.10.4	DCI based TCI state switch delay	730

8.10.5	RRC based TCI state switch delay	730

8.10.6	Active TCI state list update delay	731

8.10A	Active TCI state switching delay with CCA	731

8.10A.1	Introduction	731

8.10A.2	Known conditions for TCI state	731

8.10A.3	MAC-CE based TCI state switch delay	731

8.10A.4	DCI based TCI state switch delay	732

8.10A.5	RRC based TCI state switch delay	733

8.10A.6	Active TCI state list update delay	733

8.10B	Active TCI state switching delay for RedCap	734

8.10B.1	Introduction	734

8.10B.2	Known conditions for TCI state	734

8.10B.3	MAC-CE based TCI state switch delay	734

8.10B.4	DCI based TCI state switch delay	735

8.10B.5	RRC based TCI state switch delay	735

8.10B.6	Active TCI state list update delay	736

8.10C	Active TCI state switching delay for satellite access	736

8.10C.1	Introduction	736

8.10C.2	MAC-CE based TCI state switch delay	736

8.10C.4	DCI based TCI state switch delay	736

8.10C.5	RRC based TCI state switch delay	737

8.10C.6	Active TCI state list update delay	737

8.10D	Active TCI state switching delay for ATG	737

8.10D.2	Void	737

8.10D.6	Active TCI state list update delay	738

8.10E	Active TCI state switching delay for UE operating in FR2-1 and configured with groupBasedBeamReporting-r17	738

8.10E.1	Introduction	738

8.10E.2	Known conditions for TCI state	738

8.10E.3	MAC-CE based dual DL TCI state switch delay	739

8.10E.3.1	MAC-CE based dual DL TCI state switching delay for sDCI	739

8.10E.3.2	MAC-CE based dual DL TCI state switching delay for mDCI	739

8.10E.4	DCI based dual DL TCI state switch delay for sDCI and mDCI	739

8.10E.4.1	DCI based dual DL TCI state switching delay for sDCI	739

8.10E.4.2	DCI based dual DL TCI state switching delay for mDCI	740

8.10E.5	RRC based dual DL TCI state switch delay	740

8.10E.6	Active DL TCI state list update delay	740

8.10E.6.1	Active DL TCI state list update delay for sDCI	740

8.10E.6.2	Active DL TCI state list update delay for mDCI	740

8.11	PSCell Change	740

8.11A	PSCell Change in Carriers with CCA	741

8.11B	Conditional PSCell Change	741

8.11B.1	Introduction	741

8.11B.2	Conditional PSCell Change delay	741

8.11B.2.1	Measurement time	742

8.11D	Conditional PSCell Change in Carriers with CCA	742

8.11D.1	Introduction	742

8.11D.2	Conditional PSCell Change delay	743

8.11D.2.1	Measurement time	743

8.11E	Subsequent Conditional PSCell Change	744

8.11E.1	Introduction	744

8.11E.2	Subsequent Conditional PSCell Change delay	744

8.11E.2.1	Measurement time	745

8.12	Uplink spatial relation switch delay	745

8.12.1	Introduction	745

8.12.2	Known conditions for spatial relation when associated with DL-RS	745

8.12.3	MAC-CE based spatial relation switch delay	745

8.12.4	DCI based spatial relation switch delay	746

8.12.5	RRC based spatial relation switch delay	746

8.12A	Uplink spatial relation switch delay for RedCap	747

8.12A.1	Introduction	747

8.12A.2	Known conditions for spatial relation when associated with DL-RS	747

8.12A.3	MAC-CE based spatial relation switch delay	747

8.12A.4	DCI based spatial relation switch delay	748

8.12A.5	RRC based spatial relation switch delay	748

8.12C	Uplink spatial relation switch delay for satellite access	749

8.12C.1	Void	749

8.12C.2	Void	749

8.12C.3	Void	749

8.12C.4	Void	749

8.12C.5	Void	749

8.13	UE-specific CBW change	749

8.13.1	Introduction	749

8.13.2	UE-specific CBW change delay	749

8.13A	UE-specific CBW change for RedCap	750

8.13A.1	Introduction	750

8.13A.2	UE-specific CBW change delay	750

8.13C	UE-specific CBW change for satellite access	750

8.13C.1	Introduction	750

8.13C.2	UE-specific CBW change delay	750

8.13D	UE-specific CBW change for ATG	751

8.13D.1	Introduction	751

8.13D.2	UE-specific CBW change delay	751

8.14	Pathloss reference signal switching delay	751

8.14.1	Introduction	751

8.14.2	Known conditions for pathloss reference signal	751

8.14.3	MAC-CE based pathloss reference signal switch delay	752

8.14C	Pathloss reference signal switching delay for satellite access	752

8.14C.1	Introduction	752

8.14C.2	Known conditions for pathloss reference signal	752

8.14C.3	MAC-CE based pathloss reference signal switch delay	753

8.14D	Pathloss reference signal switching delay for ATG	753

8.14D.1	Introduction	753

8.14D.2	Known conditions for pathloss reference signal	753

8.14D.3	MAC-CE based pathloss reference signal switch delay	753

8.15	Active downlink TCI state switching delay for unified TCI	754

8.15.1	Introduction	754

8.15.4	DCI based downlink TCI state switch delay	755

8.15.5	Active Downlink TCI state list update delay	756

8.15D	Active downlink TCI state switching delay for unified TCI for ATG	757

8.15D.1	Introduction	757

8.15D.2	Void	757

8.15D.4	DCI based downlink TCI state switch delay	757

8.15D.5	Active Downlink TCI state list update delay	757

8.16	Active uplink TCI state switching delay for unified TCI	758

8.16.1	Introduction	758

8.16.4	DCI based uplink TCI state switch delay	760

8.16.5	Active Uplink TCI state list update delay	760

8.16D	Active uplink TCI state switching delay for unified TCI for ATG	762

8.16D.1	Introduction	762

8.16D.2	Void	762

8.16D.3	MAC-CE based uplink TCI state switch delay	762

8.16D.4	DCI based uplink TCI state switch delay	763

8.16D.5	Active Uplink TCI state list update delay	763

8.17	SCG Activation and Deactivation Delay	764

8.17.1	Introduction	764

8.17.2	SCG Activation Delay Requirement	764

8.17.3	SCG Deactivation Delay Requirement	765

8.18	TRP specific Link Recovery Procedures	765

8.18.1	Introduction	765

8.18.2	Requirements for TRP specific SSB based beam failure detection	766

8.18.2.1	Introduction	766

8.18.2.2	Minimum requirement	766

8.18.2.3	Measurement restriction for SSB based beam failure detection	768

8.18.3	Requirements for CSI-RS based beam failure detection	769

8.18.3.1	Introduction	769

8.18.3.2	Minimum requirement	770

8.18.3.3	Measurement restrictions for CSI-RS beam failure detection	773

8.18.4	Minimum requirement for L1 indication	774

8.18.5	Requirements for SSB based candidate beam detection	774

8.18.5.1	Introduction	774

8.18.5.2	Minimum requirement	774

8.18.5.3	Measurement restriction for SSB based candidate beam detection	777

8.18.6	Requirements for CSI-RS based candidate beam detection	778

8.18.6.1	Introduction	778

8.18.6.2	Minimum requirement	778

8.18.6.3	Measurement restriction for CSI-RS based candidate beam detection	780

8.18.7	Requirements for TRP specific Beam Failure Recovery	781

8.18.7.1	Introduction	781

8.18.7.2	Requirement	782

8.18.8	Scheduling availability of UE during TRP specific beam failure detection	782

8.18.8.1	Scheduling availability of UE performing TRP specific beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	782

8.18.8.2	Scheduling availability of UE performing TRP specific beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	782

8.18.8.3	Scheduling availability of UE performing TRP specific beam failure detection on FR2	782

8.18.8.4	Scheduling availability of UE performing TRP specific beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	783

8.18.9	Scheduling availability of UE during TRP specific candidate beam detection	783

8.18.9.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	783

8.18.9.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	784

8.18.9.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	784

8.18.9.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	784

8.19	Pre-configured measurement gap activation/deactivation delay	785

8.19.1	Introduction	785

8.19.2	Pre-configured measurement gap activation/deactivation upon DCI/timer-based BWP switch	785

8.19.2.1	Activation/deactivation upon DCI/timer-based BWP switch delay on a single CC	785

8.19.3	Pre-configured measurement gap activation/deactivation upon SCell activation/deactivation	785

8.19.4	Pre-configured measurement gap activation/deactivation upon RRC reconfiguration	785

8.19.5	Activation/deactivation delay requirements for concurrent measurement gaps with Pre-MG	785

8.19.5.1	Activation/deactivation delay requirements for non-overlapped activation/deactivation of concurrent measurement gaps with Pre- MG	786

8.19.5.2	Activation/deactivation delay requirements for fully overlapped activation/deactivation of concurrent measurement gaps with Pre- MG	786

8.19.5.3	Pre-MG activation/deactivation delay when colliding with a concurrent measurement gap	786

8.19D	Pre-configured measurement gap activation/deactivation delay for ATG	786

8.19D.1	Introduction	786

8.19D.2	Pre-configured measurement gap activation/deactivation upon DCI/timer-based BWP switch	786

8.19D.2.1	Activation/deactivation upon DCI/timer-based BWP switch delay on a single CC	786

8.19D.3	Pre-configured measurement gap activation/deactivation upon RRC reconfiguration	787

8.20	LTM PSCell Cell Switch	787

8.20.1	Introduction	787

8.20.2	LTM Cell Switch delay	788

8.20.3	Void	788

8.21	Active downlink TCI state switching delay for unified TCI for single-DCI mTRP	788

8.21.1	Introduction	788

8.21.2	Known conditions for downlink TCI state	788

8.21.3	MAC-CE based downlink TCI state switch delay	789

8.21.4	DCI based downlink TCI state switch delay	790

8.21.5	Active Downlink TCI state list update delay	790

8.22	Active downlink TCI state switching delay for unified TCI for multi-DCI mTRP	791

8.22.1	Introduction	791

8.22.2	Known conditions for downlink TCI state	791

8.22.3	MAC-CE based downlink TCI state switch delay	791

8.22.4	DCI based downlink TCI state switch delay	793

8.22.5	Active Downlink TCI state list update delay	793

8.23	Active uplink TCI state switching delay for unified TCI for single-DCI mTRP	794

8.23.1	Introduction	794

8.23.2	Known conditions for uplink TCI state	794

8.23.3	MAC-CE based uplink TCI state switch delay	794

8.23.4	DCI based uplink TCI state switch delay	796

8.23.5	Active uplink TCI state list update delay	796

8.24	Active uplink TCI state switching delay for unified TCI for multi-DCI mTRP	797

8.24.1	Introduction	797

8.24.2	Known conditions for uplink TCI state	797

8.24.3	MAC-CE based uplink TCI state switch delay	798

8.24.4	DCI based uplink TCI state switch delay	799

8.24.5	Active Uplink TCI state list update delay	799

8.25	TCI state activation for LTM candidate cell	800

8.25.1	Introduction	800

8.25.2	Known TCI state conditions	800

8.25.3	SSB based TCI state activation delay	801

9	Measurement Procedure	802

9.1	General measurement requirement	802

9.1.1	Introduction	802

9.1.2	Measurement gap	802

9.1.2.1	EN-DC: Measurement Gap Sharing	812

9.1.2.1a	SA: Measurement Gap Sharing	812

9.1.2.1b	NE-DC: Measurement Gap Sharing	813

9.1.2.1c	NR-DC: Measurement Gap Sharing	814

9.1.3	UE Measurement capability	815

9.1.3.1	EN-DC: Monitoring of multiple layers using gaps	815

9.1.3.1a	SA: Monitoring of multiple layers using gaps	816

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	816

9.1.3.1c	NR-DC: Monitoring of multiple layers using gaps	817

9.1.3.2	EN-DC: Maximum allowed layers for multiple monitoring	817

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	818

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	819

9.1.3.2c	NR-DC: Maximum allowed layers for multiple monitoring	819

9.1A.3.2	Void	820

9.1.3A	UE Measurement capability under operation mode with CCA	820

9.1.3A.1	EN-DC: Monitoring of multiple layers using gaps under CCA	820

9.1.3A.1a	SA: Monitoring of multiple layers using gaps under CCA	820

9.1.3A.2	EN-DC: Maximum allowed layers for multiple monitoring under CCA	820

9.1.3A.2a	SA: Maximum allowed layers for multiple monitoring under CCA	821

9.1.3C	UE Measurement capability under operation mode with satellite access	821

9.1.3C.1a	SA: Monitoring of multiple layers using gaps under satellite access	821

9.1.3C.2a	SA: Maximum allowed layers for multiple monitoring for SAN	822

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	822

9.1.4.1	Introduction	822

9.1.4.2	Requirements	822

9.1.5	Carrier-specific scaling factor	825

9.1.5.1	Monitoring of multiple layers outside gaps	825

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	828

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	830

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	831

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps	831

9.1.5.2	Monitoring of multiple layers within gaps	833

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	835

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	837

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps	839

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps	841

9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps	843

9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	843

9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	843

9.1.5.3	Monitoring of multiple layers within NCSG	844

9.1.5.3.1	SA mode: carrier-specific scaling factor for measurements performed within NCSG	845

9.1.5.4	L1-RSRP measurements within measurement gap	846

9.1.5.4.1	SA mode: carrier-specific scaling factor for L1-RSRP measurements performed within measurement gap	847

9.1.5.4.2	NR-DC: carrier-specific scaling factor for L1-RSRP measurements performed within measurement gap	848

9.1.6	Minimum requirement at transitions	850

9.1.7	Pre-configured measurement gap	850

9.1.7.1	Introduction	850

9.1.7.2	Requirements applicability	851

9.1.7.3	Requirements	851

9.1.7.3.1	Requirements for autonomous activation/deactivation mechanism	851

9.1.7.3.2	Requirements for network-controlled activation/deactivation mechanism	852

9.1.7.3.3	Requirements for reception/transmission during activation/deactivation	853

9.1.8	Concurrent measurement gaps	853

9.1.8.1	Introduction	853

9.1.8.2	Requirements	853

9.1.8.3	Collision between concurrent measurement gaps	854

9.1.8.4	Measurement gap related requirements of concurrent measurement gaps	854

9.1.9	Network controlled small gap	855

9.1.9.1	Introduction	855

9.1.9.2	Requirements applicability	856

9.1.10	MUSIM gaps	858

9.1.10.1	Introduction	859

9.1.10.2	Priorities for MUSIM gaps	859

9.1.10.3	Keep solution for MUSIM gaps	860

9.1.10.4	Collisions between different MUSIM gaps	860

9.1.10.5	Collisions between MUSIM gaps and measurement gaps	860

9.1.10.6	MUSIM gap related requirements	861

9.1.11	UL gap for Tx power management	861

9.1.12	Concurrent measurement gaps with Pre-MG	861

9.1.12.1	Introduction	861

9.1.12.2	Requirements	861

9.1.12.3	Collisions involving Pre-MG(s)	862

9.1.12.4	Collision between Pre-MG activation/deactivation and measurement gap	863

9.1.12.5	Pre-MG related requirements	863

9.1.13	Concurrent measurement gaps with NCSG	863

9.1.13.1	Introduction	863

9.1.13.2	Requirements	863

9.1.13.3	Collision involving NCSGs	865

9.1A	General measurement requirement for RedCap	865

9.1A.1	Introduction	865

9.1A.2	Measurement gap	865

9.1A.2.1	SA: Measurement Gap Sharing	869

9.1A.3	UE Measurement capability	870

9.1A.3.1	SA: Monitoring of multiple layers using gaps	870

9.1A.3.2	SA: Maximum allowed layers for multiple monitoring	870

9.1A.4	Capabilities for Support of Event Triggering and Reporting Criteria	870

9.1A.4.1	Introduction	870

9.1A.4.2	Requirements	871

9.1A.5	Carrier-specific scaling factor	871

9.1A.5.1	Monitoring of multiple layers outside gaps	871

9.1A.5.1.1	SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps	872

9.1A.5.2	Monitoring of multiple layers within gaps	872

9.1A.5.2.1	SA mode: carrier-specific scaling factor for SSB measurements performed within gaps	872

9.1A.6	Minimum requirement at transitions	874

9.1C	General measurement requirement for SAN	874

9.1C.1	Introduction	874

9.1C.2	Measurement gap	875

9.1C.8	Concurrent measurement gaps for SAN	877

9.1C.8.1	Introduction	877

9.1C.8.2	Requirements	877

9.1C.8.3	Collision between concurrent measurement gaps	878

9.1C.8.4	Measurement gap related requirements of concurrent measurement gaps	878

9.1C.9	Collision between SMTC and measurement gap for SAN	878

9.1C.9.1	Introduction	878

9.1C.9.2	Collision between SMTCs and measurement gap	878

9.1C.9.3	Collision between multiple SMTCs on a SAN carrier	879

9.1D	General measurement requirement for ATG	879

9.1D.1	Introduction	879

9.1D.2	Measurement gap	879

9.1D.2.1a	SA: Measurement Gap Sharing	881

9.1D.3	UE Measurement capability	882

9.1D.3.1	SA: Monitoring of multiple layers using gaps	882

9.1D.3.2	SA: Maximum allowed layers for multiple monitoring	882

9.1D.4	Void	882

9.1D.5	Carrier-specific scaling factor	882

9.1D.5.1	Monitoring of multiple layers outside gaps	882

9.1D.5.1.1	Void	883

9.1D.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements performed outside gaps	883

9.1D.5.2	Monitoring of multiple layers within gaps	883

9.1D.5.2.1	Void	884

9.1D.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements performed within gaps	884

9.1D.6	Void	885

9.1D.7	Pre-configured measurement gap	885

9.1D.7.1	Introduction	885

9.1D.7.2	Requirements applicability	885

9.1D.7.3	Requirements	885

9.1D.7.3.1	Requirements for autonomous activation/deactivation mechanism	886

9.1D.7.3.2	Requirements for network-controlled activation/deactivation mechanism	886

9.1D.7.3.3	Requirements for reception/transmission during activation/deactivation	886

9.1D.8	Capabilities for Support of Event Triggering and Reporting Criteria	887

9.1D.8.1	Introduction	887

9.1D.8.2	Requirements	887

9.1D.9	Minimum requirement at transitions	887

9.2	NR intra-frequency measurements	888

9.2.1	Introduction	888

9.2.2	Requirements applicability	891

9.2.3	Number of cells and number of SSB	891

9.2.3.1	Requirements for FR1	891

9.2.3.2	Requirements for FR2	892

9.2.4	Measurement Reporting Requirements	892

9.2.4.1	Periodic Reporting	892

9.2.4.2	Event-triggered Periodic Reporting	892

9.2.4.3	Event Triggered Reporting	892

9.2.4.4	SCell activation Triggered Reporting	893

9.2.5	Intrafrequency measurements without measurement gaps	893

9.2.5.1	Intrafrequency cell identification	893

9.2.5.2	Measurement period	900

9.2.5.3	Scheduling availability of UE during intra-frequency measurements	904

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	904

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	905

9.2.5.3.3	Scheduling availability of UE performing measurements on FR2	905

9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	907

9.2.5.4	SFTD Measurements between PCell and PSCell	907

9.2.5.4.1	Introduction	907

9.2.5.4.2	SFTD Measurement delay	907

9.2.5.4.3	SFTD Measurement Reporting Delay	908

9.2.6	Intra-frequency measurements with measurement gaps	908

9.2.6.1	Void	908

9.2.6.2	Intra-frequency cell identification	908

9.2.6.3	Intra-frequency Measurement Period	911

9.2.7	Intra-frequency measurements with NCSG	912

9.2.7.1	Intra-frequency cell identification	912

9.2.7.2	Measurement period	914

9.2.7.3	Scheduling availability during intra-frequency measurement with NCSG	915

9.2A	NR intra-frequency measurements with CCA	915

9.2A.1	Introduction	915

9.2A.2	Requirements applicability	916

9.2A.3	Number of cells and number of SSB	917

9.2A.3.1	Requirements for FR1	917

9.2A.3.2	Requirements for FR2-2	917

9.2A.4	Measurement Reporting Requirements	917

9.2A.5	Intra-frequency measurements without measurement gaps	918

9.2A.5.2	Measurement period	922

9.2A.5.3	Scheduling availability of UE during intra-frequency measurements	924

9.2A.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	925

9.2A.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	925

9.2A.5.3.3	Scheduling availability of UE performing measurements in TDD bands on FR2-2	925

9.2A.6	Intra-frequency measurements with measurement gaps	926

9.2A.6.1	Intra-frequency cell identification	926

9.2A.6.2	Intra-frequency Measurement Period	928

9.2A.7	Intra-frequency RSSI and Channel occupancy measurements	929

9.2A.7.1	Intra-frequency RSSI measurements	929

9.2A.7.2	Intra-frequency Channel occupancy measurements	931

9.2A.7.3	Scheduling restriction during RSSI and Channel Occupancy measurements in FR1	933

9.2A.7.4	Scheduling restriction during RSSI measurements in FR2-2	933

9.2B	NR intra-frequency measurements for RedCap	933

9.2B.1	Introduction	933

9.2B.2	Requirements applicability	934

9.2B.3	Number of cells and number of SSB	934

9.2B.3.1	Requirements for FR1	934

9.2B.3.2	Requirements for FR2	934

9.2B.4	Measurement Reporting Requirements	935

9.2B.4.1	Periodic Reporting	935

9.2B.4.2	Event-triggered Periodic Reporting	935

9.2B.4.3	Event Triggered Reporting	935

9.2B.5	Intra-frequency measurements without measurement gaps	936

9.2B.5.1	Intra-frequency cell identification	936

9.2B.5.2	Measurement period	938

9.2B.5.3	Scheduling availability of UE during intra-frequency measurements	939

9.2B.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	939

9.2B.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	939

9.2B.5.3.3	Scheduling availability of UE performing measurements on FR2	940

9.2B.5.3.4	Scheduling availability of HD-FDD UE performing measurements on FR1	940

9.2B.6	Intra-frequency measurements with measurement gaps	941

9.2B.6.1	Intra-frequency cell identification	941

9.2B.6.2	Intra-frequency Measurement Period	942

9.2C	NR intra-frequency measurements for SAN	943

9.2C.1	Introduction	943

9.2C.2	Requirements applicability	944

9.2C.3	Number of cells and number of SSB	944

9.2C.3.1	Requirements for FR1-NTN	944

9.2C.4	Measurement Reporting Requirements	945

9.2C.4.1	Periodic Reporting	945

9.2C.4.2	Event-triggered Periodic Reporting	945

9.2C.4.3	Event Triggered Reporting	945

9.2C.5	Intra-frequency measurements without measurement gaps	945

9.2C.5.1	Intra-frequency cell identification	945

9.2C.5.2	Measurement period	947

9.2C.5.3	Scheduling availability of UE during intra-frequency measurements	948

9.2C.5.3.1	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1-NTN	948

9.2C.5.3.2	Scheduling availability of UE performing measurements on a neighbor cell served by a different satellite in LEO	948

9.2C.6	Intra-frequency measurements with measurement gaps	949

9.2C.6.2	Intra-frequency cell identification	949

9.2C.6.3	Intrafrequency Measurement Period	950

9.2C.7	Intra-frequency measurements without measurement gaps for NTN band above 10 GHz	950

9.2C.7.1	Intra-frequency cell identification	950

9.2C.7.2	Measurement period	952

9.2C.7.3	Scheduling availability of UE during intra-frequency measurements	952

9.2C.7.3.1	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on NTN bands above 10 GHz	952

9.2C.8	Intra-frequency measurements with measurement gaps for NTN band above 10 GHz	953

9.2C.8.1	Intra-frequency cell identification	953

9.2C.8.3	Intra-frequency Measurement Period	954

9.2D	NR intra-frequency measurements for ATG	954

9.2D.1	Introduction	954

9.2D.2	Requirements applicability	954

9.2D.3	Number of cells and number of SSB	955

9.2D.3.1	Requirements for FR1	955

9.2D.4	Measurement Reporting Requirements	955

9.2D.4.1	Periodic Reporting	955

9.2D.4.2	Event-triggered Periodic Reporting	955

9.2D.4.3	Event Triggered Reporting	955

9.2D.5	Intra-frequency measurements without measurement gaps	956

9.2D.5.1	Intra-frequency cell identification	956

9.2D.5.2	Measurement period	957

9.2D.5.3	Scheduling availability of UE during intra-frequency measurements	958

9.2D.5.3.1	Scheduling availability of UE performing measurements on FR1	958

9.2D.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	959

9.2D.6	Intra-frequency measurements with measurement gaps	959

9.2D.6.1	Void	959

9.2D.6.2	Intra-frequency cell identification	959

9.2D.6.3	Intra-frequency Measurement Period	960

9.3	NR inter-frequency measurements	961

9.3.1	Introduction	961

9.3.2	Requirements applicability	964

9.3.2.1	Void	964

9.3.2.2	Void	964

9.3.3	Number of cells and number of SSB	964

9.3.3.1	Requirements for FR1	964

9.3.3.2	Requirements for FR2	964

9.3.4	Inter-frequency measurement with measurement gaps	965

9.3.4.1	Void	969

9.3.4.2	Void	969

9.3.5	Inter-frequency measurements	969

9.3.5.1	Void	971

9.3.5.2	Void	971

9.3.5.3	Void	971

9.3.6	Inter-frequency measurements reporting requirements	971

9.3.6.1	Periodic Reporting	971

9.3.6.2	Event-triggered Periodic Reporting	971

9.3.6.3	Event-triggered Reporting	971

9.3.7	Void	972

9.3.8	Inter-frequency SFTD measurement requirements	972

9.3.8.1	Introduction	972

9.3.8.2	SFTD Measurement delay	972

9.3.8.3	SFTD Measurement reporting delay	973

9.3.9	Inter-frequency measurements without measurement gaps	973

9.3.9.1	Inter-frequency Cell identification	973

9.3.9.2	Measurement period	978

9.3.9.3	Scheduling availability of UE during inter-frequency measurements when the SSB is completely contained in the active BWP of the UE	980

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	981

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	981

9.3.9.3.3	Scheduling availability of UE performing measurements on FR2	982

9.3.9.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	982

9.3.9.4	Scheduling availability of UE during inter-frequency measurements when the SSB is not completely contained in the active BWP of the UE	982

9.3.9.4.1	Scheduling availability of UE performing measurements in TDD bands on FR1	983

9.3.9.4.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	983

9.3.9.4.3	Scheduling availability of UE performing measurements on FR2	984

9.3.9.4.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	985

9.3.10	Inter-frequency measurement with NCSG	986

9.3.10.1	Inter-frequency cell identification	986

9.3.10.2	Measurement period	987

9.3.10.3	Scheduling availability during inter-frequency measurement with NCSG	988

9.3.10.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	988

9.3.10.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	989

9.3.10.3.3	Scheduling availability of UE performing measurements on FR2	989

9.3.10.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	991

9.3A	NR inter-frequency measurements in carrier frequencies with CCA	991

9.3A.1	Introduction	991

9.3A.2	Requirements applicability	992

9.3A.3	Number of cells and number of SSB	992

9.3A.3.1	Requirements for FR1	992

9.3A.3.2	Requirements for FR2-2	992

9.3A.4	Inter-frequency cell identification	992

9.3A.5	Inter-frequency measurements	994

9.3A.6	Inter-frequency measurements reporting requirements	996

9.3A.6.1	Periodic Reporting	996

9.3A.6.2	Event-triggered Periodic Reporting	996

9.3A.6.3	Event-triggered Reporting	996

9.3A.8	Inter-frequency RSSI measurements	996

9.3A.9	Inter-frequency channel occupancy measurements	997

9.3B	NR inter-frequency measurements for RedCap	998

9.3B.1	Introduction	998

9.3B.2	Requirements applicability	998

9.3B.3	Number of cells and number of SSB	999

9.3B.3.1	Requirements for FR1	999

9.3B.3.2	Requirements for FR2	999

9.3B.4	Inter-frequency measurement with measurement gaps	999

9.3B.5	Inter-frequency measurements	1001

9.3B.6	Inter-frequency measurements reporting requirements	1001

9.3B.6.1	Periodic Reporting	1001

9.3B.6.2	Event-triggered Periodic Reporting	1001

9.3B.6.3	Event-triggered Reporting	1002

9.3B.7	Inter-frequency measurements without measurement gaps	1002

9.3B.7.1	Inter-frequency Cell identification	1002

9.3B.7.2	Measurement period	1004

9.3B.7.3	Scheduling availability of UE during inter-frequency measurements	1005

9.3B.7.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	1005

9.3B.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	1005

9.3B.7.3.3	Scheduling availability of UE performing measurements on FR2	1006

9.3B.7.3.4	Scheduling availability of HD-FDD UE performing measurements on FR1	1006

9.3C	NR inter-frequency measurements for SAN	1006

9.3C.1	Introduction	1006

9.3C.2	Requirements applicability	1007

9.3C.3	Number of cells and number of SSB	1008

9.3C.3.1	Requirements for FR1-NTN	1008

9.3C.4	Inter-frequency measurement with measurement gaps	1008

9.3C.5	Inter-frequency measurements	1009

9.3C.6	Inter-frequency measurements reporting requirements	1010

9.3C.6.1	Periodic Reporting	1010

9.3C.6.2	Event-triggered Periodic Reporting	1010

9.3C.6.3	Event-triggered Reporting	1010

9.3C.7	Inter-frequency measurements without measurement gaps	1011

9.3C.7.1	Inter-frequency Cell identification	1011

9.3C.7.2	Measurement period	1012

9.3C.7.3	Scheduling availability of UE during inter-frequency measurements	1012

9.3C.7.3.1	Void	1012

9.3C.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1-NTN	1012

9.3C.8	Inter-frequency measurement with measurement gaps for NTN band above 10 GHz	1013

9.3C.9	Inter-frequency measurements for NTN band above 10 GHz	1014

9.3C.10	Inter-frequency measurements without measurement gaps for NTN band above 10 GHz	1014

9.3C.10.1	Inter-frequency Cell identification	1014

9.3C.10.2	Measurement period	1015

9.3C.10.3	Scheduling availability of UE during inter-frequency measurements	1015

9.3C.10.3.1	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on NTN bands above 10 GHz	1016

9.3D	NR inter-frequency measurements for ATG	1016

9.3D.1	Introduction	1016

9.3D.2	Requirements applicability	1016

9.3D.3	Number of cells and number of SSB	1017

9.3D.3.1	Requirements for FR1	1017

9.3D.4	Inter-frequency measurement with measurement gaps	1017

9.3D.5	Inter-frequency measurements	1018

9.3D.6	Inter-frequency measurements reporting requirements	1018

9.3D.6.1	Periodic Reporting	1018

9.3D.6.2	Event-triggered Periodic Reporting	1019

9.3D.6.3	Event-triggered Reporting	1019

9.3D.7	Void	1019

9.3D.8	Void	1019

9.3D.9	Inter-frequency measurements without measurement gaps	1019

9.3D.9.1	Inter-frequency Cell identification	1019

9.3D.9.2	Measurement period	1021

9.3D.9.3	Scheduling availability of UE during inter-frequency measurements	1021

9.3D.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	1022

9.3D.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	1022

9.4	Inter-RAT measurements	1023

9.4.1	Introduction	1023

9.4.2	NR − E-UTRAN FDD measurements	1025

9.4.2.1	Introduction	1025

9.4.2.2	Requirements when no DRX is used	1026

9.4.2.3	Requirements when DRX is used	1027

9.4.2.4	Measurement reporting requirements	1028

9.4.2.4.1	Periodic Reporting	1028

9.4.2.4.2	Event-Triggered Periodic Reporting	1028

9.4.2.4.3	Event-Triggered Reporting	1029

9.4.2.5	Scheduling Availability During NR − E-UTRAN FDD measurements with NCSG	1029

9.4.3	NR − E-UTRAN TDD measurements	1029

9.4.3.1	Introduction	1029

9.4.3.2	Requirements when no DRX is used	1029

9.4.3.3	Requirements when DRX is used	1031

9.4.3.4	Measurement reporting requirements	1032

9.4.3.4.1	Periodic Reporting	1032

9.4.3.4.2	Event-Triggered Periodic Reporting	1032

9.4.3.4.3	Event-Triggered Reporting	1033

9.4.3.5	Scheduling Availability During NR − E-UTRAN TDD measurements with NCSG	1033

9.4.4	Inter-RAT RSTD measurements	1033

9.4.4.1	NR − E-UTRAN FDD RSTD measurements	1033

9.4.4.1.1	Introduction	1033

9.4.4.1.2	Requirements	1034

9.4.4.2	NR − E-UTRAN TDD RSTD measurements	1036

9.4.4.2.1	Introduction	1036

9.4.4.2.2	Requirements	1037

9.4.5	Inter-RAT E-CID measurements	1040

9.4.5.1	NR−E-UTRAN FDD E-CID RSRP and RSRQ measurements	1040

9.4.5.1.1	Introduction	1040

9.4.5.1.2	Requirements	1040

9.4.5.1.3	Measurement Reporting Delay	1040

9.4.5.2	NR−E-UTRAN TDD E-CID RSRP and RSRQ measurements	1041

9.4.5.2.1	Introduction	1041

9.4.5.2.2	Requirements	1041

9.4.5.2.3	Measurement Reporting Delay	1041

9.4.6	NR − UTRAN FDD measurements	1041

9.4.6.1	Introduction	1041

9.4.6.2	Requirements when no DRX is used	1041

9.4.6.3	Requirements when DRX is used	1042

9.4.7	NR – E-UTRAN measurements with autonomous gaps	1044

9.4.7.1	CGI identification of an E-UTRA cell with autonomous gaps	1044

9.4.7.2	CGI reporting delay	1044

9.4.8	NR – E-UTRAN measurements without measurement gaps	1045

9.4.8.1	Introduction	1045

9.4.8.2	General requirements	1045

9.4.8.3	NR − E-UTRAN FDD measurements	1046

9.4.8.3.1	Introduction	1046

9.4.8.3.2	Requirements when no DRX is used	1046

9.4.8.3.3	Requirements when DRX is used	1047

9.4.8.3.4	Measurement reporting requirements	1047

9.4.8.3.5	Scheduling availability during NR − E-UTRAN FDD measurements	1048

9.4.8.4	NR − E-UTRAN TDD measurements	1048

9.4.8.4.1	Introduction	1048

9.4.8.4.2	Requirements when no DRX is used	1049

9.4.8.4.3	Requirements when DRX is used	1050

9.4.8.4.4	Measurement reporting requirements	1051

9.4.8.4.5	Scheduling availability during NR − E-UTRAN TDD measurements	1051

9.4A	Inter-RAT measurements for RedCap	1052

9.4A.1	Introduction	1052

9.4A.2	NR − E-UTRAN FDD measurements	1053

9.4A.2.1	Introduction	1053

9.4A.2.2	Requirements when no DRX is used	1053

9.4A.2.3	Requirements when DRX is used	1054

9.4A.2.4	Measurement reporting requirements	1055

9.4A.2.4.1	Periodic Reporting	1055

9.4A.2.4.2	Event-Triggered Periodic Reporting	1055

9.4A.2.4.3	Event-Triggered Reporting	1055

9.4A.3	NR − E-UTRAN TDD measurements	1056

9.4A.3.1	Introduction	1056

9.4A.3.2	Requirements when no DRX is used	1056

9.4A.3.3	Requirements when DRX is used	1058

9.4A.3.4	Measurement reporting requirements	1059

9.4A.3.4.1	Periodic Reporting	1059

9.4A.3.4.2	Event-Triggered Periodic Reporting	1059

9.4A.3.4.3	Event-Triggered Reporting	1059

9.4A.4	NR – E-UTRAN measurements with autonomous gaps	1060

9.4A.4.1	CGI identification of an E-UTRA cell with autonomous gaps	1060

9.4A.4.2	CGI reporting delay	1060

9.4A.4.3	CGI reporting scheduling restriction	1060

9.5	L1-RSRP measurements for Reporting	1061

9.5.1	Introduction	1061

9.5.2	Requirements applicability	1061

9.5.3	Measurement Reporting Requirements	1062

9.5.3.1	Periodic Reporting	1062

9.5.3.2	Semi-Persistent Reporting	1063

9.5.3.3	Aperiodic Reporting	1063

9.5.4	L1-RSRP measurement requirements	1063

9.5.4.1	SSB based L1-RSRP Reporting	1063

9.5.4.2	CSI-RS based L1-RSRP Reporting	1069

9.5.4A	Void	1073

9.5.4A.1	Void	1073

9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	1073

9.5.5.1	Measurement restriction for SSB based L1-RSRP	1073

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	1074

9.5.6	Scheduling availability of UE during L1-RSRP measurement	1075

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1075

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1075

9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	1076

9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1078

9.5.7	Minimum requirement at transitions	1078

9.5A	L1-RSRP measurements for Reporting under CCA	1078

9.5A.1	Introduction	1078

9.5A.2	Requirements applicability	1078

9.5A.3	Measurement Reporting Requirements	1079

9.5A.3.1	Periodic Reporting	1079

9.5A.3.2	Semi-Persistent Reporting	1079

9.5A.3.3	Aperiodic Reporting	1079

9.5A.4	L1-RSRP measurement requirements	1080

9.5A.4.1	SSB based L1-RSRP Reporting	1080

9.5A.5	Measurement restriction for L1-RSRP measurement	1082

9.5A.5.1	Measurement restriction for SSB based L1-RSRP	1082

9.5A.6	Scheduling availability of UE during L1-RSRP measurement	1083

9.5A.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1083

9.5A.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1083

9.5A.6.3	Void	1083

9.5A.6.3A	Scheduling availability of UE performing L1-RSRP measurement in case of FR1-FR2 inter-band CA	1083

9.5A.6.3B	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	1083

9.5A.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1084

9.5B	L1-RSRP measurements for Reporting for RedCap	1084

9.5B.1	Introduction	1084

9.5B.2	Requirements applicability	1084

9.5B.3	Measurement Reporting Requirements	1085

9.5B.3.1	Periodic Reporting	1085

9.5B.3.2	Semi-Persistent Reporting	1085

9.5B.3.3	Aperiodic Reporting	1086

9.5B.4	L1-RSRP measurement requirements	1086

9.5B.4.1	SSB based L1-RSRP Reporting	1086

9.5B.4.2	CSI-RS based L1-RSRP Reporting	1088

9.5B.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	1091

9.5B.5.1	Measurement restriction for SSB based L1-RSRP	1091

9.5B.5.2	Measurement restriction for CSI-RS based L1-RSRP	1091

9.5B.6	Scheduling availability of UE during L1-RSRP measurement	1091

9.5B.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1092

9.5B.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1092

9.5B.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	1092

9.5C	L1-RSRP measurements for Reporting for satellite access	1093

9.5C.1	Introduction	1093

9.5C.2	Requirements applicability	1093

9.5C.3	Measurement Reporting Requirements	1093

9.5C.3.1	Periodic Reporting	1094

9.5C.3.2	Semi-Persistent Reporting	1094

9.5C.3.3	Aperiodic Reporting	1094

9.5C.4	L1-RSRP measurement requirements	1094

9.5C.4.1	SSB based L1-RSRP Reporting	1094

9.5C.5	Measurement restriction for L1-RSRP measurement	1096

9.5C.5.1	Measurement restriction for SSB based L1-RSRP	1096

9.5C.5.2	Measurement restriction for CSI-RS based L1-RSRP	1096

9.5C.6	Scheduling availability of UE during L1-RSRP measurement	1097

9.5C.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1-NTN	1097

9.5C.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1-NTN	1097

9.5C.7	L1-RSRP measurement requirements for NTN band above 10 GHz	1097

9.5C.7.1	SSB based L1-RSRP Reporting	1097

9.5C.7.2	CSI-RS based L1-RSRP Reporting	1098

9.5C.8	Measurement restriction for L1-RSRP measurement for NTN band above 10 GHz	1099

9.5C.8.1	Measurement restriction for SSB based L1-RSRP	1099

9.5C.8.2	Measurement restriction for CSI-RS based L1-RSRP	1099

9.5C.9	Scheduling availability of UE during L1-RSRP measurement for NTN band above 10 GHz	1100

9.5C.9.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on NTN bands above 10 GHz	1100

9.5C.9.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on NTN bands above 10 GHz	1100

9.5D	L1-RSRP measurements for Reporting for ATG	1100

9.5D.1	Introduction	1100

9.5D.2	Requirements applicability	1100

9.5D.3	Measurement Reporting Requirements	1101

9.5D.3.1	Periodic Reporting	1101

9.5D.3.2	Semi-Persistent Reporting	1101

9.5D.3.3	Aperiodic Reporting	1101

9.5D.4	L1-RSRP measurement requirements	1101

9.5D.4.1	SSB based L1-RSRP Reporting	1101

9.5D.4.2	CSI-RS based L1-RSRP Reporting	1103

9.5D.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	1104

9.5D.5.1	Measurement restriction for SSB based L1-RSRP	1104

9.5D.5.2	Measurement restriction for CSI-RS based L1-RSRP	1104

9.5D.6	Scheduling availability of UE during L1-RSRP measurement	1105

9.5D.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1105

9.5D.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1105

9.6	NE-DC: Measurements	1105

9.6.1	Introduction	1105

9.6.2	SFTD Measurements	1105

9.6.2.1	Introduction	1105

9.6.2.2	SFTD Measurement requirements	1106

9.7	Cross Link Interference measurements	1106

9.7.1	Introduction	1106

9.7.2	SRS-RSRP measurements	1107

9.7.2.1	Introduction	1107

9.7.2.2	Requirements applicability	1107

9.7.2.3	Measurement Reporting Requirements	1107

9.7.2.3.1	Periodic Reporting	1107

9.7.2.3.2	Event-triggered Periodic Reporting	1107

9.7.2.3.3	Event Triggered Reporting	1107

9.7.2.4	Measurement capability	1108

9.7.2.5	SRS-RSRP measurement period	1108

9.7.3	CLI-RSSI measurements	1108

9.7.3.1	Introduction	1108

9.7.3.2	Requirements applicability	1108

9.7.3.3	Measurement Reporting Requirements	1108

9.7.3.3.1	Periodic Reporting	1108

9.7.3.3.2	Event-triggered Periodic Reporting	1109

9.7.3.3.3	Event Triggered Reporting	1109

9.7.3.4	Measurement capability	1109

9.7.3.5	CLI-RSSI measurement period	1109

9.7.4	Scheduling availability of UE during CLI measurements	1109

9.7.4.1	Scheduling availability of UE performing measurement on FR1	1109

9.7.4.2	Scheduling availability of UE performing measurement on FR2	1110

9.8	L1-SINR measurements for Reporting	1111

9.8.1	Introduction	1111

9.8.2	Requirements applicability	1111

9.8.3	Measurement Reporting Requirements	1112

9.8.3.1	Periodic Reporting	1112

9.8.3.2	Semi-Persistent Reporting	1112

9.8.4	L1-SINR measurement requirements	1113

9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1113

9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1118

9.8.5	Measurement restriction for L1-SINR measurement	1119

9.8.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	1120

9.8.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	1120

9.8.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	1122

9.8.6	Scheduling availability of UE during L1-SINR measurement	1122

9.8.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1122

9.8.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1123

9.8.6.4	Scheduling availability of UE performing L1-SINR measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1124

9.8.7	Minimum requirement at transitions	1124

9.8D	L1-SINR measurements for Reporting for ATG	1124

9.8D.1	Introduction	1124

9.8D.2	Requirements applicability	1125

9.8D.3	Measurement Reporting Requirements	1125

9.8D.3.1	Periodic Reporting	1125

9.8D.3.2	Semi-Persistent Reporting	1125

9.8D.3.3	Aperiodic Reporting	1126

9.8D.4	L1-SINR measurement requirements	1126

9.8D.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1126

9.8D.4.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured	1127

9.8D.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1128

9.8D.5	Measurement restriction for L1-SINR measurement	1129

9.8D.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	1129

9.8D.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	1130

9.8D.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	1130

9.8D.6	Scheduling availability of UE during L1-SINR measurement	1130

9.8D.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1130

9.8D.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1130

9.9	NR measurements for positioning	1131

9.9.1	Introduction	1131

9.9.1.1	General Aspects of Gap-based Measurement	1131

9.9.1.2	General Aspects of Gapless Measurement	1132

9.9.1.3	Scheduling Availability of UE during PRS Measurement without Measurement Gaps	1133

9.9.2	RSTD measurements	1134

9.9.2.1	Introduction	1134

9.9.2.2	Requirements Applicability	1134

9.9.2.3	Measurement Capability	1134

9.9.2.4	Measurement Reporting Requirements	1134

9.9.2.4.1	Void	1134

9.9.2.4.2	Void	1134

9.9.2.4.3	Void	1134

9.9.2.5	Measurements Period Requirements	1134

9.9.2.6	Void	1137

9.9.2.7	Measurements Period Requirements without Measurement Gaps	1137

9.9.2.8	Void	1140

9.9.2.9	Measurements Period Requirements with both MG and PPW	1140

9.9.2.10	Measurements Period Requirements with Bandwidth Aggregation	1141

9.9.3	PRS-RSRP measurements	1144

9.9.3.1	Introduction	1144

9.9.3.2	Requirements applicability	1145

9.9.3.3	Measurement Capability	1145

9.9.3.4	Measurement Reporting Requirements	1145

9.9.3.5	Measurement Period Requirements	1145

9.9.3.6	Measurement Period Requirements without Measurement Gaps	1148

9.9.3.7	Void	1150

9.9.3.8	Measurements Period Requirements with both MG and PPW	1150

9.9.4	UE Rx-Tx time difference measurements	1151

9.9.4.1	Introduction	1151

9.9.4.2	Requirements Applicability	1151

9.9.4.3	Measurement Capability	1151

9.9.4.4	Measurement Reporting Requirements	1151

9.9.4.5	Measurement Period Requirements	1151

9.9.4.6	Measurement Period Requirements without Measurement Gaps	1155

9.9.4.7	Void	1158

9.9.4.8	Measurements Period Requirements with both MG and PPW	1158

9.9.4.9	Measurements Period Requirements with Bandwidth Aggregation	1159

9.9.5	E-CID measurements	1163

9.9.5.1	Introduction	1163

9.9.5.2	Measurement Requirements	1163

9.9.5.2.1	Intra-frequency Measurement Requirements	1163

9.9.5.2.2	Inter-frequency Measurement Requirements	1163

9.9.5.2.3	Measurement Reporting Delay	1164

9.9.6	PRS-RSRPP measurements	1164

9.9.6.1	Introduction	1164

9.9.6.2	Requirements applicability	1164

9.9.6.3	Measurement capability	1164

9.9.6.4	Measurement reporting requirements	1164

9.9.6.5	Measurement period requirements	1164

9.9.6.6	Measurement Period Requirements without Measurement Gaps	1165

9.9.6.7	Void	1165

9.9.6.8	Measurements Period Requirements with both MG and PPW	1165

9.9.7	Measurement requirements for DL RSCPD reported with RSTD	1165

9.9.7.1	Introduction	1165

9.9.7.2	Requirements Applicability	1165

9.9.7.3	Measurement Capability	1165

9.9.7.4	Measurement Reporting Requirements	1165

9.9.7.5	Measurements Period Requirements for DL RSCPD reported with RSTD	1166

9.9.8	Measurement requirements for DL RSCP reported with UE Rx-Tx time difference	1168

9.9.8.1	Introduction	1168

9.9.8.2	Requirements Applicability	1169

9.9.8.3	Measurement Capability	1169

9.9.8.4	Measurement Reporting Requirements	1169

9.9.8.5	Measurement Period Requirements for DL RSCP and UE Rx-Tx time difference	1169

9.9A	NR measurements for positioning for RedCap	1173

9.9A.1	Introduction	1173

9.9A.1.1	General Aspects of Gap-based Measurement	1173

9.9A.1.2	General Aspects of Gapless Measurement for RedCap positioning without FH	1174

9.9A.1.3	Scheduling Availability of UE during PRS Measurement without Measurement Gaps for RedCap positioning without FH	1175

9.9A.2	RSTD measurements for RedCap	1176

9.9A.2.1	Introduction	1176

9.9A.2.2	Requirements Applicability	1176

9.9A.2.3	Measurement Capability	1176

9.9A.2.4	Measurement Reporting Requirements	1176

9.9A.2.5	Measurements Period Requirements without FH	1176

9.9A.2.5.1	Measurements Period Requirements without FH with MG	1176

9.9A.2.5.2	Measurements Period Requirements without FH without MG	1180

9.9A.2.5.3	Measurements Period Requirements without FH with both MG and PPW	1182

9.9A.2.6	Measurements Period Requirements with FH	1183

9.9A.2.6.1	Measurements Period Requirements with FH with MG	1183

9.9A.3	PRS-RSRP measurements for RedCap	1185

9.9A.3.1	Introduction	1185

9.9A.3.2	Requirements applicability	1185

9.9A.3.3	Measurement Capability	1185

9.9A.3.4	Measurement Reporting Requirements	1185

9.9A.3.5	Measurements Period Requirements without FH	1185

9.9A.3.5.1	Measurement Period Requirements without FH with MG	1185

9.9A.3.5.2	Measurement Period Requirements without FH without MG	1188

9.9A.3.5.3	Measurements Period Requirements without FH with both MG and PPW	1190

9.9A.3.6	Measurements Period Requirements with FH	1191

9.9A.3.6.1	Measurements Period Requirements with FH with MG	1191

9.9A.4	UE Rx-Tx time difference measurements for RedCap	1193

9.9A.4.1	Introduction	1193

9.9A.4.2	Requirements Applicability	1193

9.9A.4.3	Measurement Capability	1193

9.9A.4.4	Measurement Reporting Requirements	1193

9.9A.4.5	Measurement Period Requirements without FH with MG	1193

9.9A.4.6	Measurement Period Requirements without FH without MG	1193

9.9A.4.7	Measurements Period Requirements without FH with both MG and PPW	1193

9.9A.4.8	Measurements Period Requirements with FH	1194

9.9A.5	PRS-RSRPP measurements for RedCap	1195

9.9A.5.1	Introduction	1195

9.9A.5.2	Requirements Applicability	1195

9.9A.5.3	Measurement Capability	1196

9.9A.5.4	Measurement Reporting Requirements	1196

9.9A.5.5	Measurement Period Requirements without FH with MG	1196

9.9A.5.6	Measurement Period Requirements without FH without MG	1196

9.9A.5.7	Measurements Period Requirements without FH with both MG and PPW	1196

9.9A.5.8	Measurements Period Requirements with FH	1196

9.9C	NR measurements for positioning in Satellite Access	1196

9.9C.1	Introduction	1196

9.9C.1.1	General Aspects of Gap-based Measurement	1196

9.9C.1.2	General Aspects of Gapless Measurement	1197

9.9C.1.3	Scheduling Availability of UE during PRS Measurement without Measurement Gaps	1198

9.9C.2	Void	1198

9.9C.3	Void	1198

9.9C.4	UE Rx-Tx time difference measurements	1198

9.9C.4.1 Introduction	1198

9.9C.4.2 Requirements Applicability	1198

9.9C.4.3	Measurement Capability	1198

9.9C.4.4	Measurement Reporting Requirements	1198

9.9C.4.5	Measurement Period Requirements	1199

9.9C.4.6	Measurement Period Requirements without Measurement Gaps	1201

9.10	CSI-RS based L3 measurements	1203

9.10.1	Introduction	1203

9.10.2	CSI-RS based intra-frequency measurements	1203

9.10.2.1	Introduction	1203

9.10.2.2	Requirements applicability	1204

9.10.2.3	Number of cells and number of CSI-RS	1205

9.10.2.3.1	Requirements for FR1	1205

9.10.2.3.2	Requirements for FR2	1205

9.10.2.4	Measurement Reporting Requirements	1205

9.10.2.4.1	Periodic Reporting	1206

9.10.2.4.2	Event-triggered Periodic Reporting	1206

9.10.2.4.3	Event Triggered Reporting	1206

9.10.2.5	Intra-frequency measurements without measurement gaps	1206

9.10.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	1208

9.10.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	1208

9.10.2.6.2	Scheduling availability of UE performing CSI-RS based measurements in FR2	1208

9.10.3	CSI-RS based Inter-frequency measurements	1209

9.10.3.1	Introduction	1209

9.10.3.2	Requirements applicability	1209

9.10.3.3	Number of cells and number of CSI-RS resources	1210

9.10.3.3.1	Requirements for FR1	1210

9.10.3.3.2	Requirements for FR2	1210

9.10.3.4	Measurements reporting requirements	1210

9.10.3.4.1	Periodic Reporting	1210

9.10.3.4.2	Event-triggered Periodic Reporting	1210

9.10.3.4.3	Event-triggered Reporting	1210

9.10.3.5	Inter-frequency measurements with measurement gaps	1211

9.10D	CSI-RS based L3 measurements for ATG	1212

9.10D.1	Introduction	1212

9.10D.2	CSI-RS based intra-frequency measurements	1213

9.10D.2.1	Introduction	1213

9.10D.2.2	Requirements applicability	1213

9.10D.2.3	Number of cells and number of CSI-RS	1214

9.10D.2.3.1	Requirements for FR1	1214

9.10D.2.4	Measurement Reporting Requirements	1214

9.10D.2.4.1	Periodic Reporting	1214

9.10D.2.4.2	Event-triggered Periodic Reporting	1214

9.10D.2.4.3	Event Triggered Reporting	1215

9.10D.2.5	Intra-frequency measurements without measurement gaps	1215

9.10D.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	1216

9.10D.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	1217

9.10D.3	CSI-RS based Inter-frequency measurements	1217

9.10D.3.1	Introduction	1217

9.10D.3.2	Requirements applicability	1217

9.10D.3.3	Number of cells and number of CSI-RS resources	1218

9.10D.3.3.1	Requirements for FR1	1218

9.10D.3.4	Measurements reporting requirements	1218

9.10D.3.4.1	Periodic Reporting	1218

9.10D.3.4.2	Event-triggered Periodic Reporting	1218

9.10D.3.4.3	Event-triggered Reporting	1218

9.10D.3.5	Inter-frequency measurements with measurement gaps	1218

9.11	NR measurements with autonomous gaps	1220

9.11.1	Introduction	1220

9.11.2	CGI identification of an NR cell with autonomous gaps	1220

9.11.3	CGI reporting delay	1221

9.11A	NR measurements with autonomous gaps for RedCap	1221

9.11A.1	Introduction	1221

9.11A.2	CGI identification of an NR cell with autonomous gaps	1222

9.11A.3	CGI reporting delay	1222

9.11A.4	CGI reporting scheduling restriction	1223

9.11D	NR measurements with autonomous gaps for ATG	1223

9.11D.1	Introduction	1223

9.11D.2	CGI identification of an NR cell with autonomous gaps	1224

9.11D.3	CGI reporting delay	1224

9.12	Measurement for Propagation Delay Compensation	1225

9.12.1	Introduction	1225

9.12.2	Requirements Applicability	1225

9.12.3	Measurement Capability	1225

9.12.4	Measurement period requirements	1225

9.12.4.1	PRS Measurement Period	1225

9.12.4.2	TRS Measurement Period	1226

9.12.5	Measurement Reporting Requirements	1227

9.12.6	Scheduling availability during measurement for Propagation Delay Compensation	1227

9.12.7	Measurement restriction for measurement for Propagation Delay Compensation	1227

9.12.8	Measurement requirement for Propagation Delay Compensation with MUSIM gaps	1228

9.13	L1-RSRP measurements for a cell with different PCI from serving cell	1228

9.13.1	Introduction	1228

9.13.2	Requirements Applicability	1228

9.13.3	Measurement Reporting Requirements	1229

9.13.3.1	Periodic Reporting	1229

9.13.3.2	Semi-Persistent Reporting	1229

9.13.3.3	Aperiodic Reporting	1229

9.13.4	L1-RSRP measurement requirements	1230

9.13.4.1	Inter-cell SSB based L1-RSRP Reporting	1230

9.13.5	Measurement restriction for L1-RSRP measurement	1232

9.13.5.1	Measurement restriction for SSB based L1-RSRP	1232

9.13.6	Scheduling availability of UE during L1-RSRP measurement	1234

9.13.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1234

9.13.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1234

9.13.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	1234

9.13.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1235

9.13.6.5	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1	1235

9.14	NR intra-frequency L1-RSRP measurements for neighbor cell	1235

9.14.1	Introduction	1235

9.14.2	Requirements Applicability	1236

9.14.3	Measurement Reporting Requirements	1236

9.14.3.1	Periodic Reporting	1236

9.14.3.2	Semi-Persistent Reporting	1236

9.14.3.3	Aperiodic Reporting	1237

9.14.4	Number of SSB frequency layers, number of cells and number of SSBs	1237

9.14.5	L1-RSRP intra-frequency measurement requirements without measurement gaps	1237

9.14.5.1	Intra-frequency SSB based L1-RSRP reporting	1237

9.14.6	Measurement restriction for L1-RSRP measurement	1240

9.14.6.1	Measurement restriction for SSB based L1-RSRP	1240

9.14.7	Scheduling availability of UE during L1-RSRP measurement	1241

9.14.7.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1241

9.14.7.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1241

9.14.7.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	1241

9.14.7.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1242

9.14.7.5	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1	1242

9.15	NR inter-frequency L1-RSRP measurements for neighbor cell	1242

9.15.1	Introduction	1242

9.15.2	Requirements Applicability	1243

9.15.3	Measurement Reporting Requirements	1243

9.15.3.1	Periodic Reporting	1243

9.15.3.2	Semi-Persistent Reporting	1244

9.15.3.3	Aperiodic Reporting	1244

9.15.4	Number of SSB frequency layers, number of cells and number of SSBs	1244

9.15.5	L1-RSRP inter-frequency measurement requirements with measurement gaps	1244

9.15.5.1	Inter-frequency SSB based L1-RSRP reporting	1244

9.15.6	L1-RSRP inter-frequency L1-RSRP measurement requirements without measurement gaps	1245

9.15.6.1	Inter-frequency L1-RSRP measurement requirements	1245

9.15.6.1.1	Inter-frequency SSB based L1-RSRP measurement	1245

9.15.6.2	Measurement restriction for inter-frequency L1-RSRP measurement	1248

9.15.6.2.1	Measurement restriction for SSB based L1-RSRP	1248

9.15.6.3	Scheduling availability of UE during inter-frequency L1-RSRP measurements	1249

9.15.6.3.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1249

9.15.6.3.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1249

9.15.6.3.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	1250

9.15.6.3.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1250

9.15.6.3.5	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1	1250

10	Measurement Performance requirements	1251

10.1	NR measurements	1251

10.1.1	Introduction	1251

10.1.2	Intra-frequency RSRP accuracy requirements for FR1	1251

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements	1251

10.1.2.1.1	Absolute SS-RSRP Accuracy	1251

10.1.2.1.2	Relative SS-RSRP Accuracy	1252

10.1.2.2	Void	1253

10.1.2.3	Intra-frequency CSI-RSRP accuracy requirements	1253

10.1.2.3.1	Absolute CSI-RSRP Accuracy	1253

10.1.2.3.2	Relative CSI-RSRP Accuracy	1254

10.1.2B	Intra-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	1255

10.1.2B.1	Intra-frequency SS-RSRP accuracy requirements	1255

10.1.2B.1.1	Absolute SS-RSRP Accuracy	1255

10.1.2C	Intra-frequency RSRP accuracy requirements for FR1 SAN	1256

10.1.2C.1	Intra-frequency SS-RSRP accuracy requirements	1256

10.1.2C.1.1	Absolute SS-RSRP Accuracy	1256

10.1.2C.1.2	Relative SS-RSRP Accuracy	1257

10.1.3	Intra-frequency RSRP accuracy requirements for FR2	1258

10.1.3.1	Intra-frequency SS-RSRP accuracy requirements	1258

10.1.3.1.1	Absolute SS-RSRP Accuracy	1258

10.1.3.1.2	Relative SS-RSRP Accuracy	1258

10.1.3.2	Void	1259

10.1.3.3	Intra-frequency CSI-RSRP accuracy requirements	1259

10.1.3.3.1	Absolute CSI-RSRP Accuracy	1259

10.1.3.3.2	Relative CSI-RSRP Accuracy	1260

10.1.3B	Intra-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	1261

10.1.3B.1	Intra-frequency SS-RSRP accuracy requirements	1261

10.1.3B.1.1	Absolute SS-RSRP Accuracy	1261

10.1.3C	Intra-frequency RSRP accuracy requirements for FR2-NTN	1261

10.1.3C.1	Intra-frequency SS-RSRP accuracy requirements	1261

10.1.3C.1.1	Absolute SS-RSRP Accuracy	1261

10.1.3C.1.2	Relative SS-RSRP Accuracy	1262

10.1.4	Inter-frequency RSRP accuracy requirements for FR1	1263

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements	1263

10.1.4.1.1	Absolute SS-RSRP Accuracy in FR1	1263

10.1.4.1.2	Relative SS-RSRP Accuracy in FR1	1264

10.1.4.2	Void	1265

10.1.4.3	Inter-frequency CSI-RSRP accuracy requirements	1265

10.1.4.3.1	Absolute CSI-RSRP Accuracy in FR1	1265

10.1.4.3.2	Relative CSI-RSRP Accuracy in FR1	1266

10.1.4B	Inter-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	1267

10.1.4B.1	Inter-frequency SS-RSRP accuracy requirements	1267

10.1.4B.1.1	Absolute SS-RSRP Accuracy in FR1	1268

10.1.4C	Inter-frequency RSRP accuracy requirements for FR1 SAN	1269

10.1.4C.1	Inter-frequency SS-RSRP accuracy requirements	1269

10.1.4C.1.1	Absolute SS-RSRP Accuracy in FR1	1269

10.1.4C.1.2	Relative SS-RSRP Accuracy in FR1	1269

10.1.5	Inter-frequency RSRP accuracy requirements for FR2	1270

10.1.5.1	Inter-frequency SS-RSRP accuracy requirements	1270

10.1.5.1.1	Absolute SS-RSRP Accuracy	1270

10.1.5.1.2	Relative SS-RSRP Accuracy	1270

10.1.5.2	Void	1271

10.1.5.3	Inter-frequency CSI-RSRP accuracy requirements	1271

10.1.5.3.1	Absolute CSI-RSRP Accuracy	1271

10.1.5.3.2	Relative CSI-RSRP Accuracy	1272

10.1.5B	Inter-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	1273

10.1.5B.1	Inter-frequency SS-RSRP accuracy requirements	1273

10.1.5B.1.1	Absolute SS-RSRP Accuracy	1273

10.1.5C	Inter-frequency RSRP accuracy requirements for FR2-NTN	1274

10.1.5C.1	Inter-frequency SS-RSRP accuracy requirements	1274

10.1.5C.1.1	Absolute SS-RSRP Accuracy	1274

10.1.5C.1.2	Relative SS-RSRP Accuracy	1274

10.1.6	RSRP Measurement Report Mapping	1275

10.1.7	Intra-frequency RSRQ accuracy requirements for FR1	1277

10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1277

10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1	1277

10.1.7.2	Intra-frequency CSI-RSRQ accuracy requirements	1278

10.1.7.2.1	Absolute CSI-RSRQ Accuracy	1278

10.1.7B	Intra-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	1279

10.1.7B.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1279

10.1.7B.1.1	Absolute SS-RSRQ Accuracy in FR1	1279

10.1.7C	Intra-frequency RSRQ accuracy requirements for FR1 SAN	1280

10.1.7C.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1280

10.1.7C.1.1	Absolute SS-RSRQ Accuracy in FR1	1280

10.1.8	Intra-frequency RSRQ accuracy requirements for FR2	1281

10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	1281

10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2	1281

10.1.8.2	Intra-frequency CSI-RSRQ accuracy requirements	1281

10.1.8.2.1	Absolute CSI-RSRQ Accuracy	1281

10.1.8B	Intra-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	1282

10.1.8B.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	1282

10.1.8B.1.1	Absolute SS-RSRQ Accuracy in FR2	1282

10.1.8C	Intra-frequency RSRQ accuracy requirements for FR2-NTN	1283

10.1.8C.1	Intra-frequency SS-RSRQ accuracy requirements in FR2-NTN	1283

10.1.8C.1.1	Absolute SS-RSRQ Accuracy in FR2-NTN	1283

10.1.9	Inter-frequency RSRQ accuracy requirements for FR1	1284

10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1284

10.1.9.1.1	Absolute SS-RSRQ Accuracy in FR1	1284

10.1.9.1.2	Relative SS-RSRQ Accuracy in FR1	1284

10.1.9.2	Inter-frequency CSI-RSRQ accuracy requirements	1285

10.1.9.2.1	Absolute CSI-RSRQ Accuracy	1285

10.1.9.2.2	Relative CSI-RSRQ Accuracy	1286

10.1.9B	Inter-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	1287

10.1.9B.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1287

10.1.9B.1.1	Absolute SS-RSRQ Accuracy in FR1	1287

10.1.9C	Inter-frequency RSRQ accuracy requirements for FR1 SAN	1288

10.1.9C.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1288

10.1.9C.1.1	Absolute SS-RSRQ Accuracy in FR1	1288

10.1.9C.1.2	Relative SS-RSRQ Accuracy in FR1	1289

10.1.10	Inter-frequency RSRQ accuracy requirements for FR2	1289

10.1.10.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	1289

10.1.10.1.1	Absolute SS-RSRQ Accuracy in FR2	1289

10.1.10.1.2	Relative SS-RSRQ Accuracy in FR2	1290

10.1.10.2	Inter-frequency CSI-RSRQ accuracy requirements	1291

10.1.10.2.1	Absolute CSI-RSRQ Accuracy	1291

10.1.10.2.2	Relative CSI-RSRQ Accuracy	1292

10.1.10B	 Inter-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	1293

10.1.10B.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	1293

10.1.10B.1.1	Absolute SS-RSRQ Accuracy in FR2	1293

10.1.10C	Inter-frequency RSRQ accuracy requirements for FR2-NTN	1294

10.1.10C.1	Inter-frequency SS-RSRQ accuracy requirements in FR2-NTN	1294

10.1.10C.1.1	Absolute SS-RSRQ Accuracy in FR2-NTN	1294

10.1.10C.1.2	Relative SS-RSRQ Accuracy in FR2-NTN	1294

10.1.11	RSRQ report mapping	1295

10.1.12	Intra-frequency SINR accuracy requirements for FR1	1296

10.1.12.1	Intra-frequency SS-SINR accuracy requirements in FR1	1296

10.1.12.1.1	Absolute SS-SINR Accuracy in FR1	1296

10.1.12.2	Intra-frequency CSI-SINR accuracy requirements in FR1	1296

10.1.12.2.1	Absolute CSI-SINR Accuracy in FR1	1296

10.1.12C	 Intra-frequency SINR accuracy requirements for FR1 SAN	1297

10.1.12C.1	Intra-frequency SS-SINR accuracy requirements in FR1	1297

10.1.12C.1.1	Absolute SS-SINR Accuracy in FR1	1297

10.1.13	Intra-frequency SINR accuracy requirements for FR2	1298

10.1.13.1	Intra-frequency SS-SINR accuracy requirements in FR2	1298

10.1.13.1.1	Absolute SS-SINR Accuracy in FR2	1298

10.1.13.2	Intra-frequency CSI-SINR accuracy requirements in FR2	1299

10.1.13.2.1	Absolute CSI-SINR Accuracy in FR2	1299

10.1.13C	Intra-frequency SINR accuracy requirements for FR2-NTN	1299

10.1.13C.1	Intra-frequency SS-SINR accuracy requirements in FR2-NTN	1299

10.1.13C.1.1	Absolute SS-SINR Accuracy in FR2-NTN	1299

10.1.14	Inter-frequency SINR accuracy requirements for FR1	1300

10.1.14.1	Inter-frequency SS-SINR accuracy requirements in FR1	1300

10.1.14.1.1	Absolute SS-SINR Accuracy in FR1	1300

10.1.14.1.2	Relative SS-SINR Accuracy in FR1	1301

10.1.14.2	Inter-frequency CSI-SINR accuracy requirements in FR1	1302

10.1.14.2.1	Absolute CSI-SINR Accuracy in FR1	1302

10.1.14.2.2	Relative CSI-SINR Accuracy in FR1	1303

10.1.14C	 Inter-frequency SINR accuracy requirements for FR1 SAN	1304

10.1.14C.1	Inter-frequency SS-SINR accuracy requirements in FR1	1304

10.1.14C.1.1	Absolute SS-SINR Accuracy in FR1	1304

10.1.14C.1.2	Relative SS-SINR Accuracy in FR1	1305

10.1.15	Inter-frequency SINR accuracy requirements for FR2	1306

10.1.15.1	Inter-frequency SS-SINR accuracy requirements in FR2	1306

10.1.15.1.1	Absolute SS-SINR Accuracy in FR2	1306

10.1.15.1.2	Relative SS-SINR Accuracy in FR2	1306

10.1.15.2	Inter-frequency CSI-SINR accuracy requirements in FR2	1307

10.1.15.2.1	Absolute CSI-SINR Accuracy in FR2	1307

10.1.15.2.2	Relative CSI-SINR Accuracy in FR2	1308

10.1.15C	Inter-frequency SINR accuracy requirements for FR2-NTN	1309

10.1.15C.1	Inter-frequency SS-SINR accuracy requirements in FR2-NTN	1309

10.1.15C.1.1	Absolute SS-SINR Accuracy in FR2-NTN	1309

10.1.15C.1.2	Relative SS-SINR Accuracy in FR2-NTN	1309

10.1.16	SINR report mapping	1310

10.1.16.1	SS-SINR and CSI-SINR measurement report mapping	1310

10.1.17	Power Headroom	1311

10.1.17.1	Power Headroom Report	1311

10.1.17.1.1	Power Headroom Report Mapping	1311

10.1.18	PCMAX,c,f	1311

10.1.18.1	Report Mapping	1311

10.1.19	L1-RSRP accuracy requirements for FR1	1312

10.1.19.1	SSB based L1-RSRP accuracy requirements	1312

10.1.19.1.1	Absolute Accuracy	1312

10.1.19.1.2	Relative Accuracy	1313

10.1.19.2	CSI-RS based L1-RSRP accuracy requirements	1314

10.1.19.2.1	Absolute Accuracy	1314

10.1.19.2.2	Relative Accuracy	1315

10.1.19C	L1-RSRP accuracy requirements for FR1 SAN	1316

10.1.19C.1	SSB based L1-RSRP accuracy requirements	1316

10.1.19C.1.1	Absolute Accuracy	1316

10.1.19C.1.2	Relative Accuracy	1317

10.1.19C.2	CSI-RS based L1-RSRP accuracy requirements	1317

10.1.19C.2.1	Absolute Accuracy	1317

10.1.19C.2.2	Relative Accuracy	1318

10.1.19D	LTM Intra-frequency L1-RSRP accuracy requirements for FR1	1319

10.1.19D.1	SSB based intra-frequency L1-RSRP accuracy requirements	1319

10.1.19D.1.1	Absolute Accuracy	1319

10.1.19D.1.2	Relative Accuracy	1320

10.1.19E	  LTM Inter-frequency L1-RSRP accuracy requirements for FR1	1320

10.1.19E.1	SSB based Inter-frequency L1-RSRP accuracy requirements	1320

10.1.19E.1.1	Absolute Accuracy	1320

10.1.19E.1.2	Relative Accuracy	1321

10.1.20	L1-RSRP accuracy requirements for FR2	1322

10.1.20.1	SSB based L1-RSRP accuracy requirements	1322

10.1.20.1.1	Absolute Accuracy	1322

10.1.20.1.2	Relative Accuracy	1323

10.1.20.2	CSI-RS based L1-RSRP accuracy requirements	1324

10.1.20.2.1	Absolute Accuracy	1324

10.1.20.2.2	Relative Accuracy	1324

10.1.20A	  LTM Intra-frequency L1-RSRP accuracy requirements for FR2	1325

10.1.20A.1	SSB based intra-frequency L1-RSRP accuracy requirements	1325

10.1.20A.1.1	Absolute Accuracy	1325

10.1.20A.1.2	Relative Accuracy	1326

10.1.20B	  LTM Inter-frequency L1-RSRP accuracy requirements for FR2	1327

10.1.20B.1	SSB based inter-frequency L1-RSRP accuracy requirements	1327

10.1.20B.1.1	Absolute Accuracy	1327

10.1.20B.1.2	Relative Accuracy	1327

10.1.20C	L1-RSRP accuracy requirements for FR2-NTN	1328

10.1.20C.1	SSB based L1-RSRP accuracy requirements	1328

10.1.20C.1.1	Absolute Accuracy	1328

10.1.20C.1.2	Relative Accuracy	1329

10.1.20C.2	CSI-RS based L1-RSRP accuracy requirements	1329

10.1.20C.2.1	Absolute Accuracy	1329

10.1.20C.2.2	Relative Accuracy	1330

10.1.21	SFTD accuracy requirements	1331

10.1.21.1	SFTD acuracy requirements for NE-DC	1331

10.1.21.2	SFTD acuracy requirements for NR-DC	1332

10.1.21.3	Inter-frequency SFTD acuracy requirements	1333

10.1.22	CLI measurement accuracy requirements	1335

10.1.22.1	SRS-RSRP	1335

10.1.22.1.1	SRS-RSRP Accuracy	1335

10.1.22.1.2	SRS-RSRP report mapping	1336

10.1.22.2	CLI-RSSI	1337

10.1.22.2.1	CLI-RSSI Accuracy	1337

10.1.22.2.2	CLI-RSSI report mapping	1338

10.1.23	RSTD Measurements	1338

10.1.23.1	Introduction	1338

10.1.23.2	Measurement Accuracy Requirements	1338

10.1.23.3	Report mapping	1346

10.1.23.3.1	Absolute DL RSTD Measurement Reporting	1346

10.1.23.3.2	Differential Reporting for DL RSTD Measurement	1349

10.1.23.3.3	Additional Path Report Mapping for DL RSTD	1352

10.1.23A	RSTD Measurements Based on PRS Aggregation	1356

10.1.23A.1	Introduction	1356

10.1.23A.3	Report Mapping	1363

10.1.23A.3.1	Absolute DL RSTD Measurement Reporting	1363

10.1.23A.3.2	Differential Reporting for DL RSTD Measurement	1363

10.1.23A.3.3	Additional Path Report Mapping for DL RSTD	1363

10.1.24	PRS-RSRP Measurements	1363

10.1.24.1	Introduction	1363

10.1.24.2	Measurement Accuracy Requirements	1364

10.1.24.2.1	Absolute PRS-RSRP accuracy	1364

10.1.24.2.2	Relative PRS RSRP accuracy	1368

10.1.24.3	Report mapping	1372

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	1372

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	1373

10.1.24A	PRS-RSRP Measurements Based on PRS Aggregation	1375

10.1.24A.1	Introduction	1375

10.1.24A.2	Measurement Accuracy Requirements	1376

10.1.24A.2.1	Absolute PRS RSRP Accuracy Requirement	1376

10.1.24A.2.2	Relative PRS RSRP Accuracy Requirement	1376

10.1.24A.3	Report Mapping	1376

10.1.24A.3.1	Absolute PRS-RSRP Measurement Report Mapping	1376

10.1.24A.3.2	Differential Report Mapping for PRS-RSRP Measurement	1376

10.1.25	UE Rx-Tx Time Difference Measurements	1376

10.1.25.1	Introduction	1376

10.1.25.2	Measurement Accuracy Requirements	1376

10.1.25.3	Report mapping	1388

10.1.25.3.1	Absolute UE Rx-Tx Measurement Report Mapping	1388

10.1.25.3.2	Differential UE Rx-Tx Measurement Report Mapping	1391

10.1.25.3.3	Additional Path Report Mapping for UE Rx-Tx Time Difference	1394

10.1.25A	UE Rx-Tx Time Difference Measurement Based on PRS Aggregation	1397

10.1.25A.1	Introduction	1397

10.1.25A.2	Measurement Accuracy Requirements	1398

10.1.25A.3	Report mapping	1414

10.1.25C	UE Rx-Tx Time Difference Measurements in Satellite Accesss	1414

10.1.25C.1	Introduction	1414

10.1.25C.2	Measurement Accuracy Requirements	1414

10.1.25C.3	Report mapping	1415

10.1.26	FR2 P-MPR report	1415

10.1.26.1	Report mapping	1415

10.1.27	L1-SINR accuracy requirements for FR1	1416

10.1.27.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	1416

10.1.27.1.1	Absolute Accuracy	1416

10.1.27.1.2	Relative Accuracy	1417

10.1.27.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	1418

10.1.27.2.1	Absolute Accuracy	1418

10.1.27.2.2	Relative Accuracy	1420

10.1.27.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	1422

10.1.27.3.1	Absolute Accuracy	1422

10.1.27.3.2	Relative Accuracy	1424

10.1.28	L1-SINR accuracy requirements for FR2	1426

10.1.29	Intra-frequency RSRQ accuracy requirements under CCA	1433

10.1.29.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1433

10.1.29.1.1	Absolute SS-RSRQ Accuracy	1433

10.1.30	Inter-frequency RSRQ accuracy requirements under CCA	1433

10.1.30.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1433

10.1.30.1.1	Absolute SS-RSRQ Accuracy	1433

10.1.30.1.2	Relative SS-RSRQ Accuracy	1434

10.1.31	Intra-frequency SINR accuracy requirements under CCA	1435

10.1.31.1	Intra-frequency SS-SINR accuracy requirements in FR1	1435

10.1.31.1.1	Absolute SS-SINR Accuracy	1435

10.1.32	Inter-frequency SINR accuracy requirements under CCA	1435

10.1.32.1	Inter-frequency SS-SINR accuracy requirements in FR1	1435

10.1.32.1.1	Absolute SS-SINR Accuracy	1435

10.1.32.1.2	Relative SS-SINR Accuracy	1436

10.1.33	L1-RSRP accuracy requirements under CCA	1437

10.1.33.1	SSB based L1-RSRP accuracy requirements in FR1	1437

10.1.33.1.1	Absolute Accuracy	1437

10.1.33.1.2	Relative Accuracy	1437

10.1.34	RSSI measurements under CCA	1438

10.1.34.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	1438

10.1.34.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	1438

10.1.34.3	RSSI measurement report mapping	1438

10.1.35	Channel occupancy measurements under CCA	1439

10.1.35.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	1439

10.1.35.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	1439

10.1.36	Intra-frequency RSRP accuracy requirements under CCA	1439

10.1.36.1	Intra-frequency SS-RSRP accuracy requirements in FR1	1439

10.1.36.1.1	Absolute SS-RSRP Accuracy	1439

10.1.36.1.2	Relative SS-RSRP Accuracy	1440

10.1.37	Inter-frequency RSRP accuracy requirements under CCA	1440

10.1.37.1	Inter-frequency SS-RSRP accuracy requirements in FR1	1440

10.1.37.1.1	Absolute SS-RSRP	1440

10.1.37.1.2	Relative SS-RSRP Accuracy	1441

10.1.38	PRS-RSRPP Measurements	1442

10.1.38.1	Introduction	1442

10.1.38.2	Measurement Accuracy Requirements	1442

10.1.38.2.1	Absolute PRS RSRPP accuracy	1442

10.1.38.3	Report mapping	1446

10.1.38.3.1	Absolute PRS-RSRPP Measurement Report Mapping	1446

10.1.38.3.2	Differential Report Mapping for PRS-RSRPP Measurement	1447

10.1.38A	PRS-RSRPP Measurements Based on PRS Aggregation	1448

10.1. 38A.1	Introduction	1448

10.1.38A.2	Measurement Accuracy Requirements	1449

10.1.38A.2.1	Absolute PRS RSRPP accuracy	1449

10.1.38A.3	Report mapping	1449

10.1.38A.3.1	Absolute PRS-RSRPP Measurement Report Mapping	1449

10.1.38A.3.2	Differential Report Mapping for PRS-RSRPP Measurement	1449

10.1.39	UE Rx-Tx time difference measurements for RTT-based PDC	1449

10.1.39.1	Void	1449

10.1.39.2	 Measurement Accuracy Requirements for PRS	1449

10.1.39.3	 Measurement Accuracy Requirements for TRS	1452

10.1.40	Void	1456

10.1.41	FR1 DPC report	1456

10.1.41.1	Report mapping	1456

10.1.42	TDCP Measurement Report Mapping	1456

10.1.43	DL-RSCPD Measurements	1458

10.1.43.1	Introduction	1458

10.1.43.2.1	Measurement Accuracy Requirements	1458

10.1.43.3	Report Mapping	1465

10.1.43.3.1	Absolute DL RSCPD Measurement Reporting	1465

10.1.44	DL-RSCP Measurements	1466

10.1.44.1	Introduction	1466

10.1.44.2	Measurement Accuracy Requirements	1466

10.1.44.3	Report Mapping	1474

10.1.44.3.1	Relative DL RSCP Measurement Reporting	1474

10.1A	NR measurements for RedCap	1475

10.1A.1	Introduction	1475

10.1A.2	Intra-frequency RSRP accuracy requirements for FR1	1475

10.1A.2.1	Intra-frequency SS-RSRP accuracy requirements	1475

10.1A.2.1.1	Absolute SS-RSRP Accuracy	1475

10.1A.2.1.2	Relative SS-RSRP Accuracy	1476

10.1A.3	Intra-frequency RSRP accuracy requirements for FR2	1477

10.1A.3.1	Intra-frequency SS-RSRP accuracy requirements	1477

10.1A.3.1.1	Absolute SS-RSRP Accuracy	1477

10.1A.3.1.2	Relative SS-RSRP Accuracy	1477

10.1A.4	Inter-frequency RSRP accuracy requirements for FR1	1477

10.1A.4.1	Inter-frequency SS-RSRP accuracy requirements	1477

10.1A.4.1.1	Absolute SS-RSRP Accuracy in FR1	1477

10.1A.4.1.2	Relative SS-RSRP Accuracy in FR1	1478

10.1A.5	Inter-frequency RSRP accuracy requirements for FR2	1479

10.1A.5.1	Inter-frequency SS-RSRP accuracy requirements	1479

10.1A.5.1.1	Absolute SS-RSRP Accuracy	1479

10.1A.5.1.2	Relative SS-RSRP Accuracy	1479

10.1A.6	Intra-frequency RSRQ accuracy requirements for FR1	1479

10.1A.6.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1479

10.1A.6.1.1	Absolute SS-RSRQ Accuracy in FR1	1479

10.1A.7	Intra-frequency RSRQ accuracy requirements for FR2	1480

10.1A.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	1480

10.1A.7.1.1	Absolute SS-RSRQ Accuracy in FR2	1480

10.1A.8	Inter-frequency RSRQ accuracy requirements for FR1	1480

10.1A.8.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1480

10.1A.8.1.1	Absolute SS-RSRQ in FR1	1480

10.1A.8.1.2	Relative SS-RSRQ Accuracy in FR1	1481

10.1A.9	Inter-frequency RSRQ accuracy requirements for FR2	1482

10.1A.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	1482

10.1A.9.1.1	Absolute SS-RSRQ Accuracy in FR2	1482

10.1A.9.1.2	Relative SS-RSRQ Accuracy in FR2	1482

10.1A.10	 Intra-frequency SINR accuracy requirements for FR1	1482

10.1A.10.1	Intra-frequency SS-SINR accuracy requirements in FR1	1482

10.1A.10.1.1	Absolute SS-SINR Accuracy in FR1	1482

10.1A.11	Intra-frequency SINR accuracy requirements for FR2	1483

10.1A.11.1	Intra-frequency SS-SINR accuracy requirements in FR2	1483

10.1A.11.1.1	Absolute SS-SINR Accuracy in FR2	1483

10.1A.12 	Inter-frequency SINR accuracy requirements for FR1	1483

10.1A.12.1	Inter-frequency SS-SINR accuracy requirements in FR1	1483

10.1A.12.1.1	Absolute SS-SINR Accuracy in FR1	1483

10.1A.12.1.2	Relative SS-SINR Accuracy in FR1	1484

10.1A.13	 Inter-frequency SINR accuracy requirements for FR2	1485

10.1A.13.1	Inter-frequency SS-SINR accuracy requirements in FR2	1485

10.1A.13.1.1	Absolute SS-SINR Accuracy in FR2	1485

10.1A.13.1.2	Relative SS-SINR Accuracy in FR2	1485

10.1A.14	L1-RSRP accuracy requirements for FR1	1485

10.1A.14.1	SSB based L1-RSRP accuracy requirements	1485

10.1A.14.1.1	Absolute Accuracy	1485

10.1A.14.1.2	Relative Accuracy	1486

10.1A.14.2	CSI-RS based L1-RSRP accuracy requirements	1487

10.1A.14.2.1	Absolute Accuracy	1487

10.1A.14.2.2	Relative Accuracy	1488

10.1A.15	 L1-RSRP accuracy requirements for FR2	1489

10.1A.15.1	SSB based L1-RSRP accuracy requirements	1489

10.1A.15.1.1	Absolute Accuracy	1489

10.1A.15.1.2	Relative Accuracy	1489

10.1A.15.2	CSI-RS based L1-RSRP accuracy requirements	1489

10.1A.15.2.1	Absolute Accuracy	1489

10.1A.15.2.2	Relative Accuracy	1489

10.1A.16	RSTD Measurements for RedCap Positioning	1490

10.1A.16.1	Introduction	1490

10.1A.16.2	Measurement Accuracy Requirements	1490

10.1A.16.2.1	Accuracy requirement for RSTD measurement without RX FH	1490

10.1A.16.2.2	Accuracy requirement for RSTD measurement with RX FH	1497

10.1A.16.3	Report Mapping	1508

10.1A.16.3.1	Absolute DL RSTD Measurement Reporting	1508

10.1A.16.3.2	Differential Reporting for DL RSTD Measurement	1508

10.1A.16.3.3	Additional Path Report Mapping for DL RSTD	1508

10.1A.17	PRS-RSRP Measurements for RedCap positioning	1508

10.1A.17.1	Introduction	1508

10.1A.17.2	Measurement Accuracy Requirements	1508

10.1A.17.2.1	Absolute PRS RSRP Accuracy Requirement	1508

10.1A.17.2.2	Relative PRS RSRP Accuracy Requirement	1511

10.1A.17.3	Report Mapping	1511

10.1A.17.3.1	Absolute PRS-RSRP Measurement Report Mapping	1511

10.1A.17.3.2	Differential Report Mapping for PRS-RSRP Measurement	1511

10.1A.18	  UE Rx-Tx Time Difference Measurements for RedCap Positioning	1512

10.1A.18.1	Introduction	1512

10.1A.18.2	Measurement Accuracy Requirements	1512

10.1A.18.2.1	UE Rx-Tx Accuracy Requirement for 2RX RedCap UE without FH	1512

10.1A.18.2.2	UE Rx-Tx Accuracy Requirement for 1RX RedCap UE without FH	1513

10.1A.18.2.3	UE Rx-Tx Accuracy Requirement for 2RX RedCap UE with FH	1518

10.1A.18.3	Report mapping	1528

10.1A.18.3.1	Absolute UE Rx-Tx Measurement Report Mapping	1528

10.1A.18.3.2	Differential UE Rx-Tx Measurement Report Mapping	1528

10.1A.18.3.3	Additional Path Report Mapping for UE Rx-Tx Time Difference	1528

10.1A.19	PRS-RSRPP Measurements for RedCap Positioning	1528

10.1A.19.1	Introduction	1528

10.1A.19.2	Measurement Accuracy Requirements	1529

10.1A.19.2.1	Absolute PRS RSRPP accuracy	1529

10.1A.19.3	Report mapping	1531

10.1A.19.3.1	Absolute PRS-RSRPP Measurement Report Mapping	1531

10.1A.19.3.2	Differential Report Mapping for PRS-RSRPP Measurement	1532

10.2	E-UTRAN measurements	1532

10.2.1	Introduction	1532

10.2.2	E-UTRAN RSRP measurements	1532

10.2.3	E-UTRAN RSRQ measurements	1532

10.2.4	E-UTRAN RSTD measurements	1532

10.2.5	E-UTRAN RS-SINR measurements	1533

10.2.6	E-UTRAN RSRP measurements for CA/DC Idle Mode Measurements	1533

10.2.7	E-UTRAN RSRQ measurements for CA/DC Idle Mode Measurements	1533

10.2A	E-UTRAN measurements for RedCap	1534

10.2A.1	Introduction	1534

10.2A.2	E-UTRAN RSRP measurements	1534

10.2A.3	E-UTRAN RSRQ measurements	1534

10.2A.4	E-UTRAN RS-SINR measurements	1535

10.3	UTRAN FDD Measurements	1535

10.3.1	UTRAN FDD CPICH RSCP	1535

10.3.2	UTRAN FDD CPICH Ec/No	1536

10.4	V2X measurements	1536

10.4.1	Introduction	1536

10.4.2	Intra-frequency PSBCH-RSRP accuracy requirements for FR1	1536

10.4.2.1	PSBCH-RSRP Absolute Accuracy	1536

10.4.2.2	PSBCH-RSRP Relative Accuracy	1537

10.4.2A	Intra-frequency PSBCH-RSRP accuracy requirements for FR1 under CCA	1538

10.4.2A.1	PSBCH-RSRP Absolute Accuracy	1538

10.4.2A.2	PSBCH-RSRP Relative Accuracy	1538

10.4.3	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1	1539

10.4.3.1	Absolute SL-RSSI Accuracy	1539

10.4.3A	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1 under CCA	1539

10.4.3A.1	Absolute SL-RSSI Accuracy	1539

10.4.4	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1	1540

10.4.4.1	Absolute L1 SL-RSRP Accuracy	1540

10.4.4A	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1 under CCA	1540

10.4.4A.1	Absolute L1 SL-RSRP Accuracy	1540

10.4.5	Intra-Frequency Discovery Signal Measurement Accuracy Requirements	1541

10.4.5.1	Absolute Discovery Signal Measurement Accuracy	1541

10.4A	NR Sidelink Measurements for Positioning	1542

10.4A.1	Introduction	1542

10.4A.2	SL RSTD measurements	1542

10.4A.2.1	Measurement Report Mapping	1542

10.4A.2.1.1	Absolute SL RSTD Measurement Reporting	1542

10.4A.2.2	Measurement Accuracy Requirements	1543

10.4A.3	SL PRS-RSRP measurements	1545

10.4A.3.1	Measurement Report Mapping	1545

10.4A.3.1.1	Absolute SL PRS-RSRP Measurement Report Mapping	1545

10.4A.3.2	Measurement Accuracy Requirements	1546

10.4A.3.2.1	Absolute SL PRS-RSRP accuracy	1546

10.4A.4	SL Rx-Tx measurements	1547

10.4A.4.1	Measurement Report Mapping	1547

10.4A.4.1.1	Absolute SL Rx-Tx Measurement Report Mapping	1547

10.4A.4.2	Measurement Accuracy	1549

10.4A.5	SL PRS-RSRPP measurements	1550

10.4A.5.1	Measurement Report Mapping	1550

10.4A.5.1.1	Absolute SL PRS-RSRPP Measurement Report Mapping	1550

10.4A.5.2	Measurement Accuracy	1551

10.4A.5.2.1	Introduction	1551

10.4A.5.2.2	Measurement Accuracy Requirements	1552

10.4A.5.2.2.2	Absolute SL PRS-RSRPP accuracy	1552

10.4A.6	SL AoA measurements	1553

10.4A.6.1	Measurement Report Mapping	1553

10.4A.6.1.1	Absolute SL AoA Measurement Report Mapping	1553

10.4A.7	SL RTOA measurements	1554

10.4A.7.1	Measurement Report Mapping	1554

10.4A.7.1.1	Absolute SL RTOA Measurement Report Mapping	1554

11	Void	1556

12	V2X Requirements	1557

12.1	Introduction	1557

12.2	UE Transmit Timing	1557

12.2.1	Introduction	1557

12.2.2	GNSS as synchronization reference source	1558

12.2.3	NR Cell as synchronization reference source	1558

12.2.4	E-URTAN Cell as synchronization reference source	1558

12.2.5	SyncRef UE as synchronization reference source	1559

12.3	Initiation/Cease of SLSS Transmissions	1559

12.3.1	Introduction	1559

12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	1559

12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	1560

12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	1561

12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	1561

12.3A	Initiation/Cease of SLSS Transmissions with CCA	1562

12.3A.1	Introduction	1562

12.3A.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	1562

12.3A.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	1562

12.3A.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	1562

12.3A.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	1563

12.4	Selection / Reselection of V2X Synchronization Reference Source	1563

12.4A	Selection / Reselection of Sidelink Synchronization Reference Source with CCA	1565

12.5	L1 SL-RSRP measurements	1567

12.5.1	Introduction	1567

12.5.2	SL-RSRP measurements	1567

12.6	Congestion Control measurements	1568

12.7	Interruption	1568

12.7.1	Interruptions to WAN due to V2X Sidelink Communication	1568

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change	1568

12.7.3	Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink	1570

12.7.4	Interruptions to WAN at transitions between active and non-active during SL-DRX	1570

12.7.5	Interruptions to V2X sidelink at transitions between active and non-active during DRX	1571

12.7.6	Interruptions to V2X sidelink due to Active BWP switching Requirement	1571

12.7.7	Interruptions to WAN due to SyncRef UE detection and/or Sensing during SL DRX off duration	1572

12.7.8	Interruptions at NR sidelink discovery configuration	1572

12.7.9	Interruptions to WAN due to sidelink carrier addition/release	1572

12.8	Reliability of GNSS signal	1573

12.9	Scheduling availability	1573

12.9.1	Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink	1573

12.9.2	Scheduling availability of UE switching between Uu uplink  and V2X sidelink	1573

12.10	Selection / Reselection of relay UE	1574

12.10.1	Introduction	1574

12.10.2	Selection / Reselection of relay UE	1574

12.11	Component Carrier Addition and Release Delay for Sidelink Carrier Aggregation	1574

12.12	Selection / Reselection of Synchronization Reference Source for NR SL Carrier Aggregation	1575

12A	NR Sidelink Measurements for Positioning	1576

12A.1	Introduction	1576

12A.2	SL RSTD measurements	1577

12A.2.1	Introduction	1577

12A.2.3	Measurement Capability	1577

12A.2.4	Measurement Reporting Requirements	1577

12A.2.5	Measurements Period Requirements	1577

12A.3	SL PRS-RSRP measurements	1578

12A.3.1	Introduction	1578

12A.3.2	Requirements Applicability	1579

12A.3.4	Measurement Reporting Requirements	1579

12A.3.5	Measurements Period Requirements	1579

12A.4	SL Rx-Tx measurements	1580

12A.4.1	Introduction	1580

12A.4.2	Requirements Applicability	1580

12A.4.3	Measurement Capability	1580

12A.4.4	Measurement Reporting Requirements	1580

12A.4.5	Measurement Period Requirements	1581

12A.5	SL PRS-RSRPP measurements	1582

12A.5.1	Introduction	1582

12A.5.2	Requirements Applicability	1582

12A.5.3	Measurement Capability	1582

12A.5.4	Measurement Reporting Requirements	1582

12A.5.5	Measurement Period Requirements	1582

12A.6	SL AoA measurements	1583

12A.6.1	Introduction	1583

12A.6.2	Requirements Applicability	1583

12A.6.3	Measurement Capability	1583

12A.6.4	Measurement Reporting Requirements	1583

12A.6.5	Measurement Period Requirements	1584

12A.7	SL RTOA measurements	1584

12A.7.1	Introduction	1584

12A.7.2	Requirements Applicability	1585

12A.7.3	Measurement Capability	1585

12A.7.4	Measurement Reporting Requirements	1585

12A.7.5	Measurement Period Requirements	1585

13	Measurement Performance Requirements for NR gNB	1586

13.1	UL-RTOA	1586

13.1.1	Report mapping	1586

13.1.1A	Additional Path Report Mapping for UL-RTOA	1590

13.2	gNB Rx-Tx time difference	1593

13.2.1	Report mapping	1593

13.2.1A	Additional Path Report Mapping for gNB Rx-Tx	1597

13.2.2	Measurement Accuracy Requirements	1600

13.2.2.1	Introduction	1600

13.3	UL SRS RSRP measurement	1602

13.3.1	Report mapping	1602

13.3.2	Measurement accuracy requirements	1602

13.3.2.1	Introduction	1602

13.3.2.2	Requirements	1603

13.4	AoA/ZoA	1603

13.4.1	Report mapping	1603

13.5	Timing advance (TADV)	1604

13.5.1	Report mapping	1604

13.6	UL SRS RSRPP measurement	1605

13.6.1	Report mapping	1605

13.7	gNB Rx-Tx time difference measurements for RTT-based PDC	1605

13.7.1	Report mapping	1605

13.7.2	Measurement Accuracy Requirements	1606

13.7.2.1	Introduction	1606

13.7.2.2	Requirements	1606

13.8	UL-RSCP measurement	1607

13.8.1	Report mapping	1607

Annex A (normative):	Test Cases	1608

A.1	Purpose of annex	1608

A.2	Requirement classification for statistical testing	1608

A.2.1	Types of requirements in TS 38.133	1608

A.2.1.1	Time and delay requirements on UE higher layer actions	1608

A.2.1.2	Measurements of power levels, relative powers and time	1609

A.2.1.3	Implementation requirements	1609

A.2.1.4	Physical layer timing requirements	1609

A.2.1.5	Requirements under CCA	1609

A.3	RRM test configurations	1610

A.3.1	Reference measurement channels	1610

A.3.1.1	PDSCH	1610

A.3.1.1.1	FDD	1610

A.3.1.1.2	TDD	1611

A.3.1.2	CORESET for RMSI scheduling	1614

A.3.1.2.1	FDD	1614

A.3.1.2.2	TDD	1615

A.3.1.3	CORESET for RMC scheduling	1617

A.3.1.3.1	FDD	1617

A.3.1.3.2	TDD	1619

A.3.1.4	TDD UL/DL configuration	1623

A.3.1A	Reference measurement channels under CCA	1625

A.3.1A.1	PDSCH	1625

A.3.1A.1.1	TDD	1625

A.3.1A.2	CORESET for RMSI scheduling	1626

A.3.1A.2.1	TDD	1626

A.3.1A.3	CORESET for RMC scheduling	1627

A.3.1A.3.1	TDD	1627

A.3.1A.4	TDD UL/DL configuration	1627

A.3.1A.5	RMC burst transmission model	1628

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	1628

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	1628

A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup	1629

A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as CORESET	1629

A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)	1630

A.3.2.2	Void	1631

A.3.3	Reference DRX configurations	1631

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	1631

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	1631

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	1631

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity	1632

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity	1632

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms	1632

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	1632

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	1633

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms	1633

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms and TAT = 500 ms	1633

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity	1633

A.3.3.12	DRX Configuration 12: DRX cycle = 640 ms and TAT = Infinity	1634

A.3.3.13	DRX Configuration X1: DRX cycle = 80 ms and TAT = Infinity	1634

A.3.3.14	DRX Configuration 14: DRX cycle = 160 ms and TAT = Infinity	1634

A.3.4	Test Cases with Different Channel Bandwidths	1634

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	1634

A.3.4.1.1	Introduction	1634

A.3.4.1.2	Principle of testing	1635

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	1635

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	1635

A.3.5.1.1	Introduction	1635

A.3.5.1.2	Principle of Testing	1635

A.3.6	Antenna configurations	1635

A.3.6.1	Antenna configurations for FR1	1635

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	1635

A.3.6.1.1.1	Introduction	1635

A.3.6.1.1.2	Principle of testing	1635

A.3.6.1.2	Antenna connection for 8 Rx capable UEs	1638

A.3.6.1.2.1	Introduction	1638

A.3.6.1.2.2	Principle of testing	1638

A.3.6.2	Antenna configurations for FR2	1640

A.3.6A	Antenna configurations with unlicensed bands	1640

A.3.6A.1	Antenna configurations for FR1	1640

A.3.6A.1.1	Antenna connection for 4 Rx capable UEs	1640

A.3.6A.1.1.1	Introduction	1640

A.3.6A.1.1.2	Principle of testing	1640

A.3.7	EN-DC test setup	1642

A.3.7.1	Introduction	1642

A.3.7.2	E-UTRAN Serving Cell Parameters	1642

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	1642

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	1643

A.3.7A	NR FR1-FR2 test setup	1644

A.3.7B	EN-DC test setup with unlicensed bands	1644

A.3.7B.1	Introduction	1644

A.3.7B.2	E-UTRAN Serving Cell Parameters	1644

A.3.7B.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	1644

A.3.7C	LTE-FR1/FR2 test setup	1645

A.3.7D	NE-DC test setup	1645

A.3.7D.1	Introduction	1645

A.3.7D.2	E-UTRAN Serving Cell Parameters	1645

A.3.7D.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	1645

A.3.7D.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	1645

A.3.8	PRACH configurations	1646

A.3.8.1	Introduction	1646

A.3.8.2	PRACH configurations in FR1	1646

A.3.8.2.1	FR1 PRACH configuration 1	1646

A.3.8.2.2	FR1 PRACH configuration 2	1646

A.3.8.2.3	FR1 PRACH configuration 3	1647

A.3.8.2.4	FR1 PRACH configuration 4	1648

A.3.8.2.5	FR1 PRACH configuration 5	1648

A.3.8.2.6	FR1 PRACH configuration 6	1649

A.3.8.3	PRACH configurations in FR2	1649

A.3.8.3.1	FR2 PRACH configuration 1	1649

A.3.8.3.2	FR2 PRACH configuration 2	1650

A.3.8.3.3	FR2 PRACH configuration 3	1651

A.3.8.3.4	FR2 PRACH configuration 4	1651

A.3.8.3.5	FR2 PRACH configuration 5	1652

A.3.8.3.6	FR2 PRACH configuration 6	1652

A.3.8A	PRACH configurations under CCA	1653

A.3.8A.1	Introduction	1653

A.3.8A.2	PRACH configurations in FR1	1653

A.3.8A.2.1	FR1 PRACH configuration 1 under CCA	1653

A.3.8A.2.2	FR1 PRACH configuration 2 under CCA	1654

A.3.9	BWP configurations	1655

A.3.9.1	Introduction	1655

A.3.9.2	Downlink BWP configurations	1655

A.3.9.2.1	Initial BWP	1655

A.3.9.2.2	Dedicated BWP	1656

A.3.9.3	Uplink BWP configurations	1656

A.3.9.3.1	Initial BWP	1656

A.3.9.3.2	Dedicated BWP	1657

A.3.9A	BWP configurations for RedCap	1657

A.3.9A.1	Introduction	1657

A.3.9A.2	Downlink BWP configurations	1657

A.3.9A.2.1	Dedicated BWP	1657

A.3.9A.3	Uplink BWP configurations	1658

A.3.9A.3.1	Dedicated BWP	1658

A.3.10	SSB Configurations	1658

A.3.10.1	SSB Configurations for FR1	1658

A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1658

A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz	1660

A.3.10.1.6	SSB pattern 6 in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 40 MHz	1660

A.3.10.1.7	SSB pattern 7 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1660

A.3.10.1.8	SSB pattern 8 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz	1661

A.3.10.1.9	SSB pattern 9 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1661

A.3.10.1.10	SSB pattern 10 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz	1661

A.3.10.1.11	SSB pattern 11 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1662

A.3.10.1.12	SSB pattern 12 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz	1662

A.3.10.1.13	SSB pattern 13 in FR1: SSB allocation for SSB SCS=15 kHz in 3 MHz	1662

A.3.10.2	SSB Configurations for FR2	1663

A.3.10.2.1	SSB pattern 1 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1663

A.3.10.2.2	SSB pattern 2 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1663

A.3.10.2.3	SSB pattern 3 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1664

A.3.10.2.4	SSB pattern 4 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1664

A.3.10.2.5	SSB pattern 5 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1664

A.3.10.2.6	SSB pattern 6 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1665

A.3.10.2.7	SSB pattern 7 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1665

A.3.10.2.8	SSB pattern 8 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1665

A.3.10.2.9	SSB pattern 9 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1666

A.3.10.2.10	SSB pattern 10 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1666

A.3.10.2.19	SSB pattern 19 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1670

A.3.10.2.20	SSB pattern 20 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1671

A.3.10.2.21	SSB pattern 21 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1671

A.3.10.2.22	SSB pattern 22 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1672

A.3.10.2.23	SSB pattern 23 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1672

A.3.10.2.24	SSB pattern 24 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1672

A.3.10.2.25	SSB pattern 25 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1673

A.3.10.2.26	SSB pattern 26 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1673

A.3.10.2.27	SSB pattern 27 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1674

A.3.10A	SSB Configurations under CCA	1674

A.3.10A.1	SSB Configurations under CCA for FR1	1674

A.3.10A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1674

A.3.10A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1675

A.3.10A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1675

A.3.10A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1676

A.3.10B	SSB Configurations for RedCap	1676

A.3.10B.1	SSB Configurations for FR1	1676

A.3.10B.1.1	SSB pattern 1 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1676

A.3.10B.1.2	SSB pattern 2 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1677

A.3.10B.1.3	SSB pattern 3 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 20 MHz	1677

A.3.10B.1.4	SSB pattern 4 for RedCap in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1678

A.3.10B.1.5	SSB pattern 5 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1678

A.3.10B.1.6	SSB pattern 6 for RedCap in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1679

A.3.10B.1.7	SSB pattern 7 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1679

A.3.10B.2	SSB Configurations for FR2	1680

A.3.10B.2.1	SSB pattern 1 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1680

A.3.10B.2.2	SSB pattern 2 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1680

A.3.10B.2.3	SSB pattern 3 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1681

A.3.10B.2.4	SSB pattern 4 for RedCap in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1681

A.3.10B.2.5	SSB pattern 5 for RedCap in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1682

A.3.11	SMTC Configurations	1682

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	1682

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	1682

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	1682

A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms	1683

A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms	1683

A.3.11.6	SMTC pattern 6: SMTC period = 20 ms with SMTC duration = 5 ms	1683

A.3.11.7	SMTC pattern 7: SMTC period = 20 ms with SMTC duration = 5 ms	1683

A.3.11.8	SMTC pattern 8: SMTC period = 10 ms with SMTC duration = 1 ms	1683

A.3.11.9	SMTC pattern 9: SMTC period = 20 ms with SMTC duration = 1 ms	1683

A.3.11.10	SMTC pattern 10: SMTC period = 80 ms with SMTC duration = 1 ms	1684

A.3.11.11	SMTC pattern 11: SMTC period = 80 ms with SMTC duration = 5 ms	1684

A.3.11.12	SMTC pattern 12: SMTC period = 20 ms with SMTC duration = 5 ms	1684

A.3.11A	SMTC Configurations for RedCap	1684

A.3.11A.0	Introduction	1684

A.3.11A.1	SMTC pattern 1 for RedCap: SMTC period = 40 ms with SMTC duration = 1 ms	1685

A.3.11A.2	SMTC pattern 2 for RedCap: SMTC period = 80 ms with SMTC duration = 1 ms	1685

A.3.11A.3	SMTC pattern 3 for RedCap: SMTC period = 40 ms with SMTC duration = 1 ms	1685

A.3.11A.4	SMTC pattern 4 for RedCap: SMTC period = 80 ms with SMTC duration = 5 ms	1685

A.3.12	Test Cases with Different CC Configurations	1685

A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations	1685

A.3.12.1.1	Introduction	1685

A.3.12.1.2	Principle of testing	1685

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	1686

A.3.12.2.1	Introduction	1686

A.3.12.2.2	Principle of testing	1686

A.3.13	Test Cases in SA and EN-DC Operations	1686

A.3.13.1	Introduction	1686

A.3.13.2	Principle of Testing	1686

A.3.13B	 Test Cases for EN-DC and NE-DC Operations	1687

A.3.13B.1	Active BWP switch Test Cases for EN-DC and NE-DC Operations	1687

A.3.13B.1.1	Introduction	1687

A.3.13B.1.2	Principle of Testing	1687

A.3.13B.2	SFTD accuracy Test Cases for EN-DC and NE-DC Operations	1687

A.3.13B.2.1	Introduction	1687

A.3.13B.2.2	Principle of Testing	1688

A.3.14	CSI-RS configurations	1688

A.3.14.1	FDD	1688

A.3.14.2	TDD	1690

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	1695

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	1695

A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction	1695

A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	1695

A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	1696

A.3.15.3	Setup 3: 2 AoAs	1696

A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	1696

A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	1696

A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	1696

A.3.15.4.3	Setup 4c: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak for power class 6 UE supporting simultaneous reception from multiple directions	1696

A.3.15.5	Setup 5: 2 AoAs for simultaneous reception with QCL Type-D	1697

A.3.15.6	Setup 6: 3 AoAs for simultaneous reception with different QCL Type-D	1697

A.3.15.7	Setup 7: 3 AoAs	1697

A.3.15C	Angle of Arrival (AoA) for FR2-NTN RRM test cases	1697

A.3.15C.1	Setup 1: Single AoA	1697

A.3.15C.2	Setup 2: 2 AoAs	1698

A.3.16	TCI State Configuration	1698

A.3.16.1	Introduction	1698

A.3.16.2	TCI states	1698

A.3.16A	Unified TCI State Configuration	1698

A.3.16A.1	Introduction	1698

A.3.16A.2	DLorJoint TCI states	1699

A.3.16A.3	UL TCI states	1700

A.3.16B	 LTM Candidate TCI State Configuration	1700

A.3.16B.1	Introduction	1700

A.3.16B.2	LTM candidate DLorJoint TCI states	1701

A.3.16B.3	LTM candidate UL TCI states	1701

A.3.17	Configurations of CSI-RS for tracking	1702

A.3.17.1	Configuration of CSI-RS for tracking for FR1	1702

A.3.17.1.1	FDD	1702

A.3.17.1.2	TDD	1705

A.3.17.2	Configuration of CSI-RS for tracking for FR2	1709

A.3.17.2.1	TDD	1709

A.3.17.2.2	FDD	1712

A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases	1712

A.3.18.1	Introduction	1712

A.3.18.2	PRACH Power Measurement	1713

A.3.19	Test applicability for DAPS handover	1713

A.3.19.1	Introduction	1713

A.3.19.2	Principle of testing	1713

A.3.20	MsgA configurations	1713

A.3.20.1	Introduction	1713

A.3.20.2	MsgA configurations in FR1	1713

A.3.20.2.1	FR1 MsgA configuration 1	1713

A.3.20.2.2	FR1 MsgA configuration 2	1714

A.3.20.3	MsgA configurations in FR2	1715

A.3.20.3.1	FR2 MsgA configuration 1	1715

A.3.20.3.2	FR2 MsgA configuration 2	1716

A.3.20A	MsgA configurations under CCA	1717

A.3.20A.1	Introduction	1717

A.3.20A.2	MsgA configurations in FR1	1717

A.3.20A.2.1	FR1 MsgA configuration 1 under CCA	1717

A.3.20A.2.2	FR1 MsgA configuration 2 under CCA	1718

A.3.21	V2X sidelink communication	1719

A.3.21.1	Introduction	1719

A.3.21.2	Reference resource pool configurations for V2X Sidelink Communication	1720

A.3.21.3	Reference measurement channels for V2X Sidelink Communication	1723

A.3.21.4	Reference SL-DRX configurations	1724

A.3.21.4.1	SL-DRX Configuration 1: SL-DRX cycle = 40 ms	1724

A.3.21.4.2	SL-DRX Configuration 2: SL-DRX cycle = 320 ms	1724

A.3.21.4.3	SL-DRX Configuration 3: SL-DRX cycle = 640 ms	1724

A.3.21A	NR Sidelink Measurements for Positioning	1724

A.3.21A.1	Introduction	1724

A.3.21A.2	NR SL-PRS configurations	1725

A.3.21A.2.1	NR SL-PRS configurations for FR1	1725

A.3.22	CSI-IM configurations	1725

A.3.22.1	FDD	1725

A.3.22.2	TDD	1725

A.3.23	Spatial Relation Configuration	1726

A.3.23.1	Introduction	1726

A.3.23.2	Spatial Relation	1727

A.3.24	SRS configuration	1727

A.3.25	Channel bandwidth (CBW) configurations	1729

A.3.25.1	DL UE specific CBW	1729

A.3.25.2	UL UE specific CBW	1730

A.3.26	CCA model	1730

A.3.26.1	Introduction	1730

A.3.26.2	CCA model for operation on a carrier frequency with CCA in FR1	1730

A.3.26.2.1	DL CCA model	1730

A.3.26.2.2	UL CCA model	1731

A.3.26.3	CCA model for operation on a carrier frequency with CCA in FR2-2	1732

A.3.26.3.1	DL CCA model	1732

A.3.26.3.2	UL CCA model	1732

A.3.26.4	CCA model for operation on a sidelink carrier frequency with CCA	1733

A.3.26.4.1	CCA model for SyncRef UE	1733

A.3.27	Void	1734

A.3.27.1	Void	1734

A.3.27.2	 Void	1734

A.3.27.3	Void	1734

A.3.27.4	Void	1734

A.3.27.5	Void	1734

A.3.28	Discovery Burst Transmission Window configuration under CCA	1734

A.3.28.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	1734

A.3.29	Testing principles for UE capable of only NR bands with shared spectrum access	1734

A.3.29.1	Introduction	1734

A.3.29.2	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	1735

A.3.29.3	Principle of testing for UE capable of SA operation with only NR bands with shared spectrum access	1735

A.3.30	CSI-RS configurations for RRM	1736

A.3.30.1	FDD	1736

A.3.30.2	TDD	1736

A.3.31	PRS Configurations	1738

A.3.31.1	PRS Configurations for FR1	1738

A.3.31.1.1	PRS pattern 1 in FR1: SCS=15 kHz	1738

A.3.31.1.2	PRS pattern 2 in FR1: SCS=30 kHz	1739

A.3.31.2	PRS Configurations for FR2	1740

A.3.31.2.1	PRS pattern 1 in FR2: SCS=120 kHz	1740

A.3.32	NR sidelink discovery	1740

A.3.32.1	Introduction	1740

A.3.32.2	Reference resource pool configurations for NR Sidelink Discovery	1740

A.3.32.3	Principle of Testing	1741

A.3.33	PRS Processing Window (PPW) configurations	1741

A.3.34	Testing principles for test cases related to PRS measurements	1741

A.3.34.1	Introduction	1741

A.3.34.2	Test cases in RRC_INACTIVE state	1741

A.3.34.3	Test cases for PRS measurements with gaps in RRC_CONNECTED state	1742

A.3.34.4	Test cases for PRS measurements without gaps in RRC_CONNECTED state	1742

A.3.34.5	Testing principles for positioning measurements by aggregating PRS resources from multiple PFLs	1742

A.3.34.6	Testing principles for carrier phase measurement for positioning	1743

A.3.34.7	Test cases in RRC_IDLE state	1743

A.3.35	Testing principle for RedCap UE	1743

A.3.35.1	Introduction	1743

A.3.35.2	Principle of testing for FR1	1743

A.3.35.3	Principle of testing for FR2	1743

A.3.35.4	Principle of testing for PRS measurement	1743

A.3.36	Testing related to Satellite access	1744

A.3.36.1	Introduction	1744

A.3.36.1	Introduction	1744

A.3.36.2	Principle of testing GSO and NGSO scenarios	1744

A.3.36.2	Principle of testing different RRM requirements	1744

A.3.36.3	Principle of testing different ephemeris formats	1745

A.3.36.4	General setup for SIB19	1747

A.3.36.5	Satellite specific parameters configuration	1748

A.3.36.5.1	Satellite specific configuration for serving cell	1748

A.3.36.5.2	Satellite specific configuration for neighbour cell	1748

A.3.37	Reference Cell DTX configurations	1749

A.3.37.1	Cell DTX Configuration 1: Cell DTX cycle = 160 ms and TAT = Infinity	1749

A.3.38	DL-PRS Measurement Time Window configurations	1749

A.4	EN-DC tests with all NR cells in FR1	1751

A.4.1	Void	1751

A.4.2	Void	1751

A.4.3	RRC_CONNECTED state mobility	1751

A.4.3.1	Void	1751

A.4.3.2	RRC Connection Mobility Control	1751

A.4.3.2.1	Void	1751

A.4.3.2.2	Random Access	1751

A.4.3.2.2.1	4-step RA type contention based random access test in FR1 for PSCell in EN-DC	1751

A.4.3.2.2.2	4-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	1754

A.4.3.2.2.3	2-step RA type contention based random access test in FR1 for PSCell in EN-DC	1756

A.4.3.2.2.4	2-step RA type non-contention based random access test in FR1 for PSCell in EN-DC	1759

A.4.3.2.3	Void	1761

A.4.3.3	Handover with PSCell from EN-DC to EN-DC with known target PSCell in FR1	1761

A.4.3.3.1	Test Purpose and Environment	1761

A.4.3.3.2	Test Requirements	1764

A.4.4	Timing	1765

A.4.4.1	UE transmit timing	1765

A.4.4.1.1	NR UE Transmit Timing Test for FR1	1765

A.4.4.1.1.1	Test Purpose and environment	1765

A.4.4.1.1.2	Test requirements	1767

A.4.4.1.2	NR UE Transmit Timing Test for two TRPs in FR1	1768

A.4.4.1.2.1	Test Purpose and environment	1768

A.4.4.1.2.2	Test requirements	1771

A.4.4.2	UE timer accuracy	1772

A.4.4.3	Timing advance	1772

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	1772

A.4.4.3.1.1	Test Purpose and Environment	1772

A.4.4.3.1.2	Test Parameters	1772

A.4.4.3.1.3	Test Requirements	1775

A.4.5	Signaling characteristics	1775

A.4.5.1	Radio link Monitoring	1775

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	1775

A.4.5.1.1.1	Test Purpose and Environment	1775

A.4.5.1.1.2	Test Requirements	1779

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	1779

A.4.5.1.2.1	Test Purpose and Environment	1779

A.4.5.1.2.2	Test Requirements	1782

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	1782

A.4.5.1.3.1	Test Purpose and Environment	1782

A.4.5.1.3.2	Test Requirements	1785

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	1785

A.4.5.1.4.1	Test Purpose and Environment	1785

A.4.5.1.4.2	Test Requirements	1788

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	1788

A.4.5.1.5.1	Test Purpose and Environment	1788

A.4.5.1.5.2	Test Requirements	1791

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	1792

A.4.5.1.6.1	Test Purpose and Environment	1792

A.4.5.1.6.2	Test Requirements	1794

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	1795

A.4.5.1.7.1	Test Purpose and Environment	1795

A.4.5.1.7.2	Test Requirements	1797

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	1798

A.4.5.1.8.1	Test Purpose and Environment	1798

A.4.5.1.8.2	Test Requirements	1801

A.4.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	1801

A.4.5.1.9.1	Test Purpose and Environment	1801

A.4.5.1.10	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode when CD-SSB is outside active BWP	1804

A.4.5.1.10.1	Test Purpose and Environment	1804

A.4.5.1.11	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode when CD-SSB is outside active BWP	1804

A.4.5.1.11.1	Test Purpose and Environment	1804

A.4.5.1.11.2	Test Requirements	1805

A.4.5.1.12	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode for UE supporting NCD-SSB based measurement outside active BWP	1805

A.4.5.1.12.1	Test Purpose and Environment	1805

A.4.5.1.12.2	Test Requirements	1808

A.4.5.2	Interruption	1808

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1808

A.4.5.2.1.1	Test Purpose and Environment	1808

A.4.5.2.1.2	Test Requirements	1810

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1810

A.4.5.2.2.1	Test Purpose and Environment	1810

A.4.5.2.2.2	Test Requirements	1812

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1812

A.4.5.2.3.1	Test Purpose and Environment	1812

A.4.5.2.3.2	Test Requirements	1816

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1817

A.4.5.2.4.1	Test Purpose and Environment	1817

A.4.5.2.4.2	Test Requirements	1821

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1821

A.4.5.2.5.1	Test Purpose and Environment	1821

A.4.5.2.5.2	Test Requirements	1823

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1824

A.4.5.2.6.1	Test Purpose and Environment	1824

A.4.5.2.6.2	Test Requirements	1826

A.4.5.2.7	Void	1826

A.4.5.2.8	E-UTRAN - NR FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	1826

A.4.5.2.8.1	Test Purpose and Environment	1826

A.4.5.2.8.2	Test Requirements	1829

A.4.5.2.9	E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching	1829

A.4.5.2.9.1	Test Purpose and Environment	1829

A.4.5.2.9.2	Test Requirements	1832

A.4.5.2.10	E-UTRAN – NR FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell	1832

A.4.5.2.10.1	Test Purpose and Environment	1832

A.4.5.2.10.2	Test Requirements	1834

A.4.5.2.11	E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in synchronous EN-DC	1834

A.4.5.2.11.1	Test Purpose and Environment	1834

A.4.5.2.11.2	Test Requirements	1837

A.4.5.2.12	E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching in asynchronous EN-DC	1837

A.4.5.2.12.1	Test Purpose and Environment	1837

A.4.5.3	SCell Activation and Deactivation Delay	1841

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160 ms SCell measurement cycle	1841

A.4.5.3.1.1	Test Purpose and Environment	1841

A.4.5.3.1.2	Test Requirements	1847

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 640 ms SCell measurement cycle	1847

A.4.5.3.2.1	Test Purpose and Environment	1847

A.4.5.3.2.2	Test Requirements	1847

A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1	1848

A.4.5.3.3.1	Test Purpose and Environment	1848

A.4.5.3.3.2	Test Requirements	1848

A.4.5.3.4	SCell Activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command	1848

A.4.5.3.4.1	Test Purpose and Environment	1849

A.4.5.3.4.2	Test Requirements	1851

A.4.5.3.5	Direct SCell activation at SCell addition of known SCell in FR1	1851

A.4.5.3.5.1	Test Purpose and Environment	1851

A.4.5.3.5.2	Test Requirements	1856

A.4.5.3.6	Fast SCell Activation of known SCell in FR1 for 160 ms SCell measurement cycle	1856

A.4.5.3.6.1	Test Purpose and Environment	1856

A.4.5.3.6.2	Test Requirements	1859

A.4.5.3.7	Fast SCell Activation of known SCell in FR1 for 640 ms SCell measurement cycle	1860

A.4.5.3.7.1	Test Purpose and Environment	1860

A.4.5.3.7.2	Test Requirements	1860

A.4.5.3.8	SCell Activation and deactivation of unknown SCell in FR1 for UE capable of short measurement interval	1860

A.4.5.3.8.1	Test Purpose and Environment	1860

A.4.5.3.8.2	Test Requirements	1861

A.4.5.3.9	SCell Activation of unknown SCell with valid L3 measurement results in FR1 for 160 ms SCell measurement cycle	1862

A.4.5.3.9.1	Test Purpose and Environment	1862

A.4.5.3.9.2	Test Requirements	1867

A.4.5.3.10	SCell Activation of multiple unknown SCells in FR1 with L3 reporting with single activation/deactivation command in non-DRX	1868

A.4.5.3.10.1	Test Purpose and Environment	1868

A.4.5.3.10.2	Test Requirements	1870

A.4.5.3.11	TRS-based SCell Activation of SSB-less SCell in FR1 collocated inter-band	1871

A.4.5.3.11.1	Test Purpose and Environment	1871

A.4.5.3.11.2	Test Requirements	1874

A.4.5.3.12	Inter-band SSB-less Scell activation using A-TRS	1875

A.4.5.3.12.1	Test Purpose and Environment	1875

A.4.5.3.12.2	Test Requirements	1878

A.4.5.4	UE UL carrier RRC reconfiguration Delay	1878

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	1878

A.4.5.4.1.1 Test Purpose and Environment	1878

A.4.5.4.1.2	Test Requirements	1883

A.4.5.5	Beam Failure Detection and Link recovery procedures	1883

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	1883

A.4.5.5.1.1	Test Purpose and Environment	1883

A.4.5.5.1.2	Test Requirements	1887

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	1887

A.4.5.5.2.1	Test Purpose and Environment	1887

A.4.5.5.2.2	Test Requirements	1890

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1891

A.4.5.5.3.1	Test Purpose and Environment	1891

A.4.5.5.3.2	Test Requirements	1894

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1895

A.4.5.5.4.1	Test Purpose and Environment	1895

A.4.5.5.4.2	Test Requirements	1898

A.4.5.5.5	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1899

A.4.5.5.5.1	Test Purpose and Environment	1899

A.4.5.5.5.2	Test Requirements	1902

A.4.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	1903

A.4.5.5.6.1	Test Purpose and Environment	1903

A.4.5.5.6.2	Test Requirements	1906

A.4.5.5.7	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	1907

A.4.5.5.7.1	Test Purpose and Environment	1907

A.4.5.5.7.2	Test Requirements	1910

A.4.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1911

A.4.5.5.8.1	Test Purpose and Environment	1911

A.4.5.5.8.2	Test Requirements	1915

A.4.5.6	Active BWP switch	1915

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	1915

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	1915

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	1919

A.4.5.6.2	RRC-based Active BWP Switch	1924

A.4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1927

A.4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	1927

A.4.5.6.4	Simultaneous RRC-based Active BWP Switch on multiple CCs	1932

A.4.5.6.4.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	1932

A.4.5.6.4.1.1	Test Purpose and Environment	1932

A.4.5.6.4.1.2	Test Requirements	1936

A.4.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	1936

A.4.5.6.4.2.1	Test Purpose and Environment	1936

A.4.5.6.4.2.2	Test Requirements	1942

A.4.5.6.5	SCell dormancy switch	1942

A.4.5.6.5.1	E-UTRAN – NR FR1 PSCell SCell dormancy switch of single FR1 SCell outside active time	1942

A.4.5.6.5.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	1947

A.4.5.6.5.2.1	Test Purpose and Environment	1947

A.4.5.6.5.2.2	Test Requirements	1951

A.4.5.7	PSCell addition and release delay	1951

A.4.5.7.1	Addition and Release Delay of known NR PSCell	1951

A.4.5.7.1.1	Test purpose and environment	1951

A.4.5.7.1.2	Test Requirements	1954

A.4.5.8	DL Interruptions at switching between two uplink carriers	1954

A.4.5.8.1	Test Purpose and Environment	1954

A.4.5.8.2	Test Requirements	1958

A.4.5.9	UE specific CBW change	1958

A.4.5.9.1	UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN- DC	1958

A.4.5.9.1.1	Test Purpose and Environment	1958

A.4.5.9.1.2	Test Requirements	1961

A.4.5.10	PSCell activation and deactivation delay	1961

A.4.5.10.1	PSCell activation and deactivation delay	1961

A.4.5.10.1.1	Test purpose and environment	1961

A.4.5.10.1.2	Test Requirements	1963

A.4.5.11	Conditional PSCell addition and release delay (FR1 EN-DC)	1964

A.4.5.11.1	Conditional PSCell Addition and Release Delay	1964

A.4.5.11.1.1	Test purpose and environment	1964

A.4.5.11.1.2	Test Parameters	1964

A.4.5.11.1.3	Test Requirements	1966

A.4.6	Measurement procedure	1966

A.4.6.1	Intra-frequency Measurements	1966

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	1966

A.4.6.1.1.1	Test purpose and Environment	1966

A.4.6.1.1.2	Test parameters	1966

A.4.6.1.1.3	Test Requirements	1968

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	1968

A.4.6.1.2.1	Test purpose and Environment	1968

A.4.6.1.2.2	Test parameters	1969

A.4.6.1.2.2	Test Requirements	1971

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	1971

A.4.6.1.3.1	Test purpose and Environment	1971

A.4.6.1.3.2	Test parameters	1971

A.4.6.1.3.3	Test Requirements	1973

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	1973

A.4.6.1.4.1	Test purpose and Environment	1973

A.4.6.1.4.2	Test parameters	1973

A.4.6.1.4.3	Test Requirements	1975

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	1976

A.4.6.1.5.1	Test purpose and Environment	1976

A.4.6.1.5.2	Test parameters	1976

A.4.6.1.5.3	Test Requirements	1977

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	1977

A.4.6.1.6.1	Test purpose and Environment	1977

A.4.6.1.6.2	Test parameters	1977

A.4.6.1.6.3	Test Requirements	1979

A.4.6.1.7	EN-DC event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	1979

A.4.6.1.7.1	Test purpose and Environment	1979

A.4.6.1.7.2	Test parameters	1979

A.4.6.1.7.3	Test Requirements	1981

A.4.6.1.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used for UE configured with highSpeedMeasCA-Scell-r17	1981

A.4.6.1.8.1	Test Purpose and Environment	1981

A.4.6.1.8.2	Test Requirements	1985

A.4.6.1.9	EN-DC event triggered reporting tests without gap under non-DRX with NCD-SSB	1985

A.4.6.1.9.1	Test purpose and Environment	1985

A.4.6.1.9.2	Test parameters	1985

A.4.6.1.9.3	Test Requirements	1987

A.4.6.1.10	EN-DC event triggered reporting tests without gap under non-DRX when CD-SSB is outside active BWP	1987

A.4.6.110.1	Test purpose and Environment	1987

A.4.6.1.10.2	Test Requirements	1987

A.4.6.2	Inter-frequency Measurements	1987

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	1988

A.4.6.2.1.1	Test Purpose and Environment	1988

A.4.6.2.1.2	Test Requirements	1990

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	1990

A.4.6.2.2.1	Test Purpose and Environment	1990

A.4.6.2.2.2	Test Requirements	1993

A.4.6.2.3	Void	1993

A.4.6.2.4	Void	1993

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	1993

A.4.6.2.5.1	Test Purpose and Environment	1993

A.4.6.2.5.2	Test Requirements	1996

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	1996

A.4.6.2.6.1	Test Purpose and Environment	1996

A.4.6.2.6.2	Test Requirements	1998

A.4.6.2.7	Void	1999

A.4.6.2.8	Void	1999

A.4.6.2.9	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17	1999

A.4.6.2.9.1	Test Purpose and Environment	1999

A.4.6.2.9.2	Test Requirements	2002

A.4.6.3	Void	2002

A.4.6.4	L1-RSRP measurement for beam reporting	2002

A.4.6.4.1	SSB based L1-RSRP measurement when DRX is not used	2002

A.4.6.4.1.1	Test Purpose and Environment	2002

A.4.6.4.1.2	Test parameters	2002

A.4.6.4.1.3	Test Requirements	2003

A.4.6.4.2	SSB based L1-RSRP measurement when DRX is used	2004

A.4.6.4.2.1	Test Purpose and Environment	2004

A.4.6.4.2.2	Test parameters	2004

A.4.6.4.2.3	Test Requirements	2005

A.4.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	2006

A.4.6.4.3.1	Test Purpose and Environment	2006

A.4.6.4.3.2	Test parameters	2006

A.4.6.4.3.3	Test Requirements	2007

A.4.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	2008

A.4.6.4.4.1	Test Purpose and Environment	2008

A.4.6.4.4.2	Test parameters	2008

A.4.6.4.4.3	Test Requirements	2009

A.4.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	2010

A.4.6.4.5.1	Test Purpose and Environment	2010

A.4.6.4.5.2	Test parameters	2010

A.4.6.4.5.3	Test Requirements	2011

A.4.6.4.6	CSI-RS based L1-RSRP measurement when DRX is not used when CD-SSB is outside active BWP	2011

A.4.6.4.6.1	Test Purpose and Environment	2012

A.4.6.4.7	SSB based L1-RSRP measurement when DRX is not used when CD-SSB is outside active BWP	2012

A.4.6.4.7.1	Test Purpose and Environment	2012

A.4.6.4.7.2	Test Requirements	2012

A.4.6.4.8	SSB based L1-RSRP measurement for UE supporting NCD-SSB based L1 measurement outside active BWP when DRX is not used	2012

A.4.6.4.8.1	Test Purpose and Environment	2012

A.4.6.4.8.2	Test parameters	2012

A.4.6.4.8.3	Test Requirements	2014

A.4.6.5	CLI measurements	2014

A.4.6.5.1	SRS-RSRP measurement with non-DRX	2014

A.4.6.5.1.1	Test Purpose and Environment	2014

A.4.6.5.1.2	Test Parameters	2014

A.4.6.5.1.3	Test Requirements	2017

A.4.6.5.2	CLI-RSSI measurement with non-DRX	2017

A.4.6.5.2.1	Test Purpose and Environment	2017

A.4.6.5.2.2	Test Parameters	2017

A.4.6.5.2.3	Test Requirements	2018

A.4.6.6.1.2	Test Requirements	2022

A.4.6.7	L1-SINR measurement for beam reporting	2022

A.4.6.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	2024

A.4.6.7.2.1	Test Purpose and Environment	2024

A.4.6.7.2.2	Test parameters	2025

A.4.6.7.2.3	Test Requirements	2026

A.4.6.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	2026

A.4.6.7.3.1	Test Purpose and Environment	2027

A.4.6.7.3.2	Test parameters	2027

A.4.6.7.3.3	Test Requirements	2028

A.4.6.8	CSI-RS based intra-frequency Measurement	2029

A.4.6.8.1	EN-DC event triggered reporting tests without gap under DRX	2029

A.4.6.8.1.1	Test purpose and Environment	2029

A.4.6.8.1.2	Test Requirements	2031

A.4.6.9	CSI-RS based inter-frequency Measurement	2031

A.4.6.9.1	EN-DC event triggered reporting tests for FR1 cell when non-DRX is used	2031

A.4.6.9.1.1	Test Purpose and Environment	2031

A.4.6.9.1.2	Test Requirements	2033

A.4.7	Measurement Performance requirements	2035

A.4.7.1	SS-RSRP	2035

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2035

A.4.7.1.1.1	Test Purpose and Environment	2035

A.4.7.1.1.2	Test parameters	2035

A.4.7.1.1.3	Test Requirements	2040

A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2040

A.4.7.1.2.1	Test Purpose and Environment	2040

A.4.7.1.2.2	Test parameters	2040

A.4.7.1.2.3	Test Requirements	2043

A.4.7.1.3	Void	2043

A.4.7.2	SS-RSRQ	2043

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2043

A.4.7.2.1.1	Test Purpose and Environment	2043

A.4.7.2.1.2	Test Parameters	2043

A.4.7.2.1.3	Test Requirements	2047

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2047

A.4.7.2.2.1	Test Purpose and Environment	2047

A.4.7.2.2.2	Test Parameters	2047

A.4.7.2.2.3	Test Requirements	2051

A.4.7.3	SS-SINR	2051

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2051

A.4.7.3.1.1	Test Purpose and Environment	2051

A.4.7.3.1.2	Test Parameters	2051

A.4.7.3.1.3	Test Requirements	2055

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2055

A.4.7.3.2.1	Test Purpose and Environment	2055

A.4.7.3.2.2	Test Parameters	2055

A.4.7.3.2.3	Test Requirements	2058

A.4.7.4	L1-RSRP measurement for beam reporting	2058

A.4.7.4.1	SSB based L1-RSRP measurement	2058

A.4.7.4.1.1	Test Purpose and Environment	2058

A.4.7.4.1.2	Test parameters	2058

A.4.7.4.1.3	Test Requirements	2061

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2061

A.4.7.4.2.1	Test Purpose and Environment	2061

A.4.7.4.2.2	Test parameters	2062

A.4.7.4.2.3	Test Requirements	2065

A.4.7.5	SFTD accuracy	2065

A.4.7.5.1	SFTD accuracy	2065

A.4.7.5.1.1	Test Purpose and Environment	2065

A.4.7.5.1.2	Test Parameters	2065

A.4.7.5.1.3	Test Requirements	2068

A.4.7.5.2	Void	2068

A.4.7.5.3	Void	2068

A.4.7.6	CLI measurements	2068

A.4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	2068

A.4.7.6.1.1	Test Purpose and Environment	2068

A.4.7.6.1.2	Test parameters	2068

A.4.7.6.1.3	Test Requirements	2071

A.4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	2071

A.4.7.6.2.1	Test Purpose and Environment	2071

A.4.7.6.2.2	Test parameters	2072

A.4.7.6.2.3	Test Requirements	2073

A.4.7.7	L1-SINR measurement for beam reporting	2073

A.4.7.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2077

A.4.7.7.2.1	Test Purpose and Environment	2077

A.4.7.7.2.2	Test parameters	2077

A.4.7.7.2.3	Test Requirements	2080

A.4.7.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2080

A.4.7.7.3.1	Test Purpose and Environment	2080

A.4.7.7.3.2	Test parameters	2081

A.4.7.7.3.3	Test Requirements	2084

A.4.7.8	CSI-RSRP	2084

A.4.7.8.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2084

A.4.7.8.1.1	Test Purpose and Environment	2084

A.4.7.8.1.2	Test parameters	2084

A.4.7.8.1.3	Test Requirements	2088

A.4.7.8.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2088

A.4.7.8.2.1	Test Purpose and Environment	2088

A.4.7.8.2.2	Test parameters	2089

A.4.7.8.2.3	Test Requirements	2092

A.4.7.9	CSI-RSRQ	2092

A.4.7.9.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2092

A.4.7.9.1.1	Test Purpose and Environment	2092

A.4.7.9.1.2	Test Parameters	2092

A.4.7.9.1.3	Test Requirements	2096

A.4.7.9.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2096

A.4.7.9.2.1	Test Purpose and Environment	2096

A.4.7.9.2.2	Test Parameters	2096

A.4.7.9.2.3	Test Requirements	2100

A.4.7.10	CSI-SINR	2100

A.4.7.10.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2100

A.4.7.10.1.1	Test Purpose and Environment	2100

A.4.7.10.1.2	Test Parameters	2100

A.4.7.10.1.3	Test Requirements	2103

A.4.7.10.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2104

A.4.7.10.2.1	Test Purpose and Environment	2104

A.4.7.10.2.2	Test Parameters	2104

A.4.7.10.2.3	Test Requirements	2107

A.4.7.11	TDCP amplitude measurement accuracy	2107

A.4.7.11.1	TDCP amplitude measurement accuracy in EN-DC	2107

A.4.7.11.1.1	Test Purpose and Environment	2107

A.4.7.11.1.2	Test parameters	2108

A.4.7.11.1.3	Test Requirements	2109

A.4.8	Void	2109

A.4A	NE-DC test with all NR cells in FR1	2109

A.4A.1	Signaling characteristics	2109

A.4A.1.1	E-UTRAN PSCell addition	2109

A.4A.1.1.1	Test purpose and environment	2109

A.4A.1.1.2	Test Requirements	2113

A.4A.1.2	Active BWP switch	2113

A.4A.1.2.1	E-UTRAN PSCell – NR PCell FR1 DCI-based and Timer-based DL active BWP switch in non-DRX in synchronous NE-DC	2113

A.4A.1.2.1.1	Test Purpose and Environment	2113

A.4A.1.2.1.2	Test Requirements	2116

A.4A.1.3	Intra-frequency handover with E-UTRAN PSCell	2117

A.4A.1.3.1	Test purpose and environment	2117

A.4A.1.3.2	Test Requirements	2120

A.4A.1.4	Handover with PSCell from NE-DC to NE-DC with unknown target PSCell	2121

A.4A.1.4.1	Test Purpose and Environment	2121

A.4A.1.4.2	Test Parameters	2121

A.4A.1.4.3	Test Requirements	2125

A.4A.1.4.3.1	Test Requirements for NR HO	2125

A.4A.1.4.3.2	Test Requirements for LTE PSCell Change	2126

A.4A.2	Measurement performance	2126

A.4A.2.1	SFTD accuracy	2126

A.4A.2.1.1	SFTD accuracy	2126

A.4A.2.1.1.1	Test Purpose	2126

A.4A.2.1.1.2	Test Environment	2126

A.4A.2.1.1.3	Test Requirements	2129

A.5	EN-DC tests with one or more NR cells in FR2	2129

A.5.1	Void	2129

A.5.2	Void	2130

A.5.3	RRC_CONNECTED state mobility	2130

A.5.3.1	Void	2130

A.5.3.2	RRC Connection Mobility Control	2130

A.5.3.2.1	Void	2130

A.5.3.2.2	Random Access	2130

A.5.3.2.2.1	4-step RA type c ontention based random access test in FR2 for PSCell/SCell in EN-DC	2130

A.5.3.2.2.2	4-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	2133

A.5.3.2.2.3	2-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	2136

A.5.3.2.2.4	2-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	2139

A.5.3.2.3	Void	2141

A.5.3.3	Handover with PSCell with known FR2 target PSCell	2141

A.5.3.3.1	Test purpose and environment	2141

A.5.3.3.2	Test Requirements	2144

A.5.3.3.3	Void	2144

A.5.3.3.4	Void	2144

A.5.3.3.5	Void	2144

A.5.3.3.6	Void	2145

A.5.4	Timing	2145

A.5.4.1	UE transmit timing	2145

A.5.4.1.1	NR UE Transmit Timing Test for FR2	2145

A.5.4.1.1.1	Test Purpose and environment	2145

A.5.4.1.1.2	Test requirements	2147

A.5.4.1.2	NR UE Transmit Timing Test with 2-TA for FR2 UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18	2147

A.5.4.1.2.1	Test Purpose and environment	2147

A.5.4.1.2.2	Test requirements	2152

A.5.4.2	UE timer accuracy	2152

A.5.4.3	Timing advance	2152

A.5.4.3.1 EN-DC FR2 timing advance adjustment accuracy	2152

A.5.4.3.1.1 Test Purpose and Environment	2152

A.5.4.3.1.2 Test Parameters	2152

A.5.4.3.1.3	Test Requirements	2155

A.5.5	Signaling characteristics	2155

A.5.5.1	Radio link Monitoring	2155

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	2155

A.5.5.1.1.1	Test Purpose and Environment	2155

A.5.5.1.1.2	Test Requirements	2158

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	2158

A.5.5.1.2.1	Test Purpose and Environment	2158

A.5.5.1.2.2	Test Requirements	2161

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	2162

A.5.5.1.3.1	Test Purpose and Environment	2162

A.5.5.1.3.2	Test Requirements	2164

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	2164

A.5.5.1.4.1	Test Purpose and Environment	2164

A.5.5.1.4.2	Test Requirements	2167

A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	2167

A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	2170

A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	2173

A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	2176

A.5.5.1.8.2	Test Requirements	2180

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2	2180

A.5.5.1.9.1	Test Purpose and Environment	2180

A.5.5.1.9.2	Test Requirements	2181

A.5.5.1.10	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	2182

A.5.5.1.10.1	Test Purpose and Environment	2182

A.5.5.1.10.2	Test Requirements	2184

A.5.5.1.11	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode for UE supporting fast beam sweeping in multi-Rx	2184

A.5.5.1.11.1	Test Purpose and Environment	2184

A.5.5.1.11.2	Test Requirements	2187

A.5.5.1.12	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode when CD-SSB is outside active BWP	2188

A.5.5.1.12.1	Test Purpose and Environment	2188

A.5.5.1.13	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode when CD-SSB is outside active BWP	2188

A.5.5.1.13.1	Test Purpose and Environment	2188

A.5.5.1.13.2	Test Requirements	2188

A.5.5.1.14	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode for UE supporting NCD-SSB based measurement outside active BWP	2188

A.5.5.1.14.1	Test Purpose and Environment	2188

A.5.5.1.14.2	Test Requirements	2191

A.5.5.2	Interruption	2192

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	2192

A.5.5.2.1.1	Test Purpose and Environment	2192

A.5.5.2.1.2	Test Requirements	2194

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	2194

A.5.5.2.2.1	Test Purpose and Environment	2194

A.5.5.2.2.2	Test Requirements	2196

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	2196

A.5.5.2.3.1	Test Purpose and Environment	2196

A.5.5.2.3.2	Test Requirements	2198

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	2199

A.5.5.2.4.1	Test Purpose and Environment	2199

A.5.5.2.4.2	Test Requirements	2201

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	2202

A.5.5.2.5.1	Test Purpose and Environment	2202

A.5.5.2.5.2	Test Requirements	2204

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	2204

A.5.5.2.6.1	Test Purpose and Environment	2204

A.5.5.2.6.2	Test Requirements	2206

A.5.5.2.7	E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching	2207

A.5.5.2.7.1	Test Purpose and Environment	2207

A.5.5.2.7.2	Test Requirements	2209

A.5.5.2.8 E-UTRAN – NR FR2 interruptions at NR SRS carrier based switching	2209

A.5.5.2.8.1 Test Purpose and Environment	2209

A.5.5.2.8.3	Test Requirements	2211

A.5.5.2.9	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR PSCell	2211

A.5.5.2.9.1	Test Purpose and Environment	2211

A.5.5.2.9.2	Test Requirements	2214

A.5.5.3	SCell Activation and Deactivation Delay	2214

A.5.5.3.1	SCell Activation and deactivation of SCell in FR2 intra-band	2214

A.5.5.3.1.1	Test Purpose and Environment	2214

A.5.5.3.1.2	Test Requirements	2215

A.5.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 160 ms SCell measurement cycle	2216

A.5.5.3.2.1	Test Purpose and Environment	2216

A.5.5.3.2.2	Test Requirements	2218

A.5.5.3.3	Void	2218

A.5.5.3.4	Void	2218

A.5.5.3.5	SCell Activation and deactivation of SCell in FR2	2218

A.5.5.3.5.1	Test Purpose and Environment	2218

A.5.5.3.5.2	Test Requirements	2221

A.5.5.3.6	Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2	2221

A.5.5.3.6.1	Test Purpose and Environment	2221

A.5.5.3.6.2	Test Requirements	2224

A.5.5.3.7	Direct SCell activation at SCell addition of known SCell in FR2	2224

A.5.5.3.7.1	Test Purpose and Environment	2224

A.5.5.3.7.2	Test Requirements	2227

A.5.5.3.8	Fast SCell Activation of SCell in FR2 intra-band	2227

A.5.5.3.8.1	Test Purpose and Environment	2227

A.5.5.3.8.2	Test Requirements	2230

A.5.5.3.9	PUCCH SCell Activation and deactivation of known SCell in FR2	2230

A.5.5.3.9.1	Test Purpose and Environment	2230

A.5.5.3.9.2	Test Requirements	2233

A.5.5.3.10	PUCCH SCell Activation and deactivation of unknown SCell in FR2	2233

A.5.5.3.10.1	Test Purpose and Environment	2233

A.5.5.3.10.2	Test Requirements	2236

A.5.5.3.11	Multiple SCell activation and deactivation of one known PUCCH SCell and one unknown SCell in FR2	2236

A.5.5.3.11.1	Test Purpose and Environment	2236

A.5.5.3.11.2	Test Requirements	2239

A.5.5.3.12	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR2 in non-DRX	2240

A.5.5.3.12.1	Test Purpose and Environment	2240

A.5.5.3.12.2	Test Requirements	2242

A.5.5.3.13	SCell Activation and deactivation of unknown SCell in FR2 for UE in DRX, capable of small beam sweeping factors and/or short measurement interval	2243

A.5.5.3.13.1	Test Purpose and Environment	2243

A.5.5.3.13.2	Test Requirements	2246

A.5.5.3.14	PUCCH SCell activation and deactivation with FR1 PSCell based on L3 reporting after SCell activation command	2248

A.5.5.3.14.1	Test Purpose and Environment	2248

A.5.5.3.14.2	Test Requirements	2252

A.5.5.3.15	SCell Activation of unknown SCell in FR2 in non-DRX for 160 ms SCell measurement cycle with the L3 reporting during activation	2252

A.5.5.3.15.1	Test Purpose and Environment	2252

A.5.5.3.15.2	Test Requirements	2256

A.5.5.4	Void	2257

A.5.5.5	Beam Failure Detection and Link recovery procedures	2257

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	2257

A.5.5.5.1.1	Test Purpose and Environment	2257

A.5.5.5.1.2	Test Requirements	2260

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	2260

A.5.5.5.2.1	Test Purpose and Environment	2260

A.5.5.5.2.2	Test Requirements	2264

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	2264

A.5.5.5.3.1	Test Purpose and Environment	2264

A.5.5.5.3.2	Test Requirements	2267

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	2268

A.5.5.5.4.1	Test Purpose and Environment	2268

A.5.5.5.4.2	Test Requirements	2271

A.5.5.5.5	EN-DC scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	2271

A.5.5.5.5.1	Test Purpose and Environment	2271

A.5.5.5.5.2	Test Requirements	2274

A.5.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	2274

A.5.5.5.6.1	Test Purpose and Environment	2274

A.5.5.5.6.2	Test Requirements	2278

A.5.5.5.7	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	2278

A.5.5.5.7.1	Test Purpose and Environment	2278

A.5.5.5.7.2	Test Requirements	2281

A.5.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	2282

A.5.5.5.8.1	Test Purpose and Environment	2282

A.5.5.5.8.2	Test Requirements	2285

A.5.5.5.9	Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode for UE fulfilling relaxed measurement criterion	2285

A.5.5.5.9.1	Test Purpose and Environment	2285

A.5.5.5.9.2	Test Requirements	2288

A.5.5.6	Active BWP switch	2289

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	2289

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	2289

A.5.5.6.1.1.1	Test Purpose and Environment	2289

A.5.5.6.1.1.2	Test Requirements	2291

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 with FR2 SCell DL active BWP switch in non-DRX in synchronous EN-DC	2292

A.5.5.6.2	RRC-based Active BWP Switch	2295

A.5.5.6.2.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	2295

A.5.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	2298

A.5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	2298

A.5.5.6.4	SCell dormancy switch	2301

A.5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time	2301

A.5.5.6.4.1.1	Test Purpose and Environment	2301

A.5.5.6.4.1.2	Test Requirements	2304

A.5.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time	2305

A.5.5.6.4.2.1	Test Purpose and Environment	2305

A.5.5.6.4.2.2	Test Requirements	2309

A.5.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	2309

A.5.5.6.5.1	E-UTRAN – NR PSCell FR2  and NR SCell FR2 DL active BWP switch on multiple CCs with non-DRX in synchronous EN-DC	2309

A.5.5.7	PSCell addition and release delay	2312

A.5.5.7.1	Addition and Release Delay of NR PSCell	2312

A.5.5.7.1.1	Test purpose and environment	2312

A.5.5.7.1.2	Test Requirements	2314

A.5.5.8	Active TCI state switch delay	2314

A.5.5.8.1	MAC-CE based active TCI state switch	2315

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	2315

A.5.5.8.1.1.1	Test Purpose and Environment	2315

A.5.5.8.1.1.2	Test Requirements	2317

A.5.5.8.2	RRC based active TCI state switch	2318

A.5.5.8.2.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	2318

A.5.5.8.2.1.1	Test Purpose and Environment	2318

A.5.5.8.2.1.2	Test Requirements	2321

A.5.5.9	Uplink spatial relation switch delay	2321

A.5.5.9.1	MAC-CE based uplink spatial relation switch	2321

A.5.5.9.1.1	E-UTRAN – NR PSCell FR2 uplink spatial relation switch for a known spatial relation	2321

A.5.5.9.1.1.1	Test Purpose and Environment	2321

A.5.5.9.1.1.2	Test Requirements	2323

A.5.5.9.2	RRC based spatial relation switch	2323

A.5.5.9.2.1	E-UTRAN – NR PSCell FR2 spatial relation switch associated with a known DL-RS	2323

A.5.5.9.2.1.1	Test Purpose and Environment	2324

A.5.5.9.2.1.2	Test Requirements	2325

A.5.5.10	UE specific CBW change	2326

A.5.5.10.1	UE specific CBW change on FR2 NR PSCell	2326

A.5.5.10.1.1	Test Purpose and Environment	2326

A.5.5.10.1.2	Test Requirements	2328

A.5.5.11	Unified TCI state switch delay	2329

A.5.5.11.1	MAC-CE based active joint TCI state switch	2329

A.5.5.11.1.1	E-UTRAN – NR PSCell FR2 active joint TCI state switch for a known TCI state	2329

A.5.5.11.1.1.1	Test Purpose and Environment	2329

A.5.5.11.1.1.2	Test parameters	2329

A.5.5.11.1.1.3	Test Requirements	2331

A.5.5.11.2	MAC-CE based active uplink TCI state switch	2331

A.5.5.11.2.1	E-UTRAN – NR PSCell FR2 active uplink TCI state switch for a known TCI state	2331

A.5.5.11.2.1.1	Test Purpose and Environment	2331

A.5.5.11.2.1.2	Test parameters	2332

A.5.5.11.2.1.3	Test Requirements	2333

A.5.5.11.3	MAC-CE based active downlink TCI state switch	2334

A.5.5.11.3.1	E-UTRAN – NR PSCell FR2 downlink TCI state switch to cell with additional PCI for a known TCI state	2334

A.5.5.11.3.1.1	Test Purpose and Environment	2334

A.5.5.11.3.1.2	Test Parameters	2334

A.5.5.11.3.1.3	Test Requirements	2337

A.5.5.12	PSCell activation and deactivation delay	2337

A.5.5.12.1	PSCell activation and deactivation delay	2337

A.5.5.12.1.1	Test purpose and environment	2337

A.5.5.12.1.2	Test Requirements	2339

A.5.5.13	Conditional PSCell addition and release delay	2340

A.5.5.13.1	Addition and Release Delay of NR PSCell	2340

A.5.5.13.1.1	Test purpose and environment	2340

A.5.5.13.1.2	Test Requirements	2342

A.5.6	Measurement procedure	2342

A.5.6.1	Intra-frequency Measurements	2342

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	2342

A.5.6.1.1.1	Test purpose and Environment	2342

A.5.6.1.1.2	Test Requirements	2345

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	2345

A.5.6.1.2.1	Test purpose and Environment	2345

A.5.6.1.2.2	Test Requirements	2347

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	2348

A.5.6.1.3.1	Test purpose and Environment	2348

A.5.6.1.3.2	Test Requirements	2350

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	2350

A.5.6.1.4.1	Test purpose and Environment	2350

A.5.6.1.4.2	Test Requirements	2353

A.5.6.1.5	EN-DC event triggered reporting test without gap under non-DRX when CD-SSB is outside active BWP	2353

A.5.6.1.5.1	Test purpose and Environment	2353

A.5.6.1.5.2	Test Requirements	2353

A.5.6.1.6	EN-DC event triggered reporting test without gap under non-DRX	2353

A.5.6.1.6.1	Test purpose and Environment	2353

A.5.6.1.6.2	Test Requirements	2356

A.5.6.2	Inter-frequency Measurements	2356

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	2356

A.5.6.2.1.1	Test Purpose and Environment	2356

A.5.6.2.1.2	Test Requirements	2359

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	2359

A.5.6.2.2.1	Test Purpose and Environment	2359

A.5.6.2.2.2	Test Requirements	2361

A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	2362

A.5.6.2.3.1	Test Purpose and Environment	2362

A.5.6.2.3.2	Test Requirements	2364

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	2364

A.5.6.2.4.1	Test Purpose and Environment	2364

A.5.6.2.4.2	Test Requirements	2367

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	2367

A.5.6.2.5.1	Test Purpose and Environment	2367

A.5.6.2.5.2	Test Requirements	2370

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	2370

A.5.6.2.6.1	Test Purpose and Environment	2370

A.5.6.2.6.2	Test Requirements	2373

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	2374

A.5.6.2.7.1	Test Purpose and Environment	2374

A.5.6.2.7.2	Test Requirements	2376

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	2377

A.5.6.2.8.1	Test Purpose and Environment	2377

A.5.6.2.8.2	Test Requirements	2380

A.5.6.3	L1-RSRP measurement for beam reporting	2380

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used	2380

A.5.6.3.1.1	Test Purpose and Environment	2380

A.5.6.3.1.2	Test parameters	2381

A.5.6.3.1.3	Test Requirements	2382

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used	2382

A.5.6.3.2.1	Test Purpose and Environment	2382

A.5.6.3.2.2	Test parameters	2383

A.5.6.3.2.3	Test Requirements	2384

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	2384

A.5.6.3.3.1	Test Purpose and Environment	2384

A.5.6.3.3.2	Test parameters	2384

A.5.6.3.3.3	Test Requirements	2386

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	2386

A.5.6.3.4.1	Test Purpose and Environment	2386

A.5.6.3.4.2	Test parameters	2387

A.5.6.3.4.3	Test Requirements	2388

A.5.6.3.5	CSI-RS based L1-RSRP measurement when DRX is not used and when CD-SSB is outside active BWP	2388

A.5.6.3.5.1	Test Purpose and Environment	2388

A.5.6.3.6	SSB based L1-RSRP measurement when DRX is not used when CD-SSB is outside active BWP	2389

A.5.6.3.6.1	Test Purpose and Environment	2389

A.5.6.3.6.2	Test Requirements	2389

A.5.6.3.7	SSB based L1-RSRP measurement for UE supporting NCD-SSB based L1 measurement outside active BWP when DRX is not used	2389

A.5.6.3.7.1	Test Purpose and Environment	2389

A.5.6.3.7.2	Test parameters	2389

A.5.6.3.7.3	Test Requirements	2391

A.5.6.4	CLI measurements	2391

A.5.6.4.1	SRS-RSRP measurement with DRX	2391

A.5.6.4.1.1	Test Purpose and Environment	2391

A.5.6.4.1.2	Test Parameters	2391

A.5.6.4.1.3	Test Requirements	2393

A.5.6.4.2	CLI-RSSI measurement with DRX	2394

A.5.6.4.2.1	Test Purpose and Environment	2394

A.5.6.4.2.2	Test Parameters	2394

A.5.6.4.2.3	Test Requirements	2395

A.5.6.5	Measurements with autonomous gaps	2395

A.5.6.5.1 	EN-DC inter-frequency CGI identification of NR neighbor cell in FR2	2395

A.5.6.5.1.1	Test Purpose and Environment	2395

A.5.6.5.1.2	Test Requirements	2398

A.5.6.6	L1-SINR measurement for beam reporting	2398

A.5.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	2400

A.5.6.6.2.1	Test Purpose and Environment	2400

A.5.6.6.2.2	Test parameters	2400

A.5.6.6.2.3	Test Requirements	2402

A.5.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	2402

A.5.6.6.3.1	Test Purpose and Environment	2402

A.5.6.6.3.2	Test parameters	2403

A.5.6.6.3.3	Test Requirements	2404

A.5.6.7	CSI-RS based Intra-frequency Measurements	2404

A.5.6.7.1	EN-DC event triggered reporting test without gap under non-DRX	2404

A.5.6.7.1.1	Test purpose and Environment	2404

A.5.6.7.1.2	Test Requirements	2406

A.5.6.8	CSI-RS based Inter-frequency Measurements	2406

A.5.6.8.1 	EN-DC event triggered reporting tests for NR FR2 cell when DRX is used	2406

A.5.6.8.1.1	Test Purpose and Environment	2406

A.5.6.8.1.2	Test Requirements	2408

A.5.7	Measurement Performance requirements	2409

A.5.7.1	SS-RSRP	2409

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2409

A.5.7.1.1.1	Test Purpose and Environment	2409

A.5.7.1.1.2	Test parameters	2409

A.5.7.1.1.3	Test Requirements	2411

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2411

A.5.7.1.2.1	Test Purpose and Environment	2411

A.5.7.1.2.2	Test parameters	2412

A.5.7.1.2.3	Test Requirements	2414

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	2415

A.5.7.1.3.1	Test Purpose and Environment	2415

A.5.7.1.3.2	Test parameters	2415

A.5.7.1.3.3	Test Requirements	2417

A.5.7.2	SS-RSRQ	2417

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2417

A.5.7.2.1.1	Test Purpose and Environment	2417

A.5.7.2.1.2	Test Parameters	2418

A.5.7.2.1.3	Test Requirements	2419

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2419

A.5.7.2.2.1	Test Purpose and Environment	2419

A.5.7.2.2.2	Test Parameters	2419

A.5.7.2.2.3	Test Requirements	2421

A.5.7.3	SS-SINR	2421

A.5.7.3.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2421

A.5.7.3.1.1	Test Purpose and Environment	2421

A.5.7.3.1.2	Test Parameters	2421

A.5.7.3.1.3	Test Requirements	2423

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2423

A.5.7.3.2.1	Test Purpose and Environment	2423

A.5.7.3.2.2	Test Parameters	2423

A.5.7.3.2.3	Test Requirements	2424

A.5.7.4	L1-RSRP measurement for beam reporting	2424

A.5.7.4.1	SSB based L1-RSRP measurement	2425

A.5.7.4.1.1	Test Purpose and Environment	2425

A.5.7.4.1.2	Test parameters	2425

A.5.7.4.1.3	Test Requirements	2426

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2427

A.5.7.4.2.1	Test Purpose and Environment	2427

A.5.7.4.2.2	Test parameters	2427

A.5.7.4.2.3	Test Requirements	2428

A.5.7.5	CLI measurements	2429

A.5.7.5.1	EN-DC SRS-RSRP measurement accuracy with FR2 serving cell	2429

A.5.7.5.1.1	Test Purpose and Environment	2429

A.5.7.5.1.2	Test parameters	2429

A.5.7.5.1.3	Test Requirements	2431

A.5.7.5.2	EN-DC CLI-RSSI measurement accuracy with FR2 serving cell	2431

A.5.7.5.2.1	Test Purpose and Environment	2431

A.5.7.5.2.2	Test parameters	2431

A.5.7.5.2.3	Test Requirements	2433

A.5.7.6	L1-SINR measurement for beam reporting	2433

A.5.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2435

A.5.7.6.2.1	Test Purpose and Environment	2435

A.5.7.6.2.2	Test parameters	2436

A.5.7.6.2.3	Test Requirements	2437

A.5.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2438

A.5.7.6.3.1	Test Purpose and Environment	2438

A.5.7.6.3.2	Test parameters	2438

A.5.7.6.3.3	Test Requirements	2440

A.5.7.7	CSI-RSRP	2440

A.5.7.7.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2440

A.5.7.7.1.1	Test Purpose and Environment	2440

A.5.7.7.1.2	Test parameters	2440

A.5.7.7.1.3	Test Requirements	2442

A.5.7.7.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2443

A.5.7.7.2.1	Test Purpose and Environment	2443

A.5.7.7.2.2	Test parameters	2443

A.5.7.7.2.3	Test Requirements	2445

A.5.7.8	CSI-RSRQ	2446

A.5.7.8.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2446

A.5.7.8.1.1	Test Purpose and Environment	2446

A.5.7.8.1.2	Test Parameters	2446

A.5.7.8.1.3	Test Requirements	2448

A.5.7.8.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2448

A.5.7.8.2.1	Test Purpose and Environment	2448

A.5.7.8.2.2	Test Parameters	2448

A.5.7.8.2.3	Test Requirements	2450

A.5.7.9	CSI-SINR	2450

A.5.7.9.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2450

A.5.7.9.1.1	Test Purpose and Environment	2450

A.5.7.9.1.2	Test Parameters	2450

A.5.7.9.1.3	Test Requirements	2452

A.5.7.9.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2452

A.5.7.9.2.1	Test Purpose and Environment	2452

A.5.7.9.2.2	Test Parameters	2452

A.5.7.9.2.3	Test Requirements	2454

A.5.8	Void	2454

A.6	NR standalone tests with all NR cells in FR1	2456

A.6.1	SA: RRC_IDLE state mobility	2456

A.6.1.1	Cell re-selection to NR	2456

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	2456

A.6.1.1.1.1	Test Purpose and Environment	2456

A.6.1.1.1.2	Test Parameters	2456

A.6.1.1.1.3	Test Requirements	2458

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	2458

A.6.1.1.2.1	Test Purpose and Environment	2458

A.6.1.1.2.2	Test Parameters	2459

A.6.1.1.2.3	Test Requirements	2461

A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2461

A.6.1.1.3.1	Test Purpose and Environment	2461

A.6.1.1.3.2	Test Parameters	2461

A.6.1.1.3.3	Test Requirements	2464

A.6.1.1.4	Cell reselection to FR1 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2464

A.6.1.1.4.1	Test Purpose and Environment	2464

A.6.1.1.4.2	Test Parameters	2464

A.6.1.1.4.3	Test Requirements	2466

A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2467

A.6.1.1.5.1	Test Purpose and Environment	2467

A.6.1.1.5.2	Test Parameters	2467

A.6.1.1.5.3	Test Requirements	2469

A.6.1.1.6	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2470

A.6.1.1.6.1	Test Purpose and Environment	2470

A.6.1.1.6.2	Test Parameters	2470

A.6.1.1.6.3	Test Requirements	2472

A.6.1.1.7	Cell reselection to FR1 intra-frequency NR case for UE configured with highSpeedMeasFlag-r16	2473

A.6.1.1.7.1	Test Purpose and Environment	2473

A.6.1.1.7.2	Test Parameters	2473

A.6.1.1.7.3	Test Requirements	2476

A.6.1.1.8	Cell reselection to FR1 inter-frequency NR case for UE configured with highSpeedMeasInterFreq-r17	2476

A.6.1.1.8.1	Test Purpose and Environment	2476

A.6.1.1.8.2	Test Parameters	2476

A.6.1.1.8.3	Test Requirements	2479

A.6.1.1.9	Cell reselection to FR1 intra-frequency NR case for UE operating on a cell with less than 5 MHz BW	2479

A.6.1.1.9.1	Test Purpose and Environment	2479

A.6.1.1.9.3	Test Requirements	2480

A.6.1.2	Inter-RAT E-UTRAN cell re-selection	2480

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	2480

A.6.1.2.1.1	Test Purpose and Environment	2480

A.6.1.2.1.2	Test Parameters	2481

A.6.1.2.1.3	Test Requirements	2483

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	2483

A.6.1.2.2.1	Test Purpose and Environment	2483

A.6.1.2.2.2	Test Parameters	2483

A.6.1.2.2.3	Test Requirements	2486

A.6.1.2.3	Cell reselection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	2486

A.6.1.2.3.1	Test Purpose and Environment	2486

A.6.1.2.3.2	Test Parameters	2486

A.6.1.2.3.3	Test Requirements	2489

A.6.1.2.4	Cell reselection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	2489

A.6.1.2.4.1	Test Purpose and Environment	2489

A.6.1.2.4.2	Test Parameters	2490

A.6.1.2.4.3	Test Requirements	2492

A.6.1.2.5	Cell reselection to lower priority E-UTRAN cell for UE configured with highSpeedMeasFlag-r16	2492

A.6.1.2.5.1	Test Purpose and Environment	2493

A.6.1.2.5.2	Test Parameters	2493

A.6.1.2.5.3	Test Requirements	2495

A.6.1.1.7	Void	2495

A.6.2	SA: RRC_INACTIVE state mobility	2496

A.6.2.1	Configured Grant based Small Data Transmissions (CG-SDT)	2496

A.6.2.1.1	Test purpose and Environment	2496

A.6.2.1.2	Test Parameters	2497

A.6.2.1.3	Test requirements	2498

A.6.2.2	Cell reselection for positioning	2499

A.6.2.2.1	Cell reselection to FR1 intra-frequency NR case with RRC_ INACTIVE eDRX and positioning SRS	2499

A.6.2.2.1.1	Test Purpose and Environment	2499

A.6.2.2.1.2	Test Parameters	2499

A.6.2.2.1.3	Test Requirements	2503

A.6.3	RRC_CONNECTED state mobility	2503

A.6.3.1	Handover	2503

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	2503

A.6.3.1.1.1	Test Purpose and Environment	2503

A.6.3.1.1.2	Test Parameters	2503

A.6.3.1.1.3 Test Requirements	2505

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	2505

A.6.3.1.2.1	Test Purpose and Environment	2505

A.6.3.1.2.2	Test Parameters	2505

A.6.3.1.2.3	Test Requirements	2507

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	2507

A.6.3.1.3.1	Test Purpose and Environment	2507

A.6.3.1.3.2	Test Parameters	2507

A.6.3.1.3.3	Test Requirements	2509

A.6.3.1.4	SA NR - E-UTRAN handover	2509

A.6.3.1.4.1	Test Purpose and Environment	2509

A.6.3.1.4.2	Test Requirements	2512

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	2513

A.6.3.1.5.1	Test Purpose and Environment	2513

A.6.3.1.5.2	Test Requirements	2516

A.6.3.1.6	 SA NR - UTRAN FDD handover	2516

A.6.3.1.6.1	Test Purpose and Environment	2516

A.6.3.1.6.2	Test Requirements	2518

A.6.3.1.7	Intra-frequency synchronous DAPS handover in FR1	2518

A.6.3.1.7.1	Test Purpose and Environment	2518

A.6.3.1.7.2	Test Parameters	2518

A.6.3.1.7.3	Test Requirements	2521

A.6.3.1.8	Intra-frequency asynchronous DAPS handover in FR1	2521

A.6.3.1.8.1	Test Purpose and Environment	2521

A.6.3.1.8.2	Test Parameters	2522

A.6.3.1.8.3	Test Requirements	2524

A.6.3.1.9	Intra-band inter-frequency synchronous DAPS handover test in SA for FR1	2524

A.6.3.1.9.1	Test Purpose and Environment	2524

A.6.3.1.9.2	Test Parameters	2525

A.6.3.1.9.3	Test Requirements	2527

A.6.3.1.10	Intra-band inter-frequency asynchronous DAPS handover test in SA for FR1	2527

A.6.3.1.10.1	Test Purpose and Environment	2527

A.6.3.1.10.2	Test Parameters	2527

A.6.3.1.10.3	Test Requirements	2529

A.6.3.1.11	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1	2529

A.6.3.1.11.1	Test Purpose and Environment	2529

A.6.3.1.11.2	Test Parameters	2529

A.6.3.1.11.3 Test Requirements	2533

A.6.3.1.12	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1	2534

A.6.3.1.12.1	Test Purpose and Environment	2534

A.6.3.1.12.2	Test Parameters	2534

A.6.3.1.12.3	Test Requirements	2538

A.6.3.1.13	SA NR - E-UTRAN with NR PSCell addition in FR1	2538

A.6.3.1.13.1	Test Purpose and Environment	2538

A.6.3.1.13.2	Test Requirements	2543

A.6.3.1.14	SA NR - E-UTRAN handover with NR FR1 PSCell addition	2543

A.6.3.1.14.1	Test Purpose and Environment	2543

A.6.3.1.14.2	Test Requirements	2548

A.6.3.1.15	Intra-frequency handover from FR1 to FR1; known target cell configured with NCD-SSB	2549

A.6.3.1.15.1	Test Purpose and Environment	2549

A.6.3.1.15.2	Test Parameters	2549

A.6.3.1.15.3	Test Requirements	2552

A.6.3.1.16	Inter-frequency handover from FR1 to FR1; known target cell configured with NCD-SSB	2552

A.6.3.1.16.1	Test Purpose and Environment	2552

A.6.3.1.16.2	Test Parameters	2552

A.6.3.1.16.3 Test Requirements	2554

A.6.3.1.17	Handover with PSCell change delay from NR-DC (FR1-FR1) to NR-DC (FR1-FR1)	2554

A.6.3.1.17.1	Test Purpose and Environment	2554

A.6.3.1.17.2	Test Requirements	2558

A.6.3.1.18	Intra-frequency handover from FR1 to FR1; unknown target cell operating with 12 PRB SSB bandwidth	2558

A.6.3.1.18.2	Test Parameters	2558

A.6.3.1.18.3	Test Requirements	2559

A.6.3.2	RRC Connection Mobility Control	2560

A.6.3.2.1	SA: RRC Re-establishment	2560

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	2560

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	2562

A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing	2565

A.6.3.2.2	Random Access	2567

A.6.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	2567

A.6.3.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	2570

A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone	2573

A.6.3.2.2.4	2-step RA type non-contention based test in FR1 for NR standalone	2576

A.6.3.2.3	SA: RRC Connection Release with Redirection	2578

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	2578

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	2580

A.6.3.2.4	LTM PDCCH-order Random Access	2583

A.6.3.2.4.1	PDCCH-order RACH on neighbor cell in FR1 when RACH BW is within active UL BWP	2583

A.6.3.2.4.2	PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 for LTM	2587

A.6.3.2.4.3	PDCCH-order RACH on neighbor cell without L1-RSRP measurement in FR1 when RACH BW is within active UL BWP	2591

A.6.3.3	Conditional handover	2594

A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1	2594

A.6.3.3.1.1	Test Purpose and Environment	2594

A.6.3.3.1.2	Test Parameters	2594

A.6.3.3.1.3	Test Requirements	2596

A.6.3.3.2	Inter-frequency conditional handover from FR1 to FR1	2596

A.6.3.3.2.1	Test Purpose and Environment	2596

A.6.3.3.2.2	Test Parameters	2596

A.6.3.3.2.3	Test Requirements	2598

A.6.3.3.3	NR conditional handover including target MCG and target SCG from FR1-FR1 NR-DC to FR1-FR1 NR-DC	2598

A.6.3.3.3.1	Test Purpose and Environment	2598

A.6.3.3.3.2	Test Requirements	2602

A.6.3.3.4	NR conditional handover including target MCG and candidate SCG from FR1-FR1 NR-DC to FR1-FR1 NR-DC	2602

A.6.3.3.4.1	Test Purpose and Environment	2602

A.6.3.3.4.2	Test Parameters	2602

A.6.3.3.4.3	Test Requirements	2606

A.6.3.3.5	NR conditional handover including target MCG and candidate SCG from FR1-FR1 NR-DC to FR1-FR1 NR-DC with complementary conditional handover configuration	2606

A.6.3.3.5.1	Test Purpose and Environment	2606

A.6.3.3.5.2	Test Parameters	2607

A.6.3.3.5.3	Test Requirements	2610

A.6.3.3.6	NES triggering intra-frequency conditional handover from FR1 to FR1	2610

A.6.3.3.6.1	Test Purpose and Environment	2610

A.6.3.3.6.2	Test Parameters	2610

A.6.3.3.6.3	Test Requirements	2612

A.6.3.3.7	NES-based Inter-frequency conditional handover from FR1 to FR1	2612

A.6.3.3.7.1	Test Purpose and Environment	2612

A.6.3.3.7.2	Test Parameters	2612

A.6.3.3.7.3	Test Requirements	2614

A.6.3.4	LTM PCell Switch	2614

A.6.3.4.1	RACH-based Intra-frequency PCell switch from FR1 to FR1	2615

A.6.3.4.1.1	Test Purpose and Environment	2615

A.6.3.4.1.2	Test Parameters	2615

A.6.3.4.1.3	Test Requirements	2617

A.6.3.4.2	RACH based Inter-frequency LTM PCell switch from FR1 to FR1	2618

A.6.3.4.2.1	Test Purpose and Environment	2618

A.6.3.4.2.2	Test Parameters	2618

A.6.3.4.2.3	Test Requirements	2621

A.6.3.4.3	RACH-less Intra-frequency PCell switch from FR1 to FR1	2622

A.6.3.4.3.1	Test Purpose and Environment	2622

A.6.3.4.3.2	Test Parameters	2622

A.6.3.4.3.3	Test Requirements	2626

A.6.3.4.4	RACH-less Intra-frequency PCell switch from FR1 to FR1 without L1-RSRP measurement	2626

A.6.3.4.4.1	Test Purpose and Environment	2626

A.6.3.4.4.2	Test Parameters	2626

A.6.3.4.4.3	Test Requirements	2630

A.6.3.5	LTM PSCell Switch	2630

A.6.3.5.1 RACH-based intra-frequency LTM PSCell switch from FR1 to FR1	2630

A.6.3.5.1.1	Test Purpose and Environment	2630

A.6.3.5.1.2	Test Parameters	2630

A.6.3.5.1.3	Test Requirements	2635

A.6.4	Timing	2635

A.6.4.1	UE transmit timing	2635

A.6.4.1.1	NR UE Transmit Timing Test for FR1	2635

A.6.4.1.1.1	Test Purpose and environment	2635

A.6.4.1.1.2	Test requirements	2637

A.6.4.1.2	NR UE Transmit Timing Test for two TRPs in FR1	2638

A.6.4.1.2.1	Test Purpose and environment	2638

A.6.4.1.2.2	Test requirements	2640

A.6.4.2	UE timer accuracy	2641

A.6.4.3	Timing advance	2641

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	2641

A.6.4.3.1.1	Test Purpose and Environment	2641

A.6.4.3.1.2	Test Parameters	2641

A.6.4.3.1.3	Test Requirements	2644

A.6.5	Signalling characteristics	2644

A.6.5.1	Radio link Monitoring	2644

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	2644

A.6.5.1.1.1	Test Purpose and Environment	2644

A.6.5.1.1.2	Test Requirements	2647

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	2647

A.6.5.1.2.1	Test Purpose and Environment	2647

A.6.5.1.2.2	Test Requirements	2650

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	2650

A.6.5.1.3.1	Test Purpose and Environment	2650

A.6.5.1.3.2	Test Requirements	2653

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	2653

A.6.5.1.4.1	Test Purpose and Environment	2653

A.6.5.1.4.2	Test Requirements	2656

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	2656

A.6.5.1.5.1	Test Purpose and Environment	2656

A.6.5.1.5.2	Test Requirements	2659

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	2659

A.6.5.1.6.1	Test Purpose and Environment	2659

A.6.5.1.6.2	Test Requirements	2662

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	2662

A.6.5.1.7.1	Test Purpose and Environment	2662

A.6.5.1.7.2	Test Requirements	2665

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	2666

A.6.5.1.8.1	Test Purpose and Environment	2666

A.6.5.1.8.2	Test Requirements	2669

A.6.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion	2669

A.6.5.1.9.1	Test Purpose and Environment	2669

A.6.5.1.9.2	Test Requirements	2672

A.6.5.1.10	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode when CD-SSB is outside active BWP	2672

A.6.5.1.10.1	Test Purpose and Environment	2672

A.6.5.1.11	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode when CD-SSB is outside active BWP	2673

A.6.5.1.11.1	Test Purpose and Environment	2673

A.6.5.1.11.2	Test Requirements	2673

A.6.5.1.12	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for UE supporting NCD-SSB based measurement outside active BWP	2673

A.6.5.1.12.1	Test Purpose and Environment	2673

A.6.5.1.12.2	Test Requirements	2676

A.6.5.1.13	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for UE operating on a cell with less than 5 MHz BW	2676

A.6.5.1.13.1	Test Purpose and Environment	2676

A.6.5.1.13.2	Test Requirements	2677

A.6.5.1.14	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for UE operating on a cell with less than 5 MHz BW	2677

A.6.5.1.14.1	Test Purpose and Environment	2677

A.6.5.1.14.2	Test Requirements	2678

A.6.5.1.15	Radio Link Monitoring In-sync Test for FR1 PCell with 3 MHz Channel Bandwidth configured with SSB-based RLM RS in non-DRX mode	2678

A.6.5.1.15.1	Test Purpose and Environment	2678

A.6.5.1.15.2	Test Requirements	2679

A.6.5.1.16	Radio Link Monitoring In-sync Test for FR1 PCell with 3MHz Channel Bandwidth configured with SSB-based RLM RS in DRX mode	2679

A.6.5.1.16.1	Test Purpose and Environment	2679

A.6.5.1.16.2	Test Requirements	2680

A.6.5.2	Interruption	2681

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1	2681

A.6.5.2.1.2	Test Requirements	2684

A.6.5.2.2	SA interruptions at NR SRS carrier based switching	2685

A.6.5.2.2.1	Test Purpose and Environment	2685

A.6.5.2.2.2	Test Parameters	2685

A.6.5.2.2.3	Test Requirements	2687

A.6.5.2.3	SA interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in NR-CA	2687

A.6.5.2.3.1	Test Purpose and Environment	2687

A.6.5.2.3.2	Test Parameters	2687

A.6.5.2.3.3	Test Requirements	2689

A.6.5.2.4	SA interruptions at NR SRS antenna port switching	2689

A.6.5.2.4.1	Test Purpose and Environment	2689

A.6.5.2.4.2	Test Parameters	2689

A.6.5.2.4.3	Test Requirements	2691

A.6.5.3	SCell Activation and Deactivation Delay	2691

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle	2691

A.6.5.3.1.1	Test Purpose and Environment	2691

A.6.5.3.1.2	Test Requirements	2696

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	2696

A.6.5.3.2.1	Test Purpose and Environment	2696

A.6.5.3.2.2	Test Requirements	2697

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	2697

A.6.5.3.3.1	Test Purpose and Environment	2697

A.6.5.3.3.2	Test Requirements	2698

A.6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1	2698

A.6.5.3.4.1	Test Purpose and Environment	2698

A.6.5.3.4.2	Test Requirements	2701

A.6.5.3.5	Direct SCell activation at handover with known SCell in FR1	2702

A.6.5.3.5.1	Test Purpose and Environment	2702

A.6.5.3.5.2	Test Requirements	2706

A.6.5.3.6	PUCCH SCell Activation and deactivation of known SCell in FR1	2707

A.6.5.3.6.1	Test Purpose and Environment	2707

A.6.5.3.6.2	Test Requirements	2710

A.6.5.3.7	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	2710

A.6.5.3.7.1	Test Purpose and Environment	2710

A.6.5.3.7.2	Test Requirements	2713

A.6.5.3.8	SCell Activation and Deactivation of one FR1 known PUCCH SCell and one FR1 unknown SCell with single activation/deactivation command	2714

A.6.5.3.8.1	Test Purpose and Environment	2714

A.6.5.3.8.2	Test Requirements	2717

A.6.5.3.9	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR1 in non-DRX	2718

A.6.5.3.9.1	Test Purpose and Environment	2718

A.6.5.3.9.2	Test Requirements	2721

A.6.5.3.10	Fast SCell Activation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle	2721

A.6.5.3.10.1	Test Purpose and Environment	2721

A.6.5.3.10.2	Test Requirements	2724

A.6.5.3.11	SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	2725

A.6.5.3.11.1	Test Purpose and Environment	2725

A.6.5.3.11.2	Test Requirements	2725

A.6.5.3.12	SCell Activation and deactivation of unknown SCell in FR1 in DRX for UE capable of short measurement interval	2725

A.6.5.3.12.1	Test Purpose and Environment	2725

A.6.5.3.12.2	Test Requirements	2728

A.6.5.3.13	SCell Activation of multiple unknown SCells in FR1 with L3 reporting with single activation/deactivation commandin non-DRX	2728

A.6.5.3.13.1	Test Purpose and Environment	2728

A.6.5.3.13.2	Test Requirements	2733

A.6.5.3.14	SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160 ms SCell measurement cycle	2733

A.6.5.3.14.1	Test Purpose and Environment	2733

A.6.5.3.14.2	Test Requirements	2738

A.6.5.3.15	TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX	2739

A.6.5.3.15.1	Test Purpose and Environment	2739

A.6.5.3.15.2	Test Requirements	2743

A.6.5.3.16	Inter-band SSB-less SCell Activation based on A-TRS	2743

A.6.5.3.16.1	Test Purpose and Environment	2743

A.6.5.3.16.2	Test Requirements	2747

A.6.5.4	UE UL carrier RRC reconfiguration Delay	2748

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	2748

A.6.5.4.1.1	Test Purpose and Environment	2748

A.6.5.4.1.2	Test Requirements	2751

A.6.5.4.2	Void	2751

A.6.5.5	Beam Failure Detection and Link recovery procedures	2751

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	2751

A.6.5.5.1.1	Test Purpose and Environment	2751

A.6.5.5.1.2	Test Requirements	2755

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	2755

A.6.5.5.2.1	Test Purpose and Environment	2755

A.6.5.5.2.2	Test Requirements	2759

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	2759

A.6.5.5.3.1	Test Purpose and Environment	2759

A.6.5.5.3.2	Test Requirements	2762

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	2763

A.6.5.5.4.1	Test Purpose and Environment	2763

A.6.5.5.4.2	Test Requirements	2767

A.6.5.5.5	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	2767

A.6.5.5.5.1	Test Purpose and Environment	2767

A.6.5.5.5.2	Test Requirements	2770

A.6.5.5.6	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	2771

A.6.5.5.6.1	Test Purpose and Environment	2771

A.6.5.5.6.2	Test Requirements	2774

A.6.5.5.7	TRP Specific Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	2775

A.6.5.5.7.1	Test Purpose and Environment	2775

A.6.5.5.7.2	Test Requirements	2778

A.6.5.5.8	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for a UE operating on a cell with less than 5 MHz BW	2779

A.6.5.5.8.1	Test Purpose and Environment	2779

A.6.5.5.8.2	Test Requirements	2780

A.6.5.6	Active BWP switch	2780

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	2780

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of SCell with non-DRX in SA	2780

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	2785

A.6.5.6.2	RRC-based Active BWP Switch	2788

A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	2788

A.6.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	2790

A.6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2790

A.6.5.6.4	SCell dormancy switch	2796

A.6.5.6.4.1	NR FR1 PCell SCell dormancy switch of single FR1 SCell outside active time	2796

A.6.5.6.4.1.1	Test Purpose and Environment	2796

A.6.5.6.4.1.2	Test Requirements	2800

A.6.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR1 SCells inside active time	2801

A.6.5.6.4.2.1	 Test Purpose and Environment	2801

A.6.5.6.4.2.2	Test Requirements	2806

A.6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	2806

A.6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2806

A.6.5.7	DL interruptions at switching between two uplink carriers	2810

A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	2810

A.6.5.7.1.1	Test Purpose and Environment	2810

A.6.5.7.1.2	Test Requirements	2814

A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	2814

A.6.5.7.2.1	Test Purpose and Environment	2814

A.6.5.7.2.2	Test Requirements	2817

A.6.5.7A	DL interruptions at switching between two uplink carriers with two transmit antenna connectors	2817

A.6.5.7A.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	2817

A.6.5.7A.1.1	Test Purpose and Environment	2817

A.6.5.7A.1.2	Test Requirements	2821

A.6.5.7A.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	2821

A.6.5.7A.2.1	Test Purpose and Environment	2821

A.6.5.7A.2.2	Test Requirements	2823

A.6.5.7B	DL interruptions at switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	2824

A.6.5.7B.1	DL interruptions at switching between two uplink bands in FDD-TDD CA	2824

A.6.5.7B.1.1	Test Purpose and Environment	2824

A.6.5.7B.1.2	Test Requirements	2828

A.6.5.7B.2	DL interruptions at switching between two uplink bands in TDD-TDD CA	2828

A.6.5.7B.2.1	Test Purpose and Environment	2828

A.6.5.7B.2.2	Test Requirements	2833

A.6.5.7C	DL interruptions at switching between two uplink bands with two transmit antenna connectors	2833

A.6.5.7C.1	DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA	2833

A.6.5.7C.1.1	Test Purpose and Environment	2833

A.6.5.7C.1.2	Test Requirements	2839

A.6.5.7C.2	DL interruptions at switching between two uplink bands with two transmit antenna connectors in TDD-TDD CA	2839

A.6.5.7C.2.1	Test Purpose and Environment	2839

A.6.5.7C.2.2	Test Requirements	2844

A.6.5.7D	DL interruptions at UE switching across three or four uplink bands	2844

A.6.5.7D.1	DL interruptions at switching across three uplink bands in TDD-TDD CA for single TAG	2844

A.6.5.7D.1.1	Test Purpose and Environment	2844

A.6.5.7D.1.2	Test Requirements	2848

A.6.5.7D.2	DL interruptions at switching across four uplink bands in FDD-TDD CA for single TAG	2848

A.6.5.7D.2.1	Test Purpose and Environment	2848

A.6.5.7D.2.2	Test Requirements	2852

A.6.5.7D.3	DL interruptions at switching across three uplink bands in FDD-TDD CA for two TAGs	2852

A.6.5.7D.3.1	Test Purpose and Environment	2852

A.6.5.7D.3.2	Test Requirements	2856

A.6.5.7D.4	DL interruptions at switching across four uplink bands in TDD-TDD CA for two TAGs	2856

A.6.5.7D.4.1	Test Purpose and Environment	2856

A.6.5.7D.7.2	Test Requirements	2861

A.6.5.8	UE specific CBW change	2861

A.6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	2861

A.6.5.8.1.1	Test Purpose and Environment	2861

A.6.5.8.1.2	Test Requirements	2863

A.6.5.9	Pathloss reference signal switching delay	2863

A.6.5.9.1	MAC-CE based pathloss reference signal switch delay	2863

A.6.5.9.1.1	Test Purpose and Environment	2863

A.6.5.9.1.2	Test Requirements	2866

A.6.5.10	Conditional PSCell addition and release delay (FR1 NR-DC)	2866

A.6.5.10.1	Conditional PSCell Addition and Release Delay	2866

A.6.5.10.1.1	Test purpose and environment	2866

A.6.5.10.1.2	Test Parameters	2866

A.6.5.10.1.3	Test Requirements	2869

A.6.5.11	PSCell addition and release delay	2869

A.6.5.11.1	Addition and Release Delay of unknown NR FR1 PSCell	2869

A.6.5.11.1.1	Test purpose and environment	2869

A.6.5.11.1.2	Test Requirements	2871

A.6.5.12	Subsequent conditional PSCell addition/change	2872

A.6.5.12.1	Intra-frequency subsequent CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC	2872

A.6.5.12.1.1	Test purpose and environment	2872

A.6.5.12.1.2	Test Parameters	2872

A.6.5.12.1.3	Test Requirements	2874

A.6.5.12.2	Inter-frequency subsequent CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC	2875

A.6.5.12.2.1	Test purpose and environment	2875

A.6.5.12.2.2	Test Parameters	2875

A.6.5.12.2.3	Test Requirements	2878

A.6.5.13	Active TCI state switch delay	2878

A.6.5.13.1	MAC-CE based joint TCI state switch for mDCI with two TA when RTD is larger than CP	2878

A.6.5.13.1.1	Test Purpose and Environment	2878

A.6.5.13.1.2	Test Requirements	2880

A.6.6	Measurement procedure	2881

A.6.6.1	Intra-frequency Measurements	2881

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	2881

A.6.6.1.1.1	Test purpose and Environment	2881

A.6.6.1.1.2	Test parameters	2881

A.6.6.1.1.3	Test Requirements	2883

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	2883

A.6.6.1.2.1	Test purpose and Environment	2883

A.6.6.1.2.2	Test parameters	2883

A.6.6.1.2.3	Test Requirements	2885

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	2885

A.6.6.1.3.1	Test purpose and Environment	2885

A.6.6.1.3.2	Test parameters	2885

A.6.6.1.3.3	Test Requirements	2887

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	2887

A.6.6.1.4.1	Test purpose and Environment	2887

A.6.6.1.4.2	Test parameters	2888

A.6.6.1.4.3	Test Requirements	2890

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	2890

A.6.6.1.5.1	Test purpose and Environment	2890

A.6.6.1.5.2	Test parameters	2890

A.6.6.1.5.3	Test Requirements	2891

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	2891

A.6.6.1.6.1	Test purpose and Environment	2891

A.6.6.1.6.2	Test parameters	2892

A.6.6.1.6.3	Test Requirements	2893

A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	2893

A.6.6.1.7.1	Test purpose and Environment	2893

A.6.6.1.7.2	Test parameters	2893

A.6.6.1.7.3	Test Requirements	2895

A.6.6.1.8	SA event triggered reporting tests without gap under DRX for UE configured with highSpeedMeasCA-Scell-r17	2896

A.6.6.1.8.1	Test purpose and Environment	2896

A.6.6.1.8.2	Test parameters	2896

A.6.6.1.8.3	Test Requirements	2898

A.6.6.1.9	SA event triggered reporting tests with MUSIM gap configured	2898

A.6.6.1.9.1	Test purpose and Environment	2898

A.6.6.1.9.2	Test parameters	2898

A.6.6.1.9.3	Test requirements	2900

A.6.6.1.10	SA event triggered reporting tests without gap under non-DRX when CD-SSB is outside active BWP	2900

A.6.6.1.10.1	Test purpose and Environment	2900

A.6.6.1.10.2	Test Requirements	2901

A.6.6.1.11	SA event triggered reporting tests without gap under non-DRX with NCD-SSB	2901

A.6.6.1.11.1	Test purpose and Environment	2901

A.6.6.1.11.2	Test parameters	2901

A.6.6.1.11.3	Test Requirements	2902

A.6.6.1.12	SA event triggered reporting tests without gap under non-DRX with SSB index reading and 12 PRB SSB	2903

A.6.6.1.12.1	Test purpose and Environment	2903

A.6.6.1.12.2	Test parameters	2903

A.6.6.1.12.3	Test Requirements	2903

A.6.6.1.13	SA event triggered reporting tests without gap under Cell DTX	2903

A.6.6.1.13.1	Test purpose and Environment	2904

A.6.6.1.13.2	Test parameters	2904

A.6.6.1.13.3	Test Requirements	2905

A.6.6.2	Inter-frequency Measurements	2906

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2906

A.6.6.2.1.1	Test Purpose and Environment	2906

A.6.6.2.1.2	Test Requirements	2908

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2908

A.6.6.2.2.1	Test Purpose and Environment	2908

A.6.6.2.2.2	Test Requirements	2911

A.6.6.2.3	Void	2911

A.6.6.2.4	Void	2911

A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2911

A.6.6.2.5.1	Test Purpose and Environment	2911

A.6.6.2.5.2	Test Requirements	2914

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2914

A.6.6.2.6.1	Test Purpose and Environment	2914

A.6.6.2.6.2	Test Requirements	2916

A.6.6.2.7	Void	2917

A.6.6.2.8	Void	2917

A.6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern	2917

A.6.6.2.9.1	Test Purpose and Environment	2917

A.6.6.2.9.2	Test Requirements	2919

A.6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used	2919

A.6.6.2.10.1	Test Purpose and Environment	2919

A.6.6.2.10.2	Test Requirements	2921

A.6.6.2.12	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17	2924

A.6.6.2.12.1	Test Purpose and Environment	2924

A.6.6.2.12.2	Test Requirements	2926

A.6.6.2.13	SA event triggered reporting tests for FR1 with measurement gap with priority and periodic MUSIM gap configured	2927

A.6.6.2.13.1	Test Purpose and Environment	2927

A.6.6.2.13.2	Test Requirements	2929

A.6.6.2.14	SA event triggered reporting tests for FR1 with measurement gap without priority and periodic MUSIM gapconfigured	2929

A.6.6.2.14.1	Test Purpose and Environment	2929

A.6.6.2.14.2	Test Requirements	2932

A.6.6.2.15	SA event triggered reporting tests for FR1 with 3 MHz Channel Bandwidth configured without SSB time index detection when DRX is used	2932

A.6.6.2.15.1	Test Purpose and Environment	2932

A.6.6.2.15.2	Test Requirements	2933

A.6.6.3	Inter-RAT Measurements	2933

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	2933

A.6.6.3.1.1	Test Purpose and Environment	2933

A.6.6.3.1.2	Test Requirements	2936

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	2936

A.6.6.3.2.1	Test Purpose and Environment	2936

A.6.6.3.2.2	Test Requirements	2939

A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16	2940

A.6.6.3.3.1	Test Purpose and Environment	2940

A.6.6.3.3.2	Test Requirements	2943

A.6.6.4	L1-RSRP measurement for beam reporting	2943

A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used	2943

A.6.6.4.1.1	Test Purpose and Environment	2943

A.6.6.4.1.2	Test parameters	2943

A.6.6.4.1.3	Test Requirements	2945

A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used	2945

A.6.6.4.2.1	Test Purpose and Environment	2945

A.6.6.4.2.2	Test parameters	2945

A.6.6.4.2.3	Test Requirements	2947

A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	2947

A.6.6.4.3.1	Test Purpose and Environment	2947

A.6.6.4.3.2	Test parameters	2947

A.6.6.4.3.3	Test Requirements	2949

A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	2949

A.6.6.4.4.1	Test Purpose and Environment	2949

A.6.6.4.4.2	Test parameters	2949

A.6.6.4.4.3	Test Requirements	2950

A.6.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	2951

A.6.6.4.5.1	Test Purpose and Environment	2951

A.6.6.4.5.2	Test parameters	2951

A.6.6.4.5.3	Test Requirements	2952

A.6.6.4.6	Inter-cell SSB based L1-RSRP measurements on FR1 PCell when DRX is used	2953

A.6.6.4.6.1	Test Purpose and Environment	2953

A.6.6.4.6.2	Test parameters	2953

A.6.6.4.6.3	Test Requirements	2954

A.6.6.4.7	SSB based L1-RSRP measurement when DRX is not used when CD-SSB is outside active BWP	2955

A.6.6.4.7.1	Test Purpose and Environment	2955

A.6.6.4.7.2	Test Requirements	2955

A.6.6.4.8	CSI-RS based L1-RSRP measurement when DRX is not used when CD-SSB is outside active BWP	2955

A.6.6.4.8.1	Test Purpose and Environment	2955

A.6.6.4.9	SSB based L1-RSRP measurement for UE supporting NCD-SSB based L1 measurement outside active BWP when DRX is not used	2955

A.6.6.4.9.1	Test Purpose and Environment	2955

A.6.6.4.9.2	Test parameters	2956

A.6.6.4.9.3	Test Requirements	2957

A.6.6.5	Inter-RAT UTRAN FDD measurements	2957

A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1	2957

A.6.6.5.1.1	Test Purpose and Environment	2957

A.6.6.5.1.2	Test Requirements	2960

A.6.6.6	CLI measurements	2960

A.6.6.6.1	SRS-RSRP measurement with DRX	2960

A.6.6.6.1.1	Test Purpose and Environment	2960

A.6.6.6.1.2	Test Parameters	2960

A.6.6.6.1.3	Test Requirements	2962

A.6.6.6.2	CLI-RSSI measurement with DRX	2962

A.6.6.6.2.1	Test Purpose and Environment	2962

A.6.6.6.2.2	Test Parameters	2963

A.6.6.6.2.3	Test Requirements	2964

A.6.6.7	NR measurements with autonomous gaps	2964

A.6.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	2964

A.6.6.7.1.1	Test Purpose and Environment	2964

A.6.6.7.1.2	Test Parameters	2964

A.6.6.7.1.3	Test Requirements	2967

A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	2967

A.6.6.7.2.1	Test Purpose and Environment	2967

A.6.6.7.2.2	Test Requirements	2969

A.6.6.8	L1-SINR measurement for beam reporting	2970

A.6.6.8.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is used	2970

A.6.6.8.1.1	Test Purpose and Environment	2970

A.6.6.8.1.2	Test parameters	2970

A.6.6.8.1.3	Test Requirements	2972

A.6.6.8.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	2972

A.6.6.8.2.1	Test Purpose and Environment	2972

A.6.6.8.2.2	Test parameters	2972

A.6.6.8.2.3	Test Requirements	2974

A.6.6.8.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	2974

A.6.6.8.3.1	Test Purpose and Environment	2974

A.6.6.8.3.2	Test parameters	2975

A.6.6.8.3.3	Test Requirements	2976

A.6.6.9	Idle Mode CA/DC Measurements	2976

A.6.6.9.1	SA Idle mode CA/DC measurement for FR1	2976

A.6.6.9.1.1	Test Purpose and Environment	2976

A.6.6.9.1.2	Test Requirements	2980

A.6.6.9.2	 Idle mode fast CA/DC eEMR measurement for FR1 without valid reporting	2980

A.6.6.9.2.1	Test Purpose and Environment	2980

A.6.6.9.2.2	Test Requirements	2983

A.6.6.9.3	Idle mode fast CA/DC cell reselection measurement for FR1 without valid reporting	2983

A.6.6.9.3.1	Test Purpose and Environment	2983

A.6.6.9.3.2	Test Requirements	2986

A.6.6.9.4	Idle mode fast CA/DC cell reselection measurement for FR1 with valid reporting	2986

A.6.6.9.4.1	Test Purpose and Environment	2986

A.6.6.9.4.2	Test Requirements	2989

A.6.6.10	CSI-RS based intra-frequency Measurements	2989

A.6.6.10.1	SA event triggered reporting tests without gap under non-DRX	2989

A.6.6.10.1.1	Test purpose and Environment	2989

A.6.6.10.1.2	Test Requirements	2991

A.6.6.11	CSI-RS based inter-frequency Measurements	2992

A.6.6.11.1	 SA event triggered reporting tests with gap under DRX	2992

A.6.6.11.1.1	Test Purpose and Environment	2992

A.6.6.11.1.2	Test Requirements	2994

A.6.6.12	RSTD measurements	2994

A.6.6.12.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA	2994

A.6.6.12.1.1	Test Purpose and Environment	2994

A.6.6.12.1.2	Test Requirements	2998

A.6.6.12.2	NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR1 SA	2998

A.6.6.12.2.1	Test Purpose and Environment	2998

A.6.6.12.2.2	Test Requirements	3001

A.6.6.12.3	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA	3002

A.6.6.12.3.1	Test Purpose and Environment	3002

A.6.6.12.3.2	Test Requirements	3005

A.6.6.12.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA without measurement gap	3005

A.6.6.12.4.1	Test Purpose and Environment	3005

A.6.6.12.4.2	Test Requirements	3008

A.6.6.12.5	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_CONNECTED state with Rx TEG	3008

A.6.6.12.5.1	Test Purpose and Environment	3008

A.6.6.12.5.2	Test Requirements	3012

A.6.6.12.6	NR RSTD measurement reporting delay test case for PRS aggregation in FR1 SA in RRC_CONNECTED mode	3012

A.6.6.12.6.1	Test Purpose and Environment	3012

A.6.6.12.6.2	Test Requirements	3019

A.6.6.13 PRS-RSRP measurements	3019

A.6.6.13.1	PRS-RSRP reporting delay test case for single positioning frequency layer	3019

A.6.6.13.1.1	Test purpose and Environment	3019

A.6.6.13.1.2	Test Requirements	3021

A.6.6.13.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	3021

A.6.6.13.2.1	Test purpose and Environment	3021

A.6.6.13.2.2	Test Requirements	3024

A.6.6.13.3	PRS-RSRP reporting delay test case for reduced number of samples	3024

A.6.6.13.3.1	Test purpose and Environment	3024

A.6.6.13.3.2	Test Requirements	3026

A.6.6.13.4	PRS-RSRP reporting delay test case for single positioning frequency layer outside MG	3026

A.6.6.13.4.1	Test purpose and Environment	3026

A.6.6.14	UE Rx-Tx time difference measurements	3029

A.6.6.14.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	3029

A.6.6.14.1.1	Test purpose and environment	3029

A.6.6.14.1.2	Test requirements	3031

A.6.6.14.2	UE Rx-Tx time difference measurement for dual positioning frequency layers in FR1 SA	3031

A.6.6.14.2.1	Test purpose and environment	3031

A.6.6.14.2.2	Test requirements	3033

A.6.6.14.3	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with reduced sample number	3034

A.6.6.14.3.1	Test purpose and environment	3034

A.6.6.14.3.2	Test requirements	3036

A.6.6.14.4	UE Rx-Tx time difference measurement without gaps in FR1 SA	3036

A.6.6.14.4.1	Test purpose and environment	3036

A.6.6.14.4.2	Test requirements	3038

A.6.6.14.5	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with multiple RxTx TEGs	3038

A.6.6.14.4.1	Test purpose and environment	3038

A.6.6.14.4.2	Test requirements	3040

A.6.6.14.6	UE Rx-Tx time difference measurements with PRS bandwidth aggregation in FR1 SA	3041

A.6.6.14.6.1	Test purpose and environment	3041

A.6.6.14.6.2	Test requirements	3044

A.6.6.15	Idle Mode measurements of inter-RAT DC candidate cells for early reporting	3044

A.6.6.15.1	Test Purpose and Environment	3044

A.6.6.15.2	Test Requirements	3048

A.6.6.16	PRS-RSRPP measurements	3049

A.6.6.16.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state	3049

A.6.6.16.1.1	Test purpose and Environment	3049

A.6.6.16.1.2	Test Requirements	3051

A.6.6.16.2	PRS-RSRPP reporting delay test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state	3051

A.6.6.16.2.1	Test purpose and Environment	3051

A.6.6.16.2.2	Test Requirements	3053

A.6.6.16.3	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state without measurement gap	3053

A.6.6.16.3.1	Test purpose and Environment	3053

A.6.6.16.3.2	Test Requirements	3055

A.6.6.17	SA event triggered reporting tests with Pre-MG	3056

A.6.6.17.1	SA event triggered reporting tests with autonomous activation/deactivation Pre-MG	3056

A.6.6.17.1.1	Test purpose and Environment	3056

A.6.6.17.1.2	Test parameters	3056

A.6.6.17.1.3	Test Requirements	3058

A.6.6.17.2	SA event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation	3059

A.6.6.17.2.1	Test purpose and Environment	3059

A.6.6.17.2.2	Test parameters	3059

A.6.6.17.2.3	Test Requirements	3061

A.6.6.17.3	Void	3062

A.6.6.17.3.1	Void	3062

A.6.6.17.3.2	Void	3062

A.6.6.17.3.3	Void	3062

A.6.6.18	SA event triggered reporting tests with concurrent gaps	3062

A.6.6.18.1	SA event triggered reporting tests for FR1 concurrent gaps with non-overalpping scenario for SSB-based measurements in both inter-frequency layers	3062

A.6.6.18.1.1	Test Purpose and Environment	3062

A.6.6.18.1.2	Test Requirements	3064

A.6.6.18.2	SA event triggered reporting tests for FR1 concurrent gap with partially partial overalpping scenario for SSB-based measurements in both inter-frequency layers	3064

A.6.6.18.2.1	Test Purpose and Environment	3064

A.6.6.18.2.2	Test Requirements	3067

A.6.6.18.3	SA NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1	3067

A.6.6.18.3.1	Test Purpose and Environment	3067

A.6.6.18.3.2	Test Requirements	3071

A.6.6.18.4	SA event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used	3072

A.6.6.18.4.1	Test Purpose and Environment	3072

A.6.6.18.4.2	Test Requirements	3074

A.6.6.19	SA event triggered reporting tests with NCSG	3075

A.6.6.19.1	SA event triggered reporting tests with NCSG under non-DRX in FR1	3075

A.6.6.19.1.1	Test purpose and Environment	3075

A.6.6.19.1.2	Test parameters	3075

A.6.6.19.1.3	Test Requirements	3077

A.6.6.19.2	SA event triggered reporting tests for FR1 with NCSG for inter-frequency measurement	3077

A.6.6.19.2.1	Test Purpose and Environment	3077

A.6.6.19.2.2	Test parameters	3077

A.6.6.19.2.3	Test Requirements	3080

A.6.6.19.3	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG	3080

A.6.6.19.3.1	Test Purpose and Environment	3080

A.6.6.19.3.2	Test parameters	3080

A.6.6.19.3.3	Test Requirements	3083

A.6.6.19.4	Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX	3084

A.6.6.19.4.1	Test purpose and Environment	3084

A.6.6.19.4.2	Test parameters	3084

A.6.6.19.4.3	Test Requirements	3086

A.6.6.20	UE Rx-Tx time difference measurement for propagation delay compensation	3086

A.6.6.20.1	Test purpose and environment	3086

A.6.6.20.2	Test requirements	3088

A.6.6.21	UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA	3088

A.6.6.21.1	Test purpose and environment	3088

A.6.6.21.2	Test requirements	3090

A.6.6.22	SA event triggered reporting tests for concurrent measurement gaps with Pre-MG	3090

A.6.6.22.1	SA event triggered reporting tests for FR1 concurrent gap with Pre-MG with partially partial overalpping scenario for SSB-based measurements in both intra-frequency and inter-frequency layers	3090

A.6.6.22.1.1	Test Purpose and Environment	3090

A.6.6.22.1.2	Test Requirements	3093

A.6.6.22.2	SA event triggered reporting tests for concurrent gap with pre-configured gaps and network-controlled activation/deactivation	3094

A.6.6.22.2.1	Test purpose and Environment	3094

A.6.6.22.2.2	Test parameters	3094

A.6.6.22.2.3	Test Requirements	3096

A.6.6.23	SA event triggered reporting tests for concurrent measurement gaps with NCSG	3097

A.6.6.23.1	SA event triggered reporting tests for FR1 concurrent gaps with NCSG for partially partial overalpping scenario for SSB-based measurements in both inter-frequency layers [one MG + one NCSG]	3097

A.6.6.23.1.1	Test Purpose and Environment	3097

A.6.6.23.1.2	Test Requirements	3099

A.6.6.23.2	SA event triggered reporting tests for FR1 concurrent gaps with NCSG for partially partial overalpping scenario for SSB-based measurements in both inter-frequency layers [two NCSG]	3100

A.6.6.23.2.1	Test Purpose and Environment	3100

A.6.6.23.2.2	Test Requirements	3102

A.6.6.23.3	Event triggered reporting on SCC with deactivated SCell test with per-UE Con-NCSG under non-DRX	3102

A.6.6.23.3.1	Test purpose and Environment	3102

A.6.6.23.3.2	Test parameters	3103

A.6.6.23.3.3	Test Requirements	3105

A.6.6.24	SA event triggered reporting tests with NeedForGap in FR1	3105

A.6.6.24.1	SA event triggered reporting tests without gaps, with interruptions, under non-DRX	3105

A.6.6.24.1.1	Test purpose and Environment	3105

A.6.6.24.1.2	Test parameters	3105

A.6.6.24.1.3	Test Requirements	3107

A.6.6.24.2	SA event triggered reporting tests for FR1 without gap with interruption for inter-frequency measurement with SSB time index detection when DRX is not used	3107

A.6.6.24.2.1	Test Purpose and Environment	3107

A.6.6.24.2.2	Test parameters	3107

A.6.6.24.2.3	Test Requirements	3110

A.6.6.24.3	SA event triggered reporting tests for FR1 with ‘no-gap-with-interruption’, without measurement gap or DRX	3110

A.6.6.24.3.1	Test Purpose and Environment	3110

A.6.6.24.3.2	Test Requirements	3112

A.6.6.24.4	SA event triggered reporting tests for FR1 NeedForGaps without gap without interruption when DRX is not used	3112

A.6.6.24.4.1	Test Purpose and Environment	3112

A.6.6.24.4.2	Test parameters	3113

A.6.6.24.4.3	Test Requirements	3115

A.6.6.24.5	SA event triggered reporting tests without gap under non-DRX for UE indicating no-gap-no-interruption	3115

A.6.6.24.5.1	Test purpose and Environment	3115

A.6.6.24.5.2	Test parameters	3115

A.6.6.24.5.3	Test Requirements	3117

A.6.6.25	SA NR - E-UTRAN event-triggered without measurement gaps	3117

A.6.6.25.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	3117

A.6.6.25.1.1	Test Purpose and Environment	3117

A.6.6.25.1.2	Test Requirements	3120

A.6.6.25.2	SA NR - E-UTRAN event-triggered reporting without gap under non-DRX in FR1	3121

A.6.6.25.2.1	Test Purpose and Environment	3121

A.6.6.25.2.2	Test parameters	3121

A.6.6.25.2.3	Test Requirements	3122

A.6.6.25.3	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for UE capable of inter-RAT EUTRAN measurement without gap when CRS is contained within UE’s active DL BWP	3122

A.6.6.25.3.1	Test Purpose and Environment	3122

A.6.6.25.3.2	Test Requirements	3125

A.6.6.26	LTM Intra-frequency L1-RSRP measurement	3125

A.6.6.26.1	Intra-frequency SSB based L1-RSRP measurement in FR1	3125

A.6.6.26.1.1	Test Purpose and Environment	3125

A.6.6.26.1.2	Test Parameters	3126

A.6.6.26.1.3	Test Requirements	3128

A.6.6.27	LTM Inter-frequency L1-RSRP measurement with measurement gap	3128

A.6.6.27.1	Inter-frequency SSB based L1-RSRP measurement with measurement gap	3128

A.6.6.27.1.1	Test Purpose and Environment	3128

A.6.6.27.1.2	Test parameters	3128

A.6.6.27.1.3	Test Requirements	3130

A.6.6.28	LTM Inter-frequency L1-RSRP measurement without measurement gap	3130

A.6.6.28.1	Inter-frequency SSB based L1-RSRP measurement without measurement gap	3130

A.6.6.28.1.1	Test Purpose and Environment	3130

A.6.6.28.1.2	Test parameters	3131

A.6.6.28.1.3	Test Requirements	3133

A.6.6.29	RSCPD Measurements	3133

A.6.6.29.1	NR RSCPD with RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_CONNECTED state	3133

A.6.6.29.1.1	Test Purpose and Environment	3133

A.6.6.29.1.2	Test Requirements	3141

A.6.6.30	RSCP Measurements	3141

A.6.6.30.1	DL RSCP with UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	3141

A.6.6.30.1.1	Test purpose and environment	3141

A.6.6.30.1.2	Test requirements	3145

A.6.7	Measurement Performance requirements	3145

A.6.7.1	SS-RSRP	3145

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	3145

A.6.7.1.1.1	Test Purpose and Environment	3145

A.6.7.1.1.2	Test parameters	3145

A.6.7.1.1.3	Test Requirements	3149

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	3149

A.6.7.1.2.1	Test Purpose and Environment	3149

A.6.7.1.2.2	Test parameters	3149

A.6.7.1.2.3	Test Requirements	3152

A.6.7.1.3	Void	3152

A.6.7.2	SS-RSRQ	3152

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3152

A.6.7.2.1.1	Test Purpose and Environment	3152

A.6.7.2.1.2	Test Parameters	3153

A.6.7.2.1.3	Test Requirements	3156

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3156

A.6.7.2.2.1	Test Purpose and Environment	3156

A.6.7.2.2.2	Test Parameters	3157

A.6.7.2.2.3	Test Requirements	3160

A.6.7.3	SS-SINR	3160

A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3160

A.6.7.3.1.1	Test Purpose and Environment	3161

A.6.7.3.1.2	Test Parameters	3161

A.6.7.3.1.3	Test Requirements	3164

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3164

A.6.7.3.2.1	Test Purpose and Environment	3164

A.6.7.3.2.2	Test Parameters	3164

A.6.7.3.2.3	Test Requirements	3167

A.6.7.4	L1-RSRP measurement for beam reporting	3167

A.6.7.4.1	SSB based L1-RSRP measurement	3168

A.6.7.4.1.1	Test Purpose and Environment	3168

A.6.7.4.1.2	Test parameters	3168

A.6.7.4.1.3	Test Requirements	3171

A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	3171

A.6.7.4.2.1	Test Purpose and Environment	3171

A.6.7.4.2.2	Test parameters	3171

A.6.7.4.2.3	Test Requirements	3174

A.6.7.5	E-UTRAN RSRP	3174

A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell	3174

A.6.7.5.1.1	Test Purpose and Environment	3174

A.6.7.5.1.2	Test parameters	3174

A.6.7.5.1.3	Test Requirements	3177

A.6.7.6	E-UTRAN RSRQ	3177

A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell	3177

A.6.7.6.1.1	Test Purpose and Environment	3177

A.6.7.6.1.2	Test parameters	3177

A.6.7.6.1.3	Test Requirements	3181

A.6.7.7	E-UTRAN RS-SINR	3181

A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell	3181

A.6.7.7.1.1	Test Purpose and Environment	3181

A.6.7.7.1.2	Test parameters	3181

A.6.7.7.1.3	Test Requirements	3184

A.6.7.8	CLI measurements	3184

A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell	3184

A.6.7.8.1.1	Test Purpose and Environment	3184

A.6.7.8.1.2	Test parameters	3184

A.6.7.8.1.3	Test Requirements	3187

A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell	3187

A.6.7.8.2.1	Test Purpose and Environment	3187

A.6.7.8.2.2	Test parameters	3188

A.6.7.8.2.3	Test Requirements	3189

A.6.7.9	L1-SINR measurement for beam reporting	3189

A.6.7.9.2	L1-SINR measurement with SSB based CMR and dedicated IMR	3192

A.6.7.9.2.1	Test Purpose and Environment	3193

A.6.7.9.2.2	Test parameters	3193

A.6.7.9.2.3	Test Requirements	3196

A.6.7.9.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	3196

A.6.7.9.3.1	Test Purpose and Environment	3196

A.6.7.9.3.2	Test parameters	3197

A.6.7.9.3.3	Test Requirements	3200

A.6.7.10	CSI-RSRP	3200

A.6.7.10.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	3200

A.6.7.9.10.1	Test Purpose and Environment	3200

A.6.7.9.10.2	Test parameters	3200

A.6.7.10.1.3	Test Requirements	3203

A.6.7.10.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	3203

A.6.7.9.10.1	Test Purpose and Environment	3204

A.6.7.10.2.2	Test parameters	3204

A.6.7.10.2.3	Test Requirements	3207

A.6.7.11	CSI-RSRQ	3207

A.6.7.11.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3207

A.6.7.11.1.1	Test Purpose and Environment	3207

A.6.7.11.1.2	Test Parameters	3207

A.6.7.11.1.3	Test Requirements	3211

A.6.7.11.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3211

A.6.7.11.2.1	Test Purpose and Environment	3211

A.6.7.11.2.2	Test Parameters	3211

A.6.7.11.2.3	Test Requirements	3215

A.6.7.12	CSI-SINR	3215

A.6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3215

A.6.7.12.1.1	Test Purpose and Environment	3215

A.6.7.12.1.2	Test Parameters	3215

A.6.7.12.1.3	Test Requirements	3218

A.6.7.12.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3219

A.6.7.12.2.1	Test Purpose and Environment	3219

A.6.7.12.2.2	Test Parameters	3219

A.6.7.12.2.3	Test Requirements	3222

A.6.7.13	RSTD measurements	3222

A.6.7.13.1	RSTD measurement accuracy test case for single positioning frequency layer	3222

A.6.7.13.1.1	Test purpose and Environment	3222

A.6.7.13.1.2	Test Requirements	3224

A.6.7.13.2	RSTD measurement accuracy test case for dual positioning frequency layer	3224

A.6.7.13.2.1	Test purpose and Environment	3224

A.6.7.13.2.2	Test Requirements	3226

A.6.7.13.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state	3226

A.6.7.13.3.1	Test purpose and Environment	3226

A.6.7.13.3.2	Test Requirements	3228

A.6.7.13.4	RSTD measurement accuracy test case with Rx TEG	3228

A.6.7.13.5	NR RSTD measurement accuracy test case for PRS aggregation in FR1 SA in RRC_CONNECTED mode	3230

A.6.7.13.5.1	Test purpose and Environment	3230

A.6.7.13.5.2	Test Requirements	3233

A.6.7.14	PRS-RSRP measurements	3233

A.6.7.14.1	SA: measurement accuracy with PRS in FR1	3233

A.6.7.14.1.1	Test Purpose and Environment	3233

A.6.7.14.1.2	Test parameters	3233

A.6.7.14.1.3	Test Requirements	3235

A.6.7.14.2	SA: measurement accuracy with PRS in FR1 with reduced sample number	3235

A.6.7.14.2.1	Test Purpose and Environment	3235

A.6.7.14.2.2	Test parameters	3235

A.6.7.14.2.3	Test Requirements	3237

A.6.7.14.3	Void	3237

A.6.7.14.3.1	Void	3237

A.6.7.14.3.2	Void	3237

A.6.7.14.3.3	Void	3237

A.6.7.15	UE Rx-Tx time difference measurements	3237

A.6.7.15.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA	3237

A.6.7.15.1.1	Test purpose and environment	3237

A.6.7.15.1.2	Test parameters	3238

A.6.7.15.1.3	Test requirements	3239

A.6.7.15.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR1 SA	3239

A.6.7.15.2.1	Test purpose and environment	3239

A.6.7.15.2.2	Test parameters	3240

A.6.7.15.2.3	Test requirements	3241

A.6.7.15.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG	3241

A.6.7.15.3.1	Test purpose and environment	3241

A.6.7.15.3.2	Test parameters	3242

A.6.7.15.3.3	Test requirements	3244

A.6.7.15.4	UE Rx-Tx time difference measurement accuracy with PRS bandwidth aggregation in FR1 SA	3244

A.6.7.15.4.1	Test purpose and environment	3244

A.6.7.15.4.2	Test requirements	3247

A.6.7.16	PRS-RSRPP measurements	3247

A.6.7.16.1	SA: measurement accuracy with PRS in FR1	3247

A.6.7.16.1.1	Test Purpose and Environment	3247

A.6.7.16.1.2	Test parameters	3247

A.6.7.16.1.3	Test Requirements	3249

A.6.7.16.2	SA: measurement accuracy with reduced PRS samples in FR1	3249

A.6.7.16.2.1	Test Purpose and Environment	3249

A.6.7.16.2.2	Test parameters	3250

A.6.7.17	LTM L1-RSRP measurement	3251

A.6.7.17.1	Inter-frequency L1-RSRP accuracy requirements for neighbour cell in FR1	3251

A.6.7.17.1.1	Test Purpose and Environment	3251

A.6.7.17.1.2	Test parameters	3252

A.6.7.17.1.3	Test Requirements	3255

A.6.7.18	TDCP amplitude measurement accuracy	3255

A.6.7.18.1	TDCP amplitude measurement accuracy in FR1	3255

A.6.7.18.1.1	Test Purpose and Environment	3255

A.6.7.18.1.2	Test parameters	3255

A.6.7.18.1.3	Test Requirements	3256

A.6.7.19	RSCPD Measurements	3257

A.6.7.19.1	RSCPD with RSTD measurement accuracy in FR1 SA in RRC_CONNECTED	3257

A.6.7.19.1.1	Test purpose and environment	3257

A.6.7.19.1.2	Test parameters	3257

A.6.7.19.1.3	Test requirements	3260

A.6.7.20	RSCP Measurements	3260

A.6.7.20.1	RSCP with UE Rx-Tx time difference measurement accuracy in FR1 SA	3260

A.6.7.20.1.1	Test purpose and environment	3260

A.6.7.20.1.2	Test parameters	3261

A.6.7.20.1.3	Test requirements	3264

A.6.8	Measurement procedure in RRC_INACTIVE	3264

A.6.8.1	RSTD measurements	3264

A.6.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_INACTIVE state	3264

A.6.8.1.1.1	Test Purpose and Environment	3264

A.6.8.1.1.2	Test Requirements	3267

A.6.8.1.2	NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA	3267

A.6.8.1.2.1	Test Purpose and Environment	3267

A.6.8.1.2.1	Test Purpose and Environment	3267

A.6.8.1.2.2	Test Requirements	3270

A.6.8.1.3	NR RSTD measurement reporting delay test case for PRS aggregation in FR1 SA in RRC_INACTIVE state	3271

A.6.8.1.3.1	Test purpose and environment	3271

A.6.8.1.3.2	Test requirements	3274

A.6.8.1.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_INACTIVE state when eDRX cycle > 10.24s for non-RedCap UE	3275

A.6.8.1.4.1	Test Purpose and Environment	3275

A.6.8.1.4.2	Test Requirements	3278

A.6.8.2	PRS-RSRP measurements	3278

A.6.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	3278

A.6.8.2.1.1	Test purpose and Environment	3278

A.6.8.2.1.2	Test Requirements	3280

A.6.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE	3280

A.6.8.2.2.1	Test purpose and Environment	3280

A.6.8.2.2.2	Test Requirements	3282

A.6.8.2.3	PRS-RSRP reporting delay test case in RRC_INACTIVE state in FR1 with eDRX cycle > 10.24s	3283

A.6.8.2.3.1	Test purpose and Environment	3283

A.6.8.2.3.2	Test Requirements	3285

A.6.8.3	UE Rx-Tx time difference measurements	3286

A.6.8.3.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	3286

A.6.8.3.1.1	Test purpose and environment	3286

A.6.8.3.1.2	Test requirements	3288

A.6.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR1 SA	3288

A.6.8.3.2.1	Test purpose and environment	3288

A.6.8.3.2.2	Test requirements	3290

A.6.8.3.3	UE Rx-Tx time difference measurement for single positioning frequency layer with eDRX > 10.24s in FR1 SA	3291

A.6.8.3.3.1	Test purpose and environment	3291

A.6.8.3.3.2	Test requirements	3295

A.6.8.3.4	UE Rx-Tx time difference measurements with PRS bandwidth aggregation in FR1 SA	3295

A.6.8.3.4.1	Test purpose and environment	3295

A.6.8.3.4.2	Test requirements	3298

A.6.8.4	PRS-RSRPP measurements	3298

A.6.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state	3298

A.6.8.4.1.1	Test purpose and Environment	3299

A.6.8.4.1.2	Test Requirements	3300

A.6.8.4.2	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state for reduced number of samples	3301

A.6.8.4.2.1	Test purpose and Environment	3301

A.6.8.4.2.2	Test Requirements	3303

A.6.8.4.3	PRS-RSRPP reporting delay in RRC_INACTIVE with eDRX	3303

A.6.8.4.3.1	Test purpose and Environment	3303

A.6.8.4.3.2	Test Requirements	3306

A.6.8.5	RSCPD Measurements	3306

A.6.8.5.1	DL RSCPD reported with RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_INACTIVE state	3306

A.6.8.5.1.1	Test Purpose and Environment	3306

A.6.8.5.1.2	Test Requirements	3306

A.6.8.6	RSCP Measurements	3307

A.6.8.6.1	DL RSCP with UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	3307

A.6.8.6.1.1	Test purpose and environment	3307

A.6.8.6.1.2	Test requirements	3311

A.6.9	Measurement performance requirements in RRC_INACTIVE	3311

A.6.9.1	RSTD measurements	3311

A.6.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR1 in RRC_INACTIVE state	3311

A.6.9.1.1.1	Test purpose and Environment	3311

A.6.9.1.1.2	Test Requirements	3313

A.6.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_INACTIVE state	3313

A.6.9.1.2.1	Test purpose and Environment	3313

A.6.9.1.2.2	Test Requirements	3315

A.6.9.1.3	RSTD measurement accuracy for PRS aggregation in FR1 in RRC_INACTIVE state	3315

A.6.9.1.3.1	Test purpose and Environment	3315

A.6.9.1.3.2	Test Requirements	3318

A.6.9.2	PRS-RSRP measurements	3318

A.6.9.2.1	SA: measurement accuracy with PRS in FR1 in RRC_INACTIVE	3318

A.6.9.2.1.1	Test Purpose and Environment	3318

A.6.9.2.1.2	Test parameters	3318

A.6.9.2.1.3	Test Requirements	3320

A.6.9.2.2	SA: measurement accuracy with PRS in FR1 with reduced number of samples in RRC_INACTIVE state	3320

A.6.9.2.2.1	Test Purpose and Environment	3320

A.6.9.2.2.2	Test parameters	3320

A.6.9.2.2.3	Test Requirements	3322

A.6.9.3	UE Rx-Tx time difference measurements	3322

A.6.9.3.1.1	UE Rx-Tx time difference measurement accuracy in FR1 SA	3322

A.6.9.3.1.1.1	Test purpose and environment	3322

A.6.9.3.1.1.2 Test parameters	3323

A.6.9.3.1.1.3	Test requirements	3324

A.6.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples	3324

A.6.9.3.2.1	Test purpose and environment	3324

A.6.9.3.2.2	Test parameters	3324

A.6.9.3.2.3	Test requirements	3326

A.6.9.3.3	UE Rx-Tx time difference measurement accuracy with PRS bandwidth aggregation in FR1 SA	3326

A.6.9.3.3.1	Test purpose and environment	3326

A.6.9.3.3.2	Test requirements	3329

A.6.9.4	PRS-RSRPP measurements	3329

A.6.9.4.1	SA: PRS-RSRPP measurement accuracy in FR1 in RRC INACTIVE	3329

A.6.9.4.1.1	Test Purpose and Environment	3329

A.6.9.4.1.2	Test parameters	3329

A.6.9.4.1.3	Test Requirements	3331

A.6.9.4.2	SA: measurement accuracy with reduced PRS samples in FR1 in RRC INACTIVE	3331

A.6.9.4.2.1	Test Purpose and Environment	3331

A.6.9.4.2.2	Test parameters	3332

A.6.9.4.2.3	Test Requirements	3333

A.6.9.5	RSCPD Measurements	3334

A.6.9.5.1	RSCPD with RSTD measurement accuracy in FR1 SA in RRC_INACTIVE	3334

A.6.9.5.1.1	Test purpose and environment	3334

A.6.9.5.1.2	Test parameters	3334

A.6.9.5.1.3	Test requirements	3337

A.6.9.6	RSCP Measurements	3337

A.6.9.6.1	RSCP with UE Rx-Tx time difference measurement accuracy in FR1 SA	3337

A.6.9.6.1.1	Test purpose and environment	3337

A.6.9.6.1.2	Test parameters	3338

A.6.9.6.1.3	Test requirements	3341

A.6.10	Measurement Procedure in RRC_IDLE	3341

A.6.10.1	RSTD Measurements	3341

A.6.10.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_IDLE state for non-RedCap UE	3341

A.6.10.1.1.1	Test purpose and environment	3341

A.6.10.1.1.2	Test requirements	3344

A.6.10.1.2	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_IDLE state with eDRX cycle > 10.24s for non-RedCap UE	3345

A.6.10.1.2.1	Test Purpose and Environment	3345

A.6.10.1.2.2	Test Requirements	3348

A.6.10.1.3	NR RSTD measurement reporting delay test case for PRS aggregation in FR1 SA in RRC_IDLE state	3348

A.6.10.1.3.1	Test purpose and environment	3348

A.6.10.1.3.2	Test requirements	3348

A.6.10.2	 PRS-RSRP Measurements	3349

A.6.10.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_IDLE state for non-RedCap UE in FR1	3349

A.6.10.2.1.1	Test purpose and Environment	3349

A.6.10.2.1.2	Test Requirements	3351

A.6.10.2.2	PRS-RSRP reporting delay test case in RRC_IDLE state in FR1 when eDRX cycle > 10.24s	3352

A.6.10.2.2.1	Test purpose and Environment	3352

A.6.10.2.2.2	Test Requirements	3352

A.6.10.3	RSCPD Measurements	3352

A.6.10.3.1	DL RSCPD reported with RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_IDLE state	3352

A.6.10.3.1.1	Test Purpose and Environment	3352

A.6.10.3.1.2	Test Requirements	3353

A.6.11	Measurement Performance Requirements in RRC_IDLE	3353

A.6.11.1	RSTD Measurements	3353

A.6.11.1.1	NR RSTD measurement accuracy test case for single positioning frequency layer in FR1 SA in RRC_IDLE state for non-RedCap UE	3353

A.6.11.1.1.1	Test purpose and environment	3353

A.6.11.1.1.2	Test requirements	3355

A.6.11.1.2	RSTD measurement accuracy test case for single positioning frequency layer in FR1 in RRC_IDLE state with eDRX>10.24s for non-RedCap UE	3355

A.6.11.1.2.1	Test purpose and Environment	3355

A.6.11.1.2.2	Test Requirements	3357

A.6.11.1.3	NR RSTD measurement accuracy test case for PRS aggregation in FR1 SA in RRC_IDLE state	3357

A.6.11.1.3.1	Test purpose and environment	3357

A.6.11.1.3.2	Test requirements	3357

A.6.11.2	PRS-RSRP measurements	3357

A.6.11.2.1	PRS-RSRP measurement accuracy test case for non-RedCap UE in FR1 in RRC_IDLE state	3357

A.6.11.2.1.1	Test Purpose and Environment	3357

A.6.11.2.1.2	Test parameters	3358

A.6.11.2.1.3	Test Requirements	3360

A.6.11.2.2	PRS-RSRP measurement accuracy test case in RRC_IDLE state in FR1 when eDRX cycle > 10.24s	3360

A.6.11.2.2.1	Test purpose and Environment	3360

A.6.11.2.2.2	Test parameters	3360

A.6.11.2.2.3	Test Requirements	3361

A.6.11.3	RSCPD Measurements	3361

A.6.11.3.1	RSCPD with RSTD measurement accuracy in FR1 SA in RRC_IDLE	3361

A.6.11.3.1.1	Test purpose and environment	3361

A.6.11.3.1.2	Test parameters	3361

A.6. 11.3.1.3	Test requirements	3364

A.7	NR standalone tests with one or more NR cells in FR2	3366

A.7.1	SA: RRC_IDLE state mobility	3366

A.7.1.1	Cell re-selection to NR	3366

A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case	3366

A.7.1.1.1.1	Test Purpose and Environment	3366

A.7.1.1.1.2	Test Parameters	3366

A.7.1.1.1.3	Test Requirements	3368

A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case	3368

A.7.1.1.2.1	Test Purpose and Environment	3369

A.7.1.1.2.2	Test Parameters	3369

A.7.1.1.2.3	Test Requirements	3370

A.7.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	3371

A.7.1.1.3.1	Test Purpose and Environment	3371

A.7.1.1.3.2	Test Parameters	3371

A.7.1.1.3.3	Test Requirements	3373

A.7.1.1.4	Cell reselection to FR2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	3373

A.7.1.1.4.1	Test Purpose and Environment	3373

A.7.1.1.4.2	Test Parameters	3374

A.7.1.1.4.3	Test Requirements	3375

A.7.1.1.5	Cell reselection to FR2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	3376

A.7.1.1.5.1	Test Purpose and Environment	3376

A.7.1.1.5.2	Test Parameters	3376

A.7.1.1.5.3	Test Requirements	3378

A.7.1.1.6	Cell reselection to FR2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	3378

A.7.1.1.6.1	Test Purpose and Environment	3378

A.7.1.1.6.2	Test Parameters	3379

A.7.1.1.6.3	Test Requirements	3380

A.7.1.1.7	Cell reselection to FR2 intra-frequency NR case for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17	3381

A.7.1.1.7.1	Test Purpose and Environment	3381

A.7.1.1.7.2	Test Parameters	3381

A.7.1.1.7.3	Test Requirements	3383

A.7.1.1.8	Cell reselection to FR2 inter-frequency NR case for UE configured with highSpeedMeasFlagFR2-r17	3384

A.7.1.1.8.1	Test Purpose and Environment	3384

A.7.1.1.8.2	Test Parameters	3384

A.7.1.1.8.3	Test Requirements	3386

A.7.2	SA: RRC_INACTIVE state mobility	3386

A.7.2.1	Small Data Transmission	3386

A.7.2.1.1	TA validation for CG-SDT in FR2	3386

A.7.2.1.1.1	Test Purpose and Environment	3386

A.7.2.1.1.2	Test Requirements	3389

A.7.2.2	Cell reselection for positioning	3390

A.7.2.2.1	Cell reselection to FR2 intra-frequency NR case with RRC_ INACTIVE eDRX and positioning SRS	3390

A.7.2.2.1.1	Test Purpose and Environment	3390

A.7.2.2.1.2	Test Parameters	3390

A.7.2.2.1.3	Test Requirements	3392

A.7.3	RRC_CONNECTED state mobility	3392

A.7.3.1	Handover	3392

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	3392

A.7.3.1.1.1	Test Purpose and Environment	3392

A.7.3.1.1.2	Test Parameters	3392

A.7.3.1.1.3	Test Requirements	3394

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	3395

A.7.3.1.2.1	Test Purpose and Environment	3395

A.7.3.1.2.2	Test Parameters	3395

A.7.3.1.2.3	Test Requirements	3396

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	3396

A.7.3.1.3.1	Test Purpose and Environment	3396

A.7.3.1.3.2	Test Parameters	3396

A.7.3.1.3.3	Test Requirements	3398

A.7.3.1.4	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2	3398

A.7.3.1.4.1	Test Purpose and Environment	3398

A.7.3.1.4.2	Test Parameters	3398

A.7.3.1.4.3 Test Requirements	3401

A.7.3.1.5	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2	3402

A.7.3.1.5.1	Test Purpose and Environment	3402

A.7.3.1.5.2	Test Parameters	3402

A.7.3.1.5.3 Test Requirements	3405

A.7.3.1.6	Handover with PSCell from SA to EN-DC with unknown FR2 target PScell	3406

A.7.3.1.6.1	Test Purpose and Environment	3406

A.7.3.1.6.2	Test Parameters	3406

A.7.3.1.6.3	Test Requirements	3411

A.7.3.1.7	HO with PSCell from FR1 NR-SA to EN-DC with known E-UTRA PCell and known FR2 PSCell	3411

A.7.3.1.7.1	Test purpose and environment	3411

A.7.3.1.7.2	Test Requirements	3415

A.7.3.1.8	NR PSCell change delay in HO with PSCell from NR-DC to NR-DC	3416

A.7.3.1.8.1	Test Purpose and Environment	3416

A.7.3.1.8.2	Test Requirements	3419

A.7.3.1.9	Intra-frequency handover from FR2-2 to FR2-2; unknown target cell	3419

A.7.3.1.9.1	Test Purpose and Environment	3419

A.7.3.1.9.2	Test Parameters	3419

A.7.3.1.9.3	Test Requirements	3421

A.7.3.1.10	Inter-frequency handover from FR2-2 to FR2-2; unknown target cell	3422

A.7.3.1.10.1	Test Purpose and Environment	3422

A.7.3.1.10.2	Test Parameters	3422

A.7.3.1.10.3	Test Requirements	3424

A.7.3.1.11	Inter-frequency handover from FR1 to FR2-2; unknown target cell	3424

A.7.3.1.11.1	Test Purpose and Environment	3424

A.7.3.1.11.2	Test Parameters	3424

A.7.3.1.11.3	Test Requirements	3426

A.7.3.1.12	Intra-frequency handover from FR2 to FR2; known target cell configured with NCD-SSB	3427

A.7.3.1.12.1	Test Purpose and Environment	3427

A.7.3.1.12.2	Test Parameters	3427

A.7.3.1.12.3	Test Requirements	3428

A.7.3.1.13	Inter-frequency handover from FR2 to FR2; known target cell configured with NCD-SSB	3429

A.7.3.1.13.1	Test Purpose and Environment	3429

A.7.3.1.13.2	Test Parameters	3429

A.7.3.1.13.3	Test Requirements	3431

A.7.3.1.14	Handover with PSCell from FR1-FR2 NR-DC to FR1-FR1 NR-DC with target PSCell in FR1	3431

A.7.3.1.14.1	Test Purpose and Environment	3431

A.7.3.1.14.2	Test Requirements	3435

A.7.3.1.15	HO with PSCell from FR1-FR1 NR-DC to FR1-FR2 NR-DC	3435

A.7.3.1.15.1	Test Purpose and Environment	3435

A.7.3.1.15.2	Test Requirements	3440

A.7.3.2	RRC Connection Mobility Control	3440

A.7.3.2.1	SA: RRC Re-establishment	3440

A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	3440

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	3442

A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	3445

A.7.3.2.1.3.1	Test Purpose and Environment	3445

A.7.3.2.1.3.2	Test Requirements	3446

A.7.3.2.1.4	Intra-frequency RRC Re-establishment in FR2-2	3447

A.7.3.2.1.4.1	Test Purpose and Environment	3447

A.7.3.2.1.4.2	Test Requirements	3449

A.7.3.2.1.5	Inter-frequency RRC Re-establishment in FR2-2	3449

A.7.3.2.1.5.1	Test Purpose and Environment	3449

A.7.3.2.1.5.2	Test Requirements	3451

A.7.3.2.1.6	Intra-frequency RRC Re-establishment in FR2-2 without serving cell timing	3452

A.7.3.2.1.6.1	Test Purpose and Environment	3452

A.7.3.2.1.6.2	Test Requirements	3453

A.7.3.2.2	Random Access	3454

A.7.3.2.2.1	4-step RA type c ontention based random access test in FR2 for NR Standalone	3454

A.7.3.2.2.2	4-step RA type n on-contention based random access test in FR2 for NR Standalone	3457

A.7.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	3460

A.7.3.2.2.4	2-step RA type n on-contention based random access test in FR2 for NR Standalone	3463

A.7.3.2.3	SA: RRC Connection Release with Redirection	3466

A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2	3466

A.7.3.2.4	LTM PDCCH-order Random Access	3468

A.7.3.2.4.1	PDCCH-order RACH on neighbor cell in FR2 when RACH BW is within active BWP	3468

A.7.3.2.4.2	PDCCH-order RACH on inter-frequency neighbor cell in FR2	3471

A.7.3.3	Conditional Handover	3474

A.7.3.3.1	Intra-frequency conditional handover from FR2 to FR2	3474

A.7.3.3.1.1	Test Purpose and Environment	3474

A.7.3.3.1.2	Test Parameters	3474

A.7.3.3.1.2.3	Test Requirements	3475

A.7.3.3.2	Inter-frequency conditional handover from FR2 to FR2; unknown target cell	3476

A.7.3.3.2.1	Test Purpose and Environment	3476

A.7.3.3.2.2	Test Parameters	3476

A.7.3.3.2.3Test Requirements	3477

A.7.3.3.3	NES triggering intra-frequency target CHO delay From FR2 to FR2	3477

A.7.3.3.3.1	Test Purpose and Environment	3477

A.7.3.3.3.2	Test Parameters	3478

A.7.3.3.3.2.3	Test Requirements	3479

A.7.3.3.4	NES triggering inter-frequency conditional handover from FR2 to FR1	3479

A.7.3.3.4.1	Test Purpose and Environment	3479

A.7.3.3.4.2	Test Parameters	3479

A.7.3.3.4.3	Test Requirements	3481

A.7.3.3.5	NR conditional handover including target MCG and target SCG from FR1-FR2 NR-DC to FR1-FR2 NR-DC	3482

A.7.3.3.5.1	Test Purpose and Environment	3482

A.7.3.3.5.2	Test Requirements	3485

A.7.3.3.5.2.1	Test Requirements for NR conditional handover	3485

A.7.3.3.5.2.2	Test Requirements for NR PSCell change	3485

A.7.3.3.6	NR conditional Handover including target MCG and candidate SCG from FR1-FR2 NR-DC to FR1-FR2 NR-DC	3485

A.7.3.3.6.1	Test Purpose and Environment	3485

A.7.3.3.6.2	Test Parameters	3485

A.7.3.3.6.3 Test Requirements	3489

A.7.3.4	LTM PCell Switch	3489

A.7.3.4.1	RACH based Intra-frequency PCell switch from FR2 to FR2	3489

A.7.3.4.1.1	Test Purpose and Environment	3489

A.7.3.4.1.2	Test Parameters	3489

A.7.3.4.1.3	Test Requirements	3492

A.7.3.4.2	RACH-less Intra-frequency PCell switch from FR2 to FR2	3493

A.7.3.4.2.1	Test Purpose and Environment	3493

A.7.3.4.2.2	Test Parameters	3493

A.7.3.4.2.3	Test Requirements	3497

A.7.3.4.3	RACH-based Inter-frequency LTM PCell switch from FR2 to FR2	3497

A.7.3.4.3.1	Test Purpose and Environment	3497

A.7.3.4.3.2	Test Parameters	3497

A.7.3.4.3.3	Test Requirements	3500

A.7.3.5	LTM PSCell Switch	3501

A.7.3.5.1	RACH-based Intra-frequency LTM PSCell switch from FR2 to FR2	3501

A.7.3.5.1.1	Test Purpose and Environment	3501

A.7.3.5.1.2	Test Parameters	3501

A.7.3.5.1.3	Test Requirements	3505

A.7.4	Timing	3506

A.7.4.1	UE transmit timing	3506

A.7.4.1.1	NR UE Transmit Timing Test for FR2	3506

A.7.4.1.1.1	Test Purpose and environment	3506

A.7.4.1.1.2	Test requirements	3508

A.7.4.1.2	NR UE Transmit Timing Test for FR2-2	3508

A.7.4.1.2.1	Test Purpose and environment	3509

A.7.4.1.2.2	Test requirements	3511

A.7.4.1.3	NR UE Transmit Timing Test with 2-TA for FR2 UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18	3511

A.7.4.1.3.1	Test Purpose and environment	3511

A.7.4.1.3.2	Test requirements	3515

A.7.4.2	UE timer accuracy	3515

A.7.4.3	Timing advance	3515

A.7.4.3.1	SA FR2 timing advance adjustment accuracy	3515

A.7.4.3.1.1	Test Purpose and Environment	3515

A.7.4.3.1.2	Test Parameters	3516

A.7.4.3.1.3 Test Requirements	3518

A.7.4.3.2	SA FR2-2 timing advance adjustment accuracy	3518

A.7.4.3.2.1	Test Purpose and Environment	3518

A.7.4.3.2.2	Test Parameters	3518

A.7.4.3.2.3	Test Requirements	3520

A.7.5	Signaling characteristics	3523

A.7.5.1	Radio link Monitoring	3523

A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3523

A.7.5.1.1.1	Test Purpose and Environment	3523

A.7.5.1.1.2	Test Requirements	3526

A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3526

A.7.5.1.2.1	Test Purpose and Environment	3526

A.7.5.1.2.2	Test Requirements	3529

A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3529

A.7.5.1.3.1	Test Purpose and Environment	3529

A.7.5.1.3.2	Test Requirements	3531

A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3532

A.7.5.1.4.1	Test Purpose and Environment	3532

A.7.5.1.4.2	Test Requirements	3534

A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	3534

A.7.5.1.5.1	Test Purpose and Environment	3534

A.7.5.1.5.2	Test Requirements	3537

A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	3537

A.7.5.1.6.1	Test Purpose and Environment	3537

A.7.5.1.6.2	Test Requirements	3539

A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	3540

A.7.5.1.7.1	Test Purpose and Environment	3540

A.7.5.1.7.2	Test Requirements	3542

A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	3542

A.7.5.1.8.1	Test Purpose and Environment	3542

A.7.5.1.8.2	Test Requirements	3545

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	3546

A.7.5.1.9.1	Test Purpose and Environment	3546

A.7.5.1.9.2	Test Requirements	3547

A.7.5.1.10	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode for UE supporting fast beam sweeping in multi-Rx	3547

A.7.5.1.10.1	Test Purpose and Environment	3547

A.7.5.1.10.2	Test Requirements	3550

A.7.5.1.11	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode when CD-SSB is outside active BWP	3551

A.7.5.1.11.1	Test Purpose and Environment	3551

A.7.5.1.12	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode when CD-SSB is outside active BWP	3551

A.7.5.1.12.1	Test Purpose and Environment	3551

A.7.5.1.12.2	Test Requirements	3551

A.7.5.1.13	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode for UE supporting NCD-SSB based measurement outside active BWP	3551

A.7.5.1.13.1	Test Purpose and Environment	3551

A.7.5.1.13.2	Test Requirements	3554

A.7.5.2	Interruption	3554

A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2	3554

A.7.5.2.1.1	Test Purpose and Environment	3554

A.7.5.2.1.2	Test Requirements	3557

A.7.5.2.2	SA interruptions at NR SRS carrier-based switching	3557

A.7.5.2.2.1	Test Purpose and Environment	3557

A.7.5.2.2.2	Test Parameters	3557

A.7.5.2.2.3	Test Requirements	3559

A.7.5.3	SCell Activation and Deactivation Delay	3559

A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX	3559

A.7.5.3.1.1	Test Purpose and Environment	3559

A.7.5.3.1.2	Test Requirements	3561

A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2	3561

A.7.5.3.2.1	Test Purpose and Environment	3561

A.7.5.3.2.2	Test Requirements	3563

A.7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	3564

A.7.5.3.3.1	Test Purpose and Environment	3564

A.7.5.3.3.2	Test Requirements	3566

A.7.5.3.4	Direct SCell activation at SCell addition of known SCell in FR2	3567

A.7.5.3.4.1	Test Purpose and Environment	3567

A.7.5.3.4.2	Test Requirements	3569

A.7.5.3.5	Direct SCell activation at handover with known SCell in FR2	3570

A.7.5.3.5.1	Test Purpose and Environment	3570

A.7.5.3.5.2	Test Requirements	3572

A.7.5.3.6	PUCCH SCell activation and deactivation for FR1+FR2 inter-band with target SCell in FR2 and known	3573

A.7.5.3.6.1	Test Purpose and Environment	3573

A.7.5.3.6.2	Test Requirements	3575

A.7.5.3.7	PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR1 PCell	3576

A.7.5.3.7.1	Test Purpose and Environment	3576

A.7.5.3.7.2	Test Requirements	3579

A.7.5.3.8	SCell Activation and deactivation for known PUCCH SCell in FR2 inter-band in non-DRX	3580

A.7.5.3.8.1	Test Purpose and Environment	3580

A.7.5.3.8.2	Test Requirements	3583

A.7.5.3.9	PUCCH SCell Activation and deactivation of unknown SCell in FR2	3584

A.7.5.3.9.1	Test Purpose and Environment	3584

A.7.5.3.9.2	Test Requirements	3586

A.7.5.3.10	SCell Activation and deactivation of FR2 known PUCCH SCell and one FR2 unknown SCell with FR2 PCell	3587

A.7.5.3.10.1	Test Purpose and Environment	3587

A.7.5.3.10.2	Test Requirements	3590

A.7.5.3.11	PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR2 PCell	3591

A.7.5.3.11.1	PUCCH SCell activation with non-PUCCH SCell in a secondary PUCCH Group	3591

A.7.5.3.11.1.1	Test Purpose and Environment	3591

A.7.5.3.11.1.2	Test Requirements	3594

A.7.5.3.11.2	PUCCH SCell activation with non-PUCCH SCell in a primary PUCCH Group	3595

A.7.5.3.11.2.1	Test Purpose and Environment	3595

A.7.5.3.11.2.2	Test Requirements	3598

A.7.5.3.12	Void	3599

A.7.5.3.13	SCell Activation for SCell in FR2 intra-band in non-DRX	3599

A.7.5.3.13.1	Test Purpose and Environment	3599

A.7.5.3.13.2	Test Requirements	3601

A.7.5.3.14	SCell Activation for known SCell in FR2 inter-band	3601

A.7.5.3.14.1	Test Purpose and Environment	3601

A.7.5.3.14.2	Test Requirements	3603

A.7.5.3.15	PUCCH SCell activation and deactivation with FR1 PCell based on L3 reporting after SCell activation command	3604

A.7.5.3.15.1	Test Purpose and Environment	3604

A.7.5.3.15.2	Test Requirements	3608

A.7.5.3.16	PUCCH SCell activation and deactivation with FR2 PCell based on L3 reporting after SCell activation command	3608

A.7.5.3.16.1	Test Purpose and Environment	3608

A.7.5.3.16.2	Test Requirements	3611

A.7.5.3.17	SCell Activation and deactivation for SCell in FR2 inter-band in DRX for UE capable of small beam sweeping factors and/or short measurement interval	3612

A.7.5.3.17.1	Test Purpose and Environment	3612

A.7.5.3.17.2	Test Requirements	3614

A.7.5.3.18	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2, in DRX, for UE capable of small beam sweeping factors and/or short measurement interval	3616

A.7.5.3.18.1	Test Purpose and Environment	3616

A.7.5.3.18.2	Test Requirements	3619

A.7.5.3.19	SCell Activation and deactivation of FR2 unknown SCell with FR1 PCell in non-DRX with L3 reporting during activation	3621

A.7.5.3.19.1	Test Purpose and Environment	3621

A.7.5.3.19.2	Test Requirements	3624

A.7.5.3.20	SCell Activation and Deactivation of FR2 unkown SCell with FR2 PCell in non-DRX with L3 reporting during activation	3624

A.7.5.3.20.1	Test Purpose and Environment	3625

A.7.5.3.20.2	Test Requirements	3627

A.7.5.4	Void	3628

A.7.5.5	Beam Failure Detection and Link recovery procedures	3628

A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3628

A.7.5.5.1.1	Test Purpose and Environment	3628

A.7.5.5.1.2	Test Requirements	3631

A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	3632

A.7.5.5.2.1	Test Purpose and Environment	3632

A.7.5.5.2.2	Test Requirements	3635

A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	3635

A.7.5.5.3.1	Test Purpose and Environment	3635

A.7.5.5.3.2	Test Requirements	3638

A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	3638

A.7.5.5.4.1	Test Purpose and Environment	3638

A.7.5.5.4.2	Test Requirements	3641

A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3642

A.7.5.5.5.1	Test Purpose and Environment	3642

A.7.5.5.5.2	Test Requirements	3645

A.7.5.5.6	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	3645

A.7.5.5.6.1	Test Purpose and Environment	3645

A.7.5.5.6.2	Test Requirements	3648

A.7.5.5.7	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	3648

A.7.5.5.7.1	Test Purpose and Environment	3648

A.7.5.5.7.2	Test Requirements	3651

A.7.5.5.8	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode for UE fulfilling relaxed measurement criterion	3652

A.7.5.5.8.1	Test Purpose and Environment	3652

A.7.5.5.8.2	Test Requirements	3655

A.7.5.5.9	TRP specific Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	3655

A.7.5.5.9.1	Test Purpose and Environment	3655

A.7.5.5.9.2	Test Requirements	3658

A.7.5.5.10	TRP specific Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3659

A.7.5.5.10.1	Test Purpose and Environment	3659

A.7.5.5.10.2	Test Requirements	3662

A.7.5.5.11	Beam Failure Detection and Link Recovery Test for FR2-2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	3662

A.7.5.5.11.1	Test Purpose and Environment	3662

A.7.5.5.11.2	Test Requirements	3665

A.7.5.5.12	Beam Failure Detection and Link Recovery Test for FR2-2 PCell configured with CSI-RS-based BFD and LR in DRX mode	3665

A.7.5.5.12.1	Test Purpose and Environment	3665

A.7.5.5.12.2	Test Requirements	3668

A.7.5.5.13	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2-2 PCell configured with SSB-based BFD and LR in non-DRX mode	3669

A.7.5.5.13.1	Test Purpose and Environment	3669

A.7.5.5.13.2	Test Requirements	3671

A.7.5.5.14	TRP specific Beam Failure Detection and Link Recovery for FR2 PCell configured with CSI-RS-based BFD and LR and multi-Rx operation in DRX mode	3671

A.7.5.5.14.1	Test Purpose and Environment	3671

A.7.5.5.14.2	Test Requirements	3675

A.7.5.6	Active BWP switch	3675

A.7.5.6.1	DCI-based and Timer-based Active BWP Switch	3675

A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of SCell with non-DRX in SA	3675

A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of SCell with non-DRX in SA	3679

A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA	3683

A.7.5.6.1.3.1	Test Purpose and Environment	3683

A.7.5.6.1.3.2	Test Requirements	3685

A.7.5.6.1.4	NR FR2-2- NR FR2-2 DL active BWP switch of SCell with non-DRX in SA	3685

A.7.5.6.1.4.1	Test Purpose and Environment	3685

A.7.5.6.1.4.2	Test Requirements	3688

A.7.5.6.2	RRC-based Active BWP Switch	3689

A.7.5.6.2.1.1	Test Purpose and Environment	3689

A.7.5.6.2.1.2	Test Requirements	3692

A.7.5.6.2.2	NR FR2-2 DL active BWP switch of PCell with non-DRX in SA	3692

A.7.5.6.2.2.1	Test Purpose and Environment	3692

A.7.5.6.2.2.2	Test Requirements	3694

A.7.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	3695

A.7.5.6.3.1.1	Test Purpose and Environment	3695

A.7.5.6.3.1.2	Test Requirements	3697

A.7.5.6.4	SCell dormancy switch	3698

A.7.5.6.4.1	NR FR2 PCell SCell dormancy switch of single FR2 SCell inside active time	3698

A.7.5.6.4.1.1	Test Purpose and Environment	3698

A.7.5.6.4.1.2	Test Requirements	3700

A.7.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time	3701

A.7.5.6.4.2.1	 Test Purpose and Environment	3701

A.7.5.6.4.2.2	 Test Requirements	3704

A.7.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	3704

A.7.5.6.5.1	Active BWP switch on multiple SCells with non-DRX in SA	3704

A.7.5.6.5.2	NR FR2-2 Active BWP switch on multiple SCells with non-DRX in SA	3706

A.7.5.6.5.2.1	Test Purpose and Environment	3706

A.7.5.6.5.2.2	Test Requirements	3709

A.7.5.7	PSCell addition and release delay	3709

A.7.5.7.1	Addition and Release Delay of known NR PSCell	3709

A.7.5.7.1.1	Test Purpose and Environment	3709

A.7.5.7.1.2	Test Requirements	3712

A.7.5.7.2	Addition and Release Delay of unknown NR PSCell in	3712

A.7.5.7.2.1	Test Purpose and Environment	3712

A.7.5.7.2.2	Test Requirements	3714

A.7.5.7.3	Addition and Release Delay of known NR PSCell in FR2-2	3714

A.7.5.7.3.1	Test Purpose and Environment	3714

A.7.5.7.3.2	Test Requirements	3717

A.7.5.7.4	Addition and Release Delay of unknown NR PSCell in FR2-2	3717

A.7.5.7.4.1	Test Purpose and Environment	3717

A.7.5.7.4.2	Test Requirements	3719

A.7.5.8	Active TCI state switch delay	3720

A.7.5.8.1	MAC-CE based active TCI state switch	3720

A.7.5.8.2	RRC based active TCI state switch	3723

A.7.5.8.3	MAC-CE based active TCI state switch for HST FR2 scenario	3726

A.7.5.8.3.1	NR PCell FR2 HST active TCI state switch for a known TCI state	3726

A.7.5.8.3.1.1	Test Purpose and Environment	3726

A.7.5.8.3.1.2	Test Requirements	3729

A.7.5.8.3.2	NR PCell FR2 HST active TCI state switch for PC6 UE supporting tci-StateSwitchInd-r18 for a known TCI state	3730

A.7.5.8.3.2.1	Test Purpose and Environment	3730

A.7.5.8.3.2.2	Test Requirements	3733

A.7.5.8.4	DCI based active TCI state switch with m-DCI for simultaneous reception	3733

A.7.5.8.4.1	Test Purpose and Environment	3733

A.7.5.8.4.2	Test Requirements	3736

A.7.5.8.5	Single-DCI FR2 DCI based active TCI state switch with known target TCI states for simultaneous reception	3736

A.7.5.8.5.1	Test Purpose and Environment	3736

A.7.5.8.5.1.2	Test Requirements	3738

A.7.5.9	Uplink spatial relation switch delay	3739

A.7.5.9.1.1.1	Test Purpose and Environment	3739

A.7.5.9.1.1.2	Test Requirements	3741

A.7.5.9.2	RRC based spatial relation switch	3741

A.7.5.9.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	3741

A.7.5.9.2.1.1	Test Purpose and Environment	3741

A.7.5.9.2.1.2	Test Requirements	3743

A.7.5.10	UE specific CBW change	3743

A.7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	3743

A.7.5.10.1.1	Test Purpose and Environment	3743

A.7.5.10.1.2	Test Requirements	3745

A.7.5.11	UE UL carrier RRC reconfiguration Delay	3746

A.7.5.11.1	UE UL carrier RRC reconfiguration Delay	3746

A.7.5.11.1.1	Test Purpose and Environment	3746

A.7.5.11.1.2	Test Requirements	3748

A.7.5.12	Conditional PSCell addition and release delay (FR2 SA)	3748

A.7.5.12.1	Addition and Release Delay of PSCell	3748

A.7.5.12.1.1	Test purpose and environment	3748

A.7.5.12.1.2	Test Parameters	3748

A.7.5.12.1.3	Test Requirements	3750

A.7.5.13	Unified TCI state switching delay	3750

A.7.5.13.1	MAC-CE based active joint TCI state switching	3750

A.7.5.13.1.1	NR PCell FR2 active joint TCI state switch for a known TCI state	3750

A.7.5.13.1.1.1	Test Purpose and Environment	3750

A.7.5.13.1.1.2	Test parameters	3751

A.7.5.13.1.1.3	Test Requirements	3752

A.7.5.13.2	 MAC-CE based active uplink TCI state switch	3753

A.7.5.13.2.1	 NR FR2 PCell uplink TCI state switch for a known TCI state	3753

A.7.5.13.2.1.1	Test Purpose and Environment	3753

A.7.5.13.2.1.2	Test parameters	3753

A.7.5.13.2.1.3	Test Requirements	3755

A.7.5.13.3	MAC-CE based active downlink TCI state switch	3755

A.7.5.13.3.1	NR PCell FR2 active downlink TCI state switch to cell with additional PCI for a known TCI state	3755

A.7.5.13.3.1.1	Test Purpose and Environment	3755

A.7.5.13.3.1.2	Test Parameters	3755

A.7.5.13.3.1.3	Test Requirements	3758

A.7.5.13.4	sDCI MAC-CE based joint TCI state switching	3759

A.7.5.13.4.1	NR PCell FR2 dual downlink and uplink TCI state switch in sDCI for known case	3759

A.7.5.13.4.1.1	Test Purpose and Environment	3759

A.7.5.13.4.1.2	Test parameters	3759

A.7.5.13.5	MAC-CE based dual downlink TCI state switching delay for unified TCI for single-DCI mTRP	3763

A.7.5.13.5.1	NR PCell FR2 dual downlink TCI state switch in sDCI for known case	3763

A.7.5.13.5.1.1	Test Purpose and Environment	3763

A.7.5.13.5.1.2	Test Parameters	3764

A.7.5.13.5.1.3	Test Requirements	3766

A.7.5.13.6	 MAC-CE based active uplink TCI state switch for single-DCI mTRP	3766

A.7.5.13.6.1	 NR FR2 PCell uplink TCI state switch for two known TCI states	3766

A.7.5.13.6.1.1	Test Purpose and Environment	3766

A.7.5.13.6.1.2	Test parameters	3767

A.7.5.13.6.1.3	Test Requirements	3768

A.7.5.14	PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2	3769

A.7.5.14.1	Test Purpose and Environment	3769

A.7.5.14.2	Test Requirements	3771

A.7.5.15	Void	3772

A.7.5.16	UE L1-RSRP Scheduling and Measurement Restrictions on FR2-1	3772

A.7.5.16.1	Test Purpose and Environment	3772

A.7.5.16.2	Test Requirements	3774

A.7.5.17	SCG Activation and deactivation for FR1+FR1 inter-band with target PSCell in FR1	3775

A.7.5.17.1	Test Purpose and Environment	3775

A.7.5.17.2	Test Requirements	3777

A.7.5.18	Subsequent conditional PSCell addition/change	3778

A.7.5.18.1	Intra-frequency subsequent CPC from FR1-FR2 NR-DC to FR1-FR2 NR-DC	3778

A.7.5.18.1.1	Test purpose and environment	3778

A.7.5.18.1.2	Test Requirements	3781

A.7.5.18.2	Inter-frequency subsequent CPA from FR1-FR2 NR-DC to FR1-FR2 NR-DC	3781

A.7.5.18.2.1	Test Purpose and Environment	3781

A.7.5.18.2.2	Test Requirements	3784

A.7.6	Measurement procedure	3786

A.7.6.1	Intra-frequency Measurements	3786

A.7.6.1.1	SA event triggered reporting test without gap under non-DRX	3786

A.7.6.1.1.1	Test purpose and Environment	3786

A.7.6.1.1.2	Test Requirements	3788

A.7.6.1.2	SA event triggered reporting test without gap under DRX	3788

A.7.6.1.2.1	Test purpose and Environment	3788

A.7.6.1.2.2	Test Requirements	3790

A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	3790

A.7.6.1.3.1	Test purpose and Environment	3790

A.7.6.1.3.2	Test Requirements	3793

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	3793

A.7.6.1.4.1	Test purpose and Environment	3793

A.7.6.1.4.2	Test Requirements	3795

A.7.6.1.5	SA event triggered reporting test without gap under non-DRX for UE configured with highSpeedMeasFlagFR2-r17	3796

A.7.6.1.5.1	Test purpose and Environment	3796

A.7.6.1.5.2	Test Requirements	3798

A.7.6.1.6	SA event triggered reporting test without gap under non-DRX for FR2-2	3798

A.7.6.1.6.1	Test purpose and Environment	3798

A.7.6.1.6.2	Test Requirements	3800

A.7.6.1.7	SA event triggered reporting test without gap under DRX for FR2-2	3801

A.7.6.1.7.1	Test purpose and Environment	3801

A.7.6.1.7.2	Test Requirements	3803

A.7.6.1.8	SA event triggered reporting test with per-UE gaps under non-DRX for FR2-2	3804

A.7.6.1.8.1	Test purpose and Environment	3804

A.7.6.1.8.2	Test Requirements	3806

A.7.6.1.9	SA event triggered reporting test with per-UE gaps under DRX for FR2-2	3807

A.7.6.1.9.1	Test purpose and Environment	3807

A.7.6.1.9.2	Test Requirements	3809

A.7.6.1.10	SA event triggered reporting test with SSB time index detection without gap under non-DRX for FR2-2	3810

A.7.6.1.10.1	Test purpose and Environment	3810

A.7.6.1.10.2	Test Requirements	3812

A.7.6.1.11	SA event triggered reporting test with SSB time index detection with per-UE gaps under non-DRX for FR2-2	3812

A.7.6.1.11.1	Test purpose and Environment	3812

A.7.6.1.11.2	Test Requirements	3814

A.7.6.1.12	SA event triggered reporting test without gap under non-DRX when CD-SSB is outside active BWP	3815

A.7.6.1.12.1	Test purpose and Environment	3815

A.7.6.1.12.2	Test Requirements	3815

A.7.6.1.13	SA event triggered reporting test without gap under non-DRX with NCD-SSB	3815

A.7.6.1.13.1	Test purpose and Environment	3815

A.7.6.1.13.2	Test Requirements	3817

A.7.6.1.14	SA event triggered reporting test without gap under non-DRX for power class 6 UE supporting measEnhCAInterFreqFR2-r18	3818

A.7.6.1.14.1	Test Purpose and Environment	3818

A.7.6.1.14.2	Test Requirements	3819

A.7.6.2	Inter-frequency Measurements	3819

A.7.6.2.1	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR2)	3819

A.7.6.2.1.1	Test Purpose and Environment	3819

A.7.6.2.1.2	Test Requirements	3821

A.7.6.2.2	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR2)	3822

A.7.6.2.2.1	Test Purpose and Environment	3822

A.7.6.2.2.2	Test Requirements	3824

A.7.6.2.3	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR2)	3824

A.7.6.2.3.1	Test Purpose and Environment	3824

A.7.6.2.3.2	Test Requirements	3826

A.7.6.2.4	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR2)	3827

A.7.6.2.4.1	Test Purpose and Environment	3827

A.7.6.2.4.2	Test Requirements	3829

A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	3829

A.7.6.2.5.1	Test Purpose and Environment	3829

A.7.6.2.5.2	Test Requirements	3832

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1)	3832

A.7.6.2.6.1	Test Purpose and Environment	3832

A.7.6.2.6.2	Test Requirements	3835

A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	3835

A.7.6.2.7.1	Test Purpose and Environment	3835

A.7.6.2.7.2	Test Requirements	3838

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	3838

A.7.6.2.8.1	Test Purpose and Environment	3838

A.7.6.2.8.2	Test Requirements	3841

A.7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	3841

A.7.6.2.9.1	Test Purpose and Environment	3841

A.7.6.2.9.2	Test Requirements	3843

A.7.6.2.10	SA event triggered reporting test without gap under non-DRX	3844

A.7.6.2.10.1	Test Purpose and Environment	3844

A.7.6.2.10.2	Test Requirements	3845

A.7.6.2.11	SA event triggered reporting test without gap under DRX	3845

A.7.6.2.11.1	Test Purpose and Environment	3846

A.7.6.2.11.2	Test Requirements	3847

A.7.6.2.12	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is not used (PCell in FR2-2)	3848

A.7.6.2.12.1	Test Purpose and Environment	3848

A.7.6.2.12.2	Test Requirements	3850

A.7.6.2.13	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is used (PCell in FR2-2)	3850

A.7.6.2.13.1	Test Purpose and Environment	3850

A.7.6.2.13.2	Test Requirements	3853

A.7.6.2.14	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is not used (PCell in FR2-2)	3854

A.7.6.2.14.1	Test Purpose and Environment	3854

A.7.6.2.14.2	Test Requirements	3856

A.7.6.2.15	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is used (PCell in FR2-2)	3857

A.7.6.2.15.1	Test Purpose and Environment	3857

A.7.6.2.15.2	Test Requirements	3859

A.7.6.2.16	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is not used (PCell in FR1)	3860

A.7.6.2.16.1	Test Purpose and Environment	3860

A.7.6.2.16.2	Test Requirements	3864

A.7.6.2.17	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is used (PCell in FR1)	3864

A.7.6.2.17.1	Test Purpose and Environment	3864

A.7.6.2.17.2	Test Requirements	3868

A.7.6.2.18	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is not used (PCell in FR1)	3869

A.7.6.2.18.1	Test Purpose and Environment	3869

A.7.6.2.18.2	Test Requirements	3872

A.7.6.2.19	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is used (PCell in FR1)	3873

A.7.6.2.19.1	Test Purpose and Environment	3873

A.7.6.2.19.2	Test Requirements	3877

A.7.6.2.20	SA event triggered reporting tests for FR2 with measurement gap with priority and two periodic MUSIM gaps configured	3878

A.7.6.2.20.1	Test Purpose and Environment	3878

A.7.6.2.20.2	Test Requirements	3880

A.7.6.2.21	SA event triggered reporting tests for FR2 with measurement gap without priority and periodic MUSIM gap configured	3880

A.7.6.2.21.1	Test Purpose and Environment	3880

A.7.6.2.21.2	Test Requirements	3882

A.7.6.2.22	SA event triggered reporting tests with SSB time index detection when DRX is not used (PCell in FR2) for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17	3883

A.7.6.2.22.1	Test Purpose and Environment	3883

A.7.6.2.22.2	Test Requirements	3885

A.7.6.2.23	SA event triggered reporting tests without SSB time index detection when DRX is not used (PCell in FR2) for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17	3885

A.7.6.2.23.1	Test Purpose and Environment	3885

A.7.6.2.23.2	Test Requirements	3887

A.7.6.3	L1-RSRP measurement for beam reporting	3887

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	3887

A.7.6.3.1.1	Test Purpose and Environment	3887

A.7.6.3.1.2	Test parameters	3887

A.7.6.3.1.3	Test Requirements	3889

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used	3889

A.7.6.3.2.1	Test Purpose and Environment	3889

A.7.6.3.2.2	Test parameters	3889

A.7.6.3.2.3	Test Requirements	3890

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	3891

A.7.6.3.3.1	Test Purpose and Environment	3891

A.7.6.3.3.2	Test parameters	3891

A.7.6.3.3.3	Test Requirements	3892

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	3892

A.7.6.3.4.1	Test Purpose and Environment	3892

A.7.6.3.4.2	Test parameters	3893

A.7.6.3.3.3	Test Requirements	3894

A.7.6.3.5	SSB based L1-RSRP measurement when DRX is used for power class 6 UE configured with highSpeedMeasFlagFR2-r17	3894

A.7.6.3.5.1	Test Purpose and Environment	3894

A.7.6.3.5.2	Test parameters	3895

A.7.6.3.5.3	Test Requirements	3896

A.7.6.3.6	Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used	3896

A.7.6.3.6.1	Test Purpose and Environment	3896

A.7.6.3.6.2	Test parameters	3897

A.7.6.3.6.3	Test Requirements	3899

A.7.6.3.7	SSB based L1-RSRP measurement for FR2-2 when DRX is used	3899

A.7.6.3.7.1	Test Purpose and Environment	3899

A.7.6.3.7.2	Test parameters	3900

A.7.6.3.7.3	Test Requirements	3901

A.7.6.3.8	CSI-RS based L1-RSRP measurement when DRX is not used and when CD-SSB is outside active BWP	3901

A.7.6.3.8.1	Test Purpose and Environment	3901

A.7.6.3.9	SSB based L1-RSRP measurement when DRX is not used when CD-SSB is outside active BWP	3902

A.7.6.3.9.1	Test Purpose and Environment	3902

A.7.6.3.9.2	Test Requirements	3902

A.7.6.3.10	SSB based L1-RSRP measurement for UE supporting NCD-SSB based L1 measurement outside active BWP when DRX is not used	3902

A.7.6.3.10.1	Test Purpose and Environment	3902

A.7.6.3.10.2	Test parameters	3902

A.7.6.3.10.3	Test Requirements	3904

A.7.6.3.11	SSB based L1-RSRP measurement when DRX is used for power class 6 UE supporting simultaneousReceptionTwoQCL-r18	3904

A.7.6.3.11.1	Test Purpose and Environment	3904

A.7.6.3.11.2	Test parameters	3904

A.7.6.3.11.3	Test Requirements	3906

A.7.6.4	CLI measurements	3906

A.7.6.4.1	SRS-RSRP measurement with non-DRX	3906

A.7.6.4.1.1	Test Purpose and Environment	3906

A.7.6.4.1.2	Test Parameters	3906

A.7.6.4.1.3	Test Requirements	3908

A.7.6.4.2	CLI-RSSI measurement with non-DRX	3908

A.7.6.4.2.1	Test Purpose and Environment	3908

A.7.6.4.2.2	Test Parameters	3909

A.7.6.4.2.3	Test Requirements	3910

A.7.6.5.1	SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	3910

A.7.6.5.1.1	Test Purpose and Environment	3910

A.7.6.5.1.2	Test Requirements	3913

A.7.6.6	L1-SINR measurement for beam reporting	3913

A.7.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	3915

A.7.6.6.2.1	Test Purpose and Environment	3915

A.7.6.6.2.2	Test parameters	3915

A.7.6.6.2.3	Test Requirements	3916

A.7.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	3917

A.7.6.6.3.1	Test Purpose and Environment	3917

A.7.6.6.3.2	Test parameters	3917

A.7.6.6.3.3	Test Requirements	3918

A.7.6.7	CSI-RS based intra-frequency Measurements	3919

A.7.6.7.1	SA event triggered reporting test without gap under DRX for CSI-RS based intra-frequency measurement	3919

A.7.6.7.1.1	Test purpose and Environment	3919

A.7.6.7.1.2	Test Requirements	3920

A.7.6.8	CSI-RS based inter-frequency Measurements	3921

A.7.6.8.1	SA event triggered reporting tests for FR2 CSI-RS based measurement when non-DRX is used (PCell in FR2)	3921

A.7.6.8.1.1	Test Purpose and Environment	3921

A.7.6.8.1.2	Test Requirements	3923

A.7.6.9	RSTD measurements	3923

A.7.6.9.1	 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA	3923

A.7.6.9.1.1	Test Purpose and Environment	3923

A.7.6.9.1.2	Test Requirements	3927

A.7.6.9.2	 NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR2 SA	3927

A.7.6.9.2.1	Test Purpose and Environment	3927

A.7.6.9.2.2	Test Requirements	3930

A.7.6.9.3	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA	3931

A.7.6.9.3.1	Test Purpose and Environment	3931

A.7.6.9.3.2	Test Requirements	3933

A.7.6.9.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA without measurement gap	3934

A.7.6.9.4.1	Test Purpose and Environment	3934

A.7.6.9.4.2	Test Requirements	3936

A.7.6.9.5	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_CONNECTED state with Rx TEG	3937

A.7.6.9.5.1	Test Purpose and Environment	3937

A.7.6.9.5.2	Test Requirements	3940

A.7.6.9.6	NR RSTD measurement reporting delay test case for PRS aggregation in FR2 SA in RRC_CONNECTED mode	3940

A.7.6.9.6.1	Test Purpose and Environment	3940

A.7.6.9.6.2	Test Requirements	3948

A.7.6.10 PRS-RSRP measurements	3948

A.7.6.10.1 PRS-RSRP reporting delay test case for single positioning frequency layer	3948

A.7.6.10.1.1	Test Purpose and Environment	3948

A.7.6.10.1.2	Test Requirements	3950

A.7.6.10.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	3951

A.7.6.10.2.1	Test Purpose and Environment	3951

A.7.6.10.2.2	Test Requirements	3953

A.7.6.10.3	PRS-RSRP reporting delay test case for reduced number of samples	3953

A.7.6.10.3.1	Test Purpose and Environment	3953

A.7.6.10.3.2	Test Requirements	3955

A.7.6.10.4	PRS-RSRP reporting delay test case for single positioning frequency layer outside MG	3956

A.7.6.10.4.1	Test Purpose and Environment	3956

A.7.6.10.4.2	Test Requirements	3958

A.7.6.11	UE Rx-Tx time difference measurements	3958

A.7.6.11.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	3958

A.7.6.11.1.1	Test purpose and environment	3958

A.7.6.11.1.2	Test requirements	3960

A.7.6.11.2	UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA	3960

A.7.6.11.2.1	Test purpose and environment	3960

A.7.6.11.2.2	Test requirements	3962

A.7.6.11.3	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA with reduced sample number	3963

A.7.6.11.3.1	Test purpose and environment	3963

A.7.6.11.3.2	Test requirements	3964

A.7.6.11.4	UE Rx-Tx time difference measurements without gaps in FR2 SA	3965

A.7.6.11.4.1	Test purpose and environment	3965

A.7.6.11.4.2	Test requirements	3966

A.7.6.11.5	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA with RxTx TEG	3967

A.7.6.11.5.1	Test purpose and environment	3967

A.7.6.11.5.2	Test requirements	3968

A.7.6.11.6	UE Rx-Tx time difference measurements with PRS bandwidth aggregation in FR2 SA	3969

A.7.6.11.6.1	Test purpose and environment	3969

A.7.6.11.6.2	Test requirements	3972

A.7.6.12	PRS-RSRPP measurements	3972

A.7.6.12.1 PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_CONNECTED state	3972

A.7.6.12.1.1	Test Purpose and Environment	3972

A.7.6.12.1.2	Test Requirements	3975

A.7.6.12.2	PRS-RSRPP reporting delay test case for reduced number of samples for single positioning frequency layer in FR2 in RRC_CONNECTED state	3975

A.7.6.12.2.1	Test Purpose and Environment	3975

A.7.6.12.2.2	Test Requirements	3977

A.7.6.12.3	PRS-RSRPP reporting delay test case for gapless measurement in FR2	3977

A.7.6.12.3.1	Test Purpose and Environment	3977

A.7.6.12.3.2	Test Requirements	3980

A.7.6.13	UE Rx-Tx time difference measurements for PDC	3980

A.7.6.13.1	UE Rx-Tx time difference measurement for propagation delay compensation using PRS in FR2	3980

A.7.6.13.1.1	Test purpose and environment	3980

A.7.6.13.1.2	Test requirements	3981

A.7.6.13.2	UE Rx-Tx time difference measurement for propagation delay compensation using TRS in FR2	3982

A.7.6.13.2.1	Test purpose and environment	3982

A.7.6.13.2.2	Test requirements	3983

A.7.6.14	SA event triggered reporting tests with Pre-MG	3983

A.7.6.14.1	Intra-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MG in FR2	3983

A.7.6.14.1.1	Test purpose and Environment	3983

A.7.6.14.1.2	Test parameters	3984

A.7.6.14.1.3	Test Requirements	3985

A.7.6.14.2	Intra-frequency measurement test with SA event triggered reporting tests: with network-controlled activation/deactivation of Pre-MG in FR2	3986

A.7.6.14.2.1	Test purpose and Environment	3986

A.7.6.14.2.2	Test parameters	3986

A.7.6.14.2.3	Test Requirements	3988

A.7.6.15	SA event triggered reporting tests with concurrent gaps	3988

A.7.6.15.1	SA event triggered reporting tests For FR2 with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements	3988

A.7.6.15.1.1	Test Purpose and Environment	3988

A.7.6.15.1.2	Test Requirements	3990

A.7.6.15.2	SA event triggered reporting tests For FR2 with concurrent measurement gaps without SSB time index detection when DRX is not used (PCell in FR2)	3991

A.7.6.15.2.1	Test Purpose and Environment	3991

A.7.6.15.2.2	Test Requirements	3993

A.7.6.15.3	SA event triggered reporting tests for FR2 concurrent gap with partially partial overlapping scenario for SSB-based measurements and PRS-based measurement	3993

A.7.6.15.3.1	Test Purpose and Environment	3993

A.7.6.15.3.2	Test Requirements	3996

A.7.6.16	SA event triggered reporting tests with NCSG	3996

A.7.6.16.1	SA event triggered reporting test with per-UE NCSG under non-DRX	3996

A.7.6.16.1.1	Test purpose and Environment	3996

A.7.6.16.1.2	Test Requirements	3999

A.7.6.16.2	SA event triggered reporting tests on inter-frequency measurement with NCSG for FR2 when DRX is not used (PCell in FR2)	3999

A.7.6.16.2.1	Test Purpose and Environment	3999

A.7.6.16.2.2	Test Requirements	4001

A.7.6.16.3	Event triggered reporting test on deactivated SCell measurement via NCSG in FR2 in non-DRX	4002

A.7.6.16.3.1	Test Purpose and Environment	4002

A.7.6.16.3.2	Test Requirements	4004

A.7.6.17	SA event triggered reporting tests for concurrent measurement gaps with Pre-MG in FR2	4004

A.7.6.17.1	SA event triggered reporting test for FR2 with one pre-configured gap and one measurement gap	4004

A.7.6.17.1.1	Test Purpose and Environment	4004

A.7.6.17.1.2	Test Requirements	4006

A.7.6.17.2	Inter-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MGs in FR2	4007

A.7.6.17.2.1	Test purpose and Environment	4007

A.7.6.17.2.2	Test parameters	4007

A.7.6.17.2.3	Test Requirements	4009

A.7.6.18	SA event triggered reporting tests with concurrent gaps and NCSG	4010

A.7.6.18.1	SA event triggered reporting tests For FR2 with concurrent measurement gaps and NCSG without SSB time index detection when DRX is not used (PCell in FR2)	4010

A.7.6.18.1.1	Test Purpose and Environment	4010

A.7.6.18.1.2	Test Requirements	4012

A.7.6.19	SA event triggered reporting tests with NeedForGap in FR2	4013

A.7.6.19.1	SA event triggered reporting test for UE indicating NeedforInterruptionInfoNR under non-DRX and no interruption outside configured measurement gaps	4013

A.7.6.19.1.1	Test purpose and Environment	4013

A.7.6.19.1.2	Test Requirements	4015

A.7.6.19.2	SA event triggered reporting test without gap under non-DRX	4015

A.7.6.19.2.1	Test purpose and Environment	4015

A.7.6.19.2.2	Test Requirements	4017

A.7.6.19.3	SA event triggered reporting test without gap without interruption under non-DRX	4018

A.7.6.19.3.1	Test Purpose and Environment	4018

A.7.6.19.3.2	Test Requirements	4020

A.7.6.20	LTM Intra-frequency L1-RSRP measurement	4020

A.7.6.20.1	Intra-frequency SSB based L1-RSRP measurement in FR2	4020

A.7.6.20.1.1	Test Purpose and Environment	4020

A.7.6.20.1.2	Test parameters	4021

A.7.6.20.1.3	Test Requirements	4023

A.7.6.21	LTM Inter-frequency L1-RSRP measurement with measurement gap	4023

A.7.6.21.1	Inter-frequency SSB-based L1-RSRP measurement with measurement gap for LTM in FR2	4023

A.7.6.21.1.1	Test Purpose and Environment	4023

A.7.6.21.1.2	Test parameters	4023

A.7.6.21.1.3	Test Requirements	4025

A.7.6.22	LTM Inter-frequency L1-RSRP measurement without measurement gap	4025

A.7.6.22.1	Inter-frequency SSB based L1-RSRP measurement without measurement gap in FR2	4025

A.7.6.22.1.1	Test Purpose and Environment	4025

A.7.6.22.1.2	Test parameters	4026

A.7.6.22.1.3	Test Requirements	4028

A.7.6.23	Idle Mode CA/DC Measurements	4028

A.7.6.23.1	Test case for Idle mode fast CA/DC eEMR measurement for FR2 without valid reporting	4028

A.7.6.23.1.1	Test Purpose and Environment	4028

A.7.6.23.1.2	Test Requirements	4031

A.7.6.23.2	Test case for Idle mode fast CA/DC cell reselection measurement for FR2 without valid reporting	4031

A.7.6.23.2.1	Test Purpose and Environment	4032

A.7.6.23.2.2	Test Requirements	4035

A.7.6.23.3 Test case for Idle mode fast CA/DC cell reselection measurement for FR2 with valid reporting	4035

A.7.6.23.3.1	Test Purpose and Environment	4035

A.7.6.23.3.2	Test Requirements	4038

A.7.6.24	RSCPD measurements	4039

A.7.6.24.1	NR RSCPD with RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_CONNECTED state	4039

A.7.6.24.1.1	Test Purpose and Environment	4039

A.7.6.24.1.2	Test Requirements	4046

A.7.6.25	RSCP measurements	4046

A.7.6.25.1	DL RSCP with UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	4046

A.7.6.25.1.1	Test purpose and environment	4046

A.7.6.25.1.2	Test requirements	4050

A.7.7	Measurement Performance requirements	4050

A.7.7.1	SS-RSRP	4050

A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	4050

A.7.7.1.1.1	Test Purpose and Environment	4050

A.7.7.1.1.2	Test parameters	4050

A.7.7.1.1.3	Test Requirements	4052

A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	4053

A.7.7.1.2.1	Test Purpose and Environment	4053

A.7.7.1.2.2	Test parameters	4053

A.7.7.1.2.3	Test Requirements	4055

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	4056

A.7.7.1.3.1	Test Purpose and Environment	4056

A.7.7.1.3.2	Test parameters	4056

A.7.7.1.3.3	Test Requirements	4058

A.7.7.2	SS-RSRQ	4058

A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	4058

A.7.7.2.1.1	Test Purpose and Environment	4058

A.7.7.2.1.2	Test Parameters	4058

A.7.7.2.1.3	Test Requirements	4060

A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	4060

A.7.7.2.2.1	Test Purpose and Environment	4060

A.7.7.2.2.2	Test Parameters	4060

A.7.7.2.2.3	Test Requirements	4062

A.7.7.3	SS-SINR	4062

A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	4062

A.7.7.3.1.1	Test Purpose and Environment	4062

A.7.7.3.1.2	Test Parameters	4062

A.7.7.3.1.3	Test Requirements	4063

A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	4064

A.7.7.3.2.1	Test Purpose and Environment	4064

A.7.7.3.2.2	Test Parameters	4064

A.7.7.3.2.3	Test Requirements	4065

A.7.7.4	L1-RSRP measurement for beam reporting	4065

A.7.7.4.1	SSB based L1-RSRP measurement	4065

A.7.7.4.1.1	Test Purpose and Environment	4065

A.7.7.4.1.2	Test parameters	4066

A.7.7.4.1.3	Test Requirements	4067

A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	4067

A.7.7.4.2.1	Test Purpose and Environment	4067

A.7.7.4.2.2	Test parameters	4068

A.7.7.4.2.3	Test Requirements	4069

A.7.7.5	CLI measurements	4069

A.7.7.5.1	SA SRS-RSRP measurement accuracy with FR2 serving cell	4069

A.7.7.5.1.1	Test Purpose and Environment	4069

A.7.7.5.1.2	Test parameters	4070

A.7.7.5.1.3	Test Requirements	4071

A.7.7.5.2	SA CLI-RSSI measurement accuracy with FR2 serving cell	4072

A.7.7.5.2.1	Test Purpose and Environment	4072

A.7.7.5.2.2	Test parameters	4072

A.7.7.5.2.3	Test Requirements	4073

A.7.7.6	L1-SINR measurement for beam reporting	4074

A.7.7.6.1.1	Test Purpose and Environment	4074

A.7.7.6.1.2	Test parameters	4074

A.7.7.6.1.3	Test Requirements	4075

A.7.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	4076

A.7.7.6.2.1	Test Purpose and Environment	4076

A.7.7.6.2.2	Test parameters	4076

A.7.7.6.2.3	Test Requirements	4077

A.7.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	4078

A.7.7.6.3.1	Test Purpose and Environment	4078

A.7.7.6.3.2	Test parameters	4078

A.7.7.6.3.3	Test Requirements	4079

A.7.7.7	CSI-RSRP	4080

A.7.7.7.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	4080

A.7.7.7.1.1	Test Purpose and Environment	4080

A.7.7.7.1.2	Test parameters	4080

A.7.7.7.1.3	Test Requirements	4082

A.7.7.7.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	4083

A.7.7.7.2.1	Test Purpose and Environment	4083

A.7.7.7.2.2	Test parameters	4083

A.7.7.7.2.3	Test Requirements	4084

A.7.7.8	CSI-RSRQ	4085

A.7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	4085

A.7.7.8.1.1	Test Purpose and Environment	4085

A.7.7.8.1.2	Test Parameters	4085

A.7.7.8.1.3	Test Requirements	4087

A.7.7.8.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	4087

A.7.7.8.2.1	Test Purpose and Environment	4087

A.7.7.8.2.2	Test Parameters	4087

A.7.7.8.2.3	Test Requirements	4089

A.7.7.9	CSI-SINR	4089

A.7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	4089

A.7.7.9.1.1	Test Purpose and Environment	4089

A.7.7.9.1.2	Test Parameters	4089

A.7.7.9.1.3	Test Requirements	4091

A.7.7.9.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	4091

A.7.7.9.2.1	Test Purpose and Environment	4091

A.7.7.9.2.2	Test Parameters	4091

A.7.7.9.2.3	Test Requirements	4092

A.7.7.10	RSTD measurements	4093

A.7.7.10.1	RSTD measurement accuracy test case for single positioning frequency layer	4093

A.7.7.10.1.1	Test purpose and Environment	4093

A.7.7.10.1.2	Test Requirements	4094

A.7.7.10.2	RSTD measurement accuracy test case for dual positioning frequency layer	4094

A.7.7.10.2.1	Test purpose and Environment	4094

A.7.7.10.2.2	Test Requirements	4096

A.7.7.10.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR2 in RRC_CONNECTED state	4096

A.7.7.10.3.1	Test purpose and Environment	4096

A.7.7.10.3.2	Test Requirements	4098

A.7.7.10.4	RSTD measurement accuracy test case with Rx TEG	4098

A.7.7.10.4.1	Test purpose and Environment	4098

A.7.7.10.4.2	Test Requirements	4099

A.7.7.10.5	NR RSTD measurement accuracy test case for PRS aggregation in FR2 SA in RRC_CONNECTED mode	4099

A.7.7.10.5.1	Test purpose and Environment	4099

A.7.7.10.5.2	Test Requirements	4102

A.7.7.11	PRS-RSRP measurements	4102

A.7.7.11.1	SA measurement accuracy with PRS in FR2	4102

A.7.7.11.1.1	Test Purpose and Environment	4102

A.7.7.11.1.2	Test parameters	4102

A.7.7.11.1.3	Test Requirements	4104

A.7.7.11.2	SA measurement accuracy with PRS in FR2 with reduced sample number	4104

A.7.7.11.2.1	Test Purpose and Environment	4104

A.7.7.11.2.2	Test parameters	4104

A.7.7.11.2.3	Test Requirements	4106

A.7.7.12	UE Rx-Tx time difference measurements	4106

A.7.7.12.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA	4106

A.7.7.12.1.1	Test purpose and environment	4106

A.7.7.12.1.2	Test parameters	4107

A.7.7.12.1.3	Test requirements	4108

A.7.7.12.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA	4109

A.7.7.12.2.1	Test purpose and environment	4109

A.7.7.12.2.2	Test parameters	4109

A.7.7.12.2.3	Test requirements	4110

A.7.7.12.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG	4110

A.7.7.12.3.1	Test purpose and environment	4110

A.7.7.12.3.2	Test parameters	4111

A.7.7.12.3.3	Test requirements	4112

A.7.7.12.4	UE Rx-Tx time difference measurement accuracy with PRS bandwidth aggregation in FR2 SA	4113

A.7.7.12.4.1	Test purpose and environment	4113

A.7.7.12.4.2	Test requirements	4117

A.7.7.13	PRS-RSRPP measurements	4117

A.7.7.13.1	SA measurement accuracy with PRS in FR2	4117

A.7.7.13.1.1	Test Purpose and Environment	4117

A.7.7.13.1.2	Test parameters	4117

A.7.7.13.1.3	Test Requirements	4119

A.7.7.13.2	SA measurement accuracy with reduced PRS samples in FR2	4119

A.7.7.13.2.1	Test Purpose and Environment	4119

A.7.7.13.2.2	Test parameters	4119

A.7.7.13.2.3	Test Requirements	4121

A.7.7.14	L1-RSRP measurement for group-based beam reporting	4121

A.7.7.14.1	SSB based L1-RSRP measurement	4121

A.7.7.14.1.1	Test Purpose and Environment	4121

A.7.7.14.1.2	Test parameters	4121

A.7.7.14.1.3	Test Requirements	4123

A.7.7.14.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	4123

A.7.7.14.2.1	Test Purpose and Environment	4123

A.7.7.14.2.2	Test parameters	4123

A.7.7.14.2.3	Test Requirements	4125

A.7.7.15	LTM L1-RSRP measurement	4125

A.7.7.15.1	SSB based inter-frequency L1-RSRP measurement	4125

A.7.7.15.1.1	Test Purpose and Environment	4125

A.7.7.15.1.2	Test parameters	4126

A.7.7.15.1.3	Test Requirements	4127

A.7.7.16	RSCPD Measurements	4128

A.7.7.16.1	RSCPD with RSTD measurement accuracy in FR2 SA in RRC_CONNECTED	4128

A.7.7.16.1.1	Test purpose and environment	4128

A.7.7.16.1.2	Test parameters	4128

A.7.7.16.1.3	Test requirements	4130

A.7.7.17	RSCP with UE Rx-Tx time difference measurements	4130

A.7.7.17.1	RSCP with UE Rx-Tx time difference measurement accuracy in FR2 SA	4130

A.7.7.17.1.1	Test purpose and environment	4130

A.7.7.17.1.2	Test parameters	4131

A.7.7.17.1.3	Test requirements	4134

A.7.8	Measurement procedure in RRC_INACTIVE	4134

A.7.8.1	RSTD measurements	4134

A.7.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_INACTIVE state	4134

A.7.8.1.1.1	Test Purpose and Environment	4134

A.7.8.1.1.2	Test Requirements	4137

A.7.8.1.2	NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA	4137

A.7.8.1.2.1	Test Purpose and Environment	4137

A.7.8.1.2.2	Test Requirements	4140

A.7.8.1.3	NR RSTD measurement reporting delay test case for PRS aggregation in FR2 SA in RRC_INACTIVE state	4140

A.7.8.1.3.1	Test purpose and environment	4140

A.7.8.1.3.2	Test requirements	4144

A.7.8.1.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_INACTIVE state with eDRX > 10.24s	4144

A.7.8.1.4.1	Test purpose and environment	4144

A.7.8.1.4.2	Test requirements	4147

A.7.8.2	PRS-RSRP measurements	4147

A.7.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	4147

A.7.8.2.1.1	Test Purpose and Environment	4147

A.7.8.2.1.2	Test Requirements	4149

A.7.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE	4150

A.7.8.2.2.1	Test purpose and Environment	4150

A.7.8.2.2.2	Test Requirements	4152

A.7.8.2.3	PRS-RSRP reporting delay in RRC_INACTIVE with eDRX	4152

A.7.8.2.3.1	Test Purpose and Environment	4152

A.7.8.2.3.2	Test Requirements	4156

A.7.8.3	UE Rx-Tx time difference measurements	4156

A.7.8.3.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	4156

A.7.8.3.1.1	Test purpose and environment	4156

A.7.8.3.1.2	Test requirements	4158

A.7.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR2 SA	4158

A.7.8.3.2.1	Test purpose and environment	4158

A.7.8.3.2.2	Test requirements	4160

A.7.8.3.3	UE Rx-Tx time difference measurements with PRS bandwidth aggregation in FR2 SA	4160

A.7.8.3.3.1	Test purpose and environment	4160

A.7.8.3.3.2	Test requirements	4163

A.7.8.3.4	UE Rx-Tx time difference measurements for single positioning frequency layer with eDRX > 10.24s in FR2 SA	4163

A.7.8.3.4.1	Test purpose and environment	4163

A.7.8.3.4.2	Test requirements	4166

A.7.8.4	PRS-RSRPP measurements	4166

A.7.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_INACTIVE state	4166

A.7.8.4.1.1	Test Purpose and Environment	4166

A.7.8.4.1.2	Test Requirements	4169

A.7.8.4.2	PRS-RSRPP reporting delay test with reduced number of samples for single positioning frequency layer in FR2 in RRC_INACTIVE state	4169

A.7.8.4.2.1	Test Purpose and Environment	4169

A.7.8.4.2.2	Test Requirements	4171

A.7.8.4.3	PRS-RSPP reporting delay in RRC_INACTIVE state with eDRX > 10.24s in FR2	4172

A.7.8.4.3.1	Test purpose and environment	4172

A.7.8.4.3.2	Test requirements	4175

A.7.8.5	RSCPD Measurements	4175

A.7.8.5.1	DL RSCPD reported with RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_INACTIVE state	4175

A.7.8.5.1.1	Test Purpose and Environment	4175

A.7.8.5.1.2	Test Requirements	4176

A.7.8.6	RSCP Measurements	4176

A.7.8.6.1	DL RSCP with UE Rx-Tx time difference measurements in RRC_INACTIVE for single positioning frequency layer in FR2 SA	4176

A.7.8.6.1.1	Test purpose and environment	4176

A.7.8.6.1.2	Test requirements	4180

A.7.9	Measurement performance requirements in RRC_INACTIVE	4180

A.7.9.1	RSTD measurements	4180

A.7.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR2 in RRC_INACTIVE state	4180

A.7.9.1.1.1	Test purpose and Environment	4180

A.7.9.1.1.2	Test Requirements	4182

A.7.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR2 in RRC_INACTIVE state	4182

A.7.9.1.2.1	Test purpose and Environment	4182

A.7.9.1.2.2	Test Requirements	4184

A.7.9.2	PRS-RSRP measurements	4186

A.7.9.2.1	SA measurement accuracy with PRS in FR2 in RRC_INACTIVE	4186

A.7.9.2.1.1	Test Purpose and Environment	4186

A.7.9.2.1.2	Test parameters	4186

A.7.9.2.1.3	Test Requirements	4187

A.7.9.2.2	PRS-RSRP measurements with reduced number of sample in RRC_INACTIVE	4188

A.7.9.2.2.1	Test Purpose and Environment	4188

A.7.9.2.2.2	Test parameters	4188

A.7.9.2.2.3	Test Requirements	4189

A.7.9.3	UE Rx-Tx time difference measurements	4190

A.7.9.3.1	UE Rx-Tx time difference measurements in RRC_INACTIVE	4190

A.7.9.3.1.1	Test purpose and environment	4190

A.7.9.3.1.2	Test parameters	4190

A.7.9.3.1.3	Test requirements	4191

A.7.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA	4191

A.7.9.3.2.1	Test purpose and environment	4191

A.7.9.3.2.2	Test parameters	4192

A.7.9.3.2.3	Test requirements	4193

A.7.9.3.3	UE Rx-Tx time difference measurement accuracy with PRS bandwidth aggregation in FR2 SA in RRC_INACTIVE state	4193

A.7.9.3.3.1	Test purpose and environment	4193

A.7.9.3.3.2	Test requirements	4197

A.7.9.4	PRS-RSRPP measurements	4197

A.7.9.4.1	SA measurement accuracy in FR2 in RRC INACTIVE	4197

A.7.9.4.1.1	Test Purpose and Environment	4197

A.7.9.4.1.2	Test parameters	4197

A.7.9.4.1.3	Test Requirements	4199

A.7.9.4.2	SA measurement accuracy with reduced PRS samples in FR2 in RRC INACTIVE	4199

A.7.9.4.2.1	Test Purpose and Environment	4199

A.7.9.4.2.2	Test parameters	4199

A.7.9.4.2.3	Test Requirements	4201

A.7.9.5	RSCPD Measurements	4201

A.7.9.5.1	RSCPD with RSTD measurement accuracy in FR2 SA in RRC_INACTIVE	4201

A.7.9.5.1.1	Test purpose and environment	4201

A.7.9.5.1.2	Test parameters	4201

A.7.9.5.1.3	Test requirements	4203

A.7.9.6	RSCP Measurements	4203

A.7.9.6.1	RSCP with UE Rx-Tx time difference measurement accuracy in FR2 SA	4203

A.7.9.6.1.1	Test purpose and environment	4203

A.7.9.6.1.2	Test parameters	4204

A.7.9.6.1.3	Test requirements	4205

A.7.10	Measurement Procedure in RRC_IDLE	4205

A.7.10.1	RSTD Measurements	4205

A.7.10.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_IDLE state for non-RedCap UE	4205

A.7.10.1.1.1	Test purpose and environment	4205

A.7.10.1.1.2	Test requirements	4208

A.7.10.1.2	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_IDLE state with eDRX > 10.24s	4208

A.7.10.1.2.1	Test purpose and environment	4208

A.7.10.1.2.2	Test requirements	4211

A.7.10.1.3	NR RSTD measurement reporting delay test case for PRS aggregation in FR2 SA in RRC_IDLE state	4211

A.7.10.1.3.1	Test purpose and environment	4211

A.7.10.1.3.2	Test requirements	4212

A.7.10.2	PRS-RSRP Measurements	4212

A.7.10.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_IDLE state for non-RedCap UE in FR2	4212

A.7.10.2.1.1	Test Purpose and Environment	4212

A.7.10.2.1.2	Test Requirements	4216

A.7.10.2.2	PRS-RSRP reporting delay test case in RRC_IDLE state in FR2 when eDRX cycle > 10.24s	4216

A.7.10.2.2.1	Test Purpose and Environment	4216

A.7.10.2.2.2	Test Requirements	4216

A.7.10.3	RSCPD Measurements	4217

A.7.10.3.1	DL RSCPD reported with RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_IDLE state	4217

A.7.10.3.1.1	Test Purpose and Environment	4217

A.7.10.3.1.2	Test Requirements	4217

A.7.11	Measurement Performance Requirements in RRC_IDLE	4218

A.7.11.1	RSTD Measurements	4218

A.7.11.1.1	NR RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA in RRC_IDLE state for non-RedCap UE	4218

A.7.11.1.1.1	Test purpose and environment	4218

A.7.11.1.1.2	Test requirements	4219

A.7.11.1.2	RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA in RRC_IDLE state with eDRX > 10.24s	4220

A.7.11.1.2.1	Test purpose and environment	4220

A.7.11.1.2.2	Test requirements	4221

A.7.11.1.3	NR RSTD measurement accuracy test case for PRS aggregation in FR2 SA in RRC_IDLE state	4222

A.7.11.1.3.1	Test purpose and environment	4222

A.7.11.1.3.2	Test requirements	4222

A.7.11.2	PRS-RSRP measurements	4222

A.7.11.2.1	PRS-RSRP measurement accuracy test case for non-RedCap UE in FR2 in RRC_IDLE state	4222

A.7.11.2.1.1	Test Purpose and Environment	4222

A.7.11.2.1.2	Test parameters	4222

A.7.11.2.1.3	Test Requirements	4224

A.7.11.2.2	PRS-RSRP measurement accuracy test case in RRC_IDLE state in FR2 for case 2 when eDRX cycle > 10.24s	4224

A.7.11.2.2.1	Test purpose and Environment	4224

A.7.11.2.2.1	Test parameters	4225

A.7.11.2.2.2	Test Requirements	4225

A.7.11.3	RSCPD measurements	4225

A.7.11.3.1	RSCPD with RSTD measurement accuracy in FR2 SA in RRC_IDLE	4225

A.7.11.3.1.1	Test purpose and environment	4225

A.7.11.3.1.2	Test parameters	4225

A.7.11.3.1.3	Test requirements	4227

A.8	E-UTRA standalone tests for NR RRM	4229

A.8.1	Void	4229

A.8.2	RRC_IDLE state mobility	4229

A.8.2.1	Inter-RAT NR Cell re-selection	4229

A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	4229

A.8.2.1.1.1	Test Purpose and Environment	4229

A.8.2.1.1.2	Test Requirements	4232

A.8.2.1.2	E-UTRA Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	4232

A.8.2.1.2.1	Test Purpose and Environment	4232

A.8.2.1.2.2	Test Requirements	4235

A.8.2.2	E-UTRA – NR Inter-RAT Early Measruement Reporting	4236

A.8.2.2.1	E-UTRA – NR Early Measurement Reporting for NR in FR1	4236

A.8.2.2.1.1	Test Purpose and Environment	4236

A.8.2.2.1.2	Test Requirements	4238

A.8.2.2.2	E-UTRA – NR Early Measurement Reporting for NR in FR2	4239

A.8.2.2.2.1	Test Purpose and Environment	4239

A.8.2.2.2.2	Test Requirements	4241

A.8.3	RRC_CONNECTED state mobility	4241

A.8.3.1	Handover	4241

A.8.3.1.1	E-UTRAN - NR handover in FR1	4241

A.8.3.1.1.1	Test Purpose and Environment	4241

A.8.3.1.1.2	Test Requirements	4245

A.8.4	Measurement procedure	4245

A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay	4245

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	4245

A.8.4.1.1.1	Test Purpose and Environment	4245

A.8.4.1.1.2	Test Requirements	4247

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	4247

A.8.4.1.2.1	Test Purpose and Environment	4247

A.8.4.1.2.2	Test Requirements	4248

A.8.4.2	E-UTRA – NR Inter-RAT Measurements	4248

A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4248

A.8.4.2.1.1	Test Purpose and Environment	4248

A.8.4.2.1.2	Test Requirements	4251

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4251

A.8.4.2.2.1	Test Purpose and Environment	4251

A.8.4.2.2.2	Test Requirements	4254

A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4255

A.8.4.2.3.1	Test Purpose and Environment	4255

A.8.4.2.3.2	Test Requirements	4258

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4258

A.8.4.2.4.1	Test Purpose and Environment	4258

A.8.4.2.4.2	Test Requirements	4261

A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	4261

A.8.4.2.5.1	Test Purpose and Environment	4261

A.8.4.2.5.2	Test Requirements	4263

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	4264

A.8.4.2.6.1	Test Purpose and Environment	4264

A.8.4.2.6.2	Test Requirements	4265

A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	4266

A.8.4.2.7.1	Test Purpose and Environment	4266

A.8.4.2.7.2	Test Requirements	4268

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	4268

A.8.4.2.8.1	Test Purpose and Environment	4268

A.8.4.2.8.2	Test Requirements	4270

A.8.4.2.9	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	4270

A.8.4.2.9.1	Test Purpose and Environment	4270

A.8.4.2.9.2	Test Requirements	4273

A.8.4.3	E-UTRAN - NR Inter-RAT event-triggered without measurement gaps	4274

A.8.4.3.1	NR Inter-RAT event triggered reporting tests for FR2 without MG nor DRX	4274

A.8.4.3.1.1	Test Purpose and Environment	4274

A.8.4.3.1.2	Test Requirements	4275

A.8.4.3.2	NR Inter-RAT event triggered reporting tests for FR1 without gaps when DRX is not used	4276

A.8.4.3.2.1	Test Purpose and Environment	4276

A.8.4.3.2.2	Test Requirements	4279

A.8.5	Measurement performance	4279

A.8.5.1	SFTD accuracy	4279

A.8.5.1.1	SFTD accuracy	4279

A.8.5.1.1.1	Test Purpose	4279

A.8.5.1.1.2	Test Environment	4279

A.8.5.1.1.3	Test Requirements	4283

A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements	4283

A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	4283

A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	4286

A.8.5.2.1.2.1	Test Purpose and Environment	4286

A.8.5.2.1.2.2	Test Parameters	4286

A.8.5.2.1.2.3	Test Requirements	4288

A.8.5.2.2	SS-RSRQ	4288

A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	4288

A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	4291

A.8.5.2.2.2.1	Test Purpose and Environment	4292

A.8.5.2.2.2.2	Test Parameters	4292

A.8.5.2.2.2.3	Test Requirements	4293

A.8.5.2.3	SS-SINR	4293

A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	4293

A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	4296

A.8.5.2.3.2.1	Test Purpose and Environment	4296

A.8.5.2.3.2.2	Test Parameters	4297

A.8.5.2.3.2.3	Test Requirements	4298

A.9	V2X Tests	4298

A.9.1	V2X Tests in FR1	4298

A.9.1.1	Test for V2X UE Transmit Timing	4298

A.9.1.1.1 Test for GNSS as Synchronization Reference Source	4298

A.9.1.1.1.1	Test Purpose and Environment	4298

A.9.1.1.1.2	Test requirements	4299

A.9.1.1.2	Test for SyncRef UE as Synchronization Reference Source	4299

A.9.1.1.2.1	Test Purpose and Environment	4299

A.9.1.1.2.2	Test requirements	4299

A.9.1.1.3	Test for FR1 NR Cell as Synchronization Reference Source	4300

A.9.1.1.3.1	Test Purpose and Environment	4300

A.9.1.1.3.2	Test requirements	4301

A.9.1.2	Test for Initiation/Cease of S-SSB Transmission with V2X Sidelink Communication	4301

A.9.1.2.1	Test for FR1 NR Cell as synchronization reference source without gap under non-DRX	4301

A.9.1.2.1.1	Test Purpose and Environment	4301

A.9.1.2.1.2	Test Requirements	4303

A.9.1.2.2	Test for SyncRef UE as synchronization reference source	4304

A.9.1.2.2.1	Test Purpose and Environment	4304

A.9.1.2.2.2	Test Requirements	4305

A.9.1.2.3	Test for SyncRef UE as synchronization reference source when SL-DRX is used	4305

A.9.1.2.3.1	Test Purpose and Environment	4305

A.9.1.2.3.2	Test Requirements	4307

A.9.1.2.4	Test for SyncRef UE as synchronization reference source with CCA	4307

A.9.1.2.4.1	Test Purpose and Environment	4307

A.9.1.2.4.2	Test Requirements	4308

A.9.1.3	 Test for V2X Synchronization Reference Selection/Reselection	4309

A.9.1.3.1	 Test for GNSS configured as the highest priority	4309

A.9.1.3.1.1	Test Purpose and Environment	4309

A.9.1.3.1.2	Test Requirements	4310

A.9.1.3.2	 Test for FR1 NR Cell configured as the highest priority	4311

A.9.1.3.2.1	Test Purpose and Environment	4311

A.9.1.3.2.2	Test Requirements	4313

A.9.1.3.3	Test for GNSS configured as the highest priority under SL-DRX	4314

A.9.1.3.3.1	Test Purpose and Environment	4314

A.9.1.3.3.2	Test Requirements	4315

A.9.1.3.4	Test for FR1 NR Cell configured as the highest priority under SL-DRX	4316

A.9.1.3.4.1	Test Purpose and Environment	4316

A.9.1.3.4.2	Test Requirements	4318

A.9.1.4	Test for L1 SL-RSRP Measurement	4318

A.9.1.4.1	Test for V2X UE Autonomous Resource Selection/Reselection	4318

A.9.1.4.1.1	Test Purpose and Environment	4318

A.9.1.4.1.2	Test Requirements	4320

A.9.1.4.2	Test for V2X UE Resource Pre-emption	4321

A.9.1.4.2.1	Test Purpose and Environment	4321

A.9.1.4.2.2	Test Requirements	4322

A.9.1.4.3	 Test for V2X UE Resource Re-evaluation	4323

A.9.1.4.3.1	Test Purpose and Environment	4323

A.9.1.4.3.2	Test Requirements	4326

A.9.1.4.4	Test for V2X UE Autonomous Resource Selection/Reselection with Periodic Sensing	4326

A.9.1.4.4.1	Test Purpose and Environment	4326

A.9.1.4.4.2	Test Requirements	4328

A.9.1.4.5	Test for V2X UE Autonomous Resource Selection/Reselection with Contiguous Sensing	4328

A.9.1.4.5.1	Test Purpose and Environment	4328

A.9.1.4.5.2	Test Requirements	4330

A.9.1.4.6	Test for V2X UE Autonomous Resource Selection/Reselection in SL-DRX	4331

A.9.1.4.6.1	Test Purpose and Environment	4331

A.9.1.4.6.2	Test Requirements	4333

A.9.1.5	Test for Congestion Control Measurement	4333

A.9.1.5.1	Test Purpose and Environment	4333

A.9.1.5.2	Test Requirements	4336

A.9.1.6	Test for Interruption	4336

A.9.1.6.1	Test for Interruption to WAN due to V2X Sidelink Communication	4336

A.9.1.6.1.1	Test Purpose and Environment	4336

A.9.1.6.1.2	Test Requirements	4339

A.9.1.6.2	Test for interruption to WAN at transitions between active and non-active during SL-DRX in asynchronous case	4339

A.9.1.6.2.1	Test Purpose and Environment	4339

A.9.1.6.2.2	Test Requirements	4340

A.9.1.6.3	Test for Interruption at NR Sidelink Diccovery Configuration	4341

A.9.1.6.3.1	Test Purpose and Environment	4341

A.9.1.6.3.2	Test Requirements	4343

A.9.1.7	Selection / Reselection of relay UE	4343

A.9.1.7.1	Test Purpose and Environment	4343

A.9.1.7.2	Test Requirements	4347

A.9A	Tests for NR Sidelink Measurements for Positioning	4348

A.9A.1	Tests for NR Sidelink Measurements for Positioning in FR1	4348

A.9A.1.1	Measurement delay tests	4348

A.9A.1.1.1	NR SL RSTD measurement reporting delay test case in FR1 SA	4348

A.9A.1.1.1.1	Test Purpose and Environment	4348

A.9A.1.1.1.2	Test Requirements	4353

A.9A.1.1.2	SL Rx-Tx measurement delay tests	4353

A.9A.1.1.2.1	Test Purpose and Environment	4353

A.9A.1.1.2.2	Test Requirements	4356

A.9A.1.1.3	NR SL AoA measurements reporting delay test in FR1 SA	4357

A.9A.1.1.3.1	Test Purpose and Environment	4357

A.9A.1.1.3.2	Test Requirements	4360

A.9A.1.1.4	NR SL RTOA measurements reporting delay test in FR1 SA	4361

A.9A.1.1.4.1	Test Purpose and Environment	4361

A.9A.1.1.4.2	Test Requirements	4364

A.9A.1.1.5	NR SL PRS-RSRP measurement reporting delay test case in FR1 SA	4365

A.9A.1.1.5.1	Test Purpose and Environment	4365

A.9A.1.1.5.2	Test Requirements	4365

A.9A.1.1.6	NR SL PRS-RSRPP measurement reporting delay test case in FR1 SA	4365

A.9A.1.1.6.1	Test Purpose and Environment	4365

A.9A.1.1.6.2	Test Requirements	4365

A.9A.1.2	Measurement Accuracy Tests	4366

A.9A.1.2.1	NR SL RSTD measurement accuracy test case in FR1 SA	4366

A.9A.1.2.1.1	Test Purpose and Environment	4366

A.9A.1.2.1.2	Test Requirements	4369

A.9A.1.2.2	SL Rx-Tx measurement accuracy test case in FR1	4370

A.9A.1.2.2.1	Test Purpose and Environment	4370

A.9A.1.2.2.2	Test Requirements	4373

A.9A.1.2.3	NR SL PRS-RSRP measurement accuracy test case in FR1 SA	4373

A.9A.1.2.3.1	Test Purpose and Environment	4373

A.9A.1.2.3.2	Test Requirements	4374

A.9A.1.2.4	NR SL PRS-RSRPP measurement accuracy test case in FR1 SA	4374

A.9A.1.2.4.1	Test Purpose and Environment	4374

A.9A.1.2.4.2	Test Requirements	4374

A.10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	4375

A.10.1	RRC_CONNECTED state mobility	4375

A.10.1.1	RRC connection mobility control	4375

A.10.1.1.1	Random Access	4375

A.10.1.1.1.1	4-step RA type contention-based random access for NR PSCell with CCA	4375

A.10.1.1.1.1.1	Test Purpose and Environment	4375

A.10.1.1.1.1.2	Test Requirements	4376

A.10.1.1.1.1.2.1	Random Access Preamble Transmission	4377

A.10.1.1.1.1.2.2	Random Access Response Reception	4377

A.10.1.1.1.1.2.3	No Random Access Response Reception	4377

A.10.1.1.1.1.2.4	Receiving an UL grant for msg3 retransmission	4378

A.10.1.1.1.1.2.5	 Contention Resolution Timer expiry	4378

A.10.1.1.1.2	4-step RA type non-contention based random access for NR PSCell with CCA	4378

A.10.1.1.1.2.1	Test Purpose and Environment	4378

A.10.1.1.1.2.2	Test Requirements	4379

A.10.1.1.1.2.2.1	SSB-based Random Access Preamble Transmission	4380

A.10.1.1.1.2.2.2	Random Access Response Reception	4380

A.10.1.1.1.2.2.3	No Random Access Response Reception	4380

A.10.1.1.1.3	2-step RA type contention-based random access for NR PSCell with CCA	4381

A.10.1.1.1.3.1	Test Purpose and Environment	4381

A.10.1.1.1.3.2	Test Requirements	4382

A.10.1.1.1.3.2.1	MsgA Transmission	4383

A.10.1.1.1.3.2.2	MsgB Reception	4383

A.10.1.1.1.3.2.3	No MsgB Reception	4383

A.10.1.1.1.4	2-step RA type non-contention based random access for NR PSCell with CCA	4384

A.10.1.1.1.4.1	Test Purpose and Environment	4384

A.10.1.1.1.4.2	Test Requirements	4385

A.10.1.1.1.4.2.1	MsgA Transmission	4386

A.10.1.1.1.4.2.2	MsgB Reception	4386

A.10.1.1.1.4.2.3	No MsgB Reception	4386

A.10.1.2	Handover with PSCell from EN-DC to EN-DC with known target PSCell using CCA	4387

A.10.1.2.1	Test Purpose and Environment	4387

A.10.1.2.2	Test Requirements	4391

A.10.2	Timing	4392

A.10.2.1	UE transmit timing	4392

A.10.2.1.1	UE Transmit Timing Test with PSCell under DL CCA	4392

A.10.2.1.1.1	Test Purpose and environment	4392

A.10.2.1.1.2	Test requirements	4394

A.10.2.2	UE timing advance	4395

A.10.2.2.1	UE Timing Advance Adjustment Accuracy with PSCell under DL CCA	4395

A.10.2.2.1.1	Test Purpose and Environment	4395

A.10.2.2.1.2	Test Parameters	4395

A.10.2.2.1.3	Test Requirements	4397

A.10.3	Signalling characteristics	4397

A.10.3.1	Radio link monitoring	4397

A.10.3.1.1	Introduction	4397

A.10.3.1.2	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	4398

A.10.3.1.2.1	Test purpose and environment	4398

A.10.3.1.2.2	Test requirements	4400

A.10.3.1.3	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	4401

A.10.3.1.3.1	Test purpose and environment	4401

A.10.3.1.3.2	Test requirements	4403

A.10.3.1.4	Void	4403

A.10.3.1.4.1	Void	4403

A.10.3.1.4.2	Void	4403

A.10.3.1.5	Void	4404

A.10.3.1.5.1	Void	4404

A.10.3.1.5.2	Void	4404

A.10.3.2	Void	4404

A.10.3.3	SCell activation and deactivation delay	4404

A.10.3.3.1	SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle	4404

A.10.3.3.1.1	Test Purpose and Environment	4404

A.10.3.3.1.2	Test Requirements	4407

A.10.3.3.2 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle	4407

A.10.3.3.2.1	Test Purpose and Environment	4407

A.10.3.3.2.2	Test Requirements	4408

A.10.3.3.3	SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA	4408

A.10.3.3.3.1	Test Purpose and Environment	4408

A.10.3.3.3.2	Test Requirements	4408

A.10.3.4	Beam failure detection and link recovery procedures	4409

A.10.3.4.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	4409

A.10.3.4.1.1	Test Purpose and Environment	4409

A.10.3.4.1.2	Test Requirements	4412

A.10.3.4.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	4412

A.10.3.4.2.1	Test Purpose and Environment	4412

A.10.3.4.2.2	Test Requirements	4415

A.10.3.5	Active BWP switching	4416

A.10.3.5.1	UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA in EN-DC	4416

A.10.3.5.1.2	Test Requirements	4418

A.10.3.5.2	DCI-based and Timer-based Active BWP Switch	4419

A.10.3.5.2.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	4419

A.10.3.5.2.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	4422

A.10.3.5.3	RRC-based Active BWP Switch	4426

A.10.3.5.3.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	4426

A.10.3.6	PSCell addition and release delay	4428

A.10.3.6.1	Addition and Release Delay of known NR PSCell on the carrier under CCA	4428

A.10.3.6.1.1	Test purpose and environment	4428

A.10.3.6.1.2	Test Requirements	4431

A.10.3.7	Void	4431

A.10.4	Measurement procedure	4431

A.10.4.1	Intra-frequency measurements	4432

A.10.4.1.1	Event-triggered reporting tests on PSCC without gaps under non-DRX	4432

A.10.4.1.1.1	Test purpose and environment	4432

A.10.4.1.1.2	Test parameters	4432

A.10.4.1.1.3	Test Requirements	4434

A.10.4.1.2	Void	4434

A.10.4.1.3	Void	4434

A.10.4.1.4	Event-triggered reporting tests on PSCC with per-UE gaps under DRX	4434

A.10.4.1.4.1	Test purpose and environment	4434

A.10.4.1.4.2	Test parameters	4434

A.10.4.1.4.3	Test Requirements	4437

A.10.4.1.5	Void	4437

A.10.4.1.6	Void	4437

A.10.4.1.7	Void	4437

A.10.4.1.8	Void	4437

A.10.4.1.9	Void	4437

A.10.4.1.10	Void	4437

A.10.4.1.11	Void	4437

A.10.4.1.12	Void	4437

A.10.4.2	Inter-frequency measurements	4437

A.10.4.2.1	Void	4437

A.10.4.2.2	Void	4437

A.10.4.2.3	EN-DC event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used	4438

A.10.4.2.3.1	Test Purpose and Environment	4438

A.10.4.2.3.2	Test Requirements	4440

A.10.4.2.4	EN-DC event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used	4440

A.10.4.2.4.1	Test Purpose and Environment	4440

A.10.4.2.4.2	Test Requirements	4443

A.10.4.2.5	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used	4444

A.10.4.2.5.1	Test Purpose and Environment	4444

A.10.4.2.5.2	Test Requirements	4446

A.10.4.2.6	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used	4446

A.10.4.2.6.1	Test Purpose and Environment	4446

A.10.4.2.6.2	Test Requirements	4449

A.10.4.2.7	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	4450

A.10.4.2.7.1	Test Purpose and Environment	4450

A.10.4.2.7.2	Test Requirements	4453

A.10.4.2.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	4453

A.10.4.2.8.1	Test Purpose and Environment	4453

A.10.4.2.8.2	Test Requirements	4457

A.10.4.2.9	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	4458

A.10.4.2.9.1	Test Purpose and Environment	4458

A.10.4.2.9.2	Test Requirements	4461

A.10.4.2.10	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	4461

A.10.4.2.10.1	Test Purpose and Environment	4461

A.10.4.2.10.2	Test Requirements	4465

A.10.4.3	L1-RSRP measurements for beam reporting	4466

A.10.4.3.1	SSB based L1-RSRP measurement on PSCC when DRX is not used	4466

A.10.4.3.1.1	Test Purpose and Environment	4466

A.10.4.3.1.2	Test parameters	4466

A.10.4.3.1.3	Test Requirements	4467

A.10.4.3.2	SSB based L1-RSRP measurement on PSCC when DRX is used	4468

A.10.4.3.2.1	Test Purpose and Environment	4468

A.10.4.3.2.2	Test parameters	4468

A.10.4.3.2.3	Test Requirements	4469

A.10.4.3.3	SSB based L1-RSRP measurement on SCC when DRX is not used	4470

A.10.4.3.3.1	Test Purpose and Environment	4470

A.10.4.3.3.2	Test parameters	4470

A.10.4.3.3.3	Test Requirements	4471

A.10.4.3.4	SSB based L1-RSRP measurement on SCC when DRX is used	4472

A.10.4.3.4.1	Test Purpose and Environment	4472

A.10.4.3.4.2	Test parameters	4472

A.10.4.3.4.3	Test Requirements	4473

A.10.4.4	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	4474

A.10.4.4.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4474

A.10.4.4.1.1	Test Purpose and Environment	4474

A.10.4.4.1.2	Test Requirements	4477

A.10.4.4.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4478

A.10.4.4.2.1	Test Purpose and Environment	4478

A.10.4.4.2.2	Test Requirements	4481

A.10.4.4.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4482

A.10.4.4.3.1	Test Purpose and Environment	4482

A.10.4.4.3.2	Test Requirements	4485

A.10.4.4.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4485

A.10.4.4.4.1	Test Purpose and Environment	4485

A.10.4.4.4.2	Test Requirements	4489

A.10.5	Measurement performance	4489

A.10.5.1	SS-RSRP	4489

A.10.5.1.1	Intra-frequency measurement accuracy on a CCA serving cell	4489

A.10.5.1.1.1	Test Purpose and Environment	4489

A.10.5.1.1.2	Test parameters	4490

A.10.5.1.1.3	Test Requirements	4491

A.10.5.1.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4491

A.10.5.1.2.1	Test Purpose and Environment	4491

A.10.5.1.2.2	Test parameters	4492

A.10.5.1.2.3	Test Requirements	4493

A.10.5.2	SS-RSRQ	4493

A.10.5.2.1	Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4493

A.10.5.2.1.1	Test Purpose and Environment	4493

A.10.5.2.1.2	Test Parameters	4493

A.10.5.2.1.3	Test Requirements	4495

A.10.5.2.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4495

A.10.5.2.2.1	Test Purpose and Environment	4495

A.10.5.2.2.2	Test Parameters	4495

A.10.5.2.2.3	Test Requirements	4497

A.10.5.3	SS-SINR	4497

A.10.5.3.1	Intra-frequency measurement accuracy on PSCC	4497

A.10.5.3.1.1	Test Purpose and Environment	4497

A.10.5.3.1.2	Test Parameters	4497

A.10.5.3.1.3	Test Requirements	4498

A.10.5.3.2	Inter-frequency measurement accuracy on PSCC	4499

A.10.5.3.2.1	Test Purpose and Environment	4499

A.10.5.3.2.2	Test Parameters	4499

A.10.5.3.2.3	Test Requirements	4500

A.10.5.3.3	Intra-frequency measurement accuracy on SCC	4500

A.10.5.3.3.1	Test Purpose and Environment	4500

A.10.5.3.3.2	Test Parameters	4500

A.10.5.3.3.3	Test Requirements	4502

A.10.5.4	L1-RSRP measurement for beam reporting with CCA serving cell	4502

A.10.5.4.1	SSB based L1-RSRP measurement	4502

A.10.5.4.1.1	Test Purpose and Environment	4502

A.10.5.4.1.2	Test parameters	4503

A.10.5.4.1.3	Test Requirements	4504

A.10.5.5	RSSI	4504

A.10.5.5.1 	RSSI measurement accuracy on PSCC with CCA	4504

A.10.5.5.1.1	Test Purpose and Environment	4504

A.10.5.5.1.2	Test parameters	4504

A.10.5.5.1.3	Test Requirements	4506

A.10.5.5.2	RSSI measurement accuracy on SCC with CCA	4506

A.10.5.5.2.1	Test Purpose and Environment	4506

A.10.5.5.2.2	Test parameters	4506

A.10.5.5.2.3	Test Requirements	4507

A.10.5.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4507

A.10.5.5.3.1	Test Purpose and Environment	4507

A.10.5.5.3.2	Test parameters	4508

A.10.5.5.3.3	Test Requirements	4509

A.10.5.6	Channel occupancy	4509

A.10.5.6.1 	Channel occupancy measurement accuracy on PSCC with CCA	4509

A.10.5.6.1.1	Test Purpose and Environment	4509

A.10.5.6.1.2	Test parameters	4509

A.10.5.6.1.3	Test Requirements	4511

A.10.5.6.2 	Channel occupancy measurement accuracy on SCC with CCA	4511

A.10.5.6.2.1	Test Purpose and Environment	4511

A.10.5.6.2.2	Test parameters	4511

A.10.5.6.2.3	Test Requirements	4512

A.10.5.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4513

A.10.5.6.3.1	Test Purpose and Environment	4513

A.10.5.6.3.2	Test parameters	4513

A.10.5.6.3.3	Test Requirements	4514

A.11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	4517

A.11.1	RRC_IDLE state mobility	4517

A.11.1.1	Cell re-selection with both source and target NR carrier frequencies under CCA	4517

A.11.1.1.1	Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell	4517

A.11.1.1.1.1	Test Purpose and Environment	4517

A.11.1.1.1.2	Test Parameters	4517

A.11.1.1.1.3	Test Requirements	4519

A.11.1.1.2	Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell	4520

A.11.1.1.2.1	Test Purpose and Environment	4520

A.11.1.1.2.2	Test Parameters	4520

A.11.1.1.2.3	Test Requirements	4522

A.11.1.2	Cell re-selection to NR with source NR carrier frequency under CCA	4522

A.11.1.2.1	Cell reselection to FR1 inter-frequency NR case when serving cell is subject to CCA	4522

A.11.1.2.1.1	Test Purpose and Environment	4522

A.11.1.2.1.2	Test Parameters	4523

A.11.1.2.1.3	Test Requirements	4525

A.11.1.3	Cell re-selection from NR carrier with target NR carrier frequency under CCA	4526

A.11.1.3.1	Cell reselection to FR1 inter-frequency NR case when target cell is subject to CCA	4526

A.11.1.3.1.1	Test Purpose and Environment	4526

A.11.1.3.1.2	Test Parameters	4526

A.11.1.3.1.3	Test Requirements	4529

A.11.1.4	Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA	4530

A.11.1.4.1	Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA	4530

A.11.1.4.1.1	Test Purpose and Environment	4530

A.11.1.4.1.2	Test Parameters	4530

A.11.1.4.1.3	Test Requirements	4533

A.11.1.4.2	Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA	4533

A.11.1.4.2.1	Test Purpose and Environment	4533

A.11.1.4.2.2	Test Requirements	4535

A.11.2	RRC_CONNECTED state mobility	4536

A.11.2.1	Handover	4536

A.11.2.1.1	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell	4536

A.11.2.1.1.1	Test Purpose and Environment	4536

A.11.2.1.1.2	Test Parameters	4536

A.11.2.1.1.3 Test Requirements	4538

A.11.2.1.2	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4539

A.11.2.1.2.1	Test Purpose and Environment	4539

A.11.2.1.2.2	Test Parameters	4539

A.11.2.1.2.3	Test Requirements	4541

A.11.2.1.3	Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4541

A.11.2.1.3.1	Test Purpose and Environment	4541

A.11.2.1.3.2	Test Parameters	4541

A.11.2.1.3.3	Test Requirements	4543

A.11.2.1.4	Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell	4544

A.11.2.1.4.1	Test Purpose and Environment	4544

A.11.2.1.4.2	Test Parameters	4544

A.11.2.1.4.3	Test Requirements	4547

A.11.2.1.5	Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell	4547

A.11.2.1.5.1	Test Purpose and Environment	4547

A.11.2.1.5.2	Test Parameters	4547

A.11.2.1.5.3 Test Requirements	4550

A.11.2.1.6	Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell	4550

A.11.2.1.6.1	Test Purpose and Environment	4550

A.11.2.1.6.2	Test Parameters	4550

A.11.2.1.6.3	Test Requirements	4553

A.11.2.1.7	 SA NR FR1 carrier under CCA - E-UTRAN handover with known target cell	4553

A.11.2.1.7.1	Test Purpose and Environment	4553

A.11.2.1.7.2	Test Requirements	4556

A.11.2.1.8	SA NR FR1 carrier under CCA - E-UTRAN handover with unknown target cell	4557

A.11.2.1.8.1	Test Purpose and Environment	4557

A.11.2.1.8.2	Test Requirements	4560

A.11.2.1.9	Handover with PSCell from NR SA to EN-DC with known target PSCell using CCA	4560

A.11.2.1.9.1	Test Purpose and Environment	4560

A.11.2.1.9.2	Test Requirements	4566

A.11.2.2	RRC connection mobility control	4567

A.11.2.2.1	RRC re-establishment	4567

A.11.2.2.1.1	Intra-frequency RRC Re-establishment with CCA in FR1	4567

A.11.2.2.1.2	Inter-frequency RRC Re-establishment with CCA in FR1	4570

A.11.2.2.1.4	Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA	4576

A.11.2.2.2	Random Access	4579

A.11.2.2.2.1	4-step RA type contention-based random access for NR PCell with CCA	4579

A.11.2.2.2.1.1	Test Purpose and Environment	4579

A.11.2.2.2.1.2	Test Requirements	4580

A.11.2.2.2.1.2.1	Random Access Preamble Transmission	4580

A.11.2.2.2.1.2.2	Random Access Response Reception	4581

A.11.2.2.2.1.2.3	No Random Access Response Reception	4581

A.11.2.2.2.1.2.4	Receiving an UL grant for msg3 retransmission	4581

A.11.2.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI	4581

A.11.2.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI	4582

A.11.2.2.2.1.2.7	Contention Resolution Timer expiry	4582

A.11.2.2.2.2	4-step RA type non-contention based random access for NR PSCell with CCA	4582

A.11.2.2.2.2.1	Test Purpose and Environment	4582

A.11.2.2.2.2.2	Test Requirements	4583

A.11.2.2.2.2.2.1	SSB-based Random Access Preamble Transmission	4583

A.11.2.2.2.2.2.2	Random Access Response Reception	4584

A.11.2.2.2.2.2.3	No Random Access Response Reception	4584

A.11.2.2.2.3	2-step RA type contention-based random access for NR PCell with CCA	4585

A.11.2.2.2.3.1	Test Purpose and Environment	4585

A.11.2.2.2.3.2	Test Requirements	4586

A.11.2.2.2.3.2.1	MsgA Transmission	4586

A.11.2.2.2.3.2.2	MsgB Reception	4587

A.11.2.2.2.3.2.3	No MsgB Reception	4587

A.11.2.2.2.4	2-step RA type non-contention-based random access for NR PCell with CCA	4588

A.11.2.2.2.4.1	Test Purpose and Environment	4588

A.11.2.2.2.4.2	Test Requirements	4589

A.11.2.2.2.4.2.1	MsgA Transmission	4589

A.11.2.2.2.4.2.2	MsgB Reception	4590

A.11.2.2.2.4.2.3	No MsgB Reception	4590

A.11.2.2.3	RRC connection release with redirection	4591

A.11.2.2.3.1	Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	4591

A.11.2.2.3.2	Redirection from NR FR1 carrier without CCA to NR FR1 carrier with CCA	4593

A.11.3	Timing	4596

A.11.3.1	UE transmit timing	4596

A.11.3.1.1	UE Transmit Timing Test with PCell under DL CCA	4596

A.11.3.1.1.1	Test Purpose and environment	4596

A.11.3.1.1.2	Test requirements	4598

A.11.3.2	UE timing advance	4599

A.11.3.2.1	UE Timing Advance Adjustment Accuracy with PCell under DL CCA	4599

A.11.3.2.1.1	Test Purpose and Environment	4599

A.11.3.2.1.2	Test Parameters	4599

A.11.3.2.1.3	Test Requirements	4601

A.11.4	Signalling characteristics	4601

A.11.4.1	Radio link monitoring	4601

A.11.4.1.1	Introduction	4601

A.11.4.1.2	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	4602

A.11.4.1.2.1	Test purpose and environment	4602

A.11.4.1.2.2	Test requirements	4604

A.11.4.1.3	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	4605

A.11.4.1.3.1	Test purpose and environment	4605

A.11.4.1.3.2	Test requirements	4608

A.11.4.1.4	Void	4608

A.11.4.1.4.1	Void	4608

A.11.4.1.4.2	Void	4608

A.11.4.1.5	Void	4608

A.11.4.1.5.1	Void	4608

A.11.4.1.5.2	Void	4608

A.11.4.2	Void	4608

A.11.4.3	SCell activation and deactivation delay	4608

A.11.4.3.1	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle	4608

A.11.4.3.1.1	Test Purpose and Environment	4608

A.11.4.3.1.2	Test Requirements	4611

A.11.4.3.2	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 640 ms SCell measurement cycle	4611

A.11.4.3.2.1	Test Purpose and Environment	4611

A.11.4.3.2.2	Test Requirements	4612

A.11.4.3.3	SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA	4612

A.11.4.3.3.1	Test Purpose and Environment	4612

A.11.4.3.3.2	Test Requirements	4612

A.11.4.4	Beam failure detection and link recovery procedures	4613

A.11.4.4.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	4613

A.11.4.4.1.1	Test Purpose and Environment	4613

A.11.4.4.1.2	Test Requirements	4616

A.11.4.4.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	4617

A.11.4.4.2.1	Test Purpose and Environment	4617

A.11.4.4.2.2	Test Requirements	4620

A.11.4.5	Active BWP switching	4620

A.11.4.5.1	UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA	4620

A.11.4.5.1.1	Test Purpose and Environment	4620

A.11.4.5.1.2	Test Requirements	4623

A.11.4.5.2	DCI-based and Timer-based Active BWP Switch	4623

A.11.4.5.2.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA	4623

A.11.4.5.2.2	NR FR1 DL active BWP switch with non-DRX in SA	4626

A.11.4.5.3	RRC-based Active BWP Switch	4629

A.11.4.5.3.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	4629

A.11.4.6	Void	4631

A.11.5	Measurement procedure	4631

A.11.5.1	Intra-frequency measurements	4631

A.11.5.1.1	Event-triggered reporting tests on PCC without gaps under non-DRX	4631

A.11.5.1.1.1	Test purpose and environment	4631

A.11.5.1.1.2	Test parameters	4631

A.11.5.1.1.3	Test Requirements	4633

A.11.5.1.2	Event-triggered reporting tests on PCC without gaps under DRX	4633

A.11.5.1.2.1	Test purpose and environment	4633

A.11.5.1.2.2	Test parameters	4633

A.11.5.1.2.3	Test Requirements	4635

A.11.5.1.3	Void	4636

A.11.5.1.4	Void	4636

A.11.5.1.5	Void	4636

A.11.5.1.6	Void	4636

A.11.5.1.7	Void	4636

A.11.5.1.8	Void	4636

A.11.5.1.9	Void	4636

A.11.5.1.10	Void	4636

A.11.5.1.11	Void	4636

A.11.5.1.12	Void	4636

A.11.5.2	Inter-frequency measurements	4636

A.11.5.2.1	Void	4636

A.11.5.2.2	Void	4636

A.11.5.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	4636

A.11.5.2.3.1	Test Purpose and Environment	4636

A.11.5.2.3.2	Test Requirements	4639

A.11.5.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	4639

A.11.5.2.4.1	Test Purpose and Environment	4639

A.11.5.2.4.2	Test Requirements	4642

A.11.5.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	4642

A.11.5.2.5.1	Test Purpose and Environment	4642

A.11.5.2.5.2	Test Requirements	4645

A.11.5.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	4645

A.11.5.2.6.1	Test Purpose and Environment	4645

A.11.5.2.6.2	Test Requirements	4648

A.11.5.2.7	Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4649

A.11.5.2.7.1	Test Purpose and Environment	4649

A.11.5.2.7.2	Test Requirements	4651

A.11.5.2.8	Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4652

A.11.5.2.8.1	Test Purpose and Environment	4652

A.11.5.2.8.2	Test Requirements	4655

A.11.5.2.9	Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4655

A.11.5.2.9.1	Test Purpose and Environment	4655

A.11.5.2.9.2	Test Requirements	4658

A.11.5.2.10	Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4658

A.11.5.2.10.1	Test Purpose and Environment	4658

A.11.5.2.10.2	Test Requirements	4662

A.11.5.3	Inter-RAT E-UTRAN measurements	4662

A.11.5.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	4662

A.11.5.3.1.1	Test Purpose and Environment	4662

A.11.5.3.1.2	Test Requirements	4665

A.11.5.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	4665

A.11.5.3.2.1	Test Purpose and Environment	4665

A.11.5.3.2.2	Test Requirements	4669

A.11.5.4	L1-RSRP measurements for beam reporting	4669

A.11.5.4.1	SSB based L1-RSRP measurement when DRX is not used	4669

A.11.5.4.1.1	Test Purpose and Environment	4669

A.11.5.4.1.2	Test parameters	4669

A.11.5.4.1.3	Test Requirements	4671

A.11.5.4.2	SSB based L1-RSRP measurement when DRX is used	4671

A.11.5.4.2.1	Test Purpose and Environment	4671

A.11.5.4.2.2	Test parameters	4671

A.11.5.4.2.3	Test Requirements	4673

A.11.5.4.3	SSB based L1-RSRP measurement on SCC when DRX is not used	4673

A.11.5.4.3.1	Test Purpose and Environment	4673

A.11.5.4.3.2	Test parameters	4674

A.11.5.4.3.3	Test Requirements	4675

A.11.5.4.4	SSB based L1-RSRP measurement on SCC when DRX is used	4675

A.11.5.4.4.1	Test Purpose and Environment	4675

A.11.5.4.4.2	Test parameters	4676

A.11.5.4.4.3	Test Requirements	4677

A.11.6	Measurement performance	4677

A.11.6.1	SS-RSRP	4677

A.11.6.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	4677

A.11.6.1.1.1	Test Purpose and Environment	4677

A.11.6.1.1.2	Test parameters	4678

A.11.6.1.1.3	Test Requirements	4679

A.11.6.1.2	Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	4679

A.11.6.1.2.1	Test Purpose and Environment	4679

A.11.6.1.2.2	Test parameters	4679

A.11.6.1.2.3	Test Requirements	4681

A.11.6.2	SS-RSRQ	4681

A.11.6.2.1	Intra-frequency measurement accuracy	4681

A.11.6.2.1.1	Test Purpose and Environment	4681

A.11.6.2.1.2	Test Parameters	4681

A.11.6.2.1.3	Test Requirements	4683

A.11.6.2.2	Inter-frequency measurement accuracy	4683

A.11.6.2.2.1	Test Purpose and Environment	4683

A.11.6.2.2.2	Test Parameters	4683

A.11.6.2.2.3	Test Requirements	4686

A.11.6.2.3	Intra-frequency measurement accuracy on SCC	4686

A.11.6.2.3.1	Test Purpose and Environment	4686

A.11.6.2.3.2	Test Parameters	4686

A.11.6.2.3.3	Test Requirements	4688

A.11.6.2.4	Inter-frequency measurement accuracy	4688

A.11.6.2.4.1	Test Purpose and Environment	4688

A.11.6.2.4.2	Test Parameters	4688

A.11.6.2.4.3	Test Requirements	4695

A.11.6.3	SS-SINR	4695

A.11.6.3.1	Intra-frequency measurement accuracy	4695

A.11.6.3.1.1	Test Purpose and Environment	4695

A.11.6.3.1.2	Test Parameters	4695

A.11.6.3.1.3	Test Requirements	4697

A.11.6.3.2	Inter-frequency measurement accuracy	4697

A.11.6.3.2.1	Test Purpose and Environment	4697

A.11.6.3.2.2	Test Parameters	4697

A.11.6.3.2.3	Test Requirements	4698

A.11.6.3.3	Intra-frequency measurement accuracy on SCC	4698

A.11.6.3.3.1	Test Purpose and Environment	4698

A.11.6.3.3.2	Test Parameters	4699

A.11.6.3.3.3	Test Requirements	4700

A.11.6.3.4	Inter-frequency measurement accuracy	4700

A.11.6.3.4.1	Test Purpose and Environment	4700

A.11.6.3.4.2	Test Parameters	4700

A.11.6.3.4.3	Test Requirements	4704

A.11.6.4	L1-RSRP measurement for beam reporting with CCA serving cell	4705

A.11.6.4.1	SSB based L1-RSRP measurement	4705

A.11.6.4.1.1	Test Purpose and Environment	4705

A.11.6.4.1.2	Test parameters	4705

A.11.6.4.1.3	Test Requirements	4706

A.11.6.5	RSSI	4706

A.11.6.5.1	Intra-frequency RSSI measurement accuracy on PCC with CCA	4706

A.11.6.5.1.1	Test Purpose and Environment	4706

A.11.6.5.1.2	Test parameters	4706

A.11.6.5.1.3	Test Requirements	4708

A.11.6.5.2	Intra-frequency RSSI measurement accuracy on SCC with CCA	4708

A.11.6.5.2.1	Test Purpose and Environment	4708

A.11.6.5.2.2	Test parameters	4708

A.11.6.5.2.3	Test Requirements	4710

A.11.6.5.3	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4710

A.11.6.5.3.1	Test Purpose and Environment	4710

A.11.6.5.3.2	Test parameters	4710

A.11.6.5.3.3	Test Requirements	4711

A.11.6.6	Channel occupancy	4712

A.11.6.6.1	Intra-frequency channel occupancy measurement accuracy on PCC with CCA	4712

A.11.6.6.1.1	Test Purpose and Environment	4712

A.11.6.6.1.2	Test parameters	4712

A.11.6.6.1.3	Test Requirements	4713

A.11.6.6.2	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	4713

A.11.6.6.2.1	Test Purpose and Environment	4713

A.11.6.6.2.2	Test parameters	4713

A.11.6.6.2.3	Test Requirements	4715

A.11.6.6.3	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4715

A.11.6.6.3.1	Test Purpose and Environment	4715

A.11.6.6.3.2	Test parameters	4715

A.11.6.6.3.3	Test Requirements	4717

A.11.6.7	E-UTRAN RSRP	4717

A.11.6.8	E-UTRAN RSRQ	4717

A.11.6.9	E-UTRAN SINR	4717

A.12	E-UTRA Standalone Tests with at Least One NR Cell under CCA	4717

A.12.1	RRC_IDLE state mobility	4717

A.12.1.1	Inter-RAT cell re-selection to NR on a carrier frequency with CCA	4717

A.12.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA	4717

A.12.1.1.1.1	Test Purpose and Environment	4717

A.12.1.1.1.2	Test Requirements	4720

A.12.2	RRC_CONNECTED state mobility	4721

A.12.2.1	Handover	4721

A.12.2.1.1	E-UTRAN - NR with CCA handover	4721

A.12.2.1.1.1	Test Purpose and Environment	4721

A.12.2.1.1.2	Test Requirements	4724

A.12.3	Void	4725

A.12.4	Measurement procedure	4725

A.12.4.1	E-UTRANNR inter-RAT SFTD measurements	4725

A.12.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX	4725

A.12.4.1.1.1	Test Purpose and Environment	4725

A.12.4.1.1.2	Test Requirements	4727

A.12.4.2	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	4727

A.12.4.2.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4727

A.12.4.2.1.1	Test Purpose and Environment	4727

A.12.4.2.1.2	Test Requirements	4731

A.12.4.2.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4731

A.12.4.2.2.1	Test Purpose and Environment	4731

A.12.4.2.2.2	Test Requirements	4734

A.12.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4734

A.12.4.2.3.1	Test Purpose and Environment	4734

A.12.4.2.3.2	Test Requirements	4738

A.12.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4738

A.12.4.2.4.1	Test Purpose and Environment	4738

A.12.4.2.4.2	Test Requirements	4741

A.12.4.2.5	Void	4742

A.12.4.2.6	Void	4742

A.12.5	Measurement performance	4742

A.12.5.1	E-UTRANNR SFTD	4742

A.12.5.1.1	Inter-RAT SFTD accuracy with NR target cell under CCA	4742

A.12.5.1.1.1	Test Purpose	4742

A.12.5.1.1.2	Test Environment	4742

A.12.5.1.1.3	Test Requirements	4744

A.12.5.2	Void	4744

A.12.5.3	Void	4744

A.12.5.4	Void	4744

A.12.5.5	Void	4745

A.12.5.6	Void	4745

A.13	NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	4748

A.13.1	Void	4748

A.13.1.1	Void	4748

A.13.1.2	Void	4748

A.13.2	Signalling characteristics	4748

A.13.2.1	Void	4748

A.13.2.2	SCell activation and deactivation delay	4748

A.13.2.2.1	SCell Activation and Deactivation of known SCell under CCA, 160 ms SCell measurement cycle	4748

A.13.2.2.1.1	Test Purpose and Environment	4748

A.13.2.2.1.2	Test Requirements	4751

A.13.2.2.2 SCell Activation and Deactivation of known SCell under CCA, 640 ms SCell measurement cycle	4751

A.13.2.2.2.1	Test Purpose and Environment	4751

A.13.2.2.2.2	Test Requirements	4752

A.13.2.2.3	SCell Activation and Deactivation of unknown SCell under CCA	4752

A.13.2.2.3.1	Test Purpose and Environment	4752

A.13.2.2.3.2	Test Requirements	4752

A.13.2.3	Void	4753

A.13.3	Measurement procedure	4753

A.13.3.1	Intra-frequency measurements	4753

A.13.3.1.1	Event-triggered reporting tests on SCC without gaps under non-DRX	4753

A.13.3.1.1.1	Test purpose and environment	4753

A.13.3.1.1.2	Test parameters	4753

A.13.3.1.1.3	Test Requirements	4756

A.13.3.1.2	Event-triggered reporting tests on SCC without gaps under DRX	4756

A.13.3.1.2.1	Test purpose and environment	4756

A.13.3.1.2.2	Test parameters	4756

A.13.3.1.2.3	Test Requirements	4759

A.13.3.1.3	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	4759

A.13.3.1.3.1	Test purpose and environment	4759

A.13.3.1.3.2	Test parameters	4759

A.13.3.1.3.3	Test Requirements	4762

A.13.3.1.4	Event-triggered reporting tests on SCC with per-UE gaps under DRX	4763

A.13.3.1.4.1	Test purpose and environment	4763

A.13.3.1.4.2	Test parameters	4763

A.13.3.1.4.3	Test Requirements	4766

A.13.3.1.5	Void	4766

A.13.3.1.6	Void	4766

A.13.3.2	Inter-frequency measurements	4766

A.13.3.2.1	Void	4766

A.13.3.2.2	Void	4766

A.13.3.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	4766

A.13.3.2.3.1	Test Purpose and Environment	4766

A.13.3.2.3.2	Test Requirements	4769

A.13.3.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	4770

A.13.3.2.4.1	Test Purpose and Environment	4770

A.13.3.2.4.2	Test Requirements	4773

A.13.3.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	4773

A.13.3.2.5.1	Test Purpose and Environment	4773

A.13.3.2.5.2	Test Requirements	4776

A.13.3.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	4776

A.13.3.2.6.1	Test Purpose and Environment	4776

A.13.3.2.6.2	Test Requirements	4780

A.13.3.3	L1-RSRP measurements for beam reporting	4780

A.13.3.3.1	SSB based L1-RSRP measurement when DRX is not used	4780

A.13.3.3.1.1	Test Purpose and Environment	4780

A.13.3.3.1.2	Test parameters	4780

A.13.3.3.1.3	Test Requirements	4782

A.13.3.3.2	SSB based L1-RSRP measurement when DRX is used	4783

A.13.3.3.2.1	Test Purpose and Environment	4783

A.13.3.3.2.2	Test parameters	4783

A.13.3.3.2.3	Test Requirements	4785

A.13.4	Measurement performance	4785

A.13.4.1	SS-RSRP	4785

A.13.4.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	4785

A.13.4.1.1.1	Test Purpose and Environment	4785

A.13.4.1.1.2	Test parameters	4785

A.13.4.1.1.3	Test Requirements	4787

A.13.4.2	SS-RSRQ	4787

A.13.4.2.1	Intra-frequency measurement accuracy on SCC	4787

A.13.4.2.1.1	Test Purpose and Environment	4787

A.13.4.2.1.2	Test Parameters	4787

A.13.4.2.1.3	Test Requirements	4792

A.13.4.3	SS-SINR	4792

A.13.4.3.1	Intra-frequency measurement accuracy on SCC	4792

A.13.4.3.1.1	Test Purpose and Environment	4792

A.13.4.3.1.2	Test Parameters	4792

A.13.4.3.1.3	Test Requirements	4796

A.13.4.4	L1-RSRP measurement for beam reporting with CCA serving cell	4796

A.13.4.4.1	SSB based L1-RSRP measurement	4796

A.13.4.4.1.1	Test Purpose and Environment	4796

A.13.4.4.1.2	Test parameters	4797

A.13.4.4.1.3	Test Requirements	4799

A.13.4.5	RSSI	4799

A.13.4.5.1 	Intra-frequency RSSI measurement accuracy on a carrier with CCA	4799

A.13.4.5.1.1	Test Purpose and Environment	4799

A.13.4.5.1.2	Test parameters	4799

A.13.4.5.1.3	Test Requirements	4801

A.13.4.5.2	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4801

A.13.4.5.2.1	Test Purpose and Environment	4801

A.13.4.5.2.2	Test parameters	4801

A.13.4.5.2.3	Test Requirements	4803

A.13.4.6	Channel occupancy	4803

A.13.4.6.1	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	4803

A.13.4.6.1.1	Test Purpose and Environment	4803

A.13.4.6.1.2	Test parameters	4803

A.13.4.6.1.3	Test Requirements	4805

A.13.4.6.2	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4805

A.13.4.6.2.1	Test Purpose and Environment	4805

A.13.4.6.2.2	Test parameters	4805

A.13.4.6.2.3	Test Requirements	4807

A.14	NR standalone tests for Satellite access	4807

A.14.1	RRC_IDLE state mobility	4807

A.14.1.1	Cell reselection to FR1 intra-frequency NR case	4807

A.14.1.1.1	Test Purpose and Environment	4807

A.14.1.1.2	Test Parameters	4807

A.14.1.1.3	Test Requirements	4809

A.14.1.2	Cell reselection to FR1 intra-frequency NR cell for UE configured with the feature for enhanced requirements	4809

A.14.1.2.1	Test Purpose and Environment	4809

A.14.1.2.2	Test Parameters	4810

A.14.1.2.3	Test Requirements	4811

A.14.1.3	Time-based measurement initiation to FR1 intra-frequency NR cell reselection	4812

A.14.1.3.1	Test Purpose and Environment	4812

A.14.1.3.2	Test Parameters	4812

A.14.1.3.3	Test Requirements	4813

A.14.1.4	Location-based measurement initiation to FR1 intra-frequency NR cell reselection	4813

A.14.1.4.1	Test Purpose and Environment	4814

A.14.1.4.2	Test Parameters	4814

A.14.1.4.3	Test Requirements	4815

A.14.1.5	Cell reselection to FR1 inter-frequency NR case	4815

A.14.1.5.1	Test Purpose and Environment	4815

A.14.1.5.2	Test Parameters	4816

A.14.1.5.3	Test Requirements	4817

A.14.1.6	Cell re-selection to FR1 inter-frequency NR cell for UE configured with feature for enhanced requirements	4818

A.14.1.6.1	Test Purpose and Environment	4818

A.14.1.6.2	Test Parameters	4818

A.14.1.6.3	Test Requirements	4819

A.14.1.7	Time-based measurement initiation to FR1 inter-frequency cell reselection	4820

A.14.1.7.1	Test Purpose and Environment	4820

A.14.1.7.2	Test Parameters	4820

A.14.1.7.3	Test Requirements	4821

A.14.1.8	Location-based measurement initiation to FR1 inter-frequency NR cell reselection	4822

A.14.1.8.1	Test Purpose and Environment	4822

A.14.1.8.2	Test Parameters	4822

A.14.1.8.3	Test Requirements	4823

A.14.1.9	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	4823

A.14.1.9.1	Test Purpose and Environment	4823

A.14.1.9.2	Test Parameters	4824

A.14.1.9.3	Test Requirements	4825

A.14.1.10	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	4826

A.14.1.10.1	Test Purpose and Environment	4826

A.14.1.10.2	Test Parameters	4826

A.14.1.10.3	Test Requirements	4828

A.14.1.11	Cell reselection to FR1 inter-RAT for NR NTN carrier	4828

A.14.1.11.1	Test purpose and Environment	4828

A.14.1.11.2	Test parameters	4828

A.14.1.11.3	Test requirements	4830

A.14.1.12	Cell re-selection to FR1 inter-frequency NR case with TN carrier	4831

A.14.1.12.1	Test purpose and Environment	4831

A.14.1.12.2	Test parameters	4831

A.14.1.12.3	Test requirements	4833

A.14.2	RRC_CONNECTED state mobility	4833

A.14.2.1	Handover	4833

A.14.2.1.1	Intra-frequency SAN Handover from FR1 to FR1	4833

A.14.2.1.1.1	Test Purpose and Environment	4833

A.14.2.1.1.2	Test Parameters	4833

A.14.2.1.1.3	Test Requirements	4835

A.14.2.1.2	Inter-frequency SAN Handover from FR1 to FR1	4835

A.14.2.1.2.1	Test Purpose and Environment	4835

A.14.2.1.2.2	Test Parameters	4835

A.14.2.1.2.3	Test Requirements	4837

A.14.2.1.3	Intra-frequency SAN time-based conditional Handover from FR1 to FR1	4837

A.14.2.1.3.1	Test Purpose and Environment	4837

A.14.2.1.3.2	Test Parameters	4838

A.14.2.1.3.3	Test Requirements	4839

A.14.2.1.4	Inter-frequency SAN time-based conditional Handover from FR1 to FR1	4840

A.14.2.1.4.1	Test Purpose and Environment	4840

A.14.2.1.4.2	Test Parameters	4840

A.14.2.1.4.3	Test Requirements	4841

A.14.2.1.5	Intra-frequency SAN distance-based conditional Handover from FR1 to FR1	4842

A.14.2.1.5.1	Test Purpose and Environment	4842

A.14.2.1.5.2	Test Parameters	4842

A.14.2.1.5.3	Test Requirements	4844

A.14.2.1.6	Inter-frequency SAN distance-based conditional Handover from FR1 to FR1	4844

A.14.2.1.6.1	Test Purpose and Environment	4844

A.14.2.1.6.2	Test Parameters	4844

A.14.2.1.6.3	Test Requirements	4846

A.14.2.1.7	Intra-frequency intra-satellite Handover from FR2-NTN to FR2-NTN	4847

A.14.2.1.7.1	Test Purpose and Environment	4847

A.14.2.1.7.2	Test Parameters	4847

A.14.2.1.7.3	Test Requirements	4848

A.14.2.1.8	Intra-frequency SAN Handover from FR1 to FR1	4849

A.14.2.1.8.1	Test Purpose and Environment	4849

A.14.2.1.8.2	Test Parameters	4849

A.14.2.1.8.3	Test Requirements	4851

A.14.2.1.9	Intra-frequency inter-satellite handover from FR2-NTN to FR2-NTN	4851

A.14.2.1.9.1	Test Purpose and Environment	4851

A.14.2.1.9.2	Test Parameters	4851

A.14.2.1.9.3	Test Requirements	4853

A.14.2.2	RRC Connection Mobility Control	4853

A.14.2.2.1	SA: RRC Re-establishment for SAN	4853

A.14.2.2.1.1	Intra-frequency RRC Re-establishment in FR1	4853

A.14.2.2.1.2	Inter-frequency RRC Re-establishment in FR1	4855

A.14.2.2.2	Random Access	4858

A.14.2.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	4858

A.14.2.2.2.1.1	Test Purpose and Environment	4858

A.14.2.2.2.1.2	Test Requirements	4859

A.14.2.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	4860

A.14.2.2.2.2.1	Test Purpose and Environment	4860

A.14.2.2.2.2.2	Test Requirements	4862

A.14.2.2.3	RRC Connection Release with Redirection	4863

A.14.2.2.3.1	Redirection from NR in FR1 to NR in FR1	4863

A.14.2.2.3.1.1	Test Purpose and Environment	4863

A.14.2.2.3.1.2	Test Parameters	4863

A.14.2.2.3.1.3	Test Requirements	4865

A.14.2.2.4	RACH-based Hard Satellite switching with re-synchronization from FR1 to FR1	4865

A.14.2.2.4.1	Test Purpose and Environment	4865

A.14.2.2.4.2	Test Parameters	4865

A.14.2.2.4.3	Test Requirements	4867

A.14.2.2.5	RACH-less Soft Satellite switching with re-synchronization from FR1 to FR1	4867

A.14.2.2.5.1	Test Purpose and Environment	4867

A.14.2.2.5.2	Test Parameters	4867

A.14.2.2.5.3	Test Requirements	4869

A.14.2.3	Intra-frequency SAN time-based conditional Handover without L3 measurement criteria from FR1 to FR1	4870

A.14.2.3.1	Test Purpose and Environment	4870

A.14.2.3.2	Test Parameters	4870

A.14.2.3.3	Test Requirements	4871

A.14.2.4	Inter-frequency SAN time-based conditional Handover without L3 measurement criteria from FR1 to FR1	4872

A.14.2.4.1	Test Purpose and Environment	4872

A.14.2.4.2	Test Parameters	4872

A.14.2.4.3	Test Requirements	4873

A.14.3	Timing for Satellite Access	4874

A.14.3.1	UE transmit timing for Satellite Access	4874

A.14.3.1.1	NR UE Transmit Timing Test for FR1	4874

A.14.3.1.1.1	Test Purpose and environment	4874

A.14.3.1.1.2	Test requirements	4875

A.14.3.1.2	NR UE Transmit Timing Test for FR2-NTN	4876

A.14.3.1.2.1	Test Purpose and environment	4876

A.14.3.1.2.2	Test requirements	4879

A.14.3.2	Timing advance for satellite access	4879

A.14.3.2.1	SA FR1 timing advance adjustment accuracy	4879

A.14.3.2.1.1	Test Purpose and Environment	4879

A.14.3.2.1.2	Test Parameters	4879

A.14.3.2.1.3	Test Requirements	4882

A.14.3.2.3	SA FR2-NTN timing advance adjustment accuracy	4882

A.14.3.2.3.1	Test Purpose and Environment	4882

A.14.3.2.3.2	Test Parameters	4882

A.14.3.2.1.3	Test Requirements	4885

A.14.4	Signalling characteristics	4885

A.14.4.1	Radio link Monitoring	4885

A.14.4.1.1	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode	4885

A.14.4.1.1.1	Test Purpose and Environment	4885

A.14.4.1.1.2	Test Requirements	4887

A.14.4.1.2	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode	4888

A.14.4.1.2.1	Test Purpose and Environment	4888

A.14.4.1.2.2	Test Requirements	4890

A.14.4.1.3	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode	4890

A.14.4.1.3.1	Test Purpose and Environment	4890

A.14.4.1.3.2	Test Requirements	4892

A.14.4.1.4	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode	4893

A.14.4.1.4.1	Test Purpose and Environment	4893

A.14.4.1.4.2	Test Requirements	4895

A.14.4.1.5	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode	4895

A.14.4.1.5.1	Test Purpose and Environment	4895

A.14.4.1.5.2	Test Requirements	4898

A.14.4.1.6	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode	4898

A.14.4.1.6.1	Test Purpose and Environment	4898

A.14.4.1.6.2	Test Requirements	4901

A.14.4.1.7	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode	4901

A.14.4.1.7.1	Test Purpose and Environment	4901

A.14.4.1.7.2	Test Requirements	4904

A.14.4.1.8	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode	4904

A.14.4.1.8.1	Test Purpose and Environment	4904

A.14.4.1.8.2	Test Requirements	4907

A.14.4.1.9	Radio Link Monitoring Out-of-sync Test for FR2 SAN PCell configured with SSB-based RLM RS in non-DRX mode	4907

A.14.4.1.9.1	Test Purpose and Environment	4907

A.14.4.1.9.2	Test Requirements	4909

A.14.4.1.10	Radio Link Monitoring In-sync Test for FR2 SAN PCell configured with SSB-based RLM RS in non-DRX mode	4910

A.14.4.1.10.1	Test Purpose and Environment	4910

A.14.4.1.10.2	Test Requirements	4912

A.14.4.2	Beam Failure Detection and Link recovery procedures for satellite access	4913

A.14.4.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in non-DRX mode	4913

A.14.4.2.1.1	Test Purpose and Environment	4913

A.14.4.2.1.2	Test Requirements	4915

A.14.4.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in DRX mode	4916

A.14.4.2.2.1	Test Purpose and Environment	4916

A.14.4.2.2.2	Test Requirements	4918

A.14.4.2.3	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in non-DRX mode	4919

A.14.4.2.3.1	Test Purpose and Environment	4919

A.14.4.2.3.2	Test Requirements	4921

A.14.4.2.4	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in DRX mode	4922

A.14.4.2.4.1	Test Purpose and Environment	4922

A.14.4.2.4.2	Test Requirements	4924

A.14.4.2.5	Void	4925

A.14.4.2.6	Void	4925

A.14.4.3	Active BWP switch for satellite access	4925

A.14.4.3.1	DCI-based and Timer-based Active BWP Switch	4925

A.14.4.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA	4925

A.14.4.3.1.1.1	Test Purpose and Environment	4925

A.14.4.3.1.1.2	Test Requirements	4927

A.14.4.3.2	RRC-based Active BWP Switch	4927

A.14.4.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	4927

A.14.4.3.2.1.1	Test Purpose and Environment	4927

A.14.4.3.2.1.2	Test Requirements	4929

A.14.4.4	UE specific CBW change for satellite access	4929

A.14.4.4.1	UE specific CBW change on PCell in FR1 in non-DRX	4929

A.14.4.4.1.1	Test Purpose and Environment	4929

A.14.4.4.1.2	Test Requirements	4931

A.14.4.5	Pathloss reference signal switching delay	4932

A.14.4.5.1	MAC-CE based pathloss reference signal switch delay	4932

A.14.4.5.1.1	Test Purpose and Environment	4932

A.14.4.5.1.2	Test Requirements	4934

A.14.5	Measurement procedure	4934

A.14.5.1	Intra-frequency Measurements	4934

A.14.5.1.1	SA event triggered reporting tests without gap under non-DRX	4934

A.14.5.1.1.1	Test purpose and Environment	4934

A.14.5.1.1.2	Test parameters	4934

A.14.5.1.1.3	Test Requirements	4936

A.14.5.1.2	SA event triggered reporting tests without gap under DRX	4936

A.14.5.1.2.1	Test purpose and Environment	4936

A.14.5.1.2.2	Test parameters	4936

A.14.5.1.2.3	Test Requirements	4938

A.14.5.1.3	SA event triggered reporting tests without gap under non-DRX with SSB index reading	4938

A.14.5.1.3.1	Test purpose and Environment	4938

A.14.5.1.3.2	Test parameters	4938

A.14.5.1.3.3	Test Requirements	4940

A.14.5.1.4	SA event triggered reporting tests with single measurement gap under non-DRX for satellite access	4940

A.14.5.1.4.1	Test purpose and Environment	4940

A.14.5.1.4.2	Test parameters	4940

A.14.5.1.4.3	Test Requirements	4942

A.14.5.1.5	SA event triggered reporting tests with FNO concurrent gaps under DRX for satellite access	4942

A.14.5.1.5.1	Test purpose and Environment	4942

A.14.5.1.5.2	Test parameters	4942

A.15.5.1.5.3	Test Requirements	4944

A.14.5.1.6	SA event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access	4944

A.14.5.1.6.1	Test purpose and Environment	4944

A.14.5.1.6.2	Test parameters	4944

A.14.5.1.6.3	Test Requirements	4946

A.14.5.1.7	SA event triggered reporting test with SSB time index reading without gap under non-DRX for FR2-NTN	4946

A.14.5.1.7.1	Test purpose and Environment	4946

A.14.5.1.7.2	Test parameters	4946

A.14.5.1.7.3	Test Requirements	4948

A.14.5.2	Inter-frequency Measurements	4948

A.14.5.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with single gap for satellite access	4948

A.14.5.2.1.1	Test Purpose and Environment	4948

A.14.5.2.1.2	Test Requirements	4950

A.14.5.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used with single gap for satellite access	4950

A.14.5.2.2.1	Test Purpose and Environment	4950

A.14.5.2.2.2	Test Requirements	4953

A.14.5.2.3	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used with single gap for satellite access	4953

A.14.5.2.3.1	Test Purpose and Environment	4953

A.14.5.2.3.2	Test Requirements	4955

A.14.5.2.4	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in fully non-overlapped for satellite access	4955

A.14.5.2.4.1	Test Purpose and Environment	4955

A.14.5.2.4.2	Test Requirements	4957

A.14.5.2.5	void	4957

A.14.5.2.5.1	void	4957

A.14.5.2.5.2	void	4958

A.14.5.2.6	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in partially partial overalpping for satellite access	4958

A.14.5.2.6.1	Test Purpose and Environment	4958

A.14.5.2.6.2	Test Requirements	4960

A.14.5.2.7	Event triggered reporting test without gap under non-DRX	4960

A.14.5.2.7.1	Test purpose and Environment	4960

A.14.5.2.7.2	Test parameters	4960

A.14.5.2.7.3	Test Requirements	4961

A.14.5.2.8	Event triggered reporting tests without gap under DRX	4962

A.14.5.2.8.1	Test purpose and Environment	4962

A.14.5.2.8.2	Test parameters	4962

A.14.5.2.8.3	Test Requirements	4963

A.14.5.3	L1-RSRP measurement for beam reporting for satellite access	4963

A.14.5.3.1	SSB based L1-RSRP measurement for satellite access when DRX is not used	4963

A.14.5.3.1.1	Test Purpose and Environment	4964

A.14.5.3.1.2	Test parameters	4964

A.14.5.3.1.3	Test Requirements	4965

A.14.5.3.2	SSB based L1-RSRP measurement for satellite access when DRX is used	4965

A.14.5.3.2.1	Test Purpose and Environment	4965

A.14.5.3.2.2	Test parameters	4966

A.14.5.3.2.3	Test Requirements	4967

A.14.5.3.3	CSI-RS based L1-RSRP measurement for satellite access when DRX is not used	4967

A.14.5.3.3.1	Test Purpose and Environment	4967

A.14.5.3.3.2	Test parameters	4968

A.14.5.3.3.3	Test Requirements	4969

A.14.5.3.4	CSI-RS based L1-RSRP measurement for satellite access when DRX is used	4969

A.14.5.3.4.1	Test Purpose and Environment	4969

A.14.5.3.4.2	Test parameters	4969

A.14.5.3.4.3	Test Requirements	4971

A.14.5.3.5	SSB based L1-RSRP measurement when DRX is not used in FR2-NTN	4971

A.14.5.3.5.1	Test Purpose and Environment	4971

A.14.5.3.5.2	Test parameters	4971

A.14.5.3.5.3	Test Requirements	4973

A.14.6	Measurement Performance requirements	4973

A.14.6.1	SS-RSRP for SAN	4973

A.14.6.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	4973

A.14.6.1.1.1	Test Purpose and Environment	4973

A.14.6.1.1.2	Test parameters	4973

A.14.6.1.1.3	Test Requirements	4975

A.14.6.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	4975

A.14.6.1.2.1	Test Purpose and Environment	4975

A.14.6.1.2.2	Test parameters	4975

A.14.6.1.2.3	Test Requirements	4976

A.14.6.1.3	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	4976

A.14.6.1.3.1	Test Purpose and Environment	4976

A.14.6.1.3.2	Test parameters	4976

A.14.6.1.3.3	Test Requirements	4978

A.14.6.2	SS-RSRQ	4979

A.14.6.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access	4979

A.14.6.2.1.1	Test Purpose and Environment	4979

A.14.6.2.1.2	Test Parameters	4979

A.14.6.2.1.3	Test Requirements	4980

A.14.6.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access	4980

A.14.6.2.2.1	Test Purpose and Environment	4980

A.14.6.2.2.2	Test Parameters	4980

A.14.6.2.2.3	Test Requirements	4981

A.14.6.3	SS-SINR	4982

A.14.6.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4982

A.14.6.3.1.1	Test Purpose and Environment	4982

A.14.6.3.1.2	Test Parameters	4982

A.14.6.3.1.3	Test Requirements	4983

A.14.6.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4983

A.14.6.3.2.1	Test Purpose and Environment	4983

A.14.6.3.2.2	Test Parameters	4983

A.14.6.3.2.3	Test Requirements	4984

A.14.6.4	L1-RSRP measurement for beam reporting	4984

A.14.6.4.1	SSB based L1-RSRP measurement	4984

A.14.6.4.1.1	Test Purpose and Environment	4985

A.14.6.4.1.2	Test parameters	4985

A.14.6.4.1.3	Test Requirements	4986

A.14.6.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	4986

A.14.6.4.2.1	Test Purpose and Environment	4986

A.14.6.4.2.2	Test parameters	4986

A.14.6.4.2.3	Test Requirements	4987

A.14.6.4.3	SSB based L1-RSRP measurement for VSAT UE in FR2-NTN when DRX is not used	4987

A.14.6.4.3.1	Test Purpose and Environment	4987

A.14.6.4.3.2	Test parameters	4988

A.14.6.4.3.3	Test Requirements	4989

A.15	NR standalone tests with one or more NR cells in FR2-2	4992

A.15.1	SA: RRC_IDLE state mobility	4992

A.15.1.1	Cell re-selection to NR	4992

A.15.1.1.1	Cell re-selection to FR2-2 intra-frequency NR case	4992

A.15.1.1.1.1	Test Purpose and Environment	4992

A.15.1.1.1.2	Test Parameters	4992

A.15.1.1.1.3	Test Requirements	4994

A.15.1.2	Cell re-selection to FR2-2 inter-frequency NR case	4994

A.15.1.2.1	Test Purpose and Environment	4994

A.15.1.2.2	Test Parameters	4995

A.15.1.2.3	Test Requirements	4997

A.15.1.3	Cell re-selection to FR2-2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	4997

A.15.1.3.1	Test Purpose and Environment	4997

A.15.1.3.2	Test Parameters	4997

A.15.1.3.3	Test Requirements	4999

A.15.1.4	Cell re-selection to FR2-2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	5000

A.15.1.4.1	Test Purpose and Environment	5000

A.15.1.4.2	Test Parameters	5000

A.15.1.4.3	Test Requirements	5002

A.15.1.5	Cell re-selection to FR2-2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	5002

A.15.1.5.1	Test Purpose and Environment	5002

A.15.1.5.2	Test Parameters	5002

A.15.1.5.3	Test Requirements	5004

A.15.1.6	Cell re-selection to FR2-2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	5005

A.15.1.6.1	Test Purpose and Environment	5005

A.15.1.6.2	Test Parameters	5005

A.15.1.6.3	Test Requirements	5007

A.15.2	Signaling characteristics	5007

A.15.2.1	SCell Activation and Deactivation Delay	5007

A.15.2.1.1	SCell Activation and deactivation for SCell in FR2-2 intra-band in non-DRX	5007

A.15.2.1.1.1	Test Purpose and Environment	5007

A.15.2.1.1.2	Test Requirements	5009

A.15.2.1.2	SCell Activation and deactivation for FR1+FR2-2 inter-band with target SCell in FR2-2	5009

A.15.2.1.2.1	Test Purpose and Environment	5009

A.15.2.1.2.2	Test Requirements	5012

A.15.2.1.3	SCell Activation and deactivation for SCell in FR2-2 inter-band in non-DRX	5012

A.15.2.1.3.1	Test Purpose and Environment	5013

A.15.2.1.3.2	Test Requirements	5015

A.15.2.1.4	Direct SCell activation at SCell addition of known SCell in FR2-2	5016

A.15.2.1.4.1	Test Purpose and Environment	5016

A.15.2.1.4.2	Test Requirements	5018

A.15.2.1.5	Direct SCell activation at handover with known SCell in FR2-2	5019

A.15.2.1.5.1	Test Purpose and Environment	5019

A.15.2.1.5.2	Test Requirements	5021

A.15.3	RRC_CONNECTED state mobility	5022

A.15.3.1	Handover	5022

A.15.3.1.1	Intra-frequency handover from FR2-2 carrier with CCA to FR2-2 carrier with CCA; unknown target cell	5022

A.15.3.1.1.1	Test Purpose and Environment	5022

A.15.3.1.1.2	Test Parameters	5022

A.15.3.1.1.3	Test Requirements	5024

A.15.3.1.2	Inter-frequency handover from FR1 to FR2-2 carrier with CCA; unknown target cell	5025

A.15.3.1.2.1	Test Purpose and Environment	5025

A.15.3.1.2.2	Test Parameters	5025

A.15.3.1.2.3	Test Requirements	5027

A.15.4	Measurement procedure	5027

A.15.4.1	Intra-frequency Measurements	5027

A.15.4.1.1	SA event triggered reporting test without gap under non-DRX for FR2-2 with CCA	5027

A.15.4.1.1.1	Test purpose and Environment	5027

A.15.4.1.1.2	Test Requirements	5030

A.15.4.2	Inter-frequency Measurements	5031

A.15.4.2.1	SA event triggered reporting tests for FR2-2 with CCA without SSB time index detection when DRX is not used (PCell in FR2-2)	5031

A.15.4.2.1.1	Test Purpose and Environment	5031

A.15.4.2.1.2	Test Requirements	5033

A.16	NR standalone tests with all NR cells in FR1 for RedCap	5034

A.16.1	SA: RRC_IDLE state mobility for RedCap	5034

A.16.1.1	Cell re-selection to NR	5034

A.16.1.1.1	Cell re-selection to FR1 intra-frequency NR case for 1 Rx UE	5034

A.16.1.1.1.1	Test Purpose and Environment	5034

A.16.1.1.1.2	Test Parameters	5034

A.16.1.1.1.3	Test Requirements	5036

A.16.1.1.2	Cell re-selection to FR1 intra-frequency NR case for 2 Rx UE	5037

A.16.1.1.2.1	Test Purpose and Environment	5037

A.16.1.1.2.2	Test Parameters	5037

A.16.1.1.2.3	Test Requirements	5039

A.16.1.1.3	Cell re-selection to FR1 inter-frequency NR case for 1 Rx UE	5039

A.16.1.1.3.1	Test Purpose and Environment	5039

A.16.1.1.3.2	Test Parameters	5039

A.16.1.1.3.3	Test Requirements	5042

A.16.1.1.4	Cell re-selection to FR1 inter-frequency NR case for 2 Rx UE	5042

A.16.1.1.4.1	Test Purpose and Environment	5042

A.16.1.1.4.2	Test Parameters	5042

A.16.1.1.4.3	Test Requirements	5045

A.16.1.1.5	Cell re-selection to FR1 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5045

A.16.1.1.5.1	Test Purpose and Environment	5045

A.16.1.1.5.2	Test Parameters	5045

A.16.1.1.5.3	Test Requirements	5048

A.16.1.1.6	Cell re-selection to FR1 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5048

A.16.1.1.6.1	Test Purpose and Environment	5048

A.16.1.1.6.2	Test Parameters	5048

A.16.1.1.6.3	Test Requirements	5050

A.16.1.1.7	Cell re-selection to FR1 inter-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5051

A.16.1.1.7.1	Test Purpose and Environment	5051

A.16.1.1.7.2	Test Parameters	5051

A.16.1.1.7.3	Test Requirements	5053

A.16.1.1.8	Cell re-selection to FR1 inter-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5054

A.16.1.1.8.1	Test Purpose and Environment	5054

A.16.1.1.8.2	Test Parameters	5054

A.16.1.1.8.3	Test Requirements	5056

A.16.1.2	Inter-RAT E-UTRAN cell re-selection for RedCap	5057

A.16.1.2.1	Cell re-selection to higher priority E-UTRAN for 1 RX	5057

A.16.1.2.1.1	Test Purpose and Environment	5057

A.16.1.2.1.2	Test Parameters	5057

A.16.1.2.1.3	Test Requirements	5060

A.16.1.2.2	Cell re-selection to higher priority E-UTRAN for 2 RX	5060

A.16.1.2.2.1	Test Purpose and Environment	5060

A.16.1.2.2.2	Test Parameters	5060

A.16.1.2.2.3	Test Requirements	5063

A.16.1.2.3.1	Test Purpose and Environment	5063

A. 16.1.2.3.2	Test Parameters	5063

A.16.1.2.3.3	Test Requirements	5066

A.16.1.2.4.1	Test Purpose and Environment	5066

A.16.1.2.4.2	Test Parameters	5066

A.16.1.3.1.3	Test Requirements	5069

A.16.1.2.5	Cell re-selection to lower priority E-UTRAN for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5069

A.16.1.2.5.1	Test Purpose and Environment	5069

A.16.1.2.5.2	Test Parameters	5069

A.16.1.2.5.3	Test Requirements	5072

A.16.1.2.6	Cell re-selection to lower priority E-UTRAN for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5072

A.16.1.2.6.1	Test Purpose and Environment	5072

A.16.1.2.6.2	Test Parameters	5073

A.16.1.2.6.3	Test Requirements	5075

A.16.2	SA: RRC_INACTIVE state mobility for RedCap	5076

A.16.2.1	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	5076

A.16.2.1.1	NR UE CG-SDT Test in FR1 for 1 Rx RedCap UE	5076

A.16.2.1.1.1	Test purpose and Environment	5076

A.16.2.1.1.2	Test Parameters	5077

A.16.2.1.1.3	Test requirements	5079

A.16.2.1.2	NR UE CG-SDT Test in FR1 for 2 Rx RedCap UE	5079

A.16.2.1.2.1	Test purpose and Environment	5079

A.16.2.1.2.2	Test Parameters	5081

A.16.2.1.2.3	Test requirements	5082

A.16.2.2	Cell Reselection for Positioning	5083

A.16.2.2.1	Cell re-selection to FR1 intra-frequency NR case with RRC_INACTIVE eDRX and positioning SRS	5083

A.16.2.2.1.1	Test Purpose and Environment	5083

A.16.2.2.1.2	Test Parameters	5083

A.16.2.2.1.3	Test Requirements	5086

A.16.3	RRC_CONNECTED state mobility for RedCap	5086

A.16.3.1	Handover	5086

A.16.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell for 1 Rx UE	5086

A.16.3.1.1.1	Test Purpose and Environment	5086

A.16.3.1.1.2	Test Parameters	5086

A.16.3.1.1.3	Test Requirements	5088

A.16.3.1.2	Intra-frequency handover from FR1 to FR1; known target cell for 2 Rx UE	5088

A.16.3.1.2.1	Test Purpose and Environment	5088

A.16.3.1.2.2	Test Parameters	5088

A.16.3.1.2.3	Test Requirements	5090

A.16.3.1.3	Intra-frequency handover from FR1 to FR1; unknown target cell for 1 Rx UE	5090

A.16.3.1.3.1	Test Purpose and Environment	5090

A.16.3.1.3.2	Test Parameters	5091

A.16.3.1.3.3	Test Requirements	5093

A.16.3.1.4	Intra-frequency handover from FR1 to FR1; unknown target cell for 2 Rx UE	5093

A.16.3.1.4.1	Test Purpose and Environment	5093

A.16.3.1.4.2	Test Parameters	5093

A.16.3.1.5	Inter-frequency handover from FR1 to FR1; unknown target cell for 1 Rx UE	5095

A.16.3.1.5.1	Test Purpose and Environment	5095

A.16.3.1.5.2	Test Parameters	5095

A.16.3.1.5.3	Test Requirements	5097

A.16.3.1.6	Inter-frequency handover from FR1 to FR1; unknown target cell for 2 Rx UE	5098

A.16.3.1.6.1	Test Purpose and Environment	5098

A.16.3.1.6.2	Test Parameters	5098

A.16.3.1.6.3	Test Requirements	5100

A.16.3.1.7	SA NR - E-UTRAN handover for 1 Rx UE	5100

A.16.3.1.7.1	Test Purpose and Environment	5100

A.16.3.1.7.2	Test Requirements	5104

A.16.3.1.8	SA NR - E-UTRAN handover for 2 Rx UE	5104

A.16.3.1.8.1	Test Purpose and Environment	5104

A.16.3.1.8.2	Test Requirements	5107

A.16.3.1.9	SA NR - E-UTRAN handover with unknown target cell for 1 Rx UE	5107

A.16.3.1.9.1	Test Purpose and Environment	5107

A.16.3.1.9.2	Test Requirements	5110

A.16.3.1.10	SA NR - E-UTRAN handover with unknown target cell for 2 Rx UE	5110

A.16.3.1.10.1	Test Purpose and Environment	5110

A.16.3.1.10.2	Test Requirements	5114

A.16.3.2	RRC Connection Mobility Control	5114

A.16.3.2.1	SA: RRC Re-establishment	5114

A.16.3.2.1.1	Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE	5114

A.16.3.2.1.2	Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE	5117

A.16.3.2.1.3	Inter-frequency RRC Re-establishment in FR1 for 1 Rx UE	5119

A.16.3.2.1.4	Inter-frequency RRC Re-establishment in FR1 for 2 Rx UE	5122

A.16.3.2.1.5	Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE without serving cell timing	5125

A.16.3.2.1.6	Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE without serving cell timing	5128

A.16.3.2.2	Random Access	5131

A.16.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	5131

A.16.3.2.2.2	4-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	5134

A.16.3.2.2.3	4-step RA type non-contention based random access test in FR1 for NR standalone for 1 Rx UE	5137

A.16.3.2.2.4	4-step RA type non-contention based random access test in FR1 for NR standalone for 2 Rx UE	5140

A.16.3.2.2.5	2-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	5144

A.16.3.2.2.6	2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	5147

A.16.3.2.2.7	2-step RA type non-contention based test in FR1 for NR standalone for 1 RX UE	5149

A.16.3.2.2.8	2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE	5152

A.16.3.2.3	SA: RRC Connection Release with Redirection	5155

A.16.3.2.3.1	Redirection from NR in FR1 to NR in FR1 for 1 Rx UE	5155

A.16.3.2.3.2	Redirection from NR in FR1 to NR in FR1 for 2 Rx UE	5157

A.16.3.2.3.3	Redirection from NR in FR1 to E-UTRAN for 1 Rx UE	5160

A.16.3.2.3.4	Redirection from NR in FR1 to E-UTRAN for 2 Rx UE	5163

A.16.4	Timing for RedCap	5167

A.16.4.1	UE transmit timing	5167

A.16.4.1.1	NR UE Transmit Timing Test for FR1 for 1 Rx RedCap UE	5167

A.16.4.1.1.1	Test Purpose and environment	5167

A.16.4.1.1.2	Test requirements	5169

A.16.4.1.2	NR UE Transmit Timing Test for FR1 for 2 Rx RedCap UE	5169

A.16.4.1.2.1	Test Purpose and environment	5169

A.16.4.1.2.2	Test requirements	5171

A.16.4.2	Void	5171

A.16.4.3	Timing advance	5171

A.16.4.3.1	SA FR1 timing advance adjustment accuracy for 1 Rx UE	5171

A.16.4.3.1.1	Test Purpose and Environment	5171

A.16.4.3.1.2	Test Parameters	5171

A.16.4.3.1.3	Test Requirements	5173

A.16.4.3.2	SA FR1 timing advance adjustment accuracy for 2 Rx UE	5174

A.16.4.3.2.1	Test Purpose and Environment	5174

A.16.4.3.2.2	Test Parameters	5174

A.16.4.3.2.3	Test Requirements	5176

A.16.5	Signalling characteristics for RedCap	5176

A.16.5.1	Radio link Monitoring	5176

A.16.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	5176

A.16.5.1.1.1	Test Purpose and Environment	5176

A.16.5.1.1.2	Test Requirements	5179

A.16.5.1.2	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	5179

A.16.5.1.2.1	Test Purpose and Environment	5179

A.16.5.1.2.2	Test Requirements	5182

A.16.5.1.3	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	5182

A.16.5.1.3.1	Test Purpose and Environment	5182

A.16.5.1.3.2	Test Requirements	5185

A.16.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	5185

A.16.5.1.4.1	Test Purpose and Environment	5185

A.16.5.1.4.2	Test Requirements	5188

A.16.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	5188

A.16.5.1.5.1	Test Purpose and Environment	5188

A.16.5.1.5.2	Test Requirements	5191

A.16.5.1.6	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	5191

A.16.5.1.6.1	Test Purpose and Environment	5191

A.16.5.1.6.2	Test Requirements	5194

A.16.5.1.7	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	5194

A.16.5.1.7.1	Test Purpose and Environment	5194

A.16.5.1.7.2	Test Requirements	5197

A.16.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	5197

A.16.5.1.8.1	Test Purpose and Environment	5197

A.16.5.1.8.2	Test Requirements	5200

A.16.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	5200

A.16.5.1.9.1	Test Purpose and Environment	5200

A.16.5.1.9.2	Test Requirements	5203

A.16.5.1.10	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	5203

A.16.5.1.10.1	Test Purpose and Environment	5203

A.16.5.1.10.2	Test Requirements	5206

A.16.5.1.11	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	5206

A.16.5.1.11.1	Test Purpose and Environment	5206

A.16.5.1.11.2	Test Requirements	5209

A.16.5.1.12	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	5209

A.16.5.1.12.1	Test Purpose and Environment	5209

A.16.5.1.12.2	Test Requirements	5212

A.16.5.1.13	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	5213

A.16.5.1.13.1	Test Purpose and Environment	5213

A.16.5.1.13.2	Test Requirements	5215

A.16.5.1.14	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	5216

A.16.5.1.14.1	Test Purpose and Environment	5216

A.16.5.1.14.2	Test Requirements	5218

A.16.5.1.15	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	5219

A.16.5.1.15.1	Test Purpose and Environment	5219

A.16.5.1.15.2	Test Requirements	5222

A.16.5.1.16	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	5222

A.16.5.1.16.1	Test Purpose and Environment	5222

A.16.5.1.16.2	Test Requirements	5225

A.16.5.2	Beam Failure Detection and Link recovery procedures	5225

A.16.5.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 1 Rx UE	5225

A.16.5.2.1.1	Test Purpose and Environment	5225

A.16.5.2.1.2	Test Requirements	5228

A.16.5.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	5229

A.16.5.2.2.1	Test Purpose and Environment	5229

A.16.5.2.2.2	Test Requirements	5232

A.16.5.2.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 1 Rx UE	5232

A.16.5.2.3.1	Test Purpose and Environment	5232

A.16.5.2.3.2	Test Requirements	5235

A.16.5.2.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 2 Rx UE	5236

A.16.5.2.4.1	Test Purpose and Environment	5236

A.16.5.2.4.2	Test Requirements	5239

A.16.5.2.5	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 1 Rx UE	5239

A.16.5.2.5.1	Test Purpose and Environment	5239

A.16.5.2.5.2	Test Requirements	5242

A.16.5.2.6	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE	5243

A.16.5.2.6.1	Test Purpose and Environment	5243

A.16.5.2.6.2	Test Requirements	5246

A.16.5.2.7	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 1 Rx UE	5246

A.16.5.2.7.1	Test Purpose and Environment	5246

A.16.5.2.7.2	Test Requirements	5249

A.16.5.2.8	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE	5250

A.16.5.2.8.1	Test Purpose and Environment	5250

A.16.5.2.8.2	Test Requirements	5253

A.16.5.3	Active BWP switch	5253

A.16.5.3.1	DCI-based and Timer-based Active BWP Switch	5253

A.16.5.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA for 1 Rx UE	5253

A.16.5.3.1.1.1	Test Purpose and Environment	5253

A.16.5.3.1.1.2	Test Requirements	5256

A.16.5.3.1.2	NR FR1 DL active BWP switch with non-DRX in SA for 2 Rx UE	5256

A.16.5.3.1.2.1	Test Purpose and Environment	5256

A.16.5.3.1.2.2	Test Requirements	5258

A.16.5.3.2	RRC-based Active BWP Switch	5259

A.16.5.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA for 1 Rx UE	5259

A.16.5.3.2.1.1	Test Purpose and Environment	5259

A.16.5.3.2.1.2	Test Requirements	5261

A.16.5.3.2.2	NR FR1 DL active BWP switch of Cell with non-DRX in SA for 2 Rx UE	5261

A.16.5.3.2.2.1	Test Purpose and Environment	5261

A.16.5.3.2.2.2	Test Requirements	5263

A.16.5.4	UE specific CBW change	5264

A.16.5.4.1	UE specific CBW change on PCell in FR1 in non-DRX for 1 Rx UE	5264

A.16.5.4.1.1	Test Purpose and Environment	5264

A.16.5.4.1.2	Test Requirements	5266

A.16.5.4.2	UE specific CBW change on PCell in FR1 in non-DRX for 2 Rx UE	5266

A.16.5.4.2.1	Test Purpose and Environment	5266

A.16.5.4.2.2	Test Requirements	5269

A.16.6	Measurement procedure for RedCap	5269

A.16.6.1	Intra-frequency Measurements	5269

A.16.6.1.1	SA event triggered reporting tests without gap under non-DRX for 1 Rx UE	5269

A.16.6.1.1.1	Test purpose and Environment	5269

A.16.6.1.1.2	Test parameters	5269

A.16.6.1.1.3	Test Requirements	5271

A.16.6.1.2	SA event triggered reporting tests without gap under non-DRX for 2 Rx UE	5271

A.16.6.1.2.1	Test purpose and Environment	5271

A.16.6.1.2.2	Test parameters	5271

A.16.6.1.2.3	Test Requirements	5273

A.16.6.1.3	SA event triggered reporting tests without gap under DRX for 1 Rx UE	5273

A.16.6.1.3.1	Test purpose and Environment	5273

A.16.6.1.3.2	Test parameters	5273

A.16.6.1.3.3	Test Requirements	5275

A.16.6.1.4	SA event triggered reporting tests without gap under DRX for 2 Rx UE	5276

A.16.6.1.4.1	Test purpose and Environment	5276

A.16.6.1.4.2	Test parameters	5276

A.16.6.1.4.3	Test Requirements	5278

A.16.6.1.5	SA event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE	5278

A.16.6.1.5.1	Test purpose and Environment	5278

A.16.6.1.5.2	Test parameters	5278

A.16.6.1.5.3	Test Requirements	5280

A.16.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX for 2 Rx UE	5280

A.16.6.1.6.1	Test purpose and Environment	5280

A.16.6.1.6.2	Test parameters	5280

A.16.6.1.6.3	Test Requirements	5282

A.16.6.1.7	SA event triggered reporting tests with per-UE gaps under DRX for 1 Rx UE	5283

A.16.6.1.7.1	Test purpose and Environment	5283

A.16.6.1.7.2	Test parameters	5283

A.16.6.1.7.3	Test Requirements	5285

A.16.6.1.8	SA event triggered reporting tests with per-UE gaps under DRX for 2 Rx UE	5285

A.16.6.1.8.1	Test purpose and Environment	5285

A.16.6.1.8.2	Test parameters	5285

A.16.6.1.8.3	Test Requirements	5287

A.16.6.1.9	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx UE	5287

A.16.6.1.9.1	Test purpose and Environment	5287

A.16.6.1.9.2	Test parameters	5288

A.16.6.1.9.3	Test Requirements	5289

A.16.6.1.10	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx UE	5289

A.16.6.1.10.1	Test purpose and Environment	5289

A.16.6.1.10.2	Test parameters	5289

A.16.6.1.10.3	Test Requirements	5291

A.16.6.1.11	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 1 Rx UE	5291

A.16.6.1.11.1	Test purpose and Environment	5291

A.16.6.1.11.2	Test parameters	5291

A.16.6.1.11.3	Test Requirements	5293

A.16.6.1.12	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 2 Rx UE	5293

A.16.6.1.12.1	Test purpose and Environment	5293

A.16.6.1.12.2	Test parameters	5293

A.16.6.1.12.3	Test Requirements	5294

A.16.6.2	Inter-frequency Measurements	5295

A.16.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE	5295

A.16.6.2.1.1	Test Purpose and Environment	5295

A.16.6.2.1.2	Test Requirements	5297

A.16.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE	5298

A.16.6.2.2.1	Test Purpose and Environment	5298

A.16.6.2.2.2	Test Requirements	5300

A.16.6.2.3	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 1 Rx UE	5300

A.16.6.2.3.1	Test Purpose and Environment	5301

A.16.6.2.3.2	Test Requirements	5303

A.16.6.2.4	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 2 Rx UE	5303

A.16.6.2.4.1	Test Purpose and Environment	5303

A.16.6.2.4.2	Test Requirements	5305

A.16.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 1 Rx UE	5305

A.16.6.2.5.1	Test Purpose and Environment	5305

A.16.6.2.5.2	Test Requirements	5308

A.16.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 2 Rx UE	5308

A.16.6.2.6.1	Test Purpose and Environment	5308

A.16.6.2.6.2	Test Requirements	5310

A.16.6.2.7	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 1 Rx UE	5310

A.16.6.2.7.1	Test Purpose and Environment	5310

A.16.6.2.7.2	Test Requirements	5313

A.16.6.2.8	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 2 Rx UE	5313

A.16.6.2.8.1	Test Purpose and Environment	5313

A.16.6.2.8.2	Test Requirements	5315

A.16.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern for 1 Rx UE	5316

A.16.6.2.9.1	Test Purpose and Environment	5316

A.16.6.2.9.2	Test Requirements	5318

A.16.6.2.10	SA event triggered reporting tests with additional mandatory gap pattern for 2 Rx UE	5318

A.16.6.2.10.1	Test Purpose and Environment	5318

A.16.6.2.10.2	Test Requirements	5320

A.16.6.2.11	SA event triggered reporting tests for FR1 when DRX is used for 1 Rx UE	5321

A.16.6.2.11.1	Test Purpose and Environment	5321

A.16.6.2.11.2	Test Requirements	5323

A.16.6.2.12	SA event triggered reporting tests for FR1 when DRX is used for 2 Rx UE	5323

A.16.6.2.12.1	Test Purpose and Environment	5323

A.16.6.2.12.2	Test Requirements	5326

A.16.6.3	Inter-RAT Measurements	5326

A.16.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 1 Rx UE	5326

A.16.6.3.1.1	Test purpose and Environment	5326

A.16.6.3.1.2	Test Requirements	5329

A.16.6.3.2	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 2 Rx UE	5330

A.16.6.3.2.1	Test purpose and Environment	5330

A.16.6.3.2.2	Test Requirements	5333

A.16.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 1 Rx UE	5333

A.16.6.3.3.1	Test purpose and Environment	5333

A.16.6.3.3.2	Test Requirements	5337

A.16.6.3.4	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 2 Rx UE	5337

A.16.6.3.4.1	Test purpose and Environment	5337

A.16.6.3.4.2	Test Requirements	5340

A.16.6.4	L1-RSRP measurement for beam reporting	5341

A.16.6.4.1	SSB based L1-RSRP measurement when DRX is not used for 1 Rx UE	5341

A.16.6.4.1.1	Test Purpose and Environment	5341

A.16.6.4.1.2	Test parameters	5341

A.16.6.4.1.3	Test Requirements	5342

A.16.6.4.2	SSB based L1-RSRP measurement when DRX is not used for 2 Rx UE	5343

A.16.6.4.2.1	Test Purpose and Environment	5343

A.16.6.4.2.2	Test parameters	5343

A.16.6.4.2.3	Test Requirements	5344

A.16.6.4.3	SSB based L1-RSRP measurement when DRX is used for 1 Rx UE	5344

A.16.6.4.3.1	Test Purpose and Environment	5344

A.16.6.4.3.2	Test parameters	5345

A.16.6.4.3.3	Test Requirements	5346

A.16.6.4.4	SSB based L1-RSRP measurement when DRX is used for 2 Rx UE	5346

A.16.6.4.4.1	Test Purpose and Environment	5346

A.16.6.4.4.2	Test parameters	5347

A.16.6.4.4.3	Test Requirements	5348

A.16.6.4.5	CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE	5348

A.16.6.4.5.1	Test Purpose and Environment	5348

A.16.6.4.5.2	Test parameters	5349

A.16.6.4.5.3	Test Requirements	5350

A.16.6.4.6	CSI-RS based L1-RSRP measurement when DRX is not used for 2 Rx UE	5350

A.16.6.4.6.1	Test Purpose and Environment	5350

A.16.6.4.6.2	Test parameters	5351

A.16.6.4.6.3	Test Requirements	5352

A.16.6.4.7	CSI-RS based L1-RSRP measurement when DRX is used for 1 Rx UE	5352

A.16.6.4.7.1	Test Purpose and Environment	5352

A.16.6.4.7.2	Test parameters	5353

A.16.6.4.7.3	Test Requirements	5354

A.16.6.4.8	CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE	5354

A.16.6.4.8.1	Test Purpose and Environment	5354

A.16.6.4.8.2	Test parameters	5355

A.16.6.4.8.3	Test Requirements	5356

A.16.6.5	NR measurements with autonomous gaps	5357

A.16.6.5.1	SA intra-frequency CGI identification of NR neighbor cell in FR1 for 1 Rx UE	5357

A.16.6.5.1.1	Test Purpose and Environment	5357

A.16.6.5.1.2	Test Parameters	5357

A.16.6.5.1.3	Test Requirements	5359

A.16.6.5.2	SA intra-frequency CGI identification of NR neighbor cell in FR1 for 2 Rx UE	5359

A.16.6.5.2.1	Test Purpose and Environment	5359

A.16.6.5.2.2	Test Parameters	5359

A.16.6.5.2.3	Test Requirements	5361

A.16.6.5.3	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 1 Rx UE	5361

A.16.6.5.3.1	Test Purpose and Environment	5361

A.16.6.5.3.2	Test Requirements	5364

A.16.6.5.4	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 2 Rx UE	5364

A.16.6.5.4.1	Test Purpose and Environment	5364

A.16.6.5.4.2	Test Requirements	5367

A.16.6.6	RSTD Measurements	5367

A.16.6.6.1	NR RSTD measurement reporting delay test case for RedCap UE without FH in FR1 SA	5367

A.16.6.6.1.1	Test Purpose and Environment	5367

A.16.6.6.1.2	Test Requirements	5372

A.16.6.6.2	NR RSTD measurement reporting delay test case with PRS frequency hopping	5372

A.16.6.6.2.1	Test Purpose and Environment	5372

A.16.6.6.2.2	Test Requirements	5376

A.16.6.7	UE Rx-Tx Measurements	5377

A.16.6.7.1	UE Rx-Tx measurement reporting delay test case for single positioning frequency layer in FR1 SA for RedCap UE without RX FH in RRC_CONNECTED mode	5377

A.16.6.7.1.1	Test purpose and environment	5377

A.16.6.7.1.2	Test requirements	5381

A.16.6.7.2	UE Rx-Tx time difference measurement with Rx FH for single positioning frequency layer in FR1 SA in RRC_CONNECTED state	5381

A.16.6.7.2.1	Test purpose and environment	5381

A.16.6.7.2.2	Test requirements	5385

A.16.6.8	PRS-RSRP measurements	5385

A.16.6.8.1	PRS-RSRP measurement delay test case for single positioning frequency layer	5385

A.16.6.8.1.1	Test purpose and Environment	5385

A.16.6.8.1.2	Test Requirements	5389

A.16.6.8.2	PRS-RSRP measurement delay with FH in RRC_CONNECTED state in FR1	5389

A.16.6.8.2.1	Test purpose and Environment	5389

A.16.6.8.2.2	Test Requirements	5393

A.16.6.9	PRS-RSRPP Measurements	5393

A.16.6.9.1	PRS-RSRPP measurement delay without FH in RRC_CONNECTED state in FR1	5393

A.16.6.9.1.1	Test purpose and Environment	5393

A.16.6.9.1.2	Test Requirements	5395

A.16.6.9.2	PRS-RSRPP measurement with Rx FH reporting delay test case for single positioning frequency layer in FR1 SA in RRC_CONNECTED state	5396

A.16.6.9.2.1	Test purpose and Environment	5396

A.16.6.9.2.2	Test Requirements	5398

A.16.7	Measurement Performance requirements for RedCap	5398

A.16.7.1	SS-RSRP	5398

A.16.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5398

A.16.7.1.1.1	Test Purpose and Environment	5398

A.16.7.1.1.2	Test parameters	5398

A.16.7.1.1.3	Test Requirements	5402

A.16.7.1.2	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 2 RX UE	5402

A.16.7.1.2.1	Test Purpose and Environment	5402

A.16.7.1.2.2	Test parameters	5402

A.16.7.1.2.3	Test Requirements	5406

A.16.7.1.3	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5406

A.16.7.1.3.1	Test Purpose and Environment	5406

A.16.7.1.3.2	Test parameters	5406

A.16.7.1.3.3	Test Requirements	5409

A.16.7.1.4	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5409

A.16.7.1.4.1	Test Purpose and Environment	5409

A.16.7.1.4.2	Test parameters	5409

A.16.7.1.4.3	Test Requirements	5412

A.16.7.2	SS-RSRQ	5412

A.16.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5412

A.16.7.2.1.1	Test Purpose and Environment	5412

A.16.7.2.1.2	Test Parameters	5412

A.16.7.2.1.3	Test Requirements	5416

A.16.7.2.2	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5416

A.16.7.2.2.1	Test Purpose and Environment	5416

A.16.7.2.2.2	Test Parameters	5416

A.16.7.2.2.3	Test Requirements	5419

A.16.7.2.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5419

A.16.7.2.3.1	Test Purpose and Environment	5419

A.16.7.2.3.2	Test parameters	5420

A.16.7.2.3.3	Test Requirements	5423

A.16.7.2.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5423

A.16.7.2.4.1	Test Purpose and Environment	5423

A.16.7.2.4.2	Test parameters	5423

A.16.7.2.4.3	Test Requirements	5427

A.16.7.3	SS-SINR	5427

A.16.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5427

A.16.7.3.1.1	Test Purpose and Environment	5427

A.16.7.3.1.2	Test parameters	5427

A.16.7.3.1.3	Test Requirements	5430

A.16.7.3.2	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5430

A.16.7.3.2.1	Test Purpose and Environment	5430

A.16.7.3.2.2	Test parameters	5430

A.16.7.3.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5433

A.16.7.3.3.1	Test Purpose and Environment	5433

A.16.7.3.3.2	Test parameters	5433

A.16.7.3.3.3	Test Requirements	5436

A.16.7.3.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5436

A.16.7.3.4.1	Test Purpose and Environment	5436

A.16.7.3.4.2	Test parameters	5436

A.16.7.3.4.3	Test Requirements	5439

A.16.7.4	L1-RSRP measurement for beam reporting	5440

A.16.7.4.1	SSB based L1-RSRP measurement for 1 Rx UE	5440

A.16.7.4.1.1	Test Purpose and Environment	5440

A.16.7.4.1.2	Test parameters	5440

A.16.7.4.1.3	Test Requirements	5442

A.16.7.4.2	SSB based L1-RSRP measurement for 2 Rx UE	5442

A.16.7.4.2.1	Test Purpose and Environment	5442

A.16.7.4.2.2	Test parameters	5443

A.16.7.4.2.3	Test Requirements	5443

A.16.7.4.3	CSI-RS based L1-RSRP measurement on resource set with repetition off for 1 Rx UE	5443

A.16.7.4.3.1	Test Purpose and Environment	5443

A.16.7.4.3.2	Test parameters	5443

A.16.7.4.3.3	Test Requirements	5446

A.16.7.4.4	CSI-RS based L1-RSRP measurement on resource set with repetition off for 2 Rx UE	5446

A.16.7.4.4.1	Test Purpose and Environment	5446

A.16.7.4.4.2	Test parameters	5446

A.16.7.4.4.3	Test Requirements	5446

A.16.7.5	E-UTRAN RSRP	5446

A.16.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5446

A.16.7.5.1.1	Test Purpose and Environment	5446

A.16.7.5.1.2	Test parameters	5446

A.16.7.5.1.3	Test Requirements	5450

A.16.7.5.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5450

A.16.7.5.2.1	Test Purpose and Environment	5450

A.16.7.5.2.2	Test parameters	5450

A.16.7.5.2.3	Test Requirements	5453

A.16.7.6	E-UTRAN RSRQ	5453

A.16.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5453

A.16.7.6.1.1	Test Purpose and Environment	5453

A.16.7.6.1.2	Test parameters	5453

A.16.7.6.1.3	Test Requirements	5457

A.16.7.6.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5457

A.16.7.6.2.1	Test Purpose and Environment	5457

A.16.7.6.2.2	Test parameters	5457

A.16.7.6.2.3	Test Requirements	5460

A.16.7.7	RSTD measurements	5460

A.16.7.7.1	RSTD measurement accuracy test case for RedCap UE without FH	5460

A.16.7.7.1.1	Test purpose and Environment	5460

A.16.7.7.1.2	Test Requirements	5462

A.16.7.8	UE Rx-Tx measurements	5466

A.16.7.8.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA for RedCap UE without RX FH in RRC_CONNECTED mode	5466

A.16.7.8.1.1	Test purpose and environment	5466

A.16.7.8.1.2	Test parameters	5467

A.16.7.8.1.3	Test requirements	5470

A.16.7.8.2	SA: UE Rx-Tx time difference measurement accuracy with Rx FH in RRC_CONNECTED state in FR1	5470

A.16.7.8.2.1 	Test purpose and Environment	5470

A.16.7.8.2.2	Test parameters	5471

A.16.7.8.2.3	Test requirements	5474

A.16.7.9	PRS-RSRP Measurements	5474

A.16.7.9.1	PRS-RSRP measurement accuracy without FH in RRC_CONNECTED state in FR1	5474

A.16.7.9.1.1	Test Purpose and Environment	5474

A.16.7.9.1.2	Test parameters	5474

A.16.7.9.1.3	Test Requirements	5478

A.16.7.9.2	PRS-RSRP measurement accuracy with FH in RRC_CONNECTED state in FR1	5479

A.16.7.9.2.1	Test Purpose and Environment	5479

A.16.7.9.2.2	Test parameters	5479

A.16.7.9.2.3	Test Requirements	5482

A.16.7.10	PRS-RSRPP measurements	5483

A.16.7.10.1	PRS-RSRPP measurement accuracy without FH in RRC_CONNECTED state in FR1	5483

A.16.7.10.1.1	Test Purpose and Environment	5483

A.16.7.10.1.2	Test parameters	5483

A.16.7.10.1.3	Test Requirements	5486

A.16.7.10.2	SA: PRS-RSRPP measurement accuracy with Rx FH in RRC_CONNECTED state in FR1	5486

A.16.7.10.2.1	Test purpose and Environment	5486

A.16.7.10.2.2	Test parameters	5486

A.16.7.10.2.3	Test requirements	5490

A.16.8	Measurement Procedure for RedCap in RRC_INACTIVE	5490

A.16.8.1	RSTD Measurements	5490

A.16.8.1.1	NR RSTD measurement reporting delay test case for for RedCap UE without FH in FR1 SA in RRC_INACTIVE state	5490

A.16.8.1.1.1	Test Purpose and Environment	5490

A.16.8.1.1.2	Test Requirements	5494

A.16.8.1.2	NR RSTD measurement reporting delay test case with PRS frequency hopping	5494

A.16.8.1.2.1	Test Purpose and Environment	5494

A.16.8.1.2.2	Test Requirements	5498

A.16.8.1.3	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_INACTIVE state when eDRX cycle > 10.24s for RedCap UE	5498

A.16.8.1.3.1	Test Purpose and Environment	5498

A.16.8.1.3.2	Test Requirements	5502

A.16.8.2	UE Rx-Tx Measurements	5502

A.16.8.2.1	UE Rx-Tx measurement reporting delay test case for single positioning frequency layer in FR1 SA for RedCap UE without RX FH in RRC_INACTIVE mode	5502

A.16.8.2.1.1	Test purpose and environment	5502

A.16.8.2.1.2	Test requirements	5506

A.16.8.2.2	UE Rx-Tx time difference measurement with Rx FH for single positioning frequency layer in FR1 SA in RRC_INACTIVE state	5506

A.16.8.2.2.1	Test purpose and environment	5506

A.16.8.2.2.2	Test requirements	5510

A.16.8.2.3.	UE Rx-Tx time difference measurement for single positioning frequency layer with eDRX > 10.24s in FR1 SA	5510

A.16.8.2.3.1	Test purpose and environment	5510

A.16.8.2.3.2	Test requirements	5514

A.16.8.3	PRS-RSRP Measurements	5514

A.16.8.3.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	5514

A.16.8.3.1.1	Test purpose and Environment	5514

A.16.8.3.1.2	Test Requirements	5516

A.16.8.3.3	PRS-RSRP reporting delay test case in RRC_INACTIVE state in FR1 when eDRX cycle > 10.24s	5517

A.16.8.3.3.1	Test purpose and Environment	5517

A.16.8.3.3.2	Test Requirements	5519

A.16.8.4	PRS-RSRPP Measurements	5520

A.16.8.4.1	PRS-RSRPP measurement delay without FH in RRC_INACTIVE state in FR1	5520

A.16.8.4.1.1	Test purpose and Environment	5520

A.16.8.4.2	PRS-RSRPP measurement with Rx FH reporting delay test case for single positioning frequency layer in FR1 SA in RRC_INACTIVE state	5523

A.16.8.4.2.1	Test purpose and Environment	5523

A.16.8.4.2.2	Test Requirements	5525

A.16.9	Measurement Performance Requirements for RedCap in RRC_INACTIVE	5528

A.16.9.1	 RSTD Measurements	5528

A.16.9.1.1	RSTD measurement accuracy test case for RedCap UE without FH in FR1 in RRC_INACTIVE state	5528

A.16.9.1.1.1	Test purpose and Environment	5528

A.16.9.1.1.2	Test Requirements	5530

A.16.9.1.2	RSTD measurement accuracy test case for RedCap UE with FH in FR1 in RRC_INACTIVE state	5530

A.16.9.1.2.1	Test purpose and Environment	5530

A.16.9.1.2.2	Test Requirements	5532

A.16.9.2	UE Rx-Tx measurements	5532

A.16.9.2.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA for RedCap UE without RX FH in RRC_INACTIVE mode	5532

A.16.9.2.1.1	Test purpose and environment	5532

A.16.9.2.1.2	Test parameters	5533

A.16.9.2.1.3	Test requirements	5534

A.16.9.2.2	SA: UE Rx-Tx time difference measurement accuracy with Rx FH in RRC_INACTIVE state in FR1	5535

A.16.9.2.2.1	Test purpose and Environment	5535

A.16.9.2.2.2	Test parameters	5535

A.16.9.2.2.3	Test requirements	5538

A.16.9.3	PRS-RSRP Measurements	5538

A.16.9.3.1	PRS-RSRP measurement accuracy without FH in RRC_INACTIVE state in FR1	5538

A.16.9.3.1.1	Test Purpose and Environment	5538

A.16.9.3.1.2	Test parameters	5538

A.16.9.3.1.3	Test Requirements	5541

A.16.9.3.2	PRS-RSRP measurement accuracy with FH in RRC_INACTIVE state in FR1	5541

A.16.9.3.2.1	Test Purpose and Environment	5541

A.16.9.3.2.2	Test parameters	5541

A.16.9.3.2.3	Test Requirements	5544

A.16.9.4	PRS-RSRPP measurements	5544

A.16.9.4.1	PRS-RSRPP measurement accuracy without Rx FH in RRC_INACTIVE state in FR1	5544

A.16.9.4.1.1	Test purpose and Environment	5544

A.16.9.4.1.2	Test parameters	5544

A.16.9.4.1.3	Test requirements	5548

A.16.9.4.2	SA: PRS-RSRPP measurement accuracy with Rx FH in RRC_INACTIVE state in FR1	5548

A.16.9.4.2.1	Test purpose and Environment	5548

A.16.9.4.2.2	Test parameters	5548

A.16.9.4.2.3	Test requirements	5551

A.16.10	 Measurement procedure for RedCap in RRC_IDLE	5551

A.16.10.1	RSTD measurements	5551

A.16.10.1.1	NR RSTD measurement reporting delay test case for RedCap UE without FH in FR1 SA in RRC_IDLE state without eDRX	5551

A.16.10.1.1.1	Test Purpose and Environment	5551

A.16.10.1.1.2	Test Requirements	5555

A.16.10.1.2	NR RSTD measurement reporting delay test case for RedCap UE without RX FH in FR1 SA in RRC_IDLE state when eDRX > 10.24s	5555

A.16.10.1.2.1	Test Purpose and Environment	5555

A.16.10.1.2.2	Test Requirements	5559

A.16.10.2	PRS-RSRP Measurements	5559

A.16.10.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_IDLE	5559

A.16.10.2.1.1	Test purpose and Environment	5559

A.16.10.2.1.2	Test Requirements	5561

A.16.10.2.2	PRS-RSRP measurement without Rx FH reporting delay test case for single positioning frequency layer in FR1 SA in RRC_IDLE state with eDRX cycle > 10.24s	5562

A.16.10.2.2.1	Test purpose and Environment	5562

A.16.10.2.2.2	Test Requirements	5564

A.16.11	 Measurement Performance Requirements for RedCap in RRC_IDLE	5564

A.16.11.1	RSTD Measurements	5564

A.16.11.1.1	RSTD measurement accuracy test case for RedCap UE without FH in FR1 in RRC_IDLE state without eDRX	5564

A.16.11.1.1.1	Test purpose and Environment	5564

A.16.11.1.1.2	Test Requirements	5566

A.16.11.1.2	RSTD measurement accuracy test case for RedCap UE without FH in FR1 in RRC_IDLE state with eDRX > 10.24s	5567

A.16.11.1.2.1	Test purpose and Environment	5567

A.16.11.1.2.2	Test Requirements	5569

A.16.11.2	PRS-RSRP Measurements	5569

A.16.11.2.1	PRS-RSRP measurement accuracy test case for RedCap UE in FR1 in RRC_IDLE state	5569

A.16.11.2.1.1	Test Purpose and Environment	5569

A.16.11.2.1.2	Test parameters	5569

A.16.11.2.1.3	Test Requirements	5571

A.16.11.2.2	PRS-RSRP measurement without Rx FH accuracy test case for single positioning frequency layer in FR1 SA in RRC_IDLE state with eDRX cycle > 10.24s	5571

A.16.11.2.2.1	Test purpose and Environment	5571

A.16.11.2.2.2	Test Requirements	5573

A.17	NR standalone tests with one or more NR cells in FR2 for RedCap	5575

A.17.1	SA: RRC_IDLE state mobility for RedCap	5575

A.17.1.1	Cell re-selection to NR	5575

A.17.1.1.1	Cell reselection to FR2 intra-frequency NR case for 2 Rx	5575

A.17.1.1.1.1	Test Purpose and Environment	5575

A.17.1.1.1.2	Test Parameters	5575

A.17.1.1.1.3	Test Requirements	5577

A.17.1.1.2	Cell reselection to FR2 inter-frequency NR case	5577

A.17.1.1.2.1	Test Purpose and Environment	5577

A.17.1.1.2.2	Test Parameters	5577

A.17.1.1.2.3	Test Requirements	5579

A.17.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5580

A.17.1.1.3.1	Test Purpose and Environment	5580

A.17.1.1.3.2	Test Parameters	5580

A.17.1.1.3.3	Test Requirements	5582

A.17.1.1.4	Cell reselection to FR2 inter-frequency NR case for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE	5582

A.17.1.1.4.1	Test Purpose and Environment	5582

A.17.1.1.4.2	Test Parameters	5582

A.17.1.1.4.3	Test Requirements	5584

A.17.2	SA: RRC_INACTIVE state mobility for RedCap	5585

A.17.2.1	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	5585

A.17.2.1.1	TA validation for CG-SDT in FR2 for RedCap	5585

A.17.2.1.1.1	Test Purpose and Environment	5585

A.17.2.1.1.2	Test Requirements	5588

A.17.2.2	Cell Reselection for Positioning	5588

A.17.2.2.1	Cell reselection to FR2 intra-frequency NR case with RRC_INACTIVE eDRX and positioning SRS	5588

A.17.2.2.1.1	Test Purpose and Environment	5588

A.17.2.2.1.2	Test Parameters	5588

A.17.2.2.1.3	Test Requirements	5588

A.17.3	RRC_CONNECTED state mobility for RedCap	5588

A.17.3.1	Handover for RedCap	5588

A.17.3.1.1	Intra-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	5588

A.17.3.1.1.1	Test Purpose and Environment	5588

A.17.3.1.1.2	Test Parameters	5588

A.17.3.1.1.3	Test Requirements	5590

A.17.3.1.2	Inter-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	5590

A.17.3.1.2.1	Test Purpose and Environment	5590

A.17.3.1.2.2	Test Parameters	5590

A.17.3.1.2.3 Test Requirements	5592

A.17.3.2	RRC Connection Mobility Control for RedCap	5592

A.17.3.2.1	SA: RRC Re-establishment	5592

A.17.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	5592

A.17.3.2.1.1.1	Test Purpose and Environment	5592

A.17.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	5594

A.17.3.2.1.2.1	Test Purpose and Environment	5594

A.17.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	5596

A.17.3.2.1.3.1	Test Purpose and Environment	5596

A.17.3.2.1.3.2	Test Requirements	5598

A.17.3.2.2	Random Access	5598

A.17.3.2.2.1	4-step RA type contention based random access test in FR2 for NR Standalone	5598

A.17.3.2.2.1.1	Test Purpose and Environment	5598

A.17.3.2.2.1.2	Test Requirements	5600

A.17.3.2.2.2	4-step RA type non-contention based random access test in FR2 for NR Standalone	5602

A.17.3.2.2.2.1	Test Purpose and Environment	5602

A.17.3.2.2.2.2	Test Requirements	5603

A.17.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	5605

A.17.3.2.2.3.1	Test Purpose and Environment	5605

A.17.3.2.2.3.2	Test Requirements	5606

A.17.3.2.2.4	2-step RA type non-contention based random access test in FR2 for NR Standalone	5607

A.17.3.2.2.4.1	Test Purpose and Environment	5607

A.17.3.2.2.4.2	Test Requirements	5609

A.17.3.2.3	SA: RRC Connection Release with Redirection	5610

A.17.3.2.3.1	Redirection from NR in FR2 to NR in FR2	5610

A.17.3.2.3.1.1	Test Purpose and Environment	5610

A.17.3.2.3.1.2	Test Parameters	5610

A.17.3.2.3.1.3	Test Requirements	5612

A.17.4	Timing	5612

A.17.4.1	UE transmit timing	5612

A.17.4.1.1	NR UE Transmit Timing Test for FR2	5612

A.17.4.1.1.1	Test Purpose and environment	5612

A.17.4.1.1.2	Test requirements	5614

A.17.4.2	UE timer accuracy	5615

A.17.4.3	Timing advance	5615

A.17.4.3.1	SA FR2 timing advance adjustment accuracy	5615

A.17.4.3.1.1	Test Purpose and Environment	5615

A.17.4.3.1.2	Test Parameters	5615

A.17.4.3.1.3	Test Requirements	5618

A.17.5	Signaling characteristics for RedCap	5618

A.17.5.1	Radio link Monitoring for RedCap	5618

A.17.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	5618

A.17.5.1.1.1	Test Purpose and Environment	5618

A.17.5.1.1.2	Test Requirements	5621

A.17.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	5621

A.17.5.1.2.1	Test Purpose and Environment	5621

A.17.5.1.2.2	Test Requirements	5624

A.17.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	5624

A.17.5.1.3.1	Test Purpose and Environment	5624

A.17.5.1.3.2	Test Requirements	5627

A.17.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	5627

A.17.5.1.4.1	Test Purpose and Environment	5627

A.17.5.1.4.2	Test Requirements	5629

A.17.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	5630

A.17.5.1.5.1	Test Purpose and Environment	5630

A.17.5.1.5.2	Test Requirements	5632

A.17.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	5633

A.17.5.1.6.1	Test Purpose and Environment	5633

A.17.5.1.6.2	Test Requirements	5635

A.17.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	5636

A.17.5.1.7.1	Test Purpose and Environment	5636

A.17.5.1.7.2	Test Requirements	5638

A.17.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	5638

A.17.5.1.8.1	Test Purpose and Environment	5638

A.17.5.1.8.2	Test Requirements	5641

A.17.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	5642

A.17.5.1.9.1	Test Purpose and Environment	5642

A.17.5.1.9.2	Test Requirements	5643

A.17.5.2	Beam Failure Detection and Link recovery procedures	5644

A.17.5.2.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	5644

A.17.5.2.1.1	Test Purpose and Environment	5644

A.17.5.2.1.2	Test Requirements	5646

A.17.5.2.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	5647

A.17.5.2.2.1	Test Purpose and Environment	5647

A.17.5.2.2.2	Test Requirements	5650

A.17.5.2.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	5650

A.17.5.2.3.1	Test Purpose and Environment	5650

A.17.5.2.3.2	Test Requirements	5653

A.17.5.2.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	5653

A.17.5.2.4.1	Test Purpose and Environment	5653

A.17.5.2.4.2	Test Requirements	5656

A.17.5.2.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	5656

A.17.5.2.5.1	Test Purpose and Environment	5656

A.17.5.2.5.2	Test Requirements	5659

A.17.5.3	Active BWP switch for RedCap	5660

A.17.5.3.1	DCI-based and Timer-based Active BWP Switch	5660

A.17.5.3.1.1	NR FR2 DL active BWP switch with non-DRX in SA	5660

A.17.5.3.1.1.1	Test Purpose and Environment	5660

A.17.5.3.1.1.2	Test Requirements	5662

A.17.5.3.2	RRC-based Active BWP Switch	5662

A.17.5.3.2.1	NR FR2 DL active BWP switch of PCell with non-DRX in SA	5662

A.17.5.3.2.1.1	Test Purpose and Environment	5662

A.17.5.3.2.1.2	Test Requirements	5665

A.17.5.4	Active TCI state switch delay	5665

A.17.5.4.1	MAC-CE based active TCI state switch	5665

A.17.5.4.1.1	NR PCell FR2 active TCI state switch for a known TCI state	5665

A.17.5.4.1.1.1	Test Purpose and Environment	5665

A.17.5.4.1.1.2	Test Requirements	5668

A.17.5.4.2	RRC based active TCI state switch	5668

A.17.5.4.2.1	NR PCell FR2 active TCI state switch for a known TCI state	5668

A.17.5.4.2.1.1	Test Purpose and Environment	5668

A.17.5.4.2.1.2	Test Requirements	5671

A.17.5.5	Uplink spatial relation switch delay	5671

A.17.5.5.1	MAC-CE based Spatial Relation switch	5671

A.17.5.5.1.1	 NR PCell FR2 spatial relation associated with known DL-RS	5671

A.17.5.5.1.1.1	Test Purpose and Environment	5671

A.17.5.5.1.1.2	Test Requirements	5673

A.17.5.5.2	RRC based spatial relation switch	5674

A.17.5.5.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	5674

A.17.5.5.2.1.2	Test Requirements	5676

A.17.5.6	UE specific CBW change	5676

A.17.5.6.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	5676

A.17.5.6.1.1	Test Purpose and Environment	5676

A.17.5.6.1.2	Test Requirements	5678

A.17.6	Measurement procedure for RedCap	5679

A.17.6.1	Intra-frequency Measurements	5679

A.17.6.1.1	SA event triggered reporting test without gap under non-DRX	5679

A.17.6.1.1.1	Test purpose and Environment	5679

A.17.6.1.1.2	Test Requirements	5681

A.17.6.1.2	SA event triggered reporting test without gap under DRX	5681

A.17.6.1.2.1	Test purpose and Environment	5681

A.7.6.1.2.2	Test Requirements	5682

A.17.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	5682

A.17.6.1.3.1	Test purpose and Environment	5682

A.17.6.1.3.2	Test Requirements	5685

A.17.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	5685

A.17.6.1.4.1	Test purpose and Environment	5685

A.17.6.1.4.2	Test Requirements	5687

A.17.6.2	Inter-frequency Measurements	5688

A.17.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	5688

A.17.6.2.1.1	Test Purpose and Environment	5688

A.17.6.2.1.2	Test Requirements	5690

A.17.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	5690

A.17.6.2.2.1	Test Purpose and Environment	5690

A.17.6.2.2.2	Test Requirements	5692

A.17.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	5693

A.17.6.2.3.1	Test Purpose and Environment	5693

A.17.6.2.3.2	Test Requirements	5695

A.17.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2) for 2 RX UE	5695

A.17.6.2.4.1	Test Purpose and Environment	5695

A.17.6.2.4.2	Test Requirements	5697

A.17.6.3	L1-RSRP measurement for beam reporting	5698

A.17.6.3.1	SSB based L1-RSRP measurement when DRX is not used	5698

A.17.6.3.1.1	Test Purpose and Environment	5698

A.17.6.3.1.2	Test parameters	5698

A.17.6.3.1.3	Test Requirements	5698

A.17.6.3.2	SSB based L1-RSRP measurement when DRX is used	5698

A.17.6.3.2.1	Test Purpose and Environment	5698

A.17.6.3.2.2	Test parameters	5699

A.17.6.3.2.3	Test Requirements	5700

A.17.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	5700

A.17.6.3.3.1	Test Purpose and Environment	5700

A.17.6.3.3.2	Test parameters	5700

A.17.6.3.3.3	Test Requirements	5702

A.17.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	5702

A.17.6.3.4.1	Test Purpose and Environment	5702

A.17.6.3.4.2	Test parameters	5703

A.7.6.3.3.3	Test Requirements	5704

A.17.6.4.1	SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2) for 2 RX UE	5704

A.17.6.4.1.1	Test Purpose and Environment	5704

A.17.6.4.1.2	Test Requirements	5707

A.17.6.5	RSTD measurements	5707

A.17.6.5.1	NR RSTD measurement reporting delay test case for RedCap UE without FH in FR2 SA	5707

A.17.6.5.1.1	Test Purpose and Environment	5707

A.17.6.5.1.2	Test Requirements	5714

A.17.6.5.2	NR RSTD measurement reporting delay test case with PRS frequency hopping	5714

A.17.6.5.2.1	Test Purpose and Environment	5714

A.17.6.5.2.2	Test Requirements	5719

A.17.6.6	UE Rx-Tx Measurements	5720

A.17.6.6.1	UE Rx-Tx measurement reporting delay for single positioning frequency layer in FR2 SA without RX FH in RRC_CONNECTED mode	5720

A.17.6.6.1.1	Test purpose and environment	5720

A.17.6.6.1.2	Test requirements	5724

A.17.6.6.2	UE Rx-Tx time difference measurement with Rx FH for single positioning frequency layer in FR2 SA in RRC_CONNECTED state	5724

A.17.6.6.2.1	Test purpose and environment	5724

A.17.6.6.2.2	Test requirements	5728

A.17.6.7	PRS-RSRP measurements	5728

A.17.6.7.1	PRS-RSRP measurement delay test case for RedCap positioning without Rx FH in RRC_CONNECTED state in FR2	5728

A.17.6.7.1.1	PRS-RSRP measurement delay test case for single positioning frequency layer	5728

A.17.6.7.1.1.1	Test Purpose and Environment	5728

A.17.6.7.1.1.2	Test Requirements	5732

A.17.6.7.1.2	PRS-RSRP measurement delay test case for dual positioning frequency layer	5732

A.17.6.7.1.2.1	Test Purpose and Environment	5732

A.17.6.7.1.2.2	Test Requirements	5736

A.17.6.7.2	PRS-RSRP measurement delay with FH in RRC_CONNECTED state in FR2	5736

A.17.6.7.2.1	Test Purpose and Environment	5736

A.17.6.7.2.2	Test Requirements	5740

A.17.6.8	PRS-RSRPP Measurements	5740

A.17.6.8.1	PRS-RSRPP measurement delay without FH in RRC_CONNECTED state in FR2	5740

A.17.6.8.1.1	Test Purpose and Environment	5740

A.17.6.8.1.2	Test Requirements	5743

A.17.6.8.2	PRS-RSRPP measurement with Rx FH reporting delay test case for single positioning frequency layer in FR2 SA in RRC_CONNECTED state	5743

A.17.6.8.2.1	Test Purpose and Environment	5743

A.17.6.8.2.2	Test Requirements	5745

A.17.7	Measurement Performance requirements	5746

A.17.7.1	SS-RSRP	5746

A.17.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	5746

A.17.7.1.1.1	Test Purpose and Environment	5746

A.17.7.1.1.2	Test parameters	5746

A.17.7.1.1.3	Test Requirements	5748

A.17.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	5748

A.17.7.1.2.1	Test Purpose and Environment	5748

A.17.7.1.2.2	Test parameters	5748

A.17.7.1.2.3	Test Requirements	5750

A.17.7.2	SS-RSRQ	5751

A.17.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	5751

A.17.7.2.1.1	Test Purpose and Environment	5751

A.17.7.2.1.2	Test Parameters	5751

A.17.7.2.1.3	Test Requirements	5753

A.17.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell for 2 Rx UE	5753

A.17.7.2.2.1	Test Purpose and Environment	5753

A.17.7.2.2.2	Test parameters	5753

A.17.7.2.2.3	Test Requirements	5755

A.17.7.2.3	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	5755

A.17.7.3	L1-RSRP measurement for beam reporting	5755

A.17.7.3.1	SSB based L1-RSRP measurement	5755

A.17.7.3.1.1	Test Purpose and Environment	5755

A.17.7.3.1.2	Test parameters	5755

A.17.7.3.1.3	Test Requirements	5756

A.17.7.3.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	5756

A.17.7.3.2.1	Test Purpose and Environment	5756

A.17.7.3.2.2	Test parameters	5756

A.17.7.3.2.3	Test Requirements	5756

A.17.7.4	SS-SINR	5757

A.17.7.4	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell for 2Rx UE	5757

A.17.7.4.1.1	Test Purpose and Environment	5757

A.17.7.4.1.2	Test parameters	5757

A.17.7.4.1.3	Test Requirements	5759

A.17.7.5	RSTD measurements	5759

A.17.7.5.1	RSTD measurement accuracy test case for RedCap UE without FH	5759

A.17.7.5.1.1	Test purpose and Environment	5759

A.17.7.5.1.2	Test Requirements	5761

A.17.7.6	UE Rx-Tx Measurements	5763

A.17.7.6.1	UE Rx-Tx measurement accuracy for single positioning frequency layer in FR2 SA without RX FH in RRC_CONNECTED mode	5763

A.17.7.6.1.1	Test purpose and environment	5763

A.17.7.6.1.2	Test parameters	5764

A.17.7.6.1.3	Test requirements	5767

A.17.7.6.2	SA: UE Rx-Tx time difference measurement accuracy with Rx FH in RRC_CONNECTED state in FR2	5767

A.17.7.6.2.1	Test purpose and Environment	5767

A.17.7.6.2.2	Test parameters	5768

A.17.7.6.2.3	Test requirements	5771

A.17.7.7	PRS-RSRP Measurements	5771

A.17.7.7.1	PRS-RSRP measurement accuracy without FH in RRC_CONNECTED state in FR2	5771

A.17.7.7.1.1	Test Purpose and Environment	5771

A.17.7.7.1.2	Test parameters	5771

A.17.7.7.1.3	Test Requirements	5774

A.17.7.7.2	PRS-RSRP measurement accuracy with FH in RRC_CONNECTED state in FR2	5774

A.17.7.7.2.1	Test Purpose and Environment	5774

A.17.7.7.2.2	Test parameters	5775

A.17.7.7.2.3	Test Requirements	5777

A.17.7.8	PRS-RSRPP Measurements	5777

A.17.7.8.1	PRS-RSRPP measurement accuracy without FH in RRC_CONNECTED state in FR2	5777

A.17.7.8.1.1	Test Purpose and Environment	5777

A.17.7.8.1.2	Test parameters	5778

A.17.7.8.1.3	Test Requirements	5780

A.17.7.8.2	SA: PRS-RSRPP measurement accuracy with Rx FH in RRC_CONNECTED state in FR2	5780

A.17.7.8.2.1	Test purpose and Environment	5780

A.17.7.8.2.2	Test parameters	5781

A.17.7.8.2.3	Test requirements	5783

A.17.8	Measurement Procedure for RedCap in RRC_INACTIVE	5784

A.17.8.1	RSTD Measurements	5784

A.17.8.1.1	NR RSTD measurement reporting delay test case for RedCap UE without FH in FR2 SA in RRC_INACTIVE state	5784

A.17.8.1.1.1	Test Purpose and Environment	5784

A.17.8.1.1.2	Test Requirements	5787

A.17.8.1.2	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_INACTIVE state	5787

A.17.8.1.2.1	Test Purpose and Environment	5787

A.17.8.1.2.2	Test Requirements	5790

A.17.8.1.3	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_INACTIVE state with eDRX > 10.24s	5790

A.17.8.1.3.1	Test purpose and environment	5790

A.17.8.1.3.2	Test requirements	5790

A.17.8.2	UE Rx-Tx Measurements	5791

A.17.8.2.1	UE Rx-Tx measurement reporting delay for single positioning frequency layer in FR2 SA without RX FH in RRC_INACTIVE mode	5791

A.17.8.2.1.1	Test purpose and environment	5791

A.17.8.2.1.2	Test requirements	5794

A.17.8.2.2	UE Rx-Tx time difference measurement with Rx FH for single positioning frequency layer in FR2 SA in RRC_INACTIVE state	5794

A.17.8.2.2.1	Test purpose and environment	5794

A.17.8.2.2.2	Test requirements	5798

A.17.8.2.3	UE Rx-Tx time difference measurements for single positioning frequency layer with eDRX > 10.24s in FR2 SA	5798

A.17.8.2.3.1	Test purpose and environment	5798

A.17.8.2.3.2	Test requirements	5798

A.17.8.3	PRS-RSRP Measurements	5799

A.17.8.3.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	5799

A.17.8.3.1.1	Test Purpose and Environment	5799

A.17.8.3.1.2	Test Requirements	5803

A.17.8.3.2.2	Test Requirements	5807

A.17.8.3.3	PRS-RSRP reporting delay in RRC_INACTIVE with eDRX	5807

A.17.8.3.3.1	Test Purpose and Environment	5807

A.17.8.3.3.2	Test Requirements	5811

A.17.8.4	PRS-RSRPP Measurements	5811

A.17.8.4.1	PRS-RSRPP measurement delay without FH in RRC_INACTIVE state in FR2	5811

A.17.8.4.1.1	Test Purpose and Environment	5811

A.17.8.4.2	PRS-RSRPP measurement with Rx FH reporting delay test case for single positioning frequency layer in FR2 SA in RRC_INACTIVE state	5814

A.17.8.4.2.1	Test Purpose and Environment	5814

A.17.8.4.2.2	Test Requirements	5816

A.17.8.4.3	PRS-RSPP reporting delay in RRC_INACTIVE state with eDRX > 10.24s	5816

A.17.8.4.3.1	Test purpose and environment	5816

A.17.8.4.3.2	Test requirements	5816

A.17.9	Measurement Performance Requirements for RedCap in RRC_INACTIVE	5817

A.17.9.1	RSTD Measurements	5817

A.17.9.1.1	RSTD measurement accuracy test case for RedCap UE without FH in FR2 in RRC_INACTIVE state	5817

A.17.9.1.1.1	Test purpose and Environment	5817

A.17.9.1.1.2	Test Requirements	5819

A.17.9.1.2	RSTD measurement accuracy test case for RedCap UE with FH in FR2 in RRC_INACTIVE state	5819

A.17.9.1.2.1	Test purpose and Environment	5819

A.17.9.1.2.2	Test Requirements	5821

A.17.9.2	UE Rx-Tx Measurements	5821

A.17.9.2.1	UE Rx-Tx measurement accuracy for single positioning frequency layer in FR2 SA without RX FH in RRC_INACTIVE mode	5821

A.17.9.2.1.1	Test purpose and environment	5821

A.17.9.2.1.2	Test parameters	5822

A.17.9.2.1.3	Test requirements	5825

A.17.9.2.2	SA: UE Rx-Tx time difference measurement accuracy with Rx FH in RRC_INACTIVE state in FR2	5825

A.17.9.2.2.1	Test purpose and Environment	5825

A.17.9.2.2.2	Test parameters	5825

A.17.9.2.2.3	Test requirements	5828

A.17.9.3	PRS-RSRP Measurements	5828

A.17.9.3.2	PRS-RSRP measurement accuracy with FH in RRC_INACTIVE state in FR2	5830

A.17.9.3.2.1	Test Purpose and Environment	5830

A.17.9.3.2.2	Test parameters	5831

A.17.9.3.2.3	Test Requirements	5832

A.17.9.4	PRS-RSRPP Measurements	5832

A.17.9.4.1	SA: PRS-RSRPP measurement accuracy with Rx FH in RRC_INACTIVE state in FR2	5832

A.17.9.4.1.1	Test Purpose and Environment	5832

A.17.9.4.1.2	Test parameters	5833

A.17.9.4.1.3	Test Requirements	5835

A.17.9.4.2	SA: PRS-RSRPP measurement accuracy with Rx FH in RRC_INACTIVE state in FR2	5835

A.17.9.4.2.1	Test Purpose and Environment	5835

A.17.9.4.2.2	Test parameters	5836

A.17.9.4.2.3	Test Requirements	5838

A.17.10	Measurement Procedure for RedCap in RRC_IDLE	5839

A.17.10.1	RSTD Measurements	5839

A.17.10.1.1	NR RSTD measurement reporting delay test case for RedCap UE without FH in FR2 SA in RRC_IDLE state without eDRX	5839

A.17.10.1.1.1	Test Purpose and Environment	5839

A.17.10.1.1.2	Test Requirements	5842

A.17.10.1.2	NR RSTD without FH measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_IDLE state with eDRX > 10.24s	5842

A.17.10.1.2.1	Test purpose and environment	5842

A.17.10.1.2.2	Test requirements	5842

A.17.10.2	PRS-RSRP Measurements	5843

A.17.10.2.1	PRS-RSRP measurement delay test case for single positioning frequency layer in RRC_IDLE	5843

A.17.10.2.1.1	Test Purpose and Environment	5843

A.17.10.2.1.2	Test Requirements	5847

A.17.10.2.2	PRS-RSRP reporting delay test case in RRC_IDLE state in FR2 when eDRX cycle > 10.24s	5847

A.17.10.2.2.1	Test Purpose and Environment	5847

A.17.10.2.2.2	Test Requirements	5847

A.17.11	 Measurement Performance Requirements for RedCap in RRC_IDLE	5848

A.17.11.1	RSTD Measurements	5848

A.17.11.1.1	RSTD measurement accuracy test case for RedCap UE without FH in FR2 in RRC_IDLE state without eDRX	5848

A.17.11.1.1.1	Test purpose and Environment	5848

11.1.1.2	Test Requirements	5850

A.17.11.1.2	RSTD without FH measurement accuracy test case for single positioning frequency layer in FR2 SA in RRC_IDLE state with eDRX > 10.24s	5850

A.17.11.1.2.1	Test purpose and environment	5850

A.17.11.1.2.2	Test requirements	5852

A.17.11.2	PRS-RSRP Measurements	5852

A.17.11.2.1	PRS-RSRP measurement accuracy test case for RedCap UE in FR2 in RRC_IDLE state	5852

A.17.11.2.1.1	Test Purpose and Environment	5852

A.17.11.2.1.2	Test parameters	5852

A.17.11.2.2	PRS-RSRP measurement accuracy test case in RRC_IDLE state in FR2 when eDRX cycle > 10.24s	5854

A.17.11.2.2.1	Test purpose and Environment	5854

A.17.11.2.2.1	Test parameters	5855

A.17.11.2.2.2	Test Requirements	5855

A.18	E-UTRA standalone tests for NR RRM for RedCap	5855

A.18.1	RRC_IDLE state mobility	5855

A.18.1.1	Inter-RAT NR Cell re-selection	5855

A.18.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	5855

A.18.1.1.1.1	Test Purpose and Environment	5855

A.18.1.1.1.2	Test Requirements	5858

A.18.2	RRC_CONNECTED state mobility	5858

A.18.2.1	Handover	5858

A.18.2.1.1	E-UTRAN - NR handover in FR1	5858

A.18.2.1.1.1	Test Purpose and Environment	5858

A.18.2.1.1.2	Test Requirements	5862

A.18.2.2	RRC connection release with redirection	5862

A.18.2.2.1	Redirection from E-UTRA to NR FR1 for redcap UE	5862

A.18.2.2.1.1	Test Purpose and Environment	5862

A.18.2.2.1.2	Test Parameters	5862

A.18.2.2.1.3	Test Requirements	5865

A.18.3	Measurement procedure	5866

A.18.3.1	E-UTRA – NR Inter-RAT Measurements	5866

A.18.3.1.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	5866

A.18.3.1.1.1	Test Purpose and Environment	5866

A.18.3.1.1.2	Test Requirements	5869

A.18.3.1.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	5869

A.18.3.1.2.1	Test Purpose and Environment	5869

A.18.3.1.2.2	Test Requirements	5873

A.18.3.1.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	5873

A.18.3.1.3.1	Test Purpose and Environment	5873

A.18.3.1.3.2	Test Requirements	5877

A.18.3.1.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	5877

A.18.3.1.4.1	Test Purpose and Environment	5877

A.18.3.1.4.2	Test Requirements	5881

A.18.3.1.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	5881

A.18.3.1.5.1	Test Purpose and Environment	5881

A.18.3.1.5.2	Test Requirements	5883

A.18.3.1.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	5883

A.18.3.1.6.1	Test Purpose and Environment	5883

A.18.3.1.6.2	Test Requirements	5885

A.18.3.1.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	5886

A.18.3.1.7.1	Test Purpose and Environment	5886

A.18.3.1.7.2	Test Requirements	5887

A.18.3.1.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	5888

A.18.3.1.8.1	Test Purpose and Environment	5888

A.18.3.1.8.2	Test Requirements	5890

A.19	NR standalone tests for ATG	5891

A.19.1	RRC_IDLE state mobility	5891

A.19.1.1	Cell reselection to FR1 intra-frequency NR case	5891

A.19.1.1.1	Test Purpose and Environment	5891

A.19.1.1.2	Test Parameters	5891

A.19.1.1.3	Test Requirements	5892

A.19.1.2	Cell reselection to FR1 inter-frequency NR case	5892

A.19.1.2.1	Test Purpose and Environment	5892

A.19.1.2.2	Test Parameters	5892

A.19.1.2.3	Test Requirements	5894

A.19.1.3	Cell reselection to FR1 inter-frequency NR case for UE configured with hs-ATG-cellReselectionSet-r18	5895

A.19.1.3.1	Test Purpose and Environment	5895

A.19.1.3.2	Test Parameters	5895

A.19.1.3.3	Test Requirements	5897

A.19.2	RRC_CONNECTED state mobility	5898

A.19.2.1	Handover	5898

A.19.2.1.1	Intra-frequency handover from FR1 to FR1; known target cell	5898

A19.2.1.1.1	Test Purpose and Environment	5898

A.19.2.1.1.2	Test Parameters	5898

A.19.2.1.2.3	Test Requirements	5898

A.19.2.1.2	Inter-frequency handover from FR1 to FR1; unknown target cell	5899

A.19.2.1.2.1	Test Purpose and Environment	5899

A.19.2.1.2.2	Test Parameters	5899

A.19.2.1.2.3	Test Requirements	5900

A.19.2.2	Conditional Handover	5900

A.19.2.2.1	Intra-frequency distance-based conditional Handover from FR1 to FR1	5900

A.19.2.2.1.1	Test Purpose and Environment	5900

A.19.2.2.1.2	Test Parameters	5900

A.19.2.2.1.3	Test Requirements	5902

A.19.2.2.2	Inter-frequency distance-based conditional Handover from FR1 to FR1	5903

A.19.2.2.2.1	Test Purpose and Environment	5903

A.19.2.2.2.2	Test Parameters	5903

A.19.2.2.2.3	Test Requirements	5905

A.19.2.3	RRC Connection Mobility Control	5905

A.19.2.3.1	SA: RRC Re-establishment	5905

A.19.2.3.1.1	Intra-frequency RRC Re-establishment in FR1 for ATG	5905

A.19.2.3.1.1.1	Test Purpose and Environment	5906

A.19.2.3.1.1.2 Test Requirements	5906

A.19.2.3.1.2	Inter-frequency RRC Re-establishment in FR1 with unknown target cell without serving cell timing for ATG	5906

A.19.2.3.1.2.1	Test Purpose and Environment	5906

A.19.2.3.1.2.2	Test Requirements	5908

A.19.2.3.2	Random Access for ATG UE	5909

A.19.2.3.2.1.1	Test Purpose and Environment	5909

A.19.2.3.2.1.2	Test Requirements	5910

A.19.2.3.2.2.1	Test Purpose and Environment	5910

A.19.2.3.2.2.2	Test Requirements	5910

A.19.2.3.2.3	2-step RA type contention based random access test in FR1 for NR standalone	5910

A.19.2.3.2.3.1	Test Purpose and Environment	5910

A.19.2.3.2.3.2	Test Requirements	5911

A.19.2.3.2.4	2-step RA type non-contention based test in FR1 for NR standalone	5911

A.19.2.3.2.4.1	Test Purpose and Environment	5911

A.19.2.3.2.4.2	Test Requirements	5912

A.19.2.3.3.1.1	Test Purpose and Environment	5912

A.19.2.3.3.1.2	Test Parameters	5912

A.19.2.3.3.1.3	Test Requirements	5913

A.19.3	Timing	5913

A.19.3.1	UE transmit timing	5913

A.19.3.1.1	ATG UE Transmit Timing Test for FR1	5913

A.19.3.1.1.1	Test Purpose and environment	5913

A.19.3.1.1.2	Test requirements	5915

A.19.3.2	UE timer accuracy	5915

A.19.3.3	Timing advance	5915

A.19.3.3.1	SA FR1 timing advance adjustment accuracy	5915

A.19.3.3.1.1	Test Purpose and Environment	5915

A.19.3.3.1.2	Test Parameters	5915

A.19.3.3.1.3	Test Requirements	5916

A.19.4	Signalling characteristics	5916

A.19.4.1	Radio link Monitoring	5916

A.19.4.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	5916

A.19.4.1.1.1	Test Purpose and Environment	5917

A.19.4.1.1.2	Test Requirements	5919

A.19.4.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	5919

A.19.4.1.2.1	Test Purpose and Environment	5919

A.19.4.1.2.2	Test Requirements	5922

A.19.4.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	5922

A.19.4.1.3.1	Test Purpose and Environment	5922

A.19.4.1.3.2	Test Requirements	5925

A.19.4.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	5926

A.19.4.1.4.1	Test Purpose and Environment	5926

A.19.4.1.4.2	Test Requirements	5929

A.19.4.2	Beam Failure Detection and Link recovery procedures	5929

A.19.4.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	5929

A.19.4.2.1.1	Test Purpose and Environment	5929

A.19.4.2.1.2	Test Requirements	5933

A.19.4.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	5933

A.19.4.2.2.1	Test Purpose and Environment	5933

A.19.4.2.2.2	Test Requirements	5937

A.19.4.3	Active BWP switch	5937

A.19.4.3.1	DCI-based and Timer-based Active BWP Switch	5937

A.19.4.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA	5937

A.19.4.3.2	RRC-based Active BWP Switch	5940

A.19.4.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	5940

A.19.4.4	UE specific CBW change	5942

A19.4.4.1	UE specific CBW change on PCell in FR1 in non-DRX	5942

A19.4.4.1.1	Test Purpose and Environment	5942

A.19.4.4.1.2	Test Requirements	5945

A.19.4.5	Pathloss reference signal switching delay	5945

A.19.4.5.1	MAC-CE based pathloss reference signal switch delay	5945

A.19.4.5.1.1	Test Purpose and Environment	5945

A.19.4.5.1.2	Test Requirements	5947

A.19.5	Measurement procedure	5948

A.19.5.1	Intra-frequency Measurements	5948

A.19.5.1.1	SA event triggered reporting tests without gap without SSB index reading under non-DRX	5948

A.19.5.1.1.1	Test purpose and Environment	5948

A.19.5.1.1.2	Test parameters	5948

A.19.5.1.1.3	Test Requirements	5948

A.19.5.1.2	SA event triggered reporting tests with per-UE gaps under non-DRX	5949

A.19.5.1.2.1	Test purpose and Environment	5949

A.19.5.1.2.2	Test parameters	5949

A.19.5.1.2.3	Test Requirements	5949

A.19.5.1.3	SA event triggered reporting tests without gap under non-DRX with SSB index reading	5949

A.19.5.1.3.1	Test purpose and Environment	5949

A.19.5.1.3.2	Test parameters	5950

A.19.5.1.3.3	Test Requirements	5950

A.19.5.1.4	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	5950

A.19.5.1.4.1	Test purpose and Environment	5950

A.19.5.1.4.2	Test parameters	5950

A.19.5.1.4.3	Test Requirements	5951

A.19.5.2	Inter-frequency Measurements	5951

A.19.5.2.1.2	Test parameters	5951

A.19.5.2.1.3	Test Requirements	5952

A.19.5.2.2.2	Test parameters	5953

A.19.5.2.3.2	Test parameters	5953

A.19.5.2.3.3	Test Requirements	5954

A.19.5.3	L1-RSRP measurement for beam reporting for ATG	5954

A.19.5.3.1	SSB based L1-RSRP measurement when DRX is not used	5954

A.19.5.3.1.1	Test Purpose and Environment	5954

A.19.5.3.1.2	Test parameters	5954

A.19.5.3.1.3	Test Requirements	5955

A.19.5.3.2	CSI-RS based L1-RSRP measurement when DRX is not used	5955

A.19.5.3.2.1	Test Purpose and Environment	5955

A.19.5.3.2.2	Test parameters	5955

A.19.5.3.2.3	Test Requirements	5955

A.19.5.4	L1-SINR measurement for beam reporting for ATG	5955

A.19.5.4.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is not used	5955

A.19.5.4.1.3	Test Requirements	5956

A.19.5.4.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	5956

A.19.5.4.2.1	Test Purpose and Environment	5956

A.19.5.4.2.2	Test parameters	5956

A.19.5.4.2.3	Test Requirements	5957

A.19.5.4.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	5957

A.19.5.4.3.1	Test Purpose and Environment	5957

A.19.5.4.3.2	Test parameters	5957

A.19.5.4.3.3	Test Requirements	5957

A.19.5.5	NR measurements with autonomous gaps for ATG	5958

A.19.5.5.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	5958

A.19.5.5.1.1	Test Purpose and Environment	5958

A.19.5.5.1.2	Test Parameters	5958

A.19.5.5.1.3	Test Requirements	5958

A.19.6	Measurement Performance requirements	5958

A.19.6.1	SS-RSRP for ATG UE	5959

A.19.6.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	5959

A.19.6.1.1.1	Test Purpose and Environment	5959

A.19.6.1.1.2	Test parameters	5959

A.19.6.1.1.3	Test Requirements	5959

A.19.6.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	5959

A.19.6.1.2.1	Test Purpose and Environment	5959

A.19.6.1.2.2	Test parameters	5960

A.19.6.1.2.3	Test Requirements	5960

A.19.6.2	SS-RSRQ for ATG UE	5960

A.19.6.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	5960

A.19.6.2.1.1	Test Purpose and Environment	5960

A.19.6.2.1.2	Test Parameters	5960

A.19.6.2.1.3	Test Requirements	5961

A.19.6.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	5961

A.19.6.2.2.1	Test Purpose and Environment	5961

A.19.6.2.2.2	Test Parameters	5961

A.19.6.2.2.3	Test Requirements	5961

A.19.6.3	SS-SINR for ATG UE	5962

A.19.6.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	5962

A.19.6.3.1.1	Test Purpose and Environment	5962

A.19.6.3.1.2	Test Parameters	5962

A.19.6.3.1.3	Test Requirements	5962

A.19.6.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	5962

A.19.6.3.2.1	Test Purpose and Environment	5962

A.19.6.3.2.2	Test Parameters	5962

A.19.6.3.2.3	Test Requirements	5963

A.19.6.4	L1-RSRP measurement for beam reporting for ATG UE	5963

A.19.6.4.1	SSB based L1-RSRP measurement	5963

A.19.6.4.1.1	Test Purpose and Environment	5963

A.19.6.4.1.2	Test parameters	5963

A.19.6.4.1.3	Test Requirements	5964

A.19.6.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	5964

A.19.6.4.2.1	Test Purpose and Environment	5964

A.19.6.4.2.2	Test parameters	5964

A.19.6.4.2.3	Test Requirements	5964

A.19.6.5	L1-SINR measurement for beam reporting based CMR for ATG UE	5965

A.19.6.5.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	5965

A.19.6.5.1.1	Test Purpose and Environment	5965

A.19.6.5.1.2	Test parameters	5965

A.19.6.5.1.3	Test Requirements	5965

A.19.6.5.2	L1-SINR measurement with SSB based CMR and dedicated IMR	5965

A.19.6.5.2.1	Test Purpose and Environment	5965

A.19.6.5.2.2	Test parameters	5966

A.19.6.5.2.3	Test Requirements	5966

A.19.6.5.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	5966

A.19.6.5.3.1	Test Purpose and Environment	5966

A.19.6.5.3.2	Test parameters	5966

A.19.6.5.3.3	Test Requirements	5967

A.19.6.6	CSI-RSRP for ATG UE	5967

A.19.6.6.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	5967

A.19.6.6.1.1	Test Purpose and Environment	5967

A.19.6.6.1.2	Test parameters	5967

A.19.6.6.1.3	Test Requirements	5967

A.19.6.6.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	5968

A.19.6.6.2.1	Test Purpose and Environment	5968

A.19.6.6.2.2	Test parameters	5968

A.19.6.6.2.3	Test Requirements	5968

A.19.6.7	CSI-RSRQ for ATG UE	5968

A.19.6.7.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	5968

A.19.6.7.1.1	Test Purpose and Environment	5968

A.19.6.7.1.2	Test Parameters	5968

A.19.6.7.1.3	Test Requirements	5969

A.19.6.7.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	5969

A.19.6.7.2.1	Test Purpose and Environment	5969

A.19.6.7.2.2	Test Parameters	5969

A.19.6.7.2.3	Test Requirements	5970

A.19.6.8	CSI-SINR for ATG UE	5970

A.19.6.8.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	5970

A.19.6.8.1.1	Test Purpose and Environment	5970

A.19.6.8.1.2	Test Parameters	5970

A.19.6.8.1.3	Test Requirements	5970

A.19.6.8.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	5971

A.19.6.8.2.1	Test Purpose and Environment	5971

A.19.6.8.2.2	Test Parameters	5971

A.19.6.8.2.3	Test Requirements	5971

Annex B (normative):	Conditions for RRM requirements applicability for operating bands	5972

B.1	Conditions for NR RRC_IDLE state mobility	5972

B.1.1	Introduction	5972

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	5972

B.1.2A	Conditions for measurements on NR intra-frequency cells under CCA for cell re-selection	5974

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	5975

B.1.3A	Conditions for measurements on NR inter-frequency cells under CCA for cell re-selection	5975

B.1.4	Conditions for measurements on NR intra-frequency cells for cell re-selection for RedCap	5975

B.1.5	Conditions for measurements on NR inter-frequency cells for cell re-selection for RedCap	5978

B.1.6	Conditions for measurements on NR intra-frequency cells for cell re-selection for satellite access	5978

B.1.7	Conditions for measurements on NR inter-frequency cells for cell re-selection for satellite access	5978

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	5978

B.2.1	Introduction	5978

B.2.1.1	General	5978

B.2.1.2	Derivation of Minimum SSB_RP values for FR1	5979

B.2.1.3	Derivation of Minimum SSB_RP values for FR2	5979

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival	5979

B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage	5980

B.2.1.4	Gain to SS-RSRP and CSI-RSRP measurement point for FR1	5980

B.2.1.5	Gain to SS-RSRP and CSI-RSRP measurement point for FR2	5980

B.2.1.5.1	Gain to SS-RSRP and CSI-RSRP measurement point for Rx Beam Peak angle of arrival	5980

B.2.1.5.2	Gain to SS-RSRP measurement point for different frequency	5981

B.2.1.5.3	Alignment of Rough beam to Rx beam Peak	5981

B.2.1.6	Gain to PRS-RSRP measurement point for FR2	5982

B.2.1.6.1	Gain to PRS-RSRP measurement point for Rx Beam Peak angle of arrival	5982

B.2.1.7	Derivation of Minimum SSB_RP values for FR2-NTN for satellite access	5982

B.2.1.7.1	Minimum SSB_RP values for Rx Beam	5982

B.2.1.8	Gain to SS-RSRP for FR2-NTN for satellite access	5983

B.2.2	Conditions for NR intra-frequency measurements	5984

B.2.3	Conditions for NR inter-frequency measurements	5986

B.2.4	Conditions for NR L1-RSRP reporting	5988

B.2.4.1	Conditions for SSB based L1-RSRP reporting	5988

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	5990

B.2.5	Conditions for RRC connection release with redirection to NR	5992

B.2.6	Void	5994

B.2.6.1	Void	5994

B.2.6.2	Void	5994

B.2.7	Conditions for SRS-RSRP measurements	5994

B.2.8	Conditions for NR L1-SINR reporting	5995

B.2.8.1	Conditions for L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	5995

B.2.8.2	Conditions for L1-SINR reporting with SSB based CMR and dedicated IMR configured	5997

B.2.8.2.1	L1-SINR reporting with SSB based CMR and dedicated ZP-IMR configured	5997

B.2.8.2.2	L1-SINR reporting with SSB based CMR and dedicated NZP-IMR configured	5999

B.2.8.3	Conditions for L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	6001

B.2.8.3.1	L1-SINR reporting with CSI-RS based CMR and dedicated ZP-IMR configured	6001

B.2.8.3.2	L1-SINR reporting with CSI-RS based CMR and dedicated NZP-IMR configured	6003

B.2.9	Conditions for NR intra-frequency measurements under CCA	6005

B.2.10	Conditions for NR inter-frequency measurements under CCA	6005

B.2.11	Conditions for NR L1-RSRP reporting under CCA	6005

B.2.11.1	Conditions for SSB based L1-RSRP reporting	6005

B.2.12	Conditions for NR CSI-RS based intra-frequency measurements	6006

B.2.13	Conditions for NR CSI-RS based inter-frequency measurements	6007

B.2.14	Conditions for NR PRS-based measurements	6008

B.2.15	Conditions for NR intra-frequency measurements for RedCap	6010

B.2.16	Conditions for NR inter-frequency measurements for RedCap	6011

B.2.17	Conditions for NR intra-frequency measurements for satellite access	6013

B.2.18	Conditions for NR inter-frequency measurements for satellite access	6013

B.2.19	Conditions for NR L1-RSRP reporting for satellite access	6013

B.2.19.1	Conditions for SSB based L1-RSRP reporting for satellite access	6014

B.2.19.2	Conditions for CSI-RS based L1-RSRP reporting for satellite access	6014

B.2.20	Conditions for RRC connection release with redirection to NR for satellite access	6014

B.3	RRM Requirements Exceptions	6014

B.3.1	Introduction	6014

B.3.2	Receiver sensitivity relaxation for CA	6015

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	6015

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	6015

B.3.2.2.1	Inter-band carrier aggregation	6015

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	6015

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	6015

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	6015

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	6016

B.3.2.4.1	Intra-band contiguous carrier aggregation	6016

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	6016

B.3.3	Receiver sensitivity relaxation for DC	6016

B.3.3.1	Receiver sensitivity relaxation for EN-DC	6016

B.3.3.2	Receiver sensitivity relaxation for NE-DC	6016

B.3.4	Receiver sensitivity relaxation for SUL	6016

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	6016

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	6016

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	6016

B.4	Conditions for V2X	6017

B.4.1	Test parameters for GNSS signals	6017

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	6017

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	6017

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	6018

B.4.5	Conditions for PSBCH-RSRP Accuracy Requirements under CCA	6018

B.4.6	Conditions for Selection/Reselection to Intra-frequency SyncRef UE under CCA	6018

B.4.7	Conditions for L1 SL-RSRP Accuracy Requirements under CCA	6019

B.4A	Conditions for NR Sidelink Positioning Measurement Procedures and Performance Requirements	6019

B.4A.1	Conditions for NR SL-PRS based measurements	6019

B.5	High level test procedure for SAN RRM tests	6019

Annex C (informative):	Change history	6021