| 3GPP TR 38.717-01-01 V17.0.0 (2022-06) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Networks; NR intra band Carrier Aggregation for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y). (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Background	8

4.1	TR maintenance	9

5	Intra-Band Contiguous Carrier Aggregation FR1: Specific Band Combination Part	9

5.1	CA_n96 DL_n96UL	9

5.1.1	Channel bandwidths per operating band for CA	9

5.1.2	UE co-existence studies	9

5.1.3	REFSENS	9

5.2	CA_2DL_n2B_1UL_n2A	9

5.2.1	Channel bandwidths per operating band for CA	9

5.2.2	UE maximum output power for Intra-band contiguous CA	10

5.2.3	UE additional maximum output power reduction for CA	10

5.2.4	Spurious emissions for UE co-existence for intra-band contiguous CA	10

5.2.5	Reference sensitivity power level for Intra-band contiguous CA	10

5.2.6	In-band blocking	10

5.2.7	Out-of-band blocking	11

5.2.8	Narrow band blocking	11

5.3	CA_2DL_n25B_1UL_n25A	11

5.3.1	Channel bandwidths per operating band for CA	11

5.3.2	UE maximum output power for Intra-band contiguous CA	12

5.3.3	UE additional maximum output power reduction for CA	12

5.3.4	Spurious emissions for UE co-existence for intra-band contiguous CA	12

5.3.5	Reference sensitivity power level for Intra-band contiguous CA	12

5.3.6	In-band blocking	12

5.3.7	Out-of-band blocking	13

5.3.8	Narrow band blocking	13

5.4	CA_2DL_n77B_1UL_n77A	13

5.4.1	Channel bandwidths per operating band for CA	13

5.4.2	UE maximum output power for Intra-band contiguous CA	14

5.4.3	UE additional maximum output power reduction for CA	14

5.4.4	Spurious emissions for UE co-existence for intra-band contiguous CA	14

5.4.5	Reference sensitivity power level for Intra-band contiguous CA	14

5.4.6	In-band blocking	14

5.4.7	Out-of-band blocking	14

5.4.8	Narrow band blocking	14

5.5	CA_2DL_n40B_2UL_n40B	14

5.5.1	Channel bandwidths per operating band for CA	14

5.5.2	UE maximum output power for Intra-band contiguous CA	15

5.5.3	UE additional maximum output power reduction for CA	15

5.5.4	Spurious emissions for UE co-existence for intra-band contiguous CA	15

5.5.5	Reference sensitivity power level for Intra-band contiguous CA	16

5.5.6	In-band blocking	16

5.5.7	Out-of-band blocking	16

5.5.8	Narrow band blocking	16

5.6	CA_2DL_n3B_1UL_n3A	16

5.6.1	Channel bandwidths per operating band for CA	16

5.6.2	UE maximum output power for Intra-band contiguous CA	16

5.6.3	UE additional maximum output power reduction for CA	16

5.6.4	Spurious emissions for UE co-existence for intra-band contiguous CA	16

5.6.5	Reference sensitivity power level for Intra-band contiguous CA	17

5.6.6	In-band blocking	17

5.6.7	Out-of-band blocking	17

5.6.8	Narrow band blocking	18

5.7	CA_2DL_n38B_1UL_n38A	18

5.7.1	Channel bandwidths per operating band for CA	18

5.7.2	UE maximum output power for Intra-band contiguous CA	18

5.7.3	UE additional maximum output power reduction for CA	19

5.7.4	Spurious emissions for UE co-existence for intra-band contiguous CA	19

5.7.5	Reference sensitivity power level for Intra-band contiguous CA	19

5.7.6	In-band blocking	19

5.7.7	Out-of-band blocking	19

5.7.8	Narrow band blocking	20

6	Intra-Band Non-Contiguous Carrier Aggregation FR1: Specific Band Combination Part	20

6.1	CA_2DL_n71(2A)_1UL_n71A	20

6.1.1	Channel bandwidths per operating band for CA	20

6.1.2	UE co-existence studies	20

6.1.3	REFSENS	20

6.2	CA_2DL_n2(2A)_1UL_n2A	21

6.2.1	Channel bandwidths per operating band for CA	21

6.2.2	Co-existence studies	21

6.2.3	REFSENS	21

6.3	CA_2DL_n5(2A)_1UL_n5A	22

6.3.1	Channel bandwidths per operating band for CA	22

6.3.2	UE co-existence studies	22

6.3.3	REFSENS	22

6.4	CA_3DL_n77(3A)_1UL_n77A	23

6.4.1	Channel bandwidths per operating band for CA	23

6.4.2	Co-existence studies	23

6.4.3	REFSENS	23

6.5	CA_2DL_n96(2A)_1UL_n96A	23

6.5.1	Channel bandwidths per operating band for CA	23

6.5.2	UE co-existence studies	23

6.5.3	REFSENS	23

6.6	CA_2DL_n96(3A)_1UL_n96A	24

6.6.1	Channel bandwidths per operating band for CA	24

6.6.2	UE co-existence studies	24

6.6.3	REFSENS	24

6.7	CA_2DL_n96(4A)_1UL_n96A	24

6.7.1	Channel bandwidths per operating band for CA	24

6.7.2	UE co-existence studies	24

6.7.3	REFSENS	24

6.8	CA_2DL_n12(2A)_1UL_n12A	25

6.8.1	Channel bandwidths per operating band for CA	25

6.8.2	UE maximum output power for Intra-band contiguous CA	25

6.8.3	UE additional maximum output power reduction for CA	25

6.8.4	Spurious emissions for UE co-existence for intra-band contiguous CA	25

6.8.5	Reference sensitivity power level for Intra-band contiguous CA	25

6.8.6	In-band blocking	26

6.8.7	Out-of-band blocking	26

6.8.8	Narrow band blocking	26

6.9	CA_2DL_n25(2A)_1UL_n25A CA_3DL_n25(3A)_1UL_n25A	26

6.9.1	Channel bandwidths per operating band for CA	26

6.9.2	REFSENS	27

6.10	CA_2DL_n41(2A)_1UL_n41A CA_2DL_n41(A-C)_1UL_n41A CA_3DL_n41(3A)_1UL_n41A	27

6.10.1	Channel bandwidths per operating band for CA	27

6.10.2	REFSENS	27

6.11	CA_2DL_n1(2A)_1UL_n1A	28

6.11.1	Channel bandwidths per operating band for CA	28

6.11.2	Co-existence studies	28

6.11.3	REFSENS	28

7	Intra-Band Contiguous Carrier Aggregation FR2: Specific Band Combination Part	28

7.1	CA_xDL_a_yUL_b	28

7.1.1	Channel bandwidths per operating band for CA	28

7.1.2	UE co-existence studies	28

8	Intra-Band Non-Contiguous Carrier Aggregation FR2: Specific Band Combination Part	28

8.1	CA_xDL_a-a_yUL_b-b	28

8.1.1	Channel bandwidths per operating band for CA	28

8.1.2	UE co-existence studies	29

Annex A - Change history	30

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is a technical report for NR Intra-band Carrier Aggregation Rel-17 for xDL/yUL including contiguous and non-contiguous spectrum under Rel-17 time frame. The purpose is to gather the relevant background information and studies in order to address NR Intra-band Carrier Aggregation requirements for the Rel-17 band combinations requested by proponents and captured in the WID..

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-200663, “New WID: NR intra band Carrier Aggregation for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y)”, RAN#88-e

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Background

The present document is a technical report for NR Intra-band Carrier Aggregation under Rel-17 timeframe. The document covers each band combination specific issues (i.e. one sub-clause defined per band combination)

## 4.1	TR maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

# 5	Intra-Band Contiguous Carrier Aggregation FR1: Specific Band Combination Part

## 5.1	CA_n96 DL_n96UL

### 5.1.1	Channel bandwidths per operating band for CA

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink NR CA configurations | channel bandwidths for carrier [MHz] | channel bandwidths for carrier [MHz] | channel bandwidths for carrier [MHz] | channel bandwidths for carrier [MHz] | channel bandwidths for carrier [MHz] | Maximum aggregated bandwidth | Bandwidth combination set |
| CA_n96B | CA_n96B | 20, 40 | 20, 40, 60, 80 |  |  |  | 100 | 0 |
| CA_n96C | CA_n96C | 80 | 40, 60, 80 |  |  |  | 160 | 0 |
| CA_n96D | - | 80 | 80 | 60, 80 |  |  | 240 | 0 |
| CA_n96E | - | 80 | 80 | 80 | 80 |  | 320 | 0 |

### 5.1.2	UE co-existence studies

There are no co-existence issues for this combination.

### 5.1.3	REFSENS

Considering the receiver design for the higher operation frequency with for the band. And for CA_n96E, there may be gain ripple due to larger aggregation bandwidth configuration than ever, we suggest to allow [0.5]dB REFSENS relaxation as below:

Table 5.1.3-1: ΔRIB,c due to CA

| Inter-band CA combination | Operating Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n96E | n96 | 0.5 |

## 5.2	CA_2DL_n2B_1UL_n2A

### 5.2.1	Channel bandwidths per operating band for CA

Table 5.2A.1-1: Intra-band contiguous CA operating bands in FR1

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n2 | n2 |

Table 5.5A.1-1: NR CA configurations and bandwidth combination sets defined for intra-band contiguous CA

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n2B | - | 5 | 15 |  |  |  | 20 | 0 |
|  |  | 10 | 10 |  |  |  |  |  |

### 5.2.2	UE maximum output power for Intra-band contiguous CA

Not needed as uplink is single CC.

### 5.2.3	UE additional maximum output power reduction for CA

Not needed as uplink is single CC.

### 5.2.4	Spurious emissions for UE co-existence for intra-band contiguous CA

Not needed as uplink is single CC.

### 5.2.5	Reference sensitivity power level for Intra-band contiguous CA

Not needed as PCC REFSENS is same as single carrier REFSENS and DL SCC is further away from UL than DL PCC.

### 5.2.6	In-band blocking

Table 7.6A.2.1-2a: In-band blocking for intra-band contiguous CA with FDL_low  < 2700 MHz and FUL_low  < 2700 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |  |
| n2 | Finterferer (offset) | MHz | -BWchannel CA/2 –FIoffset, case 1 and BWchannel CA/2 +FIoffset, case 1 | ≤ -BWchannel CA/2 –FIoffset, case 2 and ≥ BWchannel CA/2 +FIoffset, case 2 |  |
|  | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  |
|  | Finterferer | MHz | NOTE 2 | FDL_low – 12 to FDL_high + 15 |  |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWchannel CA/2 – FIoffset, case 1; b: BWchannel CA/2 + FIoffset, case 1 NOTE 3: BWchannel CA denotes the aggregated channel bandwidth of the wanted signal NOTE 4: n48 follows the requirement in this frequency range according to the general requirement defined in Clause 7.1A. |  |  |  |  |  |

### 5.2.7	Out-of-band blocking

Table 7.6A.3-2: Out of-band blocking for intra-band contiguous CA

| NR band | Parameter | Unit | Range1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -45 | -30 | -15 |
| n2 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: BWChannel_CA denotes the aggregated channel bandwidth of the wanted signal NOTE 3: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 2700 MHz and FInterferer < 4800 MHz. For BWChannel_CA > 15 MHz, the requirement for Range 1 is not applicable and Range 2 applies from the frequency offset of 3*BWChannel_CA from the band edge. For BWChannel_CA larger than 60 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3*BWChannel_CA from the band edge. NOTE 4: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 3650 MHz and FInterferer < 5750 MHz. For BWChannel_CA≥ 40 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3*BWChannel_CA from the band edge. NOTE 5: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2700 MHz and FInterferer < 4800 MHz |  |  |  |  |  |

### 5.2.8	Narrow band blocking

Table 7.6A.4.1-1: Narrow-band blocking for intra-band contiguous CA

| NR band | Parameter | Unit | NR CA bandwidth class |  |
| --- | --- | --- | --- | --- |
|  |  |  | B | C |
| n2 | Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + NR CA Bandwidth Class specific value below |  |
|  |  |  | 16 | 16 |
|  | Puw (CW) | dBm | -55 | -55 |
|  | Fuw (offset forDf = 15 kHz, 30 kHz) | MHz | - Foffset – 0.2 / + Foffset + 0.2 | - Foffset – 0.2 / + Foffset + 0.2 |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.3.2 and A3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The PREFSENS power level is specified in Table 7.3.2-1 and Table 7.3.2-2 for two and four antenna ports, respectively. NOTE 4: The Fuw (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |

## 5.3	CA_2DL_n25B_1UL_n25A

### 5.3.1	Channel bandwidths per operating band for CA

Table 5.2A.1-1: Intra-band contiguous CA operating bands in FR1

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n25 | n25 |

Table 5.5A.1-1: NR CA configurations and bandwidth combination sets defined for intra-band contiguous CA

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n25B | - | 5 | 15 |  |  |  | 20 | 0 |
|  |  | 10 | 10 |  |  |  |  |  |

### 5.3.2	UE maximum output power for Intra-band contiguous CA

Not needed as uplink is single CC.

### 5.3.3	UE additional maximum output power reduction for CA

Not needed as uplink is single CC.

### 5.3.4	Spurious emissions for UE co-existence for intra-band contiguous CA

Not needed as uplink is single CC.

### 5.3.5	Reference sensitivity power level for Intra-band contiguous CA

Not needed as PCC REFSENS is same as single carrier REFSENS and DL SCC is further away from UL than DL PCC.

### 5.3.6	In-band blocking

Table 7.6A.2.1-2a: In-band blocking for intra-band contiguous CA with FDL_low  < 2700 MHz and FUL_low  < 2700 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |  |
| n25 | Finterferer (offset) | MHz | -BWchannel CA/2 –FIoffset, case 1 and BWchannel CA/2 +FIoffset, case 1 | ≤ -BWchannel CA/2 –FIoffset, case 2 and ≥ BWchannel CA/2 +FIoffset, case 2 |  |
|  | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  |
|  | Finterferer | MHz | NOTE 2 | FDL_low – 12 to FDL_high + 15 |  |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWchannel CA/2 – FIoffset, case 1; b: BWchannel CA/2 + FIoffset, case 1 NOTE 3: BWchannel CA denotes the aggregated channel bandwidth of the wanted signal NOTE 4: n48 follows the requirement in this frequency range according to the general requirement defined in Clause 7.1A. |  |  |  |  |  |

### 5.3.7	Out-of-band blocking

Table 7.6A.3-2: Out of-band blocking for intra-band contiguous CA

| NR band | Parameter | Unit | Range1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -45 | -30 | -15 |
| n25 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: BWChannel_CA denotes the aggregated channel bandwidth of the wanted signal NOTE 3: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 2700 MHz and FInterferer < 4800 MHz. For BWChannel_CA > 15 MHz, the requirement for Range 1 is not applicable and Range 2 applies from the frequency offset of 3*BWChannel_CA from the band edge. For BWChannel_CA larger than 60 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3*BWChannel_CA from the band edge. NOTE 4: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 3650 MHz and FInterferer < 5750 MHz. For BWChannel_CA≥ 40 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3*BWChannel_CA from the band edge. NOTE 5: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2700 MHz and FInterferer < 4800 MHz |  |  |  |  |  |

### 5.3.8	Narrow band blocking

Table 7.6A.4.1-1: Narrow-band blocking for intra-band contiguous CA

| NR band | Parameter | Unit | NR CA bandwidth class |  |
| --- | --- | --- | --- | --- |
|  |  |  | B | C |
| n25 | Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + NR CA Bandwidth Class specific value below |  |
|  |  |  | 16 | 16 |
|  | Puw (CW) | dBm | -55 | -55 |
|  | Fuw (offset forDf = 15 kHz, 30 kHz) | MHz | - Foffset – 0.2 / + Foffset + 0.2 | - Foffset – 0.2 / + Foffset + 0.2 |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.3.2 and A3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The PREFSENS power level is specified in Table 7.3.2-1 and Table 7.3.2-2 for two and four antenna ports, respectively. NOTE 4: The Fuw (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |

## 5.4	CA_2DL_n77B_1UL_n77A

### 5.4.1	Channel bandwidths per operating band for CA

Table 5.2A.1-1: Intra-band contiguous CA operating bands in FR1

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n77 | n77 |

Table 5.5A.1-1: NR CA configurations and bandwidth combination sets defined for intra-band contiguous CA

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n77B | - | 20 | 25, 30, 40 |  |  |  | 60 | 0 |
|  |  | 25 | 30 |  |  |  |  |  |

### 5.4.2	UE maximum output power for Intra-band contiguous CA

Not needed as uplink is single CC.

### 5.4.3	UE additional maximum output power reduction for CA

Not needed as uplink is single CC.

### 5.4.4	Spurious emissions for UE co-existence for intra-band contiguous CA

Not needed as uplink is single CC.

### 5.4.5	Reference sensitivity power level for Intra-band contiguous CA

Not needed as PCC REFSENS is same as single carrier REFSENS and DL SCC is further away from UL than DL PCC.

### 5.4.6	In-band blocking

Already specified for n77.

### 5.4.7	Out-of-band blocking

Already specified for n77.

### 5.4.8	Narrow band blocking

Not needed for n77.

## 5.5	CA_2DL_n40B_2UL_n40B

### 5.5.1	Channel bandwidths per operating band for CA

Table 5.5.1-1: Intra-band contiguous CA operating bands in FR1

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n40 | n40 |

Table 5.5.1-2: NR CA configurations and bandwidth combination sets defined for intra-band contiguous CA

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n40B | CA_n40B | 10,15, 20, 30, 40, 50, 60, 80 | 10, 15, 20, 30, 40, 50, 60, 80 |  |  |  | 100 | 1 |

### 5.5.2	UE maximum output power for Intra-band contiguous CA

Table 5.5.2-1: UE Power Class for intra-band contiguous CA

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n40B |  |  |  |  | 23 | +2/-21 |  |  |
| NOTE 1: If all transmitted resource blocks over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 3: For intra-band contiguous carrier aggregation the maximum power requirement shall apply to the total transmitted power over all component carriers (per UE). |  |  |  |  |  |  |  |  |

### 5.5.3	UE additional maximum output power reduction for CA

Not needed as there are no additional emission requirements.

### 5.5.4	Spurious emissions for UE co-existence for intra-band contiguous CA

Table 5.5.4-1: Requirements for uplink intra-band contiguous carrier aggregation

| NR CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n40 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 74, 75, 76, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 44 |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. NOTE 44: As exceptions, for 90 and 100 MHz aggregated bandwidth, -40 dBm/MHz is applicable in the frequency range of 2496 – 2505 MHz. |  |  |  |  |  |  |  |

### 5.5.5	Reference sensitivity power level for Intra-band contiguous CA

Not needed as TDD band.

### 5.5.6	In-band blocking

Already captured in specification for n40

### 5.5.7	Out-of-band blocking

Already captured in specification for n40

### 5.5.8	Narrow band blocking

Already captured in specification for n40

## 5.6	CA_2DL_n3B_1UL_n3A

### 5.6.1	Channel bandwidths per operating band for CA

Table 5.6.1-1: Intra-band contiguous CA operating bands in FR1

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n3 | n3 |

Table 5.6.1-1: NR CA configurations and bandwidth combination sets defined for intra-band contiguous CA

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n3B | - | 5 | 15, 20, 25, 30 |  |  |  | 60 | 0 |
|  |  | 10 | 10, 15, 20, 25, 30 |  |  |  |  |  |
|  |  | 15, 20, 25, 30 | 5, 10, 15, 20, 25, 30 |  |  |  |  |  |

### 5.6.2	UE maximum output power for Intra-band contiguous CA

Not needed as uplink is single CC.

### 5.6.3	UE additional maximum output power reduction for CA

Not needed as uplink is single CC.

### 5.6.4	Spurious emissions for UE co-existence for intra-band contiguous CA

Not needed as uplink is single CC.

### 5.6.5	Reference sensitivity power level for Intra-band contiguous CA

Based on the REFSENS requirements for intra-hand contiguous CA specified in clause 7.3A.2.1 from TS 38.101-1, exceptions are not needed as PCC REFSENS is same as single carrier REFSENS and DL SCC is further away from UL than DL PCC.

### 5.6.6	In-band blocking

Table 5.6.6-1: In-band blocking for intra-band contiguous CA with FDL_low  < 2700 MHz and FUL_low  < 2700 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |  |
| n3 | Finterferer (offset) | MHz | -BWchannel CA/2 –FIoffset, case 1 and BWchannel CA/2 +FIoffset, case 1 | ≤ -BWchannel CA/2 –FIoffset, case 2 and ≥ BWchannel CA/2 +FIoffset, case 2 |  |
|  | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  |
|  | Finterferer | MHz | NOTE 2 | FDL_low – 12 to FDL_high + 15 |  |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWchannel CA/2 – FIoffset, case 1; b: BWchannel CA/2 + FIoffset, case 1 NOTE 3: BWchannel CA denotes the aggregated channel bandwidth of the wanted signal NOTE 4: n48 follows the requirement in this frequency range according to the general requirement defined in Clause 7.1A. |  |  |  |  |  |

### 5.6.7	Out-of-band blocking

Table 5.6.7-1: Out of-band blocking for intra-band contiguous CA

| NR band | Parameter | Unit | Range1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -45 | -30 | -15 |
| n3 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: BWChannel_CA denotes the aggregated channel bandwidth of the wanted signal NOTE 3: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 2700 MHz and FInterferer < 4800 MHz. For BWChannel_CA > 15 MHz, the requirement for Range 1 is not applicable and Range 2 applies from the frequency offset of 3*BWChannel_CA from the band edge. For BWChannel_CA larger than 60 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3*BWChannel_CA from the band edge. NOTE 4: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 3650 MHz and FInterferer < 5750 MHz. For BWChannel_CA≥ 40 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3*BWChannel_CA from the band edge. NOTE 5: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2700 MHz and FInterferer < 4800 MHz |  |  |  |  |  |

### 5.6.8	Narrow band blocking

Table 5.6.8-1: Narrow-band blocking for intra-band contiguous CA

| NR band | Parameter | Unit | NR CA bandwidth class |  |
| --- | --- | --- | --- | --- |
|  |  |  | B | C |
| n3 | Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + NR CA Bandwidth Class specific value below |  |
|  |  |  | 16 | 16 |
|  | Puw (CW) | dBm | -55 | -55 |
|  | Fuw (offset forDf = 15 kHz, 30 kHz) | MHz | - Foffset – 0.2 / + Foffset + 0.2 | - Foffset – 0.2 / + Foffset + 0.2 |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.3.2 and A3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The PREFSENS power level is specified in Table 7.3.2-1 and Table 7.3.2-2 for two and four antenna ports, respectively. NOTE 4: The Fuw (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |

## 5.7	CA_2DL_n38B_1UL_n38A

### 5.7.1	Channel bandwidths per operating band for CA

Table 5.7.1-1: Intra-band contiguous CA operating bands in FR1

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n38 | n38 |

Table 5.7.1-1: NR CA configurations and bandwidth combination sets defined for intra-band contiguous CA

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n38B | - | 5 | 15, 20, 25 |  |  |  | 50 | 0 |
|  |  | 10 | 10, 15, 20, 25 |  |  |  |  |  |
|  |  | 15, 20, 25 | 5, 10, 15, 20, 25 |  |  |  |  |  |

### 5.7.2	UE maximum output power for Intra-band contiguous CA

Not needed as uplink is single CC.

### 5.7.3	UE additional maximum output power reduction for CA

Not needed as uplink is single CC.

### 5.7.4	Spurious emissions for UE co-existence for intra-band contiguous CA

Not needed as uplink is single CC.

### 5.7.5	Reference sensitivity power level for Intra-band contiguous CA

There is no REFSENS exception for this TDD intra-band contiguous CA band combination.

### 5.7.6	In-band blocking

Table 5.7.6-1: In-band blocking for intra-band contiguous CA with FDL_low  < 2700 MHz and FUL_low  < 2700 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |  |
| n38 | Finterferer (offset) | MHz | -BWchannel CA/2 –FIoffset, case 1 and BWchannel CA/2 +FIoffset, case 1 | ≤ -BWchannel CA/2 –FIoffset, case 2 and ≥ BWchannel CA/2 +FIoffset, case 2 |  |
|  | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  |
|  | Finterferer | MHz | NOTE 2 | FDL_low – 12 to FDL_high + 15 |  |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWchannel CA/2 – FIoffset, case 1; b: BWchannel CA/2 + FIoffset, case 1 NOTE 3: BWchannel CA denotes the aggregated channel bandwidth of the wanted signal NOTE 4: n48 follows the requirement in this frequency range according to the general requirement defined in Clause 7.1A. |  |  |  |  |  |

### 5.7.7	Out-of-band blocking

Table 5.7.7-1: Out of-band blocking for intra-band contiguous CA

| NR band | Parameter | Unit | Range1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -45 | -30 | -15 |
| n38 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: BWChannel_CA denotes the aggregated channel bandwidth of the wanted signal NOTE 3: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 2700 MHz and FInterferer < 4800 MHz. For BWChannel_CA > 15 MHz, the requirement for Range 1 is not applicable and Range 2 applies from the frequency offset of 3*BWChannel_CA from the band edge. For BWChannel_CA larger than 60 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3*BWChannel_CA from the band edge. NOTE 4: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 3650 MHz and FInterferer < 5750 MHz. For BWChannel_CA≥ 40 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3*BWChannel_CA from the band edge. NOTE 5: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2700 MHz and FInterferer < 4800 MHz |  |  |  |  |  |

### 5.7.8	Narrow band blocking

Table 5.7.8-1: Narrow-band blocking for intra-band contiguous CA

| NR band | Parameter | Unit | NR CA bandwidth class |  |
| --- | --- | --- | --- | --- |
|  |  |  | B | C |
| n38 | Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + NR CA Bandwidth Class specific value below |  |
|  |  |  | 16 | 16 |
|  | Puw (CW) | dBm | -55 | -55 |
|  | Fuw (offset forf = 15 kHz, 30 kHz) | MHz | - Foffset – 0.2 / + Foffset + 0.2 | - Foffset – 0.2 / + Foffset + 0.2 |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.3.2 and A3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The PREFSENS power level is specified in Table 7.3.2-1 and Table 7.3.2-2 for two and four antenna ports, respectively. NOTE 4: The Fuw (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |

# 6	Intra-Band Non-Contiguous Carrier Aggregation FR1: Specific Band Combination Part

## 6.1	CA_2DL_n71(2A)_1UL_n71A

### 6.1.1	Channel bandwidths per operating band for CA

Table 6.1.1-1: Supported bandwidth combinations for CA_n71(2A)

| NR CA Configuration | Uplink Configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum Aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A) | - | 5,10,15,20 | 5,10,15, 20 |  |  | 30 | 0 |

### 6.1.2	UE co-existence studies

There are no co-existence issues for this combination.

### 6.1.3	REFSENS

REFSENS for CA_n71(2A) need to be added in below table of TS 38.101-1. MSD values proposed are tentative values for the RAN4 #96 meeting, and these will be crosschecked and to be concluded at the following RAN4 meeting.

Table 7.3A.2.2-1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity

| CA configuration | SCS (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A) | 15 | 25RB+25RB | Wgap = 25.0 | 5 | 4.0 | FDD |
|  |  |  | Wgap = 5.0 | 20 | 0.0 |  |
|  |  | 50RB+25RB | Wgap = 20.0 | 51 | 4.6 |  |
|  |  |  | Wgap = 5.0 | 201 | 2.3 |  |
|  |  | 75RB+50RB | Wgap = 10.0 | 52 | 22.2 |  |
|  |  |  | Wgap = 5.0 | 203 | 5.2 |  |
| Note 1: Uplink resource block starts at RB postion [9] for SCS=15KHz. Note 2: Uplink resource block starts at RB postion [2] for SCS=15KHz. Note 3: Uplink resource block starts at RB postion [19] for SCS=15KHz. |  |  |  |  |  |  |

## 6.2	CA_2DL_n2(2A)_1UL_n2A

### 6.2.1	Channel bandwidths per operating band for CA

Table 6.2.1-1: Supported bandwidth combinations for CA_2DL_n2(2A)_1UL _n2A

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_n2(2A) | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  |  | 40 | 0 |

### 6.2.2	Co-existence studies

There are no co-existence issues for this combination.

### 6.2.3	REFSENS

REFSENS can be impacted by the PCC UL being closer to do the SCC DL than the nominal spacing. REFSENS values are same as for CA_n2(2A).

| CA configuration | SCS kHz | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n2(2A) | 15 | 25RB+25RB | Wgap = 55.0 | 101 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 2: Wgap is the sub-block gap between the two sub-blocks. NOTE 3: The carrier centre frequency of SCC in the DL operating band is configured closer to the UL operating band. |  |  |  |  |  |  |

## 6.3	CA_2DL_n5(2A)_1UL_n5A

### 6.3.1	Channel bandwidths per operating band for CA

Table 6.3.1-1: Supported bandwidth combinations for CA_n5(2A)

| NR CA Configuration | Uplink Configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum Aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5(2A) | - | 5,10,15,20 | 5,10,15, 20 |  |  | 25 | 0 |

### 6.3.2	UE co-existence studies

There are no co-existence issues for this combination.

### 6.3.3	REFSENS

Below analysis assumptions are used in the REFSENS analysis.

| Parameter | Value | Unit |
| --- | --- | --- |
| CIM5 | -70 | dBc |
| n5 filter T/R isolation | 53 | dB |
| Front-end loss | 4 | dB |
| Thermal noise at n5 RX ANT port | -165 | dBm/Hz |
| Transceiver effective phase noise | -140 | dBc/Hz |
| SNR requirement for QPSK | -1 | dB |

REFSENS for CA_n5(2A) need to be added in below table of TS 38.101-1.

Table 7.3A.2.2-1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity

| CA configuration | SCS (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n5(2A) | 15 | 75RB + 25RB | Wgap = 5.0 | 55 | 6.3 | FDD |
| NOTE 5: Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. |  |  |  |  |  |  |

## 6.4	CA_3DL_n77(3A)_1UL_n77A

### 6.4.1	Channel bandwidths per operating band for CA

Table 6.4.1-1: Supported bandwidth combinations for CA_3DL_n77(3A)_1UL _n77A

| NR CA Configuration | Uplink Configurations | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n77(3A) | - | 20, 40, 80, 100 | 20, 40, 80, 100 | 20, 40, 80, 100 | 300 | 0 |
|  |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 300 | 1 |

### 6.4.2	Co-existence studies

### There are no additional co-existence issues for this combination.

### 6.4.3	REFSENS

There are no REFSENS exceptions for this combination.

## 6.5	CA_2DL_n96(2A)_1UL_n96A

### 6.5.1	Channel bandwidths per operating band for CA

Table 6.5.1-1: Supported bandwidth combinations for CA_n96(2A)

| NR CA Configuration | Uplink Configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum Aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n96(2A) | - | 20, 40, 60, 80 | 20, 40, 60, 80 |  |  | 160 | 0 |

### 6.5.2	UE co-existence studies

There are no co-existence issues for this combination.

### 6.5.3	REFSENS

There are no REFSENS exceptions for this combination.

## 6.6	CA_2DL_n96(3A)_1UL_n96A

### 6.6.1	Channel bandwidths per operating band for CA

Table 6.6.1-1: Supported bandwidth combinations for CA_n96(3A)

| NR CA Configuration | Uplink Configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum Aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n96(3A) | - | 20, 40, 60, 80 | 20, 40, 60, 80 | 20, 40, 60, 80 |  | 240 | 0 |

### 6.6.2	UE co-existence studies

There are no co-existence issues for this combination.

### 6.6.3	REFSENS

There are no REFSENS exceptions for this combination.

## 6.7	CA_2DL_n96(4A)_1UL_n96A

### 6.7.1	Channel bandwidths per operating band for CA

Table 6.7.1-1: Supported bandwidth combinations for CA_n96(4A)

| NR CA Configuration | Uplink Configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum Aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n96(4A) | - | 20, 40, 60, 80 | 20, 40, 60, 80 | 20, 40, 60, 80 | 20, 40, 60, 80 | 320 | 0 |

### 6.7.2	UE co-existence studies

There are no co-existence issues for this combination.

### 6.7.3	REFSENS

The DL NCCA with four carriers CA_n96(4A) was first required in RAN4. Considering design complexity of four non-contiguous carriers, REFSENS relaxation may be needed. One possible implantation is split receiver architecture to cover non-contiguous carriers within the whole pass band. Here we propose ΔRIBNC as below:

Table 6.7.3-1: Intra-band non-contiguous CA reference sensitivity for shared spectrum

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- |
| CA_n96(4A) | 15/30/60 | 320MHz | 0.5 | TDD |

## 6.8	CA_2DL_n12(2A)_1UL_n12A

### 6.8.1	Channel bandwidths per operating band for CA

Table 5.2A.1-1: Intra-band contiguous CA operating bands in FR1

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n12(*) | n12 |

Table 5.5A.1-1: NR CA configurations and bandwidth combination sets defined for intra-band contiguous CA

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n12(2A) | - | 5 | 5 |  |  |  | 10 | 0 |

### 6.8.2	UE maximum output power for Intra-band contiguous CA

Not needed as uplink is single CC.

### 6.8.3	UE additional maximum output power reduction for CA

Not needed as uplink is single CC.

### 6.8.4	Spurious emissions for UE co-existence for intra-band contiguous CA

Not needed as uplink is single CC.

### 6.8.5	Reference sensitivity power level for Intra-band contiguous CA

MDS is re-used from CA_12A-12A.

Table 7.3A.2.2-1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity in FDD bands.

| CA configuration | SCS kHz | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n12(2A) | 15/15 | 5MHz + 5MHz | 0.0 < Wgap ≤ 7.0 | 5(RBstart=12) | 3 | FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 2: Wgap is the sub-block gap between the two sub-blocks. NOTE 3: The carrier centre frequency of SCC in the DL operating band is configured closer to the UL operating band. |  |  |  |  |  |  |

### 6.8.6	In-band blocking

Not needed requirement refers back to single carrier requirement.

### 6.8.7	Out-of-band blocking

Not needed requirement refers back to single carrier requirement.

### 6.8.8	Narrow band blocking

Not needed requirement refers back to single carrier requirement.

## 6.9	CA_2DL_n25(2A)_1UL_n25A
CA_3DL_n25(3A)_1UL_n25A

### 6.9.1	Channel bandwidths per operating band for CA

Table 6.9.1-1: Supported bandwidth combinations

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_n25(2A) | - | 5, 10, 15, 20, 25, 30, 40 | 5, 10, 15, 20, 25, 30, 40 |  |  |  | 60 | 1 |
| CA_n25(3A) | - | 5, 10, 15, 20, 25, 30, 40 | 5, 10, 15, 20, 25, 30, 40 | 5, 10, 15, 20, 25, 30, 40 |  |  | 55 | 0 |

### 6.9.2	REFSENS

Table 6.9.2-1: MSD values

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n25(2A) CA_n25(3A) | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | [24.6]11 | FDD |
| NOTE 11: For operation with three or more non-contiguous component carriers, ΔRIBNC applies to all secondary component carriers |  |  |  |  |  |  |

## 6.10	CA_2DL_n41(2A)_1UL_n41A
CA_2DL_n41(A-C)_1UL_n41A
CA_3DL_n41(3A)_1UL_n41A

### 6.10.1	Channel bandwidths per operating band for CA

Table 6.10.1-1: Supported bandwidth combinations for CA_2DL_n41(xA)_1UL _n41A

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_n41(2A) | - | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  | 190 | 3 |
| CA_n41(3A) | - | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  | 190 | 0 |
| CA_n41(A-C) |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | See CA_n41C Bandwidth Combination Set 2 in Table 5.5A.1-1 |  |  |  | 190 | 0 |

### 6.10.2	REFSENS

Since n41 is TDD there is no need to define additional REFSENS requirements.

## 6.11	CA_2DL_n1(2A)_1UL_n1A

### 6.11.1	Channel bandwidths per operating band for CA

Table 6.11.1-1: Supported bandwidth combinations

| NR CA Configuration | Uplink Configurations | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1(2A) | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 40 | 0 |

### 6.11.2	Co-existence studies

### There are no additional co-existence issues for this combination since it is single UL.

### 6.11.3	REFSENS

This combination requires MSD.

Table 7.3A.2.2-1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity in FDD bands.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1(2A) | 15/15 | 5MHz + 5MHz | 0.0 < Wgap ≤ 50.0 | 25 | 0.5 | FDD |

# 7	Intra-Band Contiguous Carrier Aggregation FR2: Specific Band Combination Part

## 7.1	CA_xDL_a_yUL_b

### 7.1.1	Channel bandwidths per operating band for CA

<Text will be added.>

### 7.1.2	UE co-existence studies

<Text will be added.>

# 8	Intra-Band Non-Contiguous Carrier Aggregation FR2: Specific Band Combination Part

## 8.1	CA_xDL_a-a_yUL_b-b

### 8.1.1	Channel bandwidths per operating band for CA

<Text will be added.>

### 8.1.2	UE co-existence studies

<Text will be added.>

# Annex A - Change history

| Date | Meeting | TDoc | Subject/Comment | New version |
| --- | --- | --- | --- | --- |
| 2020-08 | 3GPP RAN4#96-e | R4-2010680 | TR skeleton | 0.0.1 |
| 2020-08 | 3GPP RAN4#96-e | R4-2011888 | Implemented TP’s from RAN4 #96-e: R4-2011619, “TP to TR 38.717-01-01 to include CA_n71(2A)”, Ericsson, T-Mobile US | 0.1.0 |
| 2020-11 | 3GPP RAN4#97-e | R4-2015924 | Implemented TP’s from RAN4 #97-e: R4-2016329. “TP to TR 38.717-01-01 to include CA_n2(2A)”, Ericsson, Verizon R4-2016679, “TP to TR 38.717-01-01 to include CA_n5(2A)”, Ericsson, Verizon, MediaTek R4-2016339, “TP to TR 38.717-01-01 to update MSD values CA_n71(2A)”, Ericsson, T-Mobile US | 0.2.0 |
| 2021-02 | 3GPP RAN4#98-e | R4-2101891 | Implemented TP from RAN4 #98-e: R4-2100708, “TP for TR 38.717-01-01: CA_3DL_n77(3A)_1UL_n77A”, SoftBank Corp. | 0.3.0 |
| 2021-04 | 3GPP RAN4#98-bis-e | R4-2106704 | Implemented TP from RAN4 #98-bis-e: R4-2106648, “Updated TP for TR 38.717-01-01: CA_n77(3A)_BCS1”, Huawei, HiSilicon | 0.4.0 |
| 2021-05 | 3GPP RAN4#99-e | R4-2111079 | No new TP’s to implement from RAN4 #99-e | 0.5.0 |
| 2021-08 | 3GPP RAN4#100-e | R4-2113562 | No new TP’s to implement from RAN4 #100-e | 0.6.0 |
| 2021-11 | 3GPP RAN4#101-e | R4-2118494 | Implemented TP’s from RAN4 #101-e: R4-2119799, TP to TR 38.717.01-01 for NR intra band CA_n96, Charter Communications, Inc R4-2119804 TP to TR 38.717-01-01: CA_n2B, Nokia, US Cellular R4-2119805, TP to TR 38.717-01-01: CA_n25B, Nokia, US Cellular R4-2117583, TP to TR 38.717-01-01: CA_n77B, Nokia, US Cellular R4-2117584, TP to TR 38.717-01-01: CA_n12(2A), Nokia, US Cellular R4-2119820, TP for 38.717-01-01 to add CA_n25(2A) and CA_n25(3A), Ericsson, T-Mobile US R4-2119821, TP for 38.717-01-01 to add CA_n41(2A), CA_n41(3A) and CA_n41(A-C), Ericsson, T-Mobile US R4-2119824, TP to TR 38.717-01-01 Addition of CA_n1(2A), Nokia | 0.7.0 |
| 2022-02 | 3GPP RAN4#102-e | R4-2205684 | Implemented TP’s from RAN4 #101-bis-e: R4-2202174, TP to TR 38.717-01-01 Addition of CA_n40B, Nokia, NBN Implemented TP’s from RAN4 #102-e: R4-2206270, TP for TR 38.717-01-01 CA_n3B_BCS0, Huawei, HiSilicon R4-2206271, TP for TR 38.717-01-01 CA_n38B_BCS0, Huawei, HiSilicon | 0.8.0 |
| 2022-05 | 3GPP RAN4#103-e | R4-2209551 | No new TP’s at RAN4 #103-e | 0.9.0 |
| 2022-06 | 3GPP RAN #96 | RP-221323 | Presented for approval at RAN plenary. | 1.0.0 |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-06 | RAN#96 |  |  |  |  | Approved by plenary – Rel-17 spec under change control | 17.0.0 |