3GPP TSG-RAN WG4 Meeting #100-e	R4-2111757

Electronic Meeting, 16th – 27th August 2021

Source:	AT&T

Title:	TP for TR 37.717-21-11 Addition of DC_14-30_n77

Agenda item:			8.15.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.717-21-11 to add DC_14A-30A_n77A.

# 2. Text Proposal

---Start of changes---

## 5.X	DC_14-30_n77

### 5.X.1	Configurations for DC

Table 5.X.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_14A-30A_n77A | DC_14A_n77A DC_30A_n77A |

### 5.X.2	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fallback modes. It can be seen that:

-	IMD3 products are produced by Band 14 and n77 that might fall in Rx of band 30.

-	IMD3 and IMD5 products are produced by Band 30 and n77 that might fall in Rx of band 14.

### 5.X.3	∆TIB and ∆RIB values

For DC_14-30_n77, the TIB,c and RIB,c values are given in the tables below that reused the values for CA_n14-n30-n77.

Table 5.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_14-30_n77 | 14 | 0.5 |
|  | 30 | 0.3 |
|  | n77 | 0.8 |

Table 5.X.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_14-30_n77 | 14 | 0.2 |
|  | 30 | 0 |
|  | n77 | 0.5 |

### 5.X.4	Reference sensitivity exceptions

Table 5.X.4-1 shows the required MSD levels for the DC configuration. The required MSD values are derived from CA-n14A-n30A-n77A.

Table 5.X.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_14A-30A_n77A | 14 | 826.5 | 5 | 25 | 871.5 | 15.2 | IMD34 |
|  | 30 | 1742 | 5 | 25 | 2142 | N/A | N/A |
|  | n77 | 3795 | 10 | 50 | 3795 | N/A | N/A |
|  | 14 | 793 | 5 | 25 | 763 | N/A | N/A |
|  | 30 | 2310 | 5 | 25 | 2355 | 13.2 | IMD3 |
|  | n77 | 3941 | 10 | 50 | 3941 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

---End of changes---

# References