3GPP TSG-RAN WG4 Meeting #100-e	R4-2111819

Electronic Meeting, 16th – 27th August 2021

Source:	AT&T

Title:	TP for TR 37.826 Addition of DC_30A_n77A

Agenda item:			8.36.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.826 to add DC_30A_n77A.

# 2. Text Proposal

---Start of changes---

## 6.X	DC_30A_n77A

### 6.X.1	Transmitter Characteristics

#### 6.X.1.1	Maximum Output Power

Table 6.X.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_30A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |

#### 6.X.1.2	Co-existence study

The co-existence study for PC2 DC_30A_n77A can be reused from the PC3 DC_30A_n77A captured in TR 37.717-11-11, where:

- 4th order IMD products may fall into own Rx frequencies of band 30.

Therefore, additional MSD for IMD4 needs to be defined for PC2 DC_30A_n77A.

### 6.X.2	Receiver Characteristics

#### 6.X.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

Table 6.X.2.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_30A_n77A | 30 | 2310 | 5 | 25 | 2355 | 17.6 | IMD4 |
|  | n77 | 3487.5 | 10 | 50 | 3487.5 | N/A | N/A |

---End of changes---

# References