3GPP TSG-RAN4 Meeting #100-e	R4-2111854

# Electronic meeting,  16th Aug – 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Definition of generic channel BW configurations for RRM CA tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2021-08-23 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As raised the issue in R4-2108849, current RRM test configurations for NR cell are defined based on a fixed combination of sub-carrier spacing and channel bandwidth. However due to the increase of CA band combinations in FR1, we noticed that there are band combinations which cannot be configured based on the current RRM test configurations. e.g. CA_n71B, CA_n41C, etc. To solve this issue, we need to generalize the description of channel bandwidth in test configurations. For channel BW other than 10 MH for SSB SCS 15 kHz or 40 MHz for SSB SCS 30 kHz, define test parameters as follows. - Define channel bandwidth with “≥” mark in the test configuration tables to add flexibility to the CA bandwidth combination - Introduce a new parameter “BWoccupied” which is the actual BW allocation in each test parameter table (confined with 10 MHz or 40 MHz) - Configure PDSCH RMC with the same number of RBs as 10 MHz for 15 kHz SSB SCS or 40 MHz for 30 kHz SSB SCS - Configure CORESET for RMSI and RMC scheduling with same configurations as CBW of 10 MHz for 15 kHz SSB SCS or 40 MHz for 30 kHz SSB SCS - Fill in 10 MHz or 40 MHz channel BW with OCNG - Confine also bandwidth of Noc (AWGN) within BWoccupied. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Changed description of channel bandwidth in PDSCH RMC (A.3.1.1), CORESET for RMSI scheduling (A.3.1.2), and CORESET for RMC scheduling (A.3.1.3) to generalize the CBW that accommodates any bandwidths wider than or equal to 10 MHz or 40 MHz. - Added a condition to Note 2 in OCNG pattern 1 (A.3.2.1.1) to confine OCNG bandwidth within BWoccupied. - Modified test configuration tables in each test case and added “≥” before channel bandwidth to cover any BW wider than 10 MHz or 40 MHz. Also added note 2.Also added (BWchannel) after bandwidth. - Replaced the description of BWchannel by note and indicated to refer to Table 5.3.2-1 in TS 38.101-1 to generalize the bandwidth in cell specific test parameter tables. - Added a parameter “BWoccupied” in cell specific test parameter tables. - Added notes in cell specific test parameter tables to confine actual transmission bandwidth within either 10 MHz (52RBs) or 40 MHz (106 RBs) also to clarify that Io is independent of the BWchannel configured. - Modified note 2 in cell specific test parameter tables to also confine actual bandwidth of Noc within BWoccupied. - Added the definition of FC and FC,low in clause 3.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM test cases cannot cover specific CA band combinations which do not support 10 MHz or 40 MHz channel bandwidth. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, A.3.1.1, A.3.1.2, A.3.1.3, A.3.2.1.1, A.4.5.2.3, A.4.5.2.4, A.4.5.3.1, A.4.5.4.1, A.4.5.6.1.2, A.6.5.2.1, A.6.5.3.1, A.6.5.4.1, A.6.5.6.1.1 Isolated impact analysis: No change to UE requirements, changes test parameters only. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated discussion paper : R4-2108849, R4-2111852 Associated WF: R4-2108038 This CR is only for FR1. CR for FR2 will be submitted in the later meeting if the group can reach a consensus. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchaged sections skipped>>

<<Start of change>>

3.2	Symbols

For the purposes of the present document, the following symbols apply:

[…]	Values included in square bracket must be considered for further studies, because it means that a decision about that value was not taken.

BWChannel	Channel bandwidth, defined in TS 38.101-1, 38.101-2 and 38.101-3 subclause 3.2

Ês	Received energy per RE (power normalized to the subcarrier spacing) during the useful part of the symbol, i.e. excluding the cyclic prefix, at the UE antenna connector

FC	RF reference frequency on the channel raster, given in table 5.4.2.2-1 in TS 38.101-1 and 38.101-2

FC,low	The Fc of the lowest carrier, expressed in MHz

Io	The total received power density, including signal and interference, as measured at the UE antenna connector.

Ioc	The power spectral density (integrated in a noise bandwidth equal to the chip rate and normalized to the chip rate) of a band limited noise source (simulating interference from cells, which are not defined in a test procedure) as measured at the UE antenna connector.

Iot	The received power spectral density of the total noise and interference for a certain RE (power integrated over the RE and normalized to the subcarrier spacing) as measured at the UE antenna connector

The power spectral density of a white noise source (average power per RE normalised to the subcarrier spacing), simulating interference from cells that are not defined in a test procedure, as measured at the UE antenna connector

Physical Resource Block number as defined in clause 3.2 in TS 38.211.

Timing offset between uplink and downlink radio frames at the UE, as defined in clause 4.2 in TS 38.213.

Fixed timing advance offset, as defined in clause 7.1.2.2 in TS 38.133.

Configured UE transmitted power as defined in clause 6.2.4 in TS 38.101-1, 38-101-2 and 38.101-3.

PCMAX,c	Configured UE transmitted power on a serving cell c as defined in clause 6.2.4 in TS 38.101-1, 38-101-2 and 38.101-3

S	Cell Selection Criterion defined in TS 38.304, subclause 5.2.3.2 for NR

SSB_RP	Received (linear) average power of the resource elements that carry NR synchronisation burst, measured at the UE antenna connector

Srxlev	Cell selection RX level, defined in TS 38.304, subclause 5.2.3.2

Squal	Cell selection quality, defined in TS 38.304, subclause 5.2.3.2

Sintrasearch	Defined in TS 38.304 , subclause 5.2.4.7 for E-UTRAN amd 38.304 subclause 5.2.4.7 for NR

Snonintrasearch	Defined in TS 38.304 , subclause 5.2.4.7

Threshx, high	Defined in TS 38.304 , subclause 5.2.4.7

Threshx, low	Defined in TS 38.304 , subclause 5.2.4.7

Threshserving, low	Defined in TS 38.304 , subclause 5.2.4.7

TRE-ESTABLISH-REQ	The RRC Re-establishment delay requirement, the time between the moment when erroneous CRCs are applied, to when the UE starts to send preambles on the PRACH.

Tc	Basic time unit, defined in clause 4.1 of TS 38.211 [6].

Ts	Reference time unit, defined in clause 4.1 of TS 38.211 [6].

Treselection	Defined in TS 25.304, subclause 5.2.6.1.5

TreselectionRAT	Defined in TS 36.304 , subclause 5.2.4.7

TreselectionEUTRA	Defined in TS 36.304 , subclause 5.2.4.7

TreselectionUTRA	Defined in TS 36.304 , subclause 5.2.4.7

TreselectionGERANDefined in TS 36.304 , subclause 5.2.4.

Threshx, high	Defined in TS 38.304 , subclause 5.2.4.7

Threshx, low 	Defined in TS 38.304 , subclause 5.2.4.7

Threshserving, low	Defined in TS 38.304 , subclause 5.2.4.7

TUE_re-establish_delay	Time between the moments when any of the conditions requiring RRC re-establishment as defined in clause 5.3.7 in TS 38.331 [2] is detected by the UE and when the UE sends PRACH to the target PCell.

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.3.1	Reference measurement channels

A.3.1.1	PDSCH

A.3.1.1.1	FDD

Table A.3.1.1.1-1: PDSCH Reference Measurement Channels for SCS=15kHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | SR.1.1 FDD |  |  |  |  |  |  |
| Channel bandwidth | MHz | Defined in test case |  |  |  |  |  |  |
| Number of transmitter antennas |  | 1 |  |  |  |  |  |  |
| Allocated resource blocks for PDSCH Note 1 |  | 24 |  |  |  |  |  |  |
| Allocated slots per Radio Frame |  | 10 |  |  |  |  |  |  |
| Radio frame containing SSB | slots | Note 5 |  |  |  |  |  |  |
| Radio frame not containing SSB | slots | 10 |  |  |  |  |  |  |
| MCS index |  | 4 |  |  |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |  |  |
| Target Coding Rate |  | 1/3 |  |  |  |  |  |  |
| Number of control symbols |  | 2 |  |  |  |  |  |  |
| PDSCH mapping type |  | Type A |  |  |  |  |  |  |
| Information Bit Payload |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2 | bits | 1608 |  |  |  |  |  |  |
| For slots without RMSI | bits | 1864 |  |  |  |  |  |  |
| Number of Code Blocks per slot |  | 1 |  |  |  |  |  |  |
| Binary Channel Bits Per slot |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2, Note 4 | bits | 5184 |  |  |  |  |  |  |
| For slots without RMSI Note 6 | bits | 6048 |  |  |  |  |  |  |
| Note 1: Allocated outside the SMTC duration in time and in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. Note 2: PDSCH is scheduled on the slots with RMSI. Note 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in TS 38.213 [3]. Note 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. Note 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in clause A.3.10. Note 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. |  |  |  |  |  |  |  |  |

A.3.1.1.2	TDD

Table A.3.1.1.2-1: PDSCH Reference Measurement Channels for SCS=15kHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | SR.1.1 TDD |  |  |  |  |  |  |
| Channel bandwidth | MHz | Defined in test case |  |  |  |  |  |  |
| Number of transmitter antennas |  | 1 |  |  |  |  |  |  |
| Allocated resource blocks for PDSCH Note 1 |  | 24 |  |  |  |  |  |  |
| Allocated slots per Radio Frame |  |  |  |  |  |  |  |  |
| Radio frame containing SSB | slots | Note 5 |  |  |  |  |  |  |
| Radio frame not containing SSB | slots | 4 |  |  |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |  |  |
| MCS index |  | 4 |  |  |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |  |  |
| Target Coding Rate |  | 1/3 |  |  |  |  |  |  |
| Number of control symbols |  | 2 |  |  |  |  |  |  |
| PDSCH mapping type |  | Type A |  |  |  |  |  |  |
| Information Bit Payload |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2 | bits | 1608 |  |  |  |  |  |  |
| For slots without RMSI | bits | 1864 |  |  |  |  |  |  |
| Number of Code Blocks per slot |  | 1 |  |  |  |  |  |  |
| Binary Channel Bits Per slot |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2, Note 4 | bits | 5184 |  |  |  |  |  |  |
| For slots without RMSI Note 6 | bits | 6048 |  |  |  |  |  |  |
| Note 1: Allocated outside the SMTC duration in time and in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. Note 2: PDSCH is scheduled on the slots with RMSI. Note 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in TS 38.213 [3]. Note 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. Note 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in clause A.3.10. Note 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. |  |  |  |  |  |  |  |  |

Table A.3.1.1.2-2: PDSCH Reference Measurement Channels for SCS=30kHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | SR.2.1 TDD |  |  |  |  |  |  |
| Channel bandwidth | MHz | Defined in test case |  |  |  |  |  |  |
| Number of transmitter antennas |  | 1 |  |  |  |  |  |  |
| Allocated resource blocks for PDSCH Note 1 |  | 24 |  |  |  |  |  |  |
| Allocated slots per Radio Frame |  |  |  |  |  |  |  |  |
| Radio frame containing SSB | slots | Note 5 |  |  |  |  |  |  |
| Radio frame not containing SSB | slots | 10 |  |  |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |  |  |
| MCS index |  | 4 |  |  |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |  |  |
| Target Coding Rate |  | 1/3 |  |  |  |  |  |  |
| Number of control symbols |  | 2 |  |  |  |  |  |  |
| PDSCH mapping type |  | Type A |  |  |  |  |  |  |
| Information Bit Payload |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2 | bits | 1608 |  |  |  |  |  |  |
| For slots without RMSI | bits | 1864 |  |  |  |  |  |  |
| Number of Code Blocks per slot |  | 1 |  |  |  |  |  |  |
| Binary Channel Bits Per slot |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2, Note 4 | bits | 5184 |  |  |  |  |  |  |
| For slots without RMSI Note 6 | bits | 6048 |  |  |  |  |  |  |
| Note 1: Allocated outside the SMTC duration in time and in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. Note 2: PDSCH is scheduled on the slots with RMSI. Note 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in TS 38.213 [3]. Note 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. Note 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in clause A.3.10. Note 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1.. |  |  |  |  |  |  |  |  |

Table A.3.1.1.2-3: PDSCH Reference Measurement Channels for SCS=120kHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | SR.3.1 TDD | SR.3.2 TDD | SR.3.3 TDD |  |  |  |  |
| Channel bandwidth | MHz | 100 | 100 | 100 |  |  |  |  |
| Number of transmitter antennas |  | 1 | 1 | 1 |  |  |  |  |
| Allocated resource blocks for PDSCH |  | 24Note 1 | 24Note 7 | 48Note 7 |  |  |  |  |
| Allocated slots per Radio Frame |  |  |  |  |  |  |  |  |
| Radio frame containing SSB | slots | Note 5 | Note 5 | Note 5 |  |  |  |  |
| Radio frame not containing SSB | slots | 48 | 48 | 48 |  |  |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |  |  |
| MCS index |  | 4 | 4 | 4 |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK |  |  |  |  |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 |  |  |  |  |
| Number of control symbols |  | 2 | 2 | 2 |  |  |  |  |
| PDSCH mapping type |  | Type A | Type A | Type A |  |  |  |  |
| Information Bit Payload |  |  |  |  |  |  |  |  |
| For slots with RMSI | bits | 1608 | 1608 | 3104 |  |  |  |  |
| For slots without RMSI | bits | 1864 | 1864 | 3624 |  |  |  |  |
| Number of Code Blocks per slot |  | 1 | 1 | 1 |  |  |  |  |
| Binary Channel Bits Per slot |  |  |  |  |  |  |  |  |
| For slots with RMSINote 4 | bits | 5184 | 5184 | 10368 |  |  |  |  |
| For slots without RMSI Note 6 | bits | 6048 | 6048 | 12096 |  |  |  |  |
| Note 1: Allocated in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block Note 2: Void Note 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in TS 38.213 [3]. Note 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. Note 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in clause A.3.10. Note 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. Note 7: Allocated in the same resource blocks as the CORESET. Note 8: When DRX is configured, PDSCH is scheduled only while drx-onDurationTimer is running, unless otherwise specified in the test case. |  |  |  |  |  |  |  |  |

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.3.1.2	CORESET for RMSI scheduling

A.3.1.2.1	FDD

Table A.3.1.2.1-1: RMSI CORESET Reference Channel for FDD with SCS=15KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CR.1.1 FDD |  |  |  |  |  |  |
| Channel bandwidth | MHz | Defined in test case |  |  |  |  |  |  |
| Subcarrier spacing for RMSI CORESET | kHz | 15 |  |  |  |  |  |  |
| Allocated resource blocks for RMSI CORESET Note 7 |  | 24 |  |  |  |  |  |  |
| Subcarrier spacing for SSB | kHz | 15 |  |  |  |  |  |  |
| SSB and RMSI CORESET multiplexing configuration Note 7 |  | Pattern 1 |  |  |  |  |  |  |
| Offset between SSB and RMSI CORESET Note 3, 7 | RB | 0 (Note8) |  |  |  |  |  |  |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 4 |  |  |  |  |  |  |
| Number of transmitter antennas |  | 1 |  |  |  |  |  |  |
| Duration of RMSI CORESET Note 7 | symbols | 2 |  |  |  |  |  |  |
| DCI Format Note 1 |  | Note 2 |  |  |  |  |  |  |
| Aggregation level | CCE | 8 |  |  |  |  |  |  |
| DMRS precoder granularity |  | 6 |  |  |  |  |  |  |
| REG bundle size |  | 6 |  |  |  |  |  |  |
| Mapping from REG to CCE |  | Distributed |  |  |  |  |  |  |
| Cell ID |  | Note 5 |  |  |  |  |  |  |
| Payload (without CRC) | bits | Note 6 |  |  |  |  |  |  |
| Note 1: DCI formats are defined in TS 38.212. Note 2: DCI format shall depend upon the test configuration. Note 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. Note 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-11 in TS 38.213 [3]. Note 5: Cell ID shall depend upon the test configuration. Note 6: Payload size shall depend upon the test configuration. Note 7: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 0 in Table 13-1 in TS 38.213 [3] Note 8: Other values can be used to align with GSCN [13] as long as SSB does not overlap the RMC. |  |  |  |  |  |  |  |  |

A.3.1.2.2	TDD

Table A.3.1.2.2-1: RMSI CORESET Reference Channel for TDD with SCS=15KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CR.1.1 TDD |  |  |  |  |  |  |
| Channel bandwidth | MHz | Defined in test case |  |  |  |  |  |  |
| Subcarrier spacing | kHz | 15 |  |  |  |  |  |  |
| Allocated resource blocks for RMSI CORESET Note 7 |  | 24 |  |  |  |  |  |  |
| SSB and RMSI CORESET multiplexing configuration Note 7 |  | Pattern 1 |  |  |  |  |  |  |
| Offset between SSB and RMSI CORESET Note 3, 7 | RB | 0 (Note 8) |  |  |  |  |  |  |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 4 |  |  |  |  |  |  |
| Number of transmitter antennas |  | 1 |  |  |  |  |  |  |
| Duration of RMSI CORESET Note 7 | symbols | 2 |  |  |  |  |  |  |
| DCI Format Note 1 |  | Note 2 |  |  |  |  |  |  |
| Aggregation level | CCE | 8 |  |  |  |  |  |  |
| DMRS precoder granularity |  | 6 |  |  |  |  |  |  |
| REG bundle size |  | 6 |  |  |  |  |  |  |
| Mapping from REG to CCE |  | Distributed |  |  |  |  |  |  |
| Cell ID |  | Note 5 |  |  |  |  |  |  |
| Payload (without CRC) | bits | Note 6 |  |  |  |  |  |  |
| Note 1: DCI formats are defined in TS 38.212. Note 2: DCI format shall depend upon the test configuration. Note 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. Note 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-11 in TS 38.213 [3]. Note 5: Cell ID shall depend upon the test configuration. Note 6: Payload size shall depend upon the test configuration. Note 7: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 0 in Table 13-1 in TS 38.213 [3]. Note 8: Other values can be used to align with GSCN [13] as long as SSB does not overlap the RMC. |  |  |  |  |  |  |  |  |

Table A.3.1.2.2-2: RMSI CORESET Reference Channel for TDD with SCS=30KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CR.2.1 TDD |  |  |  |  |  |  |
| Channel bandwidth | MHz | Defined in test case |  |  |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |  |  |
| Allocated resource blocks for RMSI CORESET Note 7 |  | 24 |  |  |  |  |  |  |
| SSB and RMSI CORESET multiplexing configuration Note 7 |  | Pattern 1 |  |  |  |  |  |  |
| Offset between SSB and RMSI CORESET Note 3, 7 | RB | 0 (Note 8) |  |  |  |  |  |  |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 4 |  |  |  |  |  |  |
| Number of transmitter antennas |  | 1 |  |  |  |  |  |  |
| Duration of RMSI CORESET Note 7 | symbols | 2 |  |  |  |  |  |  |
| DCI Format Note 1 |  | Note 2 |  |  |  |  |  |  |
| Aggregation level | CCE | 8 |  |  |  |  |  |  |
| DMRS precoder granularity |  | 6 |  |  |  |  |  |  |
| REG bundle size |  | 6 |  |  |  |  |  |  |
| Mapping from REG to CCE |  | Distributed |  |  |  |  |  |  |
| Cell ID |  | Note 5 |  |  |  |  |  |  |
| Payload (without CRC) | bits | Note 6 |  |  |  |  |  |  |
| Note 1: DCI formats are defined in TS 38.212. Note 2: DCI format shall depend upon the test configuration. Note 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. Note 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-11 in TS 38.213 [3]. Note 5: Cell ID shall depend upon the test configuration. Note 6: Payload size shall depend upon the test configuration. Note 7: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 0 in Table 13-6 in TS 38.213 [3]. Note 8: Other values can be used to align with GSCN [13] as long as SSB does not overlap the RMC. |  |  |  |  |  |  |  |  |

Table A.3.1.2.2-3: RMSI CORESET Reference Channel for TDD with SCS=120KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CR.3.1 TDD | CR.3.2 TDD |  |  |  |  |  |
| Channel bandwidth | MHz | 100 | 100 |  |  |  |  |  |
| Subcarrier spacing | kHz | 120 | 120 |  |  |  |  |  |
| Allocated resource blocks for RMSI CORESET |  | 24Note 7 | 48 Note 9 |  |  |  |  |  |
| SSB and RMSI CORESET multiplexing configuration |  | Pattern 1 Note 7 | Pattern 1 Note 9 |  |  |  |  |  |
| Offset between SSB and RMSI CORESET Note 3 | RB | 0 (Note 8) Note 7 | 0 (Note 8) Note 9 |  |  |  |  |  |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 4 | Index 4 |  |  |  |  |  |
| Number of transmitter antennas |  | 1 | 1 |  |  |  |  |  |
| Duration of RMSI CORESET | symbols | 2 Note 7 | 2 Note 9 |  |  |  |  |  |
| DCI Format Note 1 |  | Note 2 | Note 2 |  |  |  |  |  |
| Aggregation level | CCE | 8 | 8 |  |  |  |  |  |
| DMRS precoder granularity |  | 6 | 6 |  |  |  |  |  |
| REG bundle size |  | 6 | 6 |  |  |  |  |  |
| Mapping from REG to CCE |  | Distributed | Distributed |  |  |  |  |  |
| Cell ID |  | Note 5 | Note 5 |  |  |  |  |  |
| Payload (without CRC) | bits | Note 6 | Note 6 |  |  |  |  |  |
| Note 1: DCI formats are defined in TS 38.212. Note 2: DCI format shall depend upon the test configuration. Note 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. Note 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-12 in TS 38.213 [3]. Note 5: Cell ID shall depend upon the test configuration. Note 6: Payload size shall depend upon the test configuration. Note 7: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 0 in Table 13-8 in TS 38.213 [3]. Note 8: Other values can be used to align with GSCN [13] as long as SSB does not overlap the RMC. Note 9: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 2 in Table 13-10 in TS 38.213 [3]. |  |  |  |  |  |  |  |  |

A.3.1.3	CORESET for RMC scheduling

A.3.1.3.1	FDD

Table A.3.1.3.1-1: Control Channel RMC for FDD with SCS=15KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.1.1 FDD | CCR.1.2 FDD | CCR.1.3 FDD | CCR.1.4 FDD | CCR.1.5 FDD |  |  |
| Channel bandwidth | MHz | Defined in test case | Defined in test case | Defined in test case | Defined in test case | Defined in test case |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 |  |  |
| Allocated resource blocks for CORESET Note 3 |  | 24 | 18 | 24 | 18 | 24 |  |  |
| Number of transmitter antennas |  | 1 | 1 | 1 | 1 | 1 |  |  |
| Duration of CORESET | symbols | 2 | 2 | 2 | 2 | 2 |  |  |
| REG bundle size |  | 6 | 6 | 6 | 6 | 0011000 0000000 |  |  |
| monitoringSymbolsWithinSlot |  | 1100000 0000000 | 1100000 0000000 | 1100000 0000000 | 1100000 0000000 | 6 |  |  |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |  |  |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved |  |  |
| Interleave n_shift |  | 0 | 0 | 0 | 0 | 0 |  |  |
| Interleave size |  | 2 | 2 | 2 | 2 | 2 |  |  |
| Beamforming Pre-Coder |  | N/A | N/A | N/A | N/A | N/A |  |  |
| Aggregation level | CCE | 4 | 2 | 8 | 4 | 4 |  |  |
| DCI formats |  | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |  |  |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |  |  |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration Note 3: Allocated in the resource blocks where the associated RMC is scheduled. |  |  |  |  |  |  |  |  |

A.3.1.3.2	TDD

Table A.3.1.3.2-1: Control Channel RMC for TDD with SCS=15KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.1.1 TDD | CCR.1.2 TDD | CCR.1.3 TDD | CCR.1.4 TDD | CCR.1.5 TDD |  |  |
| Channel bandwidth | MHz | Defined in test case | Defined in test case | Defined in test case | Defined in test case | Defined in test case |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 |  |  |
| Allocated resource blocks for CORESET Note 3 |  | 24 | 18 | 24 | 18 | 18 |  |  |
| Number of transmitter antennas |  | 1 | 1 | 1 | 1 | 1 |  |  |
| Duration of CORESET | symbols | 2 | 2 | 2 | 2 | 2 |  |  |
| monitoringSymbolsWithinSlot |  | 1100000 0000000 | 1100000 0000000 | 1100000 0000000 | 1100000 0000000 | 0011000 0000000 |  |  |
| REG bundle size |  | 6 | 6 | 6 | 6 | 6 |  |  |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |  |  |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved |  |  |
| Interleave n_shift |  | 0 | 0 | 0 | 0 | 0 |  |  |
| Interleave size |  | 2 | 2 | 2 | 2 | 2 |  |  |
| Beamforming Pre-Coder |  | N/A | N/A | N/A | N/A | N/A |  |  |
| Aggregation level | CCE | 4 | 2 | 8 | 4 | 4 |  |  |
| DCI formats |  | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |  |  |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |  |  |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration Note 3: Allocated in the resource blocks where the associated RMC is scheduled. |  |  |  |  |  |  |  |  |

Table A.3.1.3.2-2: Control Channel RMC for TDD with SCS=30KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.2.1 TDD | CCR.2.2 TDD | CCR.2.3 TDD |  |  |  |  |
| Channel bandwidth | MHz | Defined in test case | Defined in test case | Defined in test case |  |  |  |  |
| Subcarrier spacing | kHz | 30 | 30 | 30 |  |  |  |  |
| Allocated resource blocks for CORESET Note 3 |  | 24 | 24 | 18 |  |  |  |  |
| Number of transmitter antennas |  | 1 | 1 | 1 |  |  |  |  |
| Duration of CORESET | symbols | 2 | 2 | 2 |  |  |  |  |
| REG bundle size |  | 6 | 6 | 6 |  |  |  |  |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |  |  |  |  |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved |  |  |  |  |
| Interleave n_shift |  | 0 | 0 | 0 |  |  |  |  |
| Interleave size |  | 2 | 2 | 2 |  |  |  |  |
| Beamforming Pre-Coder |  | N/A | N/A | N/A |  |  |  |  |
| Aggregation level | CCE | 4 | 8 | 4 |  |  |  |  |
| DCI formats |  | Note 1 | Note 1 | Note 1 |  |  |  |  |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 |  |  |  |  |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration. Note 3: Allocated in the same resource blocks where the associated RMC is scheduled. |  |  |  |  |  |  |  |  |

Table A.3.1.3.2-3: Control Channel RMC for TDD with SCS=120KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.3.1 TDD | CCR.3.2 TDD | CCR.3.3 TDD | CCR.3.4 TDD | CCR.3.5 TDD | CCR.3.6 TDD | CCR.3.7 TDD |
| Channel bandwidth | MHz | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| Subcarrier spacing | kHz | 120 | 120 | 120 | 120 | 120 | 120 | 120 |
| Allocated resource blocks for CORESET Note 3 |  | 24 | 24 | 24 | 24 | 24 | 24 | 48 |
| Number of transmitter antennas |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| monitoringSlotPeriodicityAndOffset |  | sl160 0 | sl160 0 | sl160 80 | sl160 0 | sl160 0 | sl160 80 | sl160 0 |
| monitoringSlotPeriodicityAndOffset Note 4 |  | 1100000 0000000 | 0011000 0000000 | 1100000 0000000 | 1100000 0000000 | 0011000 0000000 | 1100000 0000000 | 1100000 0000000 |
| Duration of CORESET | slot | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| REG bundle size |  | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved |
| Interleave n_shift |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Interleave size |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Beamforming Pre-Coder |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Aggregation level | CCE | 4 | 4 | 4 | 8 | 8 | 8 | 4 |
| DCI formats |  | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration. Note 3: Allocated in the same resource blocks where the associated PDSCH RMC is scheduled. Note 4: monitoringSlotPeriodicityAndOffet is set to “sl1 0” if it is specifically stated that cell(s) configured with one of the control channel RMCs above shall transmit PDCCHs continuously. |  |  |  |  |  |  |  |  |

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs

Table A.3.2.1.1-1: OP.1: Generic OCNG pattern for all unused REs

| OCNG Parameters | Control Region | Data Region |
| --- | --- | --- |
| Resource allocation | Unused REs (Note 1) | Unused REs (Note 2) |
| Channel | PDCCH | PDSCH |
| Contents | Virtual UE IDs | Uncorrelated pseudo random QPSK modulated data |
| Antenna transmission scheme | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Subcarrier spacing | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Aggregation level | Same as used in PDCCH RMC | N/A |
| Code rate | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Transmit Power | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| CP length | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Note 1: REs not used in the active CORESETs where PDCCH is scheduled for the UE under test. Note 2: REs not allocated to any physical channels, CORESET, SSB or any other reference signal within the channel bandwidth of the cell, confined to BWoccupied where specified in the test case. |  |  |

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC

A.4.5.2.3.1	Test Purpose and Environment

The purpose of this test is to verify E-UTRAN PCell and NR PSCell interruptions during the measurement on the deactivated NR SCC, the UE missed ACK/NACK does not exceed the limits. This test will verify the missed ACK/NACK rate for E-UTRAN PCell and NR PSCell in EN-DC specified in TS 38.133 clause 8.2.1.2. Supported test configurations are shown in table A.4.5.2.3.1-1.

The general test parameters and NR cell specific test parameters are given in Table A.4.5.2.3.1-2 and A.4.5.2.3.1-3 below. And the E-UTRAN cell specific test parameters can refer to Table A.3.7.2.1-1. In the test there are three cells: Cell1, Cell2 and Cell3. Cell1 is LTE PCell, Cell2 and Cell3 is NR PSCell and NR deactivated SCell. Cell1 shall be configured as LTE PCell and Cell2 shall be configured as NR PSCell. The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, the UE is connected to Cell1 and Cell2 and the RRC message including measCycleSCell or allowInterruptions for the deactivated NR SCells is received at the UE antenna connector. During T1, LTE PCell and NR PSCell are continuously scheduled in DL.

Table A.4.5.2.3.1-1: Interruptions during measurements on deactivated NR SCC supported test configurations

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.4.5.2.3.1-2: General test parameters for E-UTRAN – NR interruptions during measurements on deactivated NR SCC in synchronous EN-DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2, 3 | One is E-UTRAN RF channel and the other two are NR RF channels |
| Active PCell |  | Cell1 | PCell on E-UTRAN RF channel number 1. |
| Active PSCell |  | Cell2 | PSCell on NR RF channel number 2. |
| Configured deactivated SCell |  | Cell3 | Deactivated SCell on NR RF channel number 3. |
| CP length |  | Normal | Applicable to Cell1, Cell2 and Cell3 |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| SCell measurement cycle (measCycleSCell) | ms | 640 |  |
| T1 | s | 10 |  |

Table A.4.5.2.3.1-3: NR cell specific test parameters for E-UTRAN – NR interruptions during measurements on deactivated NR SCC in synchronous EN-DC

| Parameter |  | Unit | Cell2 | Cell3 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 | FR1 |
| Duplex mode | Config 1,4 |  | FDD | FDD |
|  | Config 2,3,5,6 |  | TDD | TDD |
| TDD configuration | Config 1,4 |  | Not Applicable | Not Applicable |
|  | Config 2,5 |  | TDDConf.1.1 | TDDConf.1.1 |
|  | Config 3,6 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel | Config 1,4 |  | Note 8 | Note 8 |
|  | Config 2,5 |  | Note 8 | Note 8 |
|  | Config 3,6 |  | Note 8 | Note 8 |
| BWoccupied | Config 1,4 | RB | 52 Note 6 | 52 Note 6 |
|  | Config 2,5 |  | 52 Note 6 | 52 Note 6 |
|  | Config 3,6 |  | 106 Note 7 | 106 Note 7 |
| Initial DL BWP | Config 1,4 |  | DLBWP.0.1 | DLBWP.0.1 |
| Configuration | Config 2,5 |  | DLBWP.0.1 | DLBWP.0.1 |
|  | Config 3,6 |  | DLBWP.0.1 | DLBWP.0.1 |
| Dedicated DL BWP | Config 1,4 |  | DLBWP.1.1 | DLBWP.1.1 |
| Configuration | Config 2,5 |  | DLBWP.1.1 | DLBWP.1.1 |
|  | Config 3,6 |  | DLBWP.1.1 | DLBWP.1.1 |
| Initial UL BWP | Config 1,4 |  | ULBWP.0.1 | ULBWP.0.1 |
| Configuration | Config 2,5 |  | ULBWP.0.1 | ULBWP.0.1 |
|  | Config 3,6 |  | ULBWP.0.1 | ULBWP.0.1 |
| Dedicated UL BWP | Config 1,4 |  | ULBWP.1.1 | ULBWP.1.1 |
| Configuration | Config 2,5 |  | ULBWP.1.1 | ULBWP.1.1 |
|  | Config 3,6 |  | ULBWP.1.1 | ULBWP.1.1 |
| PDSCH Reference | Config 1,4 |  | SR.1.1 FDD | - |
| measurement channel | Config 2,5 |  | SR.1.1 TDD | - |
|  | Config 3,6 |  | SR.2.1 TDD | - |
| RMSI CORESET | Config 1,4 |  | CR.1.1 FDD | CR.1.1 FDD |
| parameters | Config 2,5 |  | CR.1.1 TDD | CR.1.1 TDD |
|  | Config 3,6 |  | CR.2.1 TDD | CR.2.1 TDD |
| PDCCH CORESET | Config 1,4 |  | CCR.1.1 FDD | CCR.1.1 FDD |
| parameters | Config 2,5 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  | Config 3,6 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| TRS configuration | Config 1,4 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  | Config 2,5 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  | Config 3,6 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| OCNG Patterns | Config 1,2,4,5 |  | OP.1 Note 6 | OP.1 Note 6 |
|  | Config 3,6 |  | OP.1 Note 7 | OP.1 Note 7 |
| SMTC Configuration |  |  | SMTC.1 | SMTC.1 |
| TCI state |  |  | TCI.State.0 | TCI.State.0 |
| SSB Configuration | Config 1,2,4,5 |  | SSB.1 FR1 | SSB.1 FR1 |
|  | Config 3,6 |  | SSB.2 FR1 | SSB.2 FR1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low |
| EPRE ratio of PSS to SSS |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 | 0 |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| NocNote 2 |  | dBm/15 kHz | -104 | -104 |
| SS-RSRP Note 3 |  | dBm/15 kHz | -87 | -87 |
| Ês/Iot |  | dB | 17 | 17 |
| Ês/Noc |  | dB | 17 | 17 |
| IoNote3 | Config 1,2,4,5 | dBm/9.36MHz | -58.96 | -58.96 |
|  | Config 3,6 | dBm/38.16MHz | -52.86 | -52.86 |
| Time offset to Cell1 Note 4 |  | s | 33 | 33 + Time offset to Cell2 |
| Time offset to Cell2 Note 5 |  | s | - | 3 |
| Propagation Condition |  |  | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modeled as AWGN of appropriate power for Noc to be fulfilled within BWoccupied. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselvess. Note 4: Receive time difference of signals received between subframe timing boundary of E-UTRA PCell and slot timing boundary of PSCell at the UE antenna connector including time alignment error between the two cells Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 8: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |  |

A.4.5.2.3.2	Test Requirements

The UE shall be continuously scheduled in LTE PCell and NR PSCell during the entire length of T1. During the time duration T1 the UE shall transmit at least 99.5% of ACK/NACK on NR PSCell.

If the NR PSCell is not in the same band as the deactivated SCell, the UE is only allowed to cause interruptions on NR PSCell immediately before and immediately after an SMTC. Each interruption on NR PSCell shall not exceed the value defined in Table A.4.5.2.3.2-1.

If the NR PSCell is in the same band as the deactivated SCell, the UE is only allowed to cause an interruption on PSCell no earlier than 1 slot before an SMTC and no later than 1 slot after the SMTC. the interruption on NR PSCell shall not exceed the value defined in Table A.4.5.2.3.2-2.

Table A.4.5.2.3.2-1: Interruption duration if the NR PSCell is not in the same band as the deactivated SCell

|  | NR Slot length (ms) | Interruption length |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |

Table A.4.5.2.3.2-2: Interruption duration if the NR PSCell is in the same band as the deactivated SCell

|  | NR Slot length (ms) | Interruption length |
| --- | --- | --- |
| 0 | 1 | 2 + SMTC duration |
| 1 | 0.5 | 2 + SMTC duration |

For synchronous inter-band EN-DC, the UE is only allowed to cause interruptions on E-UTRA PCell immediately before and immediately after an SMTC. Each interruption on E-UTRA PCell shall not exceed 1 subframe.

For synchronous intra-band EN-DC, the UE is only allowed to cause an interruption on E-UTRA PCell no earlier than 1 subframe before an SMTC and no later than 1 subframe after the SMTC. The interruption on E-UTRA PCell shall not exceed SMTC duration + 2 subframes.

The rate of correct events observed during repeated tests shall be at least 90%.

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC

A.4.5.2.4.1	Test Purpose and Environment

The purpose of this test is to verify E-UTRAN PCell and NR PSCell interruptions during the measurement on the deactivated NR SCC, the UE missed ACK/NACK does not exceed the limits. This test will verify the missed ACK/NACK rate for E-UTRAN PCell and NR PSCell in EN-DC specified in TS 38.133 clause 8.2.1. Supported test configurations are shown in table A.4.5.2.4.1-1.

The general test parameters and NR cell specific test parameters are given in Table A.4.5.2.4.1-2 and A.4.5.2.4.1-3 below. And the E-UTRAN cell specific test parameters can refer to Table A.3.7.2.1-1. In the test there are three cells: Cell1, Cell2 and Cell3. Cell1 is LTE PCell, Cell2 and Cell3 is NR PSCell and NR deactivated SCell. Cell1 shall be configured as LTE PCell and Cell2 shall be configured as NR PSCell. The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, the UE is connected to Cell1 and Cell2 and the RRC message including measCycleSCell or allowInterruptions for the deactivated NR SCells is received at the UE antenna connector. During T1, LTE PCell and NR PSCell are continuously scheduled in DL.

Table A.4.5.2.4.1-1: Interruptions during measurements on deactivated NR SCC supported test configurations

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.4.5.2.4.1-2: General test parameters for E-UTRAN – NR interruptions during measurements on deactivated NR SCC in asynchronous EN-DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2, 3 | One is E-UTRAN RF channel and the other two are NR RF channels |
| Active PCell |  | Cell1 | PCell on E-UTRAN RF channel number 1. |
| Configured PSCell |  | Cell2 | PSCell on NR RF channel number 2. |
| Configured deactivated SCell |  | Cell3 | Deactivated SCell on NR RF channel number 3. |
| CP length |  | Normal | Applicable to Cell1, Cell2 and Cell3 |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| SCell measurement cycle (measCycleSCell) | ms | 640 |  |
| T1 | s | 10 |  |

Table A.4.5.2.4.1-3: NR cell specific test parameters for E-UTRAN – NR interruptions during measurements on deactivated NR SCC in asynchronous EN-DC

| Parameter |  | Unit | Cell2 | Cell3 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 | FR1 |
| Duplex mode | Config 1,4 |  | FDD | FDD |
|  | Config 2,3,5,6 |  | TDD | TDD |
| TDD configuration | Config 1,4 |  | Not Applicable | Not Applicable |
|  | Config 2,5 |  | TDDConf.1.1 | TDDConf.1.1 |
|  | Config 3,6 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel | Config 1,4 |  | Note 8 | Note 8 |
|  | Config 2,5 |  | Note 8 | Note 8 |
|  | Config 3,6 |  | Note 8 | Note 8 |
| BWoccupied | Config 1,4 | RB | 52 Note 6 | 52 Note 6 |
|  | Config 2,5 |  | 52 Note 6 | 52 Note 6 |
|  | Config 3,6 |  | 106 Note 7 | 106 Note 7 |
| Initial BWP | Config 1,4 |  | DLBWP.0.1 | DLBWP.0.1 |
| Configuration | Config 2,5 |  | DLBWP.0.1 | DLBWP.0.1 |
|  | Config 3,6 |  | DLBWP.0.1 | DLBWP.0.1 |
| Dedicated DL BWP | Config 1,4 |  | DLBWP.1.1 | DLBWP.1.1 |
| Configuration | Config 2,5 |  | DLBWP.1.1 | DLBWP.1.1 |
|  | Config 3,6 |  | DLBWP.1.1 | DLBWP.1.1 |
| Initial UL BWP | Config 1,4 |  | ULBWP.0.1 | ULBWP.0.1 |
| Configuration | Config 2,5 |  | ULBWP.0.1 | ULBWP.0.1 |
|  | Config 3,6 |  | ULBWP.0.1 | ULBWP.0.1 |
| Dedicated UL BWP | Config 1,4 |  | ULBWP.1.1 | ULBWP.1.1 |
| Configuration | Config 2,5 |  | ULBWP.1.1 | ULBWP.1.1 |
|  | Config 3,6 |  | ULBWP.1.1 | ULBWP.1.1 |
| PDSCH Reference | Config 1,4 |  | SR.1.1 FDD | - |
| measurement channel | Config 2,5 |  | SR.1.1 TDD | - |
|  | Config 3,6 |  | SR.2.1 TDD | - |
| RMSI CORESET | Config 1,4 |  | CR.1.1 FDD | CR.1.1 FDD |
| parameters | Config 2,5 |  | CR.1.1 TDD | CR.1.1 TDD |
|  | Config 3,6 |  | CR.2.1 TDD | CR.2.1 TDD |
| PDCCH CORESET | Config 1,4 |  | CCR.1.1 FDD | CCR.1.1 FDD |
| parameters | Config 2,5 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  | Config 3,6 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| TRS configuration | Config 1,4 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  | Config 2,5 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  | Config 3,6 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| OCNG Patterns | Config 1,2,4,5 |  | OP.1 Note 6 | OP.1 Note 6 |
|  | Config 3,6 |  | OP.1 Note 7 | OP.1 Note 7 |
| SSB Configuration | Config 1,2,4,5 |  | SSB.1 FR1 | SSB.1 FR1 |
|  | Config 3,6 |  | SSB.2 FR1 | SSB.2 FR1 |
| SMTC Configuration |  |  | SMTC.1 | SMTC.1 |
| TCI state |  |  | TCI.State.0 | TCI.State.0 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low |
| EPRE ratio of PSS to SSS |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 | 0 |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| NocNote 2 |  | dBm/15 kHz | -104 | -104 |
| SS-RSRP Note 3 |  | dBm/15 kHz | -87 | -87 |
| Ês/Iot |  | dB | 17 | 17 |
| Ês/Noc |  | dB | 17 | 17 |
| IoNote3 | Config 1,2,4,5 | dBm/9.36MHz | -58.96 | -58.96 |
|  | Config 3,6 | dBm/38.16MHz | -52.86 | -52.86 |
| Time offset to Cell1 Note 4 |  | ms | 3 | 3 + Time offset to Cell2 |
| Time offset to Cell2 Note 5 |  | s | - | 3 |
| Propagation Condition |  |  | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modeled as AWGN of appropriate power for Noc to be fulfilled within BWoccupied. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselvess. Note 4: Receive time difference of signals received between subframe timing boundary of E-UTRA PCell and slot timing boundary of PSCell at the UE antenna connector including time alignment error between the two cells Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 8: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |  |

A.4.5.2.4.2	Test Requirements

The UE shall be continuously scheduled in LTE PCell and NR PSCell during the entire length of T1. During the time duration T1 the UE shall transmit at least 99.5% of ACK/NACK on NR PSCell.

If the NR PSCell is not in the same band as the deactivated SCell, the UE is only allowed to cause interruptions on NR PSCell immediately before and immediately after an SMTC. Each interruption on NR PSCell shall not exceed the value defined in Table A.4.5.2.4.2-1.

If the NR PSCell is in the same band as the deactivated SCell, the UE is only allowed to cause an interruption on PSCell no earlier than 1 slot before an SMTC and no later than 1 slot after the SMTC. the interruption on NR PSCell shall not exceed the value defined in Table A.4.5.2.4.2-2.

Table A.4.5.2.4.2-1: Interruption duration if the NR PSCell is not in the same band as the deactivated SCell

|  | NR Slot length (ms) | Interruption length |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |

Table A.4.5.2.4.2-2: Interruption duration if the NR PSCell is in the same band as the deactivated SCell

|  | NR Slot length (ms) | Interruption length |
| --- | --- | --- |
| 0 | 1 | 2 + SMTC duration |
| 1 | 0.5 | 2 + SMTC duration |

For asynchronous inter-band EN-DC, the UE is only allowed to cause interruptions on E-UTRA PCell immediately before and immediately after an SMTC. Each interruption on E-UTRA PCell shall not exceed 2 subframe.

The rate of correct events observed during repeated tests shall be at least 90%.

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.4.5.3	SCell Activation and Deactivation Delay

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle

A.4.5.3.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in clause 8.3, when the SCell in FR1 is known by the UE at the time of activation.

The supported test configurations are shown in table A.4.5.3.1.1-1 below. The test parameters are given in Tables A.4.5.3.1.1-2 and cell-specific parameters in A.4.5.3.1.1-3 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are three carriers, E-UTRA has one cell, NR has two cells. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on E-UTRA and Cell 2 (PSCell) on NR, but is not aware of Cell 3 (SCell) on NR. The UE is monitoring the PCell and PSCell. The UE shall be continuously scheduled in the PCell and PSCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 3) becomes configured on NR. The UE now starts monitoring the SCell. The test equipment sends a MAC message for activation of the SCell.

The point in time at which the MAC message is received at the UE antenna connector, in a slot # denoted m, defines the start of time period T2. The UE shall be able to report valid CSI in PSCell for the activated SCell at latest in slot m+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as defined in clause 8.3. The UE shall start reporting CSI in PSCell in slot (m+k) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PSCell interruption due to activation of SCell shall occur in the slot m+1+THARQNR slot length to slot m+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3, where Ninterruption is the interruption length given in clause 8.2. Any E-UTRA PCell interruption due to activation of SCell shall occur in the subframe m1+1+THARQEUTRA slot length to subframe m2+1+THARQ+3ms+TXEUTRA slot length+Ninterruption, where m1 and m2 are the index of the first and last subframe of E-UTRA PCell which overlaps with slot m, and Ninterruption is the interruption length given in TS 36.133 [14] clause 7.32.

Time period T3 starts when a MAC message for deactivation of SCell, sent from the test equipment to the UE in a slot # denoted n, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell in a slot n+THARQ+3msNR slot length, as defined in clause 8.3. The starting point of any PSCell interruption due to the deactivation shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3msNR slot length, as defined in clause 8.3. The starting point of any E-UTRA PCell interruption due to the deactivation shall occur in the subframe n1+1+THARQEUTRA subframe length to subframe n2+1+THARQ+3msEUTRA subframe length, where n1 and n2 are the index of the first and last subframe of E-UTRA PCell which overlaps with slot n.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PSCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell deactivation command is sent until CSI reporting for SCell is discontinued.

Table A.4.5.3.1.1-1: known FR1 SCell activation in non-DRX for 160ms SCell measurement cycle supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.4.5.3.1.1-2: General test parameters for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2,3 | One E-UTRAN radio channel (1) and two NR radio channel (2,3) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on E-UTRAN RF channel number 1. As specified in clause A.3.7.2.1 |
| Active PSCell |  | Cell 2 | Primary secondary cell on NR RF channel number 2. |
| Configured deactivated SCell |  | Cell 3 | Configured deactivated secondary cell on NR RF channel number 3 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every four slots. |
| Cell-individual offset for cells on E-UTRA RF channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on secondary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell3 timing offset to cell2 | s | 0 |  |
| Time alignment error between cell3 and cell2 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 7 | During this time the PSCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| THARQ | ms | k1NR slot length | k1 is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ feedback timing field is not present in the DCI format, the value is defined in 38.213 [3] |
| TCSI_Reporting | ms | 10+5⋅2µDL | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] µDL is the subcarrier spacing configuration for DL |
| k | slot |  | As specified in clause 4.3 of TS 38.213 [3] |

Table A. 4.5.3.1.1-3: Cell specific test parameters for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter |  |  |  |  | Unit | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| SSB ARFCN |  |  |  |  |  | freq1 |  |  | freq2 |  |  |
| Duplex mode |  | Config 1,4 |  |  |  | FDD |  |  |  |  |  |
|  |  | Config 2,3,5,6 |  |  |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1,4 |  |  |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2,5 |  |  |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | Config 1,4 |  |  | MHz | Note 7 |  |  |  |  |  |
|  |  | Config 2,5 |  |  |  | Note 7 |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  | Note 7 |  |  |  |  |  |
| BWoccupied |  | Config 1,4 |  |  | RB | 52 Note 5 |  |  |  |  |  |
|  |  | Config 2,5 |  |  |  | 52 Note 5 |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  | 106 Note 6 |  |  |  |  |  |
| DL initial BWP configuration |  | Config 1, 2, 3, 4, 5, 6 |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| DL dedicated BWP configuration |  | Config 1, 2, 3, 4, 5, 6 |  |  |  | DLBWP.1.1 |  |  |  |  |  |
| UL initial BWP configuration |  | Config 1, 2, 3, 4, 5, 6 |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| UL dedicated BWP configuration |  | Config 1, 2, 3, 4, 5, 6 |  |  |  | ULBWP.1.1 |  |  |  |  |  |
| DRx Cycle |  |  |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference |  | Config 1,4 |  |  |  | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
| measurement channel |  | Config 2,5 |  |  |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | Config 3,6 |  |  |  | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET |  | Config 1,4 |  |  |  | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| Reference Channel |  | Config 2,5 |  |  |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | Config 3,6 |  |  |  | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| RMC CORESET |  | Config 1,4 |  |  |  | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| Reference Channel |  | Config 2,5 |  |  |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | Config 3,6 |  |  |  | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| TRS configuration |  | Config 1,4 |  |  |  | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  |  |
|  |  | Config 2,5 |  |  |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | Config 3,6 |  |  |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| OCNG Patterns | Config 1,2,4,5 |  |  |  |  | OP.1 Note 5 |  |  |  |  |  |
|  | Config 3,6 |  |  |  |  | OP.1 Note 6 |  |  |  |  |  |
| SMTC configuration |  |  |  |  |  | SMTC.1 |  |  |  |  |  |
| SSB configuration |  | Config 1,2,4,5 |  |  |  | SSB.1 FR1 |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  | SSB.2 FR1 |  |  |  |  |  |
| CSI-RS configuration for CSI reporting |  | Config 1,4 |  |  |  | CSI-RS.1.1 FDD |  |  |  |  |  |
|  |  | Config 2,5 |  |  |  | CSI-RS.1.1 TDD |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  | CSI-RS.2.1 TDD |  |  |  |  |  |
| PDSCH/PDCCH |  | Config 1,2,4,5 |  |  | kHz | 15 kHz |  |  |  |  |  |
| subcarrier spacing |  | Config 3,6 |  |  |  | 30kHz |  |  |  |  |  |
| CSI reporting periodicity |  | Config 1-6 |  |  | ms | 5 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  |  |  | dBm/15kHz | -104 |  |  |  |  |  |
| Note2 |  |  |  | Config 1,2,4,5 | dBm/SCS | -104 |  |  |  |  |  |
|  |  |  |  | Config 3,6 |  | -101 |  |  |  |  |  |
|  |  |  |  |  | dB | 17 |  |  |  |  |  |
|  |  |  |  |  | dB | 17 |  |  |  |  |  |
| SS-RSRPNote3 |  |  |  | Config 1,2,4,5 | dBm/SCS | -87 |  |  |  |  |  |
|  |  |  |  | Config 3,6 |  | -84 |  |  |  |  |  |
| SCH_RP Note 3 |  |  |  |  | dBm/15 kHz | -87 |  |  |  |  |  |
| IoNote3 |  |  | Config 1,2,4,5 |  | dBm/ 9.36MHz | -58.96 |  |  |  |  |  |
|  |  |  | Config 3,6 |  | dBm/ 38.16MHz | -52.87 |  |  |  |  |  |
| Propagation condition |  |  |  |  | - | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. Note 3: SS-RSRP, Io and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2.] Note 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |  |  |  |  |  |  |  |  |

A.4.5.3.1.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell in the first available uplink resource after slot (m+k). UE is allowed to postpone CSI report to next available uplink resource if an available uplink resource is subject to interruption.  Whether CSI report in slot (m+k) was interrupted is checked by monitoring ACK/NACK sent in PCell in slot (m+k).

During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a slot m+THARQ+Tactivtion_time+TCSI_ReportingNR slot length, Tactivation_time = TFirstSSB+ 5ms, as defined in clause 8.3.

During T3 the UE shall stop sending CSI reports for SCell at latest in a slot n+THARQ+3msNR slot length, as defined in clause 8.3.

During T2 interruption of PSCell during SCell activation shall not happen outside the slot m+1+THARQNR slot length to  m+1+THARQ+3ms+TXNR slot length+Ninterruption, and interruption of E-UTRA PCell during SCell activation shall not happen outside the subframe m1+1+THARQEUTRA slot length to subframe m2+1+THARQ+3ms+TXEUTRA slot length+Ninterruption, as defined in clause 8.3.

During T3 the starting point of interruption of PSCell during SCell deactivation shall not happen outside the slot n+1+THARQNR slot length to n+1+THARQ+3msNR slot length, as defined in clause 8.3 and the starting point of interruption of E-UTRA PCell during SCell deactivation shall not happen outside the subframe n1+1+THARQEUTRA subframe length to subframe n2+1+THARQ+3msEUTRA subframe length.

The interruption of PSCell shall not be more than the values specified for EN-DC in Clause 8.2.1.2.4.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay and SCell deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a slot m+THARQ+Tactivtion_time+TCSI_ReportingNR slot length as defined in clause 8.3 then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 320 ms SCell measurement cycle

A.4.5.3.2.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in clause A.4.5.3.1.1. The supported test configurations are the same as defined in clause A.4.5.3.1.1. The test parameters are the same except those described in the following clause. The listed parameter values in Tables A.4.5.3.2.1-1 will replace the values of corresponding parameters in Tables A.4.5.3.1.1-2.

Table A.4.5.3.2.1-1: General test parameters for known FR1 SCell activation case, 320 ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |

A.4.5.3.2.2	Test Requirements

The test requirements defined in clause A.4.5.3.1.2 shall apply to this test case, except Tactivation_time will be replaced with the value TFirstSSB_MAX + Trs + 5ms.

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.4.5.4	UE UL carrier RRC reconfiguration Delay

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay

Table A.4.5.4.1-1 - Table A.4.5.4.1-4 : Void

A.4.5.4.1.1 Test Purpose and Environment

The purpose of this test is to verify that when the UE receives a RRC message implying NR UL or Supplementary UL carrier configuration, the UE shall be ready to start transmission on the newly configured carrier within the time limits specified in clause 8.4.2 and 8.4.3 for configuring and deconfiguring, respectively.

There are three cells: E-UTRAN PCell (Cell 1), FR1 PSCell (Cell 2) and FR1 SCell (Cell 3). For SCell, both NR uplink and supplementary uplink are broadcast by ServingCellConfigCommonSIB. The test parameters for PSCell and SCell are given in Table A. 4.5.4.1.1-1, Table A. 4.5.4.1.1-2, Table A. 4.5.4.1.1-3 and Table A. 4.5.4.1.1-4 below.  The test parameters and applicability for E-UTRAN PCell are defined in A.3.7.2.  The test consists two tests. In test 1, the test consists of three time periods, with duration of T1, T2 and T3 respectively. During time duration T1, NR uplink of cell 3 is configured to UE. At the start of T2, a supplementary uplink of cell3 is configured to UE through RRCReconfiguration, then UE shall start transmission both on the NR uplink and supplementary uplink. At the start of T3, the supplementary uplink is released through RRCReconfiguration.

In test 2, the test consists of three time periods, with duration of T1, T2 and T3 respectively. During time duration T1, supplementray uplink on cell 3 is configured to UE. At the start of T2, a NR uplink is configured to UE through RRCReconfiguration, then UE shall start transmission both on the NR uplink and supplementary uplink. At the start of T3, the NR uplink is released through RRCReconfiguration.

Table A.4.5.4.1.1-1: Supported test configurations

| Configuration | PSCell (Cell2) | SCell (Cell3) |
| --- | --- | --- |
| 1 | 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode | DL and UL: 15kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode; SUL: 15kHz SCS, ≥10 MHz bandwidth, SUL duplex mode |
| 2 | 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode | DL and UL: 15kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode; SUL: 15kHz SCS, ≥10 MHz bandwidth, SUL duplex mode |
| 3 | 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode | DL and UL: 30kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode; SUL: 30kHz SCS, ≥40 MHz bandwidth, SUL duplex mode |
| 4 | 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode | DL and UL: 15kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode; SUL: 15kHz SCS, 10 MHz bandwidth, SUL duplex mode |
| 5 | 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode | DL and UL: 15kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode; SUL: 15kHz SCS, 10 MHz bandwidth, SUL duplex mode |
| 6 | 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode | DL and UL: 30kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode; SUL: 30kHz SCS, ≥40 MHz bandwidth, SUL duplex mode |
| 7 | 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode | DL and UL: 15kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode; SUL: 15kHz SCS, ≥10 MHz bandwidth, SUL duplex mode |
| 8 | 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode | DL and UL: 15kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode; SUL: 15kHz SCS, ≥10 MHz bandwidth, SUL duplex mode |
| 9 | 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode | DL and UL: 30kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode; SUL: 30kHz SCS, ≥40 MHz bandwidth, SUL duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |  |

Table A.4.5.4.1.1-2: General test parameters for EN-DC UE UL carrier RRC reconfiguration Delay

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | Config 1,2,3, 4, 5, 6, 7, 8, 9 | 1, 2, 3 | Three radio channels are used for these two tests. |
| Active cell |  | Config 1,2,3, 4, 5, 6, 7, 8, 9 | Cell 1: E-UTRAN PCell Cell 2: FR1 PSCell Cell 3: FR1 SCell | E-UTRAN PCell on RF channel number 1 FR1 PSCell on RF channel number 2 FR1 SCell on RF channel number 3 |
| CP length |  | Config 1,2,3, 4, 5, 6, 7, 8, 9 | Normal |  |
| DRX |  | Config 1,2,3, 4, 5, 6, 7, 8, 9 | OFF |  |
| Measurement gap pattern Id |  | Config 1,2,3, 4, 5, 6, 7, 8, 9 | OFF |  |
| Filter coefficient |  | Config 1,2,3, 4, 5, 6, 7, 8, 9 | 0 | L3 filtering is not used |
| T1 | s | Config 1,2,3, 4, 5, 6, 7, 8, 9 | 5 |  |
| T2 | s | Config 1,2,3, 4, 5, 6, 7, 8, 9 | 5 |  |
| T3 | s | Config 1,2,3, 4, 5, 6, 7, 8, 9 | 5 |  |

Table A.4.5.4.1.1-3: NR Cell specific test parameters for EN-DC UE UL carrier RRC reconfiguration Delay on PSCell (Cell 2)

| Parameter | Unit | Test | Test 1 |  |  |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Configuration | T1 |  | T2 |  | T3 | T1 | T2 | T3 |
| Channel number |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 2 |  |  |  |  | 2 |  |  |
| TDD configuration |  | Conf 1, 2, 3 | N/A |  |  |  |  | N/A |  |  |
|  |  | Conf 4, 5, 6 | TDD Conf.1.1 |  |  |  |  | TDD Conf.1.1 |  |  |
|  |  | Conf 7, 8, 9 | TDD Conf.2.1 |  |  |  |  | TDD Conf.2.1 |  |  |
| BWchannel | MHz | Conf 1, 2, 3 | Note 6 |  |  |  |  | Note 6 |  |  |
|  |  | Conf 4, 5, 6 | Note 6 |  |  |  |  | Note 6 |  |  |
|  |  | Conf 7, 8, 9 | Note 6 |  |  |  |  | Note 6 |  |  |
| BWoccupied | RB | Conf 1, 2, 3 | 52 Note 4 |  |  |  |  | 52 Note 4 |  |  |
|  |  | Conf 4, 5, 6 | 52 Note 4 |  |  |  |  | 52 Note 4 |  |  |
|  |  | Conf 7, 8, 9 | 106 Note 5 |  |  |  |  | 106 Note 5 |  |  |
| PDSCH reference |  | Conf 1, 2, 3 | SR.1.1 FDD |  |  |  |  | SR.1.1 FDD |  |  |
| measurement |  | Conf 4, 5, 6 | SR.1.1 TDD |  |  |  |  | SR.1.1 TDD |  |  |
| channel as defined in A.3.1.1 |  | Conf 7, 8, 9 | SR 2.1 TDD |  |  |  |  | SR 2.1 TDD |  |  |
| RMSI CORESET |  | Conf 1, 2, 3 | CR.1.1 FDD |  |  |  |  | CR.1.1 FDD |  |  |
| reference |  | Conf 4, 5, 6 | CR.1.1 TDD |  |  |  |  | CR.1.1 TDD |  |  |
| measurement channel as defined in A.3.1.2 |  | Conf 7, 8, 9 | CR.2.1 TDD |  |  |  |  | CR.2.1 TDD |  |  |
| RMC CORESET |  | Conf 1, 2, 3 | CCR.1.1 FDD |  |  |  |  | CCR.1.1 FDD |  |  |
| reference |  | Conf 4, 5, 6 | CCR.1.1 TDD |  |  |  |  | CCR.1.1 TDD |  |  |
| measurement channel as defined in A.3.1.3 |  | Conf 7, 8, 9 | CCR.2.1 TDD |  |  |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern Note 1 |  | Conf 1, 2, 3, 4, 5, 6 | OP.1 Note 4 |  |  |  |  | OP.1 Note 4 |  |  |
|  |  | Config 7, 8, 9 | OP.1 Note 5 |  |  |  |  | OP.1 Note 5 |  |  |
| SSB configuration |  | Conf 1, 2, 3, 4, 5, 6 | SSB.1 FR1 |  |  |  |  | SSB.1 FR1 |  |  |
|  |  | Conf 7, 8, 9 | SSB.2 FR1 |  |  |  |  | SSB.2 FR1 |  |  |
| SMTC configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | SMTC.1 |  |  |  |  | SMTC.1 |  |  |
| CSI-RS for tracking |  | Conf 1 | TRS.1.1 FDD |  |  |  |  | TRS.1.1 FDD |  |  |
|  |  | Conf 2 | TRS.1.1 FDD |  |  |  |  | TRS.1.1 FDD |  |  |
|  |  | Conf 3 | TRS.1.1 FDD |  |  |  |  | TRS.1.1 FDD |  |  |
|  |  | Conf 4 | TRS.1.1 TDD |  |  |  |  | TRS.1.1 TDD |  |  |
|  |  | Conf 5 | TRS.1.1 TDD |  |  |  |  | TRS.1.1 TDD |  |  |
|  |  | Conf 6 | TRS.1.1 TDD |  |  |  |  | TRS.1.1 TDD |  |  |
|  |  | Conf 7 | TRS.1.2 TDD |  |  |  |  | TRS.1.2 TDD |  |  |
|  |  | Conf 8 | TRS.1.2 TDD |  |  |  |  | TRS.1.2 TDD |  |  |
|  |  | Conf 9 | TRS.1.2 TDD |  |  |  |  | TRS.1.2 TDD |  |  |
| DL initial BWP configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | DLBWP.0.1 |  |  |  |  | DLBWP.0.1 |  |  |
| DL dedicated BWP configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | DLBWP.1.1 |  |  |  |  | DLBWP.1.1 |  |  |
| UL dedicated BWP configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | ULBWP.1.1 |  |  |  |  | ULBWP.1.1 |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS | dB | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 0 |  |  |  |  | 0 |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |  |  |  |  |
| Note 2 | dBm / 15kHz | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | -102 |  |  |  |  | -102 |  |  |
|  | dBm/ SCS | Conf 1,2,3,4,5,6 | -102 |  |  |  |  | -102 |  |  |
|  |  | Conf 7,8,9 | -99 |  |  |  |  | -99 |  |  |
|  | dB | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 16 | 16 |  | 16 |  | 16 | 16 | 16 |
| Note 3 | dB | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 16 | 16 |  | 16 |  | 16 | 16 | 16 |
| SS-RSRP Note 3 | dBm/ SCS | Conf 1,2,3,4,5,6 | -86 | -86 |  | -86 |  | -86 | -86 | -86 |
|  |  | Conf 7,8,9 | -83 | -83 |  | -83 |  | -83 | -83 | -83 |
| Io Note 3 | dBm/ 9.36 MHz | Conf 1,2,3,4,5,6 | -57.9 | -57.9 |  | -57.9 |  | -57.9 | -57.9 | -57.9 |
|  | dBm/ 38.16MHz | Conf 7,8,9 | -51.8 | -51.8 |  | -51.8 |  | -51.8 | -51.8 | -51.8 |
| Propagation Condition |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | AWGN |  |  |  |  | AWGN |  |  |
| Antenna configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 1 x 2 |  |  |  |  | 1 x 2 |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. NOTE 3: , Io, and SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 6: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |  |  |  |  |  |  |  |

Table A.4.5.4.1.1-4: NR Cell specific test parameters for EN-DC UE UL carrier RRC reconfiguration Delay on SCell (Cell 3)

| Parameter | Unit | Test | Test 1 |  |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Configuration | T1 | T2 |  | T3 | T1 | T2 | T3 |
| Channel number |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 3 |  |  |  | 3 |  |  |
| TDD configuration |  | Conf 1, 4, 7 | N/A |  |  |  | N/A |  |  |
|  |  | Conf 2, 5, 8 | TDDConf.1.1 |  |  |  | TDDConf.1.1 |  |  |
|  |  | Conf 3, 6, 9 | TDDConf.2.1 |  |  |  | TDDConf.2.1 |  |  |
| BWchannel | MHz | Conf 1, 4, 7 | Note 6 |  |  |  | Note 6 |  |  |
|  |  | Conf 2, 5, 8 | Note 6 |  |  |  | Note 6 |  |  |
|  |  | Conf 3, 6, 9 | Note 6 |  |  |  | Note 6 |  |  |
| BWoccupied | RB | Conf 1, 4, 7 | 52 Note 4 |  |  |  | 52 Note 4 |  |  |
|  |  | Conf 2, 5, 8 | 52 Note 4 |  |  |  | 52 Note 4 |  |  |
|  |  | Conf 3, 6, 9 | 106 Note 5 |  |  |  | 106 Note 5 |  |  |
| PUSCH parameters for NR UL carrier |  | Conf 1, 4, 7 | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] |  | N/A | G-FR1-A3-10 in [13] | N/A |
|  |  | Conf 2, 5, 8 | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] |  | N/A | G-FR1-A3-10 in [13] | N/A |
|  |  | Conf 3, 6, 9 | G-FR1-A3-14 in [13] | G-FR1-A3-14 in [13] | G-FR1-A3-14 in [13] |  | N/A | G-FR1-A3-14 in [13] | N/A |
| PUCCH parameters For NR UL carrier |  | Conf 1, 4, 7 | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] |  | N/A | N/A | N/A |
|  |  | Conf 2, 5, 8 | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] |  | N/A | N/A | N/A |
|  |  | Conf 3, 6, 9 | Table 8.3.3.1.2-2 in [13] | Table 8.3.3.1.2-2 in [13] | Table 8.3.3.1.2-2 in [13] |  | N/A | N/A | N/A |
| PUSCH parameters for supplementary UL |  | Conf 1, 4, 7 | N/A | G-FR1-A3-10 in [13] | N/A |  | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] |
|  |  | Conf 2, 5, 8 | N/A | G-FR1-A3-10 in [13] | N/A |  | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] |
|  |  | Conf 3, 6, 9 | N/A | G-FR1-A3-14 in [13] | N/A |  | G-FR1-A3-14 in [13] | G-FR1-A3-14 in [13] | G-FR1-A3-14 in [13] |
| PUCCH parameters for supplementary UL |  | Conf 1, 4, 7 | N/A | N/A | N/A |  | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] |
|  |  | Conf 2, 5, 8 | N/A | N/A | N/A |  | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] |
|  |  | Conf 3, 6, 9 | N/A | N/A | N/A |  | Table 8.3.3.1.2-2 in [13] | Table 8.3.3.1.2-2 in [13] | Table 8.3.3.1.2-2 in [13] |
| PDSCH reference |  | Conf 1, 4, 7 | SR.1.1 FDD |  |  |  | SR.1.1 FDD |  |  |
| measurement |  | Conf 2, 5, 8 | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |
| channel as defined in A.3.1.1 |  | Conf 3, 6, 9 | SR 2.1 TDD |  |  |  | SR 2.1 TDD |  |  |
| RMSI CORESET |  | Conf 1, 4, 7 | CR.1.1 FDD |  |  |  | CR.1.1 FDD |  |  |
| reference |  | Conf 2, 5, 8 | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |
| measurement channel as defined in A.3.1.2 |  | Conf 3, 6, 9 | CR.2.1 TDD |  |  |  | CR.2.1 TDD |  |  |
| RMC CORESET |  | Conf 1, 4, 7 | CCR.1.1 FDD |  |  |  | CCR.1.1 FDD |  |  |
| reference |  | Conf 2, 5, 8 | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |  |
| measurement channel as defined in A.3.1.3 |  | Conf 3, 6, 9 | CCR.2.1 TDD |  |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern Note 1 |  | Conf 1, 2, 4, 5, 7, 8 | OP.1 Note 4 |  |  |  | OP.1 Note 4 |  |  |
|  |  | Conf 3, 6, 9 | OP.1 Note 5 |  |  |  | OP.1 Note 5 |  |  |
| SSB configuration |  | Conf 1, 2, 4, 5, 7,8 | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |
|  |  | Conf 3, 6, 9 | SSB.2 FR1 |  |  |  | SSB.2 FR1 |  |  |
| SMTC configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | SMTC.1 |  |  |  | SMTC.1 |  |  |
| CSI-RS for tracking |  | Conf 1 | TRS.1.1 FDD |  |  |  | TRS.1.1 FDD |  |  |
|  |  | Conf 2 | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  |
|  |  | Conf 3 | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |
|  |  | Conf 4 | TRS.1.1 FDD |  |  |  | TRS.1.1 FDD |  |  |
|  |  | Conf 5 | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  |
|  |  | Conf 6 | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |
|  |  | Conf 7 | TRS.1.1 FDD |  |  |  | TRS.1.1 FDD |  |  |
|  |  | Conf 8 | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  |
|  |  | Conf 9 | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |
| DL initial BWP configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | DLBWP.0.1 |  |  |  | DLBWP.0.1 |  |  |
| DL dedicated BWP configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | DLBWP.1.1 |  |  |  | DLBWP.1.1 |  |  |
| UL dedicated BWP configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | ULBWP.1.1 |  |  |  | ULBWP.1.1 |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS | dB | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 0 |  |  |  | 0 |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |  |  |  |
| Note 2 | dBm / 15kHz | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | -102 |  |  |  | -102 |  |  |
|  | dBm/ SCS | Conf 1, 2, 4, 5, 7,8 | -102 |  |  |  | -102 |  |  |
|  |  | Conf 3, 6, 9 | -99 |  |  |  | -99 |  |  |
|  | dB | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 16 | 16 |  | 16 | 16 | 16 | 16 |
| Note 3 | dB | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 16 | 16 |  | 16 | 16 | 16 | 16 |
| SS-RSRP Note 3 | dBm/ SCS | Conf 1, 2, 4, 5, 7,8 | -86 | -86 |  | -86 | -86 | -86 | -86 |
|  |  | Conf 3, 6, 9 | -83 | -83 |  | -83 | -83 | -83 | -83 |
| Io Note 3 | dBm/ 9.36 MHz | Conf 1, 2, 4, 5, 7,8 | -57.9 | -57.9 |  | -57.9 | -57.9 | -57.9 | -57.9 |
|  | dBm/ 38.16MHz | Conf 3, 6, 9 | -51.8 | -51.8 |  | -51.8 | -51.8 | -51.8 | -51.8 |
| Propagation Condition |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | AWGN |  |  |  | AWGN |  |  |
| Antenna configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 1 x 2 |  |  |  | 1 x 2 |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. NOTE 3: , Io, and SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 6: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |  |  |  |  |  |  |

A.4.5.4.1.2	Test Requirements

In test 1 the UE shall be ready to start transmission on the supplementary uplink carrier on SCell within 20ms from the start of T2.

In test 1 the UE shall stop the transmission on the supplementary uplink carrier on SCell within 20ms from the start of T3.

In test 2 the UE shall be ready to start transmission on the NR uplink carrier on SCell within 20ms from the start of T2.

In test 2 the UE shall stop the transmission on the NR uplink carrier on SCell within 20ms from the start of T3.

All of the above test requirements shall be fulfilled in order for the observed UE UL carrier configuration delay and UE UL carrier release delay to be counted as correct. The rate of correct observed UE UL carrier configuration delay and UE UL carrier release delay during repeated tests shall be at least 90%.

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC

A.4.5.6.1.2.1	Test Purpose and Environment

The purpose of this test is to verify the DL BWP switch delay requirement defined in clause 8.6, and interruption requirements for NR victim cell defined in clause 8.2.1.2.7 and interruption requirement for E-UTRA victim cell defined in clause 7.32.2.7 of TS 36.133 [15]. Supported test configurations are shown in Table A.4.5.6.1.2.1-1.

The test scenario comprises of one E-UTRA PCell (Cell 1), one NR PSCell (Cell 2) and one NR SCell (Cell 3) as given in Table A.4.5.6.1.2.1-2. Cell-specific parameters of E-UTRA PCell are specified in Table A.3.7.2.1-1 and Cell-specific parameters of NR PSCell and SCell are specified in Table A.4.5.6.1.2.1-3 below.

PDCCHs indicating new transmissions shall be sent continuously on PCell (Cell 1) and SCell (Cell 3) to ensure that the UE will have ACK/NACK sending.

PDCCHs indicating new transmissions shall be sent continuously on PSCell (Cell 2) to ensure that the UE would have ACK/NACK sending except for the time duration when BWP is switching on Cell 2 and the time duration of T2.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), Cell 2 (PSCell) on radio channel 2 (PSCC) and Cell 3 (SCell) on radio channel 3 (SCC).

-	UE is configured with 2 different UE-specific downlink bandwidth parts for PSCell, BWP-1 and BWP-2, in Cell 2 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

-	UE is configured with 1 UE-specific downlink bandwidth parts the same as initial BWP for SCell, BWP-0 in Cell 3 before starting the test.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 in PSCell.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-0 in SCell.

-	UE is configured with a bwp-InactivityTimer timer value for PSCell.

All cells have constant signal levels throughout the test.

The test consists of 3 successive time periods, with durations of T1, T2, and T3, respectively.

During T1,

Time period T1 starts when a DCI format 1_1 command for PSCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in PSCell’s slot # denoted i. The UE shall switch its bandwidth part from BWP-1 to BWP-2.

The UE shall be able to receive PDSCH at the beginning of the DL slot right after PSCell’s DL slot (i+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the PSCell no later than at the beginning of the DL slot right after slot (i+TBWPswitchDelay+k1). The UE shall be continuously scheduled on PSCell’s BWP-2 starting from the beginning of the DL slot right after slot (i+TBWPswitchDelay).

PCell(Cell 1) interruption due to BWP switch on PSCell shall occur within the BWP switch delay.

SCell(Cell 3) interruption due to BWP switch on PSCell shall occur within the BWP switch delay.

During T2, the test equipment won’t transmit DCI format for PDSCH reception on PSCell(Cell 2).

During T3,

The time period T3 starts from the slot #j, where j is the beginning slot of the DL subframe immediately after the slot wherein bwp-InactivityTimer timer expires. The UE shall switch its bandwidth part from BWP-2 back to the default bandwidth part – BWP-1.

The UE shall be able to receive PDSCH at the beginning of the DL slot right after PSCell’s DL slot (j+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the PSCell at latest at the beginning of the DL slot right after slot (j+TBWPswitchDelay+k1). The UE shall be continuously scheduled on PSCell’s BWP-1 starting from the beginning of the DL slot right after slot (j+TBWPswitchDelay).

PCell(Cell 1) interruption due to BWP switch of PSCell shall occur within the BWP switch delay.

SCell(Cell 3) interruption due to BWP switch of PSCell shall occur within the BWP switch delay.

The test equipment verifies the DL BWP switch time in PSCell by counting the slots from the time when the BWP switch command is received or bwp-InactivityTimer timer expires till an ACK is received.

The test equipment verifies that potential interruption to E-UTRA PCell and NR SCell is carried out in the correct time span by monitoring ACK/NACK sent in PCell and SCell during BWP switch of PSCell, respectively.

Table A.4.5.6.1.2.1-1: DL BWP switch supported test configurations

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: A UE which fulfils the requirements in test case A.4.5.6.1.2 can skip the test cases in A.4.5.6.1.1. Note 3: NR configuration is the same for PSCell and SCells. Note 4: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration |  |

Table A.4.5.6.1.2.1-2: General test parameters for DL BWP switch in synchronous EN-DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA radio channel is used for this test |
| NR RF Channel Number |  | 2, 3 | Two NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active PSCell |  | Cell 2 | PSCell on RF channel number 2. |
| Active SCell |  | Cell 3 | SCell on RF channel number 3. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| bwp-InactivityTimer | ms | [200] |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on PSCC. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC. |
| Cell2 timing offset to cell1 | s | 3 | Synchronous EN-DC |
| Cell3 timing offset to cell2 | s | 3 | Synchronous cells |
| T1 | s | [0.2] |  |
| T2 | s | [0.2] |  |
| T3 | s | [0.2] |  |

Table A.4.5.6.1.2.1-3: NR Cell specific test parameters for DL BWP switch in synchronous EN-DC

| Parameter |  | Unit | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 |  |
| Duplex mode | Config 1,4 |  | FDD |  |
|  | Config 2,3,5,6 |  | TDD |  |
| TDD configuration | Config 1,4 |  | Not Applicable |  |
|  | Config 2,5 |  | TDDConf.1.1 |  |
|  | Config 3,6 |  | TDDConf.2.1 |  |
| BWchannel | Config 1,4 |  | Note 7 |  |
|  | Config 2,5 |  | Note 7 |  |
|  | Config 3,6 |  | Note 7 |  |
| BWoccupied |  | Config 1,4 | RB | 52 Note 5 |  |
|  |  | Config 2,5 |  | 52 Note 5 |  |
|  |  | Config 3,6 |  | 106 Note 6 |  |
| Active BWP ID |  |  | 1, 2 | 0 |
| Initial BWP | Config 1,4 |  | DLBWP.0.2 | DLBWP.0.2 |
| Configuration | Config 2,5 |  |  |  |
|  | Config 3,6 |  |  |  |
| Active BWP-0 | Config 1,4 |  | NA | DLBWP.0.2 |
| Configuration | Config 2,5 |  |  |  |
|  | Config 3,6 |  |  |  |
| Active BWP-1 | Config 1,4 |  | DLBWP.1.3 | NA |
| Configuration | Config 2,5 |  |  |  |
|  | Config 3,6 |  |  |  |
| Active BWP-2 | Config 1,4 |  | DLBWP.1.1 | NA |
| Configuration | Config 2,5 |  |  |  |
|  | Config 3,6 |  |  |  |
| PDSCH Reference | Config 1,4 |  | SR.1.1 FDD |  |
| measurement channel | Config 2,5 |  | SR.1.1 TDD |  |
|  | Config 3,6 |  | SR2.1 TDD |  |
| RMSI CORESET | Config 1,4 |  | CR.1.1 FDD |  |
| parameters | Config 2,5 |  | CR.1.1 TDD |  |
|  | Config 3,6 |  | CR2.1 TDD |  |
| Dedicated CORESET | Config 1,4 |  | CCR.1.1 FDD |  |
| parameters | Config 2,5 |  | CCR.1.1 TDD |  |
|  | Config 3,6 |  | CCR.2.3 TDD |  |
| OCNG Patterns | Config 1,2,4,5 |  | OP.1 Note 5 |  |
|  | Config 3,6 |  | OP.1 Note 6 |  |
| SSB Configuration | Config 1,2,4,5 |  | SSB.1 FR1 |  |
|  | Config 3,6 |  | SSB.2 FR1 |  |
| SMTC Configuration |  |  | SMTC.1 |  |
| TRS Configuration | Config 1,4 |  | TRS.1.1 FDD |  |
|  | Config 2,5 |  | TRS.1.1 TDD |  |
|  | Config 3,6 |  | TRS.1.2 TDD |  |
| Antenna Configuration |  |  | 1x2 |  |
| Propagation Condition |  |  | AWGN |  |
| EPRE ratio of PSS to SSS |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 | 0 |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| NocNote 2 |  | dBm/15 kHz | [-104] | [-104] |
| SS-RSRP Note 3 |  | dBm/15 kHz | [-87] | [-87] |
| Ês/Iot |  | dB | 17 | 17 |
| Ês/Noc |  | dB | 17 | 17 |
| IoNote3 | Config 1,2,4,5 | dBm/9.36MHz | [-59] | [-59] |
|  | Config 3,6 | dBm/38.16MHz | [-61.9] | [-61.9] |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled within BWoccupied. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. Note 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |  |

A.4.5.6.1.2.2	Test Requirements

During T1, the UE shall start to send the ACK for PSCell in the DL slot right after slot (i+TBWPswitchDelay+k1).

During T3, the UE shall start to send the ACK for PSCell in the DL slot right after slot (j+TBWPswitchDelay+k11).

All of the above test requirements shall be fulfilled in order for the observed PSCell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

During T1, the start of the interruption of PCell during PSCell active BWP switch shall not happen outside the BWP switch delay.

During T3, the start of the interruption of PCell during PSCell active BWP switch shall not happen outside the BWP switch delay.

The interruption of PCell shall not be longer than the interruption duration specified for active BWP switch in clause 7.32.2.7 of TS 36.133 [15].

During T1, the start of the interruption of SCell during PSCell active BWP switch shall not happen outside the BWP switch delay.

During T3, the start of the interruption of SCell during PSCell active BWP switch shall not happen outside the BWP switch delay.

The interruption of SCell shall not be longer than the interruption duration specified for active BWP switch in clause 8.6.2.

All of the above test requirements shall be fulfilled in order for the observed PCell active BWP switch interruption to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	During T1, T3 if there are no uplink resources for reporting the ACK in the DL slot right after slot (i+TBWPswitchDelay+k1), (j+TBWPswitchDelay+k1), then the UE shall use the next available uplink resource for reporting the corresponding ACK.

Editor’s note: FFS value of k1 for type 1 and type 2 UE.

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.6.5.2	Interruption

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1

##### A.6.5.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE missed ACK/NACK rate does not exceed the limits at NR PSCell interruptions during the measurement on the deactivated NR SCC. This test will verify the missed ACK/NACK rate for PCell in standalone NR specified in clause 8.2.2.2. Supported test configurations are shown in table A.6.5.2.1.1-1.

The general test parameters and NR cell specific test parameters are given in Table A.6.5.2.1.1-2 and A 6.5.2.1.1-3 below. In the test there are two cells: Cell1 and Cell2. Cell1 is PCell, Cell2 is an NR deactivated SCell. Cell1 shall be configured as PCell and Cell2 shall be configured as SCell.

The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, the UE is connected to Cell1 and Cell2 and the RRC message including measCycleSCell or allowInterruptions for the deactivated NR SCells is received at the UE antenna connector. During T1, PCell is continuously scheduled in DL.

Table A.6.5.2.1.1-1: Interruptions during measurements on deactivated NR SCC supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD – FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD – TDD duplex mode |
| 3 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD – FDD duplex mode |
| 4 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD – TDD duplex mode |
| 5 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD – TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.6.5.2.1.1-2: General test parameters for interruptions during measurements on deactivated NR SCC in standalone NR

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | Two NR RF channels |
| Active PCell |  | Cell1 | PCell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell2 | Deactivated SCell on NR RF channel number 2. |
| CP length |  | Normal | Applicable to Cell1 and Cell2 |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| SCell measurement cycle (measCycleSCell) | ms | 640 |  |
| T1 | s | 10 |  |

Table A.6.5.2.1.1-3: NR cell specific test parameters for interruptions during measurements on deactivated NR SCC in standalone NR

| Parameter |  | Unit | Cell1 | Cell2 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 | FR1 |
| Duplex mode | Config 1 |  | FDD | FDD |
|  | Config 2,5 |  | TDD | TDD |
|  | Confiq 3 |  | TDD | FDD |
|  | Confiq 4 |  | FDD | TDD |
| TDD configuration | Config 1 |  | Not Applicable | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.1.1 | Not Applicable |
|  | Confiq 4 |  | Not Applicable | TDDConf.1.1 |
|  | Confiq 5 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel | Config 1,2,3,4 |  | Note 9 | Note 9 |
|  | Config 5 |  | Note 9 | Note 9 |
| BWoccupied | Config 1,2,3,4 | RB | 52 Note 7 | 52 Note 7 |
|  | Config 5 |  | 106 Note 8 | 106 Note 8 |
| Initial DL BWP Configuration | Config 1,2,3,4 |  | DLBWP.0.1 | DLBWP.0.1 |
|  | Config 5 |  | DLBWP.0.1 | DLBWP.0.1 |
| Dedicated DL BWP Configuration | Config 1,2,3,4 |  | DLBWP.1.1 | DLBWP.1.1 |
|  | Config 5 |  | DLBWP.1.1 | DLBWP.1.1 |
| Initial UL BWP Configuration | Config 1,2,3,4 |  | ULBWP.0.1 | ULBWP.0.1 |
|  | Config 5 |  | ULBWP.0.1 | ULBWP.0.1 |
| Dedicated UL BWP Configuration | Config 1,2,3,4 |  | ULBWP.1.1 | ULBWP.1.1 |
|  | Config 5 |  | ULBWP.1.1 | ULBWP.1.1 |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  | Config 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  | Config 3 |  | SR.1.1 TDD | SR.1.1 FDD |
|  | Confiq 4 |  | SR.1.1 FDD | SR.1.1 TDD |
|  | Confiq 5 |  | SR.2.1 TDD | SR.2.1 TDD |
| CSI-RS for tracking | Config 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  | Config 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  | Config 3 |  | TRS.1.1 TDD | TRS.1.1 FDD |
|  | Confiq 4 |  | TRS.1.1 FDD | TRS.1.1 TDD |
|  | Confiq 5 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  | Config 3 |  | CR.1.1 TDD | CR.1.1 FDD |
|  | Confiq 4 |  | CR.1.1 FDD | CR.1.1 TDD |
|  | Confiq 5 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  | Config 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  | Config 3 |  | CCR.1.1 TDD | CCR.1.1 FDD |
|  | Config 4 |  | CCR.1.1 FDD | CCR.1.1 TDD |
|  | Config 5 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| OCNG Patterns | Config 1,2,3,4 |  | OP.1 Note 7 | OP.1 Note 7 |
|  | Config 5 |  | OP.1 Note 8 | OP.1 Note 8 |
| SMTC Configuration |  |  | SMTC.1 | SMTC.1 |
| SSB Configuration | Config 1,2,3,4 |  | SSB.1 FR1 | SSB.1 FR1 |
|  | Config 5 |  | SSB.2 FR1 | SSB.2 FR1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| NocNote 2 |  | dBm/15 kHz | -104 | -104 |
| SS-RSRP Note 3 |  | dBm/15 kHz | -87 | -87 |
| Ês/Iot |  | dB | 17 | 17 |
| Ês/Noc |  | dB | 17 | 17 |
| NocNote 2 | Config 1,2,3,4 | dBm/SCS | -104 | -104 |
|  | Config 5 |  | -101 | -101 |
| IoNote3 | Config 1,2,3,4 | dBm/ 9.36MHz | -58.96 | -58.96 |
|  | Config 5 | dBm/ 38.16MHz | -52.86 | -52.86 |
| Time offset to Cell1 Note 5 |  | s | - | 3 |
| Propagation Condition |  |  | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modeled as AWGN of appropriate power for Noc to be fulfilled within BWoccupied. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselvess. Note 4: Void Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 6: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2 defined in clause 12 of TS 38.213 [3]. Note 7: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 8: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 9: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |  |

A.6.5.2.1.2	Test Requirements

The UE shall be continuously scheduled on PCell during the entire length of T1. During the time duration T1 the UE shall transmit at least 99.5% of ACK/NACK on PCell.

If the NR PCell is not in the same band as the deactivated SCell, the UE is only allowed to cause interruptions on NR PCell immediately before and immediately after an SMTC. Each interruption on NR PCell shall not exceed the value defined in Table A.6.5.2.1.2-1.

If the NR PCell is in the same band as the deactivated SCell, the UE is only allowed to cause an interruption on PCell no earlier than 1 slot before an SMTC and no later than 1 slot after the SMTC. the interruption on NR PCell shall not exceed the value defined in Table A.6.5.2.1.2-2.

Table A.6.5.2.1.2-1: Interruption duration if the PCell is not in the same band as the deactivated SCell

|  | NR Slot length (ms) | Interruption length |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |

Table A.6.5.2.1.2-2: Interruption duration if the PCell is in the same band as the deactivated SCell

|  | NR Slot length (ms) | Interruption length |
| --- | --- | --- |
| 0 | 1 | 2 + SMTC duration |
| 1 | 0.5 | 2 + SMTC duration |

The rate of correct events observed during repeated tests shall be at least 90%.

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.6.5.3	SCell Activation and Deactivation Delay

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle

A.6.5.3.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in clause 8.3, when the SCell in FR1 is known by the UE at the time of activation.

The supported test configurations are shown in table A.6.5.3.1.1-1 below. The test parameters are given in Tables A.6.5.3.1.1-2 and cell-specific parameters in A.6.5.3.1.1-3 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two NR carriers, each with one cell. Both cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1, but is not aware of Cell2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2. The UE now starts monitoring the SCC. The test equipment sends a MAC message for activation of the SCell.

The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n, defines the start of time period T2. The UE shall be able to report valid CSI in PCell for the activated SCell at latest in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as defined in clause 8.3. The UE shall start reporting CSI in PCell in slot n+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3, where Ninterruption is the interruption length given in clause 8.2.

Time period T3 starts when a MAC message for deactivation of SCell, sent from the test equipment to the UE in a slot # denoted m, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell in a slot m+THARQ+3msNR slot length, as defined in clause 8.3, and The starting point of any PCell interruption due to the deactivation shall occur in the slot m+1+THARQNR slot length to m+1+THARQ+3msNR slot length, as defined in clause 8.3.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell deactivation command is sent until CQI reporting for SCell is discontinued.

Table A.6.5.3.1.1-1: known FR1 SCell activation in non-DRX for 160ms SCell measurement cycle supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.6.5.3.1.1-2: General test parameters for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channel (1, 2) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every four slot |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 7 | During this time the PSCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| THARQ | ms | k1×NR slot length | k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [3] depends on UE’s capability |
| TCSI_Reporting | ms | 10+5⋅2µDL | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] µDL is the subcarrier spacing configuration for DL |

Table A.6.5.3.1.1-3: Cell specific test parameters for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter |  | Unit | T1 |  |  | T2 |  | T3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |
| Duplex mode | Config 1 |  |  |  |  | FDD |  |  |  |  |  |  |  |
|  | Config 2,3 |  |  |  |  | TDD |  |  |  |  |  |  |  |
| TDD configuration | Config 1 |  |  |  |  | Not applicable |  |  |  |  |  |  |  |
|  | Config 2 |  |  |  |  | TDDConf.1.1 |  |  |  |  |  |  |  |
|  | Config 3 |  |  |  |  | TDDConf.2.1 |  |  |  |  |  |  |  |
| BWchannel | Config 1,2 |  |  |  | MHz | Note 7 |  |  |  |  |  |  |  |
|  | Config 3 |  |  |  |  | Note 7 |  |  |  |  |  |  |  |
| BWoccupied | Config 1,2 |  |  |  | RB | 52 Note 5 |  |  |  |  |  |  |  |
|  | Config 3 |  |  |  |  | 106 Note 6 |  |  |  |  |  |  |  |
| Initial BWP configuration |  |  | DLBWP.0.2 |  |  |  |  |  |  |  |
| TCI state |  |  | TCI.State.0 |  |  |  |  |  |  |  |
| TRS Configuration |  |  | TRS.1.1 TDD |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel | Config 1 |  |  |  |  | SR.1.1 FDD |  | - | SR.1.1 FDD | - | SR.1.1 FDD |  | - |
|  | Config 2 |  |  |  |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  | SR.1.1 TDD |  |  |
|  | Config 3 |  |  |  |  | SR2.1 TDD |  |  | SR2.1 TDD |  | SR2.1 TDD |  |  |
| Dedicated CORESET parameters | Config 1 |  |  |  |  | CCR.1.1 FDD |  | - | CCR.1.1 FDD | - | CCR.1.1 FDD |  | - |
|  | Config 2 |  |  |  |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |
|  | Config 3 |  |  |  |  | CCR2.1 TDD |  |  | CCR2.1 TDD |  | CCR2.1 TDD |  |  |
| RMSI CORESET parameters | Config 1 |  |  |  |  | CR.1.1 FDD |  | - | CR.1.1 FDD | - | CR.1.1 FDD |  | - |
|  | Config 2 |  |  |  |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  | CR.1.1 TDD |  |  |
|  | Config 3 |  |  |  |  | CR2.3 TDD |  |  | CR2.3 TDD |  | CR2.3 TDD |  |  |
| OCNG Patterns | Config 1,2 |  | OP.1 Note 5 |  |  |  |  |  |  |  |
|  | Config 3, |  | OP.1 Note 6 |  |  |  |  |  |  |  |
| SSB Configuration |  |  |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |
| CSI-RS configuration for CSI reporting |  |  |  | Config 1 |  | CSI-RS.1.1 FDD |  |  |  |  |  |  |  |
|  |  |  |  | Config 2 |  | CSI-RS.1.1 TDD |  |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  | CSI-RS.2.1 TDD |  |  |  |  |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |  |  |  |  |
| CSI reporting periodicity |  | ms | 5 |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | Config 1,2,4,5 |  | dBm/15kHz | -104 |  |  |  |  |  |  |  |
|  |  |  | Config 3,6 |  |  | -101 |  |  |  |  |  |  |  |
|  |  | dB | 17 |  |  |  |  |  |  |  |
|  |  | dB | 17 |  |  |  |  |  |  |  |
| SS-RSRPNote3 |  |  | Config 1,2,4,5 |  | dBm/SCS | -87 |  |  |  |  |  |  |  |
|  |  |  | Config 3,6 |  |  | -84 |  |  |  |  |  |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -87 |  |  |  |  |  |  |  |
| Io Note3 |  |  |  | Config 1,2,4,5 | dBm/ 9.36MHz | -58.96 |  |  |  |  |  |  |  |
|  |  |  |  | Config 3,6 | dBm/ 38.16MHz | -52.87 |  |  |  |  |  |  |  |
| Propagation condition |  | - | AWGN |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. Note 3: SS-RSRP, Io and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |  |  |  |  |  |  |  |

A.6.5.3.1.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell in the first available uplink resource after slot (n+1+THARQ+3msNR slot length). UE is allowed to postpone CSI report to next available uplink resource if an available uplink resource is subject to interruption.

During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a slot n+THARQ+Tactivtion_time+TCSI_ReportingNR slot length, Tactivation_time = TFirstSSB+ 5ms, as defined in clause 8.3.

During T3 the UE shall stop sending CSI reports for SCell at latest in a slot m+THARQ+3msNR slot length, as defined in clause 8.3.

During T2 interruption of PCell / PSCell during SCell activation shall not happen outside the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3.

During T3 the starting point of interruption of PCell during SCell deactivation shall not happen outside the slot m+1+THARQNR slot length to m+1+THARQ+3msNR slot length, as defined in clause 8.3.

The interruption on any activated serving cell shall not be more than the values specified for SA in clause 8.2.2.2.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay and SCell deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a slot THARQ+Tactivtion_time+TCSI_ReportingNR slot length as defined in clause 8.3 then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 320ms SCell measurement cycle

A.6.5.3.2.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in clause A.6.5.3.1.1. The supported test configurations are the same as defined in clause A.6.5.3.1.1. The test parameters are the same except those described in the following clause. The listed parameter values in Tables A.6.5.3.2.1-1 will replace the values of corresponding parameters in Tables A.4.5.3.1.1-1.

Table A.6.5.3.2.1-1: General test parameters for known FR1 SCell activation case, 320ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |

A.6.5.3.2.2	Test Requirements

The test requirements defined in clause A.6.5.3.1.2 shall apply to this test case, except Tactivation_time will be replaced with the value TFirstSSB_MAX + Trs + 5ms.

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX

A.6.5.3.3.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in clause 8.3, when the SCell in FR1 is known by the UE at the time of activation.

The supported test configurations are shown in table A.6.5.3.1.1-1 below. The test parameters are given in Tables A.6.5.3.1.1-2 and cell-specific parameters in A.6.5.3.1.1-3 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two NR carriers, each with one cell. Both cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1, but is not aware of Cell2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2. The UE now starts monitoring the SCC. The test equipment sends a MAC message for activation of the SCell.

The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n, defines the start of time period T2. The UE shall be able to report valid CSI in PCell for the activated SCell at latest in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as defined in clause 8.3. The UE shall start reporting CSI in PCell in slot n+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3, where Ninterruption is the interruption length given in clause 8.2.

Time period T3 starts when a MAC message for deactivation of SCell, sent from the test equipment to the UE in a slot # denoted m, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell in a slot m+THARQ+3msNR slot length, as defined in clause 8.3, and The starting point of any PCell interruption due to the deactivation shall occur in the slot m+1+THARQNR slot length to m+1+THARQ+3msNR slot length, as defined in clause 8.3.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell deactivation command is sent until CQI reporting for SCell is discontinued.

Table A.6.5.3.3.1-1: General test parameters for unknown FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| T1 | ms | 100 | During this time the PSCell shall be known and the SCell configured, but not detected. |

A.6.5.3.3.2	Test Requirements

The test requirements defined in clause A.6.5.3.1.2 shall apply to this test case, except Tactivation_time will be replaced with the value TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms as defined in clause 8.3.

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.6.5.4	UE UL carrier RRC reconfiguration Delay

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay

Table A.6.5.4.1-1 - Table A.6.5.4.1-4 : Void

A.6.5.4.1.1	Test Purpose and Environment

The purpose of this test is to verify that when the UE receives a RRC message implying NR UL or Supplementary UL carrier configuration, the UE shall be ready to start transmission on the newly configured carrier within the time limits specified in clause 8.4.2 and 8.4.3 for configuring and deconfiguring, respectively.

There are two cells: FR1 PCell (cell 1) and FR1 SCell (cell 2). Both NR uplink and supplementary uplink are broadcast by ServingCellConfigCommonSIB. The test parameters for PCell and SCell are given in Table A. 6.5.4.1.1-1, Table A.6.5.4.1.1-2, Table A.6.5.4.1.1-3 and Table A.6.5.4.1.1-4 below.  In test 1, the test consists of three time periods, with duration of T1, T2 and T3 respectively. During time duration T1, NR uplink of cell 2 is configured to UE. At the start of T2, a supplementary uplink of cell 2 is configured to UE through RRCReconfiguration, then UE shall start transmission both on the NR uplink and supplementary uplink. At the start of T3, the supplementary uplink is released through RRCReconfiguration.

In test 2, the test consists of three time periods, with duration of T1, T2 and T3 respectively. During time duration T1, supplementray uplink on cell 2 is configured to UE. At the start of T2, a NR uplink is configured to UE through RRCReconfiguration, then UE shall start transmission both on the NR uplink and supplementary uplink. At the start of T3, the NR uplink is released through RRCReconfiguration.

Table A.6.5.4.1.1-1: Supported test configurations

| Configuration | PCell (Cell 1) | SCell (Cell 2) |
| --- | --- | --- |
| 1 | 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode | DL and UL: 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode; SUL: 15 kHz SCS, ≥10 MHz bandwidth, SUL duplex mode |
| 2 | 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode | DL and UL: 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode; SUL: 15 kHz SCS, ≥10 MHz bandwidth, SUL duplex mode |
| 3 | 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode | DL and UL: 30kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode; SUL: 30kHz SCS, ≥40 MHz bandwidth, SUL duplex mode |
| 4 | 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode | DL and UL: 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode; SUL: 15 kHz SCS, ≥10 MHz bandwidth, SUL duplex mode |
| 5 | 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode | DL and UL: 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode; SUL: 15 kHz SCS, ≥10 MHz bandwidth, SUL duplex mode |
| 6 | 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode | DL and UL: 30kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode; SUL: 30kHz SCS, ≥40 MHz bandwidth, SUL duplex mode |
| 7 | 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode | DL and UL: 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode; SUL: 15 kHz SCS, ≥10 MHz bandwidth, SUL duplex mode |
| 8 | 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode | DL and UL: 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode; SUL: 15 kHz SCS, ≥10 MHz bandwidth, SUL duplex mode |
| 9 | 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode | DL and UL: 30kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode; SUL: 30kHz SCS, ≥40 MHz bandwidth, SUL duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |  |

Table A.6.5.4.1.1-2: General test parameters for NR standalone UE UL carrier RRC reconfiguration Delay on Pcell

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | Config 1,2,3, 4, 5, 6, 7, 8, 9 | 1, 2 | Two radio channels are used for these two tests. |
| Active cell |  | Config 1,2,3, 4, 5, 6, 7, 8, 9 | Cell 1: FR1 PCell Cell 2: FR1 SCell | PCell on RF channel number 1 FR1 SCell on RF channel number 2 |
| CP length |  | Config 1,2,3, 4, 5, 6, 7, 8, 9 | Normal |  |
| DRX |  | Config 1,2,3, 4, 5, 6, 7, 8, 9 | OFF |  |
| Measurement gap pattern Id |  | Config 1,2,3, 4, 5, 6, 7, 8, 9 | OFF |  |
| Filter coefficient |  | Config 1,2,3, 4, 5, 6, 7, 8, 9 | 0 | L3 filtering is not used |
| T1 | s | Config 1,2,3, 4, 5, 6, 7, 8, 9 | 5 |  |
| T2 | s | Config 1,2,3, 4, 5, 6, 7, 8, 9 | 5 |  |
| T3 | s | Config 1,2,3, 4, 5, 6, 7, 8, 9 | 5 |  |

Table A.6.5.4.1.1-3: NR Cell specific test parameters for NR standalone UE UL carrier RRC reconfiguration Delay on PCell (Cell 1)

| Parameter | Unit | Test Configuration | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Channel number |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 1 |  |  | 1 |  |  |
| TDD configuration |  | Conf 1, 2, 3 | N/A |  |  | N/A |  |  |
|  |  | Conf 4, 5, 6 | TDD Conf.1.1 |  |  | TDD Conf.1.1 |  |  |
|  |  | Conf 7, 8, 9 | TDD Conf.2.1 |  |  | TDD Conf.2.1 |  |  |
| BWchannel | MHz | Conf 1, 2, 3 | Note 6 |  |  | Note 6 |  |  |
|  |  | Conf 4, 5, 6 | Note 6 |  |  | Note 6 |  |  |
|  |  | Conf 7, 8, 9 | Note 6 |  |  | Note 6 |  |  |
| BWoccupied | RB | Conf 1, 2, 3 | 52 Note 4 |  |  | 52 Note 4 |  |  |
|  |  | Conf 4, 5, 6 | 52 Note 4 |  |  | 52 Note 4 |  |  |
|  |  | Conf 7, 8, 9 | 106 Note 5 |  |  | 106 Note 5 |  |  |
| PDSCH reference measurement channel as defined in A.3.1.1 |  | Conf 1, 2, 3 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
|  |  | Conf 4, 5, 6 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | Conf 7, 8, 9 | SR 2.1 TDD |  |  | SR 2.1 TDD |  |  |
| RMSI CORESET reference measurement channel as defined in A.3.1.2 |  | Conf 1, 2, 3 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
|  |  | Conf 4, 5, 6 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | Conf 7, 8, 9 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| RMC CORESET reference measurement channel as defined in A.3.1.3 |  | Conf 1, 2, 3 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
|  |  | Conf 4, 5, 6 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | Conf 7, 8, 9 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern Note 1 |  | Conf 1, 2, 3, 4, 5, 6 | OP.1 Note 4 |  |  | OP.1 Note 4 |  |  |
|  |  | Config 7, 8, 9 | OP.1 Note 5 |  |  | OP.1 Note 5 |  |  |
| SSB configuration |  | Conf 1, 2, 3, 4, 5, 6 | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |
|  |  | Conf 7, 8, 9 | SSB.2 FR1 |  |  | SSB.2 FR1 |  |  |
| SMTC configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | SMTC.1 |  |  | SMTC.1 |  |  |
| CSI-RS for tracking |  | Conf 1 | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  |  |
|  |  | Conf 2 | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  |  |
|  |  | Conf 3 | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  |  |
|  |  | Conf 4 | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | Conf 5 | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | Conf 6 | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | Conf 7 | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
|  |  | Conf 8 | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
|  |  | Conf 9 | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| DL initial BWP configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| DL dedicated BWP configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | DLBWP.1.1 |  |  | DLBWP.1.1 |  |  |
| UL dedicated BWP configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | ULBWP.1.1 |  |  | ULBWP.1.1 |  |  |
| EPRE ratio of PSS to SSS | dB | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 0 |  |  | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |  |  |
| Note 2 | dBm / 15kHz | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | -102 |  |  | -102 |  |  |
|  | dBm/ SCS | Conf 1,2,3,4,5,6 | -102 |  |  | -102 |  |  |
|  |  | Conf 7,8,9 | -99 |  |  | -99 |  |  |
|  | dB | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 16 | 16 | 16 | 16 | 16 | 16 |
| Note 3 | dB | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 16 | 16 | 16 | 16 | 16 | 16 |
| SS-RSRP Note 3 | dBm/ SCS | Conf 1,2,3,4,5,6 | -86 | -86 | -86 | -86 | -86 | -86 |
|  |  | Conf 7,8,9 | -83 | -83 | -83 | -83 | -83 | -83 |
| Io Note 3 | dBm/ 9.36 MHz | Conf 1,2,3,4,5,6 | -57.9 | -57.9 | -57.9 | -57.9 | -57.9 | -57.9 |
|  | dBm/ 38.16MHz | Conf 7,8,9 | -51.8 | -51.8 | -51.8 | -51.8 | -51.8 | -51.8 |
| Propagation Condition |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | AWGN |  |  | AWGN |  |  |
| Antenna configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 1 x 2 |  |  | 1 x 2 |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. NOTE 3: , Io, and SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 6: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |  |  |  |  |  |

Table A.6.5.4.1.1-4: NR Cell specific test parameters for NR standalone UE UL carrier RRC reconfiguration Delay on SCell (Cell 2)

| Parameter | Unit | Test Configuration | Test 1 |  |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |  | T3 | T1 | T2 | T3 |
| Channel number |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 2 |  |  |  | 2 |  |  |
| TDD configuration |  | Conf 1, 4, 7 | N/A |  |  |  | N/A |  |  |
|  |  | Conf 2, 5, 8 | TDDConf.1.1 |  |  |  | TDDConf.1.1 |  |  |
|  |  | Conf 3, 6, 9 | TDDConf.2.1 |  |  |  | TDDConf.2.1 |  |  |
| BWchannel | MHz | Conf 1, 4, 7 | Note 6 |  |  |  | Note 6 |  |  |
|  |  | Conf 2, 5, 8 | Note 6 |  |  |  | Note 6 |  |  |
|  |  | Conf 3, 6, 9 | Note 6 |  |  |  | Note 6 |  |  |
| BWoccupied | RB | Conf 1, 4, 7 | 52 Note 4 |  |  |  | 52 Note 4 |  |  |
|  |  | Conf 2, 5, 8 | 52 Note 4 |  |  |  | 52 Note 4 |  |  |
|  |  | Conf 3, 6, 9 | 106 Note 5 |  |  |  | 106 Note 5 |  |  |
| PUSCH parameters for NR UL carrier |  | Conf 1, 4, 7 | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] |  | N/A | G-FR1-A3-10 in [13] | N/A |
|  |  | Conf 2, 5, 8 | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] |  | N/A | G-FR1-A3-10 in [13] | N/A |
|  |  | Conf 3, 6, 9 | G-FR1-A3-14 in [13] | G-FR1-A3-14 in [13] | G-FR1-A3-14 in [13] |  | N/A | G-FR1-A3-14 in [13] | N/A |
| PUCCH parameters For NR UL carrier |  | Conf 1, 4, 7 | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] |  | N/A | N/A | N/A |
|  |  | Conf 2, 5, 8 | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] |  | N/A | N/A | N/A |
|  |  | Conf 3, 6, 9 | Table 8.3.3.1.2-2 in [13] | Table 8.3.3.1.2-2 in [13] | Table 8.3.3.1.2-2 in [13] |  | N/A | N/A | N/A |
| PUSCH parameters for supplementary UL |  | Conf 1, 4, 7 | N/A | G-FR1-A3-10 in [13] | N/A |  | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] |
|  |  | Conf 2, 5, 8 | N/A | G-FR1-A3-10 in [13] | N/A |  | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] |
|  |  | Conf 3, 6, 9 | N/A | G-FR1-A3-14 in [13] | N/A |  | G-FR1-A3-14 in [13] | G-FR1-A3-14 in [13] | G-FR1-A3-14 in [13] |
| PUCCH parameters for supplementary UL |  | Conf 1, 4, 7 | N/A | N/A | N/A |  | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] |
|  |  | Conf 2, 5, 8 | N/A | N/A | N/A |  | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] |
|  |  | Conf 3, 6, 9 | N/A | N/A | N/A |  | Table 8.3.3.1.2-2 in [13] | Table 8.3.3.1.2-2 in [13] | Table 8.3.3.1.2-2 in [13] |
| PDSCH reference measurement channel as defined in A.3.1.1 |  | Conf 1, 4, 7 | SR.1.1 FDD |  |  |  | SR.1.1 FDD |  |  |
|  |  | Conf 2, 5, 8 | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |
|  |  | Conf 3, 6, 9 | SR 2.1 TDD |  |  |  | SR 2.1 TDD |  |  |
| RMSI CORESET reference measurement channel as defined in A.3.1.2 |  | Conf 1, 4, 7 | CR.1.1 FDD |  |  |  | CR.1.1 FDD |  |  |
|  |  | Conf 2, 5, 8 | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |
|  |  | Conf 3, 6, 9 | CR.2.1 TDD |  |  |  | CR.2.1 TDD |  |  |
| RMC CORESET reference measurement channel as defined in A.3.1.3 |  | Conf 1, 4, 7 | CCR.1.1 FDD |  |  |  | CCR.1.1 FDD |  |  |
|  |  | Conf 2, 5, 8 | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |  |
|  |  | Conf 3, 6, 9 | CCR.2.1 TDD |  |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern Note 1 |  | Conf 1, 2, 4, 5, 7, 8 | OP.1 Note 4 |  |  |  | OP.1 Note 4 |  |  |
|  |  | Conf 3, 6, 9 | OP.1 Note 5 |  |  |  | OP.1 Note 5 |  |  |
| SSB configuration |  | Conf 1, 2, 4, 5, 7,8 | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |
|  |  | Conf 3, 6, 9 | SSB.2 FR1 |  |  |  | SSB.2 FR1 |  |  |
| SMTC configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | SMTC.1 |  |  |  | SMTC.1 |  |  |
| CSI-RS for tracking |  | Conf 1 | TRS.1.1 FDD |  |  |  | TRS.1.1 FDD |  |  |
|  |  | Conf 2 | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  |
|  |  | Conf 3 | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |
|  |  | Conf 4 | TRS.1.1 FDD |  |  |  | TRS.1.1 FDD |  |  |
|  |  | Conf 5 | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  |
|  |  | Conf 6 | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |
|  |  | Conf 7 | TRS.1.1 FDD |  |  |  | TRS.1.1 FDD |  |  |
|  |  | Conf 8 | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  |
|  |  | Conf 9 | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |
| DL initial BWP configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | DLBWP.0.1 |  |  |  | DLBWP.0.1 |  |  |
| DL dedicated BWP configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | DLBWP.1.1 |  |  |  | DLBWP.1.1 |  |  |
| UL dedicated BWP configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | ULBWP.1.1 |  |  |  | ULBWP.1.1 |  |  |
| EPRE ratio of PSS to SSS | dB | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 0 |  |  |  | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |  |  |  |
| Note 2 | dBm / 15kHz | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | -102 |  |  |  | -102 |  |  |
|  | dBm/ SCS | Conf 1, 2, 4, 5, 7,8 | -102 |  |  |  | -102 |  |  |
|  |  | Conf 3, 6, 9 | -99 |  |  |  | -99 |  |  |
|  | dB | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 16 | 16 |  | 16 | 16 | 16 | 16 |
| Note 3 | dB | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 16 | 16 |  | 16 | 16 | 16 | 16 |
| SS-RSRP Note 3 | dBm/ SCS | Conf 1, 2, 4, 5, 7,8 | -86 | -86 |  | -86 | -86 | -86 | -86 |
|  |  | Conf 3, 6, 9 | -83 | -83 |  | -83 | -83 | -83 | -83 |
| Io Note 3 | dBm/ 9.36 MHz | Conf 1, 2, 4, 5, 7,8 | -57.9 | -57.9 |  | -57.9 | -57.9 | -57.9 | -57.9 |
|  | dBm/ 38.16MHz | Conf 3, 6, 9 | -51.8 | -51.8 |  | -51.8 | -51.8 | -51.8 | -51.8 |
| Propagation Condition |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | AWGN |  |  |  | AWGN |  |  |
| Antenna configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 1 x 2 |  |  |  | 1 x 2 |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. NOTE 3: , Io, and SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 6: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |  |  |  |  |  |  |

A.6.5.4.1.2	Test Requirements

In test 1 the UE shall be ready to start transmission on the supplementary uplink carrier on SCell within 20ms from the start of T2.

In test 1 the UE shall stop the transmission on the supplementary uplink carrier on SCell within 20ms from the start of T3.

In test 2 the UE shall be ready to start transmission on the NR uplink carrier on SCell within 20ms from the start of T2.

In test 2 the UE shall stop the transmission on the NR uplink carrier on SCell within 20ms from the start of T3.

All of the above test requirements shall be fulfilled in order for the observed UE UL carrier configuration delay and UE UL carrier release delay to be counted as correct. The rate of correct observed UE UL carrier configuration delay and UE UL carrier release delay during repeated tests shall be at least 90%.

<<End of change>>

<<Unchaged sections skipped>>

<<Start of change>>

A.6.5.6	Active BWP switch

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA

A.6.5.6.1.1.1	Test Purpose and Environment

The purpose of this test is to verify the DL BWP switch delay requirement defined in clause 8.6, and interruption requirement on other active serving cell defined in clause 8.2.2.2.5.

The supported test configurations are shown in Table A.6.5.6.1.1.1-1 below. The test scenario comprises of one PCell (Cell 1) and one SCell (Cell 2) as given in Table A.6.5.6.1.1.1-2. NR Cell-specific parameters are specified in Table A.6.5.6.1.1.1-3 below.

PDCCHs indicating new transmissions shall be sent continuously on PCell (Cell 1) to ensure that the UE would have ACK/NACK sending except for the time duration when BWP is switching on Cell 1 and the time duration of T2.

PDCCHs indicating new transmissions shall be sent continuously on SCell (Cell 2) to ensure that the UE will have ACK/NACK sending.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), and Cell 2 (SCell) on radio channel 2 (SCC).

-	UE is configured with 2 different UE-specific downlink bandwidth parts for PCell, BWP-1 and BWP-2, in Cell 1 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

-	UE is configured with 1 UE-specific downlink bandwidth parts the same as initial BWP for SCell, BWP-0 in Cell 2 before starting the test.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 in PCell.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-0 in SCell.

-	UE is configured with a bwp-InactivityTimer timer value for PCell.

All cells have constant signal levels throughout the test.

The test consists of 3 successive time periods, with durations of T1, T2, and T3, respectively.

During T1,

Time period T1 starts when a DCI format 1_1 command for PCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in PCell’s slot # denoted i. The UE shall switch its bandwidth part from BWP-1 to BWP-2.

The UE shall be able to receive PDSCH no later than the first DL slot that occurs after the beginning of PCell’s DL slot (i+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the PCell no later than the first UL slot that occurs after the beginning of slot (i+TBWPswitchDelay+k1). The UE shall be continuously scheduled on PCell’s BWP-2 no later than the first DL slot that occurs after the beginning of slot (i+TBWPswitchDelay).

The starting time of SCell (Cell 2) interruption due to BWP switch on PCell shall occur within the BWP switch delay.

During T2, the test equipment won’t transmit DCI format for PDSCH reception on PCell (Cell 1).

During T3,

The time period T3 starts from the slot #j, where j is the first  slot of the subframe immediately after bwp-InactivityTimer timer expires. The UE should switch its bandwidth part from BWP-2 back to the default bandwidth part – BWP-1.

The UE shall be able to receive PDSCH no later than the first DL slot that occurs after the beginning of PCell’s slot (j+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the SCell at latest on the first UL slot that occurs after the beginning of slot (j+TBWPswitchDelay+k1). The UE shall be continuously scheduled on PCell’s BWP-1 no later than the first DL slot that occurs after the beginning of slot (j+TBWPswitchDelay).

The starting time of SCell (Cell 2) interruption due to BWP switch of PCell shall occur within the BWP switch delay.

The test equipment verifies the DL BWP switch time in PCell by counting the slots from the time when the BWP switch command is received or bwp-InactivityTimer timer expires till an ACK/NACK is received.

The test equipment verifies that potential interruption to SCell is carried out in the correct time span by monitoring ACK/NACK sent in SCell during BWP switch of PCell, respectively.

Table A.6.5.6.1.1.1-1: DL BWP switch supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD -FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD – TDD duplex mode |
| 3 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD – FDD duplex mode |
| 4 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD – TDD duplex mode |
| 5 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD - TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration |  |

Table A.6.5.6.1.1.1-2: General test parameters for DL BWP switch in SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1, 2 | Two NR radio channels are used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active SCell |  | Cell 2 | SCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | For both PCell and SCell |
| bwp-InactivityTimer | ms | 200 |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC. |
| Cell2 timing offset to cell1 | s | 3 | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. |
| T1 | s | 0.2 |  |
| T2 | s | 0.2 |  |
| T3 | s | 0.2 |  |

Table A.6.5.6.1.1.1-3: NR Cell specific test parameters for DL BWP switch in SA

| Parameter |  | Unit | Cell 1 | Cell2 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 | FR1 |
| Duplex mode | Config 1 |  | FDD | FDD |
|  | Config 2,5 |  | TDD | TDD |
|  | Config 3 |  | TDD | FDD |
|  | Config 4 |  | FDD | TDD |
| TDD configuration | Config 1 |  | Not Applicable | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.1.1 | Not Applicable |
|  | Config 4 |  | Not Applicable | TDDConf.1.1 |
|  | Config 5 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel | Config 1,2,3,4 |  | Note 7 | Note 7 |
|  | Config 5 |  | Note 7 | Note 7 |
| BWoccupied |  | Config 1,2,3,4 | RB | 52 Note 5 | 52 Note 5 |
|  |  | Config 5 |  | 106 Note 6 | 106 Note 6 |
| Active BWP ID |  |  | 1, 2 | 0 |
| Initial DL BWP Configuration |  |  | DLBWP.0.2Note4 |  |
| Initial UL BWP Configuration |  |  | ULBWP.0.2Note4 |  |
| Active DL BWP-0 Configuration |  |  | N.A. | DLBWP.0.2Note4 |
| Active DL BWP-1 Configuration |  |  | DLBWP.1.1Note4 | N.A. |
| Active DL BWP-2 Configuration |  |  | DLBWP.1.3Note4 | N.A. |
| Active UL BWP-0 Configuration |  |  | N.A. | ULBWP.0.2Note4 |
| Active UL BWP-1 Configuration |  |  | ULBWP.1.1Note4 | N.A. |
| Active UL BWP-2 Configuration |  |  | ULBWP.1.3Note4 | N.A. |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  | Config 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  | Config 3 |  | SR.1.1 TDD | SR.1.1 FDD |
|  | Config 4 |  | SR.1.1 FDD | SR.1.1 TDD |
|  | Config 5 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  | Config 3 |  | CR.1.1 TDD | CR.1.1 FDD |
|  | Config 4 |  | CR.1.1 FDD | CR.1.1 TDD |
|  | Config 5 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  | Config 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  | Config 3 |  | CCR.1.1 TDD | CCR.1.1 FDD |
|  | Config 4 |  | CCR.1.1 FDD | CCR.1.1 TDD |
|  | Config 5 |  | CCR.2.3 TDD | CCR.2.3 TDD |
| OCNG Patterns | Config 1,2,3,4 |  | OP.1 Note 5 |  |
|  | Config 5 |  | OP.1 Note 6 |  |
| SSB Configuration | Config 1,2,3,4 |  | SSB.1 FR1 |  |
|  | Config 5 |  | SSB.2 FR1 |  |
| SMTC Configuration |  |  | SMTC.1 |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |  |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| NocNote 2 | Config 1,2,3,4 |  | dBm/SCS | -104 | -104 |
|  | Config 5 |  |  | -101 | -101 |
| NocNote 2 |  | dBm/15KHz | -104 | -104 |
| SS-RSRP Note 3 | Config 1,2,3,4 |  | dBm/SCS | -87 | -87 |
|  | Config 5 |  |  | -84 | -84 |
| Ês/Iot |  | dB | 17 | 17 |
| Ês/Noc |  | dB | 17 | 17 |
| IoNote3 | Config 1,2,3,4 | dBm/ 9.36MHz | -58.96 | -58.96 |
|  | Config 5 | dBm/ 38.16MHz | -52.86 | -52.86 |
| Propagation Condition |  |  | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled within BWoccupied. Note 3 SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. Note 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |  |

A.6.5.6.1.1.2	Test Requirements

During T1, the UE shall start to send the ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+k1).

During T3, the UE shall start to send the ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot (j+TBWPswitchDelay+k1).

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [7].

Depending on UE capability bwp-SwitchingDelay [2], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in Table 8.6.2-1.

All of the above test requirements shall be fulfilled in order for the observed PCell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

During T1 and T3, the start time of SCell interruption during PCell active BWP switch shall not happen outside the BWP switch delay.

The interruption of SCell shall not be longer than the interruption duration specified for active BWP switch in clause 8.2.2.2.5.

All of the above test requirements shall be fulfilled in order for the observed PCell active BWP switch interruption to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	During T1, T3 if there are no uplink resources for reporting the ACK/NACK in the first DL slot that occurs after the beginning of DL slot (i+ TBWPswitchDelay+k1), (j+ TBWPswitchDelay+k1), then the UE shall use the next available uplink resource for reporting the corresponding ACK/NACK.

<<End of change>>