3GPP TSG-RAN WG4 Meeting # 100-e		R4-2111957

Electronic Meeting, August 16-27, 2021

Agenda item:	9.9.4.5

Source: 	CATT

Title: 	Discussion on Signalling characteristics requirements for HST RRM in FR2

Document for:	Discussion

Introduction

In RAN4#99-e meeting, a way forward on RRM requirements for NR FR2 HST was approved in [1].

In this contribution, we provide the further consideration of signalling characteristics requirements for HST RRM in FR2 and give our proposals.

Discussion

In way forward [1], the signalling impact on HST RRM in FR2 is listed as below:

Signalling; RLM; FFS. Interruption; No impact identified. SCell Activation and Deactivation Delay; Not applicable to FR2 HST. UE UL carrier RRC reconfiguration delay; Not applicable to FR2 HST. Link Recovery; FFS. Active BWP switch delay; No impact identified. Active TCI state switching delay; FFS. PSCell Change; Not applicable to FR2 HST. Uplink spatial relation switch delay; FFS. UE-specific CBW change; No impact identified. Pathloss reference signal switching delay; No impact identified.

- RLM

In current Rel-15 requirements, the evaluate periods are specified with scaling factor N=8 for FR2. It increases the time with larger values of other parameters such as P, number of samples, periodicity of RS. When CPE moves along the rail track, it moves out of the cell during the evaluate periods. In the discussion of RX beams number, the initial agreement is RX beams can be reduced from 8 to 1 or 2 in different scenarios.

Proposal 1: For RLM, scaling factor can be decreased as the decision of RX beams.

- Link Recovery

In current Rel-15 requirements, the evaluate periods are specified with scaling factor N=8 for FR2. It increases the time with larger values of other parameters such as P, number of samples, periodicity of RS. When CPE moves along the rail track, it moves out of the cell during the evaluate periods. In the  discussion of RX beams number, the initial agreement is RX beams can be reduced from 8 to 1 or 2 in different scenarios.

Proposal 2: For link recovery, scaling factor can be decreased as the decision of RX beams.

- Uplink spatial relation switch delay

In current Rel-15 requirements in 8.12 of TS 38.133, MAC-CE based spatial relation switch delay, TL1-RSPR_Measurement_Period_SSB for SSB and TL1-RSRP_Measurement_Period_CSI-RS for CSI-RS are different in FR1 and FR2. According to the definition in clause 9.5.4.1 and 9.5.4.2, N= 8 is used for FR2. The same reduction of N can be reused for uplink spatial relation switch delay.

Proposal 3: For uplink spatial relation switch delay, the requirements need to be enhanced for HST to adopt the same behaviour of that scaling factor can be decreased as the decision of RX beams.

Conclusion

In this contribution, we provide the further consideration of signalling for HST RRM in FR2 and present our proposals as below:

Proposal 1: For RLM, scaling factor can be decreased as the decision of RX beams.

Proposal 2: For link recovery, scaling factor can be decreased as the decision of RX beams.

Proposal 3: For uplink spatial relation switch delay, the requirements need to be enhanced for HST to adopt the same behaviour of that scaling factor can be decreased as the decision of RX beams.

Reference

[1] R4-2108342, WF on FR2 HST RRM requirements, Nokia, Nokia Shanghai Bell