3GPP TSG-RAN WG4 Meeting # 100-e	R4-2112280

Electronic Meeting, 16 – 27 August 2021

Agenda Item:	11.8.2.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposals on BS RF requirements for support of 16QAM in NB-IoT

Document for:	Approval

# 1.	Introduction

The work item for additional enhancements for NB-IoT and LTE-MTC was approved at TSG RAN#88-e [1]. One of the objectives of this work item is to support 16QAM in NB-IoT unicast in UL and DL. BS RF requirements for support of 16QAM in NB-IoT were further discussed at TSG RAN4#99-e, but no conclusion was made.

This contribution provides further proposals on BS RF requirements for the support of 16-QAM in NB-IoT unicast in UL and DL according to the approved WF at TSG RAN4#98-bis-e [2] and the agreements in RAN1 [3].

# 2.	Discussion

2.1	NB-IoT RB power dynamic range for in-band or guard band operation or NB-IoT operation in NR in-band

It is stated in [2]:

- NB-IoT RB power dynamic range for in-band or guard band operation or NB-IoT operation in NR in-band

  - RAN4 will decide whether to use the same QPSK requirements for 16QAM.

To enhance NB-IoT coverage, the +6 dB power dynamic range is currently required for one NB-IoT QPSK RB for both E-UTRA in-band and guard band operation modes, except for guard band operation with E-UTRA 5 MHz channel bandwidth signal where BS manufacturer shall declare the NB-IoT dynamic range power it could support (as specified in TS 36.104 [4]). For NB-IoT operation in NR in-band, the +6 or +3 dB power dynamic range is currently required for one NB-IoT QPSK RB depending on the NB-IoT RB frequency position within the BS channel bandwidth, as shown in table 1 below (copied from Table 6.3.4.2-1 in TS 38.104 [5]).

Table 1: NB-IoT RB power dynamic range for NB-IoT operation in NR in-band

| BS channel bandwidth (MHz) | NB-IoT RB frequency position | NB-IoT RB power dynamic range (dB) |
| --- | --- | --- |
| 5, 10 | Any | +6 |
| 15 | Within center 77*180kHz+15kHz at each edge | +6 |
|  | Other | +3 |
| 20 | Within center 102*180kHz+15kHz at each edge | +6 |
|  | Other | +3 |
| 25, 30, 40, 50, 60, 70, 80, 90, 100 | Within center 90% of BS channel bandwidth | +6 |
|  | Other | +3 |

Considering NB-IoT coverage remains important for 16QAM, it is proposed to apply the NB-IoT RB current power dynamic range for QPSK RB transmission in NB-IoT to 16QAM RB for in-band or guard band operation or NB-IoT operation in NR in-band.

There was convergence on this proposal in discussion at TSG RAN4#99-e, the open issue is whether changes to the specs are needed to define NB-IoT RB power dynamic range for 16QAM and whether new test cases are needed. This issue is related to whether max power back off (subject to vendor’s declaration) is allowed for 16QAM. The power back off for 16QAM, if allowed, should be considered in clauses 6.1.5 and 6.1.6 of TS 36.141 [6], where currently only Prated,c is used. We do not see a need to allow power back-off for 16QAM support, as the network coverage needs to be maintained compared to that using QPSK. Therefore, we see there is no need for changes to the specifications nor new test cases to define NB-IoT RB power dynamic range for 16QAM.

Proposal 1: The current NB-IoT RB power dynamic range for QPSK RB transmission in NB-IoT should apply to 16QAM RB for in-band or guard band operation or NB-IoT operation in NR in-band. There is no need for changes to the specifications nor new test cases to define NB-IoT RB power dynamic range for 16QAM.

2.2	Error Vector Magnitude

It is stated in [2]:

- EVM limit for 16QAM DL

  - RAN4 will decide whether to define 12.5% as the limit

The EVM limit is currently specified in [4] as 17.5% for NB-IoT QPSK transmission, which is the same as for E-UTRA QPSK transmission. Considering 16QAM has a higher target SNR than QPSK, it is proposed to specify a tighter EVM limit of 12.5% for NB-IoT 16QAM transmission, which is the same as for E-UTRA 16QAM transmission.

There was no concern raised in discussion at TSG RAN4#99-e to specify 12.5% as the EVM limit for NB-IoT 16QAM transmission, the open issue is again related to whether max power back off (subject to vendor’s declaration) is allowed for 16QAM. As aforementioned, we do not see a need to allow power back-off for 16QAM support, the network coverage needs to be maintained compared to that using QPSK. To progress the work, proponent of a different EVM requirement should provide technical justifications (e.g., preliminary simulation results) in next RAN4 meeting.

Proposal 2: An EVM limit of 12.5% should be specified for NB-IoT 16QAM transmission. Proponent of a different EVM requirement should provide technical justifications (e.g., preliminary simulation results) in next RAN4 meeting.

2.3	Fixed Reference Channels for Receiver Dynamic Range

It is stated in [2]:

- 16QAM FRC for BS Rx Characteristics

  - RAN4 will decide whether new 16QAM FRC is needed for NB-IoT BS RF tests.

For E-UTRA, uplink FRCs using 16QAM are used for the receiver dynamic range requirement [4] because the higher required SNR comparing to QPSK would present a more stringent condition for the receiver dynamic range. Moreover, as recorded in TR 36.804 [7], for E-UTRA this requirement measures the effects of receiver impairments such as receiver EVM, and in order to make the test sensitive to receiver EVM impairments, the wanted signal is a 16QAM signal. Therefore, new uplink FRC using 16QAM would need to be defined for receiver dynamic range requirement of NB-IoT BS supporting 16QAM to ensure satisfactory BS performance.

The UL TBS agreed in RAN1 for 16QAM in NB-IoT uplink is provided in table 2 below.

Table 2: TBS indices are introduced for 16QAM in NB-IoT uplink

| I_TBS | I_RU |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 |  |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 |  |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | 2536 |  |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 |  |  |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 |  |  |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2536 |  |  |
| 20 | 440 | 904 | 1384 | 1864 | 2344 |  |  |  |
| 21 | 488 | 1000 | 1480 | 1992 | 2536 |  |  |  |

It has also been agreed in RAN1 that repetition is not used for 16QAM in NB-IoT uplink, where it can be used for 3, 6 and 12 subcarriers NPUSCH format 1. Note that 16QAM cannot be used for 1 subcarrier NPUSCH format 1. Considering the agreements in RAN1, and to ensure satisfactory BS receiver performance with 3, 6 and 12 subcarriers for 16QAM in NB-IoT uplink, it is proposed to define at least one of the new uplink FRCs for 16QAM as shown in table 3 below.

Table 3: FRC parameters for 16QAM in NB-IoT uplink

| Reference channel | Ax-y | Ax-y | Ax-y |
| --- | --- | --- | --- |
| Sub carrier spacing (kHz) | 15 | 15 | 15 |
| Number of tones | 3 | 6 | 12 |
| Modulation | 16-QAM | 16-QAM | 16-QAM |
| Diversity | No | No | No |
| Frequency offset | 0 | 0 | 0 |
| I_TBS / I_RU | 19 / 3 | 19 / 3 | 19 / 3 |
| Payload size (bits) | 1736 | 1736 | 1736 |
| Allocated resource units | 4 | 4 | 4 |
| Transport block CRC (bits) | 24 | 24 | 24 |
| Coding rate (target) | 3/4 | 3/4 | 3/4 |
| Coding Rate | 0.76 | 0.76 | 0.76 |
| Code block CRC size (bits) | 0 | 0 | 0 |
| Number of code blocks – C | 1 | 1 | 1 |
| Total symbols per resource unit | 144 | 144 | 144 |
| Total number of bits per resource unit | 576 | 576 | 576 |
| Tx time (ms) | 16 | 8 | 4 |
| Frequency offset | 0 | 0 | 0 |
| Channel estimation length (ms) Note 1 | 12 | 8 | 4 |
| Note 1: Channel estimation lengths are included in the table for information only. |  |  |  |

Proposal 3: To define at least one of the new uplink FRC for 16QAM as shown in table 3 above.

Preliminary link-level simulation has been performed using the FRC parameters in table 3 above, and the simulation results for the SNIR required at 70% and 95% of maximum throughput are provided in Figure 1 below. It can be seen from the figure that the required SNIR are quite closed for the different number of tones.

(a) 3 tones

(b) 6 tones

(c) 12 tones

Figure 1: SNIR required at 70% and 95% of maximum throughput for 16QAM in NB-IoT uplink

# 3.	Conclusion

This contribution has provided further proposals on BS RF requirements for the support of 16-QAM in NB-IoT unicast in UL and DL according to the approved WF and the agreements in RAN1. They are summarized as follows.

Proposal 1: The current NB-IoT RB power dynamic range for QPSK RB transmission in NB-IoT should apply to 16QAM RB for in-band or guard band operation or NB-IoT operation in NR in-band. There is no need for changes to the specifications nor new test cases to define NB-IoT RB power dynamic range for 16QAM.

Proposal 2: An EVM limit of 12.5% should be specified for NB-IoT 16QAM transmission. Proponent of a different EVM requirement should provide technical justifications (e.g., preliminary simulation results) in next RAN4 meeting.

Proposal 3: To define at least one of the new uplink FRC for 16QAM as shown in table 3 above.

# References

[1]	RP-201306, “WID revision: Additional enhancements for NB-IoT and LTE-MTC”, Huawei, HiSilicon.

[2]	R4-2105432, “WF on BS RF requirements for R17 NB-IoT 16QAM”, Huawei, HiSilicon, Ericsson.

[3]	RP-210534, “Status report for WI: Additional enhancements for NB-IoT and LTE-MTC”, Huawei.

[4]	3GPP TS 36.104: “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”.

[5]	3GPP TS 38.104: “NR; Base Station (BS) radio transmission and reception”.

[6]	3GPP TS 36.141: “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing”.