3GPP RAN WG4 Meeting #100e		R4-2112345

Electronic meeting, 16 – 27th August 2021

Agenda item:	8.2.3

Source:	Apple Inc.

Title:	A-MPR for the 6GHz unlicensed band in Region 2 and Region 3 countries

WI/SI:	NR_6GHz_unlic_full

Release:	Rel-17

Document for:	Decision

# 1	Introduction

3GPP Rel-16 NR-U WI [1] specified how the NR technology can be used on the unlicensed spectrum thus offering more resources in frequency bands, such as 5GHz and 6GHz.  While 5GHz is a well-known band for the unlicensed operation, 6GHz is a relative new band usage of which was approved recently in different regulatory regions. While the 6GHz band for the US is already part of the Rel-16 core functionality, this band is not supported in other countries, such as South Korea and Canada, which have finalised their regulatory requirements only recently (refer to the latest regulatory status updated in  [2]). Thus, RAN#92 meeting approved a new WI aim of which is to enable support of the 6GHz unlicensed band [3].

Referring to the summary of the regulatory parameters for the Region 2 and Region 3 in [4], this contribution presents A-MPR simulation results for Canada and South Korea for the so-called low power indoor (LPI) mode with NR-U PC5.

# 2	A-MPR values

As explained and summarised in [4], several Region 2 countries, such as Peru, Chile and Brazil have the same regulatory parameters at least for the LPI mode, which in turn is identical to the US FCC LPI mode. From that perspective we do not need new A-MPR simulations for those countries. On the contrary to it, Canada and South Korea have slightly different regulatory requirements, A-MPR simulations results for which are presented below.

All simulations are performed for the LPI mode with the following common operational parameters. In addition to that, country and regional specific parameters are also applied, such as maximum PSD restriction, OOBE restrictions, etc.

-	Power Class: PC5

-	Calibration: QPSK DFT-s-OFDM 100RB3 with 1dB backoff

-	ACLR: 27dB

-	SEM: NR-U

-	EVM: Not used for A-MPR simulations

-	Image rejection: -28dBc

-	Carrier leakage rejection: -28dBc

In all cases the PA was simulated with the listed impairments and the results evaluated with the given spectral requirements. The 0dB reference was obtained by finding the maximum in-band power with 1MHz aggregation bandwidth. The mask was applied accordingly. Before identifying the reference level, the carrier leakage was removed to avoid an offset in the NR-U SEM mask. Simulations are conducted for the full carrier and interlace operation with the signal setup summarized in the Table 2-1 below.

Table 2-1: Signal setup.

|  | ID | Waveform | BW | RB Setup | SCS |
| --- | --- | --- | --- | --- | --- |
| Full Allocation Single CC | 1 | DFT-s-OFDM | 20 | 100RB3 | 15 |
|  | 2 | DFT-s-OFDM | 20 | 100RB0 | 15 |
|  | 3 | DFT-s-OFDM | 20 | 50RB0 | 30 |
|  | 4 | DFT-s-OFDM | 40 | 216RB0 | 15 |
|  | 5 | DFT-s-OFDM | 60 | 162RB0 | 30 |
|  | 6 | DFT-s-OFDM | 80 | 216RB0 | 30 |
|  | 7 | CP-OFDM | 20 | 106RB0 | 15 |
|  | 8 | CP-OFDM | 20 | 51RB0 | 30 |
|  | 9 | CP-OFDM | 40 | 216RB0 | 15 |
|  | 10 | CP-OFDM | 60 | 162RB0 | 30 |
|  | 11 | CP-OFDM | 80 | 217RB0 | 30 |
| Interlaced Allocation Single CC | 12 | DFT-s-OFDM | 20 | 1RB0 every 10RBs (10x) | 15 |
|  | 13 | DFT-s-OFDM | 40 | 1RB0 every 10RBs (20x) | 15 |
|  | 14 | DFT-s-OFDM | 60 | 1RB0 every 10RBs (30x) | 30 |
|  | 15 | DFT-s-OFDM | 80 | 1RB0 every 5RBs (40x) | 30 |
|  | 16 | CP-OFDM | 20 | 1 RB0 every 10RBs (10x) | 15 |
|  | 17 | CP-OFDM | 40 | 1RB0 every 10RBs (22x) | 15 |
|  | 18 | CP-OFDM | 60 | 1RB0 every 10RBs (33x) | 30 |
|  | 19 | CP-OFDM | 80 | 1RB0 every 5RBs (44x) | 30 |

## 2.1	Canada

A-MPR simulation results for the LPI mode in accordance with the Canadian regulatory requirements are presented in Figure 2.1-1 below, where X-axis denotes a particular simulation case index (refer to Table 2-1) and Y-axis indicates the required power back-off.

Figure 2.1-1: A-MPR simulation results (for Canada LPI).

Based on the presented simulation results, we would need additional A-MPR values only for the 20MHz channel. Larger channels, such as 40 and 80MHz, do not require any additional power backoff.

Table 2.1-1:  Proposed A-MPR (for LPI in Canada)

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |
| --- | --- | --- | --- |
|  |  | 20 MHz |  |
|  |  | Full2 (dB) | Partial3 (dB) |
| DFT-s-ODFM | PI/2 BPSK | ≤ 3.0 | ≤ 5.5 |
|  | QPSK | ≤ 3.0 | ≤ 5.5 |
|  | 16 QAM | ≤ 3.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 3.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 5.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 |

Proposal 1:	Define A-MPR for NR-U PC5 LPI in Canada as provided in Table 2.1-1.

## 2.2	South Korea

A-MPR simulation results for the LPI mode in accordance with the South Korean regulatory requirements are presented in Figure 2.2-1 below, where X-axis denotes a particular simulation case index (refer to Table 2-1) and Y-axis indicates the required power back-off.

Figure 2.2-1: A-MPR simulation results (for South Korea LPI).

Based on the presented simulation results, Table 2.2-1 summarizes required A-MPR values for LPI operation with South Korean regulatory requirements. It is worth noting that the 20MHz channel is limited by the PSD requirement, whereas larger channels are limited by the -27dBm/MHz requirement. South Korean spurious emission requirement never becomes a limiting factor.

Table 2.2-1:  Proposed A-MPR (for LPI in South Korea)

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 | ≤ 5.5 | ≤ 3.5 | ≤ 4.5 | ≤ 3.0 | ≤ 4.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 5.0 | ≤ 3.5 | ≤ 5.0 |
|  | 64 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 5.0 | ≤ 3.5 | ≤ 5.0 |
|  | 256 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.0 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 64 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |

Proposal 2:	Define A-MPR for NR-U PC5 LPI in South Korea as provided in Table 2.2-1.

# 3	Conclusions

In this contribution we simulated and proposed a set of A-MPR values for the 6GHz unlicensed band required for the NR-U PC5 LPI operation in Canada and South Korea. As a summary of the paper, we suggest defining A-MPR values as provided in Table 2.1-1 and Table 2.2-1.

Proposal 1:	Define A-MPR for NR-U PC5 LPI in Canada as provided in Table 2.1-1.

Proposal 2:	Define A-MPR for NR-U PC5 LPI in South Korea as provided in Table 2.2-1.

# 4	References

- RP-1912926, "WID on NR-based Access to Unlicensed Spectrum ", Qualcomm Inc.

- 3GPP TR 37.890, "Feasibility Study on 6 GHz for LTE and NR in Licensed and Unlicensed Operations", v0.10.0

- RP-211602, "New WID for Introduction of operation in full unlicensed band 5925-7125MHz", Apple Inc.

- R4-2112344, "Overview of the 6GHz unlicensed band system and regulatory requirements in Region 2 and Region 3 countries", Apple Inc.