3GPP TSG-RAN WG4 Meeting # 100-e	R4-2112687

# Electronic Meeting, August 16-27, 2021

Agenda item:	6.1.5.1.1

Source:	Apple Inc.

Title:	Discussion on applicability of MRTD requirements to multi-TRxP

Release:	Rel-16

Document for:	Discussion

# 1. Introduction

In RAN4#99-e clarifying applicability of MRTD requirements to multi-TRxP was discussed and way forward [1] was agreed. In this contribution we present our views on clarifying the applicability of requirements in RAN4 specification.

# 2. Discussion

In RAN4#96e the following was agreed for Rel-16 eMIMO multi-TRxP transmission [2]:

- No RRM core requirement impact identified on MRTD/MTTD values specified in Rel-15;

- It is RAN4 common understanding that MRTD/MTTD requirements in clauses 7.5.3, 7.6.3 and 7.6.4 is sufficient for support the deployment with multi-DCI based and single-DCI based multi-TRxP transmission.

But the above agreement was not captured in RAN4 specification.

For multi-TRP transmission schemes, it is common understanding that signals from multi-TRP are received with CP.

From TS 38.306:

multiDCI-MultiTRP-r16 Indicates whether the UE supports multi-DCI based multi-TRP and support of fully/partially overlapping PDSCHs in time and non-overlapping in frequency. This capability applies only to BWPs where two values of coresetPoolIndex are configured. The capability signalling contains the following: - maxNumberCORESET-r16 indicates maximum number of CORESETs configured per BWP per cell in addition to CORESET 0. - maxNumberCORESETPerPoolIndex-r16 indicates maximum number of CORESETs configured per coresetPoolIndex per BWP per cell in addition to CORESET 0. - maxNumberUnicastPDSCH-PerPool-r16 indicates maximum number of unicast PDSCHs per coresetPoolIndex per slot. NOTE 1: A UE may assume that its maximum receive timing difference between the DL transmissions from two TRPs is within a Cyclic Prefix. NOTE 2: Processing capability 2 is not supported in any CC if at least one CC is configured with two values of coresetPoolIndex. NOTE 3: If UE reports value N1 for maxNumberCORESET-r16, that means UE supports up to min (N1+1, 5) CORESETs in total (including CORESET#0) if there is CORESET#0, and supports maximal N1 CORESETs if there is no CORESET#0. NOTE 4: If UE reports value N2 for maxNumberCORESETPerPoolIndex-r16, that means UE supports up to min (N2+1, 3) CORESETs in total (including CORESET#0) for a TRP if there is CORESET#0, and supports maximal N2 CORESETs for another TRP if there is no CORESET#0.

The clarification in TS 38.306 in Note 1 for UE capability to support multi-DCI based multi-TRP is for receiving signals from 2 TRPs and is not related to the MTRD requirements in TS 38.133.

The MRTD and MTTD requirements for intra-band EN-DC and CA are specified for co-located deployment assumption. But in multi-TrxP transmission the TRxPs may not be co-located.

Observation #1: The MRTD and MTTD requirements for intra-band EN-DC and CA are specified for co-located deployment assumption. But in multi-TRxP transmission the TRxPs may not be co-located.

In previous RAN4 meetings this was discussed and concluded in RAN4#96e that existing requirements are sufficient to support multi-TRxP transmissions schemes [2]. But this conclusion/ agreement was not captured in spec.

Observation #2: The applicability of MTTD/MRTD requirements that assume co-located deployment to multi-TRxP deployment is not captured in RAN4 specification.

Hence, we propose to explicitly capture that the requirements are applicable to multi-TRxP as well.

Proposal #1: Introduce applicability of MTTD/MRTD requirements to multi-TRxP.

Our proposals are captured in our CR [3].

# 3. Conclusion

In this paper, we present our views on clarifying the applicability of requirements in RAN4 specification. Our observations and proposals are captured below:

Observation #1: The MRTD and MTTD requirements for intra-band EN-DC and CA are specified for co-located deployment assumption. But in multi-TRxP transmission the TRxPs may not be co-located.

Observation #2: The applicability of MTTD/MRTD requirements that assume co-located deployment to multi-TRxP deployment is not captured in RAN4 specification.

Proposal #1: Introduce applicability of MTTD/MRTD requirements to multi-TRxP.

# Reference

- R4-2108225, “WF on NR eMIMO RRM requirement Maintenance”, Samsung.

- R4-2012146, “WF on Completing Rel-16 eMIMO RRM Core Requirement”, Samsung.

- R4-2112098, “Draft CR to 38.133 on applicability of requirements to multi-TRxP - R16”, Apple