3GPP TSG-RAN WG4 Meeting#100-e	R4-2112809

# Electronic meeting, 16 – 27 August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | CRNum | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of asymmetric BW for CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The support of asymmetric channel bandwidths in CA configurations of operating bands supporting asymmetric bandwidths is undefined. An asymmetric UL and DL channel bandwidth combination configured for a CA band combination must be contained in both an asymmetric bandwidth combination set of a band and in a supported bandwidth combination set of the CA configuration for both the UL and DL channel bandwidths. Example: for a UE supporting BCS#0 for CA_n2A-n66A and the asymnetric BCS#1 of Band n66, requirements are defined for the asummetric (UL,DL) combination (20,40) MHz but not for (25,40) MHz since the UL channel bandwidth of 25 MHz is not contained in BCS#0 of the CA configuration. A BCS for a CA configuration applies for both the UL and DL. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Sub-clause 5.5A.0: the applicability of requirements for channel bandwidths of CA configurations with one or more operating band supporting asymmetric channel bandwidths is defined. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The support of asymmetric channel bandwidths in CA configurations of operating bands (FDD) supporting asymmetric bandwidths is undefined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5A.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

## 5.5A	Configurations for CA

### 5.5A.0	General

The configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41 but with otherwise identical parameters. For brevity the said configuration for CA operating bands with Band n90 are not listed in the tables below but are covered by this specification.

Non-contiguous resource allocation and almost contiguous allocation are not applicable for each NR carrier of intra-band contiguous and non-contiguous CA configurations.

For a CA configuration with one or more operating band supporting asymmetric channel bandwidths as specified in sub-clause 5.3.6, requirements are defined for an asymmetric UL and DL channel bandwidth combination of a supported asymmetric channel bandwidth combination set for an operating band of the CA configuration when the said UL and DL channel bandwidths are also contained in a supported bandwidth combination set of the CA configuration.

### 5.5A.1	Configurations for intra-band contiguous CA

< end of changes >