3GPP TSG-RAN WG4 Meeting # 100-e	R4-2113065

# Electronic Meeting, August 16-27, 2021

Source: 	Huawei, HiSilicon

Title: 	On introduction of operation in full unlicensed band 5925-7125MHz

Agenda Item:	8.2.1

Document for:	Approval

Introduction

At RAN#92 Meeting, a new WI on introduction of operation in full unlicensed band 5925-7125MHz was approved in [1]. The objectives for the WI are,

-	Analyse regulatory requirements for countries/regions that have adopted license-exempt operation in the full 5925-7125MHz frequency range:

NOTE 1: The scope of the WI is Region 2 (Canada, Brazil, Peru, Chile) and Region 3 (South Korea) countries, regulatory parameters for which can be found from TR 37.890;

NOTE 2: Should more countries finalise the regulatory framework and/or a particular country changes the existing regulation for the full 5925-7125MHz frequency range, it can be considered within the timeline of this WI subject for further RAN discussions.

-	Depending on the regulatory requirements, determine whether they can be handled by existing NS values or whether new NS values are needed.

-	Depending on whether existing NS values cannot be re-used and/or whether new NS values are not possible, determine whether an existing band n96 can be re-used or introduction of a new band will be needed.

-	Define or update (if needed) system parameters such as channel bandwidths and channel arrangements.

-	Define or update (if needed) transmitter and receiver characteristics requirements for the UE.

-	Define or update (if needed) transmitter and receiver characteristics requirements for the BS.

In this contribution, we provide some consideration on the UE power class aspect.

Discussion

In Rel-16 for NR unlicensed operation, a new band n96 was defined according to the FCC Report and Order FCC 20-51. Standard-power operation with the use of an Automated Frequency Coordination (AFC) and low-power indoor operations are allowed per FCC 20-51.

Rel-16 defines power class 5 with max output power 20 dBm. As shown below, the FCC regulatory requirements (transmit power density and spectrum emission limits) are covered by NS_53 for low-power operations and NS_54 for standard power operation.

Table 6.2F.1-1 in 38.101: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n46 |  |  |  |  |  |  | 20 | +2/-3 |
| n96 |  |  |  |  |  |  | 20 | +2/-3 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 5 is default power class unless otherwise stated. |  |  |  |  |  |  |  |  |

Table 6.2F.1-2 in 38.101-1: Additional requirements for transmit power density

| NR Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum mean power density (dBm/MHz) |
| --- | --- | --- | --- | --- |
| n46 | NS_28 | 20, 40, 60, 80 | 5150 – 5350 | 10 |
|  |  |  | 5470 – 5725 |  |
|  | NS_29 | 20 | 5170 – 5330 | 10 |
|  |  |  | 5490 – 5730 |  |
|  |  | 40 | 5170 – 5330 | 7 |
|  |  |  | 5490 – 5730 |  |
|  |  | 60, 80 | 5170 – 5330 | 4 |
|  |  |  | 5490 – 5730 |  |
|  | NS_30 | 20, 40, 60, 80 | 5150 – 5350 | 11 |
|  |  |  | 5470 – 5725 |  |
|  | NS_31 | 20 | 5150 - 5230 | 10 |
|  |  |  | 5250 – 5350 |  |
|  |  |  | 5470 – 5725 |  |
|  |  |  | 5725 - 5850 |  |
|  |  |  | 5230 – 5250 | 4 |
|  |  | 40 | 5150 - 5230 | 7 |
|  |  |  | 5250 – 5350 |  |
|  |  |  | 5470 – 5725 |  |
|  |  |  | 5725 - 5850 |  |
|  |  |  | 5230 – 5250 | 4 |
|  |  | 60, 80 | 5150 - 5230 | 4 |
|  |  |  | 5250 – 5350 |  |
|  |  |  | 5470 – 5725 |  |
|  |  |  | 5725 - 5850 |  |
|  |  |  | 5230 – 5250 |  |
| n96 | NS_53 | 20, 40, 60, 80 | 5925 – 7125 | -1 |
|  | NS_54 | 20, 40, 60, 80 | 5925 – 6425 | 17 |
|  |  |  | 6525 – 6875 |  |

Table 6.5F.3.3.5-1 in 38.101-1: Additional requirements for NS_53 and NS_54

| Frequency band (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| f ≤ 5925 | -27 | 1 MHz |
| f ≥ 7125 | -27 |  |

In the scope of the WI, it covers Canada, Brazil, Peru, Chile in Region 2 and South Korea in Region 3. The Very Low Power operation is authorized in Canada, Brazil and South Korea with different regulatory requirement.

| Region | Country | Permissible operation (Note 1) | Frequency range | Maximum mean EIRP for in-band emissions | Maximum mean EIRP density for in-band emissions | Maximum mean EIRP density for out-of-band emissions |
| --- | --- | --- | --- | --- | --- | --- |
| Region 2 | US | SP (see 4.2.1) | 5925 – 6425MHz 6525 – 6875MHz | 36dBm (AP) 30dBm (CL) | 23dBm/MHz (AP) 17dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  |  | LPI (see 4.2.1) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5 dBm/MHz (AP) -1 dBm/MHz (CL) |  |
|  | Canada | SP (see 4.2.2) | 5925-6875 MHz | 36dBm | 23dBm/MHz |  |
|  |  | LPI (see 4.2.2) | 5925-7125 MHz | 30dBm | 5 dBm/MHz |  |
|  |  | VLP (see 4.2.2) |  | 14dBm | -8dBm/MHz |  |
|  | Brazil | LPI (see 4.2.3) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  |  | VLP (see 4.2.3) |  | 17 dBm | -5 dBm/MHz |  |
|  | Peru | LPI (see 4.2.4) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) |  |
|  | Chile | LPI (see 4.2.5) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) |  |
| Region 3 | South Korea | LPI (see 4.3.2) | 5925 – 7125MHz | 24dBm | 2dBm/MHz | -27 dBm/MHz (outside operational range) |
|  |  | VLP (see 4.3.2) | 5925 – 6425MHz | 14dBm | 1dBm/MHz | -34 dBm/MHz (outside operational range) |
| Note 1: For the exact conditions of operation, refer to the sub-clause which is mentioned in parenthesis (e.g. for the exact conditions for operating SP in US, refer to sub-clause 4.2.1.) |  |  |  |  |  |  |

Proposal: The introduction of VLP should be addressed in the WI in term of power class and regulatory requirements for Canada, Brazil and South Korea.

Conclusions

In the contribution, we provide.

Proposal: The introduction of VLP should be addressed in the WI in term of power class and regulatory requirements for Canada, Brazil and South Korea.

References

RP-211602, New WID for Introduction of operation in full unlicensed band 5925-7125MHz, Apple Inc.