3GPP TSG-RAN WG4 Meeting # 100-e	                                 R4-2113155

Electronic Meeting, August 16-27, 2021

Agenda Item:	6.1.6.2.2.2.3

Source:	Intel Corporation

Title:	Discussion on UE RX-TX time difference measurement accuracy requirements

Document for:		Discussion

# Introduction

In the previous RAN4 meeting, the core requirements for UE RX-TX time difference requirements was completed, and some open issues for the performance requirements were summarized in [1]. For some of requirements can be same as these of PRS RSTD (e.g. number of PRS occasions or PRS repetitions for defining accuracy) [2]. In this contribution we will focus on the discussion of remaining issue below which are unique for UE RX-TX time difference measurement.

- Applicability of accuracy requirements in case of NTA_offset change

- Measurement accuracy requirements

# Discussion

# Applicability of accuracy requirements in case of NTA_offset change

In previous RAN4 meeting, the impacts of Nta_offset on the UE Rx-Tx time difference requirements was addressed.

- Option 1: RAN4 not to capture applicability of UE Rx-Tx time difference accuracy requirements under NTA_offset change during the measurement period - Option 2: Clarify in section 10.1.25.2 in TS 38.133: “UE Rx-Tx time difference accuracy requirements shall not apply if NTA_offset defined in Table 7.1.2-2 in 38.133 changes during the UE Rx-Tx measurement period.”

Thus, from the specification perspective, some necessary clarification or notes is necessary which can also reflect RAN4’s agreement precisely.

Proposal 1 : UE Rx-Tx measurement requirements in TS38.133 shall be applicable unless the NTA_offset changes during the measurement period.

# Measurement accuracy requirements

In the last meeting, the UE Rx-Tx time difference accuracy requirements were agreed [CR]. But there is one of parameter combination TBD. Based on the updated simulation results with different PRS BW given in [5,Summary of simulation results], the requirement for this TBD case in [] can be:

Proposal 2a: The requirements for AWGN in FR1 based on the different PRS measurement bandwidth can be:

Table 1a. UE Rx-Tx time difference accuracy requirements in FR1

| Accuracy, Tc | Es/Iot, dB | PRS BW, PRB | PRS SCS, kHz | Repetition factor (TrepPRS*LPRS/KcombPRS) |
| --- | --- | --- | --- | --- |
| [78+margin] | -3 | ≥[24] | 15 | ≥4 |
| [59 +margin] |  | ≥[52] |  | All |
| [30 +margin] |  | >[104] |  | All |
| [12+margin] |  | ≥[24] | 30 | ≥4 |
| [30 +margin] |  | ≥[48] |  | All |
| [15 +margin] |  | ≥[132] |  | All |
| [29 +margin] |  | ≥[24] | 60 | ≥4 |
| [15 +margin] |  | ≥[64] |  | All |
| [7+margin] |  | ≥[132] |  | All |
| [101 +margin] | -13 | ≥[24] | 15 | ≥4 |
| [75 +margin] |  | ≥[52] |  | All |
| [39+margin] |  | >[104] |  | All |
| [43 +margin] |  | ≥[24] | 30 | ≥4 |
| [37+margin] |  | ≥[48] |  | All |
| [16 +margin] |  | ≥[132] |  | All |
| [36 +margin] |  | ≥[24] | 60 | ≥4 |
| [16+margin] |  | ≥[64] |  | All |
| [8 +margin] |  | ≥[132] |  | All |
| Note 1: Margin value is FFS |  |  |  |  |

Proposal 2b: The requirements for fading channel in FR1 based on the different PRS measurement bandwidth can be:

Table 1b. UE Rx-Tx time difference accuracy requirements in FR2

| Accuracy, Tc | Es/Iot, dB | PRS BW, PRB | PRS SCS, kHz | Repetition factor (TrepPRS*LPRS/KcombPRS) |
| --- | --- | --- | --- | --- |
| [137+margin] | -3 | ≥[24] | 15 | ≥4 |
| [96+margin] |  | ≥[52] |  | All |
| [62 +margin] |  | >[104] |  | All |
| [56+margin] |  | ≥[24] | 30 | ≥4 |
| [68 +margin] |  | ≥[48] |  | All |
| [44 +margin] |  | ≥[132] |  | All |
| [59 +margin] |  | ≥[24] | 60 | ≥4 |
| [42 +margin] |  | ≥[64] |  | All |
| [36 +margin] |  | ≥[132] |  | All |
| [180+margin] | -13 | ≥[24] | 15 | ≥4 |
| [98+margin] |  | ≥[52] |  | All |
| [68 +margin] |  | >[104] |  | All |
| [71+margin] |  | ≥[24] | 30 | ≥4 |
| [85 +margin] |  | ≥[48] |  | All |
| [44 +margin] |  | ≥[132] |  | All |
| [139 +margin] |  | ≥[24] | 60 | ≥4 |
| [46 +margin] |  | ≥[64] |  | All |
| [30 +margin] |  | ≥[132] |  | All |
| Note 1: Margin value is FFS |  |  |  |  |

# Conclusion

In this contribution, some considerations on UE RX-TX time difference measurement accuracy requirements are provided and the following observations and proposals can be drawn:

Proposal 1 : UE Rx-Tx measurement requirements in TS38.133 shall be applicable unless the NTA_offset changes during the measurement period

Proposal 2a: The requirements for AWGN in FR1 based on the different PRS measurement bandwidth can be:

Table 1a. UE Rx-Tx time difference accuracy requirements in FR1

| Accuracy, Tc | Es/Iot, dB | PRS BW, PRB | PRS SCS, kHz | Repetition factor (TrepPRS*LPRS/KcombPRS) |
| --- | --- | --- | --- | --- |
| [78+margin] | -3 | ≥[24] | 15 | ≥4 |
| [59 +margin] |  | ≥[52] |  | All |
| [30 +margin] |  | >[104] |  | All |
| [12+margin] |  | ≥[24] | 30 | ≥4 |
| [30 +margin] |  | ≥[48] |  | All |
| [15 +margin] |  | ≥[132] |  | All |
| [29 +margin] |  | ≥[24] | 60 | ≥4 |
| [15 +margin] |  | ≥[64] |  | All |
| [7+margin] |  | ≥[132] |  | All |
| [101 +margin] | -13 | ≥[24] | 15 | ≥4 |
| [75 +margin] |  | ≥[52] |  | All |
| [39+margin] |  | >[104] |  | All |
| [43 +margin] |  | ≥[24] | 30 | ≥4 |
| [37+margin] |  | ≥[48] |  | All |
| [16 +margin] |  | ≥[132] |  | All |
| [36 +margin] |  | ≥[24] | 60 | ≥4 |
| [16+margin] |  | ≥[64] |  | All |
| [8 +margin] |  | ≥[132] |  | All |
| Note 1: Margin value is FFS |  |  |  |  |

Proposal 2b: The requirements for fading channel in FR1 based on the different PRS measurement bandwidth can be:

Table 1b. UE Rx-Tx time difference accuracy requirements in FR1

| Accuracy, Tc | Es/Iot, dB | PRS BW, PRB | PRS SCS, kHz | Repetition factor (TrepPRS*LPRS/KcombPRS) |
| --- | --- | --- | --- | --- |
| [137+margin] | -3 | ≥[24] | 15 | ≥4 |
| [96+margin] |  | ≥[52] |  | All |
| [62 +margin] |  | >[104] |  | All |
| [56+margin] |  | ≥[24] | 30 | ≥4 |
| [68 +margin] |  | ≥[48] |  | All |
| [44 +margin] |  | ≥[132] |  | All |
| [59 +margin] |  | ≥[24] | 60 | ≥4 |
| [42 +margin] |  | ≥[64] |  | All |
| [36 +margin] |  | ≥[132] |  | All |
| [180+margin] | -13 | ≥[24] | 15 | ≥4 |
| [98+margin] |  | ≥[52] |  | All |
| [68 +margin] |  | >[104] |  | All |
| [71+margin] |  | ≥[24] | 30 | ≥4 |
| [85 +margin] |  | ≥[48] |  | All |
| [44 +margin] |  | ≥[132] |  | All |
| [139 +margin] |  | ≥[24] | 60 | ≥4 |
| [46 +margin] |  | ≥[64] |  | All |
| [30 +margin] |  | ≥[132] |  | All |
| Note 1: Margin value is FFS |  |  |  |  |

# References

- R4-2108040, “WF on requirements on NR Pos performance requirements”

- 3GPP TS 38.133 v16.8.0: "NR: Requirements for support of radio resource management "