3GPP TSG-RAN WG4 Meeting #100-e	R4-2113195

Electronic Meeting, August 16-27, 2021

Source:	ZTE Corporation

Title:          Discussion on NR FR2 HST channel model

Agenda Item:	9.9.2.3

Document for:	Approval

- Introduction

The channel model for uni-directional deployment and bi-directional deployment were discussed in RAN 4# 99-e meeting. The start point for t=0 is FFS still for uni-directional deployment and the channel model for bi-directional deployment needs further down-selection.

In this contribution, we give our views about channel model for HST_FR2.

- Discussion

In RAN4# 99-e meeting, the channel model for uni-directional deployment was captured as below[1]:

UE moving towards serving beam is considered for HST_FR2 channel model, so the switching point for channel model should be Ds_offset to the left of the RRH when CPE moves from left to right.

To align simulation results from different companies, the starting point of t = 0 can be Ds_offset to the left of the RRH which is denoted as switching point.

Proposal 1 :The starting point of t = 0 can be at the switching point which is Ds_offset to the left of RRH when CPE moves from left to right to align simulation results from different companies.

There were 4 options reached for channel model of bi-directional deployment:

These options above are strongly related to the transmission schemes. For option 2(e), whether the coverage hole affect the performance or not should be first discussed. For scenario-A one beam per RRH can provide sufficient coverage, so the Rx power of option 2(e) for scenario-B is given in figure 1. Table A-1 in appendix summarizes our evaluation assumptions for option 2(e) for scenario-B bi-directional deployment. Figure 2 depicts the beam bearing for option 2(e).

Figure 1, Rx power from different beams for scenario-B bi-directional deployment option 2(e)

Figure 2, Depiction of option 2(e) of channel model for scenario-B bi-directional deployment

It can be seen from figure 1 that the Rx power is low when CPE is near the RRH, so option 2(e) is not preferred.

Comparing between option 2(a) and option 2(b) in our accompany contribution in [2], we can find that option 2(b) provide similar coverage as option 2(a) in scenario-A at the cost of more propagation delay hopping.

Option 2(a) channel model are designed for scenario-A. If bi-directional deployment is not precluded for scenario-A option 2(a) should be supported.

The Rx power comparison between option 2(b) and 2(c) for scenario-B is shown in figure 3. Table A-2 in appendix summarizes the evaluation assumptions for these two options of scenario-B bi-directional deployment. Figure 4 depicts option 2(b) and option 2(c).

Figure 3, Rx power comparison between option 2(b) and 2(c) for scenario-B bi-directional deployment

Figure 4, Depiction of option 2(b) and option 2(c) of channel model for scenario-B bi-directional deployment

It can be seen from figure 3 that the Rx power fluctuation of option 2(b) is greater than that of option2(c).

Based on the analysis, we prefer option 2(a) for channel model of scenario-A bi-directional deployment and prefer option 2(c) for channel model of scenario-B bi-directional deployment.

Proposal 2: If bi-directional is supported, we prefer option 2(a) for channel model of scenario-A bi-directional deployment and option 2(c) for channel model of scenario-B bi-directional deployment.

- Conclusion

In this contribution, we provide the observation and proposals for HST_FR2 channel model:

Proposal 1 :The starting point of t = 0 can be at the switching point which is Ds_offset to the left of RRH when CPE moves from left to right to align simulation results from different companies.

Proposal 2: If bi-directional is supported, we prefer option 2(a) for channel model of scenario-A bi-directional deployment and option 2(c) for channel model of scenario-B bi-directional deployment.

- Reference

R4-2108661 WF on Channel Modeling for FR2 HST

R4-2113193 Discussion on NR FR2 HST scenario-A

- Appendix

Table A-1: Evaluation assumptions for scenario-B bi-directional deployment

| Parameters | Option 2(e) scenario-B bi-directional deployment |
| --- | --- |
| RRH antenna configuration | [4 8 2 1 1] dg,H=0.5λ, dg,V=0.5λ |
| CPE antenna configuration | [4 2 2 1 1] dg,H=0.5λ, dg,V=0.5λ |
| RRH antenna panel placement | -32 degree azimuth for right panel -148 degree azimuth for left panel 0 degree down tilt for right panel 0 degree down tilt for left panel |
| CPE antenna panel placement | 32 degree azimuth for right panel 148 degree azimuth for left panel 0 degree down tilt for right panel 0 degree down tilt for left panel |
| RRH Tx power | 30 dBm |
| RRH antenna height | 15 m |
| CPE antenna height | 5 m |
| Antenna loss | 3 dB |
| Paht loss model | RMaLOS PL1 TS38901 |
| Tx Beam* | Beam 1 Azimuth: -40.6013degree Elevation: 92.4843degree |
|  | Beam 2 Azimuth: -23.1968degree Elevation: 91.5043degree |
|  | Beam 3 Azimuth: -156.8014 degree Elevation: 91.5043degree |
|  | Beam 4 Azimuth: -139.3987 degree Elevation: 92.4843 degree |
| Rx Beam* | Left panel Azimuth: 148.0981 degree Elevation: 88.0063 degree |
|  | Right panel Azimuth: 31.9023 degree Elevation: 88.0063 degree |

- * The track is parallel to x-axis. The CPE moves in the positive direction of the x-axis. The y-axis coordinate of the RRH is Dmin. The 0 degree of elevation is parallel to z-axis.

Table A-2: Evaluation assumptions for scenario-B bi-directional deployment

| Parameters | Channel model options for scenario-B bi-directional deployment |  |  |
| --- | --- | --- | --- |
|  | 2(b) |  | 2(c) |
| RRH antenna configuration | [4 8 2 1 1] dg,H=0.5λ, dg,V=0.5λ |  |  |
| CPE antenna configuration | [4 2 2 1 1] dg,H=0.5λ, dg,V=0.5λ |  |  |
| RRH antenna panel placement | -26 degree azimuth for right panel -154degree azimuth for left panel 0 degree down tilt for right panel 0 degree down tilt for left panel | -18 degree azimuth for right panel -162 degree azimuth for left panel 0 degree down tilt for right panel 0 degree down tilt for left panel |  |
| CPE antenna panel placement | 26 degree azimuth for right panel 154 degree azimuth for left panel 0 degree down tilt for right panel 0 degree down tilt for left panel | 18 degree azimuth for right panel 162 degree azimuth for left panel 0 degree down tilt for right panel 0 degree down tilt for left panel |  |
| RRH Tx power | 30 dBm |  |  |
| RRH antenna height | 15 m |  |  |
| CPE antenna height | 5 m |  |  |
| RRH antenna connector loss | 3 dB |  |  |
| Paht loss model | RMaLOS PL1 TS38901 |  |  |
| Tx Beam* | Beam 1 Azimuth: -12.0948 degree Elevation: 90.8003 degree |  | Beam 1 Azimuth: -23.1986 degree Elevation: 91.5043 degree |
|  | Beam 2 Azimuth: -40.6013 degree Elevation: 92.4843 degree |  | Beam 2 Azimuth: -12.0948 degree Elevation: 90.8003 degree |
|  | Beam 3 Azimuth: -139.3987 degree Down tilt: 92.4843 degree |  | Beam 3 Azimuth: -167.9052 degree Down tilt: 90.8003 degree |
|  | - |  | Beam 4 Azimuth: -156.8014 degree Down tilt: 91.5043 degree |
| Rx Beam* | Left panel Azimuth: 153.6520 degree Elevation: 88.3577 degree |  | Left panel Azimuth: 162.3533 degree Elevation: 88.8477 degree |
|  | Right panel Azimuth: 40.6013 degree Elevation: 87.5157 degree |  | Right panel Azimuth: 17.6467 degree Elevation: 88.8477 degree |

- * The track is parallel to x-axis. The CPE moves in the positive direction of the x-axis. The y-axis coordinate of the RRH is Dmin. The 0 degree of elevation is parallel to z-axis.