3GPP TSG-RAN WG4 Meeting # 100-e 	R4-2113311

# Electronic Meeting, August. 16-27, 2021

Agenda Item:		9.13.2.1

Source: 		Qualcomm Incorporated

Title: 	Simulation assumptions for HAPS co-existence

Document for:	Approval

# Introduction

Before RAN4 #100e meeting, the simulation platforms were calibrated between companies. The HAPS performance is not quite aligned through the comparison of SINR CDF, coupling loss CDF and UR Tx power CDF. Therefore, to further calibrate the HAPS performance, we listed all the related parameters and hope to get aligned on the understanding for each parameter.

# Discussion

## Simulation assumptions

The general HAPS network structure is presented in Figure 1. A HAPS area can be divided into multiple cells with multi-beam configurations. In the coexistence study, 7 cells deployment was produced by a single HAPS. In the cases of 7 cells, the cell deployed at the center of a HAPS area is defined as “1st layer cell”, the cells on the outer side of 1st layer cell are defined as “2nd layer cell” respectively.

Figure 1. HAPS network structure

Based on the above network structure, inter-HAPS distance (B) can be calculated by HAPS area radius (A) as “B = A3”. A is equal to 100km in the coexistence study.

The HAPS and TN network parameters are compared in the Table 1.

Most HAPS parameters are the same with the agreements in last RAN4 meeting in [1]. The wrap-around highlighted in the Table 1 should be discussed if it is needed for HAPS network. If wrap-around is not considered, the interference from adjacent cell will be under-estimated for the UEs located in the edge area of 2nd layer cells. One work around method is only collecting the performance of 1st layer cell for simplification, while the antenna configurations are not the same for 1st and 2nd layer cells.

The TN network are divided into two parts due to the antenna difference:  one is with AAS antenna and the other one is with non-AAS. The TN configurations in HAPS coexistence assumptions [1] and NTN coexistence assumptions [2] are found to be different, especially for the antenna array, ISD, conducted power, and ohmic loss, etc. Since the evaluated carrier frequency is 2GHz both for HAPS and NTN, the TN network parameters for HAPS can use the same value with NTN agreements in [2]. The highlighted TN parameters means that they are not settled in HAPS and can be further discussed. For non-AAS antenna, the parameter in Table 3 is used. Only Non-AAS antenna can be used for NB-IoT.

Table 1. Network parameters for HAPS and TN with AAS antenna

| Parameter | HAPS (rural and macro) | TN AAS (rural) | TN AAS (urban) |
| --- | --- | --- | --- |
| Network layout | 7 cells [Wrap-around] | 19 sites (57 cells) wrap-around | 19 sites (57 cells) wrap-around |
| HAPS altitude/ antenna height | 20 km | [30] m | [25] m |
| Carrier frequency | 2 GHz | 2 GHz | 2 GHz |
| Bandwidth | 20 MHz | 20 MHz | 20 MHz |
| Duplex scheme | FDD | FDD | FDD |
| Noise figure | 5 dB | 5 dB | 5 dB |
| ISD | 100*3 km | [7.5] km | [0.75] km |
| Antenna array configuration | 2 x 2 for 1st layer cell 4 x 2 for 2nd layer cell | (M, N, P) = (8, 8, 1) | (M, N, P) = (8, 8, 1) |
| Antenna polarization | Linear ±45° | No polarization | No polarization |
| Element gain GE,max | [7.8] dBi | 7.1 dBi | 6.4 dBi |
| Element HPBW horizontal/vertical φ3dB, θ3dB | [65°] for both H/V | H 90° / V 54° | H 90° / V 65° |
| Element front-to-back ratio horizontal/vertical | 30 dB for both H/V | 30 dB for both H/V | 30 dB for both H/V |
| Element spacing horizontal/vertical | [0.7] for both H/V | 0.5 for H 0.9 for V | 0.5 for H 0.7 for V |
| Antenna panel tilt (from the horizon) | 90° for 1st layer cell 23° for 2nd layer cell | 3° | 10° |
| Conducted Tx power | 43 dBm | 25 dBm | 25 dBm |
| Ohmic loss | 0 | 2 dB | 2 dB |
| EIPR/cell | 56.8 dBm (1st layer cell), 59.8 dBm (2nd layer cell) | 48.1 dBm | 47.4 dBm |
| Antenna element gain pattern | Refer to Table 2 |  |  |

Table 2.  Antenna element gain pattern (TR 38.901)

| Description | Equation | Unit |
| --- | --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am | dB |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ | dBi |
| Composite array radiation pattern | AAθ,φ=AEθ,φ+10log10m=1Mn=1Nwm,nvm,n2, where vm,n=expj2πm-1dvλcosθ+n-1dhλsinθsinφ wm,n=1MNexpj2πm-1dvλsinθetilt-n-1dhλcosθetiltsinφescan | dBi |

Table 3. Non-AAS antenna pattern for TN BS

| Parameter for BS | Values |
| --- | --- |
| Antenna vertical radiation pattern (dB) | AE,V(θ″)=-min12θ″-90°θ3dB2,SLAV,θ3dB=10°,SLAV=20dB |
| Antenna horizontal radiation pattern (dB) | AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=65°,Am=20dB |
| Combining method for 3D antenna pattern (dB) | A''θ'',φ''= -min{-AE,Vθ''+AE,Hφ'', Am} |
| Maximum directional gain of an antenna GE,max | 17 dBi |
| Mechanical down tilt | Rural 3 / Urban 10 |
| Conducted Tx | 46 dBm |

The UE parameters for HAPS and NTN are compared in Table 4. The indoor UE percentage for TN is also set to 0% to keep aligned with the NTN agreements in [2].  The UE bandwidth for TN is not settled and can be further discussed.

Table 4. UE Assumptions for HAPS and TN

| Parameter | HAPS (rural and macro) | TN (rural) | TN (urban) |
| --- | --- | --- | --- |
| UE antenna element gain | 0 dBi, omni-directional | 0 dBi, omni-directional | 0 dBi, omni-directional |
| UE transmit power | 23 dBm | 23 dBm | 23 dBm |
| UE noise figure | 9 dB | 9 dB | 9 dB |
| UE height | 1.5 m | 1.5 m | 1.5 m |
| Indoor UE percentage | 0% | 0% | 0% |
| UE distribution | Uniformly distributed in the coverage area |  |  |
| Number of UE for DL | 1 | 1 | 1 |
| UE DL bandwidth | 20 MHz | 20 MHz | 20 MHz |
| Number of UE for UL | 3 | 3 | 3 |
| UE UL bandwidth 1 | [0.36] MHz | [20/3] MHz | [20/3] MHz |
| UE power control parameter 2 | gamma =1, CL-ile = 121.45 | gamma =1, CL-ile = [ ] | gamma =1, CL-ile = [ ] |
| Note 1: The UE resource allocation should be the same for different HAPS cell to ensure that there is same co channel interference for different UEs. Note 2: power control method refers to , CLx-ile = 88 + 10*log10 (200/X) + 11 – Y, where X is UL transmission BW (MHz), and Y is the BS noise figure |  |  |  |

Proposal 1: The HAPS UE UL bandwidth could be larger than 0.36 MHz since the link budget of HAPS is much better than LEO and GEO. The feasibility of wrap-around network for HAPS can be discussed. While current HAPS performance is still not quite aligned, this could be discussed later when the performance is comparable.

Proposal 2: Since the carrier frequency is 2GHz for both HAPS and NTN, the TN network parameters for HAPS can use the same value with NTN agreements, such as antenna array setting, conducted power, ISD, indoor UE percentage, etc.

## Propagation Model

The propagation model used in the simulation are listed in Table 5 for comparison.

Table 5. Propagation model for HAPS and TN

| Scenario | Radio Link | Channel model |
| --- | --- | --- |
| HAPS | HAPS to HAPS UE | NTN model |
| TN | TN BS to TN UE | TN model |
| HAPS DL to TN DL | HAPS to TN UE | NTN model |
| TN DL to HAPS DL | TN BS to HAPS UE | TN model |
| HAPS UL to TN UL | HAPS UE to TN BS | TN model |
| TN UL to HAPS UL | TN UE to HAPS | NTN model |
| Note: For both rural and urban scenarios, NTN model referred to TR 38.811, and TN model referred to section 7.4 in TR 38.901. |  |  |

# 3. Conclusion

In this paper, we clarify the HAPS and TN assumptions to get aligned about the parameter understanding. We have the following proposals:

Proposal 1: The HAPS UE UL bandwidth could be larger than 0.36 MHz since the link budget of HAPS is much better than LEO and GEO. The feasibility of wrap-around network for HAPS can be discussed. While current HAPS performance is still not quite aligned, this could be discussed later when the performance is comparable.

Proposal 2: Since the carrier frequency is 2GHz for both HAPS and NTN, the TN network parameters for HAPS can use the same value with NTN agreements, such as antenna array setting, conducted power, ISD, indoor UE percentage, etc.

# References

- R4-2108646, Simulation assumptions for HAPS co-existence

- R4-2108645, Simulation assumptions for NTN co-existence study