3GPP TSG-RAN WG4 Meeting #100-e	R4-2113643

Electronic Meeting, 16 Aug. – 27 Aug. 2021

Agenda Item:	10.7.2

Source:	Ericsson

Title:	Simulation results on Test methodology for application layer data throughput performance

Document for:	Information

# 1	Introduction

In RAN4#99-e meeting, a WF for application layer data throughput performance requirements was agreed [1]. In this contribution, simulation results for the absolute physical layer throughput with CQI and RI statistics and decode success rate will be shown. The detailed simulation assumptions are listed in table below.

| Parameter |  |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 | 40 | 100 |
| Subcarrier spacing |  |  | kHz | 15 | 30 | 120 |
| Duplex Mode |  |  |  | FDD | TDD | TDD |
| TDD Slot configuration |  |  |  | NA | FR1.30-1 | FR2.120-2 |
| Propagation channel |  |  |  | TDLA30-5 |  | TDLA30-35 |
| Antenna configuration |  |  |  | ULA Low 2x2, ULA Low 2x4 |  | ULA Low 2x2 |
| Beamforming Model |  |  |  | As defined in Annex B.4.1 |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 | 4 | 4 |
|  | CDM Type |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 5, (4,-) | Row 5, (4,-) | Row 4, (8,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | (9,-) | (9,-) | (13,-) |
|  | CSI-RS periodicity and offset |  | slot | 5/1 | 10/1 | 8/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Aperiodic | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  |  | 2 | 4 | 2 |
|  | CDM Type |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 3 (6,-) | Row 5, (4,-) | Row 3 (6,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | (13,-) | (9,-) | (13,-) |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 | 10/1 | Not configured |
|  | aperiodicTriggeringOffset |  |  | - | - | 0 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic | Periodic | Periodic |
|  | CSI-IM RE pattern |  |  | Pattern 0 | Pattern 0 | Pattern 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4,9) | (4,9) | (8,13) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 | 10/1 | Not configured |
| ReportConfigType |  |  |  | Aperiodic |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | not configured |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | not configured |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |
| Sub-band Size |  |  | RB | 8 | 16 | 8 |
| csi-ReportingBand |  |  |  | 1111111 | 1111111 | 111111111 |
| CSI-Report periodicity and offset |  |  | slot | 5/0 | 10/9 | Not configured |
| Aperiodic Report Slot Offset |  |  |  | 7 |  |  |
| CSI request |  |  |  | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 |  |  |
| reportTriggerSize |  |  |  | 1 |  |  |
| CSI-AperiodicTriggerStateList |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A | N/A | N/A |
|  |  | CodebookSubsetRestriction |  | 111111 | 111111 | 111111 |
|  |  | RI Restriction |  | N/A | N/A | N/A |
| Physical channel for CSI report |  |  |  | PUCCH | PUCCH | PUCCH |
| CQI/RI/PMI delay |  |  | ms | 8 | 9.5 | 1.375 |
| Maximum number of HARQ transmission |  |  |  | 1 | 1 | 1 |

# 2	Simulation Results

The way forward[1] encouraged to provide the simulation results for absolute throughput also with the CQI and RI statistics, decoding rate. We have the simulation results for these three test cases in the appendix.

It could be found that a lower BLER can be achieved with a relatively lower CQI/RI feedback. From system perspective, lower BLER with less HARQ feedback is preferred. Thus, we propose RAN4 not only to align the throughput results but also to consider the reasonable CQI, RI feedback and decoding rate.

The simulation results are shown in the appendix.

- FR1 FDD 2*2

| SNR (dB) | Throughput (Mbps) | BLER | Median CQI | Median RI |
| --- | --- | --- | --- | --- |
| 0 | 3.12 | 0.0349 | 2 | 1 |
| 2 | 4.73 | 0.0433 | 3 | 1 |
| 4 | 6.67 | 0.0692 | 4 | 1 |
| 6 | 8.80 | 0.0816 | 6 | 1 |
| 8 | 10.73 | 0.0837 | 6 | 1 |
| 10 | 12.94 | 0.0929 | 8 | 1 |
| 12 | 15.26 | 0.0991 | 8.5 | 1 |
| 14 | 17.53 | 0.1157 | 10 | 1 |
| 16 | 20.30 | 0.1086 | 11 | 2 |
| 18 | 22.86 | 0.1177 | 11 | 2 |
| 20 | 25.76 | 0.1185 | 8 | 2 |

- FR1 FDD 2*4

| SNR (dB) | Throughput (Mbps) | BLER | Median CQI | Median RI |
| --- | --- | --- | --- | --- |
| 0 | 6.51 | 0.08732 | 4 | 1 |
| 2 | 8.58 | 0.08751 | 6 | 1 |
| 4 | 10.64 | 0.06077 | 4 | 2 |
| 6 | 13.52 | 0.07542 | 5 | 2 |
| 8 | 17.08 | 0.0501 | 6 | 2 |
| 10 | 20.65 | 0.05247 | 7 | 2 |
| 12 | 24.40 | 0.04897 | 8 | 2 |
| 14 | 28.66 | 0.04016 | 8 | 2 |
| 16 | 32.63 | 0.05406 | 9 | 2 |
| 18 | 37.06 | 0.06683 | 11 | 2 |
| 20 | 41.93 | 0.06222 | 12 | 2 |

- FR1 TDD 2*2

| SNR (dB) | Throughput (Mbps) | BLER | Median CQI | Median RI |
| --- | --- | --- | --- | --- |
| 0 | 9.25 | 0.00559 | 2.5 | 1 |
| 2 | 14.67 | 0.02189 | 3 | 1 |
| 4 | 21.98 | 0.04285 | 5 | 1 |
| 6 | 28.86 | 0.04283 | 6 | 1 |
| 8 | 35.44 | 0.04668 | 7 | 1 |
| 10 | 42.54 | 0.04854 | 8 | 1 |
| 12 | 49.42 | 0.04536 | 9 | 1 |
| 14 | 56.53 | 0.06069 | 10 | 1 |
| 16 | 63.65 | 0.0761 | 7 | 2 |
| 18 | 73.07 | 0.07563 | 8 | 2 |
| 20 | 83.40 | 0.05934 | 8.5 | 2 |

- FR1 TDD 2*4

| SNR (dB) | Throughput (Mbps) | BLER | Median CQI | Median RI |
| --- | --- | --- | --- | --- |
| 0 | 22.0 | 0.05216 | 4 | 1 |
| 2 | 27.03 | 0.03551 | 5 | 1 |
| 4 | 33.78 | 0.02152 | 3 | 2 |
| 6 | 44.39 | 0.02059 | 4 | 2 |
| 8 | 56.34 | 0.01454 | 5 | 2 |
| 10 | 68.26 | 0.01655 | 6 | 2 |
| 12 | 81.50 | 0.01197 | 7 | 2 |
| 14 | 95.19 | 0.01014 | 8 | 2 |
| 16 | 108.61 | 0.03307 | 9 | 2 |
| 18 | 121.06 | 0.06203 | 10 | 2 |
| 20 | 132.83 | 0.08083 | 11 | 2 |

- FR2 TDD 2*2

| SNR (dB) | Throughput (Mbps) | BLER | Median CQI | Median RI |
| --- | --- | --- | --- | --- |
| 0 | 21.62 | 0.03705 | 5 | 1 |
| 2 | 29.87 | 0.037 | 6 | 1 |
| 4 | 39.86 | 0.02368 | 7 | 1 |
| 6 | 51.31 | 0.02507 | 9 | 1 |
| 8 | 62.15 | 0.03583 | 9 | 1 |
| 10 | 74.48 | 0.03131 | 11 | 1 |
| 12 | 87.31 | 0.03045 | 11 | 1 |
| 14 | 100.1 | 0.02255 | 9 | 2 |
| 16 | 113 | 0.0234 | 9 | 2 |

# References

- R4-2108102, “WF on 5G NR UE Application Layer Data Throughput Performance”, Qualcomm Inc.

# Appendix

Test 1(FR1 FDD)

- Throughput

- RI and CQI statistics

- BLER

Test 2(FR1 TDD)

- Throughput

- RI and CQI statistics

- BLER

Test 3(FR2)

- Throughput

- RI and CQI statistics

- BLER