3GPP TSG-RAN4 Meeting #100-e	R4-2113653

# Electronic Meeting, 16th – 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | n/a | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DRAFT CR to TS 38.101-2 Introducing extension of FR2 to cover up to 71GHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz |  |  |  |  |  | Date: |  |  | 2021-07-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The existing frequency range designation FR2 needs to be updated to cover also the 52.6 – 71GHz frequency range |  |  |  |  |  |  |  |  |
| Summary of change: |  | At RAN#92-e it was agreed to include the new frequencies 56.2 – 71GHz in the existing frequency range definition by etxending the current FR2 definition up to 71GHz. And to introduce two sub frequency ranges FR2-1 and FR2-2. FR2-1 is equvalent to the existing FR2 definition 24250 – 52600 MHz and FR2-2 is defined for 52600 – 71000 MHz. It was also agreed that whenever FR2 is referred, both FR2-1 and FR2-2 frequency sub-ranges shall be considered. This is captured in the note in the updated table. It was also agreed that the designations of FR2-1 and FR2-2 shall only be used when needed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 52.6 – 71GHz frequency range will not be covered in the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 5	Operating bands and channel arrangement

## 5.1	General

The channel arrangements presented in this clause are based on the operating bands and channel bandwidths defined in the present release of specifications.

NOTE:	Other operating bands and channel bandwidths may be considered in future releases.

Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which NR can operate according to this version of the specification are identified as described in Table 5.1-1.

Table 5.1-1: Definition of frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1 | 410 MHz – 7125 MHz |
| FR2 | FR2-1: 24250 MHz – 52600 MHz |
|  | FR2-2: 52600 MHz – 71000 MHz |
| NOTE: Whenever the FR2 is solely referred, both FR2-1 and FR2-2 frequency sub-ranges shall be considered in this release. |  |

The present specification covers FR2 operating bands.

## 5.2	Operating bands