3GPP TSG-RAN WG4 #100-e	R4- 2113656

Electronic meeting, 16th – 27th of August 2021

Source: 	Ericsson

Title:  	On Beam Correspondence requirements for FR2 HST UE

Agenda Item:	9.9.3.2

Document for:	Discussion

# Introduction

During RAN#89-e, a WI was approved relating to developing high speed train requirements for FR2.

Part of the WI objectives is to evaluate if PC4 UE requirements need to be updated.

Objective on UE core requirement copied from the WID in [1]:

- Specify the UE RF core requirements for power class 4 if identified

  - Introduction for beam correspondence requirements for PC4 if identified

## UE RF WF at RAN4#98-bis-e with regards to Beam Correspondance

At RAN4#99-e a WF on HST for FR2 was agreed in R4-2107861[4]. For Beam Correspondence requirements the following is stated:

Beam Correspondence requirement for FR2 HST UE:

- For Rel-15 Beam Correspondence:

  - FR2 HST UE (roof-mounted UE type) shall mandatorily support beamCorrespondenceWithoutUL-BeamSweeping.

- For Rel-16 Beam Correspondence:

  - Whether or not FR2 HST UE need to support Rel-16 optional feature beamCorrespondenceSSB-based-r16:

    - FFS the benefits.

    - If not, no need to define corresponding requirement for Rel-16 optional feature beamCorrespondenceSSB-based-r16 for Rel-17 FR2 HST UE.

  - Whether or not UE need to support Rel-16 optional feature beamCorrespondenceCSI-RS-based-r16:

    - FFS the benefits.

    - If not, no need to define corresponding requirement for Rel-16 optional feature beamCorrespondenceCSI-RS-based-r16 for Rel-17 FR2 HST UE.

# Discussion on Beam Correspondence

In the discussion paper “On UE beam and pwr requirements for FR2 HST” (R4-2113655) [5] we analyze and propose to limit the spherical coverage requirements as well as increasing the output pwr for HST FR2 UEs. In [5] we also suggest using PC1 as baseline for UE power and spherical coverage.

Background: The support for Beam Correspondence is fundamental for FR2 operation. By means of beam correspondence, the UE can determine its TX beam used for transmission to a gNB based on knowledge of the RX beam used to receive data from the said gNB.

A UE supporting beam correspondence can transmit an UL signal in the direction of receiving a received DL reference signal. Beam correspondence is important both in connected mode and for initial access. For the latter, the random-access response and msg4 are spatially QCL with the SSB (same DL beam), but there are no means for the UE to form its UL beams (msg1 and msg3) other than beam correspondence.

The current (not mandatory) Rel-15 requirements on beam correspondence are only verified in connected mode and assumed to be met if the UE meets the spherical coverage requirements. However, for a UE with inadequate beam correspondence support (compliance with the spherical coverage requirement is optional), the determination of the TX beam for connected mode operation must be improved by SRS sweeping. These UEs would not indicate beamCorrespondenceWithoutUL-BeamSweeping in their UE capability and would have to rely on UL SRS (beam) sweeping.

During the discussion on Beam Correspondence requirements for HST FR2 at the RAN4#98-e meeting several companies argued for excluding Beam Correspondence with “Bit-0” I.e. without beam sweeping (I.e. UE not indicating support of “beamCorrespondenceWithoutUL-BeamSweeping”).

Beam correspondence with UL beam sweeping is also not possible at initial access.

Observation 1: In the WF in [4) it was agreed that FR2 HST UE shall mandatory support beamCorrespondenceWithoutUL-BeamSweeping.

As of now there is no beam correspondence requirement for PC4 and PC4 UE is to be considered as the baseline UE Power Class for HST FR2. Furthermore, there are no Beam correspondence requirements for PC1, in [5] we suggest to base UE power and spherical requirements on PC1 instead of PC4 or PC5.

Observation 2: There are no Beam correspondence requirements defined for PC1 or PC4 in TS38.101-2.

As stated in TS38.101-2 for Beam Correspondence:

### 6.6.4	Beam correspondence for power class 3

#### 6.6.4.1	General

The beam correspondence requirement for power class 3 UEs consists of three components: UE minimum peak EIRP (as defined in Clause 6.2.1.3), UE spherical coverage (as defined in Clause 6.2.1.3), and beam correspondence tolerance (as defined in Clause 6.6.4.2). The beam correspondence requirement is fulfilled if the UE satisfies one of the following conditions, depending on the UE's beam correspondence capability IE beamCorrespondenceWithoutUL-BeamSweeping, as defined in TS 38.306 [14]:

Proposal 1: Beam correspondence requirements for HST FR2 UE’s are needed.

# Conclusion

Based on the summary in clause 2 we propose and observe the following:

Observation 1: In a HST deployment scenario there is a need for UEs to support BC without sweeping.

Observation 2: There are no Beam correspondence requirements defined for PC1 or PC4 in TS38.101-2.

Proposal 1: Beam correspondence requirements for HST FR2 UE’s are needed and shall be without UL beam sweeping.

# References

RP-202118, WID, “New WID on NR support for high speed train scenario in FR2”, Nokia, Samsung

R4-2103240, “WF on NR support for HST in FR2”, Samsung

R4-2105491, “WF on UE RF requirement for FR2 HST”, Samsung

R4-2107861, “WF on UE RF requirement for FR2 HST”, Samsung

R4-2113655, “On UE beam and pwr requirements for FR2 HST”, Ericsson