3GPP TSG-RAN WG4 Meeting#100     	R4-2113699

E-meeting, 16th – 27th  Aug, 2021

Title:	TP to TR 38.717-04-02 Addition of CA_n1A-n28A-n40-n78A

Source:	Nokia

Agenda item:	8.12.2

Document for:	Approval

# Introduction

This is a TP to TR 38.717-04-02 Addition of CA_n1A-n28A-n40-n78A

********************** Start of TP *********************************

## 5.2.x	CA_n25-n66-n71-n77

### 5.2. x.1	Operating bands for CA

Table 5.2.x-1: Inter-band CA operating bands

| NR CA Band | NR Band (Table 5.2-1 in TS38.101-1[2]) |
| --- | --- |
| CA_n1-n28-n40-n78 | n1, n28, n40, n78 |

#### 5.2.x.2	Channel bandwidths per operating band for CA

Table 5.2.x.2-1: Supported bandwidths

| NR CA Configuration | UL Config | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n28A-n40A-n78A | CA_n1A-n28A CA_n1A-n40A CA_n1A-n78A CA_n28A-n40A CA_n28A-n78A CA_n40A-n78A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n1A-n28A-n40B-n78A | CA_n1A-n28A CA_n1A-n40A CA_n1A-n78A CA_n28A-n40A CA_n28A-n78A CA_n40A-n78A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | See CA_n40B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

#### 5.2.x.3	UE Co-existence study

The coexistence studies have been captured into the constituent fallback modes in TR 38.717-03-02 and in TS 38.101-1, there is no additional harmonic and intermodulation impact for the additional band receiver.

#### 5.2.x.5	REFSENS requirements

There are no additional MSD requirements for this band combination.

********************** End of TP *********************************