3GPP TSG-RAN WG4 Meeting #100-e	R4-2113735

# Electronic meeting, August, 2021

Source:		Ericsson

Title:			TP to TR 38.847

Agenda item:		8.3.1

Document for:		Approval

# Introduction

In RAN#88e, a WID to introduce the 47 GHz band was approved. The purpose of the WID is to specify NR band n262 covering the entire 47.2-48.2 GHz in order to accommodate operations in any part of this range.

In last RAN4#99-e meeting, many agreements were reached on UE PC1, 2 and 5 and associated CR was approved [2]. This TP is proposing to capture those agreements in the 47GHz TR.

# Text Proposal

<Start of the change>

## 8.1	UE specific

### 8.1.1	Transmitter characteristics

The following requirements define the maximum output power radiated by the UE for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The minimum output power values for EIRP are found in Table 8.1.1-1. The requirement is verified with the test metric of total component of EIRP (Link=TX beam peak direction, Meas=Link angle). The requirement for the UE which supports a single FR2 band is specified in Table 8.1.1-1. The requirement for the UE which supports multiple FR2 bands is specified in both Table 8.1.1-1 and Table 8.1.1-4.

Table 8.1.1-1: UE minimum peak EIRP for power class 1, 2, 3 and 4

| Operating band | Min peak EIRP (dBm) |  |  |  |
| --- | --- | --- | --- | --- |
|  | PC1 | PC2 | PC3 | PC4 |
| n262 | 34.2 | 22.9 | 16.0 | 28.3 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance |  |  |  |  |

The maximum output power values for TRP and EIRP are found on the Table 8.1.1-2. The max allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode and the total component of EIRP (Link=TX beam peak direction, Meas=Link angle.

Table 8.1.1-2: UE maximum output power limits for power class 1, 2, 3 and 4

| Operating band | Power Class | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- | --- |
| n262 | PC1 | 35 | 55 |
|  | PC2 | 23 | 43 |
|  | PC3 | 23 | 43 |
|  | PC4 | 23 | 43 |

The minimum EIRP at the 50th percentile of the distribution of radiated power measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 8.1.1-3 below. The requirement is verified with the test metric of the total component of EIRP (Link=Beam peak search grids, Meas=Link angle). The requirement for the UE which supports a single FR2 band is specified in Table 8.1.1-3. The requirement for the UE which supports multiple FR2 bands is specified in both Table 8.1.1-3 and Table 8.1.1-4.

Table 8.1.1-3: UE spherical coverage for power class 1, 2, 3 and 4

| Operating band | Min EIRP |  |  |  |
| --- | --- | --- | --- | --- |
|  | PC1 at 85 %-tile CDF (dBm) |  | PC2 at 60 %-tile CDF (dBm) |  | PC3 at 50 %-tile CDF (dBm) |  | PC4 at 20 %-tile CDF (dBm) |
| n262 |  | 26.0 |  | 11.0 |  | 2.9 |  | 16.2 |
| NOTE 1: Minimum EIRP at 50 %-tile CDF is defined as the lower limit without tolerance |  |  |  |  |

For the UEs that support multiple FR2 bands, minimum requirement for peak EIRP and EIRP spherical coverage in Tables 8.1.1-1 and 8.1.1-3 shall be decreased per band, respectively, by the peak EIRP relaxation parameter MBP,n and EIRP spherical coverage relaxation parameter MBS,n, as defined in Table 8.1.1-4.

Table 8.1.1-4: UE multi-band relaxation factors for power class 3

| Band | MBP,n (dB) | MBS,n (dB) |
| --- | --- | --- |
| n257 | 0.73 | 0.73 |
| n258 | 0.6 | 0.7 |
| n259 | 0.5 | 0.4 |
| n260 | 0.51 | 0.41 |
| n261 | 0.52,4 | 0.74 |
| n262 | 0.7 | 0.7 |
| Note 1: n260 peak and spherical relaxations are 0 dB for UE that exclusively supports n261+n260 Note 2: n261 peak relaxation is 0 dB for UE that exclusively supports n261+n260 Note 3: n257 peak and spherical relaxations are 0 dB for UE that exclusively supports n261+n257 Note 4: n261 peak and spherical relaxations are 0 dB for UE that exclusively supports n261+n257 |  |  |

The minimum output power shall not exceed the values specified in Table 8.8.1-5 for each operating band supported. The minimum power is verified in beam locked mode with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 8.1.1-5: Minimum output power for power class 1, 2, 3 and 4

| Operating band | Power Class | Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- | --- |
| n262 | PC1 | 50 | 4 | 47.58 |
|  |  | 100 | 4 | 95.16 |
|  |  | 200 | 4 | 190.20 |
|  |  | 400 | 4 | 380.28 |
| n262 | PC2, PC3 and PC4 | 50 | -13 | 47.58 |
|  |  | 100 | -13 | 95.16 |
|  |  | 200 | -13 | 190.20 |
|  |  | 400 | -13 | 380.28 |

The transmit OFF power is defined as the TRP in the channel bandwidth when the transmitter is OFF. The transmitter is considered OFF when the UE is not allowed to transmit on any of its ports.

The transmit OFF power shall not exceed the values specified in Table 8.8.1-6 for each operating band supported. The requirement is verified with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid).

Table 8.8.1-6: Transmit OFF power

| Operating band | Channel bandwidth / Transmit OFF power (dBm) / measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n262 | -35 | -35 | -35 | -35 |
|  | 47.58 MHz | 95.16 MHz | 190.20 MHz | 380.28 MHz |

Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency. ACLR requirement is specified for a scenario in which adjacent carrier is another NR channel.

NR Adjacent Channel Leakage power Ratio (NRACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency at nominal channel spacing. The assigned NR channel power and adjacent NR channel power are measured with rectangular filters with measurement bandwidths specified in Table 8.8.1-7.

If the measured adjacent channel power is greater than –35 dBm then the NRACLR shall be higher than the value specified in Table 8.8.1-7. The requirement is verified in beam locked mode with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid).

Table 8.8.1-7: General requirements for NRACLR

|  | Channel bandwidth / NRACLR / Measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| NRACLR for band n262 | 16 dB | 16 dB | 16 dB | 16 dB |
| NR channel measurement bandwidth (MHz) | 47.58 | 95.16 | 190.20 | 380.28 |
| Adjacent channel centre frequency offset (MHz) | +50 / -50 | +100 / -100 | +200 / -200 | +400 / -400 |

This table 8.8.1-8 specifies the requirements for coexistence with protected bands.

Table 8.8.1-8: Requirements

| NR Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band/frequency range | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n260 | NR Band 257 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 262 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n261 | NR Band 260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | NR Band 262 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n262 | NR Band 260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |

### 8.1.2	Receiver characteristics

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1) with peak reference sensitivity specified in Table 8.1.2-1. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link Angle).

For the UEs that support multiple FR2 bands, the minimum requirement for Reference sensitivity in Table 8.1.2-1 shall be increased per band, respectively, by the reference sensitivity relaxation parameter ∆MBP,n as specified in clause 8.1.1.  The requirement for the UE which supports a single FR2 band is specified in Table 8.1.2-1. The requirement for the UE which supports multiple FR2 bands is specified in both Table 8.1.2-1 and Table 8.1.1-4.

Table 8.1.2-1: Reference sensitivity for power class 1, 2, 3 and 4

| Operating band | Power Class | REFSENS (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n262 | PC1 | -92.5 | -89.5 | -86.5 | -83.5 |
|  | PC2 | -86.8 | -83.8 | -80.8 | -77.5 |
|  | PC3 | -82.8 | -79.8 | -76.8 | -73.8 |
|  | PC4 | -91.0 | -88.0 | -85.0 | -82.0 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 |  |  |  |  |  |

The REFSENS requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to that specified in Table 8.1.2-1.

Unless given by Table 7.3.2.1-3 in TS 38.101-2, the minimum requirements for reference sensitivity shall be verified with the network signalling value NS_200 (Table 6.2.3.1-1 in TS 38.101-2) configured.The maximum EIS at the 50th percentile of the CCDF of EIS measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 8.1.2-2 below. The requirement is verified with the test metric of EIS (Link=Spherical coverage grid, Meas=Link angle).

For the UEs that support multiple FR2 bands, the minimum requirement for EIS spherical coverage in Table 8.1.2-2 shall be increased per band, respectively, by the EIS spherical coverage relaxation parameter ∆MBS,n as specified in clause 8.1.1.  The requirement for the UE which supports a single FR2 band is specified in Table 8.1.2-2. The requirement for the UE which supports multiple FR2 bands is specified in both Table 8.1.2-2 and Table8.1.1-4.

Table 8.1.2-2: EIS spherical coverage for power class 1, 2, 3 and 4

| Operating band | Power Class | EIS at 50th %-tile CCDF (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n262 | PC1 | -84.3 | -81.3 | -78.3 | -75.3 |
|  | PC2 | -74.9 | -71.9 | -68.9 | -65.9 |
|  | PC3 | -69.7 | -66.7 | -63.7 | -60.7 |
|  | PC4 | -78.9 | -75.9 | -72.9 | -69.9 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in Annex E.2.1. |  |  |  |  |  |

The requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to that specified in Table 8.1.2-2.

Unless given by Table 7.3.2.1-3 in TS 38.101-2, the minimum requirements for reference sensitivity shall be verified with the network signalling value NS_200 (Table 6.2.3.1-1 in TS 38.101-2) configured.

<End of the change>

# Conclusion

This contribution provides the text proposal of expected changes to TR 38.847. It is proposed to agree on this text proposal.

# References

- RP-201232, “New WID on introduction of NR 47 GHz band”

- R4-2107791, Introduction of n262 UE RF requirements, Nokia, Nokia Shanghai Bell