3GPP TSG-RAN WG4 Meeting #100-e	R4-2113736

# Electronic meeting, August 16th-27th, 2021

Source:		Ericsson

Title:			APT 600MHz - TP to TR 38.860 – Conclusion

Agenda item:		10.4.4

Document for:		Approval

# Introduction

The Asia-Pacific Telecommunity (APT) sent a LS ([1]) to 3GPP RAN/RAN4 asking 3GPP for feedback on the feasibility of 2 different frequency arrangements in the scope of harmonizing 470-698 MHz band for ITU Region 3. A corresponding Study Item ([2]) was approved in last RAN#90e meeting to address this LS.

In last RAN4#99-e meeting, the different options (B1, B2 and B2a) were further discussed and corresponding TPs were agreed ([3], [4] and [5]). This TP to TR 38.860 is proposing a tentative conclusion.

# Text Proposal

<Start of the change>

# 9.	Conclusion

Option B1 would require that an additional 5 MHz broadcast spectrum is repurposed. The main advantage of this option is that it could be supported with a single 40 MHz filter, even if this is still challenging. UEs could meet band n71 requirements. With option B1, the network cannot allow n71 UEs since the duplex spacing is different.

With option B2, a single UE filter could not be considered and some degradation might be expected for full range due to the 6 MHz gap only. UEs could meet band n71 requirements. Also, option B2 network could allow n71 legacy UEs by using multiple FBI (as long as option B2 SEM is the same as NR or as for n71).

Option B2a would introduce a new band overlapping n71.

<End of the change>

# Conclusion

This contribution provides the text proposal on summary of expected changes to TR 38.860. It is proposed to agree on this text proposal.

# References

- RP-202143, Frequency arrangements for IMT in the band 470 – 703 MHz, LS from Asia-Pacific Telecommunity.

- RP-202924	, New SID on extended 600 MHz NR band TSG RAN 90e

- R4-2108000, TP for TR 38.860:  Filter option B1, Qualcomm

- R4-2107891, TP for TR 38.860: Filter option B2, Nokia

- R4-2107999, TP to TR38.860 on band plan and duplex filter considerations for 600 MHz, Apple