3GPP TSG-RAN WG4 Meeting #100-e	R4-2113748

# Electronic meeting, August 16th-27th, 2021

Source:		Ericsson

Title:			RMR 900MHz band – General

Agenda item:		8.4.1

Document for:		Approval

# Introduction

The 900MHz NR band for European RMR WI ([1]) has been revised in RAN#92e meeting to introduce a new 900MHz band for Railway in Europe.

After some initial discussion  in last RAN4#99-e meeting, this contribution is further addressing the general and system parameters aspects for this new band.

# Discussion

## ECC Decision(20)02

ECC Decision(20)02 has specified the LRTC for the new 900MHz RMR band and for BS, in Annex A2.1, the following restriction has been mentioned:

The rationale for this restriction is a bit unclear, there is no detailed justification in any ECC Report but it should be noted that:

- The lowest GSM-R carrier in the band is centred at 919.6 MHz.

- CEPT has extrapolated the definition of a NR 5.6 MHz channel, with a transmission bandwidth of 28 RBs between 919.6725 MHz and 924.7125 MHz.

- CEPT Report 313 ([3]) mentions “For FRMCS, the lowest possible RB starts around 919.6 MHz.”

In last RAN4#99-e meeting, contribution [4] made the following observations, requesting for further inputs:

Actually, if those observations might be correct, the intention for those observations are unclear to us: ECC Decision doesn’t mandate that the lowest RB edge of a 5 MHz NR signal shall be positioned at 919.6 MHz, but it mandates its frequency position is at least at 919.6 MHz.

So, considering that a 5 MHz NR signal will have a minimum guard band of 242.5 kHz, its lowest RB edge will be located at 919.6425 MHz, which is aligned with ECC Decision.

Based on this conclusion, our understanding is that no special attention should be taken on the channel raster for the 900MHz RMR band.

Observation: A 5 MHz NR signal will have its lowest edge RB located at frequency 919.6425 MHz, which is compliant with ECC Decision(20)02.

## NB-IoT

According to ECC Decision(20)02 ([2]), deployment of NB-IoT in the RMR 900MHz band is possible under following conditions:

- Standalone operation mode is allowed with a maximum EIRP specified in Table 4 of this Decision (see 2.1).

- Guard band operation mode is not allowed.

- In-band operation mode is allowed but without power boosting.

Power boosting has always been a key feature of NB-IoT, enabling enhanced coverage of devices located in location with very bad coverage.

NB-IoT standalone mode has no impact on NR specifications, this mode is specified in LTE specifications.

It should also be noted that the latest revision of the WI ([1]) doesn’t mention any support of NB-IoT.

Based on those observations, we would propose the following:

Proposal1: NB-IoT support in [900 MHz RMR] band is not considered in this WI.

## System parameters

### Band definition

The new 900MHz RMR band should have the next band number available in TS 38.104 v17.2.0, which is: n100.

It shall be added to list of NR operating bands in FR1 as proposed below:

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n100 | 874.4 MHz – 880.0 MHz | 919.4 MHz – 925 MHz | FDD |

### BS and UE channel bandwidth

Following channel BWs shall be added to the table listing the supported channel per operating bands:

| NR band / SCS / BS channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
|  | 15 | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
| n100 | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### Channel raster

No guard band is mentioned in ECC Decision(20)02 for the 900MHz RMR band.

This band shall use a 100 kHz channel raster and the NR-ARFCN shall be defined as below:

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n100 | 100 | 174880 – <20> – 176000 | 183880 – <20> – 185000 |

### Synchronization raster

The synchronization raster information for the new 900MHz RMR band shall be specified as below:

| NR operating band | SS Block SCS | SS Block pattern (NOTE 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n100 | 15 kHz | Case A | 2303 – <1> – 2307 |

# Conclusion

In this contribution, we studied the generla aspects of introducing the 900MHz RMR band, based on ECC Decision (20)02. We made the following observation and proposals:

Observation: A 5 MHz NR signal will have its lowest edge RB located at frequency 919.6425 MHz, which is compliant with ECC Decision(20)02.

Proposal1: NB-IoT support in [900 MHz RMR] band is not considered in this WI.

Proposal2: Specify the following system parameters for the 900MHz RMR band:

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n100 | 874.4 MHz – 880.0 MHz | 919.4 MHz – 925 MHz | FDD |

| NR band / SCS / BS channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
|  | 15 | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
| n100 | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n100 | 100 | 174880 – <20> – 176000 | 183880 – <20> – 185000 |

| NR operating band | SS Block SCS | SS Block pattern (NOTE 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n100 | 15 kHz | Case A | 2303 – <1> – 2307 |

# References

- RP-211495, Introduction of 900MHz NR band for Europe for Rail Mobile Radio (RMR), UIC

- ECC Decision(20)02,  Harmonised use of the paired frequency bands 874.4-880.0 MHz and 919.4-925.0 MHz and of the unpaired frequency band 1900-1910 MHz for Railway Mobile Radio (RMR), 20 November 2020

- ECC Report 313, Technical study for co-existence between RMR in the 900 MHz range and other applications in adjacent bands

- R4-2111054, Discussion on channel raster aspects for RMR900, Huawei