3GPP TSG-RAN WG4 Meeting #100-e	R4-2113804

# Electronic Meeting, 16th - 27th Aug, 2021

Title: 	Discussion on UE demodulation requirements for FR2 HST

Source: 	Huawei, HiSilicon

Agenda item:	9.9.5.2

Document for:	Discussion

Background

During RAN#99-e meeting, WF [1] on Demodulation requirement for FR2 HST was approved. In this contribution, we share our views about the UE demodulation requirements for FR2 HST.

Discussion

Test Scope

Doppler frequency

- Doppler frequency for PDSCH requirement - Doppler frequency for PDSCH requirement in Bi-directional deployment scenario, if Bi-directional deployment scenario is introduced - Option 1: 9722Hz targeting 350km/h at 30GHz - Option 2: 7000Hz with the smallest RS range of frequency offset estimation - Doppler frequency for PDSCH requirement in Uni-directional deployment scenario, if Uni -directional deployment scenario is introduced - 9722Hz targeting 350km/h at 30GHz

The maximum supporting velocity is inversely proportional to the interval of the two closest symbols on which reference signals are transmitted. The detail theory analysis value using TRS or SSB for frequency tracking is shown in Table 2.1.1-1 below, with the assumption that 30 GHz carrier frequency and 120 kHz SCS.

Table 2.1.1-1 Maximum supporting velocity, fc=29.5GHz, SCS=120kHz

| Physical channel | Reference signal(s) | Maximum FOE capability [Hz] | velocity [km/h] |
| --- | --- | --- | --- |
| PDSCH | TRS | 13989 | 252 |
|  | SSB | 28029 | 505 |

We can see that it is feasible to use SSB+TRS for tracking frequency offset for downlink to support 350km/h, using SSB for coarse frequency offset estimation and TRS for precise frequency offset estimation. So we prefer to define 350km/h requirements, i.e. Doppler of 9722Hz targeting 350km/h at 30GHz for both Bi-directional and Uni-directional RRH deployment.

It is feasible to use SSB+TRS for tracking frequency offset for downlink to support 350km/h, using SSB for coarse frequency offset estimation and TRS for precise frequency offset estimation.

Define 350km/h requirements, i.e. Doppler of 9722Hz targeting 350km/h at 30GHz for both Bi-directional and Uni-directional RRH deployment.

Requirement for scenario A or B

- PDSCH requirement for Uni/Bi-directional scenario in scenario A and scenario B - Option 1: Define PDSCH requirement with Uni/Bi-directional scenario for both A and B, Define the test applicability rule to reduce the test effort - Option 2: Define requirements for both scenario A/B, and Uni/Bi-directional deployment, and not define any applicability between them - Option 3 : Consider output of FR2 HST deployment scenario discussion whether to cover scenario A - Option 4 : RAN4 define two test cases for HST FR2 - Test 1: HST single tap (Uni-directional) with scenario A - Test 2: DPS (Uni-directional) with scenario B - If RAN4 agree to consider both Uni-directional and Bi-directional deployment, either test 1 or 2 apply Bi-directional model

Different deployment scenarios can be deployed in different region and the different algorithm can be used for different deployment scenarios. It is not suitable to only define performance for only one deployment scenario or define some applicability rules between different deployment scenarios, otherwise the UE maybe not work under certain deployment scenario. Therefore, we prefer to define requirements for both scenario A/B and uni/bi-directional deployment, and not define any applicability rule between them.

Define requirements for both scenario A/B and uni/bi-directional deployment, and not define any applicability rule between them.

Transmission schemes

- DPS transmission scheme - Option 1: - Define PDSCH requirement with DPS scheme 1a in Uni-directional scenario for scenario A. FFS scheme 1b - Define PDSCH requirement with DPS scheme 1a and 1b in Uni-directional scenario for scenario B, FFS the number of TCI state configured - Define PDSCH requirement with DPS scheme 1a in Bi-directional scenario for scenario A and scenario B. FFS scheme 1b - Option 2: Define both DPS transmission scheme 1a and 1b for both Bi-directional and Uni-directional deployment - Option 2a : RAN4 define both scheme 1a and 1b if the performance is same, but define the same applicability rule as Rel-16 HST, i.e., if a UE declared supporting > 1 TCI states, the UE will pass scheme 1b and skipped scheme 1a test cases, and if a UE only support 1 TCI state, the UE need to pass scheme 1a and skip scheme 1b test cases.

In our view, another panel can be used for beam search using TDM. We don’t see any feasibility issue for DPS transmission scheme 1b for both Uni-directional and Bi-directional deployment. Therefore, we prefer to define both DPS scheme 1a and scheme 1b for both Bi-directional and Uni-directional deployment. Same applicability rule as Rel-16 HST between DPS 1a and DPS 1b can be used.

There is no any feasibility issue for DPS transmission scheme 1b for both Bi-directional and Uni-directional deployment.

Define both DPS transmission scheme 1a and 1b for both Bi-directional and Uni-directional deployment, and same applicability rule as Rel-16 HST between DPS 1a and DPS 1b can be used..

Test Setup

CBW

- CBW - Option 1: 100MHz CBW - Option 2: 200MHz CBW - Option 3: Align the CBW configuration for PDSCH and PUSCH

Typically, CPE is serving for all users in the train, it is reasonable to provide higher throughput than the normal UE. So we prefer to use 200MHz for PDSCH tests under FR2 HST scenario.

Use 200MHz for PDSCH tests under FR2 HST scenario.

Simulations

Here we provide the simulation results for FR2 HST PDSCH for alignment.

Table 3-1 Ideal simulation results for PDSCH for HST FR2

| Case number | CBW(MHz) | MCS | SNR (dB) |
| --- | --- | --- | --- |
| 1 | 100 | 17 | 10.24 |
| 2 | 200 | 17 | 10.35 |

Figure 3-1 Ideal simulation results for PDSCH for HST FR2

There is negligible performance difference between the case with 100MHz CBW and 200MHz CBW.

Proposals

In this contribution, we discuss on demodulation performance for NR UE HST FR2. Our observations and proposals are:

- It is feasible to use SSB+TRS for tracking frequency offset for downlink to support 350km/h, using SSB for coarse frequency offset estimation and TRS for precise frequency offset estimation.

- There is no any feasibility issue for DPS transmission scheme 1b for both Bi-directional and Uni-directional deployment.

- There is negligible performance difference between the case with 100MHz CBW and 200MHz CBW.

- Define 350km/h requirements, i.e. Doppler of 9722Hz targeting 350km/h at 30GHz for both Bi-directional and Uni-directional RRH deployment.

Define requirements for both scenario A/B and uni/bi-directional deployment, and not define any applicability rule between them.

Define both DPS transmission scheme 1a and 1b for both Bi-directional and Uni-directional deployment, and same applicability rule as Rel-16 HST between DPS 1a and DPS 1b can be used..

Use 200MHz for PDSCH tests under FR2 HST scenario.

Reference

R4-2108637, WF on Demodulation requirement for FR2 HST, RAN4#99-e, Samsung