TSG-RAN Working Group 4 (Radio) meeting #100-e	R4-2113952

Electronic Meeting, 16th – 26th August 2021

Source:	Ericsson

Title:	System and Link Simulation Results for DL 1024 QAM

Agenda item:	9.6.2.1

Document for:	Approval

# 1	Introduction

During RAN4#99-e an agreement on further link and system simulation assumptions were made in order to fine tune the needed simulation permutations for RAN4#100-e meeting.  In this contribution the updated results are presented for both system and link simulations.

Following the results, Section 2.3, provided is the author’s analysis on the results and the proposed outcome on BS EVM requirement level and the BS class aspects.  During past RAN4 meetings there were some companies concerned that wide area base stations should not be introduced due to the lack of gain or benefit seen by introducing the higher modulation (1024 QAM), the simulation results presented in this contribution illustrate the advantage for which all BS classes would benefit.

# 2	Discussion

In previous RAN4 meeting system simulations were discussed as an additional means to help contribute to the decision on BS class applicability.  With the main question of how many users would benefit or see gain from the addition of a higher order modulation, 1024 QAM.  Unlike FR2 the FR1 benefits from channel conditions which lends itself to larger coverage realizations, as such the wide area deployment option should not be excluded.

## 2.1	Link Level Simulation Assumptions and Results

The following table has been adopted from [3] to incorporate parameters identified as needing more consideration for BS EVM requirement.

Table 2.1-1 link level simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2 GHz |
| CBW | 40 MHz |
| SCS | 15kHz |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 10ns delay spread, Maximum Doppler frequency: 5Hz TDL-D 10ns delay spread, Maximum Doppler frequency: 5Hz |
| MCS | 256QAM: MCS 24 in TS 38.214 Table 5.1.3.1-2: MCS index table 2 for PDSCH, and other MCSs are not precluded 1024QAM: MCS 24 in the following Table accroding to the agreement in RAN1 #104, and other MCSs are not precluded MCS Index; Modulation Order; Target code Rate R x [1024]; Spectral. IMCS; Qm; efficiency. 23; 10; 805.5; 7.8662. 24; 10; 853; 8.3301. 25; 10; 900.5; 8.7939. 26; 10; 948; 9.2578. |
| Precoding | Precoding configuration defined in 38.101-4 Section 7.2 for fading channels; follow PMI |
| Symbol type | CP-OFDM |
| HARQ | 8 processes, maximum 4 transmissions |
| RV sequence | {0, 2, 3, 1} |
| RANK | 1 and 2 |
| BS antenna configuration | 1 and 2 |
| UE antenna configuration | 4 |
| Antenna correlation (Tx and Rx) | Low correlation |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PDSCH configuration | Type A mapping, Start symbol 1, Duration 13 (for D slots) |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| txEVM | 2%, 2.5%, 3% |
| rxEVM | 0.5%, 1%, 1.5%, 2%, 3% |

Based upon the above parameters, the following initial results below for rank 1. Black curves correspond to 256QAM MCS24. Note this simulation does not assume UE Rx EVM, that is RxEVM 0%. We also ran the simulation with rank 2, but the peak rate is not reached even with 256QAM MCS24 and TxEVM 2% for the multi-path fading condition such as TDL-A or TDL-D.

| (a) TDLA10 5Hz | (b) TDLD10 5Hz |
| --- | --- |

Figure 1	1Tx Rank 1.

| (a) TDLA10 5Hz | (b) TDLD10 5Hz |
| --- | --- |

Figure 2	2Tx, Rank 1.

## 2.2	System Level Assumptions and Results

As agreed WF [1] states the assumptions of the system simulations are based upon R4-2110664 [2].  The UE distribution has been selected of 100% outdoor in order to fully understand the impact and gain on introducing WA BS class.  Additionally, results have also considered full buffer.

## 2.3	Simulation Result Analysis

The following is a summary table the corresponding DL SNR is selected from the system level simulations are then related to the most optimal MCS at each SNR from the link simulations where channel conditions (either TDL-A or TDL-D) where chosen for best MCS with highest throughput.

The table combines link and system simulation results and shows that 40% of users experience a DL SINR of at least 25 dB,  which has ability to sustain a link utilizing 1024 QAM (either 1 or 2 Tx).  At this DL SINR a link of 256 QAM may also be a possibility depending on the link level channel conditions.  However, for almost 20% of users the DL SINR is >30dB and  1024 QAM provides gains compared to 256 QAM.

Table 1: Percentage most optimal MCS of users

| CDF [%] | DL SINR | 1 Tx, Rank 1 | 2 Tx, Rank 1 |
| --- | --- | --- | --- |
| 60 | 20 | 256 QAM (MCS24, EVM 2.5%) | 256 QAM (MCS24, EVM 2.5%) |
| 40 | 25 | 256 QAM (MCS23, EVM 2.5%) 1024 QAM (MCS23, EVM 2%) | 1024 QAM (MCS23, EVM 2.5%) |
| 20 | 30 | 1024 QAM (MCS25, EVM 2%) 1024 QAM (MCS24, EVM 2.5%) | 1024 QAM (MCS25, EVM 2.5%) |
| 10 | 35 | 1024 QAM (MCS25, EVM 2.5%) 1024 QAM (MCS24, EVM 2%) | 1024 QAM (MCS26, EVM 2%) |
| 5 | 40 | 1024 QAM (MCS26, EVM 2%) | 1024 QAM (MCS26, EVM 2.5%) |

Observation: Analysis shows that at least 20% of users can benefit from 1024 QAM even in WA BS class.

Proposal 1: Wide Area BS should not be excluded from introduction of DL 1024 QAM in NR FR1.

# 3	Conclusion

In this contribution the following key observations and proposals were made based upon analysis.

Observation: Analysis shows that at least 20% of users can benefit from 1024 QAM even in WA BS class.

Proposal 1: Wide Area BS should not be excluded from introduction of DL 1024 QAM in NR FR1.

# 4	References

- R4-2108634, “WF on BS requirements, class applicability and system simulations”, Moderator (Ericsson)

- R4-2110664, “BS EVM requirements for support of 1024 QAM”, Huawei, HiSilicon, CMCC, China Unicom

- R4-2108077, “WF on Link level simulation assumptions”, CATT.