3GPP TSG-RAN WG4 Meeting # 100-e 	    R4-2114045

Electronic Meeting 16th August – 27th August 2021

Agenda Item:	6.1				NR Positioning Support
6.1.6.2.3.2		SRS-RSRP requirements

Source: 	Ericsson

Title:	SRS-RSRP requirements

Document for:	Discussion

# Introduction

- SRS BW grouping for defining SRS-RSRP accuracy requirements

  - FR1

    - 24 ≤  BW < 48 (requirements will be defined for Ês/Iot ≥ 3dB only)

    - 48 ≤  BW < 132

    - 132 ≤ BW

  - FR2

    - 32 ≤  BW < 64 (requirements will be defined for Ês/Iot ≥ 3dB only)

    - 64 ≤  BW < 132

    - 132 ≤ BW

As discussed in RAN4#99-e, bandwidth grouping in terms of RBs to define SRS-RSRP measurement accuracy requirements were agreed [1].

- RF calibration margin differs between gNB type 1-C and other gNB types:

  - X= [2.5] dB for gNB type 1-C

  - X= [4] dB for gNB type 1-H, 1-O and 2-O

Furthermore, besides the addition of RF margin, where [2.5]dB and [4]dB are to be added on top of simulation results accuracies for type 1-C and 1-H, 1-O and 2-O respectively, the differences in simulation results across companies per SRS configuration are to be handled in terms of an additional margin which is dependent on the standard deviation across companies’ simulation results for said SRS configurations.

- SRS-RSRP average (SRS-RSRPavg) without RF margin used for baseline accuracy requirements is derived as follows:

  - SRS-RSRPavg = [ceil(2*avg(CDF90(SRS-RSRP)))/2] + 0.5 dB (If std dev > 0.25 dB)

# Discussion SRS-RSRP requirements analysis

To derive measurement accuracy requirements for gNB Rx-Tx, further simulations were conducted for which the simulation data points, e.g. bandwidth in units of RB differed from simulations that already were performed. Since the new data points do not contribute to the definition of SRS-RSRP measurement accuracy requirements due to the bandwidth grouping specifically for SRS-RSRP measurement accuracy requirements and no outliers could be identified from the new data points, it is proposed to keep the baseline values as agreed last meeting.

Simulation results collected in RAN4#99-e can be used to define SRS-RSRP measurement accuracy requirements, since no outliers can be identified in the further simulation results

Furthermore, it is proposed to add the RF margins as proposed in last meetings, namely:

- X= 2.5 dB for gNB type 1-C

- X= 4 dB for gNB type 1-H, 1-O and 2-O

RF calibration margin differs between gNB type 1-C and other gNB types:

- X= 2.5 dB for gNB type 1-C

- X= 4 dB for gNB type 1-H, 1-O and 2-O

This leads to SRS-RSRP measurement accuracy requirements as follows:

Table 13.3.2.2-1 gNB SRS-RSRP absolute accuracy requirements in FR1 for gNB type 1-C

| Accuracy | Conditions |  |
| --- | --- | --- |
|  | SRS Ês/Iot | SRS bandwidth range |
| dB | dB | RB |
| ±4 | Ês/Iot ≥ +3 | 24 ≤ BW < 48 |
| ±4 |  | 48 ≤ BW < 132 |
| ±4 |  | 132 ≤ BW |
| ±6.5 | Ês/Iot ≥ -13 | 48 ≤ BW < 132 |
| ±5.5 |  | 132 ≤ BW |

Table 13.3.2.2-2 gNB SRS-RSRP absolute accuracy requirements in FR1 for gNB type 1-H and 1-O

| Accuracy | Conditions |  |
| --- | --- | --- |
|  | SRS Ês/Iot | SRS bandwidth range |
| dB | dB | RB |
| ±5.5 | Ês/Iot ≥ +3 | 24 ≤ BW < 48 |
| ±5.5 |  | 48 ≤ BW < 132 |
| ±5.5 |  | 132 ≤ BW |
| ±8 | Ês/Iot ≥ -13 | 48 ≤ BW < 132 |
| ±7 |  | 132 ≤ BW |

Table 13.3.2.2-3 gNB SRS-RSRP absolute accuracy requirements in FR2 for gNB type 2-O

| Accuracy | Conditions |  |
| --- | --- | --- |
|  | SRS Ês/Iot | SRS bandwidth range |
| dB | dB | RB |
| ±5.5 | Ês/Iot ≥ +3 | 32 ≤ BW < 64 |
| ±5.5 |  | 64 ≤ BW < 132 |
| ±5.5 |  | 132 ≤ BW |
| ±8 | Ês/Iot ≥ -13 | 64 ≤ BW < 132 |
| ±7 |  | 132 ≤ BW |

This implementation of RF margin values and additional margin to cope with standard deviation across companies’ simulation results per SRS configuration, as shown in above tables is also depicted in [2].

# Conclusion

- Simulation results collected in RAN4#99-e can be used to define SRS-RSRP measurement accuracy requirements, since no outliers can be identified in the further simulation results

- RF calibration margin differs between gNB type 1-C and other gNB types:

- X= 2.5 dB for gNB type 1-C

- X= 4 dB for gNB type 1-H, 1-O and 2-O

Table 13.3.2.2-1 gNB SRS-RSRP absolute accuracy requirements in FR1 for gNB type 1-C

| Accuracy | Conditions |  |
| --- | --- | --- |
|  | SRS Ês/Iot | SRS bandwidth range |
| dB | dB | RB |
| ±4 | Ês/Iot ≥ +3 | 24 ≤ BW < 48 |
| ±4 |  | 48 ≤ BW < 132 |
| ±4 |  | 132 ≤ BW |
| ±6.5 | Ês/Iot ≥ -13 | 48 ≤ BW < 132 |
| ±5.5 |  | 132 ≤ BW |

Table 13.3.2.2-2 gNB SRS-RSRP absolute accuracy requirements in FR1 for gNB type 1-H and 1-O

| Accuracy | Conditions |  |
| --- | --- | --- |
|  | SRS Ês/Iot | SRS bandwidth range |
| dB | dB | RB |
| ±5.5 | Ês/Iot ≥ +3 | 24 ≤ BW < 48 |
| ±5.5 |  | 48 ≤ BW < 132 |
| ±5.5 |  | 132 ≤ BW |
| ±8 | Ês/Iot ≥ -13 | 48 ≤ BW < 132 |
| ±7 |  | 132 ≤ BW |

Table 13.3.2.2-3 gNB SRS-RSRP absolute accuracy requirements in FR2 for gNB type 2-O

| Accuracy | Conditions |  |
| --- | --- | --- |
|  | SRS Ês/Iot | SRS bandwidth range |
| dB | dB | RB |
| ±5.5 | Ês/Iot ≥ +3 | 32 ≤ BW < 64 |
| ±5.5 |  | 64 ≤ BW < 132 |
| ±5.5 |  | 132 ≤ BW |
| ±8 | Ês/Iot ≥ -13 | 64 ≤ BW < 132 |
| ±7 |  | 132 ≤ BW |

# References

- R4-2108314		“WF on gNB positioning measurement requirements”, Ericsson

- R4-2114048		“Corrections to gNB SRS-RSRP measurement in 38.133”, Ericsson