3GPP TSG-RAN WG4 Meeting #100-e	R4-2114202

Online Meeting, Aug 16 – 27, 2021

Agenda item:	8.26.2.2

Source: 	Qualcomm Incorporated

Title: 	Introducing NR-U 100 MHz carrier bandwidth in bands n46 and n96

Document for:	Discussion

# Introduction

A revised basket WID for adding support of new channel bandwidths to existing NR bands was approved in RAN#92-e meeting [1]. The revised WID includes the introduction of NR-U 100MHz carrier bandwidth (CBW) in bands n46 and n96, captured in table 4.1.3-1 therein and partially reproduced below.

In this brief contribution we will discuss channel rasters for the new CBW in both bands.

| Band | Channel bandwidth and SCS | Contact name, company | Contact email | Other supporting companies | Justification | Additional information | status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n46 | 100MHz (30-60 kHz) | Gene Fong, Qualcomm | gfong@qti.qualcomm.com | Ericsson, Qualcomm, Charter Communications Inc., Skyworks, HPE, Nokia, Cablelabs | 100MHz bandwidth is added to improve shared spectrum throughput and utillization | The same importance shall be given to ensuring co-existence with other technologies (e.g. WiFi) as for other channel bandwidths, taking into account the regulatory requirements that apply in different regions. | New |
| n96 | 100MHz (30-60 kHz) | Gene Fong, Qualcomm | gfong@qti.qualcomm.com | Ericsson, Qualcomm, Charter Communications Inc., Skyworks, HPE, Nokia, Cablelabs | 100MHz bandwidth is added to improve shared spectrum throughput and utillization | The same importance shall be given to ensuring co-existence with other technologies (e.g. WiFi) as for other channel bandwidths, taking into account the regulatory requirements that apply in different regions. | New |

# Channel raster for 100 MHz carrier bandwidth

Coexistence between NR-U and other technologies such as Wifi should be a key consideration in the formulation of a channel raster for 100 MHz CBW. In RAN#92-e, a note was added to the WID [1] stating that when adding 100 MHz CBW to n46 and n96 “the same importance shall be given to ensuring co-existence with other technologies (e.g. WiFi) as for other channel bandwidths, taking into account the regulatory requirements that apply in different regions.” In this respect, the considerations applied to the channel raster of NR-U 60 MHz CBW provides a good reference, since 60 MHz CBW is not supported by WiFi and there both smaller and larger CBW supported by Wifi, as in the case of NR-U 100 MHz.

Similar to the channel raster for NR-U 60 MHz CBW (see Appendix), the channel raster for NR-U 100 MHz CBW should ensure that each channel overlaps with no more than five 20 MHz Wifi channels. In some cases, it may be possible to place channels adjacent to the band edge, so that they overlap with only four 20 MHz Wifi channels. Otherwise, inevitably, a 100 MHz NR-U carrier will have partial overlap with some 40 MHz and 80 MHz Wifi channels. The number of impacted Wifi carriers impacted by a 100 MHz NR-U carrier should be limited to three 40 MHz, two 80 MHz and one 160 MHz carrier.

Even though partial overlap with multiple Wifi channels cannot be avoided in all cases, that does not necessarily imply a coexistence issue. Note that by using type A multi-channel access, NR-U transceivers are capable of accessing only 20 MHz sub-channels that are not occupied by other users, thereby ensuring fair access to the media by other wireless technologies, including Wifi.

Channel rasters for NR-U 100 MHz CBW in band n46 have been proposed in previous RAN4 meetings [2,3,4]. Our proposed channel raster for n46, shown in Table 1, is closer to the proposals in [2,3].

In [4], a rather limited channel raster for NR-U 100 MHz was proposed to try to avoid partial overlap with Wifi channels with CBW larger than 20 MHz. However, this was accomplished by relegating NR-U 100 MHz to band-edge channels. In our view, such an approach would not be fair because band-edge channels are burdened with more strict emissions requirements, which would require very high A-MPR.

To reiterate, coexistence issues should be addressed effectively by means of efficient/flexible channel access procedures and equitable spectral emissions requirements. As noted before, NR-U supports multi-channel access procedures to facilitate coexistence with other wireless technologies. Additionally, in some deployment scenarios of interest for NR-U, e.g. private networks, coexistence considerations would be non-existent or may be easily avoided. To enable such scenarios, NR-U supports semi-static channel access procedures “intended for environments where the absence of other technologies is guaranteed e.g., by level of regulations, private premises policies” [5]. Adopting an overly conservative approach that statically limits access to unlicensed spectrum by NR-U would be even less justifiable in such scenarios.

Figure 1: Proposed channels for NR-U 100 MHz in U-NII-1 and U-NII-2A.

Figure 2: Proposed channels for NR-U 100 MHz in U-NII-2C.

Figure 3: Proposed channels for NR-U 100 MHz in U-NII-3 and U-NII-4.

| Nominal channel freq (MHz) | NREF (NR-ARFCN) | FREF (MHz) |
| --- | --- | --- |
| 5200 | 746668 | 5200.02 |
| 5220 | 748000 | 5220.00 |
| 5240 | 749332 | 5239.98 |
| 5260 | 750668 | 5260.02 |
| 5280 | 752000 | 5280.00 |
| 5300 | 753332 | 5299.98 |
| 5520 | 768000 | 5520.00 |
| 5540 | 769332 | 5539.98 |
| 5560 | 770668 | 5560.02 |
| 5580 | 772000 | 5580.00 |
| 5600 | 773332 | 5599.98 |
| 5620 | 774668 | 5620.02 |
| 6540 | 776000 | 5640.00 |
| 5660 | 777332 | 5659.98 |
| 5680 | 778668 | 5680.02 |
| 5785 | 785668 | 5785.02 |
| 5805 | 787000 | 5805.00 |
| 5825 | 788332 | 5824.98 |
| 5845 | 789668 | 5845.02 |
| 5865 | 791000 | 5865.00 |

Table 1: Channel raster for NRU 100 MHz in band n46

Proposal 1: Adopt the channel raster in Table 1 for NR-U 100 MHz CBW in band n46.

Similarly, for band n96 we propose a channel raster following the same channel alignment constraints with respect to Wifi channels. The proposed channel raster for NR-U 100 MHz in band n96 is shown in Table 2. Wifi channels for CBW=160 MHz are assumed to have center frequencies at 6025 MHz, 6185 MHz, 6345 MHz, 6505 MHz, 6665 MHz, 6825 MHz, and 6985 MHz.

| Nominal channel freq (MHz) | NREF (NR-ARFCN) | FREF (MHz) |
| --- | --- | --- |
| 5995 | 799668 | 5995.02 |
| 6015 | 801000 | 6015.00 |
| 6035 | 802332 | 6034.98 |
| 6055 | 803668 | 6055.02 |
| 6155 | 810332 | 6154.98 |
| 6175 | 811668 | 6175.02 |
| 6195 | 813000 | 6195.00 |
| 6215 | 814332 | 6214.98 |
| 6315 | 821000 | 6315.00 |
| 6335 | 822332 | 6334.98 |
| 6355 | 823668 | 6355.02 |
| 6375 | 825000 | 6375.00 |
| 6475 | 831668 | 6475.02 |
| 6495 | 833000 | 6495.00 |
| 6515 | 834332 | 6514.98 |
| 6535 | 835668 | 6535.02 |
| 6635 | 842332 | 6634.98 |
| 6655 | 843668 | 6655.02 |
| 6675 | 845000 | 6675.00 |
| 6695 | 846332 | 6694.98 |
| 6795 | 853000 | 6795.00 |
| 6815 | 854332 | 6814.98 |
| 6835 | 855668 | 6835.02 |
| 6855 | 857000 | 6855.00 |
| 6955 | 863668 | 6955.02 |
| 6975 | 865000 | 6975.00 |
| 6995 | 866332 | 6994.98 |
| 7015 | 867668 | 7015.02 |
| 7035 | 869000 | 7035.00 |
| 7055 | 870332 | 7054.98 |
| 7075 | 871668 | 7075.02 |

Table 2: Channel raster for NRU 100 MHz in band n96

Proposal 2: Adopt the channel raster in Table 2 for NR-U 100 MHz CBW in band n96.

# Conclusions

Proposal 1: Adopt the channel raster in Table 1 for NR-U 100 MHz CBW in band n46.

Proposal 2: Adopt the channel raster in Table 2 for NR-U 100 MHz CBW in band n96.

# Appendix

Excerpt from TS 38.101-1 clause 5.4.2 for reference.

Table 5.4.2.3-2: Allowed NREF (NR-ARFCN) for operation in Band n46

| Channel Bandwidth | Allowed NREF |
| --- | --- |
| 10 MHz | 782000, 788668 |
| 20 MHz | 744000, 745332, 746668, 748000, 749332, 750668, 752000, 753332, 754668, 756000, 765332, 766668, 768000, 769332, 770668, 772000, 773332, 774668, 776000, 777332, 778668, 780000, 781332, 783000, 784332, 785668, 787000, 788332, 789668, 791000, 792332, 793668 |
| 40 MHz | 744668, 746000, 748668, 751332, 754000, 755332, 766000, 767332, 770000, 772668, 775332, 778000, 780668, 783668, 786332, 787668, 790332, 793000 |
| 60 MHz | 745332, 746668, 748000, 752000, 753332, 754668, 766668, 768000, 769332, 773332, 774668, 778668, 780000, 784332, 785668, 787000, 791000, 792332 |
| 80 MHz | 746000, 747332, 752668, 754000, 767332, 768668, 774000, 779332, 785000, 786332, 791668 |
| NOTE: 10 MHz channel bandwidth shall only apply in certain regions where the absence of non 3GPP technologies can be guaranteed on a long-term basis in this version of specification. |  |

Table 5.4.2.3-3: Allowed NREF (NR-ARFCN) for operation in Band n96

| Channel Bandwidth | Allowed NREF |
| --- | --- |
| 20 MHz | 797000, 798332, 799668, 801000, 802332, 803668, 805000, 806332, 807668, 809000, 810332, 811668, 813000, 814332, 815668, 817000, 818332, 819668, 821000, 822332, 823668, 825000, 826332, 827668, 829000, 830332, 831668, 833000, 834332, 835668, 837000, 838332, 839668, 841000, 842332, 843668, 845000, 846332, 847668, 849000, 850332, 851668, 853000, 854332, 855668, 857000, 858332, 859668, 861000, 862332, 863668, 865000, 866332, 867668, 869000, 870332, 871668, 873000, 874332 |
| 40 MHz | 797668, 800332, 803000, 805668, 808332, 811000, 813668, 816332, 819000, 821668, 824332, 827000, 829668, 832332, 835000, 837668, 840332, 843000, 845668, 848332, 851000, 853668, 856332, 859000, 861668, 864332, 867000, 869668, 872332 |
| 60 MHz | 798332, 799668, 803668, 805000, 809000, 810332, 814332, 815668, 819668, 821000, 825000, 826332, 830332, 831668, 835668, 837000, 841000, 842332, 846332, 847668, 851668, 853000, 857000, 858332, 862332, 863668, 867668, 869000, 873000 |
| 80 MHz | 799000, 804332, 809668, 815000, 820332, 825668, 831000, 836332, 841668, 847000, 852332, 857668, 863000, 868332 |

# References

[1] RP-211599, Revised Basket WID on adding channel bandwidth support to existing NR bands

[2] R4-2007323, Further considerations on 100MHz CBW in NR-U, Huawei/HiSilicon

[3] R4-20074717, Discussion on 100MHz for NR-U, ZTE

[4] R4-2104484, Co-existence challenges with NR-U 100MHz channel bandwidth and other technologies, Charter Communications, Cable Labs.

[5] 3GPP TS 37.213, clause 4.3