3GPP TSG-RAN WG4 Meeting #100-e 	R4-2114226

Online, 16 - 27 Aug 2021

Title:	Response LS to MSG TFES

Response to:	LS- New blocking requirement for Band 1 BSs for protection from RMR in 1900-1910

Work Item:	-

Source:	3GPP RAN WG4

To:	MSG TFES

Cc:	3GPP TSG RAN

Contact Person:

Name:	Richard Kybett

Tel. Number:

E-mail Address:	richard.kybett@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

3GPP RAN4 would like to thank MSG TFES for the LS on “New blocking requirement for Band 1 BSs for protection from RMR in 1900-1910”

In the LS MSG TFES has requested feedback from RAN4 on 4 questions in the context of an additional sensitivity requirement for band 1 BS with regards a 10MHz interferer in 1900 – 1910 MHz.

RAN4 provides the following response to the questions:

- Is a 10 MHz EUTRA channel supported by current signal generators in the labs?

Yes.

Existing RAN4 specifications and the defined test configurations require 10MHz modulated signals for some other requirements and as such signal generators in labs are capable of generating 10MHz channel BW signals.

- Does a 10 MHz EUTRA interferer definition differ significantly from the current 5 MHz EUTRA interferer definition in TS36.104 clause 7.6.1.1 specifically table 7.6.1.1-2?

No, table 7.6.1.1-2 specified the interferer channel bandwidth and annex C (of the same TS (3GPP 36.141) specified the interferer definition.

For E-UTRA or E-UTRA with NB-IoT (in-band and/or guard band operation) BS, the interfering signal shall be a PUSCH containing data and reference symbols. Normal cyclic prefix is used. The data content shall be uncorrelated to the wanted signal and modulated according to clause 5 of TS36.211. Mapping of PUSCH modulation to receiver requirement are specified in table C-1.1.

Table C-1: Modulation of the interfering signal

| Receiver requirement | Modulation |
| --- | --- |
| In-channel selectivity | 16QAM |
| Adjacent channel selectivity and narrow-band blocking | QPSK |
| Blocking | QPSK |
| Receiver intermodulation | QPSK |

This definition can be applied to any E-UTRA channel BW so simply defining the interferer channel BW as 10MHz is sufficient.

- Is the measurement uncertainty for a test with -30 dBm interferer and 1 dB desens (option 1) different from the measurement uncertainty for a test with -19.5 dBm and 6 dB desens
(option 2) for a test of blocking performance?

The RAN 4 measurement uncertainty calculation for in-band blocking below 3Gz is as follows:

MU=MUWanted2+MUInterfer2+ACLR effect

MU=0.72+12+0.4=1.6dB

Applying this methodology both option 1 and option 2 would have the same MU as the existing in-band blocking requirement. Although the higher interferer level in option 2 may present additional test set up challenges in order to maintain the same interferer accuracy and ACLR effect contributions.

- Does the new requirement present a testing issue compared to existing blocking performance requirements?

No additional testing concerns have been identified.

3GPP RAN4 would like to also inform MFG/TFES that a detailed background of MU calculations for the BS radiated requirements is documented in the TR 37.941 “Radio Frequency (RF) conformance testing background for radiated Base Station (BS) requirements”. Whilst focussed on the radiated requirements this document also offers insight into how the conducted requirements were derived.

2. Actions:

None.

3. Date of Next TSG-RAN4 Meetings:

TSG-RAN4 Meeting #101-e	November 01 – 12, 2021	Online

TSG-RAN4 Meeting #102	February 21 – 25, 2022		Athens, Greece