3GPP TSG-RAN WG4 Meeting # 100-e 	R4-2114227

# Electronic Meeting, 16 –  27 Aug, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.176-1 | CR |  | rev |  | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR to TS 38.176-1 - Corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Perf |  |  |  |  |  | Date: |  |  | 2021-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Errors were found in the text. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Term BS still existsi in some coppied text where it should be IAB as follows: Table 4.1.2.2-1 BS chanel BW should be IAB channel BW 6.3.1.2.1 – BS should be IAB-DU 6.3.1.3.1 – BS should be IAB-DU 6.3.1.3.4.2 – step 3) BS should be IAB-DU Table C.1-1, the note covering 4.2-6GHz should be note 1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements are not clear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.2.2, 6.3.1.2.1, 6.3.1.3.1, 6.3.1.3.4.2, C.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

#### 4.1.2.2	Measurement of transmitter

Table 4.1.2.2-1: Maximum Test System uncertainty for transmitter tests

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2 IAB output power | ±0.7 dB, f ≤ 3 GHz ±1.0 dB, 3 GHz < f ≤ 6 GHz (Note) |  |
| 6.3.1 IAB-DU Output power dynamics | ± 0.4 dB |  |
| 6.3.2 IAB-MT Output power dynamics | ±0.7 dB, BW ≤ 40MHz ±1.0 dB, 40MHz < f ≤ 100MHz |  |
| 6.4.1 Transmit OFF power | ±2.0 dB, f ≤ 3 GHz ±2.5 dB, 3 GHz < f ≤ 6 GHz (Note) |  |
| 6.4.2 Transmitter transient period | N/A |  |
| 6.5.2.1 IAB-DU Frequency error | ± 12 Hz |  |
| 6.5.2.2 IAB-MT Frequency error | ±15 Hz, f ≤ 3.0GHz ±36 Hz, f > 3.0GHz |  |
| 6.5.3 EVM | ± 1% |  |
| 6.5.4 Time alignment error | ± 25 ns |  |
| 6.6.2 Occupied bandwidth | 10 MHz IAB Channel BW: ±100 kHz 15 MHz, 20 MHz, 25 MHz, 30 MHz, 40 MHz, 50 MHz IAB Channel BW: ±300 kHz 60 MHz, 70 MHz, 80 MHz, 90 MHz, 100 MHz IAB Channel BW: ±600 kHz |  |
| 6.6.3 Adjacent Channel Leakage power Ratio (ACLR) | ACLR/ CACLR BW ≤ 20MHz: ±0.8 dB BW > 20MHz: ±1.2 dB Absolute power ±2.0 dB, f ≤ 3 GHz Absolute power ±2.5 dB, 3 GHz < f ≤ 6 GHz (Note) CACLR BW ≤ 20MHz: ±0.8 dB BW > 20MHz: ±1.2 dB CACLR absolute power ±2.0 dB, f ≤ 3 GHz CACLR absolute power ±2.5 dB, 3 GHz < f ≤ 6 GHz (Note) |  |
| 6.6.4 Operating band unwanted emissions | ±1.5 dB, f ≤ 3 GHz ±1.8 dB, 3 GHz < f ≤ 6 GHz (Note) |  |
| 6.6.5.5.1.1 Transmitter spurious emissions, Mandatory Requirements | 9 kHz < f ≤ 4 GHz: ±2.0 dB 4 GHz < f ≤ 19 GHz: ±4.0 dB 19 GHz < f ≤ 26 GHz: ±4.5 dB |  |
| 6.6.5.5.1.2 Transmitter spurious emissions, Additional spurious emission requirements | ±2.0 dB for > -60 dBm, f ≤ 3 GHz ±2.5 dB, 3 GHz < f ≤ 4.2 GHz ±3.0 dB, 4.2 GHz < f ≤ 6 GHz ±3.0 dB for ≤ -60 dBm, f ≤ 3 GHz ±3.5 dB, 3 GHz < f ≤ 4.2 GHz ±4.0 dB, 4.2 GHz < f ≤ 6 GHz |  |
| 6.6.5.2.3 Transmitter spurious emissions, Co-location | ±3.0 dB |  |
| 6.7 Transmitter intermodulation (interferer requirements) This tolerance applies to the stimulus and not the measurements defined in 6.6.3, 6.6.4 and 6.6.5 | The value below applies only to the interfering signal and is unrelated to the measurement uncertainty of the tests in 6.6.3 (ACLR), 6.6.4 (OBUE) and 6.6.5 (spurious emissions) which have to be carried out in the presence of the interferer. ±1.0 dB | The uncertainty of interferer has double the effect on the result due to the frequency offset |
| NOTE: Test system uncertainty values for 4.2 GHz < f ≤ 6 GHz apply for IAB operates in licensed spectrum only. |  |  |

------------------------------ Next Modified section ------------------------------

##### 6.3.1.2.1	Definition and applicability

The RE power control dynamic range is the difference between the power of an RE and the average RE power for an IAB-DU at maximum carrier output power (Pmax,c,TABC, or Pmax,c,AC) for a specified reference condition.

For IAB type 1-H this requirement shall apply at each TAB connector supporting transmission in the operating band.

------------------------------ Next Modified section ------------------------------

##### 6.3.1.3.1	Definition and applicability

The IAB-DU total power dynamic range is the difference between the maximum and the minimum transmit power of an OFDM symbol for a specified reference condition.

For IAB type 1-H this requirement shall apply at each TAB connector supporting transmission in the operating band.

NOTE:	The upper limit of the dynamic range is the OFDM symbol power for an IAB-DU when transmitting on all RBs at maximum output power. The lower limit of the total power dynamic range is the average power for single RB transmission. The OFDM symbol shall carry PDSCH and not contain RS or SSB.

------------------------------ Next Modified section ------------------------------

###### 6.3.1.3.4.2	Procedure

For IAB type 1-H where there may be multiple TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in annex D.1.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect the single-band connector(s) under test as shown in annex D.1.1 for IAB type 1-H. All connectors not under test shall be terminated.

2)	Set each connector under test to transmit according to the applicable test configuration in clause 4.8 using the corresponding test models in clause 4.9.2 at Prated,c,TABC for IAB type 1-H (D.21).

3)	For IAB type 1-H, set the IAB-DU to transmit a signal according to:

-	NR-FR1-TM3.1a if 256QAM is supported without power back off, or

-	NR-FR1-TM3.1 if 256QAM is supported with power back off, or

-	NR-FR1-TM3.1 if 256QAM is not supported by IAB-DU.

4)	Measure the OFDM symbol TX power (OSTP) as defined in the annex H.

5)	For IAB type 1-H, set to transmit a signal according to:

NR-FR1-TM2a if 256QAM is supported, or

NR-FR1-TM2 if 256QAM is not supported;

6)	Measure the OFDM symbol TX power (OSTP) as defined in the annex H.

In addition, for multi-band connectors, the following steps shall apply:

7)	For a multi-band connectors and single band tests, repeat the steps above per involved operating band where single band test configurations and test models shall apply with no carrier activated in the other operating band.

------------------------------ Next Modified section ------------------------------

# C.1	Measurement of transmitter

Table C.1-1: Derivation of test requirements (Transmitter tests)

| Test | Minimum requirement in TS 38.174 [2] | Test Tolerance (TT) | Test requirement in the present document |
| --- | --- | --- | --- |
| 6.2 IAB output power | See TS 38.174 [2], clause 6.2 | Normal and extreme conditions: 0.7 dB, f ≤ 3.0 GHz 1.0 dB, 3.0 GHz < f ≤ 6GHz (Note 1) | Formula: Upper limit + TT, Lower limit - TT |
| 6.3.1 IAB-DU Output power dynamics | See TS 38.174 [2], clause 6.3.1 | 0.4 dB | Formula: Total power dynamic range – TT (dB) |
| 6.3.2 IAB-MT Total power dynamic range | See TS 38.174 [2], clause 6.3.2 | ±0.7 dB, BW ≤ 40MHz ±1.0 dB, 40MHz < f ≤ 100MHz | Formula: Total power dynamic range – TT (dB) |
| 6.4.1 Transmitter OFF power | See TS 38.174 [2], clause 6.4.1 | 2.0 dB, f ≤ 3.0 GHz 2.5 dB, 3.0 GHz < f ≤ 6 GHz (Note 1) | Formula: Minimum Requirement + TT |
| 6.4.2 Transient period | See TS 38.174 [2], clause 6.4.2 | N/A |  |
| 6.5.2.1 IAB-DU Frequency error | See TS 38.174 [2], clause 6.5.1.1 | 12 Hz | Formula: Frequency Error limit + TT |
| 6.5.2.2 IAB-MT Frequency error | See TS 38.174 [2], clause 6.5.1.2 | ±15 Hz, f ≤ 3.0GHz ±36 Hz, f > 3.0GHz | Formula: Frequency Error limit + TT |
| 6.5.4 Modulation quality (EVM) | See TS 38.174 [2], clause 6.5.2 | 1% | Formula: EVM limit + TT |
| 6.5.4 Time alignment error | See TS 38.174 [2], clause 6.5.3.1 | 25ns | Formula: Time alignment error limit + TT+ TT |
| 6.6.2 Occupied bandwidth | See TS 38.174 [2], clause 6.6.2 | 0 Hz | Formula: Minimum Requirement + TT |
| 6.6.3 Adjacent Channel Leakage Power Ratio (ACLR) | See TS 38.174 [2], clause 6.6.3 | ACLR/CACLR: BW ≤ 20MHz: 0.8dB BW > 20MHz: 1.2 dB Absolute ACLR/CACLR: 0 dB | Formula: ACLR Minimum Requirement - TT Absolute limit +TT |
| 6.6.4 Operating band unwanted emissions | See TS 38.174 [2], clause 6.6.4 | Offsets < 10MHz 1.5 dB, f ≤ 3.0GHz 1.8 dB, 3.0GHz < f ≤ 6GHz (Note 1) Offsets ≥ 10MHz 0dB | Formula: Minimum Requirement + TT |
| 6.6.5.5.1 General transmitter spurious emissions requirements Category A | See TS 38.174 [2], clause 6.6.5.2.1 | 0dB | Formula: Minimum Requirement + TT |
| 6.6.5.5.1 General transmitter spurious emissions requirements Category B | See TS 38.174 [2], clause 6.6.5.2.1 | 0dB | Formula: Minimum Requirement + TT |
| 6.6.5.5.2 Additional spurious emissions requirements | See TS 38.174 [2], clause 6.6.5.2.2 | 0dB | Formula: Minimum Requirement + TT |
| 6.6.5.5.3 Co-location with other base stations | See TS 38.174 [2], clause 6.6.5.2.3 | 0dB | Formula: Minimum Requirement + TT |
| 6.7 Transmitter intermodulation | See TS 38.174 [2], clause 6.7 | 0dB | Formula: Ratio + TT |
| NOTE 1: TT values for 4.2 GHz < f ≤ 6.0 GHz apply for IAB operates in licensed spectrum only. NOTE 2: TT values are applicable for normal condition unless otherwise stated. |  |  |  |

------------------------------ Next Modified section ------------------------------

------------------------------ Next Modified section ------------------------------

----------------------------- End of modified section ------------------------------