3GPP TSG-RAN WG4 Meeting # 100-e	R4-2114412

# Electronic Meeting, August 16-27, 2021

Source: 	Huawei

Title: 	On the NTN bands numbering

Agenda Item:	9.13.1.4

Document for:	Discussion

# Introduction

During previous RAN4#99-e, a WF on general aspects of the NTN was agreed in [1]. One of the agreements was related to the NTN bands and their numbering (slide #13):

Proposal 2-1-1-1: The first NTN band will have the following frequency range definition: 1980-2010 MHz in UL and 2170-2200 MHz in DL. Its band number is FFS. Note: Companies are encouraged to provide a NTN band numbering scheme for next RAN4 meeting.

In this contribution we provide our views on the NTN bands numbering.

# Discussion

The NR bands are currently numbered separately for the FR1 and FR2 frequency ranges, as extracted below. FR2 bands numbering was decided to start from n267, in order to leave the range up to 256 (8 bits) for the future FR1 bands.

## 5.2 Operating bands NR is designed to operate in the operating bands defined in table 5.2-1 and 5.2-2. NB-IoT is designed to operate in the NR operating bands n1, n2, n3, n5, n7, n8, n12, n14, n18, n20, n25, n28, n41, n65, n66, n70, n71, n74, n90 which are defined in Table 5.2-1. Table 5.2-1: NR operating bands in FR1 NR operating band; Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high; Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high; Duplex mode. n1; 1920 MHz – 1980 MHz; 2110 MHz – 2170 MHz; FDD. n34; 2010 MHz – 2025 MHz; 2010 MHz – 2025 MHz; TDD. n65; 1920 MHz – 2010 MHz; 2110 MHz – 2200 MHz; FDD. n84; 1920 MHz – 1980 MHz; N/A; SUL. n951; 2010 MHz – 2025 MHz; N/A; SUL. n996; 1626.5 MHz -1660.5 MHz; N/A; SUL. Table 5.2-2: NR operating bands in FR2 NR operating band; Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL,low – FUL,high FDL,low – FDL,high; Duplex mode. n257; 26500 MHz – 29500 MHz; TDD. n258; 24250 MHz – 27500 MHz; TDD. n259; 39500 MHz – 43500 MHz; TDD. n260; 37000 MHz – 40000 MHz; TDD. n261; 27500 MHz – 28350 MHz; TDD. n262; 47200 MHz – 48200 MHz; TDD.

Furthermore, discussion on the NR bands numbering was also recently held in NR_ext_to_71GHz-Core WI (NR operation in 52.6 – 71 GHz range), with the conclusions captured in [2]. It was agreed not to specify any new band numbering range for the NR operation in 52.6 – 71 GHz range, and to reuse the reserved band numbers in FR2 for 60 GHz band.

In case of NTN band numbering, the following is observed:

- Number of future NTN bands is expected to be relative low,

- The NTN operation will reuse the NR radio interface, reusing the already specified SCS and channel bandwidths, i.e. there is no good technical reason to make specific distinction of the NTN operation in NR specifications,

- For the NTN channel bandwidths: those are expected to be introduced in a band-specific manner, following the existing approach in NR specifications.

Based on the above and considering the agreement on the FR1 band for NTN (i.e. 1980-2010 MHz in UL and 2170-2200 MHz in DL), the following is proposed:

Proposal 1: agree on the NTN bands numbering based on the following principles:

- No separate NR bands numbering range for NTN,

- NTN bands numbering to reuse the existing band numbering range for FR1 (and for FR2, if needed in future).

Proposal 2: The first NTN band to be allocated the next available FR1 band number, i.e. n100 (the number to be confirmed to avoid conflict with other spectrum work items).

It shall be noted, that the existing NR operating bands list captures various aspects, such as e.g. consideration of the NB-IoT (i.e. only selected NR bands are specified for the NB-IoT operation), as captured by the sentence above table 5.1-1. For the NTN, similar clarification may be considered.

Based on the above discussion, the possible NTN example bands implementation may look as in the example below:

## 5.2 Operating bands NR is designed to operate in the operating bands defined in table 5.2-1 and 5.2-2. NB-IoT is designed to operate in the NR operating bands n1, n2, n3, n5, n7, n8, n12, n14, n18, n20, n25, n28, n41, n65, n66, n70, n71, n74, n90 which are defined in Table 5.2-1. NTN is designed to operate in the NR operating band n100, [NTNband#2_n101], [NTNband#3_n263] which are defined in Table 5.2-1 [and Table 5.2-2]. Table 5.2-1: NR operating bands in FR1 NR operating band; Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high; Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high; Duplex mode. n1; 1920 MHz – 1980 MHz; 2110 MHz – 2170 MHz; FDD. n34; 2010 MHz – 2025 MHz; 2010 MHz – 2025 MHz; TDD. n65; 1920 MHz – 2010 MHz; 2110 MHz – 2200 MHz; FDD. n84; 1920 MHz – 1980 MHz; N/A; SUL. n951; 2010 MHz – 2025 MHz; N/A; SUL. n996; 1626.5 MHz -1660.5 MHz; N/A; SUL. n100; 1980 MHz – 2010 MHz; 2170 MHz – 2200 MHz; FDD. [NTNband#2_n101]; TBD; TBD; TBD. Table 5.2-2: NR operating bands in FR2 NR operating band; Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL,low – FUL,high FDL,low – FDL,high; Duplex mode. n257; 26500 MHz – 29500 MHz; TDD. n258; 24250 MHz – 27500 MHz; TDD. n259; 39500 MHz – 43500 MHz; TDD. n260; 37000 MHz – 40000 MHz; TDD. n261; 27500 MHz – 28350 MHz; TDD. n262; 47200 MHz – 48200 MHz; TDD. [NTNband#3_n263]; TBD; TDD.

# Conclusion

Based on the discussion, the following proposals are formulated:

Proposal 1: agree on the NTN bands numbering based on the following principles:

- No separate NR bands numbering range for NTN,

- NTN bands numbering to reuse the existing band numbering range for FR1 (and for FR2, if needed in future).

Proposal 2: The first NTN band to be allocated the next available FR1 band number, i.e. n100 (the number to be confirmed to avoid conflict with other spectrum work items).

# References

[1] 	R4-2100899	WF on [312] NTN_Solutions_Part1, (NTN general part), Moderator, THALES, RAN4#99-e

[2]	R4-2107876	WF on [145] NR_ext_to_71GHz Part1, Intel Corporation (Moderator), RAN4#99-e