3GPP TSG-RAN WG4 #100-e	R4-2114415

Electronic Meeting, August 16-27, 2021

Agenda item:	9.12.2.3

Source:	MediaTek Inc.

Title:	TP to TR 38.833: Scenario for LTE CRS interference handling for NR UE

Document for:	Approval

- Introduction

In this contribution, we propose TP for the scenario section of CRS interference handling in TR 38.833.

- Reference

- R4-2108662, “WF on CRS interference handling in scenarios with overlapping spectrum for LTE and NR”, China Telecom

- TP to TR 38.833

The following TP to TR 38.860 is proposed for approval.

<< Start of text proposal >>

### 5.1.1	Scenario

Dynamic spectrum sharing (DSS) provides a very useful migration path from LTE to NR. To help faster 5G rollout, the network operators can deploy 5G using their existing LTE frequency bands and base stations dynamically share the resources based on the traffic load. As the time and frequency resources are shared between NR UE and LTE UE, the core requirement for DSS is that existing essential channels of LTE, such as CRS, should be still transmitted for backward compatibility. The always-on CRS signals from interference cells will interfere NR UE.

To evaluate performance of different CRS interference handling schemes, two scenarios with overlapping spectrum for LTE and NR are considered below in Figure 1.

| Scenario 1: Serving and interference cells are both operating in DSS (NR+LTE) mode | Scenario 2: Serving cell is operating in NR mode and interference cell is operating in LTE mode |
| --- | --- |
| Figure 1. Scenarios for CRS interference in overlapping spectrum for LTE and NR |  |

<< End of text proposal >>