3GPP TSG-RAN4 Meeting #100-e		R4-214456

# Electronic Meeting, August 16-27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE Rx-Tx measurement requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To introduce missing or undefined Rx-Tx measurement requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following aspects of the Rx-Tx measurement requirements are corrected, added or updated: - PRS/SRS proximity condition - UE Rx-Tx measurement procedure under cell TA command change - UE Rx-Tx measurement procedure under cell NTA_offset change - UE Rx-Tx measurement procedure under cell change |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE Rx-Tx measurement requirement and UE behaviour will be undefined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGES----------------------------

#### 9.9.4.5 Measurement Period Requirements

When physical layer receives last of NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message from LMF via LPP [34], UE shall be able to measure multiple (up to the UE capability specified in clause 9.9.4.3) UE Rx-Tx time difference measurements as defined in TS 38.215 [4] in configured positioning frequency layers within the measurement period TUERxTx,Total ms.

TUERxTx, Total=i=1LTUERxTx,i+L-1*maxTeffect,i.

where i is the index of positioning frequency layer,

TUERxTx,i is the measurement period for UE Rx-Tx time difference measurements in positioning frequency layer i as further defined in this clause,

L is total number of positioning frequency layers, and

Teffect,i is the periodicity of the UE Rx-Tx time difference measurement in positioning frequency layer i as defined further in this clause.

TUERxTx,i=CSSFi*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast

Where

CSSFi is the carrier-specific scaling factor for NR PRS-based measurement in the positioning frequency layer i as defined in clause 9.1.5.2,

NRxBeam,i is the scaling factor for Rx beam sweeping, and NRxBeam,i=1 if positioning frequency layer i is in FR1 and NRxBeam,i=8 if positioning frequency layer i is in FR2,

Lavailable_PRS,i is the time duration of available PRS can be measured in the positioning frequency layer i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26].

NPRS,islot is the maximum number of DL PRS resources of positioning frequency layer i configured in a slot,

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSysmbols in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in clause 4.2.7.2 of TS 37.355 [34],

N’ is UE capability for number of DL PRS resources that it can process in a slot corresponding to maxNumOfDL-PRS-ResProcessedPerSlot as specified in clause 6.4.3  of TS 37.355 [34],

Nsample is the number of UE Rx-Tx time difference measurement samples and Nsample= [4],

Tlast is the measurement duration for the last UE Rx-Tx time difference measurement sample, including the sampling time and processing time, Tlast = Ti + Tavailable_PRS,i ,

Teffect,i is periodicity of UE Rx-Tx time difference measurement in positioning frequency layer i:

Teffect,i= TiTavailable_PRS,i*Tavailable_PRS,i

where

Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34],

Tavailable_PRS,i= LCMTPRS,i,MGRPi, the least common multiple between TPRS,i and MGRPi

MGRPi is the measurement gap repetition periodicity in positioning frequency layer i.

TPRS,i is the PRS resource periodicity in positioning frequency layer i. If the positioning frequency layer i has more than one DL PRS resource sets with different PRS periodicities with muting,  TperPRS with muting=Nmuting*TperPRS, the least common multiple of TperPRS with muting among DL PRS resource sets is used to derive the measurement period of that positioning frequency layer.

TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. If bitmap b1  for higher-layer parameter DL-PRS-MutingPattern is provided, and TperPRS*TmutingPRS ≤10240ms, then Nmuting=TmutingPRS*min(L,10240TperPRS*TmutingPRS); otherwise, if bitmap b1 is not provided or TperPRS*TmutingPRS>10240ms, then Nmuting=1. TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and L is the size of the bitmap b1.

Note: For the purpose of calculating TPRS,i, only the PRS resources fully or partially covered by the MG are considered.

The time TUERxTx,i starts from the first MG instance aligned with DL PRS resources of positioning frequency layer i closest in time after both the NR-Multi-RTT-RequestLocationInformation message and NR-Multi-RTT-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

Note: No per-positioning frequency layer requirement is applied in scenarios when multiple positioning frequency layers are configured.

The UE Rx-Tx time difference measurement is restarted if HO occurs during the measurement period and after SRS reconfiguration on the target cell is complete. The UE shall continue the ongoing UE Rx-Tx time difference measurement after the HO if the HO does not impact the SRS configuration. However, is this case the UE Rx-Tx time difference measurement period (TUERxTx,total,HO) shall be as follows:

TUERxTx,total,HO=TUERxTx,total+K*Teffect+THO

Where:

-	K is the number of times the handover occurs during TUERxTx,total,HO;

-	Teffect is the largest Teffect,i among all the positioning frequency layers;

-	THO is the time during which the UE Rx-Tx time difference measurement may not be possible due to handover; it can be up to Tinterrupt as defined in clause 6.1.

The measurement requirements do not apply for a PRS resource:

-	if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i or

-	if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

When PRS-RSRP is configured for multi-RTT, the UE Rx-Tx time difference measurements and PRS-RSRP measurements are performed over the same measurement period.

If the uplink transmission timing changes due to network-configured TA during the UE Rx-Tx measurement period then:

- The UE shall discard the UE Rx-Tx time difference measurement, and

- The UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If the NTA_offset defined in Table 7.1.2-2 changes changes during the UE Rx-Tx measurement period then:

- The UE shall discard the UE Rx-Tx time difference measurement, and

- The UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

The requirements in this clause apply provided that any SRS transmission is within [-160, 160] ms of at least one DL PRS resource of each of the TRPs in the NR-Multi-RTT-ProvideAssistanceData.

----------------------END OF CHANGES----------------------------