3GPP TSG-RAN WG4 Meeting # 100-e                                                         R4-2114758

Electronic Meeting, Aug. 16-Aug. 27, 2021

Source:	ZTE Corporation

Title:	TP for 36.717-04-01: CA_1A-7A-20A-38A

Agenda Item:	11.3.2

Document for:	Approval

- Introduction

This contribution provides a text proposal to 1 UL for DL CA_1A-7A-20A-38A for TR 36.717-04-01.

- Reference

- TR 36.717-04-01,LTE Advanced inter-band CA Rel-17 for x bands DL (x=4, 5) with 1 band UL in Rel-17, v0.4.0

- Text Proposal

----- Start of TP -----

  - 5.x		CA_1-7-20-38

    - 5.x.1	Channel bandwidths per operating band for CA

Table 5.x.1-1: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-7A-20A-38A | - | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

    - 5.x.2	∆TIB and ∆RIB values

Table 5.x.2-1: IB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-7-20-38 | 1 | 0.6 |
|  | 7 | 0.5 |
|  | 20 | 0.5 |
|  | 38 | 0.5 |

Table 5.x.2-2: R IB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-7-20-38 | 1 | 0 |
|  | 7 | 0 |
|  | 20 | 0 |
|  | 38 | 0.2 |

    - 5.x.3 	REFSENS requirements

No additional REFSENS / MSD are needed since it is already covered in fallback band combination.

----- End of TP -----