3GPP TSG-RAN WG4 Meeting #100-e                                R4-2114808

Online, 16th-27th Aug, 2021

Source:	Samsung, Verizon

Title:	TP for TR 37.717-21-11: DC_13-48_n77

Agenda item:			8.15.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.717-21-11 to include DC_13-48_n77 according to the request in [1].

2. Reference

- R4-2110666 WID revision: Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL).

- R4-2110665 TR 37.717-21-11 v0.5.0_DC of 2 LTE band and 1 NR band

# 3. Text Proposal

<Start of Text Proposal>

5.x	DC_13-48_n77

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_13A-48A_n77A DC_13A-48A-48A_n77A | DC_13A_n77A |

### 5.x.2	Co-existence studies

DC_48_n77 is not used alone, and no uplink inter-band EN-DC is specified for the configuration since the minimum requirements apply only when there is non-simultaneous Tx/Rx operation between E-UTRA B48 and NR n77 carriers (please refer to Rev 2 of R4-2112654 for DC_48_n77 this meeting). Thus no co-existence study is needed for TDD Band 48.

### 5.x.3	∆TIB and ∆RIB values

For DC_13-48_n77, the TIB,c and RIB,c values are specified in the tables below, reusing DC_13_n77 values.

Table 5.x.3-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13-48_n77 | 13 | 0.5 |
|  | 48 | 0.8 |
|  | n77 | 0.8 |

Table 5.x.3-2: ΔRIB due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13-48_n77 | 13 | 0.2 |
|  | 48 | 0.5 |
|  | n77 | 0.5 |

### 5.x.4	REFSENS requirements

Based on 5.x.2 discussion above, REFSENS exceptions are not expected.

<End of Text Proposal>