3GPP TSG RAN WG4 Meeting #100-e	 	R4-2114929

Electronic meeting, 16th – 27th August, 2021

Title:	LS on signaling for power class 1.5

Response to:

Release:	Release 17

Work Item:	High-power UE (power class 1.5) operation in NR bands n77 and n78 (HPUE_PC1_5_n77_n78)

Source:	[Qualcomm Incorporated]

To:	TSG RAN WG2

Cc:

Contact person:	Gene Fong

gfong<at>qti<dot>qualcomm<dot>com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

The objectives for the work item on high power UE (power class 1.5) operation in NR bands n77 and n78 (HPUE_PC1_5_n77_n78) as well as the work item on higher power UE PC1.5 in NR band n79 (NR_UE_PC1_5_n79) include the development of requirements for UE mobile device and fixed wireless access (FWA) device.  In the development of requirements for FWA, RAN4 agreed that there is a benefit to introduce a new maximum duty cycle capability to facilitate the FWA device to meet Maximum Permissible Exposure (MPE) requirements.

RAN4 envisions that this new capability would be analogous to the existing maxUplinkDutyCycle-PC2-FR1 but with  allowed set of values as 10%, 15%, 20%, 25%, 30%, 40%, 50%, 60%, 70%, 80%, 90%, and 100%.  In case the UE does not report the capability, it should be assumed that the UE will mitigate MPE autonomously by P-MPR or by other means and no restriction on scheduled uplink duty cycle is requested.  RAN4 suggests that this capability is per band, FR1 only, limited to devices supporting power class 1.5, with a possible name as maxUplinkDutyCycle-MPE-FR1.  RAN4 requests, if possible, this new capability be introduced as early as possible since power class 1.5 for Band n41 was already introduced in Release 16 and defined to be release independent to Release 15.

# 2	Actions

To TSG RAN WG2

ACTION: 	RAN4 respectfully requests RAN2 to define a new UE capability signaling element allowing the PC 1.5 UE to report its maximum uplink duty cycle capability for compliance with MPE.

# 3	Dates of next TSG RAN WG4 meetings

TSG RAN WG4 Meeting #101-e		November 1 – 12, 2021	          Electronic meeting

TSG RAN WG4 Meeting #102-e		February 21 – 25, 2022	          TBC