3GPP TSG-RAN WG4 Meeting #100-e	R4-2114961

E-meeting, 16th – 27th  Aug, 2021

Title:	WF on FR2 UL CA

Source:	Samsung

Agenda item:	9.4.2.2

Document for:	Approval

WF – PA-PA interaction

- how to incorporate PA-PA interaction

- GTW Agreement: Down-select to Option 1 and Option 2.

- Option 1: Included in CA MPR

- Option 2: Included in relaxations X and Y

- Option 3: No need to include.

- further check if FR1 approach can apply to FR2, i.e. Option 1 (or why FR2 approach can not apply to FR2, i.e. Option 2)

- FFS PA-PA interaction aspect

- Option 1: Only inter panel interaction is considered

- Option 2: Depends on activation status

- Option 3: Others (both or none, etc)

- if MPRPA-PA is defined, FFS how to modify MPR equation and how to apply MPRPA-PA (per band or per band combination)

WF – X and Y for CA_n257A_n259A

- X and Y for min peak EIRP.

  - Options

    - Option 1: X=Y=3.5 dB

    - Option 2: X=3.7 dB and Y=3.5 dB

    - Option 3: 1UL ΔTIB,P,n = MBP,n  and 2UL ΔTIB,P,n = X2/Y2+MBP,n

    - Option 4: ΔTIB,P,n shall be ΔRIB,P,n -1 dB

    - Option 5: X=Y=9 dB

    - Option 6: X and Y is not needed but UL CA relaxation included in CA MPR

  - Agreement

    - Before deciding X and Y values, agreement on how to incorporate PA-PA interaction is needed. And further study on relaxation framework is encouraged.

- X and Y for spherical coverage.

  - Options

    - Option 1: X=Y=3.5 dB

    - Option 2: X=3.7 dB and Y=3.5 dB

    - Option 3: ΔTIB,S,n= X2/Y2+2.7 dB.

    - Option 4: ΔTIB,S,n shall be ΔRIB,s,n -1 dB.

    - Option 5: X=Y=9 dB

    - Option 6: X and Y should incorporate MPRPA-PA

  - Agreement

    - Before deciding X and Y values, agreement on how to incorporate PA-PA interaction is needed. And further study on relaxation framework is encouraged.

Note: X is the relaxation value for n257 compared with its single cc requirement; Y is the relaxation value of n259 compared with its single cc requirement.

WF – Total power concept

- applicability of total power concept

  - Options

    - Option 1: Yes it is and is defined as the sum of EIRP in peak direction of two band shall not exceed the 43 dBm

    - Option 2: Applied to TRP

    - Option 3: Applied to minimum peak EIRP

    - Option 4: Applied to EIRP spherical coverage

    - Option 5: Not needed, cannot be used for MPE and power consumption and thermal issues; can be handled with P-MPR

  - Agreement

    - No new requirements than the per-band based requirement package of max EIRP, max TRP, min peak EIRP, EIRP spherical coverage.

    - Further study the impact of total power concept to max EIRP, max TRP, min peak EIRP, EIRP spherical coverage, and how to address it.

WF – power configuration

- RAN4 needs to further discuss the power configuration side condition for FR2 inter-band UL CA MOP. UE power consumption under MOP status should be addressed and further study how to address it.

    - Option 1: maximum output power transmission is only required for tested band and a relative low output power can be configured for non-tested band

    - other options are not precluded