3GPP TSG-RAN WG4 Meeting # 100-e	      	              			R4-2114983

Electronic Meeting, 16th -27th August, 2021

Title: 	WF on MPR for intra-band V2X con-current operation

Source: 	LG Electronics

Agenda item:	9.15.5.4

Document for:	Approval

Background [1]

- During RAN4#99-e & RAN4 #100-e meeting, RAN4 discussed the required MPR values for intra-band V2X con-current operation.

- In RAN4 #99-e meeting, RAN4 agreed to the detail MPR requirements for intra-band V2X con-current operation UE in R4-21107870 as follows.

  - Issue 3-2-3: MPR for PC2 UE

  - Issue 3-2-3: MPR for PC2 UE

Background [2]

- However, interested companies proposed to revise MPR requirements for V2X UE in RAN4 #100-e meeting as follow,

  - Issue 3-1-1: MPR for intra-band V2X con-current UE

    - Option 1: Consider MPR requirements for intra-band V2X con-current operation in R4-2112341 to keep the agreed MPR requirements in RAN4 #99-e meeting

    - Option 2: Specify 2dB MPR for 23dBm 64QAM, 4dB MPR for 23dBm 256QAM, 4.5dB MPR for 26dBm 64QAM and 7dB MPR for 26dBm 256QAM, no matter what the RB allocation is.

    - Option 3: The MPR values given in 38.101-3 [2] for cellular EN-DC shall adopt for V2X intra-band con-current operation where separate RF chains are used for Uu and SL.

- Therefore, RAN4 needs further discussion to specify the MPR requirements and needs more input to understand the reason for the relaxed MPR values based on the evidence for intra-band V2X con-current operation UE.

Discussion on the candidate Options

- Issue 3-1-1: MPR for intra-band V2X con-current UE

  - Observation for Option 2 in Background [2]

    - In R4-2113410, the proponent did not provide any evidence to relax the agreed MPR requirements.

    - RAN4 can revise the MPR requirements based on the MPR simulation results when the interested company provides the detailed simulation results based on the RAN4 agreed MPR simulation assumptions.

  - Observation for Option 3 in Background [2]

    - In R4-2112284, the proponent proposed to reuse EN-DC MPR requirements for intra-band V2X con-current operation.

    - But the MPR requirement of EN-DC UE did not consider the difference between V2X specific channel and NR UE’s channel such as S-SSB transmission, PSFCH multiplexing, and PSSCH/PSCCH multiplexing. So it is impossible to reuse the EN-DC MPR requirements for intra-band V2X con-current UE.

WF: MPR requirements

- Issue 3-1-1: MPR for intra-band V2X con-current UE

  - Candidate options

    - Option 1: Consider MPR requirements for intra-band V2X con-current operation in R4-2112341 to keep the agreed MPR requirements in RAN4 #99-e meeting

    - Option 2: Specify 2dB MPR for 23dBm 64QAM, 4dB MPR for 23dBm 256QAM, 4.5dB MPR for 26dBm 64QAM and 7dB MPR for 26dBm 256QAM, no matter what the RB allocation is.

    - Option 3: The MPR values given in 38.101-3 [2] for cellular EN-DC shall adopt for V2X intra-band con-current operation where separate RF chains are used for Uu and SL.

- Agreement

  - RAN4 adopts Option 1 for MPR requirements for intra-band V2X con-current operation UE.

  - RAN4 can approve the related TP to capture the agreed MPR requirements with [ ].

  - RAN4 allows revision of MPR requirements until RAN4 #102 meeting based on the MPR simulation results from interested companies.

- Reference

- R4-2114735, “Email discussion summary for [100-e][135] NRSL_enh_Part2,” Moderator (CATT), RAN4#100-e

- R4-2107870, “WF on MPR requirements for intra-band con-current V2X operation,” LGE, RAN4 #99-e

- R4-2112284, “MPR specifications for intra-band con-current V2X operation,” Qualcomm, RAN4 #100-e

- R4-2112341, “TP on MPR for NR V2X intra-band con-current operation with Uu,” LGE, RAN4 #100-e

- R4-2113410, “Discussion on MPR requirements for intra-band con-current V2X operation,” Huawei, RAN4 #100-e