3GPP TSG-RAN WG4 Meeting # 100-e																   R4-2115015

Electronic Meeting, August. 16-27, 2021

Agenda item:			8.25

Source:	Moderator (CATT)

Title:	Email discussion summary for [100-e][115] NR_LTE_V2X_PC5_combos

Document for:	Information

Introduction

In this email discussion summary, the newly introduced band combinations of V2X con-current operation and remaining open issues will be discussed.

The candidate targets of this email discussion for 1st round and 2nd round:

- 1st round

  - Companies to provide comments on open issues involved.

  - Companies to provide comments on TPs and CRs involved.

- 2nd round

  - Further check the revised TPs and CRs if any.

  - Recommend the final status of TPs and CRs.

Topic #1: Introduce new band combinations

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2111947 | CATT | Title: On notation for V2X band combinations Observation 1: NR CA inter-band band combinations are represented as CA_nX-nY, in which only NR bands are involved and “-” between two bands is used. Observation 2: EN-DC inter-band band combinations are represented as DC_X_nY, in which both LTE band and NR band are involved. NE-DC inter-band band combinations are represented as DC_nX_Y, in which both NR band and LTE band are involved and “_” between two bands is used. NR DC inter-band band combinations are represented as DC_nX-nY, in which only NR bands are involved and “-” between two bands is used. Observation 3: For CA and DC, “nX-nY” is used for notation of inter-band band combinations that involve only NR bands, while “X_nY” or “nX_Y” is used for notation of inter-band band combinations that involve both LTE band and NR band. Observation 4: LTE V2X inter-band band combinations for con-current operation are represented as V2X_X-Y, in which only LTE bands are involved and “-” between two bands is used. Proposal 1: To adopt either Option 1 or Option 2 for NR V2X inter-band band combinations. Option 1: Reuse the principle of CA and DC, i.e. V2X_nX-nY, V2X_X_nY, V2X_nX_Y Option 2: Use “_” for all cases, i.e. V2X_nX_nY, V2X_X_nY, V2X_nX_Y |
| R4-2111949 | CATT | Title: CR for TS 38.101-3, Introduce new band combination of V2X_3A_n47A |
| R4-2111950 | CATT | Title: TR 37.875, Band combinations for V2X con-current operation |
| R4-2112285 | Qualcomm Incorporated | Title: Calculation of MSD for n79 for V2X_n79A-n47A and V2X_n79A-47A and accompanying TP Proposal 1: The MSD value of 3.3 dB given in table 2 be adopted for band n79 in V2X_n79A-47A Proposal 2: The MSD value of 3.3 dB given in table 3 be adopted for band n79 V2X_n79A-n47A Proposal 3: MSDs for n79, n47 and B47 in CA combinations V2X_n79A-47A and V2X_n79A-n47A apply for all BWs and SCS combinations. |
| R4-2113420 | Huawei, HiSilicon | Title: Discussion and TP for TR 37.875 on MSD for V2X_n79A-n47A and V2X_n79A_47A |
| R4-2111948 | CATT | Title: TP on V2X_3A_n47A coexistence study and band combos notation correction |

Open issues summary

Based on above contributions, the following sub-topics and issues are summarized.

Sub-topic 1-1: Sensitivity degradation for V2X_n79A-n47A and V2X_n79A_47A

Issue 1-1-1: MSD for V2X_n79A-n47A and V2X_n79A_47A

- Proposals

  - Option 1: Adopt the following MSD (3.3dB) for Uu band proposed in paper R4-2112285.

Table 6.2.4.1-2: Reference sensitivity exceptions (MSD) due to cross band isolation for Uu

| UL band | SL band | SCS of UL/DL band (kHz) | LCRB of UL band | UL band BW(MHz) | LCRB of SL band | SL band BW (MHz) | MSD value of Uu band (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n79 | n47 | 15 | 216 | 40 | 50 | 10 | 3.3 |

Table 6.3.4.4-2: Reference sensitivity exceptions (MSD) due to cross band isolation for Uu

| UL band | SL band | SCS of UL/DL band (kHz) | LCRB of UL band | UL band BW(MHz) | LCRB of SL band | SL band BW (MHz) | MSD value of Uu band (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n79 | n47 | 15 | 216 | 40 | 50 | 10 | 3.3 |

  - Option 2: Adopt the following MSD (1dB) for Uu band proposed in paper R4-2113420.

Table 6.2.4.1-1: Reference sensitivity exceptions (MSD) due to cross band isolation for V2X

| UL band | SL band | SCS of UL/DL band (kHz) | LCRB of UL band | UL band BW(MHz) | LCRB of SL band | SL band BW (MHz) | MSD value of SL band (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n79 | n47 | 15 | 216 | 40 | 50 | 10 | 3.3 |
| n47 | n79 | 15 | 50 | 10 | 216 | 40 | 1 |

Table 6.3.4.4-1: Reference sensitivity exceptions (MSD) due to cross band isolation for NR V2X

| UL band | DL band | SCS of UL/DL band (kHz) | LCRB of UL band | UL band BW(MHz) | LCRB of DL band | DL band BW (MHz) | MSD value of DL band (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n79 | 47 | 15 | 216 | 40 | 50 | 10 | 3.3 |
| 47 | n79 | 15 | 50 | 10 | 216 | 40 | 1 |

  - Option 3: Adopt the average value of MSD.

- Recommended WF

  - Need more discussion.

Issue 1-1-2: Whether MSD apply for all CBW/SCS combinations

- Proposals

  - Option 1: YES

  - Option 2: NO

- Recommended WF

  - Need more discussion.

Sub-topic 1-2: Notation for V2X band combinations

Issue 1-2-1: Notation for V2X band combinations

- Proposals

  - Option 1: Reuse the principle of CA and DC, i.e. V2X_nX-nY, V2X_X_nY, V2X_nX_Y

  - Option 2: Use “_” for all cases, i.e. V2X_nX_nY, V2X_X_nY, V2X_nX_Y

- Recommended WF

  - Need more discussion.

Companies views’ collection for 1st round

Open issues

Issue 1-1-1: MSD for V2X_n79A-n47A and V2X_n79A_47A

| Company | Comments |
| --- | --- |
| CATT | Option 3. Also separate tables for Uu and V2X would be preferred. |
| Huawei | Option 2, the diversity gain is not considered in R4-2112285. Thus, 1dB de-sense is reasonable. |
| LGE | Option 2 is reasonable, But different Table is needed because n47 do not have UL band. |
| Qualcomm | Option 1: These numbers are based on PA measurements and commercially available components and the only difference between the 2 options is the amount of PA noise impacting the receiver. Diversity gain does not have to be considered in our calculation as we are taking the REFSENS value from 38.101-1, which already accounts for diversity, and adding the PA noise component to it. |

Issue 1-1-2: Whether MSD apply for all CBW/SCS combinations

| Company | Comments |
| --- | --- |
| CATT | If the MSD is considering the worst case of self-interference, it can apply for all CBW/SCS combinations. |
| Huawei | Option 2, it’s just applicable for specific configuration just like MSD due intermodulation. |
| LGE | Generally, MSD can be impacted case by case. MSD impacted fre. Range is full overlapped in the whole Rx band, then option 1 is right, but some CBW can be impacted in partially overlapped case. The MSD verifying perspective, the MSD only tested in the specific test configuration. In LTE, NR CA/DC, some specific band combination (5MHz, 10MHz in LTE/NR band, 40MHz in n79) are used to verify the MSD requirements. In SL operation. 10MHz CBW is baseline to verify the MSD. |
| Qualcomm | Option 1: Every BW/SCS combination should have an MSD associated with it. So as not to overly complicate the spec we propose using the MSD values calculated for n47/B47 (BW=10MHz) with n79 (BW=40MHz) for all the BW/SCS combinations |

Issue 1-2-1: Notation for V2X band combinations

| Company | Comments |
| --- | --- |
| CATT | OK with both options. |
| Huawei | Based on the TR 38.862, we can follow the option 1. |
| LGE | In specification, RAN4 can follow same principle of NR Uu as option 1. My intention is also to follow option 1 for notation of SL band combination. Only different point is SL band is located in the end. |
| Qualcomm | Option 1 |

CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2111949 (CR for TS 38.101-3, Introduce new band combination of V2X_3A_n47A) |  |
| R4-2112285 (Calculation of MSD for n79 for V2X_n79A-n47A and V2X_n79A-47A and accompanying TP) | Huawei: Diversity gain is not considered. Thus, 1dB de-sense is reasonable. |
| R4-2113420 (Discussion and TP for TR 37.875 on MSD for V2X_n79A-n47A and V2X_n79A_47A) | Qualcomm: Cannot agree with this TP as we think the MSD is too small given the PA noise that we have seen. |
| R4-2111948 (TP on V2X_3A_n47A coexistence study and band combos notation correction) | Qualcomm: agree with tis TP |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |
| Issue 1-1-1: MSD for V2X_n79A-n47A and V2X_n79A_47A | Tentative agreements: Separate MSD tables for V2X and Uu should be used. Candidate options: - Option 1: Adopt the following MSD (3.3dB) for Uu band proposed in paper R4-2112285. - Option 2: Adopt the following MSD (1dB) for Uu band proposed in paper R4-2113420. - Option 3: Adopt the average value of MSD. Recommendations for 2nd round: Further discuss whether to consider diversity gain in MSD and decide a final MSD. |
| Issue 1-1-2: Whether MSD apply for all CBW/SCS combinations | Tentative agreements: Option 2: NO. MSDs for n79, n47 and B47 in V2X_n79A-47A and V2X_n79A-n47A do not apply for all BWs and SCS combinations. Whether MSDs apply for all BW and SCS combinations should be based on each specific case. Candidate options: NONE Recommendations for 2nd round: No further discussion needed. |
| Issue 1-2-1: Notation for V2X band combinations | Tentative agreements: Option 1: Reuse the principle of CA and DC, i.e. V2X_nX-nY, V2X_X_nY, V2X_nX_Y Candidate options: NONE Recommendations for 2nd round: No further discussion needed. Moderator: One WF will be assigned to mainly capture these agreements for traceability and open issues. This WF can be referred to for future CRs on 38.101-1/3 that will change previous V2X notations. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2111949 | Agreed |
| R4-2112285 | To be revised |
| R4-2113420 | Noted |
| R4-2111948 | Approved |

Discussion on 2nd round (if applicable)

Open issues summary

Issue 1-1-1: MSD for V2X_n79A-n47A and V2X_n79A_47A

- Proposals

  - Option 1: Adopt the MSD (3.3dB) for Uu band proposed in paper R4-2112285.

  - Option 2: Adopt the MSD (1dB) for Uu band proposed in paper R4-2113420.

  - Option 3: Adopt the average value of MSD.

- Recommended WF

  - Further discuss whether to consider diversity gain in MSD and decide a final MSD.

Companies views’ collection for 2st round

Open issues

Issue 1-1-1: MSD for V2X_n79A-n47A and V2X_n79A_47A

| Company | Comments |
| --- | --- |
| LGE | Prefer option 2 and can compromised with option 3. RAN4 need to check the table format for REFSENS requirements for V2X_n79A-n47A and V2X_n79A_47A UE. |
| CATT | We can accept the average value 2.2dB MSD. |
| Qualcomm | Prefer option 1. The MSD values calculated in draft R4-2114904 (revision of R4-2112285) are based on measurements and commercially available components. These calculations also take RX diversity gain into account. Based on these measurements it is seen that an MSD of 3.3 dB is required for n79. |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2114904 (revision of R4-2112285) |  |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2114904 | Noted |
| R4-2114903 | Approved |

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on … | YYY |  |
| LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |
| WF on general issues of V2X band combinations | CATT |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2111947 | On notation for V2X band combinations | CATT | Noted |  |
| R4-2111949 | CR for TS 38.101-3, Introduce new band combination of V2X_3A_n47A | CATT | Agreed |  |
| R4-2111950 | TR 37.875, Band combinations for V2X con-current operation | CATT | For email approval |  |
| R4-2112285 | Calculation of MSD for n79 for V2X_n79A-n47A and V2X_n79A-47A and accompanying TP | Qualcomm Incorporated | To be revised |  |
| R4-2113420 | Discussion and TP for TR 37.875 on MSD for V2X_n79A-n47A and V2X_n79A_47A | Huawei, HiSilicon | Noted |  |
| R4-2111948 | TP on V2X_3A_n47A coexistence study and band combos notation correction | CATT | Approved |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2114904 | Calculation of MSD for n79 for V2X_n79A-n47A and V2X_n79A-47A and accompanying TP | Qualcomm Incorporated | Noted |  |
| R4-2114903 | WF on general issues of V2X band combinations | CATT | Approved |  |
| R4-2111950 | TR 37.875, Band combinations for V2X con-current operation | CATT | For email approval |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| CATT | Yuan Gao | gaoyuan@catt.cn |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)