3GPP TSG-RAN WG4 Meeting #100-e	R4-2115064

Electronic Meeting, Aug. 16 – Aug. 27, 2021

Agenda Item: 10.6.1, 10.6.2, 10.6.3, 10.6.4, 10.6.5

Source:	Nokia

Title:	WF on ‘Optimizations of pi/2 BPSK uplink power in NR’ in Rel-17

Document for: Approval

# 1	TX Signal placement in channel bandwidth

The latest email discussion summary [1] captures the issue in the following way:

Issue 1-1-1: TX Signal placement in channel bandwidth Companies agreed to study this issue further to determine whether a signal placement restriction within a given BW is required Recommendation Capture as agreement in WF

Proposed agreement #1:

Study the Tx signal placement in channel bandwidth further to determine whether a signal placement restriction within a given BW is required.

| Company | Comment |
| --- | --- |
| Qualcomm | Agree with proposed agreement |
| Apple | We support this proposal |
| Huawei | We support this proposal. |

Agreed WF:

Study the Tx signal placement in channel bandwidth further to determine whether a signal placement restriction within a given BW is required.

# 2	Adjustment of ACLR requirements

The latest email discussion summary [1] captures the issue in the following way:

Issue 1-1-2: Do ACLR requirements need to be adjusted if power is boosted by more than 3dB. (moderator’s edit: relative to 29 dBm) Companies agreed that if the output power is increased beyond PC1 levels, the ACLR would have to be re-evaluated. Recommendation Capture as agreement in WF

Proposed agreement #2:

If the UE output power reaches the next power class, the ACLR requirement is re-evaluated

| Company | Comment |
| --- | --- |
| Qualcomm | Agree with proposed agreement |
| OPPO | Ok with proposed agreement |
| Apple | We support this proposal |
| Huawei | Support the proposal |

Agreed WF:

If the UE output power reaches the next power class, the ACLR requirement is re-evaluated

# 3. 	UE type considered for power enhancement

The latest email discussion summary [1] captures the issue in the following way:

Issue 1-1-3: UE type considered for power enhancement in SI Companies agreed to initially address UE handhelds with PC2 as a baseline Recommendation Capture as agreement in WF

Proposed agreement #3:

UE handheld with PC2 is addressed as a baseline for the power enhancement

| Company | Comment |
| --- | --- |
| Qualcomm | Agree with proposed agreement |
| OPPO | Ok with proposed agreement |
| Apple | We support this proposal |
| Huawei | Support the proposal |

Agreed WF:

UE handheld with PC2 is addressed as a baseline for the power enhancement

# 4.	TR skeleton on optimization of pi/2 BPSK uplink power in NR

The latest email discussion summary [1] captures the issue in the following way:

Issue 1-3-1: TR skeleton on optimization of pi/2 BPSK uplink power in NR - Proposals - Option 1: Updated TR skeleton is agreeable. - Option 2: Needs further updating One company wanted to add one paragraph for “Net gain analysis of combined Tx and Rx impacts”

Proposed agreement #4:

Updated TR skeleton including one additional paragraph (“Net gain analysis of combined Tx and Rx impacts”) is agreed

| Company | Comment |
| --- | --- |
| Qualcomm | We are agreeable to update TR |

Agreed WF:

Updated TR skeleton including one additional paragraph (“Net gain analysis of combined Tx and Rx impacts”) is agreed

# 5. 	Study of power enhancement for PC2 UE

The latest email discussion summary [1] captures the issue in the following way:

Issue 1-1-4: Study of power enhancement for PC2 UE Some companies wanted to set a power enhancement target for the SI prior to doing PA measurements. While another company thought it better to set a power enhancement value based on PA measurements. Recommendation: - Further discuss in 2nd round - Discussion could focus on - Motivation for establishing a power target apriori Should a power enhancement value be based on PA measurements from participating companies

The way forward is to continue discussion in the 2nd round.

Proposed agreement #5:

Limit the maximum output power to 29dBm at antenna port for PC2 amplifiers single PA architecture and use this as 0dB MPR reference

| Company | Comment |
| --- | --- |
| Qualcomm | The power enhancement value should be based on PA measurements. Could proponents explain the motivation for establishing a power target apriori? What happens if the established target cannot be achieved? |
| OPPO | Agree with QC comment. |
| IITH, IITM, CEWIT, Reliance Jio, Tejas Networks | We need to study and determine achievable power levels. We cannot set targets at this stage. |
| SaankhyaLabs | Agree with IITH comments |
| Apple | Since this is the first meeting discussing this topic, we are open to collect further input. Eventually we could make it a tentative agreement, so it is more of a guideline than something set in stone. |
| Huawei | We are ok to have further discussion. But we could not have a target which beyond the implementation capability for the baseline assumption. |

’

Agreed WF:

No conclusion, more discission needed in coming meeting

# 6. 	UE signaling and network configuration

The latest email discussion summary [1] captures the issue in the following way:

Issue 1-1-5: UE signaling and network configuration Most companies thought that it may be too early to discuss signaling. Some companies thought that reusing existing UE signaling was a good starting point. Other companies thought that signaling is conditional on what is agreed. Recommendation: Signaling can be discussed further in future meetings

The way forward is to defer the discussion related to UE ignaling and network configuration until the scope of Rel-17 (candidate) solutions is more mature.

Proposed agreement #6:

Post pone to WI phase.

| Company | Comment |
| --- | --- |
| Qualcomm | Agree with proposed agreement |
| OPPO | Ok with proposed agreement |
| Apple | We accept the proposed agreement |
| Huawei | Ok with the proposal |

Agreed WF:

Din on UE signaling and network configuration are post poed to WÌ Phase

# 7.	Transparent spectral shaping

The latest email discussion summary [1] captures the issue in the following way:

Issue 1-2-1: Transparent spectral shaping Most companies supported keeping the Rel-15 approach (i.e. transparent spectral shaping to gNB receiver) in Rel-17 where both DMRS and data are shaped and the actual pulse shaping filter is left for UE implementation. One company thought that to improve performance specifying the shaping filter was necessary Recommendation: Further discussion in 2nd round

The way forward is to continue discussion in the 2nd round.

Proposed agreement #7:

Keep the Rel-15 approach (i.e. transparent spectral shaping to gNB receiver) in Rel-17 where both DMRS and data are shaped and the actual pulse shaping filter is left for UE implementation.

| Company | Comment |
| --- | --- |
| Qualcomm | Keep rel-15 approach in Rel-17 unless a different scheme can be justified |
| OPPO | Ok with proposed agreement |
| IITH, IITM, CEWIT, Reliance Jio, Tejas Networks | The study requires a specific shaping filter to be narrowed down and we are not yet ready to make this decision. |
| SaankhyaLabs | In agreement with comments from IITH above. |
| Apple | While we would prefer to keep Rel-15 approach we recognize that this is a study item. We wonder if the decision is made too early. If it is found that a certain shaping filter or set of filter would provide improvement to the overall performance then it should still be possible to define specific requirements. We would like to propose to have agreement #7 as a tentative agreement. |
| Huawei | OK with the proposed agreement. |

Agreed WF:

No conclusion, more discussion is needed in coming meetings

# 8.	Pulse shaping filter characteristics

The latest email discussion summary [1] captures the issue in the following way:

Issue 1-2-2: Pulse shaping filter characteristics Most companies supported following the general RAN4 principle that the requirements are implementation agonistic, and which shaping filter type is selected is up to UE implementation and should not be limited to certain type during the study One company thought that to improve performance specifying the shaping filter was necessary Recommendation: - Company responses were consistent between issues 1-2-1 and 1-2-2 so in the future these two topics can be discussed together Further discussion in 2nd round

The way forward is to continue discussion in the 2nd round.

Proposed agreement #8:

RAN4 principle that the requirements are implementation agonistic, and which shaping filter type is selected is up to UE implementation and should not be limited to certain type during the study

| Company | Comment |
| --- | --- |
| Qualcomm | See answer to section 7 |
| OPPO | Ok with proposed agreement |
| IITH, IITM, CEWIT, Reliance Jio, Tejas Networks | Same answer as section 7 |
| SaankhyaLabs | Agree with comments from IITH. Please refer to answer in Section 7 |
| Apple | Similar answer to section 7: We recognize that this is a study item and we wonder if the decision is made too early. If it is found that a certain shaping filter or set of filter would provide improvement to the overall performance then it should still be possible to define specific requirements. We would like to propose to have agreement #8 as a tentative agreement. |
| Huawei | Ok with the proposed agreement |

Agreed WF:

No conclusion, more discussion is needed in coming meetings

# 9.	Spectral flatness requirements (PRB ≥16)

The latest email discussion summary [1] captures the issue in the following way:

Issue 1-2-3: Spectral flatness requirements (PRB ≥16) Most companies elected to keep the current spectral flatness requirements for large PRB allocations (e.g. ≥ 16 PRBs) (moderator’s comment: current spectral flatness refers to spec in section 6.4.2.4.1, TS 38.101-1) One company decided to postpone making a decision on this until more results are available. Recommendation: Continue discussion in future meetings

The way forward is to continue discussion in the 2nd round.

Proposed agreement #9:

keep the current spectral flatness requirements for large PRB allocations (e.g. ≥ 16 PRBs as in section 6.4.2.4.1, TS 38.101-1.

| Company | Comment |
| --- | --- |
| Qualcomm | See answer to section 7 |
| OPPO | Ok with proposed agreement |
| IITH, IITM, CEWIT, Reliance Jio, Tejas Networks | Same answer as section 7. |
| SaankhyaLabs | Agree with comments from IITH |
| Apple | Please see our answers in section 7 and 8 |
| Huawei | Ok with the proposed agreement |

Agreed WF:

No conclusion, more discussion is needed in coming meetings

# 10.	Spectral flatness requirements (PRB <16)

The latest email discussion summary [1] captures the issue in the following way:

Issue 1-2-4: Spectral flatness requirements (PRB <16) Most companies agreed that tighter spectral flatness requirements for small PRB allocations to optimize net gain needs further discussion One company wanted to adopt tighter spectral flatness requirements Recommendation Propose study of PRB sensitive filtering in WF

The way forward is to continue discussion in the 2nd round.

Proposed agreement #10:

Discuss further tighter spectral flatness requirements compared to current specification for small PRB allocations to optimize net gain.

| Company | Comment |
| --- | --- |
| Qualcomm | Agree with proposed agreement |
| OPPO | Ok with proposed agreement |
| Apple | We agree with proposal |
| Huawei | Ok with the proposal |

Agreed WF:

Discuss further tighter spectral flatness requirements compared to current specification for small PRB allocations to optimize net gain.

# References

[1] R4-2114747, “Email discussion summary for [100e][147] FS_NR_Opt_pi2BPSK”, Qualcomm