3GPP TSG-RAN WG4 Meeting # 100-e 	      R4-2115351

Electronic Meeting, August 16-27, 2021

Agenda item:			9.16.7.1, 9.16.7.2, 9.16.7.3

Source:	Qualcomm

Title:	WF on R17 NR_ext_to_71GHz_RRM_1

Document for:	Approval

# Introduction

This WF captures all agreements and open issues for the WI NR_ext_to_71_GHz based on the GTW and email discussion [100-e][230] NR_ext_to_71GHz_RRM_1.

Agreements from the GTW

Agreements from the first-round email discussion

Agreements from the second-round email discussion

# Topic #1: General

### Sub-topic 1-1 Working assumptions and Deployment scenarios

Issue 1-1-1: Deployment scenarios

GTW Agreement

- Define the requirements for the following deployment scenarios with equal priority:

  - Standalone single carrier and CA in FR2-2

  - FR2-2 CA and DC with anchor on FR1

    - Note: the scenario may be further adjusted pending further discussion in the RF session

Issue 1-1-2: Working assumptions for creating new FR2-2 RRM requirements

Working assumptions:

- FR2 RRM requirements apply for the operation in FR2-2, unless there is technical justification for a revision of the FR2 requirements.

- For RRM requirements defined with SCS already, higher SCS (e.g., 480 kHz and 960 kHz) applicable for 52.6 – 71 GHz can be defined (if needed) as function of SCS within FR2; For RRM requirements defined with FR2, it needs check if involvements of new SCS’s, division of FR2-1/FR2-2 or FR2 already can cover it.

### Sub-topic 1-2: Rx beam sweeping scaling factor

Issue 1-2-1: Rx beam sweeping scaling factor

GTW agreements:

- Rx beam sweeping scaling factor

  - Further study whether new scaling factor is needed for FR2-2 considering RF session conclusions on UE antenna array assumptions and UE power classes and the difference with FR2-1 assumptions

  - Rx beam sweeping factor from FR2-1 can be used as a starting point for analysis

### Sub-topic 1-3: Scheduling restrictions

Issue 1-3-1: Scheduling restrictions

Agreement:

- Further investigate whether to update the scheduling restriction for L1/L3 measurement considering at least the following aspects with further inputs from RAN1 and RF:

  - Beam switching time

  - Synchronization assumptions with large SCS

### Sub-topic 1-4: Random access

Issue 1-4-1: Random access requirements for new SCS/preamble sequence length

Agreements:

RAN4 to re-use existing random access requirements for new SCS and preamble sequence length in the beginning. New requirements, if needed, could be further discussed.

### Sub-topic 1-5: Measurement procedures

Issue 1-5-1: Measurement capability

Agreements:

Re-use FR2-1 measurement capability of number of cells and beams for FR2-2.

Issue 1-5-5: Simulations for Cell detection, PBCH detection and SSB measurements

Open items:

- Discuss whether simulations are needed to study the impact of higher SCS on the following

  - Cell detection

  - PBCH detection for SSB index detection

  - SSB measurements

- If yes, also provide inputs on the simulation assumptions.

# Topic #2: Timing requirements

### Sub-topic 2-1: UE transmit timing error

Issue 2-1-1: General principles in defining the Te requirements

GTW agreements:

- Further study percentage of UL CP length Te can occupy without impacting UL system performance?

  - Option 1: Keep the Te within the same percentage of the CP length as existing SCS

  - Option 2: 30%

  - Option 3: 50%

- Further study achievable Te from UE perspective

- Study different combinations of SSB SCS and UL signal SCS for FR2-2

### Sub-topic 2-2: UE timer accuracy

Issue 2-2-1: UE timer accuracy requirements

Agreements:

RAN4 not to define new UE Timer accuracy requirements for the operation above 52.6 GHz.

### Sub-topic 2-3: Timing advance

Issue 2-3-1: Timing advance

Agreements:

The UE shall adjust the timing of its transmissions with a relative accuracy better than or equal to the UE Timing Advance adjustment accuracy requirement in the below table.

Note: Revisit if certain implementation issues are identified.

| UL Sub Carrier Spacing(kHz) | 15 | 30 | 60 | 120 | 480 | 960 |
| --- | --- | --- | --- | --- | --- | --- |
| UE Timing Advance adjustment accuracy | ±256 Tc | ±256 Tc | ±128 Tc | ±32 Tc | [±8 Tc] | [±4 Tc] |

### Sub-topic 2-4: MTTD/MRTD

Issue 2-4-1: MTTD/MRTD

Agreements:

- Consider FR2-1 requirements for 120kHz SCS for FR2-2 as baseline.

- RAN4 to further discuss whether new MRTD/MTTD requirements are needed for SCS of 480/960 kHz based on the agreed deployment scenarios

  - Identify other parameters that needs to be considered for the discussion

  - Consider using the current FR2-1 MRTD requirements and rules for FR2-2

# Topic #3: Interruptions

### Sub-topic 3-2: General principles

Issue 3-2-1: Applicability of FR2-1 interruption durations

Agreements:

Define the interruption requirements for FR2-2 based on the same assumptions (RF retunings, AGC etc) as in legacy FR2 (FR2-1).

- The baseline interruption time (Ti) in FR2-2 to be same as FR2-1

  - Nslot,interrupted = ⌈ Ti/Tslot ⌉ for synchronous scenario

  - Nslot,interrupted = ⌈ Ti/Tslot ⌉ + 1, for asynchronous scenario

Issue 3-2-2: Impact of MRTD

Agreements:

The interruption requirements for FR2-2 shall consider the MRTD between CCs where additional slot may be allowed (asynchronous scenarios) when the MRTD is non-negligible considering the slot length of 480 kHz and 960 kHz.

### Sub-topic 3-3: Interruption requirements

Issue 3-3-1: Interruptions at SCell addition/release (inter-band CA)

Agreements:

Update Table 8.2.2.2.1-1 in TS 38.133 with 480/960 kHz subcarrier spacing as below:

Interruption length X1 for SCell addition/release for inter-band CA

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 | 1 |  |
| 1 | 0.5 | 2 |  |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 4 |
|  |  | Either aggressor cell or victim cell is on FR1 | 5 |
| 3 | 0.125 | Aggressor cell is on FR2 | 8 |
|  |  | Aggressor cell is on FR1 | 9 |
| 5 | 0.03125 | Aggressor cell is on FR2-2 | 32 |
|  |  | Aggressor cell is on FR2-1 | FFS |
|  |  | Aggressor cell is on FR1 | FFS |
| 6 | 0.015625 | Aggressor cell is on FR2-2 | 64 |
|  |  | Aggressor cell is on FR2-1 | FFS |
|  |  | Aggressor cell is on FR1 | FFS |

Issue 3-3-2: Interruptions at SCell addition/release (intra-band CA)

Agreements:

- Update Table 8.2.2.2.1-2 in TS 38.133 with 480/960 kHz subcarrier spacing as below:

Interruption duration for SCell addition/release for intra-band CA

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 + TSMTC_duration * Nslotsubframe,μ |
| 1 | 0.5 | 2 + TSMTC_duration * Nslotsubframe,μ |
| 2 | 0.25 | 4 + TSMTC_duration * Nslotsubframe,μ |
| 3 | 0.125 | 8 + TSMTC_duration * Nslotsubframe,μ |
| 5 | 0.03125 | 32 + TSMTC_duration * Nslotsubframe,μ |
| 6 | 0.015625 | 64 + TSMTC_duration * Nslotsubframe,μ |
| NOTE 1: TSMTC_duration measured in subframes is - the longest SMTC duration among all above active serving cells and the SCell being added when one SCell is added; - the longest SMTC duration among all active serving cells in the same band when one SCell is released. NOTE 2: Nslotsubframe,μ is as defined in TS 38.211 [6]. |  |  |

Issue 3-3-3: Interruptions at SCell activation/deactivation (inter-band CA)

Agreements:

- Update Table 8.2.2.2.2-1 in TS 38.133 with 480/960 kHz subcarrier spacing as below:

Interruption length X2 for SCell activation/deactivation for inter-band CA

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 | 1 |  |
| 1 | 0.5 | 1 |  |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 |
|  |  | Aggressor cell is on FR1 | 5 |
| 5 | 0.03125 | Aggressor cell is on FR2-2 | 16 |
|  |  | Aggressor cell is on FR2-1 | FFS |
|  |  | Aggressor cell is on FR1 | FFS |
| 6 | 0.015625 | Aggressor cell is on FR2-2 | 32 |
|  |  | Aggressor cell is on FR2-1 | FFS |
|  |  | Aggressor cell is on FR1 | FFS |

Issue 3-3-4: Interruptions at SCell activation/deactivation (intra-band CA)

Agreements:

- Update Table 8.2.2.2.2-2 in TS 38.133 with 480/960 kHz subcarrier spacing as below:

Interruption duration for SCell activation/deactivation for intra-band CA

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 + TSMTC_duration * Nslotsubframe,μ |
| 1 | 0.5 | 1 + TSMTC_duration * Nslotsubframe,μ |
| 2 | 0.25 | 2 + TSMTC_duration * Nslotsubframe,μ |
| 3 | 0.125 | 4 + TSMTC_duration * Nslotsubframe,μ |
| 5 | 0.03125 | 16 + TSMTC_duration * Nslotsubframe,μ |
| 6 | 0.015625 | 32 + TSMTC_duration * Nslotsubframe,μ |
| NOTE 1: TSMTC_duration measured in subframes is - the longest SMTC duration among all above active serving cells and the SCell being added when one SCell is added; - the longest SMTC duration among all active serving cells in the same band when one SCell is released. NOTE 2: Nslotsubframe,μ is as defined in TS 38.211 [6]. |  |  |

Issue 3-3-5: Interruptions during measurements on deactivated SCC

Agreements:

- Update Table 8.2.2.2.3-1 in TS 38.133 with 480/960 kHz subcarrier spacing as below:

- Interruption duration for measurement on deactivated SCell for intra-band CA

|  | NR Slot length (ms) | X (slots) | Interruption length (slots) |
| --- | --- | --- | --- |
| 0 | 1 | 1 | 2 + TSMTC_duration * Nslotsubframe,μ |
| 1 | 0.5 | 1 | 2 + TSMTC_duration * Nslotsubframe,μ |
| 2 | 0.25 | 2 | 4 + TSMTC_duration * Nslotsubframe,μ |
| 3 | 0.125 | 4 | 8 + TSMTC_duration * Nslotsubframe,μ |
| 5 | 0.03125 | 16 | 32 + TSMTC_duration * Nslotsubframe,μ |
| 6 | 0.015625 | 32 | 64 + TSMTC_duration * Nslotsubframe,μ |
| NOTE 1: TSMTC_duration measured in subframes is the longest SMTC duration among all above active serving cells and the deactivated SCell to be measured; NOTE 2: Nslotsubframe,μ is as defined in TS 38.211 [6]. |  |  |  |

Issue 3-3-6: Interruptions due to Active BWP switching Requirement

Agreements:

- Update Table 8.2.2.2.1-1 in TS 38.133 with 480/960 kHz subcarrier spacing as below:

Interruption length X

|  | NR Slot | Interruption length X (slots) |
| --- | --- | --- |
|  | length (ms) |  |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |
| 2 | 0.25 | 3 |
| 3 | 0.125 | 5 |
| 5 | 0.03125 | 17 |
| 6 | 0.015625 | 33 |
| Note1: void |  |  |

Issue 3-3-7: Interruptions due to UE-specific CBW change

Agreements:

- Update Table 8.2.2.2.1-1 in TS 38.133 with 480/960 kHz subcarrier spacing as below:

Interruption length X

|  | NR Slot | Interruption length X (slots) |
| --- | --- | --- |
|  | length (ms) |  |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |
| 2 | 0.25 | 3 |
| 3 | 0.125 | 5 |
| 5 | 0.03125 | 17 |
| 6 | 0.015625 | 33 |

Issue 3-3-8: Interruptions at NR SRS carrier based switching

Agreements:

- For Standalone NR Carrier Aggregation, when SRS carrier based switching is performed between carriers, the interruption requirements on any active serving cell can be shown in Table 1 and Table 2:

Table 1: Interruption length X1 (slot)

|  | NR Slot length | SRS carrier | Interruption length X1 (slots) |  |
| --- | --- | --- | --- | --- |
|  | (ms) of victim cell | switching time (us)Note 1 | Sub carrier spacing for agressor cell (kHz) |  |
|  |  |  | 15 | 30 |
| 0 | 1 | ≤ 200 | 2 | 2 |
|  |  | 300, 500 | 2 | 2 |
|  |  | 900 | 2 | 2 |
| 1 | 0.5 | ≤ 200 | 3 | 2 |
|  |  | 300, 500 | 3 | 3 |
|  |  | 900 | 4 | 4 |
| 2 | 0.25 | ≤ 200 | 4 | 3 |
|  |  | 300, 500 | 5 | 4 |
|  |  | 900 | 7 | 6 |
| 3 | 0.125 | ≤ 200 | 7 | 5 |
|  |  | 300, 500 | 9 | 7 |
|  |  | 900 | 12 | 10 |
| 5 | 0.03125 | ≤ 200 | 22 | 15 |
|  |  | 300, 500 | 31 | 24 |
|  |  | 900 | 44 | 37 |
| 6 | 0.015625 | ≤ 200 | 42 | 28 |
|  |  | 300, 500 | 61 | 47 |
|  |  | 900 | 87 | 73 |
| Note1: NR SRS carrier switching time is UE capability indicated by higher layer parameter SRS-SwitchingTimeNR. |  |  |  |  |

Table 2: Interruption length X2 (slot)

|  | NR Slot | SRS carrier | Interruption length X2 (slots) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | length (ms) of victim cell | switching time (us) Note 1 | Sub carrier spacing for agressor cell (kHz) |  |  |  |
|  |  |  | 60 | 120 | 480 | 960 |
| 0 | 1 | ≤ 200 | 2 | 2 | 2 | 2 |
| 1 | 0.5 | ≤ 200 | 2 | 2 | 2 | 2 |
| 2 | 0.25 | ≤ 200 | 3 | 3 | 2 | 2 |
| 3 | 0.125 | ≤ 200 | 4 | 4 | 3 | 3 |
| 5 | 0.03125 | ≤ 200 | 11 | 10 | 8 | 8 |
| 6 | 0.015625 | ≤ 200 | 21 | 18 | 15 | 15 |
| Note1: NR SRS carrier switching time is UE capability indicated by higher layer parameter SRS-SwitchingTimeNR. |  |  |  |  |  |  |

# Reference

[1] R4-2115405	, Email discussion summary: [100-e][235] NR_ext_to_71GHz_RRM_1, Qualcomm