3GPP TSG-RAN WG4 Meeting #100-e 												R4-2115623

Electronic Meeting, August 16-27, 2021

Agenda item:			9.2.1

Source:	Moderator (vivo)

Title:	Email discussion summary for [100-e][332] FR1_TRP_TRS_Part1

Document for:	Information

Introduction

This email summary covers the discussion for General aspects, SA test methodology and configuration, EN-DC test methodology and configuration of TRP TRS WI.

List of candidate target of email discussion for 1st round and 2nd round:

- 1st round: Finalize the updated workplan for TRP TRS WI, and discuss open issues.

- 2nd round: Make decisions on the open issues.

Topic #1: General and Work plan

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2112364 | Apple | Observation 1: The 3GPP-developed FR1 TRP/TRS conformance test requirements are expected to be reflected in the ETSI Harmonized Standard, to be used as the pass/fail limit to determine compliance with the European regulation. Observation 2: Market surveillance requirements associated with OTA pass/fail limits imply that any device out of the population of conforming terminals distributed in the market shall comply with the OTA pass/fail limits. Observation 3: Similarr to the European market, China's regulators have defined a framework to certify UEs based on OTA pass/fail limits, including provisions for market surveillance. Observation 4: When defining OTA requirements, 3GPP should consider not only the impact of measurement uncertainty but also the impact of manufacturing and assembly tolerances on the conformance test requirement due to the understanding that regulators assign pass/fail limits to this requirement with the expectation that any device available in the market shall comply. Proposal 1: RAN4 should discuss how to consider the impact of UE manufacturing and assembly tolerances in the OTA requirement specification work. Proposal 2: RAN4 should define the TRP/TRS requirements for NR FR1 UEs in stand-alone mode as a priority, although effort to define EN-DC test methodology can proceed in parallel with the SA work. Proposal 3: 3GPP should include scope for RedCap UEs in the FR1 TRP/TRS WID. |
| R4-2112861 | CMCC | Proposal#1. SA n41 PC2 TRP and TRS from operator’s side should be considered. Proposal 2 related to antenna switching ON/OFF is not discussed in this thread. |
| R4-2113975 | vivo | TP to TR 38.834 on general aspects |
| R4-2113977 | vivo | Proposal 1: Select one channel bandwidth per band for TRP and TRS tests for NR FR1. Default channel bandwidth is mid channel bandwidth defined in TS 38.508-1 Table 4.3.1.0A-1, but another bandwidth can be considered on case by case basis based on operators’ requests. Proposal 2: The low, mid, high, test frequencies for the selected mid CBW for each NR operating bands defined in Clause 4.3.1.1 in TS 38.508-1 should be measured for TRP and TRS. Proposal 3: The common test parameters summarized in Table 1 is recommended for FR1 TRP test. Table 1: Common parameters for TRP UL configuration CBW [MHz]; UL Modulation; SCS [kHz]; Allocation RB; RB Start; DL. 5; DFT-s-OFDM QPSK; 15; 12; 6; N/A. 10; 15; 25; 12. 15; 15; 36; 18. 20; 15; 50; 25. 40; 15; 108; 54. 50; 30; 64; 32. 60; 30; 81; 40. 100; 30; 135; 67. Proposal 4：For FR1 TRP OTA test, select the UL reference measurement channel configuration as in conductive Maximum Output Power test for NR FR1 per band, i.e. Annex A.2 UL reference measurement channels in TS 38.521-1. Proposal 5: The common test parameters summarized in Table 2 is recommended for FR1 TRS test. Table 2: Common parameters for TRS DL and UL CBW [MHz]; DL; UL Modulation; SCS [kHz]; DL Allocation RB; UL RB allocation. 5; CP-OFDM QPSK; DFT-s-OFDM QPSK; 15; FULL RB, in Table 7.3.2.4.1-2 in TS 38.521-1; Uplink configuration in Table 7.3.2.4.1-3 in TS 38.521-1. 10; 15. 15; 15. 20; 15. 40; 15. 50; 30. 60; 30. 100; 30. Proposal 6: For TRS test, select the reference measurement channel configuration for NR FR1 as defined for the conductive REFSENS minimum requirements and adopt UL allocation per band as defined in TS38.521-1 Table 7.3.2.4.1-3. Proposal 7: For FR1 TRP, adopt constant sample step of 15° both in theta () and phi () directions; for FR1 TRS, adopt constant sample step of 30° both in theta () and phi () directions. |
| R4-2113979 | vivo | Regular LS for the coordination between RAN4 and RAN5 on TRP TRS WI |
| R4-2113980 | vivo | Updated work Plan of TRP TRS WI |
| R4-2113985 | OPPO, vivo | Reserved |
| R4-2113911 | OPPO | Proposal: Configure the same RB numbers for both the centre and edge of the channel/band for TRP test. |
| R4-2114026 | Huawei Tech.(UK) Co.. Ltd | Proposal 1: reuse the configurations defined in section 6.2 and 7.3 of TS 38.521-1, but only test TRP and TRS under QPSK modulation. |
| R4-2112574 | Samsung | Proposal 2: RAN4 discuss framework for wide range band on how to select low/mid/high channels. One candidate is to adopt low/mid/high channels covering the entire frequency range. |

Open issues summary

Sub-topic 1-1 Workplan

Issue 1-1-1: Updated Workplan for FR1 TRP TRS WI

- Proposals

  - Proposal: Approve the updated workplan for FR1 TRP TRS WI in [R4-2113980]

- Recommended WF

Issue 1-1-2: Prioritization of TRP TRS WI

- Proposals

  - Proposal: RAN4 should define the TRP/TRS requirements for NR FR1 UEs in stand-alone mode as a priority, although effort to define EN-DC test methodology can proceed in parallel with the SA work.

- Recommended WF

Issue 1-1-3: TRP TRS working scope extension

- Proposals

  - Proposal: 3GPP should include scope for RedCap UEs in the FR1 TRP/TRS WID.

- Recommended WF

Sub-topic 1-2 General test parameters for TRP TRS

Issue 1-2-1: Channel Bandwidth for TRP TRS test

- Proposals

  - Proposal: Select one channel bandwidth per band for TRP and TRS tests for NR FR1. Default channel bandwidth is mid channel bandwidth defined in TS 38.508-1 Table 4.3.1.0A-1, but another bandwidth can be considered on case by case basis based on operators’ requests.

- Recommended WF

Issue 1-2-2: Test frequencies for each band

- Proposals

  - Proposal 1: The low, mid, high, test frequencies for the selected mid CBW for each NR operating bands defined in Clause 4.3.1.1 in TS 38.508-1 should be measured for TRP and TRS. (vivo)

  - Proposal 2: RAN4 discuss framework for wide range band on how to select low/mid/high channels. One candidate is to adopt low/mid/high channels covering the entire frequency range. (Samsung)

- Recommended WF

Issue 1-2-3: Measurement grid for TRP/TRS

- Proposals

  - Proposal: For FR1 TRP, adopt constant sample step of 15° both in theta () and phi () directions; for FR1 TRS, adopt constant sample step of 30° both in theta () and phi () directions.

- Recommended WF

Sub-topic 1-3 Common parameter for TRP

Issue 1-3-1: Common parameters for TRP UL

- Proposals

  - Proposal 1: The common test parameters summarized in Table 1 is recommended for FR1 TRP test. (vivo)

- Table 1: Common parameters for TRP UL configuration

| CBW [MHz] | UL Modulation | SCS [kHz] | Allocation RB | RB Start | DL |
| --- | --- | --- | --- | --- | --- |
| 5 | DFT-s-OFDM QPSK | 15 | 12 | 6 | N/A |
| 10 |  | 15 | 25 | 12 |  |
| 15 |  | 15 | 36 | 18 |  |
| 20 |  | 15 | 50 | 25 |  |
| 40 |  | 15 | 108 | 54 |  |
| 50 |  | 30 | 64 | 32 |  |
| 60 |  | 30 | 81 | 40 |  |

  - Proposal 2: Configure the same RB numbers for both the centre and edge of the channel/band for TRP test. (OPPO)

Proposal 3: reuse the configurations defined in section 6.2 of TS 38.521-1, but only test TRP under QPSK modulation. (Huawei)

- Recommended WF

Issue 1-3-2: TRP RMC

- Proposals

  - Proposal: For FR1 TRP OTA test, select the UL reference measurement channel configuration as in conductive Maximum Output Power test for NR FR1 per band, i.e. Annex A.2 UL reference measurement channels in TS 38.521-1.

- Recommended WF

Sub-topic 1-4 Common parameter for TRS

Issue 1-4-1: Common parameters for TRS DL and UL

- Proposals

  - Proposal 1: The common test parameters summarized in Table 2 is recommended for FR1 TRS test. (vivo)

- Table 2: Common parameters for TRS DL and UL

| CBW [MHz] | DL | UL Modulation | SCS [kHz] | DL Allocation RB | UL RB allocation |
| --- | --- | --- | --- | --- | --- |
| 5 | CP-OFDM QPSK | DFT-s-OFDM QPSK | 15 | FULL RB, in Table 7.3.2.4.1-2 in TS 38.521-1 | Uplink configuration in Table 7.3.2.4.1-3 in TS 38.521-1 |
| 10 |  |  | 15 |  |  |
| 15 |  |  | 15 |  |  |
| 20 |  |  | 15 |  |  |
| 40 |  |  | 15 |  |  |
| 50 |  |  | 30 |  |  |
| 60 |  |  | 30 |  |  |
| 100 |  |  | 30 |  |  |

Proposal 2: reuse the configurations defined in section 7.3 of TS 38.521-1, but only test TRS under QPSK modulation. (Huawei)

- Recommended WF

Issue 1-4-2: TRS RMC

- Proposals

  - Proposal: For TRS test, select the reference measurement channel configuration for NR FR1 as defined for the conductive REFSENS minimum requirements and adopt UL allocation per band as defined in TS38.521-1 Table 7.3.2.4.1-3.

- Recommended WF

Sub-topic 1-5 UE manufacturing impacts consideration

Issue 1-5: How to treat the UE manufacturing impacts

- Proposals

  - Proposal: RAN4 should discuss how to consider the impact of UE manufacturing and assembly tolerances in the OTA requirement specification work.

- Recommended WF

Sub-topic 1-6 Operator requests

Issue 1-6: Test requests from operator

- Proposals

  - Proposal. SA n41 PC2 TRP and TRS from operator’s side should be considered. (CMCC)

- Recommended WF

Companies views’ collection for 1st round

Open issues

Sub topic 1-1 Workplan

| Company | Comments |
| --- | --- |
| OPPO | Issue 1-1-1: Updated Workplan for FR1 TRP TRS WI Considering some controversial topics in core part may not be concluded completely by March of 2022, it is proposed that core part topics can be continuously considered afterward. Issue 1-1-2: Prioritization of TRP TRS WI Support the proposal. Issue 1-1-3: TRP TRS working scope extension RedCap UEs can be included in the working scope, but should be 2nd priority. |
| Qualcomm | Issue 1-1-2: Prioritization of TRP TRS WI As agreed in WID, both SA and NSA should be specified based on the operators’request. There is no need to prioritize SA since most of aspects on test method and requirement NSA should be reused between SA and NSA. Issue 1-1-3: TRP TRS working scope extension OK to consider RedCap but agree with OPPO that it should be 2nd priority. |
| MediaTek | Issue 1-1-2: Prioritization of TRP TRS WI Support the proposal. Issue 1-1-3: TRP TRS working scope extension OK to consider RedCap but agree with OPPO that it should be 2nd priority. |
| Huawei | Issue 1-1-1: Updated Workplan for FR1 TRP TRS WI Could MU part by RAN5 start in parallel ? Issue 1-1-2: Prioritization of TRP TRS WI Support the proposal Issue 1-1-3: TRP TRS working scope extension If extension is already being considered, taking on RedCap may exasperate the situation. Besides, the commercial availability of RedCap phones may be limited during this WI. Perhaps RedCap could be considered for R18. |
| Samsung | Issue 1-1-2: Prioritization of TRP TRS WI Support the proposal Issue 1-1-3: TRP TRS working scope extension We share similar view as Huawei. Considering the limitations on RAN4 resources and Redcap UE availability, maybe it is possible in next release. |
| Xiaomi | Issue 1-1-2: Prioritization of TRP TRS WI Support the proposal. This is actually our proposal last meeting. Issue 1-1-3: TRP TRS working scope extension Agree to include Redcap UEs in the scope. From our understanding, the test method should be the same, but the limit might be different so most of the work will be example tests. |
| CAICT | Issue 1-1-2: Prioritization of TRP TRS WI At the test methodology development stage, since most of aspects can be reused, there is no need to prioritize SA. At the performance requirement development stage, we can further discuss the prioritization of SA. Issue 1-1-3: TRP TRS working scope extension It is not sure that the discussion of RedCap UEs can be completed in FR1 TRP/TRS WI with the R17 timeline, so it could be the 2nd priority or be considered for R18. |
| vivo | Issue 1-1-1: Updated Workplan for FR1 TRP TRS WI Some feedback: The workplan follows the approved Time Budget of WI, whether the core part or performance part would be extended, that depends on RAN plenary decision. The MU part is started in RAN5 this meeting, so works in parallel with RAN4. Issue 1-1-2: Prioritization of TRP TRS WI Support the proposal Issue 1-1-3: TRP TRS working scope extension Share similar view with some companies, considering the tight timeline of the WI, further extension of the working scope within Rel-17 is not suggested. In addition, RedCap RF requirement discussion is just started in RAN4, after concluding the RF requirements, then consider the development of RedCap OTA requirements as next step in Rel-18 would be reasonable. |
| Apple | Issue 1-1-2: Prioritization of TRP TRS WI As the proponent, we support this approach. To Qualcomm: the proposed prioritization does not remove any objectives but allows the group to get started with the SA requirement work so that we have a fighting chance to try to conclude the SA OTA requirements within the Rel-17 timeframe. Issue 1-1-3: TRP TRS working scope extension As the proponent, we support the inclusion of RedCap into the scope of the work item. With many thanks to companies' comments above, we understand the concerns related to device availability. Perhaps one way forward could be to recommend to RAN to include the objective into the WID as a second priority within the Rel-17 timeframe. This way, if the work item is extended into Rel-18, the RedCap objective can be addressed on equal footing with with handheld UEs. |

Sub topic 1-2 General test parameters for TRP TRS

| Company | Comments |
| --- | --- |
| OPPO | Issue 1-2-1: Channel Bandwidth for TRP TRS test Support the proposal. Issue 1-2-2: Test frequencies for each band Support both Proposal 1 and Proposal 2. Actually, the low, middle, high test frequencies in 38.508-1 comply with the rule of covering the entire frequency range of the band. Issue 1-2-3: Measurement grid for TRP/TRS The proposed measurement grid for TRP TRS is agreeable. |
| R&S | Issue 1-2-1: Channel Bandwidth for TRP TRS test We agree with the proposal, although mid channel bandwidth in TS 38.508-1 lists multiple options depending on the band. Similar approach to LTE in TR 37.902 could be considered so the bandwidth is fixed for all bands to use one reference bandwidth (e.g. 10MHz for FDD bands and 20MHz for TDD bands). With this approach, test results (mostly for TRS) would be directly comparable to TR 37.902 / TS 37.544. There could be exemptions defined for some bands, to be considered on a case by case basis based on operators’ requests. Issue 1-2-2: Test frequencies for each band We support proposal 1 to follow TS 38.508-1. Issue 1-2-3: Measurement grid for TRP/TRS We support the proposal. It helps to keep consistency with the TS 37.902 and other standardization groups. |
| CMCC | Issue 1-2-1: Channel Bandwidth for TRP TRS test Considering actual commercial development, n41 CBW is proposed to be high test channel bandwidth as 38.508-1. |
| Huawei | Issue 1-2-1: Channel Bandwidth for TRP TRS test Support the proposal. Issue 1-2-2: Test frequencies for each band Support proposal 1. Issue 1-2-3: Measurement grid for TRP/TRS Support the proposal. |
| Samsung | Issue 1-2-1: Channel Bandwidth for TRP TRS test Generally fine with this proposal. Agree to select only one BW for low/mid/high channel. The mid channel BW defined in 38.508-1 is a promising candidate. Further check may be needed on Note1 of TS 38.508-1 Table 4.3.1.0A-1 (For UEs with limited UE channel bandwidth capability, if mid channel BW is not supported by the UE, select the closest channel BW to the average channel BW of all supported channel bandwidths among all SCSs. If there are two channel bandwidths that have same distance to the mathematical center, the higher one is selected. This shall apply only for Rel 15 UEs.) Especially for TRS, the BW parameter will directly affect final requirement. Issue 1-2-2: Test frequencies for each band Proposal 1 and 2 are not contradicted. We prefer to define low/mid/high channel covering the entire band (aligned with TS38.501-1). Issue 1-2-3: Measurement grid for TRP/TRS We support the proposal for consistency. |
| Xiaomi | Issue 1-2-1: Channel Bandwidth for TRP TRS test We support the proposal. Further consider if additional CBW is introduced per operator request, can we scale the limit or further work need to be done. Issue 1-2-2: Test frequencies for each band We also don’t see the conflict of Proposal 1 and 2. Agree with both of them. Issue 1-2-3: Measurement grid for TRP/TRS We support the proposal. |
| CAICT | Issue 1-2-1: Channel Bandwidth for TRP TRS test We generally agree with this proposal and support to follow TS38.501-1. We share the same view with R&S. Similar approach to LTE in TR 37.902 could be considered so the bandwidth is fixed for all bands. There could be exemptions defined for some bands, to be considered on a case by case basis based on operators’ requests. Issue 1-2-2: Test frequencies for each band We support proposal 1 to follow TS38.501-1. We also don’t see the conflict of Proposal 1 and 2. Issue 1-2-3: Measurement grid for TRP/TRS We support the proposal. |
| vivo | Issue 1-2-1: Channel Bandwidth for TRP TRS test The detailed bandwidth for some specific bands can be further discussed. Issue 1-2-2: Test frequencies for each band Support proposal 1, to align the test frequencies with conducted testing. Issue 1-2-3: Measurement grid for TRP/TRS. Support the proposal to reuse the measurement grid of LTE TRP TRS. |
| Apple | Issue 1-2-3: Measurement grid for TRP/TRS Support the proposal |

Sub topic 1-3 Common parameter for TRP

| Company | Comments |
| --- | --- |
| OPPO | Issue 1-3-1: Common parameters for TRP UL Proposal 1 is actually an example of Proposal 2, and can be agreed. Regarding Proposal 3, there are three DFT-s-OFDM QPSK cases required for testing, i.e. Inner Full, Inner 1RB Left, Inner 1RB Right. We propose only Inner Full case performed. Issue 1-3-2: TRP RMC Support the proposal. |
| R&S | Issue 1-3-1: Common parameters for TRP UL There seem to be overlap between the three proposals, although Proposal 1 is a more detailed definition of the test parameters. The following generic proposal could be considered under the assumption that the channel bandwidth / SCS per band has not been selected yet (related to Issue 1-2-1): NR FR1 Test parameters for TRP (UL) shall follow section 6.2.1.4.1 “Initial conditions” in TS 38.521-1, using only the following parameters (according to Test ID #4): DL configuration is N/A UL Modulation = DFT-s-OFDM QPSK UL RB allocation = Inner Full (as listed in Table 6.1-1 “Common uplink configuration”) It has to be noted that LTE test parameters are also required for EN-DC. Issue 1-3-2: TRP RMC We support the proposal. It has to be noted that LTE test parameters are also required for EN-DC. |
| CMCC | Issue 1-3-1: Common parameters for TRP UL In order to consider antenna performance in a large range frequency more fully, UL RB allocation is proposed that left channel uses Inner 1RB left, right channel uses Inner 1RB right and mid channel uses Inner full RB with 100MHz bandwidth. |
| Huawei | Issue 1-3-1: Common parameters for TRP UL Agree with Oppo’s comment on proposal 3 Issue 1-3-2: TRP RMC Support the proposal. |
| Samsung | Issue 1-3-1: Common parameters for TRP UL Agree with Inner_Full RB configuration and DFT-s-OFDM QPSK UL modulation Issue 1-3-2: TRP RMC Support the proposal. |
| CAICT | Issue 1-3-1: Common parameters for TRP UL Agree to use Inner Full RB configuration |
| vivo | Issue 1-3-1: Common parameters for TRP UL Support Proposal 1 to fix some common parameters this meeting. By default, Inner full is always adopted for OTA testing. Issue 1-3-2: TRP RMC Support the proposal |

Sub topic 1-4 Common parameter for TRS

| Company | Comments |
| --- | --- |
| OPPO | Issue 1-4-1: Common parameters for TRS DL Similar view with Issue 1-3-1. Support Proposal 1. Issue 1-4-2: TRS RMC Support the proposal. |
| R&S | Issue 1-4-1: Common parameters for TRS DL and UL Similar to issue Issue 1-3-1, the proposal could be generalized as follows until channel bandwidths and SCS are confirmed per band: NR FR1 Test parameters for TRS (DL) shall follow section 7.3.2.4.1 “Initial conditions” in TS 38.521-1, using only the following parameters (according to Test ID #4): DL Modulation = CP-OFDM QPSK DL RB allocation = Full RB (as listed in Table 7.3.2.4.1-2 “Downlink Configuration of each RB allocation”) UL Modulation = DFT-s-OFDM QPSK UL RB allocation = REFSENS (as listed in Table 7.3.2.4.1-3 “Uplink configuration for reference sensitivity”) It has to be noted that LTE test parameters are also required for EN-DC. Issue 1-4-2: TRS RMC To keep the consistency, RMC for TRS should refer to UL and DL reference measurement channel as specified in Annexes A.2.2.2, A.2.3.2, A.3.2 and A.3.3 in TS 38.521-1. It has to be noted that LTE test parameters are also required for EN-DC. |
| vivo | Issue 1-4-1: Common parameters for TRS DL Support the general common parameters in P1. It is important to align the common channel bandwidth and SCS mapping first, to avoid the discussion of detailed parameters per band. For specific band requested by operators, we can further retune the parameter next step through case by case manner. Issue 1-4-2: TRS RMC Agree with RS, both UL and DL should be considered. |

Sub topic 1-5 UE manufacturing impacts consideration

| Company | Comments |
| --- | --- |
| OPPO | Issue 1-5: How to treat the UE manufacturing This topic is supposed to be discussed in Performance Part of WI. Generally, it’s not only an issue of OTA test, but also for entire UE test industry. |
| R&S | Issue 1-5: How to treat the UE manufacturing We agree with OPPO. This proposal is related to requirements and thus to the Performance part of this WI |
| Qualcomm | Issue 1-5: How to treat the UE manufacturing Agree with OPPO and R&S that this should be discussed in performance part. Besides, it is not easy to specify the impact of UE manufacturing in the requirements since it depeds on many aspects. |
| Xiaomi | Issue 1-5: How to treat the UE manufacturing We share similar understanding that this proposal relates to all the requirements. RAN4 work is to define core requirements and we are not sure if this “test industry” issue can be concluded within RAN4. |
| CAICT | Issue 1-5: How to treat the UE manufacturing We share similar understanding with other companies. |
| vivo | Issue 1-5: How to treat the UE manufacturing Thanks for the paper from Apple, we believe this aspect should be further studied. More inputs from interested companies for next-step analyses are encouraged. |
| Apple | As the proponent of the proposal, we would like to at least enable the discussion in RAN4 how to address this issue, and we are fine to come back to this topic in the performance part of the work item. As we outlined in our contribution, the impact of the OTA core requirements, which RAN4 defines, eventually translates to regulation in some regions or countries, and the statistical significance of passing or failing the limit should be considered in the RAN4 work. This is definitely not a test industry issue but is a fundamental issue and is part of the meaning of "minimum core requirement." |

Sub topic 1-6 Operator requests

| Company | Comments |
| --- | --- |
| OPPO | Issue 1-6: Test requests from operator PC2 for n41 SA is already considered in the WID. |
| CMCC | Issue 1-6: Test requests from operator Band n41 for PC2 satisfies CMCC’s development requirement. |
| vivo | Issue 1-6: Test requests from operator In the WID, PC3 and PC2 are both included. Is the intension to prioritize n41 SA PC2 for next step? |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2113975 | Moderator: Postponed |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic 1-1 Workplan | Issue 1-1-1: Updated Workplan for FR1 TRP TRS WI Moderator: 2 companies (OPPO, Huawei) share views on the workplan, Rapporteur provide the feedback and clarification. Tentative agreements: - With the clarification feedback, the updated Workplan looks agreeable. Recommendations for 2nd round: - N/A Issue 1-1-2: Prioritization of TRP TRS WI Moderator: 7 companies (OPPO, MTK, Huawei, Samsung, Xiaomi, vivo, Apple) support to prioritize the SA part of Performance work. 2 companies (Qualcomm, CAICT) share negative views. Recommendations for 2nd round: - Further discuss to check whether group can reach consensus on some high-level guidance for this topic. Issue 1-1-3: TRP TRS working scope extension Moderator: 5 companies (OPPO, Qualcomm, MTK, Xiaomi, Apple) support to add RedCap UE into the WI scope. 4 companies (Huawei, Samsung, CAICT, vivo) think this work can be done within Rel-18 timeline. Recommendations for 2nd round: - Further discuss |
| Sub-topic 1-2 General test parameters for TRP TRS | Issue 1-2-1: Channel Bandwidth for TRP TRS test Moderator: Generally, all the companies support the proposal. However, some detailed configuration for specific band should be discussed case by case. One company indicates that n41 CBW is proposed to be high test channel bandwidth as 38.508-1. Agreements: - Select one channel bandwidth per band for TRP and TRS tests for NR FR1. Default channel bandwidth is mid channel bandwidth defined in TS 38.508-1 Table 4.3.1.0A-1, but another bandwidth can be considered on case by case basis based on operators’ requests. - Further check detailed parameters for specific band is needed. Recommendations for 2nd round: - N/A. Issue 1-2-2: Test frequencies for each band Moderator: Generally, all the companies support proposal 1, and some companies indicate that Proposal 2 is also not conflict with Proposal 1. Agreements: - The low, mid, high, test frequencies for the selected mid CBW for each NR operating bands defined in Clause 4.3.1.1 in TS 38.508-1 should be measured for TRP and TRS. - RAN4 can further discuss framework for wide range band on how to select low/mid/high channels. Recommendations for 2nd round: - N/A. Issue 1-2-3: Measurement grid for TRP/TRS Moderator: all the companies support the proposal. Agreements: - For FR1 TRP, adopt constant sample step of 15° both in theta () and phi () directions; for FR1 TRS, adopt constant sample step of 30° both in theta () and phi () directions. Recommendations for 2nd round: - N/A. |
| Sub-topic 1-3 Common parameter for TRP | Issue 1-3-1: Common parameters for TRP UL Moderator: generally, all the companies support proposal 1. 6 companies (OPPO, R&S, Huawei, Samsung, CAICT, vivo) support Inner Full UL RB allocation for TRP. 1 company (CMCC) suggest different RB allocation for Low/mid/high channel of single band. Tentative Agreements: - Table 1: Common parameters for TRP UL configuration CBW [MHz]; UL Modulation; SCS [kHz]; Allocation RB; RB Start; DL. 5; DFT-s-OFDM QPSK; 15; 12; 6; N/A. 10; 15; 25; 12. 15; 15; 36; 18. 20; 15; 50; 25. 40; 15; 108; 54. 50; 30; 64; 32. 60; 30; 81; 40. Recommendations for 2nd round: - Further discuss whether Inner Full can be adopted as the only RB allocation. Issue 1-3-2: TRP RMC Moderator: all the interested companies support the proposal. Agreements: - For FR1 TRP OTA test, select the UL reference measurement channel configuration as in conductive Maximum Output Power test for NR FR1 per band, i.e. Annex A.2 UL reference measurement channels in TS 38.521-1. Recommendations for 2nd round: - For EN-DC the LTE configuration is needed, but suggest to define it next meeting. - No 2nd round discussion for this issue. |
| Sub-topic 1-4 Common parameter for TRS | Issue 1-4: Common parameters for TRS DL and UL Moderator: compared with generalized parameter proposed by R&S, it would be good to define bandwidth and SCS mapping directly, i.e. follow Table 2 approach. Tentative Agreements: - Table 2: Common parameters for TRS DL and UL CBW [MHz]; DL; UL Modulation; SCS [kHz]; DL Allocation RB; UL RB allocation. 5; CP-OFDM QPSK; DFT-s-OFDM QPSK; 15; FULL RB, in Table 7.3.2.4.1-2 in TS 38.521-1; Uplink configuration in Table 7.3.2.4.1-3 in TS 38.521-1. 10; 15. 15; 15. 20; 15. 40; 15. 50; 30. 60; 30. 100; 30. - RMC for TRS should refer to UL and DL reference measurement channel as specified in Annexes A.2.2.2, A.2.3.2, A.3.2 and A.3.3 in TS 38.521-1. - LTE test parameters are also required for EN-DC Recommendations for 2nd round: - Confirm the above tentative agreements Issue 1-4-2: TRS RMC Tentative Agreements: - For TRS test, select the reference measurement channel configuration for NR FR1 as defined for the conductive REFSENS minimum requirements. - RMC for TRS should refer to UL and DL reference measurement channel as specified in Annexes A.2.2.2, A.2.3.2, A.3.2 and A.3.3 in TS 38.521-1. - LTE test parameters are also required for EN-DC Recommendations for 2nd round: - Confirm the above tentative agreements |
| Sub-topic 1-5 UE manufacturing impacts consideration | Issue 1-5: How to treat the UE manufacturing impacts Moderator: majority companies (OPPO, R&S, Qualcomm, Xiaomi, CAICT) think this is an issue of performance part. 2 companies (vivo, Apple) think we can discuss this issue at an early stage. Recommendations for 2nd round: - N/A |
| Sub-topic 1-6 Operator requests | Issue 1-6: Test requests from operator Moderator: n41 for PC2 is already included in the working scope. if the intention from operator is not the prioritization of this case, then no further 2nd round discussion is needed. Recommendations for 2nd round: - N/A |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Topic #2: SA test methodology

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2112363 | Apple | Observation 1: RAN4 should study whether the assumption D=300 mm is valid for higher frequencies. Observation 2: RAN4 should study whether the radiating structure needs to enclose the DUT and hand phantom for all test setups. Observation 3: RAN4 should define separate BHH and H requirements for devices with width ≤ 72 mm and width > 72 mm. |
| R4-2113976 | vivo | Proposal 1: Endorse the above calibration procedure for SA and EN-DC. Proposal 2: Endorse the above general TRP measurement procedure for SA. |
| R4-2114026 | Huawei Tech. (UK) Co.. Ltd | Proposal 2: SA TRP tests should be limited to single carrier only. Proposal 3: Multiple probe approach could be considered for measurement time reduction. |

Open issues summary

Sub-topic 2-1 General aspect for SA

Issue 2-1-1: SA TRP

- Proposals

  - SA TRP tests should be limited to single carrier only.

- Recommended WF

Issue 2-1-2: SA test setup

- Proposals

  - Single-antenna and multiple-antennas anechoic chambers can both be applied.

- Recommended WF

Sub-topic 2-2 Calibration procedure for SA and EN-DC

Issue 2-2: Common calibration procedure

- Proposals

  - Endorse the general calibration procedure for SA and EN-DC in R4-2113976

- Recommended WF

Sub-topic 2-3 Test procedure for SA TRP

Issue 2-3: SA TRP Test procedure

- Proposals

  - Endorse the general TRP measurement procedure for SA in R4-2113976

- Recommended WF

Sub-topic 2-4 	Phantoms impacts

Moderator: in the WF [R4-2108620], the measurement distance for TRP TRS was agreed:

- Measurement distance for FR1 TRP TRS test system

  - The effective antenna approach defined in TR 38.827 for RTS method should be adopted for FR1 TRP TRS testing, the minimum range length is suggested to be 1.2 m.

Issue 2-4-1: Whether radiating structure should enclose DUT and phantoms

- Proposals

  - RAN4 should study whether the radiating structure needs to enclose the DUT and hand phantom for all test setups.

- Recommended WF

Issue 2-4-2: Whether different use scenario should be defined for devices with different width

- Proposals

  - RAN4 should define separate BHH and H requirements for devices with width ≤ 72 mm and with width > 72 mm.

- Recommended WF

Companies views’ collection for 1st round

Open issues

Sub topic 2-1 General aspect for SA

| Company | Comments |
| --- | --- |
| OPPO | Issue 2-1-1: SA TRP SA with 1cc is the 1st priority, which is stated in the WID. Issue 2-1-2: SA test setup Agree with the proposal. Leave the flexibility to the implementation. |
| Qualcomm | Issue 2-1-1: SA TRP OK with proposal |
| Samsung | Issue 2-1-1: SA TRP Support the proposal. Issue 2-1-2: SA test setup Support the proposal.. |
| CAICT | Issue 2-1-1: SA TRP Support the proposal. Issue 2-1-2: SA test setup Support the proposal. |
| vivo | Issue 2-1-1: SA TRP Support the proposal. This is already agreement in the WID. Issue 2-1-2: SA test setup Given the test setup for LTE TRP TRS will be reused as much as possible, the multi-probe and single-antenna system can both be adopted for TRP TRS test. |
| Apple | Issue 2-1-1: SA TRP Support the proposal to test single carrier only |

Sub topic 2-2 Calibration procedure for SA and EN-DC

| Company | Comments |
| --- | --- |
| OPPO | Issue 2-2: Common calibration procedure The basic calibration procedure is better to be complied with TR 25.914 and TR 37.902. |
| R&S | Issue 2-2: Common calibration procedure The high-level description of the calibration procedure in R4-2113976 seems fine, but it requires further improvements: - The wording in TR 25.914, clause 7.3, gives some more details on the background for such a method (i.e. gain transfer method). It is recommended to use most parts of the wording there. - Dipole is referred in many places as the reference antenna, although other types are also used in OTA test systems. The wording need to be generalized. - In the same way, the usage of a VNA is assumed although combinations of generator/receiver could be also used. |
| vivo | Issue 2-2: Common calibration procedure This is general common calibration procedure which needs more details, if translated to Text Proposals to TR. Agree the comments above, the intension is to further refine the procedure, given the content in 25.914 has been so many years, some update and refined wording for NR TRP TRS maybe better for readers. |

Sub topic 2-3 Test procedure for SA TRP

| Company | Comments |
| --- | --- |
| OPPO | Issue 2-3: SA TRP Test procedure The proposed test procedure is acceptable. Detail refinement is needed. |
| R&S | Issue 2-3: SA TRP Test procedure We agree with OPPO. Text seems fine from high level perspective but need further refinement. |
| Qualcomm | Issue 2-3: SA TRP Test procedure The test procedure looks good. Further refinement might be needed. |

Sub topic 2-4 Phantoms impacts

| Company | Comments |
| --- | --- |
| OPPO | Issue 2-4-1: Whether radiating structure should enclose DUT and phantoms FFS is needed. Issue 2-4-2: Whether different use scenario should be defined for devices with different width For current CTIA hand phantoms for DUT width <=72mm and >72mm, OTA performance gap can not be ignored. Therefore, separate requirement for DUT width <=72mm and >72mm is reasonable. |
| R&S | Issue 2-4-1: Whether radiating structure should enclose DUT and phantoms The typical approach for OTA test systems is to consider a test volume that fully encompasses the DUT and phantoms, and the actual test volume is defined by the positions used for the Quality of Quiet Zone measurements. That shouldn’t be mixed up with the radiating aperture considered for the minimum measurement distance determination. For the latter, the moderator comment shows the reference to the agreed effective antenna approach for minimum range length reused from TR 38.827. Issue 2-4-2: Whether different use scenario should be defined for devices with different width Similar to Issue 1-5, this one is related to the requirement definition and thus should be treated under the Performance phase of this WI. Even though, hand phantoms grip for devices >72mm width is different from that one defined ≤72mm. |
| Apple | Issue 2-4-1: Whether radiating structure should enclose DUT and phantoms As the proponent of this proposal, we suggest RAN4 should study whether the radiating structure needs to enclose the DUT and hand phantom for all test setups to ensure that we correctly specify the test volume for hand phantom test cases. Issue 2-4-2: Whether different use scenario should be defined for devices with different width As the proponent of this proposal, we support and don't see any harm in reaching an agreement. Indeed, this aspect allows us to structure the performance part of the work related to hand phantom test cases. |

CRs/TPs comments collection

For the suggested wording of reply LS, please share comments in the table below.

| CR/TP number | Comments collection |
| --- | --- |
| xx | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| 2.2.1 Sub-topic 2-1 General aspect for SA | Issue 2-1-1: SA TRP Agreements: - SA TRP tests should be limited to single carrier only Recommendations for 2nd round: - N/A Issue 2-1-2: SA test setup Agreements: - Single-antenna and multiple-antennas anechoic chambers can both be applied. Recommendations for 2nd round: - N/A |
| 2.2.2 Sub-topic 2-2 Calibration procedure for SA and EN-DC | Issue 2-2: Common calibration procedure Moderator: the general procedure content is acceptable, but needs further improvement. Tentative agreement: - Select the general procedure outlined in R4-2113976 as a starting point. Further improve the wording. Recommendations for 2nd round: - Detailed suggestions on improvement of the wording is needed. Then, a corresponding TP will be prepared next RAN4 meeting. |
| 2.2.3 Sub-topic 2-3 Test procedure for SA TRP | Issue 2-3: SA TRP Test procedure Tentative Agreements: - General test procedure in R4-2113976 as a starting point. Further improve the wording. Recommendations for 2nd round: - Detail refinement suggestion is encouraged. |
| 2.2.4 Sub-topic 2-4 Phantoms impacts | Issue 2-4-1: Whether radiating structure should enclose DUT and phantoms Recommendations for 2nd round: - Further discuss Issue 2-4-2: Whether different use scenario should be defined for devices with different width Recommendations for 2nd round: - Further discuss |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Topic #3: EN-DC test method

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2112249 | Qualcomm Incorporated | Proposal 1: Fixed power splitting between LTE and NR is adopted for EN-DC TRP and TRS measurement. Proposal 2: The scheme of 50%-50% power splitting with only fixed 50% power for LTE, e.g., for PC3, 20dBm LTE and no upper power limit setting for NR is adopted for EN-DC TRP measurement. Proposal 3: It is proposed to adopt the same power splitting scheme for EN-DC TRP and TRS measurements. |
| R4-2112362 | Apple | Proposal 1: Methods for measuring the TRP of UEs which employ dynamic power sharing are not in the scope of this work item Proposal 2: The EN-DC power configuration for the TRP/TRS test shall follow the RAN5 LTE anchor agnostic approach. |
| R4-2112574 | Samsung | Proposal 1: adopt Option 2a (maximize NR power) for ENDC SISO OTA test. |
| R4-2112607 | Xiaomi | Observation 1: The LTE uplink coverage is equally important to NR uplink coverage. Observation 2: There is TRP difference for same NR band with different EN-DC combinations. Observation 3: To fully consider the impact between LTE and NR branch, the significant power difference is not considerable. Proposal 1: It is proposed to use the option 1-b as EN-DC power configuration. |
| R4-2113912 | OPPO | Proposal: Get aligned on the understanding of fixed power splitting and dynamic power sharing, i.e. a UE indicating a capability for dynamic power sharing between E-UTRA and NR for EN-DC is in the scope of DUT using fixed power splitting approach. |
| R4-2113978 | vivo | Observation 1: Based on RAN plenary decision, only NR carrier should be measured under EN-DC mode. Proposal 1: For EN-DC power splitting, select Option 1b: rough 50%-50% power splitting with only fixed 50% power for LTE, e.g., for PC3, 20dBm LTE and no upper power limit setting for NR TRP. Proposal 2: EN-DC Tx Power splitting for TRP and TRS testing should be configured as the same. Proposal 3: Special OTA test method for DPS function is not precluded but can be further discussed as 2nd priority. |
| R4-2113983 | OPPO | TP to TR 38.834 |
| R4-2114530 | Huawei, HiSilicon | Observation1: For conducted test, FR1 inter-band EN-DC uses 50%-50% power splitting for the test cases with simultaneous E-UTRA and NR transmission. Proposal 1: For FR1 EN-DC TRP test, Power splitting between LTE and NR uses similar configurations as conducted test of UE maximum output power which is p-NR-FR1 = p-MaxEUTRA-r15 = 20 for Power Class 3 UE, p-NR-FR1 = p-MaxEUTRA-r15 = 23 for Power Class 2 UE, i.e. option 1a in the WF. Poaposal2: It is not necessary to develop OTA test method dedicated to quantify the DPS function of FR1 UEs. Poaposal3: Adopt same EN-DC Tx Power splitting for TRP and TRS test. |

Open issues summary

Sub-topic 3-1 EN-DC power splitting

Moderator: Agreed in the WF [R4-2108620]:

- Power splitting between LTE and NR

  - For EN-DC OTA testing, the fixed-power-splitting approach should be configured, the following options can be considered:

    - Option 1: UE transmit LTE and NR with a 50%-50% equal power splitting under EN-DC mode.

      - Option 1a: exact 50%-50% power splitting with fixed 50% power for each RAT, e.g. for PC3, 20 dBm LTE and 20 dBm NR

      - Option 1b: rough 50%-50% power splitting with only fixed 50% power for LTE, e.g., for PC3, 20dBm LTE and no upper power limit setting for NR

    - Option 2: UE transmit a significant different power for LTE and NR under EN-DC mode

      - Option 2a: maximum power for NR and minimized power for LTE (stable LTE connection should be confirmed with, e.g. 10dBm UL power)

      - Option 2b: maximum power for LTE and minimized power for NR (stable NR connection should be confirmed with, e.g. 10dBm UL power)

    - Option 3: other configuration is not precluded

Issue 3-1-1: EN-DC power splitting

- Proposals

  - Proposal 1: The scheme of 50%-50% power splitting with only fixed 50% power for LTE, e.g., for PC3, 20dBm LTE and no upper power limit setting for NR is adopted for EN-DC TRP measurement. (QC, Xiaomi, vivo)

  - Proposal 2: The EN-DC power configuration for the TRP/TRS test shall follow the RAN5 LTE anchor agnostic approach. (Apple)

  - Proposal 3: adopt Option 2a (maximize NR power) for ENDC SISO OTA test. (Samsung)

  - Proposal 4: For FR1 EN-DC TRP test, Power splitting between LTE and NR uses similar configurations as conducted test of UE maximum output power which is p-NR-FR1 = p-MaxEUTRA-r15 = 20 for Power Class 3 UE, p-NR-FR1 = p-MaxEUTRA-r15 = 23 for Power Class 2 UE, i.e. option 1a in the WF. (Huawei)

- Recommended WF

Issue 3-1-2: UL power configuration for TRP and TRS

- Proposals

  - Proposal: It is proposed to adopt the same power splitting scheme for EN-DC TRP and TRS measurements. (QC, vivo, Huawei, HiSilicon)

- Recommended WF

Sub-topic 3-2 DPS test

Issue 3-2: DPS

- Proposals

  - Proposal 1: Methods for measuring the TRP of UEs which employ dynamic power sharing are not in the scope of this work item. (apple)

  - Proposal 2: It is not necessary to develop OTA test method dedicated to quantify the DPS function of FR1 UEs. (Huawei)

  - Proposal 3: Get aligned on the understanding of fixed power splitting and dynamic power sharing, i.e. a UE indicating a capability for dynamic power sharing between E-UTRA and NR for EN-DC is in the scope of DUT using fixed power splitting approach. (OPPO)

  - Proposal 4: Special OTA test method for DPS function is not precluded but can be further discussed as 2nd priority. (vivo)

- Recommended WF

Companies views’ collection for 1st round

Open issues

Sub topic 3-1 EN-DC power splitting

| Company | Comments |
| --- | --- |
| OPPO | Issue 3-1-1: EN-DC power splitting Support proposal 1. Roughly equal power splitting is more reasonable case in the real network, besides, more NR radiation power is expected. Issue 3-1-2: UL power configuration for TRP and TRS Support the proposal. |
| R&S | Issue 3-1-1: EN-DC power splitting We support Proposals 1 or 4, i.e. options 1a and 1b from WF [R4-2108620]. In case Proposal 3 is agreed (i.e. option 2a from WF), it has to be ensured that LTE connection is kept stable, and thus set the LTE link to be configured at ≤10dBm. Issue 3-1-2: UL power configuration for TRP and TRS We support the proposal. |
| Qualcomm | Issue 3-1-1: EN-DC power splitting Support proposal 1. Even though it was agree that 3GPP would not define LTE requirements for EN-DC, proposal 1 is a general approach which can be used for other regions. Issue 3-1-2: UL power configuration for TRP and TRS Support the proposal. |
| MediaTek | Issue 3-1-1: EN-DC power splitting Fine for proposal 1. Issue 3-1-2: UL power configuration for TRP and TRS Fine for the proposal. |
| Samsung | Issue 3-1-1: EN-DC power splitting We support proposal 3 and also fine with proposal 2. LTE TRP has been measured in LTE standalone mode. For NR TRP, the power of NR side should be emphasized. Considering different power classes and different UE architectures, equal power splitting will make the configuration vague and fragmented in standard. So an approach focusing on NR power is preferred. Issue 3-1-2: UL power configuration for TRP and TRS It is preferred to discuss UL configuration for TRS after ENDC power splitting issue is finalized.. |
| Xiaomi | Issue 3-1-1: EN-DC power splitting Support proposal 1 as already stated in our discussion paper. Issue 3-1-2: UL power configuration for TRP and TRS Fine for the proposal. |
| Huawei, HiSilicon | Issue 3-1-1: EN-DC power splitting We support proposal 4. Regarding proposal 1, it is not clear to us why “no upper power limit setting for NR” dedicated for OTA? We prefer consistent setting with conducted test. |
| CAICT | Issue 3-1-1: EN-DC power splitting Support proposal 1. 50%-50% power splitting is reasonable in the real network. Issue 3-1-2: UL power configuration for TRP and TRS Support the proposal. |
| vivo | Issue 3-1-1: EN-DC power splitting Support proposal 1. Issue 3-1-2: UL power configuration for TRP and TRS For TRP and TRS test, the UL configuration should be the same. Support the proposal. |
| AT&T | Issue 3-1-1: EN-DC power splitting We support proposal 3 to focus the results on NR. Adopting 50/50 power split in OTA test environment would require p-Max signalling which results in an additional uncertainty in the results. Issue 3-1-2: UL power configuration for TRP and TRS We agree with Samsung that the UL configuration for TRS should be discussed after the ENDC power splitting issue is finalized. |
| Apple | Issue 3-1-1: EN-DC power splitting As the proponent of Proposal 2, we respectfully urge companies who are supporting the 50/50 split to reconsider their position. In our understanding, the objective of the EN-DC TRP test is to measure the radiated power of the FR1 carrier in order to define a minimum requirement on the design of the corresponding antenna. A 50/50 split between the LTE and NR CCs needlessly introduces complexity into the test setup (how to ensure 50/50 split is actually maintained, since we agreed in RAN #92 not to measure the LTE carrier and also since the link antenna paths is not calibrated in the test setup?) and can introduce conditions for the prioritization of transmission power reduction (see 38.213). All of these aspects abstract the meaning of the test from the original intent of defining a metric on the antenna performance. Some NR capable UEs might support cetain NR bands only in EN-DC configurations, and this test case should be seen as a way to enable the TRP and TRS tests of the NR carrier. The simplest solution is to reuse the already RAN5-defined anchor agnostic approach. |

Sub topic 3-2 DPS test

| Company | Comments |
| --- | --- |
| OPPO | Issue 3-2: DPS For DPS, two questions should be answered. The first one, is the DUT indicating a capability of DPS in the scope of WID? Our view is that this kind of DUT should be included in the scope. Actually, most commercial UEs already configured with DPS. And it does not affect current power configuration discussion. Secondly, is specific OTA method for DPS function validation needed? Our position is negative, because DPS function can be easily validated through conductive test. |
| Qualcomm | Issue 3-2: DPS We support proposal 1 and 2. TRP testing targets to verify the antenna efficiency. DPS could be verified in conductive testing. |
| MediaTek | Issue 3-2: DPS Support ”Proposal 2: It is not necessary to develop OTA test method dedicated to quantify the DPS function of FR1 UEs.” |
| Samsung | Issue 3-2: DPS We share similar view as OPPO. if UE supporting DPS is in scope and will be measured with fix power splitting, then it is not necessary to develop OTA method to verify DPS function. |
| Xiaomi | Issue 3-2: DPS Support proposal 2. |
| vivo | Issue 3-2: DPS In our understanding, given fixed power splitting is configured, there is no DPS during SISO OTA testing, no matter the device support DSP or not. The fixed power splitting based test method is the basic approach to test NR carrier performance under EN-DC mode. If RAN4 has interests to develop test method to identify the DPS function, this special OTA test method can be studied as 2nd priority or further studied in Rel-18 timeline. |
| Apple | Issue 3-2: DPS As the proponent of the proposal, we support Proposal 1. We see convergence between our proposal and Proposal 2, although we should be clear that RAN4 will not define a new test methods for DPS UEs and also will not define different OTA requirements for DPS UEs. |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2113983 | Moderator: Postponed |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| 3.2.1 Sub-topic 3-1 EN-DC power splitting | Issue 3-1-1: EN-DC power splitting Moderator:7 companies (OPPO, R&S, Qualcomm, MKT, Xiaomi, CAICT, vivo) support Proposal 1. 2 companies (Samsung, AT&T) support Proposal 3. Two companies (Apple, Samsung) support Proposal 2. Two companies (Huawei, R&S) support Proposal 4. Recommendations for 2nd round: - Further discuss Issue 3-1-2: UL power configuration for TRP and TRS Moderator:7 companies (OPPO, R&S, Qualcomm, MKT, Xiaomi, CAICT, vivo) support Proposal 1. 2 companies (Samsung, AT&T) suggest to make decision on EN-DC power splitting first. Recommendations for 2nd round: - Further discuss |
| 3.2.2 Sub-topic 3-2 DPS test | Issue 3-2: DPS Moderator: majority view believes special OTA test method to quantify DPS function and performance is not needed. Tentative agreement: - RAN4 will not define a new OTA test method to quantify DPS function and performance. In addition, no new requirements will be defined for DPS UEs. Recommendations for 2nd round: - Confirm the above statement as agreements. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Topic #4: Reply LS from CTIA

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2114591 | CTIA Certification | Introduction CTIA Certification would like to thank 3GPP RAN4 for their June 10, 2021 LS on Head and Hand Phantoms for 5G FR1 OTA Testing in R4-2108621. CTIA Certification is interested in cooperating with 3GPP on the development of harmonized OTA testing methods for the industry. Response It could be possible for 3GPP to reference the phantoms within the CTIA Certification OTA test plan. A no-cost non-exclusive license agreement could be created giving 3GPP the permission to reference the appropriate sections of the test plan along with “©CTIA Certification. Referenced with permission.” In addition to the head and hand phantoms, CTIA Certification has defined a forearm phantom as well. Action If a license agreement is of interest to 3GPP, CTIA Certification requests 3GPP to please identify the specific phantoms 3GPP would like to reference. |

Open issues summary

Moderator: The reply LS R4-2114591 from CTIA on Head and Hand Phantoms for 5G FR1 OTA Testing is received.

Sub-topic 4-1 Permission from CTIA

Issue 4-1-1: A no-cost non-exclusive license agreement

- Whether a no-cost non-exclusive license agreement with “©CTIA Certification. Referenced with permission.” can be adopted by 3GPP

  - WI Rapporteur will further check with MCC

- Recommended WF

  - No discussion in RAN4. WI rapporteur will contact 3GPP MCC for the confirmation.

Sub-topic 4-2 Specific phantom to reference

Issue 4-2-1: Specific phantom to reference

- Initial views on the phantoms selection for reference.

- Recommended WF

  - Companies can share view on the above aspect.

Companies views’ collection for 1st round

Open issues

Sub topic 4-2-1 Specific phantom to reference

| Company | Comments |
| --- | --- |
| OPPO | Head phantom and hand phantoms for speech mode and browsing mode is needed. Other phantoms are FFS. |
| R&S | For phantoms already defined in 3GPP TR 25.914, this group shall decide whether to reuse that information or introduce references to phantom definition in CTIA OTA Test Plan seeking harmonization among test specifications. For phantoms not included in 3GPP TR 25.914, the reference to wide grip hand phantom (i.e. devices 72mm < width ≤ 92mm) is required. Reference to other phantoms (e.g. forearm) is only required in case IoT devices are considered in the scope of this WI. |
| vivo | In the WID the follow device types are considered: - Smartphone - Considering UEs with antenna configurations of 1Tx, 2Tx, 2 Rx and 4 Rx - Tablet - Laptop embedded equipment (LEE) - Laptop mounted equipment (LME) Given Redcap UEs are not within the scope of Rel-17 TRP TRS WI, forearm phantom for wearable device will not be used. Based on the agreed use scenarios in the WF R4-2108620, currently only head and hand phantoms are needed to reference. Forearm phantom can be considered when redcap UEs are within the scope or next release. |
| Apple | We would like to find a way to correctly refer to all applicable phantoms (those which are already captured in TR 25.914, wide grip, and forearm). The forearm phantom is relevant for RedCap UEs, if this scope could be agreed to be added to the work item. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub topic 4-2-1 Specific phantom to reference | Sub topic 4-2-1 Specific phantom to reference Moderator: Based on companies view, at least head and hand phantoms should be used. Regarding forearm phantom, there is no aligned understanding. Tentative agreements: - At least head and hand phantoms should be referenced. Recommendations for 2nd round: - Further discuss how to treat other phantoms. - Further discuss whether to reuse partial phantom information defined in 3GPP TR 25.914. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on FR1 TRP TRS | vivo | General WF for the WI |
| TP work split for TR 38.834 drafting | vivo, CAICT, Samsung, Qualcomm, Apple, Huawei, HiSilicon, OPPO, Xiaomi, R&S, MVG |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2112249 | Discussion on EN-DC test methodology | Qualcomm Incorporated | Noted |  |
| R4-2112362 | EN-DC power configuration for TRP/TRS | Apple | Noted |  |
| R4-2112363 | Views on test cases which employ hand phantoms | Apple | Noted |  |
| R4-2112364 | General considerations of OTA requirements | Apple | Noted |  |
| R4-2112574 | Discussion on power-split and measurement channels | Samsung | Noted |  |
| R4-2112607 | on EN-DC power setting | Xiaomi | Noted |  |
| R4-2112861 | Requirements of NR Band n41 Test Configuration and Multiple Antenna Test Methodology | CMCC | Noted |  |
| R4-2113911 | Parameter configuration for SA TRP | OPPO | Noted |  |
| R4-2113912 | Power split and DPS | OPPO | Noted |  |
| R4-2113975 | TP to TR38.834 on general aspects | vivo | Return to |  |
| R4-2113976 | Discussion on SA test method | vivo | Noted |  |
| R4-2113977 | Views on general aspects of TRP TRS WI | vivo | Noted |  |
| R4-2113978 | Discussion on EN-DC test method | vivo | Noted |  |
| R4-2113979 | LS to RAN5 on FR1 TRP TRS WI progress (RAN4#100e) | vivo | Revised |  |
| R4-2113980 | Updated workplan of TRP TRS WI | vivo | Approved |  |
| R4-2113983 | TP for TR 38.834 v0.1.0 | OPPO | Return to |  |
| R4-2113985 | TR 38.834 v0.1.0 | OPPO, vivo | Return to |  |
| R4-2114026 | On FR1 SA TRP-TRS test | Huawei Tech.(UK) Co.. Ltd | Noted |  |
| R4-2114530 | on FR1 EN-DC TRP-TRS test methodology | Huawei, HiSilicon | Noted |  |
| R4-2114591 | Reply LS on Head and Hand Phantoms for 5G FR1 OTA Testing | CTIA Certification | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents