3GPP TSG-RAN WG4 Meeting #100-e	R4-2115632

Electronic Meeting, 16 August – 27 May 2021

Agenda Item:	8.3.5

Source:	Ericsson

Title:	Way forward on UE demodulation on NR 47GHz band

Document for:	Approval

# Way forward

Applicability of 64QAM Rank 2 (TS38.101-4 Table 7.2.2.2.1-4 Test 2-6) and 16QAM Rank 1 with Enhanced Receiver Type 1 (TS38.101-4 Table 7.2.2.2.1-5 Test 3-1)

- 64QAM Rank 2 (TS38.101-4 Table 7.2.2.2.1-4 Test 2-6) is applicable up to 48200MHz including band n262 with extra margin: 0.5dB

- Apply the same applicability to 16QAM Rank 1 with Enhanced Receiver Type 1 (TS38.101-4 Table 7.2.2.2.1-5 Test 3-1)

Applicability of 256QAM Rank 1 (TS38.101-4 Table 7.2.2.2.1-3 Test 1-4)

- 256QAM Rank 1 test is applicable to up 48200MHz (including band n262) with extra margin. FFS for the extra margin.

  - Testability depends on the test system.

  - Companies are encouraged to study further to decide the required extra margin with the following assumptions:

    - Phase noise models:

      - PN model Example 2 in TR 38.803

      - Interesting companies can also provide evaluations for other PN models, such as PN model in R4-2010176

    - Carrier frequency: 48.2GHz vs. 39GHz

    - Phase noise compensation at the UE receiver

      - CPE compensation