3GPP TSG-RAN WG4 Meeting #100-e                	 R4-2115732

Electronic Meeting, Aug 16-27, 2021

Agenda Item:	9.12.2.1.2

Source:	Ericsson

Title:	WF on CSI requirements for inter-cell interference MMSE-IRC

Document for:	Approval

# Introduction

This is the WF to capture all agreements and open issues in [100-e][328] for WF on CSI requirements for inter-cell interference MMSE-IRC in RAN4#100e meeting.

# CSI requirements

## Issue 1-1: Whether to define CQI reporting requirements

- Agreement:

  - Define CQI reporting requirements for MMSE-IRC receiver for inter-cell interference

Issue 1-2: Whether to define PMI reporting requirements

- Option 1: Yes

- Option 2: Need further discussion

- Option 3: No

# Interference Model

Issue 2-1: CSI-IM on target cell

- Agreement:

  - CSI-IM overlaps with PDSCH from interference is the baseline scenario for defining CSI reporting requirement.

  - Other scenarios are FFS.

    - Option 1: Only consider overlapping with PDSCH from interference

    - Option 2: Further include overlapping with NZP CSI-RS from interference

    - Option 3: Further include overlapping with Interf. cell CSI-IM

    - Option 4: Make further down selection based on simulation results and also take practical network configuration into account.

      - If using target CSI-IM overlaps with neighbor NZP CSI-RS, target CSI-IM port number (X) shall be equal to neighbor cell NZP CSI-RS port number

Issue 2-2: NZP CSI-RS on target cell

- Option 1: Overlaps with PDSCH from interference

- Option 2: Overlaps with NZP CSI-RS from interference

- Option 3: Make further down selection based on simulation results and also take practical network configuration into account.

  - if using target NZP CSI-RS overlaps with neighbor NZP CSI-RS, same port number for the 2 cells shall be configured.

# Simulation assumption for CQI requirements

Issue 3-1: Test configuration

- Agreement:

- Serving cell

  - Reuse assumptions from Sections 6.2.2.1.2.1, 6.2.2.2.2.1, 6.2.2.2.2.1, 6.2.3.2.2.1 in TS 38-101-4.

- Interference cell

  - 1 TX with static channel

  - NZP CSI-RS and CSI-IM is based on outcome of Issue 2-2-1 and Issue 2-2-2

  - INR configuration

    - Option 1: 10.04 dB (DIP -0.41dB)

    - Other options are not precluded

      - Based on the discussion of Deployment and Number of explicitly modeled interference cells in MMSE-IRC receiver for inter-cell interference – Demodulation requirements.

# Reference

[1] R4-2108665, “WF on CQI reporting requirements for inter-cell interference MMSE-IRC”, Ericsson, RAN4#99e, May’21