3GPP TSG-RAN WG4 Meeting # 100-e 					R4-2115792

Electronic Meeting, 16 – 27 August 2021

Agenda item:			5.1.9.1, 5.1.9.2, 5.2.2.4.1, 6.1.9.4.1, 6.1.3.3, 6.1.5.2

Source:	Moderator (Apple)

Title:	Email discussion summary for [99e][320] Demod_Maintenance_UE

Document for:	Information

# Introduction

In this email thread for UE Demod Maintenance, the following topics will be covered:

- Rel-15 NR UE Demodulation and CSI requirements maintenance (5.1.9.1, 5.1.9.2)

- Rel-15 LTE UE Demodulation and CSI requirements maintenance (5.2.2.4.1)

- Rel-16 NR V2X Demodulation maintenance (6.1.3.3)

- Rel-16 NR eMIMO Demodulation maintenance (6.1.5.2)

- Rel-16 NR UE Demodulation and CSI requirements maintenance (6.1.9.1.4.1)

# Topic #1: Rel-15 NR UE Demodulation and CSI requirements maintenance

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2111874 | Anritsu Corporation | Draft CR: Introduction of a new propagation definition Added TDLA30-70Hz to Table B.2.2-1. |
| R4-2111875 | Anritsu Corporation | Draft CR: Introduction of a new propagation definition Cat-A CR |
| R4-2111876 | Anritsu Corporation | Draft CR: Introduction of a new propagation definition Cat-A CR |
| R4-2111892 | Anritsu Corporation | TDD UL-DL pattern alignment for intra-band EN-DC performance TC Observation 1: Currently only the performance specification (i.e. TS 38.101-4) does not take into account of the TDD UL-DL pattern for intra-band EN-DC (TDD band). Proposal 1: RAN4 clarifies a policy on the tests of intra-band EN-DC for TDD bands whether they are to be measured or not. Proposal 2: In a case RAN4 decides that intra-band EN-DC tests should be carried out, then clarify another policy on a treatment of the TDD patterns for NR, especially with which cannot be aligned with LTE. (e.g. FR1.30-6) Option 1: For TDD intra-band EN-DC case, use the UL-DL configuration FR1.30-4 as a substitute for other configurations which cannot align the UL-DL pattern between LTE and NR. Whether it is possible to change SNR or modulation is FFS. Option 2: Skip the corresponding test configurations in case it is not possible to align the UL-DL configuration between LTE and NR. |
| R4-2111893 | Anritsu Corporation | Draft CR: CR to RI reporting parameter settings Corrected typo on CSI-IM resource Type: Periodic -> Aperiodic in Table 8.4.2.2-1 |
| R4-2111894 | Anritsu Corporation | Draft CR: CR to RI reporting parameter settings Cat-A CR |
| R4-2111895 | Anritsu Corporation | Draft CR: CR to RI reporting parameter settings Cat-A CR |
| R4-2111896 | Anritsu Corporation | Draft CR: CR to reporting granularity for PMI TCs PDSCH & PDSCH DMRS precoding configuration for ramdom precoding are added to test parameters in PMI TCs. |
| R4-2111897 | Anritsu Corporation | Draft CR: CR to reporting granularity for PMI TCs Cat-A CR |
| R4-2111898 | Anritsu Corporation | Draft CR: CR to reporting granularity for PMI TCs Cat-A CR |
| R4-2113125 | Intel Corporation | Draft CR on CSI reference measurement channels Type of CSI-RS for PDSCH scheduling assumptions in CSI RMC is clarified. |
| R4-2113126 | Intel Corporation | Draft CR on CSI reference measurement channels Cat-A CR |
| R4-2113127 | Intel Corporation | Draft CR on CSI reference measurement channels Cat-A CR |
| R4-2113624 | Ericsson | Draft CR on TS38.101-4 Correction of parameter configurations in Rel-15 Correction of some of the parameter configuraions and formula |
| R4-2113625 | Ericsson | Draft CR on TS38.101-4 Correction of parameter configurations in Rel-16 Cat-A CR |
| R4-2113626 | Ericsson | Draft CR on TS38.101-4 Correction of parameter configurations in Rel-17 Cat-A CR |

## Open Issues Summary

### Sub-topic 1-1 TDD UL-DL pattern alignment for intra-band EN-DC performance TC

Issue 1-1-1: Should tests for intra-band EN-DC for TDD bands be measured or carried out

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - Discuss further

Issue 1-1-2: Clarification on treatment of the TDD patterns for NR

If tests for intra-band EN-DC for TDD bands are carried out, clarification on treatment of the TDD patterns for NR especially which cannot be aligned with LTE (e.g. FR1.30-6).

- Proposals

  - Option 1: For TDD intra-band EN-DC case, use the UL-DL configuration FR1.30-4 as a substitute for other configurations which cannot align the UL-DL pattern between LTE and NR. Whether it is possible to change SNR or modulation is FFS.

  - Option 2: Skip the corresponding test configurations in case it is not possible to align the UL-DL configuration between LTE and NR.

- Recommended WF

  - Discuss further

## Companies’ views collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub-topic 1-1 Issue 1-1-1: Issue 1-1-2: |
| Huawei, HiSilicon | Sub-topic 1-1 Issue 1-1-1: Option 1: Yes, but with clarification of Option 2 in next Issue 1-1-2. Issue 1-1-2: Option 2. During the NR Rel-15 performance requirements discussion, UL-DL configuration for LTE TDD is config 2 that is used in real network, i.e. DDDSU. the aligned NR TDD UL-DL configuration for FR1 is: DDDSU for 15kHz SCS (i.e. FR1.15-1), 7DS2U for 30kHz SCS (i.e. FR1.30-1), and later added DDDSUUDDDD for 30kHz SCS (i.e. FR1.30-4); For NR TDD UL-DL configuration that cannot be aligned with LTE’s, the following test applicability rule is captured in section 9.1.1 of TS 38.101-4 - The FR1 EN-DC test cases with the NR TDD DL-UL configurations which are not aligned with LTE's can be tested on the corresponding EN-DC band combinations where UE supports simultaneous transmission and reception. |
| Ericsson | Sub-topic 1-1 For this issue, we have same understanding as Huawei. TS38.101-4 9.1.1 has clearly described the applicability of FR1 EN-DC tests for TDD. We don’t think any additional applicability/clarification is needed. |
| Qualcomm | Sub-topic 1-1 Same view as Huawei and Ericsson. For the particular case of FR1.30-6, it might be ok to skip based on applicability rule mentioned by Huawei because we can test those test conditions with already defined test for FR1.30-1A, which has same test conditions except TDD config. Update 8/19/21 Based on response from Anritsu, we are ok to skip the test for the case of FR1.30-6 since there is a similar test for different TDD config. But we think that it should be considered case-by-case basis. We don’t want UE to skip some tests which are trying to test some other aspect (other than TDD config) as well. |
| Intel | Sub-topic 1-1 We share the same view as companies above. Existing note from Secction 9.1.1 should solve the all issues. |
| Anritsu | Thanks all for the comments. As for the applicability rule which is captured in section 9.1.1 of TS 38.101-4, in our understanding the corresponding UE capability is as follows (simultaneousRxTxInterBandENDC). If our reference is correct, it seems this capability seems dedicated only to the Inter-band EN-DC (TDD-TDD) and there is no description which is related to the intra-band EN-DC as we raised the issue in our paper. |
| Huawei | We agree that the applicability rule in section 9.1.1 is for inter-band. But for intra-band EN-DC, if no aligned TDD pattern for test, we prefer to skip the related test, i.e. Option 2 |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2111874 (Anritsu) | Huawei: static propagation with no external noise sources applied is used for LTE Cell for EN-DC demodulation test, we did not observe any necessity to introduce this new propagation condition TDLA30-70 without used in any test cases. |
|  | Ericsson: We understand the intention is to use TDLA30-70 for RRM tests. However TDLA30-70 is not used in TS38.101-4 and it is strange to specify it in TS38.101-4 just for the RRM tests (TS38.133). Since the delay profile TDLA30 has been specified in TS38.101-4, we expect the fading simulator can implement the correct channel model by applying the Doppler frequency of 70Hz. Or is there any issue if we don’t specify TDLA30-70? |
|  | Qualcomm: Can you please clarify which RRM test cases are you referring to in this CR? As per our understanding, all fading RRM tests are defined with TDLA30-10 or TDLC300-100. |
|  | Intel: It’s not clear for us why we need to define channel model in 38.101-4 which is not used for any test in this document. |
|  | Apple: We don’t think its necessary to define TDLA30-70 in 38.101-4 for RRM tests. |
|  | Anritsu: The motivation why we submitted this CR is to use the condition in RRM spec as Ericsson kindly mentioned. It is also because the definition of propagation condition is only defined in 38.101-4 and nowhere in 38.133. But if the group is OK that we simply specify TDLA30-70 (or something not defined in 38.101-4) in RRM TCs, we are fine to note this CR. To Qualcomm: Please refer to the CR R4-2111871 for 38.133 (also written on the coverpage of this CR R4-2111874). Associated RRM TCs are A.6.6.3.1.1, A.6.6.3.2.1, A.8.4.2.1.1, A.8.4.2.2.1, A.8.4.2.3.1 and A.8.4.2.4.1. There are description of TDLA30 with no doppler frequency information, or sometimes specified incorrectly by ETU70 even though it should be for NR cell. |
| R4-2111893 (Anritsu) | Company A: |
|  | Company B: |
| R4-2111896 (Anritsu) | Company A: |
|  | Company B: |
| R4-2113125 (Intel) | Ericsson: We are fine with this clarification. but 'CSI-RS for beam refinement' is used only for FR2. Maybe we don’t need 'CSI-RS for beam refinement' in FR1 (Table A.4-2 and Table A.4-3). |
|  | Qualcomm: In our view, we should skip TRS at least for test cases which require CQI reporting to align with reference resource definition in 38.214. It might be ok to have grant on TRS slots for PMI reporting tests. There are already some differences in the test setup for CQI vs PMI reporting tests, such as number of HARQ transmissions for the same reason. This could be another exception and we don’t see the need to align it. |
|  | Intel: @Ericsson: Thank you for comments. We will update the FR1 CSI RMC tables. @Qualcomm: We just try to reflect the previous meetings agreement. Our understanding is that same assumptions were assumed for PDSCH scheduling assumption for all CSI test cases. Please let us know if we miss any agreement from the past on different PDSCH scheduling assumptions for PMI and CQI/RI tests. As for 38.214, as we commented for the similar discussion for another topic, our understanding of 38.214 is that it provides the guidelines to UE for deriving the CQI index. Same time, there are no any restrictions on PDSCH scheduling in slots with TRS in case CQI reporting is configured. Also, 38.214 provides the similar guidelines for deriving the PMI index. However, we have PDSCH scheduling for slots with TRS for PMI tests. |
|  | Apple: It is also our understanding that PDSCH is not scheduled in slots with TRS in addition to slots with CSI-RS for acquisition for CQI/RI reporting test cases. |
|  | Intel2: @Apple: Same comments as for Qualcomm. We just try to reflect the previous meetings agreement. Please let us know if we miss any agreement from the past on different PDSCH scheduling assumptions for PMI and CQI/RI tests. @All: Probably companies can double check the assumptions which they have used for link level analysis for Rel-15 CQI/RI requirements defintion. If we’ve used different assumptions on PDSCH scheduling then we can asume that PDSCH scehduling in slots with TRS doesn’t have big impact on CQI/RI requiremnets, because alignement for definition of requiremnets was reached even in case we use different assumptions on PDSCH scheduling. In this case we can align PDSCH scheduling assumptions for all CSI tests based on agreement in the past. |
| R4-2113624 (Ericsson) | Huawei: We don’t think it is necessary to just modify the symbol in the formula. Also as per Table 7.4.1.5.3-1: CSI-RS locations within a slot in TS 38.211, there is only k0 for Row 5, no k1, it is not necessary to specify k1. - Ericsson: For the PMI formula, we think the description for section 6 and 8 should be aligned otherwise there will be confusing for further testing. Regarding to the description for the first subcarrier index in the PRB used for CSI-RS (k0, k1 ), we think now we have misalignment on section for CQI and section for PMI, which means that we can either modify the CQI part to (k0,k1) albeit we don’t configure k1 for Row 5 like what we did in PMI section: or we can modify the PMI section like the one in CQI section: |
|  | Intel: We are fine with update of gamma equation. As for CSI-RS configuration, we suggest to remove k1 parameter with ‘-’ value for scenarios where k1 cannot be configured (i.e. number of ports 1, 2 or 4) to avoid any confusion. |
|  | Huawei: Thanks for clarification about the formula updates to align section 6 and section 8, it is fine for us for the update. For CSI-RS configuration, we also prefer to remove k1 parameters considering no k1 for Row 5. |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 | Issue 1-1-1: Should tests for intra-band EN-DC for TDD bands be measured or carried out Companies’ views are that intra-band EN-DC tests for TDD bands should be carried out with further clarification. Tentative agreements: Yes with further clarification. Issue 1-1-2: Clarification on treatment of the TDD patterns for NR Companies pointed out that we already have clarification in section 9.1.1, and p.c. indicated that it’s for inter-band EN-DC and they would like to clarify for intra-band EN-DC. Candidate options: - Option 1: Skip the corresponding test configurations in case it is not possible to align the UL-DL configuration between LTE and N (existing note in section 9.1.1 is sufficient). - Option 2: Consider on case-by-case basis to avoid skipping tests that test features other than TDD config. Recommendations for 2nd round: Further discuss options above and how to capture the applicability if needed. |

### CRs/TPs

.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2111874 (Anritsu) | Recommend as ‘Not Pursued’ Companies don’t see the necessity to introduce this change in TS 38.101-4. Anritsu’s clarified the intention and are fine with not introducing the change. |
| R4-2113125 (Intel) | Recommend as ‘Return to’ in 2nd round. The purpose of not scheduling PDSCH in CSI-RS slots is to have the same code rate in all slots for tests with CQI reporting/ follow CQI. Companies could further check on assumptions used for PDSCH scheduling for CQI reporting tests for alignment in Rel-15 Discuss following options:. Option 1: Align PDSCH scheduling assumptions for all CSI tests (PMI, CQI, RI) Option 2: Use different PDSCH scheduling assumptions for PMI tests and CQI/RI tests |

## Discussion in 2nd round

Issue 1-1-2: Clarification on treatment of the TDD patterns for NR intra-band EN-DC for TDD bands

For intra-band EN-DC for TDD bands, clarification on treatment of the TDD patterns for NR especially which cannot be aligned with LTE (e.g. FR1.30-6).

- Proposals

  - Option 1: Skip the corresponding test configurations in case it is not possible to align the UL-DL configuration between LTE and NR (existing note in section 9.1.1 is sufficient).

  - Option 2: Consider on case-by-case basis to avoid skipping tests that test features other than TDD config.

- Recommended WF

Further discuss the options and how to capture the applicability if needed.

| Company | Comments |
| --- | --- |
| Anritsu | Thank you all for having a discussion during the 1st round. We feel there are almost no differences between the option 1 and 2 above. But we rather prefer Option 1 and also would like to clarify the case to skip the test by TDD pattern basis to avoid including ambiguity in the spec (i.e. skip FR1.30-6, and test FR1.30-1 and FR1.30-4 patterns by utilizing the feature of LTE SF offset.). Because the expression “case by case” in option 2 may create a difference among TE vendors when implementing the corresponding test case. By the way, our image of option 1 and 2 are as follows. Option 1: On condition that we may use the feature of LTE SF offset, we can test FR1.30-1 and FR1.30-4. We skip the pattern FR1.30-6 since we cannot adjust the pattern between LTE and NR even with the SF offset feature. Here LTE SF offset feature means that we simply shift the beginning of a frame for LTE or NR by a unit of subframes. Option 2: It is not so clear what “test features other than TDD config” means. Our interpretation is that PDSCH demod tests are skipped since those tests include patterns other than FR1.30-1. And other tests (PDCCH, CSI, etc.) can be carried out since those tests normally use FR1.30-1. These tests also need to utilize the LTE SF offset feature anyway. It is appreciated if companies can provide some examples of patterns to understand the image of case-by-case. |
| Huawei | Option 1.If any confusions for intra-band EN-DC with consideration of the note in section 9.1.1 for inter-band EN-DC,additional note can be added to skip the related test cases if no aligned TDD pattern between LTE and NR for intra-band EN-DC. |
| Intel | Support Option 1. We share the same view as Huawei. |
| Qualcomm | At this point, we only have this issue with FR1.30-6, so we are ok with either option. But I would like to comfirm with other companies if they think that this issue is there for any other TDD config other than FR1.30-6 so that we could double check befor agreeing to Option 1. |

Issue 1-2-1: Clarification on assumptions for CSI reporting tests

Companies to check assumptions used for PDSCH scheduling in Rel-15. Discuss if the clarification in Intel’s CR (R4-2113125) in needed.

- Proposals

  - Option 1: Align PDSCH scheduling assumptions for all CSI tests (PMI, CQI, RI)

  - Option 2: Use different PDSCH scheduling assumptions for PMI tests and CQI/RI tests

- Moderator’s observation

The purpose of not scheduling PDSCH in CSI-RS slots is to have the same code rate in all slots for tests with CQI reporting/ follow CQI. Also, the code-rate increase is around 5% if PDSCH is scheduled in slots with TRS, compared to slots with no TRS. The code rate change would be 2 slots and every 20ms.

- Recommended WF

Companies to check on assumptions used for PDSCH scheduling for CQI reporting tests for alignment in Rel-15 and discuss the options above. Further discuss if this clarification in this CR is needed.

| Company | Comments |
| --- | --- |
| Huawei | Option 1. It is better to align all CSI tests.We share the same understanding as Intel, the specification of TS 38.214 just provides guidance for UE to derive the CQI, in the real network, there are no any restrictions on PDSCH scheduling in slots with TRS in case CQI reporting is configured. |
| Apple | The idea behind not scheduling PDSCH in CSI-RS slots is to not have code-rate changes for CQI reporting tests. But PMI reporting need not be affected by it. For the existing PMI reporting requirements we don’t schedule PDSCH in slots for NZP CSI-RS for CSI acquisition and CSI for BM (in FR2), but schedule PDSCH in slots with CSI-RS for tracking. Given that the code rate is small, we are fine to use the same PDSCH scheduling assumptions for all CSI tests. We are fine to go with option 1. |
| Intel | Support Option 1. We just try to follow our understanding of previous agreement and align assumptions for all CSI tests. PDSCH will be scheduled only in the second TRS slot, because the first TRS slot contains the CSI-RS for CSI acquisition and will be skipped. Therefore, we will have only one slot with slightly different code rate in comparison to others, which does not affect the performance with CQI reporting. |
| Qualcomm | We don’t have any concern on whether PMI reporting test has PDSCH scheduled on TRS slots or not. Our main concern is with CQI/RI reporting tests where we also have 20dB SNR test cases. In those cases, having PDSCH scheduled on TRS slots may impact the performance. Also, since RAN4 defines minimum requirements, we will prefer to follow reference resource settings for CQI reporting regardless of SNR as was argued even in Rel-15 discussion and that’s why we do have all the settings in CQI/RI reporting tests matching to reference resource setting. We don’t see why we have to change that agreement. We already have some differences in CQI/RI and PMI reporting tests such as number of HARQ transmissions, reporting type etc. So, we don’t see why we have to align everything for all the tests. If companies prefer to align the tests, we would rather prefer not scheduling PDSCH on TRS slots in PMI reporting tests instead of other way around. |
| Intel | Based on our understanding, PDSCH scheduling in slot with TRS will not affect CQI/RI requirements, because we have relative requirements. Same time, taking into account concern from QC, we can postpone the decision on this draft CR till next meeting. We will provide the additional analysis next meeting. |

### CRs/TPs comments

| CR/TP number | Comments collection |
| --- | --- |
| R4-2113125 (Intel) | Moderator: To be revised based on comments for Issue 1-2-1. |
|  | Company B: |
| R4-2115668 Revision of R4-2113624 (Ericsson) | Apple: Suggest to update as Row 5, (4). Also, suggest to update other k0 and l0 for other CSI-RS configurations. |
|  | Ericsson: To Apple, just wondering that does the intention of changing to ‘Row 5, (4)’ is to avoid potential mis-leading on ‘Row 5 or 4’? Another observation for your second suggestion is that for other k0 and l0 configurations, mainly are for NZP-CSI-RS, sometimes there is k1(Row 8), sometimes there is no k1(Row 3). So if we modify it like k0 for ZP-CSI-RS, the name of the parameter in NZP-CSI-RS would be sometimes ‘First subcarrier index in the PRB used for CSI-RS (k0, k1 )’ and sometimes ‘First subcarrier index in the PRB used for CSI-RS (k0)’. We wonder which is better? Modify this parameter as ‘First subcarrier index in the PRB used for CSI-RS (k0, k1 ), Row *, (x,y)’ when there is k1. And modify this parameter as ‘First subcarrier index in the PRB used for CSI-RS (k0), Row *, (x)’ when there is no k1. Leave this parameter in NZP-CSI-RS configuration as it is since for all the CSI test cases they are aligned. |
|  | Apple2: We prefer to unify the way its specified for ZP CSI-RS and NZP CSI-RS and prefer option 1. Current change to unify ZP CSI-RS between CQI, PMI as Row 5, 4 is fine. |
|  | Huawei: We share the similar view with Apple that remove the k1 and “-” for CSI-RS configuration including both ZP CSI-RS and NZP CSI-RS. Please also consider the following updates in your revision: - No k1 for Row 3 and Row 4 - No l1 for Row 3, Row 4, Row 5 and Row 8 |

## Summary of 2nd round

Issue 1-1-2: Clarification on treatment of the TDD patterns for NR intra-band EN-DC for TDD bands

For intra-band EN-DC for TDD bands, clarification on treatment of the TDD patterns for NR especially which cannot be aligned with LTE (e.g. FR1.30-6).

Tentative Agreements: Skip the corresponding test configurations in case it is not possible to align the UL-DL configuration between LTE and NR (existing note in section 9.1.1 is sufficient).

Issue 1-2-1: Clarification on assumptions for CSI reporting tests

Companies to check assumptions used for PDSCH scheduling in Rel-15. Discuss if the clarification in Intel’s CR (R4-2113125) in needed.

Tentative Agreements: Postpone the CR and discuss further in next meeting.

# Topic #2: Rel-15 LTE UE Demodulation and CSI requirements maintenance

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2111843 | Anritsu Corporation | Draft CR to Reference Channel Parameters in UE Category M1 Table A.3.3.2.1-4: Fixed Reference Channel two antenna ports Change value of Max. Throughput average to 0.0187Mbps. On the Note5, change value of scheduled subframes to 5th subframe and every 12ms. |
| R4-2111844 | Anritsu Corporation | Draft CR to Reference Channel Parameters in UE Category M1 Cat-A CR |
| R4-2111845 | Anritsu Corporation | Draft CR to Reference Channel Parameters in UE Category M1 Cat-A CR |

## Open Issues Summary

None

## Companies’ views collection for 1st round

### Open issues

None

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2111843 (Anritsu) | Qualcomm: One thing we wanted to check is how you are planning to configure the MPDCCH period. Your proposal is to schedule one grant per 12ms with MPDCCH repetitions rmax = 4. We assume you would configure the MPDCCH period rmax*G (where G= mpdcch-StartSF-UESS below) to either 4 (G=1) or 6 (G=1.5). Alternatively, you could change the schedule to one grant per 10 ms by setting rmax*G = 4*2.5 = 10. Then all the RMCs in Table A.3.3.2.1-4 would have the same scheduling rate of 1 every 10 ms. 36.331 sec 6.3.2 mpdcch-config-r13 CHOICE { release NULL, setup SEQUENCE { csi-NumRepetitionCE-r13 ENUMERATED {sf1, sf2, sf4, sf8, sf16, sf32}, mpdcch-pdsch-HoppingConfig-r13 ENUMERATED {on,off}, mpdcch-StartSF-UESS-r13 CHOICE { fdd-r13 ENUMERATED {v1, v1dot5, v2, v2dot5, v4, v5, v8, v10}, tdd-r13 ENUMERATED {v1, v2, v4, v5, v8, v10, v20, spare1} }, mpdcch-NumRepetition-r13 ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128, r256}, mpdcch-Narrowband-r13 INTEGER (1.. maxAvailNarrowBands-r13) } } OPTIONAL -- Need ON mpdcch-StartSF-UESS Starting subframe configuration for an MPDCCH UE-specific search space, see TS 36.213 [23]. Value v1 corresponds to 1, value v1dot5 corresponds to 1.5, and so on.. |
|  | Ericsson: Comment to Qualcomm. When we implemented this requirement, we set mPDCCH-NumRepetition=4 and mpdcch_startSF_UESS=1. This means MPDCCH search space is scheduled every 4 subframes (4 ms). FRC proposed by Anritsu specifies TE schedules MPDCCH every 12 ms, which means TE schedule MPDCCH every 3rd UE-specific search space. |
|  | Anritsu: To Qualcomm: As also mentioned by Ericsson, our intention is to set mPDCCH-startSF-UESS = 1 (G=1), not G=2.5. Please also see our idea of the scheduling below. In case we apply G=2.5, we have a concern with the throughput degradation due to the longer MPDCCH/PDSCH transmittion period. |

## Summary for 1st round

### Open issues

None

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2111843 (Anritsu) | Agreeable based on further clarification from companies. |

# Topic #3: NR V2X Demodulation maintenance

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2112668 | LG Electronics Inc. | Draft CR for Abbreviations for V2X demodulation Add Abbreviations for NR V2X |
| R4-2112671 | LG Electronics Inc. | Draft CR for Abbreviations for V2X demodulation Cat-A CR |

## Open Issues Summary

None

## Companies’ views collection for 1st round

### Open issues

None

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2112668 (LG) | Huawei: Thanks for the additions of abbreviations for V2X. It is better to add abbreviations for SCI. |
|  | LG: Thanks for checking carefully. I will revise the draft CR by adding abbreviation for SCI. |

## Summary for 1st round

### Open issues

None

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2112668 (LG) | Revised |

## Discussion in 2nd round

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2115669 Revision of R4-2112668 (LG) | Huawei: Thanks for your revision, further comments on the coversheet: 1) No revision mark in the coversheet 2) The affected test specification TS 38.521-4 |

# Topic #4: eMIMO Demodulation maintenance

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2112101 | Apple | Draft CR to 38.101-4 on Applicability for multi-TRxP test cases-R16 Added applicability for single DCI multi-TRxP based on UE capability of maxNumberActiveTCI-PerBWP > n1. |
| R4-2112102 | Apple | Draft CR to 38.101-4 on Applicability for multi-TRxP test cases-R17 Cat-A CR |
| R4-2112688 | Apple | Discussion on applicability for multi-TRxP demodulation requirements Observation #1: For multi-DCI transmission scheme for multiTRxP UE indicates maxNumberCORESETPerPoolIndex-r16, which is sufficient to meet the condition to receive DCI and PDSCH from 2 TCI states. Observation #2: For multi-TRxP Single DCI transmission schemes there is no UE capability indication that indicates support of receiving with 2 active TCI states. Proposal #1: Add applicability for single DCI multi-TRxP based on UE capability of maxNumberActiveTCI-PerBWP > n1. |

## Open Issues Summary

### Sub-topic 4-1 Applicability of requirements for multi-TRxP

If the multi-TRP test cases will be applicable to UEs that supports maxNumberActiveTCI-PerBWP ≥ 2 in addition to the capability for different multi-TRP transmission schemes.

Issue 4-1-1: Applicability of requirements for multi-DCI multi-TRxP

Multi-DCI multi-TRxP requirements don’t need additional applicability based on maxNumberActiveTCI-PerBWP.

- Proposals

  - Option 1: Yes.

  - Option 2: No.

- Recommended WF

  - Discuss further

Issue 4-1-2: Applicability of requirements for single-DCI multi-TRxP

Single-DCI multi-TRxP requirements need additional applicability based on maxNumberActiveTCI-PerBWP.

- Proposals

  - Option 1: Yes.

  - Option 2: No.

- Recommended WF

  - Discuss further

## Companies’ views collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub-topic 4-1 Issue 4-1-1 Issue 4-1-2 |
| Huawei, HiSilicon | Sub-topic 4-1 Issue 4-1-1: Option 1: Yes. As indicated in the discussion paper R4-2112688, the capability of multiDCI-multiTRP-Parameters-r16 includes the capability of maxNumberActivatedTCI-States-r16. Issue 4-1-2: Test cases defined for single DCI based transmission schemes include the following three scenarios: - singleDCI-SDM-scheme-r16 - supportFDM-SchemeA-r16 - supportInter-slotTDM-r16 As per TS 38.331/306, no information about the supported number of activated TCI states in the singleDCI-SDM-scheme-r16 and supportFDM-SchemeA-r16, based on our understanding, UE should support two activated TCI states to support SDM and FDM for single DCI based transmission schemes by default, so no additional capability need to be specified, or clarification is added that support of maxNumberActiveTCI-PerBWP > n1 is mandatory if needed. But for TDM, UE can support 1 or 2 activated TCI states, additional capability maxNumberTCI-states-r16 is defined for supportInter-slotTDM-r16, so the test applicability for test cases of single DCI based inter-slot TDM for multi-TRP can be updated to include this additional capability for the supported number of activated TCI states. |
| Samsung | Issue 4-1-1 We are ok with option 1 Issue 4-1-2 We are ok with option 1,to add the additoinal additional applicability based on maxNumberActiveTCI-PerBWP for inter-slot TDM scheme, for other single-DCI schemes, we are open to discuss whether it is needed or not, since there is no additional capabilty specfied in case of UE supported FDM-SchemeA-r16 or singleDCI-SDM-scheme-Parameters-r16. |
| Ericsson | Sub-topic 4-1 Issue 4-1-1: To avoid the misunderstanding of answer: - Multi-DCI multi-TRxP requirements don’t need additional applicability based on maxNumberActiveTCI-PerBWP. Issue 4-1-2: We are ok to add the additional applicability based on maxNumberActiveTCI-PerBWP for UE demodulation requirements of single-DCI based SDM/FDM/TDM schemes specified in TS38.101-4 because these test cases are set based on the assumption UE monitors two active TCI states. |
| Intel | Sub-topic 4-1 We share the same view as Huawei. For Multi-DCI and Single DCI Inter Slot TDM schemes we have dedicated parameters with information about supported maximum number of TCI states. As for Single DCI SDM and FDM schemes, taking into account that RAN1 have not defined any dedicated fields for number of supported TCIs for these schemes and, also, there is no any connection between Rel-15 feature maxNumberActiveTCI-PerBWP and Rel-16 features singleDCI-SDM-scheme-Parameters-r16 and supportFDM-SchemeA-r16 in RAN1 specifications, we assume that reporting of support of certain Single DCI scheme should be sufficient for testing. |
| Apple | @ Huawei, Thanks for pointing out the additional capability for TDM scheme. For Single DCI SDM and TDM schemes since there is no additional capability to indicate for monitoring 2 active TCI states, it would be reasonable to add applicability based on maxNumberActiveTCI-PerBWP >n1. For Single DCI TDM, we should add applicability based on maxNumberTCI-states-r16 =2. |
| Intel2 | For Single DCI SDM and TDM schemes, based on our understanding and discussion with RAN 1 colleagues, in case UE reports support of singleDCI-SDM-scheme-Parameters-r16 and supportFDM-SchemeA-r16, we can assume that such UE supports 2 active TCI states, because it cannot operate for such schemes without support of 2 active TCI states. Probably comapnies can double check with they RAN1 collegues this part. |
| Huawei | As we commented, we have the similar understanding as Intel: UE should support two activated TCI states to support SDM and FDM for single DCI based transmission schemes by default, so no additional capability need to be specified, or clarification is added that support of maxNumberActiveTCI-PerBWP > n1 is mandatory if needed. |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2112101 (Apple) | Apple: We will update based on conclusion on issue 4-1-2 |
|  | Company B: |
|  | Samsung: pending on the issue 4-1-1 and 4-1-2 |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic #4-1 | Issue 4-1-1: Applicability of requirements for multi-DCI multi-TRxP Whether multi-DCI multi-TRxP requirements don’t need additional applicability based on maxNumberActiveTCI-PerBWP. Tentative agreements: No additional capability is needed for multi-DCI multi-TRxP requirements. Issue 4-1-2: Applicability of requirements for single-DCI multi-TRxP Whether single-DCI multi-TRxP requirements need additional applicability based on maxNumberActiveTCI-PerBWP. UE capability for single DCI TDM also includes additional capability indication for TCI states: For sDCI TDM requirements, applicability based on maxNumberTCI-states-r16 =2 should be added. For sDCI SDM and FDM schemes there is no additional UE capability. - 3 companies think additional applicability based on maxNumberActiveTCI-PerBWP > n1 is needed since tests are with 2 active TCI states. - 2 companies think that UE should indicate maxNumberActiveTCI-PerBWP > n1 if UE indicates support for sDCI SDM and FDM schemes. Tentative agreements: For Single DCI TDM requirements add applicability based on maxNumberTCI-states-r16 =2. Candidate options: For Single DCI SDM and FDM requirements: - Option1: Add applicability based on maxNumberActiveTCI-PerBWP >n1. - Option2: Add applicability based on maxNumberActiveTCI-PerBWP >n1, with clarification that it is mandatory. - Option 3: No additional applicability based on UE capability is needed. Recommendations for 2nd round: Further discuss options above for Single DCI SDM and FDM requirements. |

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2112101 (Apple) | Revised |

## Discussion in 2nd round

Issue 4-1-2: Applicability of requirements for single-DCI multi-TRxP

Agreement in round 1: For Single DCI TDM requirements add applicability based on maxNumberTCI-states-r16 =2.

For Single DCI SDM and FDM requirements:

  - Option1: Add applicability based on maxNumberActiveTCI-PerBWP >n1.

  - Option2: Add applicability based on maxNumberActiveTCI-PerBWP >n1, with clarification that it is mandatory.

  - Option 3: No additional applicability based on UE capability is needed.

Recommended WF:

Discuss additional applicability for single DCI SDM and FDM requirements.

| Company | Comments |
| --- | --- |
| Samsung | We support option 3, since if UE can support singleDCI-SDM-scheme-Parameters-r16 and supportFDM-SchemeA-r16, it should be implicitly implied that maxNumberActiveTCI-PerBWP >n1 is supported by this UE. |
| Huawei | Option 2.to ensure backward compatibility, we can set maxNumberActiveTCI-PerBWP = n2 --- Update Aug 25 --- Our understanding about Option 1 and Option 2, UE should be mandated to report maxNumberActiveTCI-PerBWP = n2 during the testing.If companies still have different views on whether it is mandatory to support maxNumberActiveTCI-PerBWP = n2, they can check with RAN1 colleagues or check RAN1 common understanding after this meeting and we can make conclusion for next meeting. |
| Apple | Option 1. There is no indication in 38.306 that UE should indicate maxNumberActiveTCI-PerBWP = n2 with support for SDM or FDM transmission schemes. Since we agree that UE should support maxNumberActiveTCI-PerBWP > n1 for single DCI requirements, its better to capture this in requirements applicability. |
| Intel | Option 3. Based on our discussion with RAN1 colleagues, if UE reports support of one of these schemes, then it means that such UE supports the two active TCI states. Same time, maxNumberActiveTCI-PerBWP feature was introduced in Rel-15 for other scenarios/purposes, and we should not mandate the signalling of capability which was not designed for considered multi-TRP scenarios. |
| Qualcomm | We prefer Option 1. We are not sure what is the difference between Option 1 and Option 2. But since it is not explicitly said anywhere in the spec that UE needs to support multiple TCI states under the mTRP schemes capabilities, it will be safer to add this capability. |

### CRs/TPs comments

| CR/TP number | Comments collection |
| --- | --- |
| R4-2115670 Revision of R4-2112101 (Apple) | Moderator: To be updated based on WF from 1st round and 2nd round comments |
|  | Intel: We think that due to no consensus reached on Issue 4-1-2, this draft CR should cover only “Single-DCI based inter-slot TDM” test this meeting. We can continue discussion on SDM and FDM Scheme A next meeting. |
|  | Apple: Based on the comments received for Issue 4-1-2, we have updated to draftCR to only add additional applicability for single DCI TDM requirements. We can further check on SDM and FDM Scheme A in in next meeting. Updated version: Draft R4-2115670_DraftCR 101-4 Applicability mTRP Demod-R16_v2.docx |
|  | Intel 2: We just realized that applicability rule for inter-slot TDM is defined in table for mandatory features. Same time, support of inter-slot TDM is optional features. Should not maxNumberTCI-states-r16 also be defined in table with optional features? |
|  | Apple2: Thanks for the careful check! We updated the CR to include applicability in optional features table: Draft R4-2115670_DraftCR 101-4 Applicability mTRP Demod-R16_v3.docx |

## Summary of 2nd round

Issue 4-1-2: Applicability of requirements for single-DCI multi-TRxP

Tentative Agreements: Discuss if additional applicability for single DCI SDM and FDM requirements is required in next meeting.

# Topic #5: Rel-16 NR UE Demodulation and CSI requirements maintenance

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2112957 | NTT DOCOMO, INC. | Draft CR FR1 EN-DC power imbalance requirements Update Note2 in Table 9.1.1-3 according to the related agreeements as following: “UE supports both intra-band contiguous and non-contiguous EN-DC requirements for supported inter-band EN-DC combinations” |
| R4-2112958 | NTT DOCOMO, INC. | Draft CR FR1 EN-DC power imbalance requirements Cat-A CR |
| R4-2113369 | Ericsson, Apple | Draft CR to 38.101-4: Correction of SNR levels for 0.001% BLER PDSCH requirement SNR values increased by 0.5dB to account for the span margin. |
| R4-2113370 | Ericsson, Apple | Draft CR to 38.101-4: Correction of SNR levels for 0.001% BLER PDSCH requirement Cat-A CR |
| R4-2113773 | Huawei,HiSilicon | draft CR: Updates to PDSCH FRC in TS 38.101-4 for Rel-16 Change the number of CBs for FRC of R.PDSCH.1-16.1 TDD and R.PDSCH.1-16.2 TDD in Table A.3.2.2.2-16 from 2 to 4 |
| R4-2113774 | Huawei,HiSilicon | draft CR: Updates to PDSCH FRC in TS 38.101-4 for Rel-17 Cat-A CR |
| R4-2114036 | MediaTek inc. | CR to TS38.101-4 on URLLC requirements (Rel-16) - Add R.PDSCH. 1-2.6 FDD in Table A.3.2.1.1 according to R4-2103901, also modifiy Tables 5.2.2.1.8-3 and 5.2.3.1.8-3 - Add R.PDSCH. 2-2.6 TDD in Table A.3.2.2.2 according to R4-2103901, also modifiy Tables 5.2.2.2.8-3 and 5.2.3.2.8-3 - Moddify R.PDSCH. 5-10.1 TDD to R.PDSCH. 5-11.1 TDD in Table A.3.2.2.5-11. Aslo modify the Table 7.2.2.2.2-3 - Modifty the reference channel R.PDSCH.1-16.1 TDD, R.PDSCH.1-17.1 TDD to R.PDSCH.2-16.1 TDD, R.PDSCH.2-17.1 TDD respectively. Also modify Tables 5.2.2.2.6-3, 5.2.2.2.7-3, 5.2.3.2.6-3 and 5.2.3.2.7-3 - Remove the redundant ‘space’ in section 7.2.2.2.3 |
| R4-2114038 | MediaTek inc. | CR to TS38.101-4 on URLLC requirements (Rel-17) Cat-A CR |

## Open Issues Summary

None

## Companies’ views collection for 1st round

### Open issues

None

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2112957 (Docomo) | SoftBank: Support the proposal in this CR. |
|  | Company B: |
| R4-2113369 (Ericsson) | Huawei: Coversheet issues: - “Source to TSG” is empty, R4 should be added. - “Other specs affected” should be updated to indicated the affected test specification TS 38.521-4. |
|  | Company B: |
| R4-2113773 (Huawei) | Ericsson: We agree the number of code blocks should be 4. We propose to merge to R4-2114036. |
|  | Company B: |
| R4-2114036 (MediaTek) | Huawei: Maybe the square brackets can be removed for this meeting to align with other parts. Table 5.2.2.2.8-3 and Table 7.2.2.2. 3-3 are not aligned with the latest specification of TS 38.101-4 version 16.5.0 about the SNR value, square bracket still exists in the latest specification. |
|  | Apple: Ref chan R.DSCH.1-2.6 FDD Added in A.3.2.1.1-1, should be added in table for 16QAM if necessary. We also realize that R.PDSCH.1-2.5 FDD is present in both QPSK and 16QAM tables, and should be removed from QPSK table. Change in 5.2.2.2.7 is not needed, and no need to add R.PDSCH.2-17.1 in Annex. |
|  | Intel2: Based on our understanding, R.DSCH.1-2.5 FDD from Table A.3.2.1.1-1 is for requirements from Section 5.2.2.1.13 and 5.2.3.1.13 (FDD Multi-TRP Single DCI FDM Scheme A) and R.PDSCH.1-2.5 FDD from Table A.3.2.1.1-2 is for requirements from Section 5.2.2.1.8 and 5.2.3.1.8 (FDD Pre-emption requirements). Same time, FRC for TDD pre-emption requirements was not implemented (This is covered by this CR). To resolve the issue for FDD test, we suggest to move R.DSCH.1-2.5 FDD from Table A.3.2.1.1-1 to Table A.3.2.1.1-2 and keep numbering (but update the title from R.DSCH to R.PDSCH). Same time, rename R.PDSCH.1-2.5 from Table A.3.2.1.1-2 to R.PDSCH.1-2.6 to align with FRC numbering which we will have for TDD case. |
| MediaTek | Thanks for the comments. @ Huawei: - We remove the bracket in Table 5.2.2.2.7-3, Table 5.2.2.2.8-3, Table 5.2.3.2.7-3 and Table 7.2.2.2. 3-3 @ Apple and Intel: - We delete R.DSCH.1-2.5 FDD in Table A.3.2.1.1-1 - We move R.DSCH.1-2.5 FDD to Table A.3.2.1.1-2 as R.PDSCH.1-2.5 FDD - We rename R.PDSCH.1-2.5 FDD in Table A.3.2.1.1-2 to R.PDSCH.1-2.6 FDD - We think reference channel number should follow the last two digit of table number. Hence, we just change R.PDSCH.1-17.1 to R.PDSCH.2-17.1, just like the change from R.PDSCH.1-16.1 to R.PDSCH.2-16.1. We upload the revised version for further review. |

## Summary for 1st round

### Open issues

None

### CRs/TPs

Provided in section 6.

## Discussion in 2nd round

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2115671 Revision of R4-2113369 (Ericsson) | Apple: We are ok with the updates. |
|  | Company B: |
| R4-2115672 Revision of R4-2114036 (MediaTek) | Apple: 1. In Table A.3.2.2.2-16: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 - what is the purpose to change the reference channel from R.PDSCH.1-16.1 TDD to R.PDSCH.2-16.1 TDD and R.PDSCH.1-16.2 TDD to R.PDSCH.2-16.2 TDD 2. In Table A.3.2.2.2-17: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-2 - what is the purpose to change from R.PDSCH.1-17.1 TDD to R.PDSCH.2-17.1 TDD The reason given in first round is: We think reference channel number should follow the last two digit of table number. Hence, we just change R.PDSCH.1-17.1 to R.PDSCH.2-17.1, just like the change from R.PDSCH.1-16.1 to R.PDSCH.2-16.1. But its not clear which last 2 digit from table we try to match. 3. In Table Table A.3.2.1.1-2 renaming R.PDSCH.1-2.5 FDD in to R.PDSCH.1-2.6 FDD would cause a mis-match in the reference channel used for tests in Table 5.2.2.1.13-3, Table 5.2.3.1.13-3 |
|  | MediaTek: For the first and second comments: The reference measurement channels for PDSCH are defined in Annex A.3.2. To my understanding, the structure of captions for PDSCH tables is A.3.2.x.y-z. x=1 is for FDD and x = 2 is for TDD. y is for the subcarrier spacing or E-UTRA, take FDD as example, b=1 is for 15kHz, b=2 is for 30kHz, etc. z is used for modulation order, TDD pattern or some other different configuration. If you check the reference measure channel under table A.3.2.x.y-z, almost all of reference channels follow the last two digit of table number. For example, the PDSCH reference channels in Table A.3.2.2.1-3 are R.PDSCH.1-3.1, R.PDSCH.1-3.2 and R.PDSCH.1-3.3. Only the PDSCH reference channel in Table A.3.2.2.2-16, Table A.3.2.2.2-17 and Table A.3.2.2.2-18 do not follow the rule. The PDSCH reference channels in these three tables are start with R.PDSCH.1-16.1 That is the reason for the modification. (There is a corresponding CR to address Table A.3.2.2.2-18 in NR-U WI) For the third comment: There are two “R.PDSCH.1-2.5 FDD” in TS38.101-4 v.16.5.0. One R.DSCH.1-2.5 FDD (not R.PDSCH.1-2.5 FDD) in Table A.3.2.1.1-1 and one R.PDSCH.1-2.5 FDD in Table A.3.2.1.1-2. R.DSCH.1-2.5 FDD in Table A.3.2.1.1-1 is used for Table 5.2.2.1.13-3 and Table 5.2.3.1.13-3. R.PDSCH.1-2.5 FDD in Table A.3.2.1.1-2 is used for Table 5.2.2.1.8-3 and Table 5.2.3.1.8-3. To resolve the contention for two “R.PDSCH.1-2.5 FDD”. 1. Follow the rule I mentioned above, I correct R.DSCH.1-2.5 FDD to R.PDSCH.1-2.5 FDD and place it under the Table A.3.2.1.1-2. 2. I modify R.PDSCH.1-2.5 FDD in Table A.3.2.1.1-2 to R.PDSCH.1-2.6 FDD and then also modify Table 5.2.2.1.8-3 and Table 5.2.3.1.8-3. |
|  | Apple2: The first 2 issues are now clear. Regarding the 3rd one. My understanding is that in the current spec v16.5.0 R.DSCH.1-2.5 FDD in Table A.3.2.1.1-1 if for requirements for preemption indication (5.2.x.1.8). And, the reference channel R.PDSCH.1-2.5 FDD in Table A.3.2.1.1-2 is for mTRP FDM Scheme A requirement (5.2.x.1.13). Hence by this change, the reference channel in tables 5.2.x.1.13-3 is no longer correct. The only change should be to add R.PDSCH.1-2.6 FDD in Table A.3.2.1.1-2 (previously R.DSCH.1-2.5 FDD in Table A.3.2.1.1-1), without any modification to existing R.PDSCH.1-2.5 FDD. |
|  | MediaTek2: I think R.DSCH.1-2.5 FDD in Table A.3.2.1.1-1 is for single-DCI based FDM Scheme A. You can refer to R4-2103830, where the MCS table for reference channel is 64QAMLowSE. As for R.PDSCH.1-2.5 FDD in Table A.3.2.1.1-2, it is for PDSCH pre-emption. You can refer to R4-2103901, where the MCS table for reference channel is 64QAM as it is for eMBB UE. I just added a “P” for R.PDSCH.1-2.5 FDD in Table A.3.2.1.1-1 and moved it to Table A.3.2.1.1-2 as the MCS is 16QAM (not QPSK). Also, I did not modify the reference channel in Table 5.2.x.1.13-3. Hence, I think the reference channel for single-DCI based FDM Scheme is still correct with such modifications. As for the original R.PDSCH.1-2.5 FDD in Table A.3.2.1.1-2, I changed it to R.PDSCH.1-2.6 FDD for PDSCH pre-emption and also modify Table 5.2.x.1.8. |

# Recommendations for Tdocs

## 1st round

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| Topic #1 |  |  |  |  |
| R4-2111874 | Introduction of a new propagation definition | Anritsu | Not pursued | Companies don’t see need for introducing TDLA30-70 in 38.101-4. |
| R4-2111893 | CR to RI reporting parameter settings | Anritsu | Agreeable |  |
| R4-2111896 | CR to reporting granularity for PMI TCs | Anritsu | Agreeable |  |
| R4-2113125 | Draft CR on CSI reference measurement channels | Intel | Return to | Companies need to check the assumptions on PDSCH scheduling for CQI/RI requirements. |
| R4-2113624 | Draft CR on TS38.101-4 Correction of parameter configurations in Rel-15 | Ericsson | Revised |  |
| Topic #2 |  |  |  |  |
| R4-2111843 | Draft CR to Reference Channel Parameters in UE Category M1 | Anritsu | Agreeable |  |
| Topic #3 |  |  |  |  |
| R4-2112668 | Draft CR for Abbreviations for V2X demodulation | LGE | Revised |  |
| Topic #4 |  |  |  |  |
| R4-2112101 | Draft CR to 38.101-4 on Applicability for multi-TRxP test cases-R16 | Apple | Revised |  |
| Topic #5 |  |  |  |  |
| R4-2112957 | Draft CR FR1 EN-DC power imbalance requirements | DOCOMO | Agreeable |  |
| R4-2113369 | Draft CR to 38.101-4: Correction of SNR levels for 0.001% BLER PDSCH requirement | Ericsson, Apple | Revised | Address coversheet issues |
| R4-2113773 | draft CR: Updates to PDSCH FRC in TS 38.101-4 for Rel-16 | Huawei | Agreeable |  |
| R4-2114036 | CR to TS38.101-4 on URLLC requirements (Rel-16) | MediaTek | Revised |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2113125 | Draft CR on CSI reference measurement channels | Intel | Postponed |  |
| R4-2115668 | Draft CR on TS38.101-4 Correction of parameter configurations in Rel-15 | Ericsson | Agreeable |  |
| R4-2115669 | Draft CR for Abbreviations for V2X demodulation | LGE | Agreeable |  |
| R4-2115670 | Draft CR to 38.101-4 on Applicability for multi-TRxP test cases-R16 | Apple | Agreeable |  |
| R4-2115671 | Draft CR to 38.101-4: Correction of SNR levels for 0.001% BLER PDSCH requirement | Ericsson, Apple | Agreeable |  |
| R4-2115672 | CR to TS38.101-4 on URLLC requirements (Rel-16) | MediaTek | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Samsung | Yunchuan Yang | yc0301.yang@samsung.com |
| Ericsson | Jiakai Shi | jiakai.shi@ericsson.com |
| Qualcomm | Gaurav Nigam | gnigam@qti.qualcomm.com |
| Intel | Dmitry Belov | dmitry.belov@intel.com |
| Apple | Manasa Raghavan | Manasa.raghavan@apple.com |
| Anritsu | Osamu Yamashita | Osamu.Yamashita@anritsu.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)