3GPP TSG-RAN WG4 Meeting # 100-e 												R4-2115807

Electronic Meeting, 16 - 27 August, 2021

Agenda item:			9.1

Source:	Moderator (Samsung)

Title:	Email discussion summary for [100-e][331] NR_MIMO_OTA

Document for:	Information

Introduction

Contributions submitted to AI 9.1 NR MIMO OTA WI and AI 6.1.9.5 MIMO OTA SI maintenance are captured in this email discussion.

In the RAN4#99-e meeting, next steps of NR MIMO OTA WI were captured in the WF.

Next steps:

  - Further Discuss the pass/fail limit and reference figure of channel model validation

  - Further discuss the Maximum downlink RS-EPRE for FR2

  - Further discuss FR2 blocking issue

  - Further discuss FR2 simulation activity

  - Further discuss whether RAN4 should consider the case that more than 18 points can not reach 70%TP for FR2 MIMO OTA

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: agree draft CRs in AI 6.1.9.5, finalize the time plan of FR1 lab alignment, and discuss open issues of NR MIMO OTA WI.

- 2nd round: agree TPs, make decisions on the open issues.

Topic #1: General and Testing methodologies

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2112977 | vivo | Rapporteur input to TS38.151 This paper provides the rapporteur input to TS38.151 for essential corrections and editorial issue. |
| R4-2114381 | Keysight Technologies UK Ltd | TP on Channel Model and DUT Positioning Clarifications |
| R4-2112978 | vivo,CAICT | TP to TS38.151 on BS beam configuration |
| R4-2114528 | Huawei, HiSilicon | TP to TS 38.151 on FR1 2x2 BS beam selection |
| R4-2114529 | Huawei, HiSilicon | update simulation results on FR1 2x2 channel models |
| R4-2114534 | Huawei, HiSilicon | Discussion on probe weight Proposal 1: An initial reference set of probe weight is published to performance requirements for FR2 and aligning simulation, and feedback from TE/CE vendors is encouraged to reach a consensus on a set of weights. |
| R4-2114535 | Huawei, HiSilicon | TP to TS38.151 v0.4.0 on FR2 Base Station beam configuration |
| R4-2112862 | CMCC | Consideration on Probe#3 of FR2 MIMO OTA Proposal. The blocking issue of probe#3 should be closed due to the fact that the optimized weight of Probe#3 is almost zero. |
| R4-2112979 | vivo | TP to TS38.151 on Minimum Number of Slots and Power Control |
| R4-2113915 | OPPO | The FR2 blocking issue Proposal: Probe 3’s blocking issue can be ignored when the following situation is satisfied. The improved three-step approach gives the results that Probe 3 experiences blocking small enough. Or A declaration of the weight of Probe 3 is small enough in the implementation of the selected FR2 MIMO OTA channel model. |
| R4-2114380 | Keysight Technologies UK Ltd | On Blocking Issue for FR2 MIMO OTA Observation 1: From a visual perspective, blocking from Probe #1 is worse than from Probe #3. Observation 2: The CST asymptotic solver accurately predicts the theoretical S21 between mm-wave horn antennas. Observation 3: Issues with CST’s asymptotic solver currently prevent the evaluation of Step 2 and 3 simulations regardless of reference antenna position. Observation 4: The probe weights of Probe #1 are significantly higher than those of Probe #3. Proposal 1: If the blocking effects still need to be quantified to resolve this blocking issue, keep this discussion open and interested parties are encouraged to provide simulation results. |
| R4-2113854 | Spirent Communications | Channel Model Targets Proposal 1. Use the targets presented in this contribution for the different spatial channel model parameters validation. |
| R4-2113858 | CAICT, CMCC | Reference Channel Emulation Curves for FR1 In this contribution spatial channel model validation targets for FR1 are proposed. |
| R4-2114382 | Keysight Technologies UK Ltd | Reference Channel Emulation Curves Proposal 1: take the presented FR1 reference curves into account to determine the validation reference curves and pass/fail limits. |
| R4-2114025 (reserved) | CAICT, Keysight Technologies UK Ltd, Spirent Communications, CMCC | Reference curves for FR1 CDL-C Uma |
| R4-2114383 | Keysight Technologies UK Ltd, Spirent Communications, CMCC, CAICT | Reference Channel Emulation Curves for Validation Purposes |
| R4-2112976 (reserved) | vivo | 3GPP TS 38.151 v0.5.0 |
| R4-2114503 | Huawei, HiSilicon | Discussion on FR2 channel model validation Proposal 1: whether to use the base station beam for FR2 channel model validation needs to be further clarified. Proposal 2: The near field to far-field conversion need to be explained in detail in D.3.4 FR2 PAS similarity percentage. In addition, a TP to TS38.151 v0.4.0 on FR2 PAS similarity percentage was added to the appendix of the paper. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1 Base Station beam configuration

FR1 and FR2 Base Station beam configuration was clarified and agreed in last meeting. Accordingly TPs to capture these agreements have been submitted to this meeting. Since multiple TPs are contributing to the same clause, it is suggested to discuss here as open issue before agreeing final TP.

Issue 1-1: Base Station beam configuration

- Proposals

  - TP in R4-2112978 on FR1 and FR2 (vivo, CAICT)

  - TP in R4-2114381 on FR1 and FR2 (Keysight)

  - TP in R4-2114528 on FR1 (Huawei, HiSilicon)

  - TP in R4-2114535 on FR2 (Huawei, HiSilicon)

- Recommended WF

  - Single merged TP (Revision of R4-2114381) is recommended

Sub-topic 1-2 Channel model validation for FR1

Companies shared the reference targets of FR1 channel models validation in R4-2113854, R4-2113858 and R4-2114382, including target values and reference curves for PDP, Temporal and Spatial correlation, V/H Ratio for CDL-C UMa and CDL-C UMi respectively. Alignment among companies is needed so that we can approve a set of reference values for channel model validation.

Issue 1-2: Reference validation targets for FR1

- Proposals

  - Option 1: R4-2113854 (Spirent)

  - Option 2: R4-2113858 (CAICT, CMCC)

  - Option 3: R4-2114382 (Keysight)

  - Joint TDoc R4-2114383  for CDL-C UMa

- Recommended WF

  - for CDL-C Uma, it is recommended to agree R4-2114383;

  - for CDL-C UMi, it is encouraged to align the reference curve in this meeting.

Sub-topic 1-3 FR2 probe blocking issue

Issue 1-3: FR2 probe blocking issue

- Proposals

  - Option 1: The blocking issue of probe#3 should be closed due to the fact that the optimized weight of Probe#3 is almost zero.(CMCC)

  - Option 2: Probe 3’s blocking issue can be ignored when the following situation is satisfied: Either “The improved three-step approach gives the results that Probe 3 experiences blocking small enough” or “A declaration of the weight of Probe 3 is small enough in the implementation of the selected FR2 MIMO OTA channel model.” (OPPO)

  - Option 3: If the blocking effects still need to be quantified to resolve this blocking issue, keep this discussion open and interested parties are encouraged to provide simulation results. (Keysight)

- Recommended WF

  - it is highly encouraged to conclude this issue in this meeting

Sub-topic 1-4 FR2 probe weights

Issue 1-4: FR2 probe weights

- Proposals

  - Proposal 1:  An initial reference set of probe weight is published to performance requirements for FR2 and aligning simulation, and feedback from TE/CE vendors is encouraged to reach a consensus on a set of weights. (R4-2114534)

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

Sub topic 1-1 Base Station beam configuration

| Company | Comments |
| --- | --- |
| OPPO | For FR1 BS beam configuration, the wording in R4-2112978 (vivo, CAICT) is preferred. |
| Keysight | Draft revision of R4-2114381 was harmonized between Keysight, vivo, CAICT, Huawei |
| CAICT | Draft revision of R4-2114381 was harmonized between Keysight, vivo, CAICT, Huawei. The wordings in both R4-2112978 and Revision of R4-2114381 are acceptable. |

Sub topic 1-2 Channel model validation for FR1

| Company | Comments |
| --- | --- |
| OPPO | The averaged CDL-C UMa curves in R4-2114383 can be agreed. PDP and temporal correlation curves from three contributors have perfectly coincided. For spatial correlation, there are some deviations when fc > 1GHz. Fortunately, most of deviations happen on points with spatial correlation < 0.3, and experiences tell us that throughput performance has minor impact on those kind of points. |
| Keysight | Agree to R4-2114383 |
| CMCC | Agree to R4-2114383 |
| CAICT | For CDL-C UMa, support the proposed reference channel emulation curves in R4-2114383. For CDL-C UMi, offline alignment between CAICT/CMCC, Keysight, and Spirent is in progress. |
| Spirent | Agree to R4-2114383. For CDL-C UMi spatial correlation is the only parameter that is not completely aligned. A revision of R4-2114383 can capture PDP, Autocorrelation, V/H agreements for CDL-C UMi. We propose to have another round of harmonization to minimize the differences in spatial correlation. |

Sub topic 1-3 FR2 probe blocking issue

| Company | Comments |
| --- | --- |
| OPPO | Considering the inputs from R4-2112862(CMCC) and R4-2114534(Huawei, HiSilicon), we agree to close the blocking issue based on that the weight of Probe 3 is small enough to be ignored. |
| Keysight | Supporting closing this issue (Option 1) |
| CMCC | Support Option1 due to that the optimized weight of Probe#3 is almost zero from CMCC side. More CE vendors are encouraged to clarify whether weight of Probe#3 is small enough to ignore blocking issue. |
| Qualcomm | Support option 1. |
| CAICT | If it is the consensus of CE vendors that the weight of Probe#3 is small enough, we can support closing the blocking issue (Option 1). |
| Spirent | Support closing this issue |
| Samsungng | Support closing this issue (Option 1) |
| Apple | Support Keysight R4-2114380 Proposal 1. Blocking effects still need to be better understood in the context of multiple system implementations. |

Sub topic 1-4 FR2 probe weights

| Company | Comments |
| --- | --- |
| Keysight | The best way to evaluate the applicability of the given probe weights, would be to check that the simulated PSP with the proposed weights is matching with the earlier PSP simulation alignment results from TE/CE vendors. |
| Huawei, HiSilicon | With only 2 decimal place, the PSP is almost 90%, but it obviously not accurate enough for real chamber. We have concern that chamber with even the similar PSP but different probe weight, UE could behave different performance level. Considering UE verification currently is processed by authorized agency. If the weight cannot be unified within 3GPP, the conformance among different OTA chambers may be not consistent. Thus, we proposed to go with the probe weight in R4-2114534 as RAN4 assumption. Or we are open to accept more input and analysis. |
| Apple | Agree with Keysight comment. In addition, correlation between fundamental simulation and measurements are also instrumental to increase modelling confidence and accuracy |

CRs/TPs comments collection

For close-to-finalize Wis and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing Wis, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2112977 | Company A |
|  | Company B |
| R4-2112978 | vivo: as suggested by Moderator, single merged big TP for channel model related topic is better. So this TP can be marked as “merged”. |
|  | Company B |
| R4-2112979 | Company A |
|  | Company B |
| R4-2114381 | Apple - KS text addition on B.2.1 (system setup) overlaps with Vivo R4-2112981 listed on this document section 3.1. Such editorial change doesn’t seems to help to reduce the original ambiguity on TR 38.827, A.3. The Figure A.3-1 has arbitrary reduced ripple regions. Understanding that there are no consensus among system vendor manufacturers on the embodiment or material used for the positioner. And no manufacturer declaration about the DUT antenna system placement/topology is mentioned. Improved clarification covering such aspects might be needed. |
|  | Keysight: the overlap has been resolved in the revision of R4-2114381 |
| R4-2114528 | Company A |
|  | Company B |
| R4-2114535 | Company A |
|  | Company B |
| R4-2112976 (3GPP TS 38.151 v0.5.0) | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic 1-1 Base Station beam configuration | Issue 1-1: Base Station beam configuration Draft revision of R4-2114381 was harmonized between Keysight, vivo, CAICT, Huawei. Tentative Agreements: - The base Station beam configuration part in REV of R4-2114381 CM Clarifications_v2.docx is agreeable. Recommendations for 2nd round: Finalize the single merged TP to capture above agreements. |
| Sub-topic 1-2 Channel model validation for FR1 | Issue 1-2: Reference validation targets for FR1 For CDL-C UMa, companies are supportive to reference channel emulation curves in R4-2114383. For CDL-C UMi, offline alignment is ongoing and further harmonization on spatial correlation is expected. Tentative agreements: - For CDL-C UMa, reference channel emulation curves in R4-2114383 is endorsed. Recommendations for 2nd round: Further discuss CDL-UMi and capture aligned parts into revision of R4-2114383. |
| Sub-topic 1-3 FR2 probe blocking issue | Issue 1-3: FR2 probe blocking issue 7 companies are supportive to close this issue, 1 company would like to keep it open for better understanding of blocking issue in the context of multiple system implementations. Recommendations for 2nd round: Further discuss on this topic in WF discussion. |
| Sub-topic 1-4 FR2 probe weights | Issue 1-4: FR2 probe weights 3 companies shared views on this issue. 2 companies commented that PSP based on the proposed probe weights should be simulated to match previous results. Huawei points out that different probe weights among labs may lead to performance variation in conformance test. Apple also points out that correlation between simulation and measurement should be considered. Tentative agreements: - NA Recommendations for 2nd round: Further discuss on this topic in 2nd round. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2112977 | agreeable |
| R4-2112978 | merged |
| R4-2112979 | agreeable |
| R4-2114381 | To be revised |
| R4-2114528 | merged |
| R4-2114535 | merged |
| R4-2112976 | Return to |

Discussion on 2nd round

Sub-topic 1-2 Channel model validation for FR1

Issue 1-2: Reference validation targets for FR1

- Background

  - for CDL-C Uma, reference channel emulation curves in R4-2114383 was endorsed in 1st round

  - for CDL-C Umi, interested  companies have aligned on PDP, Autocorrelation, V/H ratio except spatial correlation

- Proposal

  - Have another round of harmonization to minimize the differences in spatial correlation (Spirent).

- Recommended WF

  - further discuss on spatial correlation, and capture available agreements for both CDL-C UMa and CDL-C UMi into R4-2115759 (REV of R4-2114383)

| Company | Comments |
| --- | --- |
| Keysight | The goal is indeed to revise R4-2114383 to include CDL-C UMi reference curves (based on the averages from KS, Spirent, CMCC). A draft will be provided shortly |
| CAICT | We agree with Keysight. We target to decide the reference curves for both CDL-C Uma and CDL-C Umi in this meeting, including the spatial correlation for CDL-C Umi. |

Sub-topic 1-4 FR2 probe weights

Issue 1-4: FR2 probe weights

- Proposals

  - Propose to go with the probe weight in R4-2114534 as RAN4 assumption. Or we are open to accept more input and analysis. (Huawei)

- Recommended WF

  - further discussion to address Huawei’s concern (chamber with even the similar PSP but different probe weight, UE could behave different performance level) is encouraged

| Company | Comments |
| --- | --- |

Sub-topic 1-5 Channel model validation for FR2

Issue 1-5: Channel model validation for FR2

- Proposals (R4-2114503)

  - Proposal 1:  whether to use the base station beam for FR2 channel model validation needs to be further clarified.

  - Proposal 2:  The near field to far-field conversion need to be explained in detail in D.3.4 FR2 PAS similarity percentage.

  - In addition, a TP to TS38.151 v0.4.0 on FR2 PAS similarity percentage was added to the appendix of the paper.

- Recommended WF

  - please companies comment on both proposal and TP

| Company | Comments |
| --- | --- |
| Keysight | On P1: The base station beam is emulated by the (radio) channel emulator. Therefore, base station beam effects are and must be included in the FR2 channel model validation procedure On P2: We agree and could write additional description for next meeting On TP: the TP is acceptable |
| Qualcomm | For proposal 1: We share the similar view with Keysight. The beam effect of BS should be included in the FR2 channel model validation procedure. We are OK with the TP |

Summary for 2nd round

Open issues

|  | Status summary |
| --- | --- |
| Sub-topic 1-2 Channel model validation for FR1 | Issue 1-2: Reference validation targets for FR1 - Agreements - For CDL-C UMa and CDL-C UMi, reference channel emulation curves in R4-2115759 is endorsed; - Companies are encouraged to bring proposals and analysis for pass limit of channel validation and target to conclude in Nov 2021 RAN4 meeting. - A check point for offline alignment among interested parties before November is encouraged - Given the reference emulation curves are based on the average approach from several companies, the pass/fail limit should consider different implementations. |
| Sub-topic 1-4 FR2 probe weights | Issue 1-4: FR2 probe weights - GTW Agreements - Given probe weights belongs to implementation, no further effort on alignment of probe weights. |
| Sub-topic 1-5 FR2 probe blocking issue | Issue 1-3: Channel model validation for FR2 The questions in the proposals have got answered. The TP in annex on FR2 PAS similarity percentage is agreeable and hence a formal TDoc R4-2115811 is allocated for this TP. |

CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2112976 (draft TS) | For email approval |
| R4-2115757 (from R4-2114381) | agreeable |
| R4-2115811 (TP from annex of discussion paper R4-2114503) | agreeable |

Topic #2: Performance requirement

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2112573 | Samsung | Discussion on FR1 performance requirements Proposal 1: for TRMS derivation based on CDF approach per-band, a higher percentile value than 85%-tile is expected and 95%-tile is a candidate. Observation 1: Uncertainty of CDF data processing can be reduced by increasing the minimum device number, or applying high order interpolation in data processing. Proposal 2: further discuss the interpolation method in CDF data processing, and one candidate is linear interpolation. |
| R4-2112980 | vivo,CAICT | Proposal on preliminary FR1 MIMO OTA MU value assessment Observation 1: For LTE SISO OTA and LTE MIMO OTA test, the example/estimation of measurement uncertainty is done in RAN4 before defining the TRP/TRS and TRMS requirements. Observation 2: The definition of LTE TRP/TRS and LTE MIMO OTA performance requirements, preliminary measurement uncertainty assessment and test tolerances is a package which is developed in RAN4, since each component directly impacts the UE RF core requirements. The final values are recommended to RAN5 via LS for OTA test requirements in [7][8]. Observation 3: Due to the workload and time sensitive issue, the FR2 OTA MU and TT are defined in RAN5 based on the agreed work split between RAN4 and RAN5. However, the example MU assessment for RF requirements MOP and REFSENS have still be defined in RAN4. Observation 4: NR MIMO OTA MU value assessment and potential TT has not been discussed in RAN4. Proposal: RAN4 should discuss the MU assessment for FR1 MIMO OTA, example expanded uncertainty should be analysed. |
| R4-2113312 | CAICT, OPPO | Time plan for FR1 lab alignment and requirement development Proposal: Adopt the above time plan and corresponding actions for FR1 MIMO OTA lab alignment and performance requirement development, and update the progress in the following RAN4 meetings. |
| R4-2113914 | OPPO | Refinement on lab alignment activity Proposal: Adopt the above PADs roaming scheme to guarantee the lab alignment activity performed effectively and on-time. |
| R4-2112245 | Qualcomm Incorporated | Discussion on FR2 MIMO OTA performance requirements Proposal 1: Specify maximum downlink power at the center of QZ rather than RS-EPRE for FR2 MIMO OTA testing. Proposal 2: RAN4 to agree -66dBm/120kHz as FR2 maximum downlink power for the frequency up to 43.5GHz. Proposal 3: RAN4 to agree the pass criterion for PC3 UE to be 18 or more test points meeting or greater than 70% maximum throughput. The additional criterion, e.g., 90% TP outage level is FFS. Proposal 4: If the UE could not meet the criterion in proposal 3 due to the limitation on the parameter of maximum downlink power, the measurement channel bandwidth can be revisited, e.g., from 100MHz to 50MHz, to achieve higher downlink Pmax power. The additional criterion, e.g., 90% TP outage level is FFS. Proposal 5: TE/CE vendors to share the variation range for AoA/ZoA, PAS, power, delay, etc., those impacting by 6 probes which is also necessary for FR2 channel validation. |
| R4-2113033 | MediaTek Beijing Inc. | Proposal on FR2 MIMO simulator alignment Proposal: Do a fundamental scenario simulation as Fig 1 firstly, for FR2 MIMO OTA simulator alignment. |
| R4-2114504 | Huawei, HiSilicon | Discussion on FR2 MIMO OTA simulation Proposal 1: If the time window severely affects the UE throughput, it is helpful for the channel simulator vendors to provide some useful information about the length of the time window. Proposal 2: CE/TE vendors are encouraged to provide helpful information for FR2 MIMO OTA simulation. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 2-1 Time plan for FR1 lab alignment and requirement development

In contribution R4-2113312, a time plan for FR1 lab alignment and requirement development is provided:

| Lab alignment & requirement development activities | Action required | Action deadline |
| --- | --- | --- |
| Lab volunteer application | Lab volunteer submits application to Rapporteur | 2021/10/15 |
| PAD list frozen | RAN4 decides PAD list from PAD candidates | 2021/11/12 |
| Channel model validation | Lab volunteer submits channel model validation results to RAN4 for review | 2022/1/14 |
| Performance alignment start | PADs arrive at lab volunteers | 2022/1/14 Note: Performance alignment can start before the date once the lab volunteer is approved |
| Performance alignment finish | Lab volunteers should finish the PADs measurement in 7 days and send the measurement results to Rapporteur | 2022/3/31 |
| Performance alignment result review | RAN4 review the performance alignment results | RAN4 #102-bis-e |
| FR1 MIMO OTA measurement data collection | Aligned labs share results into data pool | RAN4 #103-e |

Lab volunteers and Performance alignment device (PAD) providers should note the deadline.

Issue 2-1: Time plan for FR1 lab alignment and requirement development

- Proposals

  - Adopt the above time plan and corresponding actions for FR1 MIMO OTA lab alignment and performance requirement development, and update the progress in the following RAN4 meetings.

- Recommended WF

  - TBA

Sub-topic 2-2 measurement device handling

Issue 2-2: measurement device handling

- Proposals

  - Adopt the PADs roaming scheme to guarantee the lab alignment activity performed effectively and on-time. (R4-2113914)

    - Every PAD should be measured in three laboratory volunteers or more;

    - Every delivery of PADs (including customs clearance) should be not extend 14 days;

    - Every PAD should have a backup sample which will be measured in the first lab volunteer and then delivered together with the PAD during the following round robin test.

  - other

- Recommended WF

  - It is encouraged to further discuss on measurement device handling in lab alignment and test campaign.

Sub-topic 2-3 data processing in FR1 TRMS derivation

Issue 2-3-1: percentile value of CDF

- Proposals

  - For TRMS derivation based on CDF approach per-band, a higher percentile value than 85%-tile is expected and 95%-tile is a candidate. (R4-2112573)

  - other

- Recommended WF

  - TBA.

Issue 2-3-2: interpolation method in CDF data processing

- Proposals

  - Further discuss the interpolation method in CDF data processing, and one candidate is linear interpolation. (R4-2112573)

  - other

- Recommended WF

  - TBA.

Sub-topic 2-4 FR1 MU assessment

Issue 2-4: FR1 MU assessment

- Proposals

  - RAN4 should discuss the MU assessment for FR1 MIMO OTA, example expanded uncertainty should be analysed (R4-2112980)

  - other

- Recommended WF

  - TBA.

Sub-topic 2-5 FR2 maximum downlink power and Figure of Merit

Maximum downlink power and Figure of Merit are correlated, so it is better to be discussed under the same sub-topic.

Issue 2-5-1: FR2 maximum downlink power

- Proposals

  - Proposal 1: Specify maximum downlink power at the center of QZ rather than RS-EPRE for FR2 MIMO OTA testing. (R4-2112245)

  - Proposal 2: RAN4 to agree -66dBm/120kHz as FR2 maximum downlink power for the frequency up to 43.5GHz. (R4-2112245)

  - other

- Recommended WF

  - TBA.

Issue 2-5-2: FR2 Figure of Merit

- Proposals

  - Proposal 3: RAN4 to agree the pass criterion for PC3 UE to be 18 or more test points meeting or greater than 70% maximum throughput. The additional criterion, e.g., 90% TP outage level is FFS. (R4-2112245)

  - Proposal 4: If the UE could not meet the criterion in proposal 3 due to the limitation on the parameter of maximum downlink power, the measurement channel bandwidth can be revisited, e.g., from 100MHz to 50MHz, to achieve higher downlink Pmax power. The additional criterion, e.g., 90% TP outage level is FFS. (R4-2112245)

  - other

- Recommended WF

  - TBA.

Sub-topic 2-6 FR2 simulation

Issue 2-6-1: information for FR2 simulation

- Proposals

  - Proposal 1:  If the time window severely affects the UE throughput, it is helpful for the channel simulator vendors to provide some useful information about the length of the time window. (R4-2114504)

  - Proposal 2:  CE/TE vendors are encouraged to provide helpful information for FR2 MIMO OTA simulation. (R4-2114504)

  - Proposal 3: TE/CE vendors to share the variation range for AoA/ZoA, PAS, power, delay, etc., those impacting by 6 probes which is also necessary for FR2 channel validation. (R4-2112245)

  - other

- Recommended WF

  - TBA.

Issue 2-6-2: FR2 simulator alignment

- Proposals

  - Proposal: Do a fundamental scenario simulation as Fig 1 firstly, for FR2 MIMO OTA simulator alignment. (R4-2113033)

  - other

- Recommended WF

  - TBA.

Companies views’ collection for 1st round

Open issues

Sub topic 2-1 Time plan for FR1 lab alignment and requirement development

| Company | Comments |
| --- | --- |
| OPPO | Support the proposal. |
| CMCC | Support the proposal. |
| CAICT | Support the proposal. Another issue is when collecting the measurement data, how to avoid the same UE model being measured in several test labs. It will affect defining requirements. A possible solution is to share the device list. Further discussion is suggested. |
| Samsung | Support the proposal. Agree with CAICT on the consideration of device. In requirement stage, device allocation among labs should be considered. Time plan information for device related is also encouraged. |
| vivo | It is a good idea to define a clear time plan for lab alignment and measurement results collection activity. Some detailed aspects should be considered to make the whole progress transparent and controllable. - Rapporteur initiate the lab alignment activity in the NR MIMO OTA reflector to call for volunteers for lab alignment, interested companies provide feedback in the reflector. - Interested companies share volunteered device for alignment activity in the NR MIMO OTA reflector, frozen the PAD list. - The channel model validation limits should be defined first, if the channel model validation results can not meet the criteria, the test lab can not join the lab alignment activity. - Delivery the PAD to each permitted test lab, the progress in each lab should be shared in the NR MIM OTA reflector for easy tracing, when PAD comes in and testing is finalized. - Based on previous experience, the PAD measurement results should NOT be shared to anyone, before submitting to the following RAN4 meeting. Comparison and alignment analyses should only be done in RAN4 meeting. - In case, labs fail to reach alignment for specific test case or test band, there should be one chance for the lab to optimize/check the test system for re-alignment. So one more meeting cycle for failed lab is allowed. Further discuss how to select the device for final measurement results collection, avoid that the same UE model is measured in several test labs and the performance plays dominate role in the CDF cure which is analyzed just from limited number (i.e. 15) of devices. |
| Xiaomi | Support the proposal. |
| CAICT | Response to vivo: Thanks for your constructive comments. We will carefully consider the comments from different companies and refine the time plan. We agree to allow one more chance for the failed labs to the check the test system for re-alignment, but meanwhile the aligned labs can proceed to the next procedure. Each action deadline should remain unchanged to avoid the whole progress being affected by the failed labs. |

Sub topic 2-2 measurement device handling

| Company | Comments |
| --- | --- |
| OPPO | Support the proposal. The delivery of PADs will be a big challenge for the round robin test under current situation. It should be carefully and seriously considered. |
| CMCC | Support this proposal, considering the efficiency. |
| CAICT | We support to adopt a PAD roaming scheme, but more details should be taken into account. E.g., It is difficult to determine how many lab volunteers can be permitted before the “Lab volunteer application” and “Channel model validation” stages. So it may not be guaranteed that each PAD can be measured in at least 3 lab volunteers. Thus, we suggest to further discuss the PAD roaming scheme and freeze it in the next RAN4 #101-e meeting. |
| Samsung | It is good to adopt a PAD roaming scheme. Ideally, the same PAD is better to be measured in all labs, but some trade-off may be also acceptable considering limited time. For the 14days delivery time, not sure if it is a controllable item. Agree to also prepare back up PAD but it should be carefully distinguished between reference UE and backup UE. |
| vivo | The intension is good and helpful. Several aspects need to consider: - How many labs can join the lab alignment activity depends on the channel model validation results. Prefer not limit the number of labs at this stage, further discuss the minimum number of labs for alignment activity, when the channel model validation results are submitted and checked. - For each PAD, there should NO backup sample selected, to ensure the same UE is measured in each lab. If something happens for a device in any of the test labs, e.g. can not connect or test, then that device is removed out of the PAD list finally. |
| Apple | Support the proposal. If PADs allows, preliminary conducted measurements done on reference lab will ensure PADs stability, and provide reference data for additional volunteer labs to investigate conducted performance drift prior to OTA measurements |

Sub topic 2-3	Sub-topic 2-3 data processing in FR1 TRMS derivation

| Company | Comments |
| --- | --- |
| OPPO | Issue 2-3-1: percentile value of CDF The analysis in R4-2112573 is reasonable. Therefore, higher percentile value is expected. The proper XX%-tile can be further discussed. Issue 2-3-2: interpolation method in CDF data processing Linear interpolation is acceptable for CDF data processing. |
| MediaTek | Issue 2-3-1: percentile value of CDF The proposal concept is made sense. Issue 2-3-2: interpolation method in CDF data processing The proposal concept is made sense. |
| Huawei, Hisilicon | Issue 2-3-1: percentile value of CDF Support the proposal to have higher percentile value than 85%-tile, 95%-tile is a good candidate |
| CAICT | Issue 2-3-1: percentile value of CDF Considering the lack of enough measurement results at present, we suggest to further discuss the percentile value of CDF. Issue 2-3-2: interpolation method in CDF data processing Support the proposal. |
| Samsung | Issue 2-3-1: percentile value of CDF Support the proposal to have higher percentile value than 85%-tile, 95%-tile is a good candidate. Issue 2-3-2: interpolation method in CDF data processing Support the proposal and linear interpolation is acceptable |
| vivo | Issue 2-3-1: percentile value of CDF It is good to consider the potential percentile of the CDF, however it is so early to make decision before reviewing the measurement results. Issue 2-3-2: interpolation method in CDF data processing Linear interpolation is a good starting point. |

Sub topic 2-4 FR1 MU assessment

| Company | Comments |
| --- | --- |
| OPPO | Support the proposal. The MU assessment should be discussed in RAN4. |
| Keysight | The WID states: “The Measurement Uncertainty (MU) aspects, including potentially test tolerances, and test procedures will be handled in RAN WG5.” and it should therefore be understood that the previous approach, i.e., determine MU and TT in RAN4 and suggest RAN5 to implement those, is no longer applicable. However, a very preliminary MU assessment could be considered in RAN4 with the understanding that RAN5 is responsible to work on and finalize the MU/TT. |
| Huawei, Hisilicon | Support the proposal. The common understanding of MU/TT is important for requirement discussion. We had proposed the same at the beginning of WI, still support. |
| CAICT | Support the proposal to discuss the MU assessment in RAN4. |
| Samsung | We also support the proposal given MU is correlated with requirement. |
| vivo | Given RAN5 is responsible for the Conformance testing spec, indeed, the final MU value and TT is decided by RAN5. However, as discussed in the paper R4-2112980, the example MU assessment and potential assumed TT should be discussed in RAN4 before companies decide the final TRMS requirements. The WID would need to be updated to reflect this aspect. |
| Apple | Support Keysight comments |
| Keysight | While we agree that the preliminary MUs might be valuable to get an idea of the approximate MU of test systems for NR FR1 MIMO OTA, we do not agree that RAN4 should discuss (potential) TTs as this is clearly a RAN5 responsibility. The objective for MU/TT discussions to be held in RAN4 stated above and in the corresponding contribution, i.e., to correlate the TRMS requirement with MU/TT, seems to indicate that the MU is taken into account twice in the requirements discussions: once for the core requirement in RAN4 and again for the test requirement in RAN5. That should be avoided unless we provide guidance to RAN5 that the core requirements already took MU/TT into account. Maybe some clarification could be provided how the MU is planned to be used for requirement definitions in RAN4? |

Sub topic 2-5 FR2 maximum downlink power and Figure of Merit

| Company | Comments |
| --- | --- |
| Keysight | Issue 2-5-1: At this point, we cannot agree to the proposed max DL power in proposal 2 based on offline discussions with QC on the missing cable losses and an adjusted backoff with fading. |
| Qualcomm | Issue 2-5-1: FR2 maximum downlink power We are OK to consider the additional cable losses of -8dB. For the backoff, now we are referring to the value in specified TR38810. The input from companies on the backoff are welcome. Issue 2-5-2: FR2 Figure of Merit The FR2 MIMO OTA requirements were agreed to calculate with 18 test points for PC3 so we support proposal 1 and proposal 2. We have already discussed FR2 FoM for many meetings. It is encouraged to have the agreement for FR2 FoM. |
| MediaTek | Issue 2-5-2: FR2 Figure of Merit To Qualcomm: Could you clarify proposal4 meaning? Does it mean that we may define “channel bandwidth is 50MHz directly”, while we observe feasible maximum downlink power is too low, during test method/requirement discussion stage? |
| Samsung | Issue 2-5-1: FR2 maximum downlink power Agree with Qualcomm that maximum downlink power is specified at centre of QZ, it seems the same as FR1. Further discussion is needed whether to change the EPRE term. Issue 2-5-2: FR2 Figure of Merit Our understanding is that FoM depends on the finalization of FR2 maximum downlink power. |
| Huawei, HiSilicon | Issue 2-5-1: FR2 maximum downlink power We are fine to further discuss max DL power based on -66dBm/120kHz at center of QZ. For the cable loss, more input and analysis from TE vendor is encouraged . Issue 2-5-2: FR2 Figure of Merit Agree with Samsung. |
| Xiaomi | Issue 2-5-1: FR2 maximum downlink power Agree to specify the max downlink power at centre of QZ. How to translate this with EPRE term needs further study. Issue 2-5-2: FR2 Figure of Merit Agree that to finalize the max downlink power first and then further discuss the FoM. |
| Keysight | Issue 2-5-1: FR2 maximum downlink power We hope to provide additional details/feedback regarding the max DL power in round 2; we just cannot agree with the current proposal since the cable losses have not been taken into account and since the backoff is only for a modulated but unfaded signal. |

Sub topic 2-6 FR2 simulation

| Company | Comments |
| --- | --- |
| OPPO | Specifying scenario simulation is helpful to alignment. Detail discussion is needed. |
| Qualcomm | Issue 2-6-1: information for FR2 simulation We support proposal 2 and 3. It would be helpful to move forward if we can get the confirmation from TE/CE vendors in this meeting. Issue 2-6-2: FR2 simulator alignment Agree with the proposal. The simulation results in our paper of R4-2112245 is in line with the proposed scenario. |
| MediaTek | Issue 2-6-1: information for FR2 simulation In principle, we prefer to have more complete condition definition for simulation alignment purpose. Hence, we think all proposal 1 & 2 & 3 are fine in simulator alignment stage. Issue 2-6-2: FR2 simulator alignment As proponent, we support the proposal. If interested companies can do this fundamental scenario simulation, it would be easier to clarify & mitigate potential difference. The whole simulator is really a large system with many parameters and assumptions. And we are glad to know R4-2112245 is already in line with the proposed scenario. |
| Huawei, HiSilicon | Issue 2-6-1: information for FR2 simulation Both Proposal 2 and Proposal 3 illustrate the importance of TE/CE vendors inputs. We provide probe weight in test methodology section. If no information could be provided by TE vendors, we propose to use the weights in R4-2114534, and also apply test assumption with the weighs which can be added into TS 38.151. Thus, the test and simulation can be highly aligned. And we believe it can accelerate simulation work. For Proposal 1, feedback from CE vendors about the time window is expected. A clarifying question to QC: how long is the time window for the simulation results in R4-2114504? Issue 2-6-2: FR2 simulator alignment Agree with MediaTek. |
| Apple | Issue 2-6-2: FR2 simulation alignment Support to start a simulation campaign with basic fundamental model, to build model maturity and confidence equivalent basic measurements could be used for baseline simulation/measurement correlation. |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic 2-1 Time plan | Issue 2-1: Time plan for FR1 lab alignment and requirement development Most companies support the proposed time plan. Some constructive suggestions are also provided by some companies. Recommendations for 2nd round: Revise the time plan to address the comments collected. Target is to agree the time plan in this meeting. |
| Sub-topic 2-2 measurement device handling | Issue 2-2: measurement device handling Most companies support the proposed PAD roaming scheme, while some constructive suggestion are also provided by some companies. Recommendations for 2nd round: Further discuss this topic in WF discussion. |
| Sub-topic 2-3 data processing in FR1 TRMS derivation | Issue 2-3-1: percentile value of CDF 6 companies commented to this issue. 4 companies are supportive to define a higher percentile value than 85%-tile. 2 companies commented that it is too early to make decision. Tentative agreements: - further study the percentile value of CDF Recommendations for 2nd round: Try to capture tentative agreement and possible candidate values in WF, and further discuss whether the percentile value should be defined before test campaign. Issue 2-3-2: interpolation method in CDF data processing Most companies are supportive to adopt linear interpolation at least as a starting point. Tentative agreements: - linear interpolation as starting point for CDF data processing Recommendations for 2nd round: Further discuss this topic in WF |
| Sub-topic 2-4 FR1 MU assessment | Issue 2-4: FR1 MU assessment 7 companies shared views on this issue. 5 companies are supportive to define MU in RAN4, while 2 companies prefer that RAN4 only define preliminary MU, and final MU and TT will be handled by RAN5. Tentative agreements: - At least preliminary MU assessment shall be done in RAN4 Recommendations for 2nd round: Further discuss on this topic in GTW and WF. |
| Sub-topic 2-5 FR2 maximum downlink power and Figure of Merit | Issue 2-5-1: FR2 maximum downlink power Companies are supportive to define maximum downlink power at center of QZ. About detailed value, more input on cable losses and an adjusted backoff with fading are needed. Tentative agreements: - maximum downlink power shall be specified at centre of QZ Recommendations for 2nd round: Further discuss on this topic in 2nd round. TE vendors’ input on cable losses and an adjusted backoff with fading are encouraged. Issue 2-5-2: FR2 Figure of Merit Many companies point out that it is necessary to finalize the max downlink power first and then further discuss the FoM. Tentative agreements: - finalize the max downlink power first and then further discuss the FoM Recommendations for 2nd round: Further discuss on above tentative agreement to check if it is agreeable. |
| Sub-topic 2-6 FR2 simulation | Issue 2-6-1: information for FR2 simulation Companies commented that more information inputs are needed for FR2 simulation, including the length of time window, variation range for AoA/ZoA, PAS, power, delay, etc. No input from TE/CE vendors yet. Tentative agreements: - NA Recommendations for 2nd round: Keep this issue open in 2nd round. TE/CE vendors’ involvement is encouraged. Issue 2-6-2: FR2 simulator alignment Companies are all supportive to MediaTek’s proposal. Tentative agreements: - Do a fundamental scenario simulation as Fig 1 of R4-2113033 firstly, for FR2 MIMO OTA simulator alignment. Recommendations for 2nd round: Confirm above tentative agreement in WF. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Sub-topic 2-1 Time plan for FR1 lab alignment and requirement development

Issue 2-1: Time plan for FR1 lab alignment and requirement development

- Background

  - A revised time plan will be provided by CAICT and OPPO. So R4-2113312 will be revised to R4-2115758 to address companies’ comments.

- Recommended WF

  - Comments to the REV of R4-2113312 are encouraged. The target is to approve the initial time plan in R4-2115758 in this meeting

| Company | Comments |
| --- | --- |

Sub-topic 2-3 data processing in FR1 TRMS derivation

Issue 2-3-1: percentile value of CDF

- whether the percentile value should be defined before test campaign (from moderator base on 1st round comments)

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - TBA.

| Company | Comments |
| --- | --- |

Sub-topic 2-4 FR1 MU assessment

Issue 2-4: FR1 MU assessment

- Proposals

  - Option 1: RAN4 should discuss the MU assessment for FR1 MIMO OTA, example expanded uncertainty should be analysed (R4-2112980)

  - Option 2: a very preliminary MU assessment could be considered in RAN4 with the understanding that RAN5 is responsible to work on and finalize the MU/TT.

- Recommended WF

  - Clarifications are encouraged on how the MU is planned to be used for requirement definitions in RAN4.

| Company | Comments |
| --- | --- |

Sub-topic 2-5 FR2 maximum downlink power and Figure of Merit

Issue 2-5-1: FR2 maximum downlink power

- Proposals

  - Proposal 1: Specify maximum downlink power at the center of QZ rather than RS-EPRE for FR2 MIMO OTA testing. (R4-2112245)

  - Proposal 2: RAN4 to agree -66dBm/120kHz as FR2 maximum downlink power for the frequency up to 43.5GHz. (R4-2112245)

  - other

- Recommended WF

  - Further discussion is welcomed, and TE vendors’ input on cable losses and an adjusted backoff with fading are encouraged.

Issue 2-5-2: FR2 Figure of Merit

- If the tentative agreement “finalize the max downlink power first and then further discuss the FoM” is agreeable

  - Option 1: Yes, agreeable

  - Option 2: No, not agreeable.

- Recommended WF

  - TBA.

| Company | Comments |
| --- | --- |

Sub-topic 2-6 FR2 simulation

Issue 2-6-1: information for FR2 simulation

- Proposals

  - Proposal 1:  If the time window severely affects the UE throughput, it is helpful for the channel simulator vendors to provide some useful information about the length of the time window. (R4-2114504)

  - Proposal 2:  CE/TE vendors are encouraged to provide helpful information for FR2 MIMO OTA simulation. (R4-2114504)

  - Proposal 3: TE/CE vendors to share the variation range for AoA/ZoA, PAS, power, delay, etc., those impacting by 6 probes which is also necessary for FR2 channel validation. (R4-2112245)

  - other

- Recommended WF

  - Comments from TE/CE vendors are encouraged.

| Company | Comments |
| --- | --- |
| Keysight | We believe the definition of the FR2 requirements should focus more on measurements instead of simulations |
| vivo | Support the comments from Keysight. |
| Qualcomm | FR2 requirements could definitely be define with measurement but due to the lack of supporting devices. Simulation approach was also agreed as feasible approach to derive the FR2 MIMO OTA requirements. In addition, the simulation approach was used to derive the FR2 requirements such as EIS, EIRP, etc. in rel-15. We have agreed the simulation assumptions. In this meeting, the scenario is agreed as well. We suggest triggering the simulation calibration by offline before Nov meeting. To response Huawei’s comment from the first round “A clarifying question to QC: how long is the time window for the simulation results in R4-2114504?” In our paper of R4-2112245, we run the simulation for 10s as the time window. |
| MediaTek | Both simulation and measurement approaches shall be considered as agreed. |

Summary for 2nd round

Open issues

|  | Status summary |
| --- | --- |
| Sub-topic 2-1 Time plan | Issue 2-1: Time plan for FR1 lab alignment and requirement development - Agreements - The Time plan for FR1 lab alignment and requirement development in R4-2115758 is agreed. |
| Sub-topic 2-3 data processing in FR1 TRMS derivation | Issue 2-3-1: percentile value of CDF - Agreements - The percentile value of CDF to derive final TRMS requirements will be discussed after lab alignment is finalized and before test campaign. |
| Sub-topic 2-4 FR1 MU assessment | Issue 2-4: FR1 MU assessment - Agreements - RAN4 should discuss the preliminary MU assessment for FR1 MIMO OTA including example expanded uncertainty, final MU and TT will be decided by RAN5. - WID is suggested to update in Sep RAN plenary meeting, to reflect this working scope extension, for easy tracing the progress in the status report of WI. |
| Sub-topic 2-5 FR2 maximum downlink power and Figure of Merit | Issue 2-5-1: FR2 maximum downlink power - Agreements - Maximum downlink power shall be specified at centre of QZ. - Companies’ input on this topic is encouraged. - Finalize max DL power level at next meeting Issue 2-5-2: FR2 Figure of Merit - Agreements - The FoM will be further discussed after the max downlink power is finalized. |
| Sub-topic 2-6 FR2 simulation | Issue 2-6-1: information for FR2 simulation - Agreement - RAN4 should identify which information (except for FR2 probe weights) should be further provided and aligned to accelerate FR2 simulation activity. |

CRs/TPs

N.A.

Topic #3: TR38.827 maintance

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2112981 | vivo | Draft CR to TR38.827:correct Positioning ambiguities |
| R4-2112982 | vivo | Draft CR to TR38.827:power validation |

Open issues summary

No open issues. Please comment to section 3.3.2 directly.

Companies views’ collection for 1st round

Open issues

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2112981 | Apple - Overlap with KS R4-2114381. Such editorial change doesn’t seems to help to reduce the original ambiguity on TR 38.827, A.3. The Figure A.3-1 has arbitrary reduced ripple regions. Understanding that there are no consensus among system vendor manufacturers on the embodiment or material used for the positioner. And no manufacturer declaration about the DUT antenna system placement/topology is mentioned. Improved clarification covering such aspects might be needed. |
|  | Keysight: the overlap has been resolved in the revision of R4-2114381 |
| R4-2112982 | Apple - Needs clarification on vivo’s motivation to change the Table 7.4.1-1 title removing reference to NR FR1 Quiet Zone Validation while maintaining the original NR FR2 respective table title on 7.4.1-2. A direct reference to quite zone validation is a straight forward and important concept at low FR1 frequencies. Edition on step 6 “horizontally polarized sleeve dipole measured in at least four orthogonal horizontal positions”. A H pol sleeve dipole can only be placed on 2 different orthogonal positions, unless the objective is to investigate ripple due sleeve dipole manufacturing tolerances and/or placement in the chamber |
|  | Company B |

Summary for 1st round

Open issues

N.A.

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2112981 | To be revised |
| R4-2112982 | To be revised |

Discussion on 2nd round

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2115760 (REV of R4-2112981) | vivo: response to apple This the CR for 38.827, the content will be aligned with the revision of R4-2114381 from Keysight for TS 38.151. |
|  | Company B |
| R4-2115761 (REV of R4-2112982) | vivo: response to apple Thanks for the comments. The Quiet Zone Validation in title was mistakenly removed, has been corrected in the revision. Regarding the second comment for step 6, this was previous agreements in the WF R4-2106092. This CR just implements the agreements into TR. |
|  | Company B |

Summary for 2nd round

Open issues

N.A.

CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2115760 (from R4-2112981) | Agreeable |
| R4-2115761 (from R4-2112982) | Agreeable |

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on NR MIMO OTA | vivo, CAICT |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2112245 | Discussion on FR2 MIMO OTA performance requirements | Qualcomm Incorporated | Noted |  |
| R4-2112573 | Discussion on FR1 performance requirements | Samsung | Noted |  |
| R4-2112862 | Consideration on Probe#3 of FR2 MIMO OTA | CMCC | Noted |  |
| R4-2112976 | 3GPP TS 38.151 v0.5.0 | vivo | For email approval | draft TS |
| R4-2112977 | Rapporteur input to TS38.151 | vivo | Agreeable |  |
| R4-2112978 | TP to TS38.151 on BS beam configuration | vivo,CAICT | Merged |  |
| R4-2112979 | TP to TS38.151 on Minimum Number of Slots and Power Control | vivo | Agreeable |  |
| R4-2112980 | Proposal on preliminary FR1 MIMO OTA MU value assessment | vivo,CAICT | Noted |  |
| R4-2113033 | Proposal on FR2 MIMO simulator alignment | MediaTek Beijing Inc. | Noted |  |
| R4-2113312 | Time plan for FR1 lab alignment and requirement development | CAICT, OPPO | Revised |  |
| R4-2113854 | Channel Model Targets | Spirent Communications | Noted |  |
| R4-2113858 | Reference Channel Emulation Curves for FR1 | CAICT, CMCC | Noted |  |
| R4-2113914 | Refinement on lab alignment activity | OPPO | Noted |  |
| R4-2113915 | The FR2 blocking issue | OPPO | Noted |  |
| R4-2114025 | Reference curves for FR1 CDL-C Uma | CAICT, Keysight Technologies UK Ltd, Spirent Communications, CMCC | Withdrawn |  |
| R4-2114380 | On Blocking Issue for FR2 MIMO OTA | Keysight Technologies UK Ltd | Noted |  |
| R4-2114381 | TP on Channel Model and DUT Positioning Clarifications | Keysight Technologies UK Ltd | Revised |  |
| R4-2114382 | Reference Channel Emulation Curves | Keysight Technologies UK Ltd | Noted |  |
| R4-2114383 | Reference Channel Emulation Curves for Validation Purposes | Keysight Technologies UK Ltd, Spirent Communications, CMCC, CAICT | Revised | To be revised to capture more agreements |
| R4-2114503 | Discussion on FR2 channel model validation | Huawei, HiSilicon | Noted | TP from annex of discussion paper R4-2114503 is agreeable and captured in new TDoc R4-2115811 |
| R4-21121145 | Discussion on FR2 MIMO OTA simulation | Huawei, HiSilicon | Noted |  |
| R4-2114528 | TP to TS 38.151 on FR1 2x2 BS beam selection | Huawei, HiSilicon | Merged |  |
| R4-2114529 | update simulation results on FR1 2x2 channel models | Huawei, HiSilicon | Noted |  |
| R4-2114534 | Discussion on probe weight | Huawei, HiSilicon | Noted |  |
| R4-2114535 | TP to TS38.151 v0.4.0 on FR2 Base Station beam configuration | Huawei, HiSilicon | Merged |  |
| R4-2112981 | Draft CR to TR38.827:correct Positioning ambiguities | vivo | Revised |  |
| R4-2112982 | Draft CR to TR38.827:power validation | vivo | Revised |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2115756 | WF on NR MIMO OTA | vivo, CAICT | Agreeable |  |
| R4-2115757 | TP on Channel Model and DUT Positioning Clarifications | Keysight Technologies UK Ltd | Agreeable |  |
| R4-2115758 | Time plan for FR1 lab alignment and requirement development | CAICT, OPPO | Agreeable |  |
| R4-2115759 | Reference Channel Emulation Curves for Validation Purposes | Keysight Technologies UK Ltd, Spirent Communications, CMCC, CAICT | Agreeable |  |
| R4-2115760 | Draft CR to TR38.827:correct Positioning ambiguities | Vivo | Agreeable |  |
| R4-2115761 | Draft CR to TR38.827:power validation | Vivo | Agreeable |  |
| R4-2115811 | TP to TS38.151 on FR2 PAS similarity percentage | Huawei | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| MediaTek Inc. | Ting-Wei Kang | ting-wei.kang@mediatek.com |
| CAICT | Xuan Yi Siting Zhu | yixuan@caict.ac.cn zhusiting@caict.ac.cn |
| Spirent Communications | Alfonso Rodriguez-Herrera | Alfonso.Rodriguez@spirent.com |
| Samsung | Bozhi Li | bozhi.li@samsung.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)